3GPP TSG-RAN WG2 #127bis	R2-2408816

# Hefei, China, 14-18 October 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.300 | CR | 0918 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Coexistence of NCR, LTM and CPAC with (e)RedCap |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NetConRepeater, NR_Mob_enh2-Core, NR_redcap_enh-Core |  |  |  |  |  | Date: |  |  | 2024-10-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | According to section 16.13.2, it seems that it is possible for a eRedCap UE to perform LTM and CPAC procedures, but this is not according to what has been agreed during Rel-18. Also, from the same section it seems that it is possible for a eRedCap UE to be an NCR node, but this is not according to what has been agreed during Rel-18. The NCR node is very much like IAB node and it makes sense to capture the same restriction for NCR, same as there is for IAB |  |  |  |  |  |  |  |  |
| Summary of change: |  | Section 16.13.2 - Clarified that an (e)RedCap UE cannot perform LTM and CPAC procedures. - Clarified that an (e)RedCap UE cannot be an NCR node. Impact Analysis Impacted 5G architecture options: NR SA Impacted functionality: (e)RedCap, LTM, CPAC, and NCR Inter-operability: 1. If the network is implemented according to the CR and the UE is not, there is no inter-operability issue 2. If the UE is implemented according to the CR and the network is not, there is no inter-operability issue |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If the CR is not approved, an (e)RedCap UE may perform LTM and CPAC procedure but this is not according to what has been agreed in Rel-18. Also, it seems that specification allows for an (e)RedCap UE to have NCR capabiltities, while this is not according to what has been agreed in Rel-18. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 16.13.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGES

### 16.13.2	Capabilities

CA, MR-DC, DAPS, CPA, CPC, CPAC, LTM, NCR and IAB related capabilities are not supported by (e)RedCap UEs, as defined together with other limitations in TS 38.306 [11]. It is up to the network to prevent (e)RedCap UEs from using radio capabilities not intended for (e)RedCap UEs.

END OF CHANGES