3GPP TSG-RAN WG2 #127bis	R2-2408820

# Hefei, China, 14th – 18th October 2024

Agenda Item:	7.4.2

Source:	Ericsson

Title:	Summary of RRC proposals for feMob

Document for:	Discussion

# 1	Introduction

In this contribution is provide a summary of the RRC proposals for FeMob with some possible conclusion for each of them. Please, note that stage 2 and capabilities corrections are left out from this document.

# 2	Discussion

Hereafter the contributions submitted to AI 7.4.2 are listed and a possible outcome for what is proposed is suggested:

| Tdoc # | Company | Proposal(s) | Suggested conclusion |
| --- | --- | --- | --- |
| R2-2408432 | Google | P1: For inter-DU LTM, shared preamble resources as agreed upon by RAN2 can be formed by combining SSB index, PRACH mask index, and preamble index. P2: The 'SSB Positions In Burst' parameter is essential for the source gNB-DU to calculate the RA-RNTI, which in turn allows for associating a TA value with a UE when RACH occasion differentiation is required. | P1: To be discussed. Preamble index should be enough for the target DU to identify to which source DU the TA value should be sent, but we can discuss if something else is needed. P2: To be discussed. The RA-RNTI is calculated by the target DU and is still questionable how this field can be used by the source DU. The source DU may not have any association between the RA-RNTI and the C-RNTI of the UE. |
| R2-2407993 | CATT | P1: On how to identify the power control parameters after LTM cell switch, RAN2 down selects between the following two solutions: - Solution 1: clarify in the field description of ul-powerControl-r18 in CandidateTCI-State and CandidateTCI-UL-State that it refers to the uplink-PowerControlToAddModList configured in the SpCell of the candidate configuration; - Solution 2: introduce uplink-PowerControlToAddModList-r18 and uplink-PowerControlToReleaseList-r18 in LTM-TCI-Info to align RAN1 LS, the ul-powerControl-r18 in CandidateTCI-State and CandidateTCI-UL-State refer to the power control parameters in LTM-TCI-Info; | P1: Not Pursued. Current field description seems quite clear. Also, what is proposed is not correct at the field ul-powerControl is part of the CandidateTCI-State within the LTM-TCI-Info IE. The ASN.1 structure is quite clear about this. Also, there is no need to implement any list if the power control parameters are provided within the CandidateTCI-State, which is what RAN2 has agreed. |
| R2-2407994 | CATT | P1: UE release/clear all current common radio configurations except for PCell upon subsequent CPAC execution. P2: Clarify in 5.3.5.13.8 that each srb-Identity included in RadioBearerConfig is referred to the selected subsequent CPAC candidate configuration. P3: Move the field description of securityCellSetId from ConditionalReconfiguration field descriptions to CondReconfigToAddMod field descriptions. | P1. To be agreed. Probably the best would be to align the procedural text to what we have for LTM. P2. Not Pursued. The procedural text seems fine and the change is anyway strange. How something that is in the CPAC candidate configuration be part of the current UE configuration? P3. Not Pursued. This is just a matter of taste, but current text works fine. |
| R2-2408126 | Vivo | P1: Propose to delte the following sentence in the s-measure field description: “This field is not applicable to L1 measurements configured with ltm-CSI-ResourceConfigToAddModList-r18.” | P1. Not Pursued. This was already discussed in last meeting, and not agreed. I would like to now repeat this discussion again. If there is some issue from RAN4 point if view, RAN4 can send RAN2 an LS. |
| R2-2408436 | Samsung | P1: RAN2 to confirm that the intra-SN subsequent CPAC and CHO can be configured together. P2: To avoid the mismatch in CHO confguration between UE an MN and potential RLF, when intra-SN subsequent CPAC and CHO are configured together, consider the below options: - UE keeps the CHO configuration after subsequent CPAC execution stored in SCG VarConditionalReconfig (preferred). - Introduce new inter-node signalling from SN to MN to inform that subsequent CPAC execution is performed for intra-SN CPC. P3: RAN2 is kindly asked to confirm that the shared preamble resource is referring the RACH resource (i.e., combination of multiple parameters) rather than preamble only. P4: The candidate gNB-DU needs provide SFN, where the preamble is transmitted, along with the RA-RNTI and the TA value for a specific UE. P5: The candidate gNB-DU needs provide the SSB position in burst to the source gNB-DU when providing the early UL sync configuration | P1: To be discussed maybe in Rel-17. This seems to be a co-existence issue which starts from Rel-17. Not sure if Rel-18 is the right place to discuss this. P2: See comment on P1. P3: To be discussed. Preamble index should be enough for the target DU to identify to which source DU the TA value should be sent, but we can discuss if something else is needed. P4: To be discussed. The RA-RNTI is calculated by the target DU and is still questionable how this field can be used by the source DU. The source DU may not have any association between the RA-RNTI and the C-RNTI of the UE. P5: See comment on P4. |
| R2-2408523 | ZTE, CATT, Ericsson | P1: From RAN2 perspective, CG-Config and CG-CandidateList are related to different configurations of intra-SN subsequent CPAC in MN format, but they are not required to must be sent together in the same SN Modification Required XnAP message. P2: RAN2 understands that two XnAP messages can be sent from the SN to the MN to transfer CG-Config message and CG-CandidateList messages separately. It requires some modification or clarification in the current stage-2 procedure. The detailed message and procedure can be up to RAN3 discussion. | P1: To be discussed P2: To be discussed |
| R2-2408678 | Google | P1: CR proposes to not support co-existence of LTM with CHO, CPAC, CPA, CPC, and DAPS | P1: Not pursued. The co-existence aspect of LTM with other mobility features have been already discussed. Restrictions have been already captured in stage 2 (whenever necessary). In general, RRC is not a good place to capture such kind of aspects. |
| R2-2408751 | Nokia | P1: The procedure text changes introduced in TS38.330 V 18.3.0 related to update of VarConditionalReconfig during SCPAC execution is modified to support subsequent CPA. The description for subsequentCondExecutionCond updated to enable the above P2: In SK-CounterConfiguration-r18 , the size of sk-CounterConfigToReleaseList-r18 and sk-CounterConfigToAddModList-r18 should be updated as 1..maxSecurityCellSet-r18-1 as follows P3: For the SCPAC scenario where initial serving SN/PSCell is not part of SCPAC , the Network should ensure that the servingCellSetID is set to a unique value different than the security-cell-set-ID of the candidates | P1. To be discussed. This part of the procedural text has been discussed again and again. Would be good to take a final conclusion on whether to do this change or not. P2: Not pursued, I guess 9 is because there are cases where the serving cell is not one of the subsequent CPAC configuration. P3: To be agreed, but is already the understanding. Not sure anything needs to be captured in RRC. |
| R2-2408818 | Ericsson | CR to be used to collect all agreed changes. |  |
| R2-2408819 | Ericsson | P1: The instance of “MN format” are spelled out since the terminology does not really any meaning. P2: In the LTM cell switch MAC CE the TCI state is a mandatory field and thus the field rrc-SSB-Subset it become meaninless when the CG is provided. P3: Some parameter introduced for when the TRS configuration is provided in LTM-TCI-Info needs to be repeated at every CSI-RS resources but in some cases, such as intra-frequency, this seems unnecessary. Would be good to give the opportunity to the network to re-used the fields which are already in SpCellConfig P4: Clarify in the Annex B2 that the RRCReconfiguration shall not be sent unprotected if it includes the LTM-Config. | P1: To be agreed. This is in line which the exercise already done in 37.340. P2: To be agreed. It seems a straightforward change given what has been agreed for the LTM cell switch MAC CE. P3. To be discussed. We would need to discuss if we want to allow some signalling to be saved or we stick to what we have at the moment. Would be nice anyway to not repeat continuously the same field for all the configured CSI-RS resources. P4. To be agreed. According to what is captured in RRC, this is already the case. The change seems straightforward. |
| R2-2408945 | Nokia | P1: RAN2 responds to Question 1 by stating no additional information is needed in RACH Configuration sent over F1AP. P2: RAN2 responds to Question 2 by stating ssb-PositionInBurst should be signalled in EarlyUL-SyncConfig over F1AP. P3: Stage 2 changes. P4: RAN2 to confirm the multiple LTM recovery attempts is an issue and to discuss possible ways to fix it in specifications. | P1: To be discussed. Preamble index should be enough for the target DU to identify to which source DU the TA value should be sent, but we can discuss if something else is needed. P2: To be discussed. The RA-RNTI is calculated by the target DU and is still questionable how this field can be used by the source DU. The source DU may not have any association between the RA-RNTI and the C-RNTI of the UE. P3: To be discussed in stage 2 discussions. P4: Not pursued. Current specification already limit the execution of the LTM fast RLF recovery to one attempt. Not sure if anything more is needed. |
| R2-2408258 | MediaTek | P1: RAN2 to clarify the understanding of maxNumberJointTCI-AcrossCells in FG45-3a: - Understanding 1: The volume of this component includes the legacy TCI state indicated by FG23-1-1 UnifiedJointTCI-r17, which represent a total maximum number of activated TCI states that UE can support. - Understanding 2: The volume of this component only includes LTM TCI states indicated in LTM-TCI-info. The value of UnifiedJointTCI-r17 is not taken into account P3: If understanding 1 is the common understanding, RAN2 should further discuss the solution to address the discontinuous value issue for maxNumberJointTCI-AcrossCells in FG45-3a. - Option 1: Change the value of maxNumberJointTCI-AcrossCells in FG45-3a from ENUMERATED {n1, n2, n3, n4, n8, n16, n32} to INTEGER (1..32). - Option 2.1: Add an new component in this IE to represent the maximum number of MAC-CE activated joint LTM TCI states across candidate cells. - Option 2.2: Add an new IE (new UE capability) to represent the maximum number of MAC-CE activated joint LTM TCI states across candidate cells. P4: Add a new field in LTM-Candidate IE to indicate the number of serving cells configured by this LTM candidate configuration. | P1: Wait for RAN1. Even if the Understanding 1 should be fine, RAN1 is planning to discussing and also update the value range of this capability. RAN2 should wait for inputs from RAN1. P3: See comment on P1. P4: Not pursued. I expect the UE capabilities to perform the ASN.1 early decoding and also the capabilities about how many cells can be stored to be agnostic of what is configured in the LTM candidate configuration. The UE reports this capabilities and the network should always provide a configuration which is according to what the UE is capable of. From this point of view, this is just an optimization but nothing which is strictly essential. |
| R2-2408325 | MediaTek | The CR seems to be empty, but I guess is related to the tdoc in R2-2408258 | Check comments for R2-2408258 |
| R2-2408452 | Vodafone, Ericsson | Changes proposed to stage 2 | To be addressed in the stage 2 discussions |
| R2-2408522 | ZTE, Ericsson | Changes proposed to stage 2 | To be addressed in the stage 2 discussions |
| R2-2408632 | Lenovo | Changes proposed to stage 2 | To be addressed in the stage 2 discussions |
| R2-2409137 | Huawei | P1: The UE that indicates support of interFreqL1-MeasConfig-r18 (R1 45-1a) can indicate that it can do L1 inter-frequency measurements of SSB in the serving BC, by indicating a new per UE capability interFreqL1-OnlyInBC-r18. P2: Discuss whether a candidate cell could belong to a band that is not the band of a serving cell but is overlapping in frequency with the band of a serving cell, and its SSB could be in the DL active BWP. P3: If this is possible, clarify that if the UE indicates interFreqL1-OnlyInBC-r18, interFreqSSB-L1-MeasWithoutGaps-r18 only applies to SSBs of candidate cells in the BC. P4: Discuss whether to capture that the UE that indicates support of interFreqL1-MeasConfig-r18 shall indicate support of interFreqSSB-L1-MeasWithoutGaps-r18 in the same BC or support of ltm-InterFreqMeasGap-r18 or both | P1-P4. To be discussed. This is an optimization to reduce signalling but in some cases, it seems that it could useful. Nevertheless, RAN2 needs to first to discuss whether to have this optimization at this point in time and also how to correctly set the capability bit that is proposed. I suppose this will not be an easy discussion but we can try to collect companies opinion if there is time. |

According to what is discussed above, the following proposals are made.

The following proposals are suggested to be agreed:

Subsequent CPAC

UE release/clear all current common radio configurations except for PCell upon subsequent CPAC execution.

Confirm that For the SCPAC scenario where initial serving SN/PSCell is not part of SCPAC , the Network should ensure that the servingCellSetID is set to a unique value different than the security-cell-set-ID of the candidates.

Instead of using “MN format” it is spelled out what this does mean in the RRC specification.

LTM

Clarify that the field rrc-SSB-Subset is not used for LTM in case a CG is configured.

Clarify in the Annex B2 that the RRCReconfiguration shall not be sent unprotected if it includes the LTM-Config.

The following proposals are suggested to be discussed ONLINE:

RAN2 to discuss, inter-DU LTM, whether shared preamble resources are supported and whether preamble index is enough or whether this is done by a combination of SSB index, PRACH mask index, and preamble index.

RAN2 to clarify whether the 'SSB Positions In Burst' parameter is essential for the source gNB-DU to calculate the RA-RNTI, which in turn allows for associating a TA value with a UE when RACH occasion differentiation is required.

RAN2 to clarify whether the candidate gNB-DU needs provide SFN, where the preamble is transmitted, along with the RA-RNTI and the TA value for a specific UE.

RAN2 to confirm that the intra-SN subsequent CPAC and CHO can be configured together.

If CPAC and CHO can be configured together, to avoid the mismatch in CHO confguration between UE an MN and potential RLF, when intra-SN subsequent CPAC and CHO are configured together, consider the below options:

  - UE keeps the CHO configuration after subsequent CPAC execution stored in SCG VarConditionalReconfig (preferred).

  - Introduce new inter-node signalling from SN to MN to inform that subsequent CPAC execution is performed for intra-SN CPC.

From RAN2 perspective, CG-Config and CG-CandidateList are related to different configurations of intra-SN subsequent CPAC in MN format, but they are not required to must be sent together in the same SN Modification Required XnAP message.

RAN2 understands that two XnAP messages can be sent from the SN to the MN to transfer CG-Config message and CG-CandidateList messages separately. It requires some modification or clarification in the current stage-2 procedure. The detailed message and procedure can be up to RAN3 discussion.

The procedure text changes introduced in TS38.330 V 18.3.0 related to update of VarConditionalReconfig during SCPAC execution is modified to support subsequent CPA.  The description for subsequentCondExecutionCond  updated to enable the above

RAN2 to discuss whether to allow network to use fields configured in SpCellConfig in case the TRS configuration is configured in the LTM-TCI-Info IE.

The remaining proposals are suggested to be discussed OFFLINE:

Discuss the remaining RRC proposals (including the ones suggested to not be pursued) offline, except for stage2 and capabilities.

# 3	Conclusion

Based on the discussion in the previous sections we propose the following:

The following proposals are suggested to be agreed:

Subsequent CPAC

Proposal 1	UE release/clear all current common radio configurations except for PCell upon subsequent CPAC execution.

Proposal 2	Confirm that For the SCPAC scenario where initial serving SN/PSCell is not part of SCPAC , the Network should ensure that the servingCellSetID is set to a unique value different than the security-cell-set-ID of the candidates.

Proposal 3	Instead of using “MN format” it is spelled out what this does mean in the RRC specification.

LTM

Proposal 4	Clarify that the field rrc-SSB-Subset is not used for LTM in case a CG is configured.

Proposal 5	Clarify in the Annex B2 that the RRCReconfiguration shall not be sent unprotected if it includes the LTM-Config.

The following proposals are suggested to be discussed ONLINE:

Proposal 6	RAN2 to discuss, inter-DU LTM, whether shared preamble resources are supported and whether preamble index is enough or whether this is done by a combination of SSB index, PRACH mask index, and preamble index.

Proposal 7	RAN2 to clarify whether the 'SSB Positions In Burst' parameter is essential for the source gNB-DU to calculate the RA-RNTI, which in turn allows for associating a TA value with a UE when RACH occasion differentiation is required.

Proposal 8	RAN2 to clarify whether the candidate gNB-DU needs provide SFN, where the preamble is transmitted, along with the RA-RNTI and the TA value for a specific UE.

Proposal 9	RAN2 to confirm that the intra-SN subsequent CPAC and CHO can be configured together.

Proposal 10	If CPAC and CHO can be configured together, to avoid the mismatch in CHO confguration between UE an MN and potential RLF, when intra-SN subsequent CPAC and CHO are configured together, consider the below options:

a.	UE keeps the CHO configuration after subsequent CPAC execution stored in SCG VarConditionalReconfig (preferred).

b.	Introduce new inter-node signalling from SN to MN to inform that subsequent CPAC execution is performed for intra-SN CPC.

Proposal 11	From RAN2 perspective, CG-Config and CG-CandidateList are related to different configurations of intra-SN subsequent CPAC in MN format, but they are not required to must be sent together in the same SN Modification Required XnAP message.

Proposal 12	RAN2 understands that two XnAP messages can be sent from the SN to the MN to transfer CG-Config message and CG-CandidateList messages separately. It requires some modification or clarification in the current stage-2 procedure. The detailed message and prcedure can be up to RAN3 discussion.

Proposal 13	The procedure text changes introduced in TS38.330 V 18.3.0 related to update of VarConditionalReconfig during SCPAC execution is modified to support subsequent CPA.  The description for subsequentCondExecutionCond  updated to enable the above

Proposal 14	RAN2 to discuss whether to allow network to use fields configured in SpCellConfig in case the TRS configuration is configured in the LTM-TCI-Info IE.

The following proposals are suggested to be discussed OFFLINE:

Proposal 15	Discuss the remaining RRC proposals (including the ones suggested to not be pursued) offline, except for stage2 and capabilities.