3GPP TSG-RAN WG2 Meeting #127bis					R2-2408835

Hefei, China, 14 - 18 October, 2024

Title: 	Discussion on MRO enhancements for Rel-18 mobility features

Source: 	Huawei, HiSilicon

Agenda item:	8.10.2

Document for:	Discussion/Decision

- Introduction

In this paper, we mainly analyze possible RAN2 impacts for LTM, CHO with candidate SCGs and subsequent CPAC.

- Discussion

Discussion on LTM

Per the agreement from RAN2#125bis, RAN2 has agreed on the definition of too early LTM, too late LTM and LTM to wrong cell, and to focus on the MCG LTM first. RAN2#126 meeting has achieved the following agreements:

We correct the definition of scenario 3a as follows: Case 3a: the UE detects HOF/RLF in the LTM target cell and performs reestablishment procedure in a cell other than the source cell and the target cell.

If available, log the L1 measurements for serving cell, target cell and other LTM candidate cells in RLF report, upon RLF or mobility failure.

Reuse the existing approach of using timeConnFailure and the reconnectCellId in RLF-report also for LTM failures, details TBD.

Log the LTM cell ID upon performing recovery an LTM candidate cell, details TBD e.g. which field.

Extend lastHO-Type in RLF-Report to indicate the LTM cell switch as last executed mobility procedure.

RAN2#127 made the following agreements:

Only the field description associated to the timeConnFailure IE needs to be updated accordingly. Otherwise, we don’t expect any further specification impact for timeConnFailure and reconnectCellId is foreseen, TBC.

introduce a new field in RLF report to indicate the LTM recovery cell id.

RAN2 include the specific access type in the RLF report, i.e. whether it is RA-based or RA-less cell switch. FFS details, e.g. if explicit or implicitly signalled.

We do not include in the RLF report, the time since the last reception of PDCCH order towards the target cell till reception of cell switch command.

RA-based access will not be a new triggering condition for the SHR report.

HO interruption time will not be a new triggering condition for the SHR report

Reuse the existing approach of using timeUntilReconnection in RLF-report also for LTM failure scenarios.

We will not define and log timeSinceLTM-Reconfig (like timeSinceCHO-Reconfig) within RLF-report and SHR in LTM failure and near failure cases.

We will not log interruption time in SON reports (e.g. SHR) for LTM.

We aim to log some info to deduce the ltmCandidate (similar like choCandidate) in SHR to indicate whether a neighbour cell is an LTM candidate cell or not, TBD if explicit/implicit.

Log L3 measurements for serving cell, target cell and other LTM candidate cells in RLF report, upon RLF or mobility failure. RAN2 assumes this is already possible with existing spec.

To facilitate the further discussion, we illustrate the steps during the LTM procedure to have a retro on the design of Rel-18 LTM feature, compared to those during legacy L3 handover.

Figure 1: Handover procedures for Legacy L3 HO and LTM

The division of failure cases for LTM is similar as those for handover, since the LTM is built and augmented on top of L3 handover. For handovers, the SON features including the radio link failure (RLF) report and successful handover report (SHR) provides the UE feedback information. The RLF and SHR could be augmented to also provide useful feedback information to address the LTM mobility issues. We will discuss the possible MRO enhancement for those SON reports.

RLF report for LTM

- A FFS from last RAN2 meeeting is whether the specific access type, i.e. RACH-based or RACH-less, is explicitly or implicitly indicated in RLF report. The specific access type is determined by whether the UE has available TA after reception of LTM cell switch command, which could be gained either from network triggered PDCCH ordered TA acquisition procedure or UE-based TA measurement. Based on the agreed logging information from previous RAN2 meeting, we could not derive whether it is RA-based or RA-less LTM cell switch execution implicitly. Therefore, it is suggested to indicate the specific access type explicitly.

Proposal 1: RAN2 include the specific access type in the RLF report explicitly.

Per current specification, the UE could perform the UE-based TA measurement and perform PDCCH ordered TA acquisition simultaneously. If there are two valid TA, i.e. one from cell switch command and the other measured by UE itself, the UE applies the former one. If there is only either one TA of those two, the UE simply applies it. In addition to the access type, the UE also indicates that whether the network provides the TA in the cell switch command. So that, the network could distinguish between the TA from the network and the TA from UE-based measurement to make proper adjustment of the LTM related parameters, e.g. whether to enable the UE-based TA measurement.

Proposal 2: RAN2 include in the RLF report whether the network provides the TA in the cell switch command.

Furthermore, we discuss the beam info indicated in the LTM cell switch command. In case of RACH-less cell switch, the UL transmission may fail due to the bad choice of the target beam. It is suggested that UE logs the beam info in the cell switch command. Combining this beam info with the L1 measurement result at the failure, the network could perform analysis regarding the choice of target beam.

Proposal 3: RAN2 agree to enhance the RLF report with the beam information contained in the LTM cell switch command.

SHR report for LTM

Another distinct nature of LTM is that the UE could perform a RACH-less cell switch towards the target cell with valid TA acquired from the PDCCH order’s TA acquisition or UE-based TA measurement. It is beneficial for the UE to recognize the RA-based cell switch as a near failure, since the UE is supposed to benefit from the LTM feature by performing a RACH-less access. Therefore, a RA-based access could be viewed as a sub-optimal LTM mobility incident. We recommend, the RA-based access is a SHR trigger for LTM feature.

If the UE is configured with LTM config, the UE has no available valid TA upon reception of cell switch command and has to perform RA-based access towards target cell, which causes long UP interruption time. It is beneficial to recognize the RA-based cell switch as a near failure, since the UE is supposed to benefit from the LTM feature by performing a RACH-less access and thus avoiding long UP interruption time according to Rel-18 WID of LTM.

One thing we would like to clarify, the UE attempt the RA-based access until T304 expiry, which is equivalent to declaration of HOF. Before T304 expiry, there is no such thing as fall back from RA-less to RA-based. Only HOF occurs, the UE may perform LTM based recovery, which is based on RA using LTM cell configuration. However, we call it an LTM recovery procedure instead of fallback procedure.

Observation 1: Before T304 expiry, there is no such thing as fall back from RACH-less to RACH-based. Only HOF occurs, the UE may perform LTM based recovery, which is based on RA using LTM cell configuration. However, we call it an LTM recovery procedure instead of fallback procedure.

Proposal 4: Fall back from RACH-less to RACH-based would not be a new trigger condition for SHR report.

In last meeting, there is a discussion on the number of RLC re-transmission as a new trigger for SHR report. However, there is no RLC re-transmission for the first UL data transmission on the target cell when the UE is attempting to access the target cell, since RLC re-transmission at the UE side is triggered by the feedback from the network, i.e. RLC status report from the target cell. It is noteworthy that the RLC re-transmission only applies to the connected UE. Therefore, it is not technically feasible to introduce the threshold for RLC re-transmission to evaluate the quality of LTM cell switch execution.

Proposal 5: Do not introduce trigger related to RLC re-transmission for SHR since there is no RLC re-transmission during LTM cell switch execution.

RAN2 has excluded the RACH-based access and HO interruption time as new trigger conditions for SHR. There is no doubt to reuse the timer related trigger, i.e. T310/T312/T304, similar to legacy L3 handover. Once the timer related trigger is satisfied, the network would better know whether the handover is a LTM cell switch or L3 handover, in order to adjust the parameter precisely and efficiently, e.g. whether it is the CU or DU to optimize accordingly. Therefore, we propose, an indication of  LTM cell switch is introduced to SHR report.

Proposal 6: Introduce an indication of LTM cell switch to SHR report.

- Furthermore, RAN2 has agreed to indicate the RACH-based or RACH-less access in the RLF report. Similarly, the logging of access type is needed for SHR report and RA report.

Proposal 7: Introduce an indication of specific access type, i.e. RACH-based of RACH-less, to SHR report and RA report.

A another FFS is whether to indicate the LTM candidate cells in SHR explicitly or implicitly. There are two alternatives as follows:

Alt.1: indicate a LTM candidate in the neighbour cell measurement results

Alt.2: introduce a LTM candidate cell list

For alt.1, it support to indicate the LTM candidate for both RLF report and SHR report, by a flag of the LTM candidate within the MeasResultNR explicitly for each neighbour cell, since the two reports contain the neighbour cell measurement results.  However, for alt.2, a separate LTM candidate list needs to be introduced to RLF and SHR report individually. Therefore, we propose to adopt alt.1.

Proposal 8: Introduce a flag of the LTM candidate within the MeasResultNR explicitly for each neighbour cell.

Discussion on CHO with candidate SCGs

About assumptions on configurations

According to RAN2#126 Chair Notes [1], for the assumptions on configurations, the following options were proposed for discussions:

Option 1: R18 configurations of CHO with candidate SCGs

Option 2: R17 configurations of CHO with target SCG

Option 3: R16 configurations of CHO

Some companies proposed to discuss Option 1 only, while some companies proposed to discuss mixed configurations, e.g. Option 1 + Option 2, or Option 1 + Option 3. Since the Option 1 is the target case for this Rel-18 feature objective, the usage of mixed configurations are just a supplement to Option 1 in case that these parameters of Option 1 could not be configured properly and thus raise the risk of mobility failure issues.

We think RAN2 can focus on Option 1 first, and other options (including mixed configuration) can be de-prioritized.

Proposal 9: RAN2 focus on only R18 configurations of CHO with candidate SCGs first.

Discussions on scenarios and issues

At RAN2#126, we submitted [2] and we showed 9 failure cases for CHO with candidate SCGs. As below, we provide more technical analysis  on these scenarios and possible issues.

Discussions on Case 2)

For Case 2), it is about too late handover, and then UE will suffer RLF on PCell.

Figure 4: Handover procedure for Case 2)

At RAN2#126 meeting, RAN2 made the following agreements, which can be used for Case 2).

RAN2 to enhance the RLF report with additional information regarding the state of the two execution conditions. We see later if we also can enhance the SCGFailureInformation report.

For RLF report, currently the IE lastHO-Type is used to indicate the type of the last executed handover before the last detected connection failure, and it has defined CHO and DAPS. For Case 2), if a new HO type CHO with candidate SCGs can be added, the network can do optimizations based on this kind of handover. For now, we see no strong need to enhance SCGFailureInformation for Case 2).

Proposal 10: For Case 2) where execution conditions for PCell/PScell are met but handover to PCell/PScell fails, it is proposed RAN2 to discuss the need of introducing a new HO type CHO with candidate SCGs in the IE lastHO-Type in RLF report.

Discussions on Case 7)

For Case 7), both PCell mobility and PScell mobility are triggered. And then, either PCell or PScell or both fail.

Figure 5: Handover procedure for Case 7)

For Case 7), if PCell fails and PScell succeeds, the UE will do RRC re-establishment and also generate RLF report. In this case, the UE can indicate the cause (e.g. PCell failure+PScell success) to the network, so that the network can optimize configuration parameters correspondingly.

If PCell succeeds and PScell fails, the UE will change its PCell successfully, but PScell change fails. Later, The network can do something to handle it. So SCGFailureInformation enhancement may be needed. For example, MN associated with failed SCG can be considered in order to reflect CHO with candidate SCGs.

If both PCell and PScell fail, similar to the first case for Case 7), the cause can be indicated to the network for optimizations.

At RAN2#127 meeting, the following agreements were made. In Case 7), the failure may be caused by too long time between CHO execution and CPC execution, so it is helpful for the UE to record the time between two execution conditions in RLF report.

UE includes following information in RLF report:

- b.	Time information regarding condition fulfilment for CHO with candidate SCGs. Details are FFS. We consider both the case when both CHO condition and associated CPC condition are fulfilled, and the case when CHO (or CPC) is fulfilled but CPC (or CHO) conditions are not fulfilled.

- c.	Measurement results of PCells and PSCells.

Proposal 11: For the case of PCell failure and PScell success, and the case of PCell failure and PScell failure, it is proposed RAN2 to discuss the need of introducing the causes in the RLF report.

Proposal 12: For the case of PCell success and PScell failure, it is proposed RAN2 to discuss the need of introducing MN associated with failed SCG in SCGFailureInformation message.

Proposal 13: For the case that both CHO and CPC are met, and then either PCell or PSCell or both fail,  it is proposed RAN2 to agree on including the time between two execution conditions in RLF report.

Discussions on Case 8)

For Case 8), both PCell and PScell change successfully, and then either PCell or PScell or both fail in a short time. This belongs to "failure shortly after a successful handover".

Figure 6: Handover procedure for Case 8)

We think Case 8) can be categorized into near failure case, and then successful handover report can be enhanced for PCell/PScell mobility.

Firstly, at RAN2#127 meeting, the following agreements were made, and they are FFS for reports other than RLF report. For the time period between CHO execution time and CPC execution time, it can be included in either SHR or SPR. In addition, the trigger based on the time period can be considered, e.g. if the time period is above a threshold, it may be near failure, and such successful handover reports should be triggered.

UE includes following information in RLF report:

- b.	Time information regarding condition fulfilment for CHO with candidate SCGs. Details are FFS. We consider both the case when both CHO condition and associated CPC condition are fulfilled, and the case when CHO (or CPC) is fulfilled but CPC (or CHO) conditions are not fulfilled.

- c.	Measurement results of PCells and PSCells.

Secondly, for Case 8), the UE may generate SHR and SPR. Then whether/how to co-ordinate both reports may be discussed. For example, the UE may generate SHR for PCell mobility and generate SPR for PScell mobility in Case 8), and two reports are actually related to one single R18 mobility procedure (i.e. CHO with candidate SCGs).

Proposal 14: Time period between CHO execution time and CPC execution time can be added in SHR or SPR.

Proposal 15: For a successful CHO with candidate SCGs, time period between CHO execution time and CPC execution time could be one triggering condition for SHR or SPR.

Proposal 16: For a successful mobility procedure of CHO with candidates SCGs, RAN2 to discuss whether SHR and SPR need to be correlated or not.

Other scenarios (see no issues and no enhancements needed)

Apart from the above scenarios, there are more scenarios but we observe no issues and thus no enhancements are needed.

For Case 1), it is about too late handover, and PCell/PScell mobility is not triggered. In this case, we think legacy SON reporting can work.

Figure 7: Handover procedure for Case 1)

For Case 4), we think CHO failure reporting can be followed. In this case, no candidate SCG mobility is triggered, so no extra information needs to be collected.

If R16 CHO fails, legacy SON reporting is followed.

Figure 8: Handover procedure for Case 4)

For Case 5), it is CHO only and SCG is not changed. This assumes that UE has both R16 CHO and R18 CHO with candidate SCGs. If R16 CHO shortly RLF, legacy SON reporting is followed.

Figure 9: Handover procedure for Case 5)

For Case 6), it is CHO only and SCG is not changed. This assumes that UE has both R17 (CHO+SCG)+R18 CHO with candidate SCGs. If R17 CHO+SCG succeeds, and SCG failure happens quickly, legacy SCGFailureInformation is followed.

Figure 10: Handover procedure for Case 6)

For 9d, there are two failures during the access procedure, priority of this case? According to internal discussion, when UE is in DC and stable state, Fast MCG recovery can happen. So 9b does not exist.

Figure 11: Handover procedure for Case 9)

Discussion on subsequent CPAC

At RAN2#127, there were some discussions, and P12 was postponed.

R2-2406959	Discussion on MRO enhancements for R18 mobility features	CMCC	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

Focus on P12

Proposal 12: Include the subsequent CPAC related information in MHI, e.g. the visited SCPAC candidate PSCell(s), the time stays in the candidate PSCell(s), the PSCell(s) measurement result with the indication about the PSCell(s) is for SCPAC, etc.

-	LG is OK with the proposal, but this seems to be existing already. Qualcomm agrees. Lenovo wants to check the spec in more detail. Session chair thinks that companies seem to need more time to check the spec and we can come back to this next meeting.

Here we would like to provide our views on P12.

Intuitively, it might be helpful for the network to know some history information during the following CPAC, with which the S-CPAC configuration can be optimized. However, it seems that the useful information has already been included in legacy procedures.

VisitedCellInfoList field descriptions

timeSpent This field indicates the duration of stay in the cell or in any cell selection state and/or camped on any cell state in NR or E-UTRA approximated to the closest second. If included in VisitedPSCellInfo, it indicates the duration of stay in the PSCell or without any PSCell. If the duration of stay exceeds 4095s, the UE shall set it to 4095s.

visitedCellId This field indicates the visited cell id including NR and E-UTRA cells.

According to TS 38.331, VisitedPSCellInfoList is included in MobilityHistoryReport as listed above. The visited cell and the staying time are contained in this message. There also exists several mechanisms that support recording and reporting the measurement results, e.g. MDT. Besides, the network naturally knows whether a PSCell is in the S-CPAC candidate cells. As discussed above, we think the network can evaluate the S-CPAC with the information included in the legacy messages. Therefore, we see no strong motivation to introduce the proposed information or the S-CPAC indication into MHI.

Proposal 17: RAN2 to agree that the MHI related information may enhance the S-CPAC optimization, but it is not necessary to enhance MHI in RAN2 scope.

- Conclusion

Based on the discussions above, we have the following observations and proposals:

LTM

Observation 1: Before T304 expiry, there is no such thing as fall back from RACH-less to RACH-based. Only HOF occurs, the UE may perform LTM based recovery, which is based on RA using LTM cell configuration. However, we call it an LTM recovery procedure instead of fallback procedure.

Proposal 1: RAN2 include the specific access type in the RLF report explicitly.

Proposal 2: RAN2 include in the RLF report whether the network provides the TA in the cell switch command.

Proposal 3: RAN2 agree to enhance the RLF report with the beam information contained in the LTM cell switch command.

Proposal 4: Fall back from RACH-less to RACH-based would not be a new trigger condition for SHR report.

Proposal 5: Do not introduce trigger related to RLC re-transmission for SHR since there is no RLC re-transmission during LTM cell switch execution.

Proposal 6: Introduce an indication of LTM cell switch to SHR report.

Proposal 7: Introduce an indication of specific access type, i.e. RACH-based of RACH-less, to SHR report and RA report.

Proposal 8: Introduce a flag of the LTM candidate within the MeasResultNR explicitly for each neighbour cell.

CHO with candidate SCGs

Proposal 9: RAN2 focus on only R18 configurations of CHO with candidate SCGs first.

Proposal 10: For Case 2) where execution conditions for PCell/PScell are met but handover to PCell/PScell fails, it is proposed RAN2 to discuss the need of introducing a new HO type CHO with candidate SCGs in the IE lastHO-Type in RLF report.

Proposal 11: For the case of PCell failure and PScell success, and the case of PCell failure and PScell failure, it is proposed RAN2 to discuss the need of introducing the causes in the RLF report.

Proposal 12: For the case of PCell success and PScell failure, it is proposed RAN2 to discuss the need of introducing MN associated with failed SCG in SCGFailureInformation message.

Proposal 13: For the case that both CHO and CPC are met, and then either PCell or PSCell or both fail,  it is proposed RAN2 to agree on including the time between two execution conditions in RLF report.

Proposal 14: Time period between CHO execution time and CPC execution time can be added in SHR or SPR.

Proposal 15: For a successful CHO with candidate SCGs, time period between CHO execution time and CPC execution time could be one triggering condition for SHR or SPR.

Proposal 16: For a successful mobility procedure of CHO with candidates SCGs, RAN2 to discuss whether SHR and SPR need to be correlated or not.

Subsequent CPAC

Proposal 17: RAN2 to agree that the MHI related information may enhance the S-CPAC optimization, but it is not necessary to enhance MHI in RAN2 scope.

References

Draft_R2_126_Meeting_Report_v2

R2-2405334, MRO enhancements for Rel-18 mobility features, Huawei, HiSilicon

Draft_RAN2_127_Meeting_Report_v1