3GPP TSG-RAN WG2 Meeting #127bis	R2-2408849

Hefei, China, Oct 14th – 18th, 2024

Agenda item:	8.4.3

Source:	China Telecom

Title:	RRM Measurement Relaxation and Offloading in RRC_IDLE /INACTIVE Mode

Document for:	Discussion

- Introduction

In RAN #102 meeting, a new work item on Low-power wake-up signal and receiver for NR (LP-WUS/WUR) was approved. In RAN2#127 meeting, the following agreements and open issues on authorization were made:

RAN2 only discuss RRM measurement offloading/relaxation for LP-WUS UEs.

For serving cell measurement offloading (i.e., there is no serving cell measurement by MR):

    - The entry conditions for serving cell measurement offloading can be defined as at least MR greater than a certain RSRP threshold, and LR could also be considered.

    - The exit condition is based on the LR measurement results.

In view of the fact that, this paper mainly discusses the RRM measurement relaxation and offloading of RRC_IDLE/INACTIVE UE MR for serving and neighbour cells.

- Discussion

2.1 The offloading criterion for serving cell

As one of the objectives of WID, the trigger conditions for serving cell measurement offloading from MR to LR need to be studied.  From the perspective of RAN2, the thresholds for RRM measurement offloading should satisfy similar criteria (e.g. Rel-16 PowSav or Rel-17 RedCap) for relaxing neighbour cell RRM measurement or even with more stringent requirements or thresholds. For instance, when the current criteria for relaxing neighbour cell RRM measurements, such as not being at the cell edge and/or exhibiting low mobility, must be fulfilled, along with the requirement of a strong enough received LP-WUS signal. In such cases, the UE could use the LP-WUS and offload RRM serving cell measurement from MR to LP-WUR.

Proposal 1: The current triggering criteria for RRM measurement relaxation in Rel-16 PowSav and Rel-17 RedCap can be used as a baseline for RRM measurement offloading.

If partial UE serving cell RRM measurements are offloaded from MR to LP-WUR, the evaluation of offload exiting could be done based on measurements from both LP-WUR and MR. Thus, some joint evaluation of threshold based on measurements from both LP-WUR and MR could be considered.

Proposal 2: It is proposed to consider joint evaluation of threshold based on measurements from both LP-WUR and MR.

2.2 RRM Measurement Relaxation for Neighbour Cells

In RAN2#126 meeting, it was agreed that the RRM measurement of the neighboring cell can only be performed by MR.

If the entry conditions for LP-WUS monitoring are fulfilled, before performing the serving cell measurement of MR, it should be determined whether the criteria for performing neighboring cell measurement are met:

- If the signal quality of the serving cell is good enough and neighboring cell measurement does not need to be performed, there is no need to wake up MR. For example,when UE is near the centre of the cell, the UE is not likely to perform cell re-selection to a neighbor cell of equal or lower priority.

- If the neighboring cell measurement is met, then it should be determined whether neighboring cell measurement relaxation can be performed.

Proposal 3: Before waking MR to perform the neighboring cell measurement relaxation, it should be determined whether the criteria for performing neighboring cell measurement are met.

If UE does not perform measurement relaxation by LP-WUR, MR can apply NR legacy behaviors for RRM measurement. Neighbour cell measurement relaxation can reuse the legacy Rel-16 criteria for not at cell edge and low mobility. However, if a UE is operating with LP-WUR, the number of transitions from LP-WUR to MR should be minimized to obtain the power saving gain. As stated in TR 38.869 [2], using the MR for RRM measurements has a significant impact on UE energy consumption and reduces the power saving gain. Thus, the neighbor cell RRM measurements by MR should be relaxed once the entry condition of LP-WUS monitoring is fulfilled. The entry condition of LP-WUS monitoring can be regarded as the criteria for neighbor cell RRM measurement relaxation by MR.

Proposal 4: The entry condition of LP-WUS monitoring can be used as the criteria for MR to relax RRM measurement for neighbor cells.

- Conclusions

Based on the discussion in the previous sections we propose the following:

Proposal 1: The current triggering criteria for RRM measurement relaxation in Rel-16 PowSav and Rel-17 RedCap can be used as a baseline for RRM measurement offloading.

Proposal 2: It is proposed to consider joint evaluation of threshold based on measurements from both LP-WUR and MR.

Proposal 3: Before waking MR to perform the neighboring cell measurement relaxation, it should be determined whether the criteria for performing neighboring cell measurement are met.

Proposal 4: The entry condition of LP-WUS monitoring can be used as the criteria for MR to relax RRM measurement for neighbor cells.

- References

- RP-240801 Revised WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR)

- TR 38.869, Study on low-power wake up signal and receiver for NR.