3GPP TSG-RAN WG2 Meeting #127bis                                    	R2-2408905

Hefei, China, Oct. 14th – 18th, 2024

Agenda item:	8.9.2

Source:	Novamint, Sateliot

Title:	Considerations on multi-satellite for S&F Satellite operation

Document for:	Discussion and Decision

- Introduction

In the last RAN2 meeting the following agreements were made related to S&F Satellite operation.

RAN2#127 Agreement

Agreements:

1.	RAN2 adopts the SA2 study conclusions on the possible S&F architectures as the baseline for further discussion (RAN2 will only consider the full CN and spit-MME payload options)

2.	RAN2 will consider both single satellite pass and multiple satellite pass scenarios

3.	RAN2 will consider both MO and MT data within scope

4.	UE is informed whether its serving satellite is currently operating in S&F via System Information broadcast (FFS if we also need a static indication that in general the NW supports the feature)

5.	RAN2 does not further discuss whether legacy UEs will always need to be barred in a S&F network, at least not before further progress in SA2 discussion. If there will be a need for this, mechanisms to bar legacy UEs are already in place and no further impact on RAN2 specs in expected.

7.	RAN2 agrees there will be a way to bar legacy UEs (using legacy cellBarred and/or cellBarred-NTN bit) and still allow R19 S&F capable UEs. FFS on the exact solution (e.g. new barring bits or whether this is linked to some other indication)

In this contribution, we discuss and focus on the “multiple satellite” aspects of the Store and Forward Satellite operation.

- Discussion

A. Multiple satellite pass scenarios versus single satellite pass scenarios

It is important to note that due to the nature of a NGSO constellation, the serving satellite of a UE is always changing as allowing a UE to be served by different satellites within the constellation is key to reducing the revisit time for the UE.

Note that the revisit time with a single satellite in typical LEO orbits may be around 12h or longer [2]. Therefore, defining a S&F solution in which a UE is only able to be served by a single satellite of a NGSO constellation, referred to as single-satellite S&F solution, would mean that a UE will only be able to send/receive data e.g. twice a day on average.

This case is even reinforced for a moving UE.

Therefore, the support of a Store and Forward Satellite operation solution able to serve a UE via multiple satellites (in other words, not restricting the interaction of a UE and the network to a single satellite of the constellation), referred to as multi-satellite S&F solution, should be considered in the architecture from the beginning.

Observation 1: The support of multi-satellite is a fundamental principle of NGSO constellation - single satellite is a substract of multi satellite i.e. any solution working with multiple satellite will work with only one satellite while a solution made only for single satellite may not work with multiple satellites.

The support of multi-satellite in Store and Forward Satellite operation (multi-satellite S&F) should be considered as the baseline and general case, while restricting the number of satellites that can provide service to a given UE to one (single-satellite S&F) is a particular deployment case.

B. Satellite ID

In SA2 [2], a "S&F Monitored List" is being considered as the way to inform the UE about the identifiers of the potential serving satellites within a constellation. The S&F Monitoring List is to be sent to UE during a NAS procedure (e.g. Attach, TAU) and includes the list of satellites for the same PLMN with which the UE can attempt to retry a NAS procedure or perform any subsequent NAS procedure, including MO/MT data transfer. The S&F Monitoring List can be updated by an MME every time the UE performs a NAS procedure.

The use of the “S&F Monitored List” relies on the assumption that the UE should be able to determine which is the “SatelliteId” of the satellites in the constellation.

In this respect, a “satelliteId-r17” identifier was introduced in R17 as part of SIB32 to provide satellite assistance information related to discontinuous coverage of the multiple satellites, but there is not explicit indication of which of those satellite identifiers corresponds to the serving one. On the other hand, in R18, a new identifier “satelliteId-r18” has been introduced into SIB31 as part of the serving satellite reference location, but apparently there is no connection/relation between the values that could be used in “satelliteId-r17” and in “satelliteId-r18” fields for different purposes. SatelliteId-r18 is also used in other SIBs.

Observation 2: Current specs include different definitions and uses of satellite identifiers (“satelliteId-r17”, “satelliteId-r18”) in different SIBs (SIB31/32/33/3/5). Relation, if any, between the use of these identifiers is not explicitly stated. How the UE is expected to determine a single/unique satellite identifier for the serving satellite that can be used in the “S&F Monitored List” could be open to different interpretations.

RAN2 to define the way that the UE is expected to determine the satelliteId of a serving satellite in a multi-satellite S&F scenario.

- Conclusion

Following observations and proposals are made:

Observation 1: The support of multi satellite is a fundamental principle of NGSO constellation - single satellite is a substract of multi satellite i.e. any solution working with multiple satellite will work with only one satellite while a solution made only for single satellite may not work with multiple satellites.

Observation 2: Current specs include different definitions and uses of satellite identifiers (“satelliteId-r17”, “satelliteId-r18”) in different SIBs (SIB31/32/33/3/5). Relation, if any, between the use of these identifiers is not explicitly stated. How the UE is expected to determine a single/unique satellite identifier for the serving satellite that can be used in the “S&F Monitored List” could be open to different interpretations.

- The multi-satellite support for Store and Forward Satellite operation (multi-satellite S&F) should be considered as the baseline and general case, while restricting the number of satellites that can serve to a given UE to one (single-satellite S&F) is a particular deployment case.

RAN2 to define the way that the UE is expected to determine the satelliteId of a serving satellite in a multi-satellite S&F scenario.

- References

RP-242397, Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3, RAN#105, 2024-09, Melbourne, AU.

R2-2107453, “On LEO satellite flyover timing and discontinuous coverage”; Eutelsat, August 9th - 27th, 2021, RAN2#115-e

TR 23.700-29, “Study on integration of satellite components in the 5G architecture; Phase 3”, SA#104, 2024-06, Shanghai, CN.