3GPP TSG-RAN WG2 Meeting #127bis	R2-2408922

Hefei, China, Oct 14th – 18th, 2024

Agenda Item:	8.1.2.3

Source:	InterDigital Inc.

Title:	LCM for positioning

Document for:	Discussion

# 1. Introduction

In RAN2-127, LCM for UE-sided model for the beam management use case was discussed and the following agreements were made [1]:

Agreements on procedures - Step 1: Network sends UECapabilityEnqiry message to initiate the procedure to a UE reporting its AI/ML supported functionalities. - Step 2: UE sends UECapablityInformation message to network, containing supported functionalities at the UE side. - “Step 3”: Following configurations are provided from NW to UE: 1) UE is allowed to do UAI reporting via OtherConfig. 2) Network may provide NW-side additional condition. FFS on the RRC signalling and whether it is mandatory or optional. 3) FFS on configuration (e.g. inference configuration) of supported functionalities. FFS on the content of configuration. - UE decides the applicable functionalities based on NW-side additional conditions (if provided), UE-side additional conditions (internally known by UE) and model availability in device. FFS whether other configuration can be considered by UE (e.g. inference configuration). FFS how the applicable functionality is decided if NW-side additional condition is not provided in step 3. - “Step 4”: UE reports applicable functionality in the following scenarios: 1) Upon being configured to provide applicable functionality and upon change of applicable functionality via UAI 2) As response to NW-side additional condition requesting applicable functionality reporting in step 3, FFS other network configuration (e.g. inference configuration), FFS via UAI or RRCReconfigurationComplete, etc - Step 5: 1) Network configures inference configuration to UE after applicable functionality reporting, if inference configuration based on supported functionality is not provided in Step 3 (i.e. inference configuration is provided in Step 5). 2) If inference configuration based on supported functionality is provided in Step 3, it is up to network implementation whether to provide an updated configuration or not. - The applicable functionality may be activated by receiving its inference configuration when it is provided in Step 5. FFS the initial activation state. FFS on initial state of applicable functionality if inference configuration of supported functionality is provided in Step 3. FFS on additional L1/L2 signalling for activation/deactivation. FFS if multiple applicable functionalities can be activated at the same time. FFS what is the granularity of functionality

In this contribution, we will discuss which of these agreements could also be applicable for the positioning use case, specifically positioning use case 1.

# 2. Discussion

Regarding the capability exchange for positioning, it has already been agreed in RAN2-126 to use the LPP capability transfer procedures [2], which is aligned with what is agreed for the beam management case. Thus, we propose the following for the initial steps of the LCM procedure:

Proposal 1:

Step 1: Network (LMF) sends RequestCapabilities LPP message to initiate the procedure to a UE reporting its AI/ML positioning functionalities.

Step 2: UE sends ProvideCapabilities LPP message to network (LMF), containing supported AI/ML positioning functionalities at the UE side.

In RAN2-125bis, both reactive and proactive reporting of applicable functionalities were agreed to be supported for both the beam management and positioning use cases [3]:

Agreements for positioning and beam management

- Support proactive reporting of UE-sided applicable functionality, e.g., the UE reports its applicable AI/ML functionalities via UAI message/LPP message.

- Support reactive reporting of UE-sided applicable functionality.  The NW configures AI/ML functionalities via RRC/LPP message.  FFS what the configuration contains. FFS how to report applicable functionality and what is applicable functionality

3	FFS how the two approaches will be specified and whether we can combine them into one procedure.    FFS how to report applicable functionality, what is applicable functionality, how the UE determines which function is applicable or not (if it is needed)

Our understanding is that for reactive reporting, the network can request the UE for immediate reporting of applicable positioning functionalities and the UE responds immediately. On the other hand, for the proactive reporting, the network can request the UE to report when (later) a positioning functionality (or functionalities) become(s) applicable.

Thus, we propose the following to be agreed:

Proposal 2 (for reactive reporting):

Step 3a: Network (LMF) sends an LPP message to the UE, requesting it about the applicable AIML positioning functionalities. The LPP message and its details are FFS.

Step 4a: In response, the UE sends an LPP message indicating the applicable functionalities. The LPP message and its details are FFS.

Proposal 3 (for proactive reporting):

Step 3b: Network (LMF) sends an LPP message to the UE, configuring it to provide a report when AIML positioning functionality (or functionalities) become(s) applicable. The LPP message and its details are FFS. FFS if the same message is used for step 3a and step 3b.

Step 4b: The UE sends an LPP response message when the functionality(ies) indicated in step 3b become applicable. The LPP message and its details are FFS. FFS if the same message is used for step 4a and step 4b.

As in the beam management use case, input from RAN1 is required regarding whether the UE needs to be configured with inference configuration for performing applicability determination. In addition, unlike the beam management use case where RAN1 has made a working assumption that network side additional conditions may be signalled via associated ID(s) and this can be used for determining functionality applicability determination, there is no consensus in RAN1 regarding the need/usage of network side additional conditions. Thus, these aspects can be reconsidered based on RAN1 progress/input.

Proposal 4:  Wait for RAN1 progress regarding whether UE needs to be provided with inference configuration and/or network side additional conditions for performing functionality applicability determination.

One aspect that was discussed for the beam management use case was whether a L1/L2 signalling is needed for activation/deactivation of a given functionality (after the applicability reporting). For the positioning use case, the LMF, and not the gNB, is the controlling entity for the LCM. Also, there is no strict latency requirement regarding the activation/deactivation of the AIML positioning functionality. Considering these, activation/deactivation via LPP signalling is the preferred approach.

Proposal 5:  LPP signalling is used for the activation/deactivation of AI/ML positioning functionalities. The LPP message is FFS.

# 3. Conclusions

In this contribution, the LCM procedure for the positioning use case (specifically use case 1) were discussed and the following proposals are made:

Proposal 1:

Step 1: Network (LMF) sends RequestCapabilities LPP message to initiate the procedure to a UE reporting its AI/ML positioning functionalities.

Step 2: UE sends ProvideCapabilities LPP message to network (LMF), containing supported AI/ML positioning functionalities at the UE side.

Proposal 2 (for reactive reporting):

Step 3a: Network (LMF) sends an LPP message to the UE, requesting it about the applicable AIML positioning functionalities. The LPP message and its details are FFS.

Step 4a: In response, the UE sends an LPP message indicating the applicable functionalities. The LPP message and its details are FFS.

Proposal 3 (for proactive reporting):

Step 3b: Network (LMF) sends an LPP message to the UE, configuring it to provide a report when AIML positioning functionality (or functionalities) become(s) applicable. The LPP message and its details are FFS. FFS if the same message is used for step 3a and step 3b.

Step 4b: The UE sends an LPP response message when the functionality(ies) indicated in step 3b become applicable. The LPP message and its details are FFS. FFS if the same message is used for step 4a and step 4b.

Proposal 4:  Wait for RAN1 progress regarding whether UE needs to be provided with inference configuration and/or network side additional conditions for performing AIML positioning functionality applicability determination.

Proposal 5:  LPP signalling is used for the activation/deactivation of AI/ML positioning functionalities. The LPP message is FFS.

# 4. References

[1]	RAN2-127 Chairman Notes, Maastricht, The Netherlands, August 2024

[2]	RAN2-126 Chairman Notes, Fukuoka, Japan, May 2024

[3]	RAN2-125bis Chairman Notes, Changsha, China, April 2024