3GPP TSG-RAN WG2 #127-bis	R2-2408932

# Hefei, China, 14th – 18th October 2024

Agenda Item:	 8.1.2.3

Source:	Ericsson

Title:	LCM For Positioning

Document for:	Discussion

# 1	Introduction

In this paper we discuss the below aspects for the 1st priority Case 1 (UE-based positioning with UE-side model, direct AI/ML positioning):

- Proposal Related to Positioning LCM from Email Discussion [POST126][032] [AI/ML PHY] LCM [R2-2406381]

- Terminology: Dynamic capability or Capability associated with an applicability condition

- Proactive and Reactive AI/ML for positioning procedure difference and commonality

- New Method or update of existing method

- Stage 2 Draft CR

# 2	Discussion

## Email Discussion

As part of email discussion of [POST126][032][AI/ML PHY] LCM [R2-2406381], the below proposal was suggested for positioning.

Proposal 7: For positioning Case 1, following understanding of applicable functionality reporting (proactive/reactive) is considered:

1) UE can report a change in applicable functionality by sending applicable functionalities upon change via LPP signaling (unsolicited information transfer). UE can also provide applicable functionality reporting as a response to LMF request (solicited information transfer). FFS on the order of inference configuration and applicable functionality reporting.

2) As baseline, the applicable functionality is initially activated by receiving configuration for the corresponding applicable functionalities. FFS on additional L1/L2 signaling for activation/deactivation.

Related to P7 of email discussion [032], it is important to note that the RTT of LPP signaling can be up to 40ms and there is expectation that end-to-end latency is 100ms. In such case, we should avoid too much/excessive signaling exchange between UE and LMF. It is possible that LMF upon receiving the capability and applicability condition configures the fallback option in case if it is unable to derive the results using AI/ML positioning technique. This can thus reduce the need of signaling exchange and there would be predictable behaviour on what the UE should do.

Proactive approach implies that UE provides its capability and associated applicability condition without LMF requesting. This approach is already supported by LPP; the below 1st Figure is reactive approach whereas the 2nd figure is proactive approach.

Figure 5.1.1-1: LPP Capability Transfer procedure

Figure 5.1.2-1: LPP Capability Indication procedure

Proactive approach does not imply that UE autonomously decide how the positioning measurements or location estimates has to be performed, that decision should be performed by the LMF. For both, proactive or reactive approach, the LMF should configure the fallback options when AI/ML model cannot be used for any reason (e.g: low battery, no applicable condition).

In RAN2#127 below terminology was agreed

Agreements on definitions

- Supported functionalities refer to functionalities that UE can indicate by using UE capability information (via RRC/LPP signalling)

- Applicable functionalities refers to functionalities that the UE is ready to apply for inference

- Activated functionalities refers to functionalities already enabled for performing inference

UE capability for AI/ML functionality for positioning is static and is reported by UE to LMF. The LMF may query using new LPP messages on applicability condition and get response from UE whether the condition is met or not. The UE may also provide its applicability condition and should include the applicability condition/validity criteria as below:

- Area IDs for which the model(s) is valid

- PRS configurations for which the model(s) is valid

- Validity duration timer (considering battery/processing capacity)

The LMF may decide to change PRS configuration to fulfil the inference criteria.

The term dynamic capability should not be used, rather it is capability with additional conditions.

There would be excessive signaling If UE reports every time one of its additional conditions is not met. This may risk that QoS for positioning in terms of latency is not met.

LMF configures the fallback options (e.g.: whether to use legacy method or abort and use new method) upon receiving applicability conditions.

UE shall not autonomously decide what steps to take when applicability conditions (mismatch between training and inference) inconsistency is observed. It follows the fallback configuration that LMF configures. This is valid for both proactive and reactive approaches.

LMF forwards the AI/ML UE capability and applicability conditions to AMF for storage for subsequent usage.

## New Method or Update of Existing method

In order to minimize specification impact, we think update of existing method as shown below for a potential stage2 update as below. The changes needed as compared to existing DL-TDOA UE based positioning method is shown in yellow highlights.

#### 8.X.1.1	UE-based positioning with UE-side model

##### 8.X.1.1.1	UE-based positioning with UE-side model Positioning procedure

UE-based positioning with UE-side model is based upon DL-TDOA positioning method.

Figure 8.X.1.1-1: UE Based DL-TDOA positioning method with UE side AI/ML model.

0.	The LMF may use the procedure in Figure 8.X.1.1-1 to obtain the TRP information required for DL-TDOA positioning.

1.	The LMF may request the positioning capabilities of the target device using the LPP Capability Transfer procedure described in clause 7.1.2.1. The UE includes capability for UE-based UE side direct AI/ML.

2.	The LMF sends an LPP Provide Assistance Data message to the target device as described in clause 8.10.3.1.2.1. The message includes any required assistance data for the target device to perform the necessary DL-PRS measurements and enquires if applicability condition for the AI/ML model is met.

3. The UE responds and indicates its applicabilty conditions such as PRS Configuration, validity area, time.

4.	The LMF sends an LPP Request Location Information message to request Location information using AI/ML model inference and any fallback configurations.

5.	The target device performs the channel measurements based upon DL-PRS and generates the result using the AI/ML model inference.

6.	The target device reports the location information to the LMF in a LPP Provide Location Information message and providing information whether the result has been derived using AI/ML model.

Note: If the UE does not indicate that the location was derived using AI/ML, it would imply that UE failed to use the AI/ML model for inference (e.g. additional conditions for using the model inference were not met, battery or processing issue etc.).

7.	The LMF may provide model performance feedback to the target device.

Existing DL-TDOA positioning method is updated for AI/ML positioning Case 1.

RAN2 to agree to the Text Proposal for stage 2 as captured from Step 1 to Step 7 above as baseline.

## Training Data Collection for Case 1

### 2.3.1 Using Legacy Procedure for AD request for training

As per RAN1 agreements (Only keeping option for Case 1):

Agreement For training data collection of Case 1, in terms of DL PRS configuration for collecting training data, RAN1 study the following options on assistance data, using legacy mechanisms as a starting point: Option A. (UE initiated) UE makes a request to LMF on the preferred DL PRS configuration for training data collection, e.g., on-demand PRS. LMF makes the decision on determining the DL PRS configuration for training data collection and provides the assistance data to the UE. Note: the UE can be a PRU and/or a Non-PRU UE. Note: as in existing specification, the DL PRS configurations in the assistance data from LMF to UE are based on DL PRS configuration coordinated between LMF and gNB.

The training data collection for Case 1, which only requires PRS configuration, is similar to existing procedure where UE requests assistance data of PRS configuration for legacy positioning measurements. Thus legacy procedure/ways for UE to request PRS configuration including UE-initiated assistance data request and UE-initiated on-demand PRS procedure can be reused. The training data collection for Case 1 is part of procedure for AI/ML positioning, and the PRS configuration request for this should not be either prioritised or deprioritized compared with legacy positioning, thus UE does not include the purpose of request when sending request for PRS configuration.

For data collection for AI/ML models on the UE side, the UE may request assistance data to LMF using UE-initiated assistance data request procedure as specified in clause 8.12.3.1.2.2 of TS 38.305 or UE-initiated on-demand PRS procedure as specified in clause 7.6.2 of TS 38.305 without the need to include the purpose of request.

### 2.3.2 Generating Label for data collection and associated parameters

RAN1 Agreements Working Assumption For training data generation of AI/ML based positioning Case 1, the measurement and its related data (e.g., timestamp) are generated by PRU and/or Non-PRU UE. Agreement For training data collection of AI/ML based positioning, the collected data sample can include the following components: Part A: - channel measurement - quality indicator of channel measurement - time stamp of channel measurement Part B: - ground truth label (or its approximation) - quality indicator of label - time stamp of label Note: “Part A” and “Part B” terminologies are only for RAN1 discussion purpose, and may not be used in specification. Note: contents in Part A and Part B may or may not be generated by different entities. Note: Part A and/or Part B, and their contents may or may not apply for each case FFS: detailed definition of channel measurement Working Assumption For training data generation of AI/ML based positioning Case 1, the label and its related data (e.g., time stamp) can be generated by: - PRU - Non-PRU UE with estimated location - LMF Note: transfer of the label and its related data is out of RAN1 scope.

AI/ML is used in area where there can be NLOS. Further, the PRS symbols may not be transmitted with perfect sync and there may be Real Time Difference. The LMF should be able to guide what is the expected positioning error during training phase to the UE based upon related Integrity parameters (e.g., standard deviation of RTD) so that the UE can understand if the obtained positioning can be used as a label (ground truth) or should be discarded. Similar to legacy expectedRSTD where LMF provides the expected RSTD value so that the UE can filter any outliers, the LMF for AI/ML training may provide maximum uncertainty/error so that the UE can judge if the obtained positioning can be considered as ground truth or should be discarded.

Similar to legacy expectedRSTD where LMF provides the expected RSTD value so that the UE can filter any outliers, the LMF for the case of AI/ML training can provide maximum uncertainty/error so that the UE can judge if the obtained positioning can be considered as ground truth or should be discarded.

LMF provides the guidance for trustworthy label based upon Positioning Integrity (maximum allowed Positioning error or uncertainty for a trustworthy label)

# Conclusion

In the previous sections we made the following observations:

Observation 1	The term dynamic capability should not be used, rather it is capability with additional conditions.

Observation 2	There would be excessive signaling If UE reports every time one of its additional conditions is not met. This may risk that QoS for positioning in terms of latency is not met.

Based on the discussion in the previous sections we propose the following:

Proposal 1	LMF configures the fallback options (e.g.: whether to use legacy method or abort and use new method) upon receiving applicability conditions.

Proposal 2	UE shall not autonomously decide what steps to take when applicability conditions (mismatch between training and inference) inconsistency is observed. It follows the fallback configuration that LMF configures. This is valid for both proactive and reactive approaches.

Proposal 3	LMF forwards the AI/ML UE capability and applicability conditions to AMF for storage for subsequent usage.

Proposal 4	Existing DL-TDOA positioning method is updated for AI/ML positioning Case 1.

Proposal 5	RAN2 to agree to the Text Proposal for stage 2 as captured from Step 1 to Step 7 above as baseline.

Proposal 6	For data collection for AI/ML models on the UE side, the UE may request assistance data to LMF using UE-initiated assistance data request procedure as specified in clause 8.12.3.1.2.2 of TS 38.305 or UE-initiated on-demand PRS procedure as specified in clause 7.6.2 of TS 38.305 without the need to include the purpose of request.

Proposal 7	LMF provides the guidance for trustworthy label based upon Positioning Integrity (maximum allowed Positioning error for a trustworthy label)

# References

Tdoc Number, Title, Source, Meeting, Date

Spec number, Title, Source, Version, Date