3GPP TSG-RAN WG2 Meeting #127bis		                               R2-2409049

Hefei, China, 14-18 October, 2024

Agenda Item:		8.7.4.1

Source:			MediaTek Inc.

Title:				LCP enhancements for LCH with delay critical data

Document:			Discussion and decision.

### 1	Introduction

This document discusses the LCP enhancement according to the revised objective in R19 XR WID [1].

-	Specify Enhancements for support of UL scheduling to enable high XR capacity while meeting delay requirements/avoiding too late PDUs, as follows [RAN2]:

-	Specify additional Logical Channel priority handling using delay/deadline information of packets;

In SI phase, there are still some FFS points should be further discussed:

- For delay-aware LCP enhancement, RAN2 considers the following option to override/adjust the priority of LCH based on delay/deadline information as a baseline:

  - Use additional priority configured to LCHs in case of these LCHs with delay-critical data

- FFS whether the priority only applies to delay-critical data within the LCH or for the whole LCH

- FFS whether a separate remaining time threshold can be configured for delay aware LCP (i.e. different from the one used for DSR).

- FFS whether/how additional priority impacts intra-UE prioritization (can be discussed in stage-3)

We will discuss above FFS points one by one.

### 2	Discussion

In RAN2 #126 meeting [2], we reach following agreements

- For delay-aware LCP enhancement, RAN2 considers the following option to override/adjust the priority of LCH based on delay/deadline information as a baseline:

  - Use additional priority configured to LCHs in case of these LCHs with delay-critical data

Based on above agreements, it is straightforward to introduce a new priority RRC IE in LogicalChannelConfig, and this new priority is used when this logical channel has delay-critical data.

Proposal 1: Introduce a new priority RRC IE in NR RRC spec for logical channel with delay-critical data.

In the same meeting, we also reach another agreement:

- LCP prioritization within a logical channel will not be considered in RAN2 discussions => that means the priority of the logical channel should be the same.

Therefore, we propose that the priority should apply to the whole LCH.

Proposal 2: The priority of the logical channel is changed to a signalled ‘delay-critical priority’ value when it has delay-critical data and the priority apply to whole LCH.

A simple example TP is shown below to achieve UL scheduling enhancement for LCH wity delay critical data.

-- MAC spec --

##### 5.4.3.1.1	General

The Logical Channel Prioritization (LCP) procedure is applied whenever a new transmission is performed.

RRC controls the scheduling of uplink data by signalling for each logical channel per MAC entity:

-	priority where an increasing priority value indicates a lower priority level;

-	prioritisedBitRate which sets the Prioritized Bit Rate (PBR);

-	bucketSizeDuration which sets the Bucket Size Duration (BSD).

RRC additionally controls the LCP procedure by configuring mapping restrictions for each logical channel:

-	allowedSCS-List which sets the allowed Subcarrier Spacing(s) for transmission;

-	maxPUSCH-Duration which sets the maximum PUSCH duration allowed for transmission;

-	configuredGrantType1Allowed which sets whether a configured grant Type 1 can be used for transmission;

-	allowedServingCells which sets the allowed cell(s) for transmission;

-	allowedCG-List which sets the allowed configured grant(s) for transmission;

-	allowedPHY-PriorityIndex which sets the allowed PHY priority index(es) of a dynamic grant for transmission;

-	allowedHARQ-mode which sets the allowed UL HARQ mode for transmission.

The following UE variable is used for the Logical channel prioritization procedure:

-	Bj which is maintained for each logical channel j.

The MAC entity shall initialize Bj of the logical channel to zero when the logical channel is established.

For each logical channel j, the MAC entity shall:

1>	increment Bj by the product PBR × T before every instance of the LCP procedure, where T is the time elapsed since Bj was last incremented;

1>	if the value of Bj is greater than the bucket size (i.e. PBR × BSD):

2>	set Bj to the bucket size.

1>	if logical channel j has delay critical data:

2>	set priority to delayCriticalPriority.

1>	else:

2>	set priority to priority.

NOTE:	The exact moment(s) when the UE updates Bj between LCP procedures is up to UE implementation, as long as Bj is up to date at the time when a grant is processed by LCP.

--

The last FFS point, whether a separate remaining time threshold can be configured for delay aware LCP (i.e. different from the one used for DSR). We don’t see the motivation to introduce another new parameter to make the spec more complicated.

Proposal 3: No need to introduce another new remaining time threshold for delay aware LCP (i.e., the one used for DSR can be reused).

### 3	Conclusion

This document promulgated the following proposals:

Proposal 1: Introduce a new priority RRC IE in NR RRC spec for logical channel with delay-critical data.

Proposal 2: The priority of the logical channel is changed to a signalled ‘delay-critical priority’ value when it has delay-critical data and the priority apply to whole LCH.

Proposal 3: No need to introduce another new remaining time threshold for delay aware LCP (i.e., the one used for DSR can be reused).

### 4	Reference

[1]	RP-241771, Revised WID on XR (eXtended Reality) for NR Phase 3, Nokia (Rapporteur).

[2]	R2-2405706, Report from session on R18 MBS, R18 QoE and R19 XR, Session chair (Huawei).