3GPP TSG-RAN WG2 #127-bis	R2-2409052

Hefei, China, 14th – 18th October 2024

Source:	NTT DOCOMO, INC.

Title:	Discussion on LP-WUS for CONNECTED state

Document for:	Discussion and decision

Agenda Item:	8.4.4

- Introduction

This contribution provides our view on open issues related to procedures for LP-WUS in CONNECTED.

- Discussion

  - Reuse of legacy parameters for Option 1-2

According to the assumption made in RAN1#118, in Option 1-2, LP-WUS monitoring to trigger PDCCH monitoring is configured separately from C-DRX cycle configuration.

RAN1#118 [1] Working Assumption From RAN1 perspective, for RRC CONNECTED mode, PDCCH monitoring is triggered by LP-WUS with C-DRX configuration - Support Option 1-1: LP-WUS monitoring according to the LP-WUS monitoring configuration before drx-onDurationTimer to trigger the starting of the drx-onDurationTimer. - Support Option 1-2: LP-WUS monitoring outside at least legacy C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring. - FFS whether/how to support both Option 1-1 and Option 1-2 simultaneously configured for the same UE Note: Above can be revisited considering RAN2 decisions.

An example of LP-WUS configuration is depicted in Figure 1. The LP-WUS MOs are assumed to be located not to overlap C-DRX active time.

Figure X. LP-WUS MOs are located outside of C-DRX active time in Option 1-2.

In this section, we discuss parameters to configure LP-WUS MO cycle especially in terms of whether to reuse legacy parameters for C-DRX.

On-duration timer of PDCCH monitoring window

In RAN2#127, we discussed on-duration timer of PDCCH monitoring window based on the following proposal. However, RAN2 could not conclude which way to go, reuse legacy onDurationTimer for C-DRX or define a new timer.

RAN2#127 [2] Proposal 9a: For option 1-2-2, after LP-WUS triggers the UE to perform PDCCH monitoring, the UE starts one timer. When the timer is running, the UE monitors PDCCH. The timer can be new defined timer or legacy drx-inactivityTimer.

According to RAN1 assumption on Option 1-2 (cited above), the LP-WUS MO is configured separately from C-DRX cycle. This means that Option 1-2 for LP-WUS in CONNECTED is an independent feature of the legacy C-DRX, thus we do not see necessity to reuse onDurationTimer for C-DRX. Moreover, if RAN1 concludes to allow coexistence of Option 1-1 and Option 1-2 in the future, both (1) the C-DRX-bound LP-WUS cycle and (2) a different LP-WUS cycle can be simultaneously configured. If we agree to reuse onDurationTimer for C-DRX for Option 1-2, that leads to a restriction of configuration that LP-WUS for (1) and (2) must have the same length of PDCCH monitoring. We do not think such a restriction is reasonable, therefore we propose to define a new timer.

Proposal 1.	For Option 1-2, define a new timer for PDCCH monitoring after LP-WUS triggers the UE to perform the monitoring.

MO periodicity

According to RAN1 assumption on Option 1-2 (cited above), LP-WUS Mos are assumed to be located outside of C-DRX on-duration time. Considering only this assumption, we could discuss some restrictions on the configuration of LP-WUS MO cycle, e.g., the LP-WUS MO cycle is always the same as C-DRX cycle in order to avoid overlapping of LP-WUS MO and C-DRX on-duration time.

However, in fact RAN1 has not discussed whether to support an exception handling to allow overlapping of LP-WUS MO and C-DRX on-duration time. Thus, we understand that RAN1 is still possible to support such case, so RAN2 can wait for RAN1 before discussing restrictions on P-WUS MO cycle configuration.

Observation 1.	RAN1 is still possible to allow overlapping of LP-WUS MO and C-DRX on-duration time.

- Summary and proposal

Proposal 1.	For Option 1-2, define a new timer for PDCCH monitoring after LP-WUS triggers the UE to perform the monitoring.

Observation 1.	RAN1 is still possible to allow overlapping of LP-WUS MO and C-DRX on-duration time.

- References

[1]	“RAN1 Chair’s Notes,” RAN1#118.

[2]	R2-2406577, “Analysis on LP-WUS for RRC_CONNECTED Mode,” CATT, RAN2#127.