3GPP TSG RAN WG2#127bis	R2-2407924

Hefei, China, 14th - 18th October 2024

3GPP TSG-RAN WG3 Meeting #125		R3-244793

Maastricht, Netherland, 19 – 23 August 2024

Title:	LS on Early TA acquisition for the inter-DU scenario

Response to:

Release:	Rel-18

Work Item:	NR_Mob_enh2-Core

Source:	RAN3

To:	RAN2

Cc:	RAN1

Contact person:	Jing-Rong Hsieh

jinghsieh@google.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

# 1	Overall description

- RAN2#121bis-e (R2-2304553) From RAN2 perspective, to enable shared preamble resource among multiple UEs, it is beneficial that the information that identifies the allocated CFRA resource (i.e., SS/PBCH index, RACH occasion, and Random Access Preamble index) can be indicated in the PDCCH order (as legacy intra-cell PDCCH order). - RAN2#123-bis (R2-2311332) RAN2 understands that the source DU needs to know the early RACH configuration for each candidate cell, so that the source cell can know how to set the PDCCH order information for early RACH; The candidate DU provides the TA value and its associated information to the source DU via the CU, e.g. preamble index, RO information (i.e. RA-RNTI) and candidate cell identity, so that the source DU can identify the UE. RAN3 can design the necessary network signalling.

Based on the previous RAN2 agreements, RAN3 had discussed the necessary parameters required for early TA acquisition for the inter-DU LTM scenario. RAN3 designed the Early UL Sync Configuration IE in F1AP including a RACH configuration (containing the RRC EarlyUL-SyncConfig IE) and a preamble index list assigned for the source gNB-DU. However, RAN3 lacks clarity on whether such design adequately supports the utilization of CFRA resources for inter-DU LTM scenario as effectively as it does for intra-DU LTM.

Therefore, RAN3 seeks to understand the following two questions for early TA acquisition in inter-DU LTM:

Question 1: Are shared preamble resources from the RAN2 agreement, consisting of the combination of SSB index, PRACH mask index, and preamble index, necessary, or is the preamble index list sufficient?

Question 2: When using the RA-RNTI, is “SSB Positions In Burst” a necessary parameter to associate the TA value with a specific UE?

# 2	Actions

To RAN2

ACTION: 	RAN3 kindly asks RAN2 to provide answers to the questions above.

# 3	Dates of next RAN WG 3 meetings

RAN3#125bis	14 – 18 October 2024	Hefei, China

RAN3#126	18 – 22 November 2024	Orlando, USA