3GPP TSG-RAN WG2 Meeting #128			     R2-2409740

Orlando, US, November 18 – 22, 2024

Agenda Item:	8.7.5 (NR_XR_Ph3-Core)

Source: 	LG Electronics Inc.

Title:         	Further details of RLC enhancements for XR

Document for: 	Discussion and Decision

Introduction

This contribution discusses further details of unnecessary retransmission and timely RLC retransmissions based on the RAN2 agreements so far. In addition, we present our views on questions in the SA2 LS (S2-2410999).

Discussion

Avoiding unnecessary retransmission

- RAN2 confirm the previous baseline assumption: the RLC receiving window always advances to any given RLC SN before the transmitting window does. - RAN2 will adopt a “combined” approach for avoiding unnecessary RLC retransmissions, i.e. - TX side stops transmissions of an outdated SDU - RX side abandons the SDU based on a local timer - Rx informs Tx side about the abandoned SDUs, as a baseline we assume existing SR can be reused unless issues are identified - FFS if some C-PDU handling is needed to avoid C-PDU discard - FFS if some indication is sent from Tx to Rx. The assumption is this is not a full status report, but something simple (if needed)

As per the RAN2 agreements above, RAN2 decided to adopt the “combined’ approach for avoiding unnecessary RLC retransmissions, but some companies still wanted to have an indication from Tx to Rx and two FFSes are remained.

One FFS is whether an indication is sent from Tx to Rx. During the offline discussion after XR session in the last meeting, we heard that one reason to support the indication from Tx to Rx is to reduce reordering time at the Rx. However, unlike LTE, NR RLC does not have a re-ordering functionality and only out-of-order delivery is supported. Even though the new indication from Tx to Rx is introduced and received at the Rx, there are no RLC SUDs waiting for re-ordering in the Rx window and it is useless indication for reducing re-ordering time at the RLC layer.

In our view, this indication may be useful only when the Tx window is stalled and no more new SDUs can be transmitted without advancing Tx window. However, we don’t think that Tx window stalling occurs frequently and there is a problem to advance Tx window based on the STATUS report in most cases. Another drawback is that additional synchronization mechanism would be needed to avoid unnecessary SDU discard at the Rx side since Rx side may discard received SDUs due to un-synchronization between Tx window and Rx window. Thus, it is sufficient to advance Rx window first and then Tx window advances based on the STATUS report.

Proposal 1. An indication from Tx to Rx is not supported, i.e., RAN2 only support that Rx window advances first and then Tx window advances based on the STATUS report.

Another FFS is whether some C-PDU handling is needed to avoid C-PDU discard. According to the contribution (R2-2409208) to support this FFS, their concern is that the combined approach with timer-based discard at the Rx may mistakenly discard PDCP control PDUs. However, the PDCP Control PDU is used to convey followings and the PDCP control PDU except a UDC feedback can be used for UM DRB.

  - a PDCP status report;

  - an interspersed ROHC feedback;

  - an EHC feedback;

  - a UDC feedback;

  - a PDCP SN gap report.

Considering that the UM DRB does not provide lossless transmission, it is obvious that loss of a PDCP control PDU would not cause a problem and special handling to avoid PDCP control PDU loss would not be needed so far. As long as PDCP control PDU can be used for both UM DRB and AM DRB, same principle should be applied to AM DRB as well and special handling to avoid PDCP control PDU loss would not be needed for AM DRB.

Someone may argue that a UDC feedback is only used for AM DRB and loss of the PDCP control PDU including a UDC feedback would cause a problem. In our view, however, whenever the network identifies out of synchronization between the compression buffer and de-compression buffer, the network can transmit a PDCP control PDU with a UDC checksum error notification to the UE until the network receives a PDU including a UDC header with FR=1 from the UE. This means that the network implementation can properly handle loss of the PDCP control PDU including a UDC feedback without any special handling at the UE. NOTE that a RLC control PDU is not located in the Tx and Rx window and no problem is identified.

Proposal 2. Special handling to avoid PDCP control PDU discard is not needed.

Since it is agreed that the Rx side abandons the SDU based on a local timer, RAN2 need to determine what the local timer is as a next step. The following two options can be considered for this local timer:

- Option 1: a new RLC timer is introduced;

- Option 2: t-reordering timer in PDCP is used at the RLC entity, i.e., RLC entity abandons a RLC SDU causing the SN gap after expiry of t-reordering timer in PDCP.

According to the current specification, when a missing RLC SDU is detected at the RLC entity, the PDCP entity would also detect a SN gap and then start the t-Reordering timer. After expiry of t-Reordering timer, the PDCP entity moves the receiving window forward and RLC SDU having SN lower than RX_DELIV in the PDCP entity would be discarded. Based on this observation, both options work well and only difference is dependency. In option 1, the RLC entity has its own timer to detect obsolete RLC SDUs independently and doesn’t need to check t-reordering timer and RX_DELIV in PDCP. In the option 2, however, there should be interaction between RLC and PDCP. For example, the PDCP needs to indicate to RLC entity after expiry of t-reordering timer and the RLC entity needs to check RX_DELIV to update RLC status variables. In our view, from complexity and specification impact point of view, the option 1 is clearer and better than the option 2.

Proposal 3. A new RLC timer at the Rx is introduced to determine obsolete RLC SDUs.

For the STATUS report in the “combined’ approach, when the Rx side generates the STATUS report after advancing Rx window based on abandoned RLC SDUs, it is clear that the abandoned RLC SDUs should be positively acknowledged in the STATUS report since if the abandoned RLC SDUs are not positively acknowledged in the STATUS report, the Tx side cannot synchronize the Tx window with the Rx window.

Proposal 4. The abandoned RLC SDUs determined by a new RLC timer are positively acknowledged in the STATUS report.

Timely RLC retransmissions

- RAN2 confirm that existing mechanisms are insufficient to resolve the timely RLC retransmission problem and RLC enhancements for timely RLC retransmission are investigated in Rel-19. - Exclude enhanced status reporting. - Focus the discussion on autonomous retransmission and polling enhancements, e.g. we need to understand how each option affects the capacity and packet delay

The basic RLC AM retransmission mechanism is based on a STATUS report. When the transmitting entity receives the STATUS report, the transmitting entity can retransmit only RLC SDU which is indicated by NACK in the STATAU report. This means that the transmitting entity has to delay the retransmission of the RLC SDU until the STATUS report is received. In addition, even though the transmitting entity receives the STATUS report, if the received STATUS report does not include NACK for the RLC SDU in the transmitting window, the transmitting entity cannot retransmit the RLC SDU and should wait for the next STATUS report including NACK for the RLC SDU. For this reason, RAN2 should consider two approaches together for ensuring timely RLC retransmissions: one is how to get a STATUS report for timely retransmission and another is how to retransmit the RLC SDU without the STATUS report since the received STATUS report may not include NACK for the delay-critical RLC SDU timely.

For how to get a STATUS report for timely retransmission, the current STATUS report can be triggered by polling at the transmitting entity or a SN gap detection at the receiving entity. As per the current RLC specification, the transmitting entity can include a poll bit to request a STATUS report when the number of transmitted RLC PDUs or the amount of transmitted bytes reaches a certain threshold, but there is no condition to check delay status of a RLC SDU. In this situation, even though the delay-critical RLC SDU is transmitted, the transmitting entity cannot include a poll bit unless the current condition for polling is satisfied. The transmitting entity may not receive a STATUS report before expiry of discard timer for the delay-critical RLC SDU since there is no poll included in the last transmission and the receiving entity may not detect any missing RLC SDUs. As a result, the delay-critical RLC SDU may not have another retransmission opportunity before exceeding PDB and this becomes an out-dated RLC SDU unnecessarily. We think that it would be important to receive the STATUS report after the delay-critical RLC SDU transmission to receive another retransmission opportunity and satisfy the PDB of the delay-critical RLC SDU. Thus, the transmitting entity should include a poll when the delay-critical RLC SDU is submitted to the lower layer for transmission. This enhancement does not have capacity issue and packet delay can be reduced because more retransmission opportunities can be given before exceeding PDB.

Proposal 5. The transmitting entity include a poll when the RLC SDU having remaining time lower than a threshold is submitted to the lower layer.

Even if polling enhancements to receive a STATUS report is adopted, it should be also considered how to retransmit the RLC SDU without the STATUS PDU since RLC PDUs can be missed anytime and there is still possibility not to receive the STATUS report timely. For instance, if the PDU including a poll is missed, another poll bit can be included again after expiry of a poll retransmission timer or satisfying another triggering condition which would take an unacceptable latency for delay-critical RLC SDUs. In addition, even the RLC PDU including a poll is successfully transmitted, the receiving entity cannot know exactly which RLC SDU is a delay-critical RLC SDU at the transmitting entity. The receiving entity cannot selectively include necessary NACK for the delay-critical RLC SDU and NACK in the STATUS report is determined only based on the identified missing SN from the receiving window. It would be hard to receive the STATUS report at the time point when the transmitting entity wants for retransmission of delay-critical RLC SDU. To overcome this limitation of RLC AM retransmission mechanism, we think that if remaining time of a RLC SDU in the transmitting window becomes below a threshold, e.g., the RLC SDU is indicated as a delay-critical RLC SDU by the PDCP, this delay-critical RLC SDU should be considered for retransmission without receiving NACK for this delay-critical RLC SDU.

For concerns on capacity loss due to blind autonomous retransmission, when remaining time of a RLC SDU becomes lower than a threshold, a delay status report at the MAC layer would be already triggered and transmitted depending on the configured value of the threshold. The network would give the UE both an uplink grant and the STATUS report together after receiving a DSR. In this situation, the transmitting entity can cancel the retransmission of the RLC SDU having lower than the threshold based on the positive acknowledgment in the STATUS report. This means that capacity loss would be marginal since only RLC SDUs which are not yet positively acknowledged can be retransmitted although autonomous retransmission is determined based on remaining time of a RLC SDU. From packet delay perspective, autonomous retransmission can save much time for retransmission because this does not need to wait a RTT (round trip time) to receive a STATUS report for determining retransmission of a delay-critical RLC SDU.

Observation 1. If the transmitting entity considers to retransmit the RLC SDU having remaining time lower than the threshold, capacity loss may be marginal since only RLC SDUs which are not yet positively acknowledged can be retransmitted.

Observation 2. If the transmitting entity considers to retransmit the RLC SDU having remaining time lower than the threshold, delay for retransmission can be reduced largely since there is no need to wait a RTT to receive a STATUS report.

For autonomous retransmission, there is another proposal to retransmit the RLC SDU based on HARQ feedback. However, this proposal requires huge impact to both RAN1 and RAN2 since explicit HARQ ACK/NACK for each uplink transmission should be newly defined. We think that it is unreasonable to introduce explicit HARQ ACK/NACK only for RLC retransmission determination and this should not be considered for this enhancement.

Proposal 6. If remaining time of a RLC SDU in the transmitting window becomes lower than a threshold, this RLC SDU should be considered for retransmission without receiving NACK for this RLC SDU.

Proposal 7. RLC autonomous retransmission based on HARQ feedback is not supported.

Discussion on the SA2 LS in S2-2410999

Contents of the SA2 LS in S2-2410999: Normative work for AL-FEC awareness at NG-RAN will be based on the following principles: - When requesting or updating QoS for a DL flow, an AF may provide to PCF/NEF - together with PDU Set QoS parameters - the content ratio for the flow. An AF may provide either PSIHI or content ratio for a flow. - If a PCF receives the content ratio from an AF, then the PCF may include the content ratio in PCC rules that it provides to the SMF. - The SMF provides content ratio to NG-RAN when establishing/modifying a QoS flow. - If NG-RAN supports PDU Set content ratio awareness and has received content ratio information for a QoS flow, then NG-RAN may consider content ratio information as follows: - If NG-RAN needs to discard PDUs of the QoS flow during congestion, then NG-RAN may use the content ratio information for discarding. NOTE 1: The principles concluded for AL-FEC awareness at RAN (as above) is subject to RAN-level enhancements of RLC Acknowledged Mode (RLC AM) for XR as per RP-240791 and further feasibility analyses and feedback from RAN WG2. RLC UM is not intended to be supported in this release due to concerns shared so far from RAN WG2 (see R2-2405781), pending further status update from RAN WG2. NOTE 2: RAN can, e.g. primarily discard obsolete AL-FEC PDUs of each PDU Set. NOTE 3: There is no difference in terms of how many PDUs NG-RAN will be discarded for a given QoS flow. In other words, NG-RAN will not discard more PDUs if content ratio information is provided for a QoS flow compared to when no content ratio information is provided for the same QoS flow. This scheme merely allows NG-RAN to select the PDUs to be discarded to ensure that the original content can still be recovered at the receiver. NOTE 4: Since there is no difference how many PDUs will be discarded, there are no impacts to existing congestion control algorithms. SA2 defined the content ratio as the ratio of PDUs of a PDU Set that are needed at the UE to be able to reconstruct the original content. Related to this agreement, SA2 would like to ask the following questions: Question 1: Please provide feedback on feasibility of use of content ratio information for discarding of DL PDUs during congestion for RLC AM mode based on the above principles. Please also provide feedback on applicability of RLC AM for XR services, also considering the RAN-level enhancements of RLC Acknowledged Mode (RLC AM) with small packet delay budget as per RP-240791. With respect to RLC UM, RAN indicated in R2-2405781 that "RAN2 does not think it is possible for NG-RAN to determine reliably whether a PDU has been successfully delivered over an unacknowledged-mode data bearer." After sending the previous LS in S2-2405625, SA2 clarified the principles for potential use of content ratio information for discarding of PDUs during congestion as listed above. Question 2: Please provide feedback on feasibility of use of content ratio information for discarding of DL PDUs during congestion for RLC UM based on the above principles.

Given that both Q1 and Q2 are related to discarding of DL PDUs, RAN2 should focus on DL traffic and do not need to discuss about UL traffic to answer those two questions.

For the first question, as per the RLC specification, when receiving a positive acknowledgement for an RLC SDU, the transmitting AM RLC entity shall send an indication to the upper layers of successful delivery of the RLC SDU. This means that the NG-RAN can know which DL PDU is successfully delivered based on this indication. However, considering DL PDU transmission over the CU-DU split architecture, RLC AM transmission is performed in the DU but the discarding of DL PDU is performed at the PDCP layer in the CU-UP. It is clear that there should be RAN3 signalling interactions and this requires additional delay to determine whether a specific DL PDU is successfully delivered or not. It is not sure that the NG-RAN is able to get this information about successful delivery of DL PDUs early enough to correctly determine whether the remaining DL PDUs should be discarded or not.

In addition, if remaining DL PDUs in a PDU set are discarded considering content ratio information for AL-FEC, this would create a PDCP SN gap and the NG-RAN would send the PDCP SN gap report to the UE. If the NG-RAN wants to avoid the PDCP SN gap report transmission to the UE, the NG-RAN should re-associate SN to remove the PDCP SN gap, but this requires to perform ciphering again for DL PDUs in a next PDU set with a new COUNT value. This means that the NG-RAN cannot have pre-processing gain for DL PDUs in a PDU set. Considering that satisfying short delay budget is a very important factor for XR services, losing pre-processing gains to reduce transmission latency is undesirable.

Another consideration point is impact on PDCP. Currently, PDU set discard and PSI based SDU discard are already introduced in PDCP to support XR services. If the NG-RAN wants to additionally discard remaining SDUs within a PDU set based on the content ratio information for AL-FEC of the PDU set, the NG-RAN should have another discard mechanism in the PDCP which makes the PDCP discarding operation more complicated. Furthermore, if the content ratio information for AL-FEC of a PDU set is dynamically changed, complexity of the PDCP discard operation should be even worse. Of course, if only NG-RAN does this additional discard operation and the UE does not need to do this, there may be no or marginal RAN2 specification impact, but it would increase network implementation complexity, especially on PDCP complexity.

For the applicability of RLC AM for XR services, there is no restriction to use RLC AM for XR services, but Rel-18 or earlier release RLC AM may not be suitable for XR service since RLC AM retransmission mechanism is not well adapted for short packet delay budget of XR traffic.

Proposal 8. For the Question 1, RAN2 should include the following observations in the reply LS:

- From RAN2 perspective, it may be feasible to use of content ratio information for discarding of DL PDUs during congestion for RLC AM mode, but it has negative impact on RAN performance and network implementation complexity, especially PDCP complexity.

For the second question, RAN2 already replied about UM data bearer as shown in below box. The clear RAN2 conclusion is that only relying on HARQ feedback to determine successful delivery is unreliable and the NG-RAN cannot determine reliably whether a PDU has been successfully delivered over RLC UM. It is obvious that discarding DL PDUs based on unreliable information is risky and should be avoided. Thus, it is infeasible to use of content ratio information for discarding DL PDUs during congestion for RLC UM.

Reply LS on Application-Layer FEC Awareness at RAN in R2-2405781: - Can NG-RAN determine whether a PDU was successfully delivered over an unacknowledged mode data bearer? If so, does NG-RAN get this information sufficiently early to decide whether or not to drop subsequent AL-FEC packets? Answer: RAN2 does not think it is possible for NG-RAN to determine reliably whether a PDU has been successfully delivered over an unacknowledged-mode data bearer.

Proposal 9. For the Question 2, RAN2 should include the following observations in the reply LS:

- From RAN2 perspective, it is infeasible to use of content ratio information for discarding DL PDUs during congestion for RLC UM.

Conclusion

Based on the above discussions, we present the following observations and proposals:

Observation 1. If the transmitting entity considers to retransmit the RLC SDU having remaining time lower than the threshold, capacity loss may be marginal since only RLC SDUs which are not yet positively acknowledged can be retransmitted.

Observation 2. If the transmitting entity considers to retransmit the RLC SDU having remaining time lower than the threshold, delay for retransmission can be reduced largely since there is no need to wait a RTT to receive a STATUS report.

Proposal 1. An indication from Tx to Rx is not supported, i.e., RAN2 only support that Rx window advances first and then Tx window advances based on the STATUS report.

Proposal 2. Special handling to avoid PDCP control PDU discard is not needed.

Proposal 3. A new RLC timer at the Rx is introduced to determine obsolete RLC SDUs.

Proposal 4. The abandoned RLC SDUs determined by a new RLC timer are positively acknowledged in the STATUS report.

Proposal 5. The transmitting entity include a poll when the RLC SDU having remaining time lower than a threshold is submitted to the lower layer.

Proposal 6. If remaining time of a RLC SDU in the transmitting window becomes lower than a threshold, this RLC SDU should be considered for retransmission without receiving NACK for this RLC SDU.

Proposal 7. RLC autonomous retransmission based on HARQ feedback is not supported.

Proposal 8. For the Question 1, RAN2 should include the following observations in the reply LS:

- From RAN2 perspective, it may be feasible to use of content ratio information for discarding of DL PDUs during congestion for RLC AM mode, but it has negative impact on RAN performance and network implementation complexity, especially PDCP complexity.

Proposal 9. For the Question 2, RAN2 should include the following observations in the reply LS:

- From RAN2 perspective, it is infeasible to use of content ratio information for discarding DL PDUs during congestion for RLC UM.