3GPP TSG-RAN WG2 Meeting #128	R2-2409768

Orlando, USA, 18th – 22nd Nov. 2024

Source:	vivo

Title:	Discussion on RRM measurement gaps enhancements

Agenda Item:	8.7.3

Document for:	Discussion and Decision

- Introduction

In RAN#127bis [1], the following agreements were made regarding the MG skipping for XR transmission enhancements:

Agreements for RRM measurement gap skipping - RAN2 assumes that at least some impact on DSR from MG skipping can be avoided by NW implementation. FFS whether there is an impact which would require some specification changes/enhancements. - No need to have delay-aware LCP enhancements specific for MG skipping, i.e. MG skipping and delay-aware LCP are designed as independent features - RAN2 can further evaluate whether there is any impact on DRX from MG skipping. For the moment, the issue is unclear. - RAN2 will focus its work on supporting the solution chosen by RAN1/RAN4 - RAN2 can discuss whether there is a need to additionally have other solution (e.g. RRC-based) which can be decided after RAN1/RAN4 evaluation and knowing more details of DCI-based solution

In this contribution, we first discuss the enhancements related to MG skipping for DSR and DRX. Then we discuss the UE assistant information and RRC-based MG skipping solution for the MG skipping enhancement.

- Discussion

DSR enhancement in case of cancellable MG

In RAN2#127bis meeting [1], DSR enhancement in case of cancellable MG was discussed, and the following agreement was reached:

- RAN2 assumes that at least some impact on DSR from MG skipping can be avoided by NW implementation. FFS whether there is an impact which would require some specification changes/enhancements.

The DSR is used by the network to timely understand the delay status of UL packets in the UE buffer. This allows the network can decide whether it needs to allocate more scheduling resources to speed the data transmission or to adjust the scheduling strategy. If DSR is reported before a cancellable MG, the network can determine whether to indicate UE to skip the MG, enabling it to be used for transmitting delay-critical UL packets. In such cases, it is essential that the DSR can be reported with a sufficient lead time before the MG occasion so that there is enough time for the UE to process the DCI indicating MG skipping.

- It is a reasonable assumption that a gNB can determine the cancellable MG skipping based on the DSR report received with a certain offset before the cancellable MG occasion.

As illustrated in Figure 1, if DSR is triggered during a cancellable MG or close to a cancellable MG (i.e., there is not enough time for reporting the DSR and receiving the MG skipping DCI due to MG), then the network cannot receive DSR in time to determine and indicate the MG skipping. This results in wasted radio resources and increased packet delays.

- DSR report is delayed when DSR is triggered during MG

- There is not enough time for DSR report before MG.

Figure 1 example for gNB cannot get DSR in time for configuring MG cancellation

- Depending on the overlapping status between the PDCP discardTimer running duration and the cancellable MG during, the DSR transmission is delayed and the gNB cannot determine MG skipping in time in the following two cases:

- The DSR is triggered during the MG and can only be transmitted after the MG.

- The DSR is triggered before the MG, but the offset to the cancellable MG is too short for gNB to receive DSR or indicate the MG skipping.

Network implementation can partially address this issue, for example, by configuring a higher remaining time threshold that takes into account the length of MG to trigger DSR earlier. However, indiscriminately increasing this threshold could also lead to frequent triggering of DSR. Furthermore, even with a high threshold configured, scenarios like those depicted in Figure 1 may still occur because overlapping between the PDCP discardTimer duration and MG duration can happen regardless of the threshold setting.

Therefore, early DSR triggering and reporting before a cancellable MG should be supported when there is an overlapping between PDCP discardTimer running duration and MG duration.

- RAN2 to consider early DSR triggering and reporting before a cancellable MG when there is an overlapping between PDCP discardTimer running duration and MG duration.

There may be cases where a triggered DSR occurs too close to the MG and the DSR report can only be initiated after the MG (i.e., the second scenario in Figure 1). In our opinion, such a triggered DSR should not be canceled due to the MG, because even a delayed DSR report could still be valuable. For example, in case of light network situation, there might still be enough time and radio resources for the network to schedule the delay-critical data transmission even if the DSR report is delayed by an MG. Even if some packets are successfully transmitted and exceed the PDB, they may still be useful for the decoding of other packets, considering the possible inter-PDU relationship in XR service.

- Even if a triggered DSR cannot be transmitted before a cancellable MG and is transmitted to gNB after the MG, it can be still helpful somehow for delay-critical data transmission after MG.

- Do not support cancellation of DSR that cannot be transmitted early enough before a cancellable MG.

DRX enhancements in case of cancellable MG

According to the RAN1 working assumption, DCI is used to dynamically indicate the skipping of a particular measurement gap/restriction. As illustrated in Figure 2 below, if the start time of a cancellable MG overlaps with DRX inactive time, the gNB will be unable to notify the UE of the MG cancellation through DCI because the UE will not monitor PDCCH during DRX inactive time.

Figure 2 Example of unable to notify MG cancellation by gNB in case of DRX operation

- When UE falls to sleep according to DRX pattern before a cancellable MG, the gNB is not able to notify the UE to cancel this MG.

If the cancellable MG is not skipped and it coincides with the DRX active period, the UE will be unable to perform data transmission. To address this issue, UE should be allowed to temporarily monitor PDCCH before a cancellable MG. One solution is to suspend the DRX operation, which means that the UE would continue to monitor PDCCH regardless of the established DRX pattern before a cancellable MG. Alternatively, DRX timers could be extended to ensure that the UE is in its active period before a MG occasion (for example, by extending the first DRX OnDurationTimer as shown in Figure 2, enabling the UE to monitor PDCCH and receive DCI indication to skip the upcoming MG).

- Allow UE to temporarily monitor PDCCH by considering the following methods:

  - Suspending DRX operation (i.e., UE continues to monitor PDCCH regardless of DRX pattern) before a cancellable MG.

  - Extending current DRX timers

UE assistant information

Discussion on whether and what kind of UE assistance information should be introduced for measurement gap/restrictions related enhancements have taken place in RAN1. Since measurement occasions affect RAN4, it should be RAN4 to make the decision on this issue. RAN1 has sent an LS to RAN4 [2] in RAN1#117 meeting. In our opinion, RAN2 should wait for RAN4’s conclusions regarding UE assistance information related to measurement occasions. If RAN4 agrees to introduce information related to measurement occasions in UAI, then RAN2 can proceed with discussing the corresponding signaling and procedural design.

- RAN2 waits for RAN4’s conclusions on UE assistance information related to measurement occasions before further discussion.

RRC-based MG skipping solution

In RAN2#127bis meeting, the RRC-based MG (Measurement Gap) skipping solution was discussed, and many companies expressed interest in supporting this RRC-based approach. From RAN2 perspective, we believe that the RRC-based solution offers advantages over the DCI-based solution in at least the following two scenarios:

- Scenario 1: When MG occasions overlap with data whose arrival can be predicted.

- Scenario 2: When CG scheduling is configured.

For scenario 1, given that XR traffic is quasi-periodic (e.g., with periodicity of 16.67ms with jitter), it is possible to predict the overlap of MG occasions with a significant portion of packets based on the traffic period, jitter range, and MG configuration. Therefore, it is more appropriate to employ an RRC-based MG skipping solution (i.e., RRC configures the MG skipping patterns) rather than a DCI-based solution for these packets. The RRC-based solution addresses situations where the overlap of MG occasions and data can be anticipated, whereas the DCI-based solution addresses situations where such overlapping cannot be predicted due to packet arrival jitter and changes in data burst size. The RRC and DCI-based solutions are complementary and could be jointly utilized for measurement gap skipping.

As for scenario 2, the CG scheduling method is a kind of main scheduling method for UL XR traffic, if CG is configured and DCI-based MG skipping solution is used, the network may need to indicate UE before the CG occasions to skip the MG every time with DCI which have no data to schedule, which is inefficient.

Considering the above analysis, we think RAN2 should support the RRC-based MG skipping solution. Naturally, detailed discussions about this approach can start once we have more information about the DCI-based solution from RAN1/4 and determine what is required for the RRC-based solution.

- RAN2 confirms that RRC-based solution indicating the occasions where measurement gaps/restrictions should be skipped is supported.

- Conclusion

In this paper, we have discussed the DSR and DRX enhancements in case of cancellable MGs, and then we have discussed the UAI and RRC-based MG skipping solution. The following observations and proposals are made:

DSR enhancement in case of cancellable MG

- It is a reasonable assumption that a gNB can determine the cancellable MG skipping  based on the DSR report received with a certain offset before the cancellable MG occasion.

- Depending on the overlapping status between the PDCP discardTimer running duration and the cancellable MG during, the DSR transmission is delayed and the gNB cannot determine MG skipping in time in the following two cases:

- The DSR is triggered during the MG and can only be transmitted after the MG.

- The DSR is triggered before the MG, but the offset to the cancellable MG is too short for gNB to receive DSR or indicate the MG skipping.

- Even if a triggered DSR cannot be transmitted before a cancellable MG and is transmitted to gNB after the MG, it can be still helpful somehow for delay-critical data transmission after MG.

- RAN2 to consider early DSR triggering and reporting before a cancellable MG when there is an overlapping between PDCP discardTimer running duration and MG duration.

- Do not support cancellation of DSR that cannot be transmitted early enough before a cancellable MG.

DRX enhancements in case of cancellable MG

- When UE falls to sleep according to DRX pattern before a cancellable MG, the gNB is not able to notify the UE to cancel this MG.

- Allow UE to temporarily monitor PDCCH by considering the following methods:

  - Suspending DRX operation (i.e., UE continues to monitor PDCCH regardless of DRX pattern) before a cancellable MG.

  - Extending current DRX timers

UE assistant information

- RAN2 waits for RAN4’s conclusions on UE assistance information related to measurement occasions before further discussion.

RRC-based MG skipping solution

- RAN2 confirms that RRC-based solution indicating the occasions where measurement gaps/restrictions should be skipped is supported.

- References

- RAN2-127bis, Chair notes

- R1-2405736, LS on UE assistance information, RAN1#117