3GPP TSG-RAN WG2 Meeting #128	R2-2409865

Orlando, USA, 18 - 22, Nov, 2024

Agenda Item:	8.1.3

Source:	Xiaomi

Title:	Discussion on NW-side data collection

Document for:	Discussion

Introduction

For NW side model, data collection is essential to collect training data. In this contribution, we discuss open issues of NW-side data collection.

Discussion

Common procedure for BM and Pos

If the data is collected and reported by UE, data collection procedure can be divided to two steps. UE first initiates data collection and log data. Then, UE can report the data to NW based on report trigger. In the following sections, we would discuss data collection initiation and data report respectively.

Data collection control

Considering training data may be very large, data logging shall be supported, which means UE had started to collect data before report. One important aspect is how to initiate UE to perform data collection. The simplest way is NW can send explicit indication to start or stop data collection. It’s FFS whether multiple data collection configuration can be provided to UE. We understand the data would be use cases specific, e.g. BM and CSI. Even in the same use case, the data may be sub use case specific. For example, one data sample of spatial BM prediction consists measurement results of set A and B beam at the same time. However, one data sample of temporal BM prediction requires historic measurement results of set B beam and one measurement result of set A beam. Therefore, NW should be allowed to provide multiple data collection configurations for different (sub) use cases.

Proposal 1: Multiple data collection configuration can be configured to UE.

A subsequent question is whether dynamic activation/deactivation of data collection is supported. We understand the real question is whether L1/L2 signalling is needed. It’s always possible for NW to setup/release the data collection configuration by RRC. From the LS from RAN1 [1], it can be seen the latency requirement of training data collection is rather relaxed. There is no need to introduce L1/L2 dynamic control from latency point of view.

Proposal 2: L1/L2 signalling based dynamic activation/deactivation of data collection is not supported.

It’s agreed the radio condition-based event triggered logging is supported. We understand NW may only need data UE collected under certain radio channel condition. Because model may be different under different radio channel condition. Radio channel may change due to UE movement and NW may need to frequent send indication to start or stop data collection. In this release the use cases are all within serving cell. So the radio condition should only based on serving cell’s radio condition. RSRP/RSRQ/SINR can be supported.

Proposal 3: Only serving cell’s radio condition is supported in logging event.

Proposal 4: RSRP/RSRQ/SINR can be supported in logging event.

The straightforward way to support event is based on threshold. Similar as legacy MR event, the event fulfilment can be based on radio condition, e.g. RSRP, above or lower than the threshold. One open issue is whether TTT like timer is needed. In legacy TTT like timer is introduced to avoid frequent measurement report. However, the data logging event trigger data logging, but doesn’t trigger report. There is no need to support TTT like timer for data logging event.

Proposal 5: Logging event fulfilment is evaluated based on radio condition above or lower than the threshold. Threshold is configured by NW.

Proposal 6: TTT like timer is not supported for data logging event.

It’s agreed measurements for data collection purposes and logging based can be controlled based on power state of the UE. It’s FFS whether the UE stops autonomously or it reports to the NW. We understand the UE shall stop the data collection autonomously. Because it’s up to UE implementation how to determine power state. UE may not be able to continue the data collection due to low power or power control. UE has no choice but to stop the data collection.

Proposal 7: UE autonomously stops data collection if power state doesn’t allow UE to collect data.

The data logging could reduce the number of reports. However, it would also consume UE storage, which increase the complexity and cost. It shall be optional for UE to support data logging.

Proposal 8: It’s optional for UE to support data logging for NW side data collection.

Even if UE supports the data logging, it’s possible that different UE chipset/vendor has different implementation. The supported storage size may be different. As indicated in [1], different type of training data may require different storage size. Also, the data report configuration may be associated with the storage size, e.g. longer report period would result in UE storing more data. UE can report the supported data logging storage size to NW. NW can decide which data is to be logged by UE and the appropriate configuration, e.g. report period.

Proposal 9: UE can support different data logging storage size, which can be indicated by UE capability.

Data report

It’s agreed UE can indicates the availability of logged data to NW to assist NW to trigger UEInformationRequest. However, it’s not clear the trigger of the availability. We understand the availability indication shall be triggered upon UE power state low. If UE power state is low, UE would stop the data collection. If UE has logged data, UE can report the data availability to NW. NW can decide whether UE shall report or keep the data and wait for power state recover. This is useful, since NW may not know whether UE has collected any data before power state low. Also, if logged data has reached buffer limit, UE can’t log more data. NW should be told and NW shall indicate UE to report the logged data as soon as possible. As discussed above, NW may perform different handling in these two cases. Therefore, NW also need to know the availability indication is triggered by power state low or buffer.

Proposal 10: UE triggers data availability indication report in following cases,

- if power state is low and there is logged data.

- if logged data reached buffer limit

Proposal 11: UE indicates the data power state low or reaching buffer limit in the data availability indication.

It’s not clear which RRC message is used to report this availability indication. According to the above discussion, the trigger can occur at any time. So UE should be able to initiate the RRC message anytime. Potential message can be UAI or MR or new message. We understand this indication is similar as assistance information. So, UAI is preferred.

Proposal 12: UAI is used to report the data availability indication.

After data report, UE can release the logged collected data. UE doesn’t need to log the old data anymore. It can be released, so that new data can be collected.

Proposal 13: UE releases the logged data after report.

If NW has collected enough data, NW may release the data collection configuration. In this case, UE shall also release the logged data. But if NW modify the data collection configuration, e.g. modify set A, it’s not clear whether UE shall release or keep the logged data. We understand UE shall still keep the logged data, since NW may still need the old data. If NW doesn’t need the old data, NW modify the configuration by release and setup procedure. So, NW has the flexibility to control whether UE can keep the data implicitly.

Proposal 14: If NW release the data collection configuration, UE releases the logged data.

Proposal 15: If NW modify the data collection configuration, UE keeps the logged data.

Mobility handling

RAN1 agreed the NW side additional condition can be associated with a ID. This ID is unique within a cell. Therefore, different cell may be associated with different IDs. Furthermore, it’s difficult to ensure the ID is global unique. Therefore, it’s possible the same ID means different NW side additional condition in two cells. When UE moves from one cell to another, UE may log data collected from multiple cells. These cells may be associated with different NW side additional condition. Therefore, when UE reports the data, UE shall indicate the data and the associated NW information. The associated NW information can be cell ID or associated ID.

Proposal 16: if logged data are collected from multiple cells, UE report the logged data and associated NW information. The associated NW information can be cell ID or associated ID.

NW-side data collection for Pos

(2nd priority) Case 2b: UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning

(1st priority) Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning

(1st priority) Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning

(2nd priority) Case 2b: UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning

(1st priority) Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning

(1st priority) Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning

For AI positioning, there are there use cases with the AI model deployed at Network side, however, we only consider the first priority use cases in this section.

For positioning use cases, when considering LMF-side inference, it is assumed that the LPP protocol should be applied to the data collected by UE and terminated at LMF, while the NRPPa protocol should be applied to the data collected by gNB and terminated at LMF. While for LMF-side performance monitoring, it is assumed that the LPP protocol should be applied to the data collected by UE and terminated at LMF, while the NRPPa protocol should be applied to the data collected by gNB and terminated at LMF.

Note:	For gNB- and OAM-centric data collection, there may be a need to consult with RAN3 and SA5 whether/how OAM is to be involved.

Note:	For possible impacts due to positioning use cases, there may be a need to consult with RAN3 whether/how NRPPa is to be involved.

For positioning use cases, when considering LMF-side inference, it is assumed that the LPP protocol should be applied to the data collected by UE and terminated at LMF, while the NRPPa protocol should be applied to the data collected by gNB and terminated at LMF. While for LMF-side performance monitoring, it is assumed that the LPP protocol should be applied to the data collected by UE and terminated at LMF, while the NRPPa protocol should be applied to the data collected by gNB and terminated at LMF.

Note:	For gNB- and OAM-centric data collection, there may be a need to consult with RAN3 and SA5 whether/how OAM is to be involved.

Note:	For possible impacts due to positioning use cases, there may be a need to consult with RAN3 whether/how NRPPa is to be involved.

According to the TR 38.843 [2], we had made some conclusions on NW-sided data collection and Model training for positioning as below.

Model Training:

o	For gNB-side model, training data can be generated by the gNB, while the termination point for training data may include the gNB, or OAM.

	Note: RAN2 identified the case in which LMF may be used for gNB-side model training. However, no conclusion was reached, as this depends on the RAN1 progress.

o	For LMF-side model, the LMF is the termination point for training data.

Model Training:

o	For gNB-side model, training data can be generated by the gNB, while the termination point for training data may include the gNB, or OAM.

	Note: RAN2 identified the case in which LMF may be used for gNB-side model training. However, no conclusion was reached, as this depends on the RAN1 progress.

o	For LMF-side model, the LMF is the termination point for training data.

Based on above conclusion, we have some understandings on the data collection for positioning. For Case 3a, if the termination point for training data is the gNB, there may be no specification impact on data collection. However, if the termination point is the LMF, the NRPPa protocol should be used to report the dataset from the gNB to the LMF for model training. Since we haven’t agreed that the LMF is the termination point for training data in Case 3a, we won’t consider the LMF as the termination point for now.

For Case 3b, the LMF is responsible for data collection, and the NRPPa protocol should be used to report the dataset from the gNB to the LMF for model training.

Proposal 17: For Case 3a, if the termination point for training data is the gNB, there may be no specification impact on data collection.

Proposal 18: For Case 3b, the LMF is responsible for data collection, and the NRPPa protocol should be used to report the dataset from the gNB to the LMF for model training.

For the network side data collection for AI positioning, SA2 had made some progresses as below [3]:

Figure 5.18.1-1: Data collection by LMF to train the AI/ML based positioning model using UE measurements

The LMF needs to obtain input data for AI/ML based positioning model training and may also request the UE to provide the UE location. The AI/ML based positioning model is trained to perform UE positioning for UEs located in an area of interest that may expand over multiple TA or cover multiple NG-RAN nodes.

1.	The LMF starts data collection for the purpose to train a AI/ML Model for UE Positioning. The LMF subscribes to AMF to retrieve the list of SUPIs located in an area of interest using Namf_EventExposure_Subscriber_Request (Target of Event Reporting = "any UE", Event ID = "UEs in/out area of interest".

2.	The AMF send Namf_EventExposure_Subscriber_Response ("list of SUPIs in the area of interest").

3.	The LMF checks the user consent for data collection for model training.

4.	For each UE that provides consent to data collection for model training and can support data collection the LMF initiates a request for input data.

The LMF needs to obtain input data for AI/ML based positioning model training and may also request the UE to provide the UE location. The AI/ML based positioning model is trained to perform UE positioning for UEs located in an area of interest that may expand over multiple TA or cover multiple NG-RAN nodes.

1.	The LMF starts data collection for the purpose to train a AI/ML Model for UE Positioning. The LMF subscribes to AMF to retrieve the list of SUPIs located in an area of interest using Namf_EventExposure_Subscriber_Request (Target of Event Reporting = "any UE", Event ID = "UEs in/out area of interest".

2.	The AMF send Namf_EventExposure_Subscriber_Response ("list of SUPIs in the area of interest").

3.	The LMF checks the user consent for data collection for model training.

4.	For each UE that provides consent to data collection for model training and can support data collection the LMF initiates a request for input data.

According to SA2 conclusion, the LME will request the UE in an area of interest to perform data collection. Which means that the UE should stop data collection if the UE is not in the area of the intertest. Therefore, we think the LMF should send the area of interest to the UE for UE performing data collection.

Proposal 19: For Case 3b, the LMF sends the area of interest of data collection to the UE when requesting it to perform the data collection.

For positioning, the UE measures the PRS based on the PRS configuration provided by the LMF, which is common for all UEs. Consequently, the PRS configurations are identical for both data collection and positioning. Therefore, the LMF may also indicate the specific PRS configuration to the UE when requesting data collection.

Proposal 20: For Case 3b, the LMF may indicate the PRS configuration for the UE performing data collection when LMF request the UE to perform the data collection.

Conclusion

Based on the discussion in section 2, we have following proposals:

For both BM and Pos:

Data collection control

Proposal 1: Multiple data collection configuration can be configured to UE.

Proposal 2: L1/L2 signalling based dynamic activation/deactivation of data collection is not

supported.

Proposal 3: Only serving cell’s radio condition is supported in logging event.

Proposal 4: RSRP/RSRQ/SINR can be supported in logging event.

Proposal 5: Logging event fulfilment is evaluated based on radio condition above or lower than the threshold. Threshold is configured by NW.

Proposal 6: TTT like timer is not supported for data logging event.

Proposal 7: UE autonomously stops data collection if power state doesn’t allow UE to collect data.

Proposal 8: It’s optional for UE to support data logging for NW side data collection.

Proposal 9: UE can support different data logging storage size, which can be indicated by UE capability.

Data report

Proposal 10: UE triggers data availability indication report in following cases,

- if power state is low and there is logged data.

- if logged data reached buffer limit

Proposal 11: UE indicates the data power state low or reaching buffer limit in the data availability indication.

Proposal 12: UAI is used to report the data availability indication.

Proposal 13: UE releases the logged data after report.

Proposal 14: If NW release the data collection configuration, UE releases the logged data.

Proposal 15: If NW modify the data collection configuration, UE keeps the logged data.

Mobility handling

Proposal 16: if logged data are collected from multiple cells, UE report the logged data and associated NW information. The associated NW information can be cell ID or associated ID.

For Pos:

Proposal 17: For Case 3a, if the termination point for training data is the gNB, there may be no specification impact on data collection.

Proposal 18: For Case 3b, the LMF is responsible for data collection, and the NRPPa protocol should be used to report the dataset from the gNB to the LMF for model training.

Proposal 19: For Case 3b, the LMF sends the area of interest of data collection to the UE when requesting it to perform the data collection.

Proposal 20: For Case 3b, the LMF may indicate the PRS configuration for the UE performing data collection when LMF request the UE to perform the data collection.

Reference

[1] R2-2311720	Reply LS on Data Collection Requirements and Assumptions

[2] TR 38.843 v18.0.0 Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface

[3] TS 23.273-j00, Technical Specification Group Services and System Aspects; 5G System (5GS) Location Services (LCS)