3GPP TSG-RAN WG2 Meeting #128	R2-2409991

Orlando, USA, Nov 18th – Nov 22nd, 2024

Agenda item:	8.3.5

Source:	Qualcomm Incorporated

Title:	Simulation assumptions and methodology for Measurement Event prediction, RLF prediction, and SLS

WID/SID:	FS_NR_AIML_Mob – Release 19

Document for:	Discussion and Decision

# Introduction

In the last RAN2 meeting (RAN2 #127-bis [1]), the focus was on two topics – Measurement Event prediction, Simulation results for RRM measurement prediction – and very good progress was made on both the topics; for the agreements made during the meeting, please refer to the Chair’s notes [1].

In the last RAN2 meeting (RAN2 #127-bis [1]), a post-meeting email discussion, [POST127bis][022][AI mobility], was set up to discuss the simulation assumptions, parameters, and methodology for Measurement Event prediction, RLF prediction, and System Level Simulation (SLS).

In this contribution, we discuss some aspects of Measurement Event prediction, RLF prediction, and SLS, that were brought up in the post-meeting email discussion [POST127bis][022][AI mobility].

# Measurement Event prediction

In the email discussion [POST127bis][022][AI mobility], there is a discussion on how to define Indirect and Direct Event prediction. We provide below our understanding regarding the definitions.

Indirect Measurement Event prediction for temporal domain Case A

In this case, the prediction of occurrence of the measurement event can consider both the predicted measurements of the prediction window (PW) and the historical actual measurements, as shown in Figure 1. In the example in Figure 1, at the current time, the historical actual measurements as well as the predicted measurements satisfy the entering condition of the event, e.g., A3, for the entire TTT (Time to trigger) duration. Considering both the predicted measurements and the historical actual measurements enables the UE to transmit the Measurement Event prediction report to the network in a timely manner.

Figure 1: Indirect Event prediction for temporal domain Case A

Indirect Measurement Event prediction for temporal domain Case B

In this case, the prediction of occurrence of the measurement event can consider both the predicted measurements of the prediction and observation windows and the historical actual measurements, as shown in Figure 2. The predicted measurements in the observation window are also the measurements that are skipped by the UE.

Figure 2: Indirect Event prediction for temporal domain Case B

Indirect Measurement Event prediction for inter-frequency measurements

In inter-frequency RRM measurement prediction, the UE predicts the measurements of a target frequency based on the measurements of the serving frequency.

Based on the above discussion, we have the following proposal for the definitions.

Proposal 1. The following are the definitions of Indirect Event prediction for the various cases.

Indirect Measurement Event prediction for temporal domain Case A

In this case, the prediction of occurrence of measurement event at a future time is based on the predicted measurement(s) of the prediction window (PW) and the historical actual measurement(s), i.e., the predicted and historical actual measurements satisfy the event condition, e.g., event A3, for a TTT (time to trigger) duration.

Indirect Measurement Event prediction for temporal domain Case B

In this case, the prediction of occurrence of measurement event at a future time or at the current time is based on the historical actual measurement(s) and the predicted measurement(s) in the prediction and/or observation windows.

Indirect Measurement Event prediction for inter-frequency measurements

In this case, the prediction of occurrence of measurement event at a future time or at the current time is based on the historical actual measurement(s) of the serving frequency and the predicted measurement(s) of the target frequency.

Direct Measurement Event prediction

It was agreed in previous RAN2 meeting that “For direct measurement event prediction, the model output is the probability of event occurrence within a time window.” Furthermore, the input for the AI/ML model can be the RRM measurements; other inputs are not precluded.

Proposal 2. For Direct Measurement Event prediction, the time window is taken to start at the current time and extends into a time into the future, which is a parameter of the simulations. For the probability threshold parameter, it should be determined based on accuracy metric (KPI) performance that is targeted to be achieved.

# RLF prediction

We propose similar definitions for Indirect RLF prediction and Direct RLF prediction as for the measurement event prediction case.

Proposal 3. The following are the definitions for Indirect RLF prediction and Direct RLF prediction.

Indirect RLF prediction

In Indirect RLF prediction, the prediction of occurrence of RLF at a future time is based on the predicted SINR measurement(s) of the prediction window (PW) and the historical actual measurements (temporal domain prediction Case A), i.e., based on the predicted and the historical actual measurements, N311 consecutive in-sync indications are not predicted to occur and T310 expires.

Direct RLF prediction

For Direct RLF prediction, the time window is taken to start at the current time and extends into a time into the future, which is a parameter of the simulations. Direct RLF prediction is based on historical actual SINR measurements and optionally, other inputs.

Discussion regarding simulating interference for the purposes of computing SINR for RLF prediction

In the email discussion [POST127bis][022][AI mobility], the UE is assumed to receive interference from all neighbor cells. A 3-tier deployment scenario can be considered for this purpose, which yields more instances where SINR can be computed and RLF prediction performed. For a UE in a serving cell, most of the interference comes from the cells of the surrounding six sites. Interference also comes from the two co-site cells, as we consider 3 sectors/cell per site. E.g., in the 3-tier deployment scenario of Figure 3, a UE in a sector in site 1 experiences interference due to transmissions in sites 2 through 7, and from the neighbor sectors in site 1, i.e., from the two co-site cells.

Figure 3: 3-tier deployment scenario

On the interference modeling issue, we think that there is no need to model interference from neighbor cells for the SINR computation, and the reason for this is the following. For RLF detection, the UE measures RLM-RS reference signals which could be SSBs or CSI-RSs. During RLM-RS processing at the UE, a processing gain (in dB) can be assumed to be applied – an example of a value of the processing gain is 10dB. This effectively results in suppression of interference due to transmissions from the neighbor cells. The processing gain comes from the fact that RLM-RS processing involves correlation with known sequences, e.g., in the PSS and the SSS in the case the RLM-RS is SSB.

For the serving cell, the received signal strength at the UE can be computed based on the top few strongest beams or the strongest beam.

If companies would still like to model interference from neighbor cells, then we do not prefer that the approach where the neighbor cells are assumed to be “fully loaded” is used since it is a worst-case scenario and would overestimate the interference. Of the following three options proposed in the email discussion [POST127bis][022][AI mobility],

- Option 1: the interference comes from the beam with maximum RSRP of neighbor cells,

- Option 2: the interference comes from fixed beam pattern of neighbor cells,

- Option 3: the interference comes from randomly chosen beam(s) of neighbor cells,

our preference is a randomized approach similar as Option 3.

To summarize, we have the following observations and proposals.

Observation 1. There is no need to model interference from neighbor cells for the SINR computation since RLM-RS processing at the UE involves a processing gain (an example value of the processing gain is 10dB), that effectively results in suppression of interference due to transmissions from the neighbor cells. The processing gain comes from the fact that RLM-RS processing involves correlation with known sequences, e.g., in the PSS and SSS in the case the RLM-RS is SSB.

Proposal 4. Our preference is not to model interference from neighbor cells for the SINR computation.

Observation 2. If companies would still like to model interference from neighbor cells, then we do not prefer that the approach where the neighbor cells are assumed to be “fully loaded” is used since it is a worst-case scenario and would overestimate the interference.

Proposal 5. If companies would like to model interference from neighbor cells, our preference is as follows.

- Processing gain is applied in the SINR computation. Recommended value for the processing gain is 10dB.

- Use an approach similar as Option 3 (interference comes from randomly chosen beams(s) of neighbor cells). Details are FFS.

# 4 System Level Simulation (SLS)

In this section, we discuss handover modeling and timeline for SLS.

At time t0, we assume that the UE predicts that event A3 condition will be satisfied for TTT duration, at the future time t1. The UE transmits a Measurement Event prediction report to the source gNB (see Figure 3). The Measurement Event prediction report can contain the source cell and target cell measurement predictions for the prediction window duration of time and the source cell and target cell historical actual measurements.

Figure 4: Aspects of handover modelling for SLS

Based on the received Measurement Event prediction report, the source gNB may initiate handover preparation procedure for the target cell.

Let’s consider the case where at a later time t2, the event A3 is triggered by the actual measurements, and that the network hasn’t transmitted a Handover Command to the UE by then. Following legacy behavior, UE transmits an RRC Measurement Report to the source gNB.

Upon receiving the RRC Measurement Report:

- If the target cell indicated in the RRC Measurement Report is the same as that indicated in the Measurement Event prediction report, the source gNB waits for the ongoing handover preparation to complete, and then transmits a Handover Command message to the UE.

- If the target cell indicated in the RRC Measurement Report is different from that indicated in the Measurement Event prediction report, the source gNB initiates another handover preparation procedure for the indicated target cell and transmits a Handover Command to the UE upon completion of the procedure.

A similar behavior is followed if the UE transmits another Measurement Event prediction report following an initial report.

Proposal 6. If the UE predicts that the measurement event will be triggered in the future, the UE transmits a Measurement Event prediction report to the source gNB. The Measurement Event prediction report can contain the source cell and target cell measurement predictions for the prediction window duration of time and the source cell and target cell historical actual measurements.

Proposal 7. Based on the received Measurement Event prediction report, the source gNB may initiate handover preparation procedure for the target cell.

Proposal 8. If the source gNB receives subsequently an RRC Measurement Report from the UE:

- If the target cell indicated in the RRC Measurement Report is the same as that indicated in the Measurement Event prediction report, the source gNB waits for the ongoing handover preparation to complete, and then transmits a Handover Command message to the UE.

- If the target cell indicated in the RRC Measurement Report is different from that indicated in the Measurement Event prediction report, the source gNB initiates another handover preparation procedure for the indicated target cell and transmits a Handover Command to the UE upon completion of the procedure.

Proposal 9. A similar procedure as in the above proposal is followed if the UE transmits another Measurement Event prediction report following an initial report.

# 3 Conclusions

Based on the above discussions, we recommend that RAN2 discuss the following observations and proposals.

Measurement Event prediction

Proposal 1. The following are the definitions of Indirect Event prediction for the various cases.

Indirect Measurement Event prediction for temporal domain Case A

In this case, the prediction of occurrence of measurement event at a future time is based on the predicted measurement(s) of the prediction window (PW) and the historical actual measurement(s), i.e., the predicted and historical actual measurements satisfy the event condition, e.g., event A3, for a TTT (time to trigger) duration.

Indirect Measurement Event prediction for temporal domain Case B

In this case, the prediction of occurrence of measurement event at a future time or at the current time is based on the historical actual measurement(s) and the predicted measurement(s) in the prediction and/or observation windows.

Indirect Measurement Event prediction for inter-frequency measurements

In this case, the prediction of occurrence of measurement event at a future time or at the current time is based on the historical actual measurement(s) of the serving frequency and the predicted measurement(s) of the target frequency.

Proposal 2. For Direct Measurement Event prediction, the time window is taken to start at the current time and extends into a time into the future, which is a parameter of the simulations. For the probability threshold parameter, it should be determined based on accuracy metric (KPI) performance that is targeted to be achieved.

RLF prediction

Proposal 3. The following are the definitions for Indirect RLF prediction and Direct RLF prediction.

In Indirect RLF prediction, the prediction of occurrence of RLF at a future time is based on the predicted SINR measurement(s) of the prediction window (PW) and the historical actual measurements (temporal domain prediction Case A), i.e., based on the predicted and the historical actual measurements, N311 consecutive in-sync indications are not predicted to occur and T310 expires.

For Direct RLF prediction, the time window is taken to start at the current time and extends into a time into the future, which is a parameter of the simulations. Direct RLF prediction is based on historical actual SINR measurements and optionally, other inputs.

Simulating interference for the purposes of computing SINR for RLF prediction

Observation 1. There is no need to model interference from neighbor cells for the SINR computation since RLM-RS processing at the UE involves a processing gain (an example value of the processing gain is 10dB), that effectively results in suppression of interference due to transmissions from the neighbor cells. The processing gain comes from the fact that RLM-RS processing involves correlation with known sequences, e.g., in the PSS and SSS in the case the RLM-RS is SSB.

Proposal 4. Our preference is not to model interference from neighbor cells for the SINR computation.

Observation 2. If companies would still like to model interference from neighbor cells, then we do not prefer that the approach where the neighbor cells are assumed to be “fully loaded” is used since it is a worst-case scenario and would overestimate the interference.

Proposal 5. If companies would like to model interference from neighbor cells, our preference is as follows.

- Processing gain is applied in the SINR computation. Recommended value for the processing gain is 10dB.

- Use an approach similar as Option 3 (interference comes from randomly chosen beams(s) of neighbor cells). Details are FFS.

System Level Simulation (SLS)

Proposal 6. If the UE predicts that the measurement event will be triggered in the future, the UE transmits a Measurement Event prediction report to the source gNB. The Measurement Event prediction report can contain the source cell and target cell measurement predictions for the prediction window duration of time and the source cell and target cell historical actual measurements.

Proposal 7. Based on the received Measurement Event prediction report, the source gNB may initiate handover preparation procedure for the target cell.

Proposal 8. If the source gNB receives subsequently an RRC Measurement Report from the UE:

- If the target cell indicated in the RRC Measurement Report is the same as that indicated in the Measurement Event prediction report, the source gNB waits for the ongoing handover preparation to complete, and then transmits a Handover Command message to the UE.

- If the target cell indicated in the RRC Measurement Report is different from that indicated in the Measurement Event prediction report, the source gNB initiates another handover preparation procedure for the indicated target cell and transmits a Handover Command to the UE upon completion of the procedure.

Proposal 9. A similar procedure as in the above proposal is followed if the UE transmits another Measurement Event prediction report following an initial report.

# References

[1] Chair notes, RAN2 #127-bis meeting.