3GPP TSG-RAN WG2 Meeting #128	R2-2410013

Orlando, USA, Nov 18th – 22nd, 2024

# Agenda item:	8.6.4

# Source:	ETRI

# Title:	Discussion on conditional intra-CU LTM

# Document for:	Discussion, Decision

- Introduction

In RAN#127-bis meeting, agreements on conditional LTM design have been made as the followings [1]:

Agreements on Conditional LTM - Source cell sends the conditional LTM configuration via RRCReconfiguration to UE, which includes the LTM candidate configurations, and the corresponding execution conditions. - Event LTM3-like and LTM5-like are used as the conditional LTM execution condition. FFS on reuse of CHO conditions. - Source cell and each candidate cell provides its own execution condition for conditional LTM. - It is DU to generate the L1 execution condition. FFS on a case that L3 measurement is used. - RACH-less Conditional intra-CU LTM is supported. - RACH based conditional intra-CU LTM is supported. - UE based TA measurement mechanism is supported for conditional intra-CU LTM. - PDCCH ordered early TA acquisition is supported for conditional LTM. - Rel-18 Early candidate TCI State activation/deactivation is supported for conditional intra-CU LTM. - For RACH-less conditional LTM, CG-based first UL transmission on target cell is supported. FFS on DG-based approach. - The LTM completion defined for Rel-18 intra-CU LTM is reused for conditional LTM

In this paper, we discuss open issues to be considered to support conditional intra-CU LTM.

- Discussion

  - Basic procedure for conditional LTM

General configuration

Conditional LTM is similar to network triggered LTM in that the network determines multiple candidate cells for LTM based on the UE’s L3 measurement reports, and a cell switch is determined based on measurements from both the serving cell and candidate cells. It is also same that no extra RRC signaling is required for subsequent cell switches among candidate cells configured by the network.

The key distinction between conditional LTM and network triggered LTM lies in which node initiates the cell switch to a target cell. In network triggered LTM, a serving cell selects the target cell based on L1 measurement reports from the UE. In contrast, in conditional LTM, the UE itself decides the target cell based on the measurement results and execution conditions configured by the source cell.

Since candidate cells for both conditional and network triggered LTMs are determined and configured based on the same L3 measurement reports from the UE, there is no reason for a network to configure candidate cells separately for each LTM type. Instead, the network can designate the same candidate cells—or a subset of those configured for the network triggered LTM—as candidate cells for the conditional LTM.

Observation 1: Since candidate cells for both conditional and network triggered LTMs are determined and configured based on the same L3 measurement results, there is no reason for a network to configure candidate cells separately for each LTM type.

In a scenario where a subset of network triggered LTM candidate cells is configured as conditional LTM candidate cells, the network triggered LTM is used as a primary LTM and the conditional LTM can be used as a complementary LTM to enhance the robustness of LTM cell switching. In most cases, the network determines a target cell based on L1 measurement report and requests the UE to perform cell switch to the target cell by transmitting CSC MAC CE. Only if a channel condition between the UE and the serving cell degrades rapidly, the UE performs a cell switch to a target cell of which execution conditions are fulfilled without an instruction from the serving cell.

Observation 2: If a subset of network triggered LTM candidate cells is configured as conditional LTM candidate cells, the network triggered LTM is used as a primary LTM and the conditional LTM can be used as a complementary LTM to enhance the robustness of LTM cell switching.

In the last RAN2 meeting, some companies expressed their concerns on the support of PDCCH ordered TA acquisition function in conditional LTM. They argued that PDCCH ordered TA acquisition could not be triggered timely because a UE does not report the measurement results in conditional LTM, and additional schemes such as new trigger events or UE requests would be required to support PDCCH ordered TA acquisition. Those arguments may make sense if separate candidate cells are configured for conditional LTM and network triggered LTM. However, if candidate cells for conditional LTM are configured as a subset of those for network triggered LTM, PDCCH ordered TA acquisition can be supported without specification changes because the UE reports measurement results of these conditional LTM candidate cells.

Additionally, if separate candidate cells are configured for conditional LTM and network triggered LTM, RAN2 would need to reassess the maximum number of LTM candidate cells to account for conditional LTM. If the maximum number of LTM candidate cells is increased, additional network resources may be required to support conditional LTM and network triggered LTM simultaneously. On the other hand, if candidate cells for conditional LTM are configured as a subset of those for network triggered LTM, both LTM types can be supported without the need for additional network resources.

Proposal 1: candidate cells for conditional LTM are a subset of candidate cells for network triggered LTM.

Conditional LTM preparation phase

In RAN2#127-bis, RAN2 has agreed to support PDCCH ordered early TA acquisition for conditional LTM, but how to deliver a TA value was not discussed. There may be two possible options.

- Option 1: Introduce a new MAC CE

- Option 2: Reuse the CSC MAC CE with a new parameter which indicates the purpose of the current CSC MAC CE (a cell switch command or TA delivery)

Proposal 2: RAN2 is kindly asked to discuss how to deliver a TA value for PDCCH ordered early TA acquisition.

In the network triggered LTM, the source cell manages the validity of the TA and requests the UE to repeat the early TA acquisition procedure if the TA becomes invalid. If the previous proposal is agreed, that is, candidate cells for the conditional LTM is configured as a subset of those configured for the network triggered LTM, the TA can be managed in the same way. The source cell triggers early TA acquisition procedure based on the UE’s measurement report, delivers UE the TA that received from a candidate cell, maintains the validity of the TA, and triggers the early TA acquisition procedure again if the TA becomes invalid and the UE is still connected to the source cell.

Proposal 3: the source cell maintains a TA value for PDCCH ordered early TA acquisition before conditional LTM execution.

The following figure illustrates the proposed stage-2 signaling procedure for the conditional intra-CU LTM. The agreements made in the last RAN2 meeting and the proposals included in this paper are captured in the figure.

Figure 1. Signaling procedure for conditional intra-CU LTM

Proposal 4: Signaling procedure in Figure 1 is take as the starting point for the discussion of conditional intra-CU LTM signaling procedure.

  - Mixture of conditional LTM and network triggered LTM

Conditional handover is introduced to reduce the possibility of RLF during handover and can be configured with a normal L3 handover. For example, a source cell can send a HO command to a UE after configuring the UE with candidate cell configurations for CHO. The UE will follow the L3 handover procedure if it receives HO command before any CHO execution condition is satisfied. However, if a CHO execution condition is satisfied before the HO command is received from the source cell, the UE will proceed with the CHO procedure.

Similar to the L3 conditional handover, conditional LTM can be configured to reduce the probability of RLF during cell switching when network triggered intra-CU LTM is active. Furthermore, if candidate cells for conditional LTM are configured as a subset of those for network triggered LTM, as described in section 2.1, a mixture of conditional LTM and network triggered intra-CU LTM can be supported with minimal effort.  In this approach, the current LTM-Config and LTM-Candidate IEs can be reused with minor modification to support the mixed configuration. The only modification is to include execution conditions in configurations of conditional LTM candidate cells, enabling the UE to identify candidate cells designated for conditional LTM.

Proposal 5: conditional LTM and intra-CU LTM can be configured simultaneously.

Proposal 6: A UE identifies an LTM candidate cell as a conditional LTM candidate cell if the LTM candidate configuration includes execution conditions for conditional LTM.

According to WID [2], subsequent LTM should be supported in conditional LTM. In practice, there is no significant difference between conditional LTM and network triggered intra-CU LTM. In both cases, multiple cells within the same CU are prepared for potential cell switching, enabling subsequent switches among these prepared candidate cells without the need for RRC messaging. Since the switch is restricted to intra-CU, a cell switch in one LTM type does not affect the candidate cell configurations of the other LTM type. Consequently, the UE does not need to release the conditional LTM cell configuration after an intra-CU LTM and should also retain the conditional LTM cell configuration following a conditional LTM for subsequent LTM, unless explicitly released by the network.

Proposal 7: The UE keeps its conditional LTM cell configuration after a conditional LTM or intra-CU LTM for subsequent conditional LTM unless there are explicitly released by a network.

In the RAN2#125-bis meeting, it was agreed that the UE can be configured with a mixture of intra-CU and inter-CU candidate cells. If the previous proposals for supporting a mixture of conditional LTM and intra-CU LTM are agreed, the possibility of supporting a mixture of conditional LTM and inter-CU LTM should also be considered.

This scenario differs somewhat from a mixture of conditional LTM and intra-CU LTM. If an inter-CU LTM cell switch occurs while both conditional LTM and inter-CU LTM are configured, the target cell in a different CU may become a new source cell. Since conditional LTM is limited to cells within the same CU, the current conditional LTM configurations become irrelevant if the CU of the conditional LTM cells differs from the CU of the new source cell.

However, the UE does not necessarily need to release its conditional LTM candidate cell configurations, as it may perform a cell switch back to the previous serving cell sometime. Instead, the UE simply stops evaluating the execution conditions for conditional LTM candidate cells associated with the previous serving CU and begins evaluating the execution conditions for other conditional LTM candidate cells belonging to the current serving CU.

In the RAN2#127 meeting, it was agreed to introduce a new Rel-19 ID in RRC for UE to decide whether to perform RLC and PDCP re-establishment when performing LTM cell switch. After a cell switch, the UE can use the same Rel-19 ID to decide whether to continue evaluating the execution conditions for conditional LTM. Specifically, the UE stops evaluating the execution conditions for a LTM candidate cell if the candidate cell’s Rel-19 ID differs from that of the current serving cell. Conversely, if the Rel-19 ID of an LTM candidate cell matches that of the current serving cell, the UE starts evaluating the execution conditions for that cell after the cell switch.

Proposal 8: a mixture of conditional LTM and network triggered inter-CU LTM is supported.

Proposal 9: The UE keeps its conditional LTM cell configuration after a conditional LTM or inter-CU LTM for subsequent conditional LTM unless there are explicitly released by a network.

Proposal 10: the UE evaluates the execution conditions only for the candidate cells whose Rel-19 ID matches that of the current serving cell.

- Conclusions

In this paper, we discuss conditional intra-CU LTM related issue. Following are the proposals made in this paper:

Observation 1: Since candidate cells for both conditional and network triggered LTMs are determined and configured based on the L3 measurement reports from the UE, the network can designate a subset of those configured for network triggered LTMs as candidate cells for conditional LTM type.

Observation 2: If a subset of network triggered LTM candidate cells is configured as conditional LTM candidate cells, the network triggered LTM is used as a primary LTM and the conditional LTM can be used as a complementary LTM to enhance the robustness of LTM cell switching.

Proposal 1: candidate cells for conditional LTM are a subset of candidate cells for network triggered LTM.

Proposal 2: RAN2 is kindly asked to discuss how to deliver a TA value for PDCCH ordered early TA acquisition.

Proposal 3: the source cell maintains a TA value for PDCCH ordered early TA acquisition before conditional LTM execution.

Proposal 4: Signaling procedure in Figure 1 is take as the starting point for the discussion of conditional intra-CU LTM signaling procedure.

Proposal 5: conditional LTM and intra-CU LTM can be configured simultaneously.

Proposal 6: A UE identifies an LTM candidate cell as a conditional LTM candidate cell if the LTM candidate configuration includes execution conditions for conditional LTM.

Proposal 7: The UE keeps its conditional LTM cell configuration after a conditional LTM or intra-CU LTM for subsequent conditional LTM unless there are explicitly released by a network.

Proposal 8: a mixture of conditional LTM and network triggered inter-CU LTM is supported.

Proposal 9: The UE keeps its conditional LTM cell configuration after a conditional LTM or inter-CU LTM for subsequent conditional LTM unless there are explicitly released by a network.

Proposal 10: the UE evaluates the execution conditions only for the candidate cells whose Rel-19 ID matches that of the current serving cell.

- References

- Chair notes RAN2#127-bis;

- RP-242356 Revised Work Item: NR mobility enhancements Phase 4; Apple, China Telecom;