3GPP TSG-RAN WG2 Meeting #128	R2-2410106

Orlando, USA, Nov. 18th – 22nd, 2024

Agenda item:	8.13.4

Source: 	China Telecom

Title:	Discussion on service continuity for multi-hop relay

WID/SID:	NR_SL_relay_enh2

Document for:	Discussion and Decision

- Introduction

In RAN#104, a new WI on NR sidelink multi-hop relay was approved in RP-241609 [1]. In Rel-17, single hop U2N relay was introduced to further extend the coverage of Uu and sidelink. Additional sidelink relay enhancements are needed to support some remaining (and new) requirements for Proximity Services captured in TS 22.278 and TS 22.261, e.g. multi-hop support for UE-to-Network Relay. Based on that, the following objective on service continuity for multi-hop relay will be specified in Rel-19 NR sidelink multi-hop relay WI.

Copied from WID in RP-241609 - Specify the following intra-gNB service continuity scenarios for multi-hop U2N relay based on Rel-17/18 procedures (for remote UE): First Priority: - Intra-gNB multi-hop indirect to direct path switching using existing framework - Intra-gNB multi-hop indirect to single-hop indirect path switching using existing framework Second Priority in order of importance: - Intra-gNB direct to multi-hop indirect path switching - Intra-gNB single-hop indirect to multi-hop indirect path switching The scenarios C and D are limited to path switching to a target indirect path consisting of the last relay UE in “direct” RRC Connected mode and all the other intermediate relay(s) in “indirect” RRC Connected mode to the same cell. NOTE: The current existing measurement framework and existing data forwarding mechanism should be reused.

In this contribution, we would like to share our opinions on general aspects of service continuity for multi-hop U2N relay.

- Discussion

Intra-gNB multi-hop indirect to direct path switching

In Rel-17/18 U2N relay, the following events are introduced to support service continuity.

- Event X1: Serving L2 U2N Relay UE becomes worse than absolute threshold1 AND NR Cell becomes better than another absolute threshold2;

- Event X2: Serving L2 U2N Relay UE becomes worse than absolute threshold;

- Event Y1: PCell becomes worse than absolute threshold1 AND candidate L2 U2N Relay UE becomes better than another absolute threshold2;

- Event Y2: Candidate L2 U2N Relay UE becomes better than absolute threshold;

- Event Z1: Serving L2 U2N Relay UE becomes worse than absolute threshold1 AND candidate L2 U2N Relay UE becomes better than another absolute threshold2

Regarding intra-gNB multi-hop indirect to direct path switching, it may happen when the serving U2N relay becomes worse and NR cell becomes better. In this case, Event X1 could be used. The remote UE shall consider the first relay as serving L2 U2N relay for evaluating Event X1. When the PC5 quality between remote UE and the first relay is worse than threshold1 and Uu quality between remote UE and a target NR Cell becomes better than threshold2, the remote UE may initial the measurement report procedure. Based on the measurement report, the gNB could select a NR Cell within the same gNB and send RRCReconfiguration message to the remote UE to trigger the path switching procedure.

Proposal 1: For intra-gNB multi-hop indirect to direct path switching, remote UE switched to an NR Cell within the same gNB is supported. The NR Cell could be same as or different from the previous serving cell of remote UE.

Proposal 2: Event X1 could be used for intra-gNB multi-hop indirect to direct path switching. The remote UE shall consider the first relay as serving L2 U2N relay for evaluating Event X1.

Intra-gNB multi-hop indirect to single-hop indirect path switching

For intra-gNB multi-hop indirect to single-hop indirect path switching, taking two-hop switching to single-hop as an example, there may be the following cases.

- Case 2-1: Remote-relay1-relay2-gNB(Cell1) TO remote-relay3-gNB(Cell1)

- Case 2-2: Remote-relay1-relay2-gNB(Cell1) TO remote-relay3-gNB(Cell2)

- Case 2-3: Remote-relay1-relay2-gNB(Cell1) TO remote-relay1-gNB (Cell1)

- Case 2-4: Remote-relay1-relay2-gNB(Cell1) TO remote-relay1-gNB (Cell2)

For Case 2-1 and 2-2, it may happen when the serving U2N relay becomes worse and candidate U2N relay becomes better. In this case, Event Z1 could be used. The remote UE shall consider the first relay as serving L2 U2N relay for evaluating Event Z1. When the PC5 quality between remote UE and the first relay is worse than threshold1 and PC5 quality between remote UE and a candidate relay becomes better than threshold2, the remote UE may initial the measurement report procedure. Based on the measurement report, the gNB may select a target U2N relay within the same gNB and send RRCReconfiguration message to the remote UE to trigger the path switching procedure.

Proposal 3: For intra-gNB multi-hop indirect to single-hop indirect switching, remote UE switched to a different relay within the same gNB is supported. The serving cell of the new relay could be the same as or different from the previous serving cell of the remote UE.

Proposal 4: Event Z1 could be used for intra-gNB multi-hop indirect to direct path switching. The remote UE shall consider the first relay as serving L2 U2N relay for evaluating Event Z1.

For Case 2-3 and 2-4, it may happen when PC5 quality between relay1 and relay2 becomes worse and Uu quality between relay1 and NR cell becomes better. If Event X1 is configured for relay1, relay1 may initial the measurement report procedure. Based on the measurement report, the gNB may switch relay1 onto direct Uu path. If the PC5 quality between remote UE and relay1 is stable, whether to support service continuity for remote UE in these scenarios shall be further discussed. Generally, it is more like a kind of “group HO”, which is not supported in previous release.

Proposal 5: RAN2 clarifies whether to support service continuity of Case 2-3 and 2-4 for intra-gNB multi-hop indirect to single-hop indirect switching.

Intra-gNB direct to multi-hop indirect path switching

Regarding intra-gNB direct to multi-hop indirect path switching, it may happen when PCell becomes worse and candidate U2N relay becomes better. In this case, Event Y1 could be used. When Uu quality between (remote) UE and PCell becomes worse than threshold1 and the PC5 quality between (remote) UE and candidate U2N relay is better than threshold2, the (remote) UE may initial the measurement report procedure. Based on the measurement report, the gNB may select a target relay within the same gNB and send RRCReconfiguration message to the (remote) UE to trigger the path switching procedure.

Proposal 6: For intra-gNB direct to multi-hop indirect path switching, (remote) UE switched to candidate U2N relay within the same gNB is supported. The target U2N relay could be same as or different from the previous serving cell of (remote) UE.

Proposal 7: Event Y1 could be used for intra-gNB direct to multi-hop indirect path switching. The PC5 quality between the remote UE and candidate U2N relay/candidate first relay is considered for evaluating Event Y1.

Intra-gNB single-hop indirect to multi-hop indirect path switching

For intra-gNB single-hop indirect to multi-hop indirect path switching, taking two-hop switching to single-hop as an example, there may be the following cases.

- Case 4-1: Remote-relay1-gNB(cell1) TO remote-relay1-relay2-gNB(cell1)

- Case 4-2: Remote-relay1-gNB(cell1) TO remote-relay1-relay2-gNB(cell2)

- Case 4-3: Remote-relay1-gNB(cell1) TO remote-relay3-relay4-gNB(cell1)

- Case 4-4: Remote-relay1-gNB(cell1) TO remote-relay3-relay4-gNB(cell2)

For Case 4-3 and 4-4, it may happen when the serving U2N relay becomes worse and candidate U2N relay becomes better. In this case, Event Z1 could be used. The remote UE shall consider the first relay as serving L2 U2N relay for evaluating Event Z1. When the PC5 quality between remote UE and the first relay is worse than threshold1 and PC5 quality between remote UE and a candidate relay becomes better than threshold2, the remote UE may initial the measurement report procedure. Based on the measurement report, the gNB may select a target U2N relay within the same gNB and send RRCReconfiguration message to the remote UE to trigger the path switching procedure.

Proposal 8: For Intra-gNB single-hop indirect to multi-hop indirect path switching, remote UE switched to a different relay within the same gNB is supported. The serving cell of the new relay could be the same as or different from the previous serving cell of the remote UE.

Proposal 9: Event Z1 could be used for intra-gNB multi-hop indirect to direct path switching. The remote UE shall consider the first relay as serving L2 U2N relay for evaluating Event Z1.

For Case 4-1 and 4-2, it may happen when Uu quality between relay1 and NR cell becomes worse and PC5 quality between relay1 and candidate relay becomes better. If Event Y1 is configured for relay1, relay1 may initial the measurement report procedure. Based on the measurement report, the gNB may switch relay1 onto indirect path via relay2. If the PC5 quality between remote UE and relay1 is stable, whether to support service continuity for remote UE in these scenarios shall be further discussed. Generally, it is more like a kind of “group HO”, which is not supported in previous release.

Proposal 10: RAN2 clarifies whether to support service continuity of Case 4-1 and 4-2 for intra-gNB single-hop indirect to multi-hop indirect path switching.

- Conclusion

In this contribution, we discuss general aspects on service continuity for multi-hop U2N relay. We kindly ask RAN2 to consider the corresponding proposals listed as below.

Proposal 1: For intra-gNB multi-hop indirect to direct path switching, remote UE switched to an NR Cell within the same gNB is supported. The NR Cell could be same as or different from the previous serving cell of remote UE.

Proposal 2: Event X1 could be used for intra-gNB multi-hop indirect to direct path switching. The remote UE shall consider the first relay as serving L2 U2N relay for evaluating Event X1.

Proposal 3: For intra-gNB multi-hop indirect to single-hop indirect switching, remote UE switched to a different relay within the same gNB is supported. The serving cell of the new relay could be the same as or different from the previous serving cell of the remote UE.

Proposal 4: Event Z1 could be used for intra-gNB multi-hop indirect to direct path switching. The remote UE shall consider the first relay as serving L2 U2N relay for evaluating Event Z1.

Proposal 5: RAN2 clarifies whether to support service continuity of Case 2-3 and 2-4 for intra-gNB multi-hop indirect to single-hop indirect switching.

Proposal 6: For intra-gNB direct to multi-hop indirect path switching, (remote) UE switched to candidate U2N relay within the same gNB is supported. The target U2N relay could be same as or different from the previous serving cell of (remote) UE.

Proposal 7: Event Y1 could be used for intra-gNB direct to multi-hop indirect path switching. The PC5 quality between the remote UE and candidate U2N relay/candidate first relay is considered for evaluating Event Y1.

Proposal 8: For Intra-gNB single-hop indirect to multi-hop indirect path switching, remote UE switched to a different relay within the same gNB is supported. The serving cell of the new relay could be the same as or different from the previous serving cell of the remote UE.

Proposal 9: Event Z1 could be used for intra-gNB multi-hop indirect to direct path switching. The remote UE shall consider the first relay as serving L2 U2N relay for evaluating Event Z1.

Proposal 10: RAN2 clarifies whether to support service continuity of Case 4-1 and 4-2 for intra-gNB single-hop indirect to multi-hop indirect path switching.

- Reference

RP-241609, New WID on NR sidelink multi-hop relay, LG Electronics, InterDigital, FirstNet

3GPP TS 38.300 18.2.0: "NR; Radio Resource Control (RRC) protocol specification".