3GPP TSG-RAN WG2 Meeting #128	R2-2410180

Orlando, USA, Nov 18th – 22nd, 2024

Agenda Item:	8.6.3

Source: 	ASUSTeK

Title:  	Discussion on L1 measurement reporting for LTM

Document for:	Discussion and Decision

- Introduction

In the previous meeting RAN2#127bis, L1 event triggered report for LTM was discussed and several topics were postponed, including detailed designed of TTT and contents of MR MAC CE:

7. TTT operation per beam or per cell (for a candidate cell)? - Option1: Per beam (P1 in R2-2408260, MediaTek) - Option2: Per cell (P2 in R2-2408525, ZTE) 8. TTT operation on current beam changing - Option1: TTT is not reset (P1 in R2-2408613, NEC) - Option2: TTT is reset (P4 in R2-2408948, Nokia) We will make decision next meeting. … 12. Contents of MR MAC CE 12.2 Additional information … B. Multiple serving beams (up to N) (P13 in R2-2408759, Ericsson) Proposal 13: The network can configure the UE to report the serving cell in the event-triggered L1 measurement report. Noted. [Samsung]: Do not see a need. In event triggered MR, the UE doesn’t distinguish separate beams from the serving cell. UE just measure current beam whatever is. It’s different than Rel-18 LTM. [Apple]: Agree with Samsung. [Huawei]: Assume there is no real difference between Rel-18 LTM and Rel-19 LTM. [OPPO]: Agree with Samsung, Apple. [Xiaomi]: Feel sympathy on the proposal. It could be helpful if supported. [LG]: Assume no additional efforts to support it. [Apple]: For the serving cell, we already have beam management. If needed, we can count on it. [Nokia]: Agree with Samsung and Apple. [Qualcomm]: One thing clear is that we shouldn’t do additional measurements on the serving beams for event triggered MR. [MediaTek]: If multiple serving beams are included in LTM resource configuration, then anyway the UE will perform measurement on each serving beam. [Ericsson]: Proposal is made based on the assumption NW configures serving cell into a candidate configuration also. C. Candidate beams (P12 in R2-2407988, CATT) (P6-7 in R2-2408499, OPPO) - Candidate beams (i.e., beams associated to the same event configuration) which have fulfilled the event condition are included in the MAC CE measurement report according to limitation on the max number of candidate beams configured by NW. - Other candidate beams (even though they haven’t fulfilled the event condition) can be included according to limitation on the max number of candidate beams configured by NW. [Session chair]: It was already included as FFS in the previous discussion, so let’s skip it.

In this contribution, we continue to discuss details for L1 measurement report for LTM.

- Discussion

TTT operation for event-triggered report

During the previous meeting, TTT operation regarding beam change was discussed. In our view, the intention for a network to provide a Serving Cell beam change is mostly to provide/activate a beam with better quality for the UE. It would be redundant and could cause extra reporting overhead if the UE continues the TTT evaluation after the Serving Cell beam changes. For example, as depicted in Fig.1, the UE will trigger unnecessary beam report due to insufficient evaluation time of the new current beam, e.g. the quality of the new current beam meet the event condition for a short period of time even though the quality of the beam is becoming better afterwards. In order to avoid unwanted triggering of beam report, it is proposed that in response to a current beam change of a Serving Cell, corresponding TTT of triggering events with evaluation related to Serving Cell quality (i.e., LTM2, LTM3, and LTM5) should be reset.

Fig. 1

Proposal 1:	In response to a current beam change of a Serving Cell, TTT associated with trigger event LTM2, LTM3, and LTM5 should be is reset.

Content of MR MAC CE for LTM

In the previous meeting, it was agreed that beam information, beam quality of N beams, where N is configured by NW, and triggered event information are included in the MAC CE as baseline. Since MR for LTM is transmitted via MAC CE, one issue could occur is when an available UL grant cannot accommodate all information generated by the UE (e.g., cannot accommodate beam information of all N beams). Therefore, we propose to support truncated format of MR MAC CE for LTM. How to determine the order to include different beam information into truncated MAC CE can be further discussed.

Proposal 2:	Support truncated format of MR MAC CE for LTM. Detail FFS.

- Conclusion

We have the following proposals for LTM-related RA reports:

Proposal 1:	In response to a current beam change of a Serving Cell, TTT associated with trigger event LTM2, LTM3, and LTM5 should be is reset.

Proposal 2:	Support truncated format of MR MAC CE for LTM. Detail FFS.

- Reference