3GPP TSG-RAN WG2 Meeting #128	R2-2410183

Orlando, USA, Nov.  18th – 22nd, 2024

Agenda Item:	8.13.2

Source:	ASUSTeK

Title:	Remaining issues on multi-hop U2N Relay Discovery message forwarding

Document for:	Discussion and Decision

1	Introduction

RAN2#127 made the following agreements on multi-hop U2N Relay Discovery:

Agreements:

A candidate last relay UE for discovery or (re)selection can be in any RRC state, but not outside network coverage.  FFS if acting as a relay requires it to transition to RRC_CONNECTED.

Some or all of the intermediate relay UEs on a multi-hop indirect path can be inside or outside the network coverage.

[POST127][401][Relay] Multi-hop relay discovery and (re)selection [1] further raises the following proposals on the Intermediate Relay UE:

Proposal 1a: A lower bound is not required for the intermediate Relay UE.

Proposal 2: RAN2 understands that the intermediate Relay UE can initiate the announcement message when receiving announcement message or, at least, when the intermediate Relay UE has the information for the announcement message provided by the last Relay UE. The detail can be discussed further.

Proposal 3: The discovery announcement message can be forwarded at the intermediate Relay UE when the SD-RSRP or SL-RSRP between the last/parent intermediate Relay UE and itself is above a configured threshold.

Proposal 4: The following model B discovery forwarding condition can be baseline for further discussion:

- For Model B, the intermediate Relay UE forwards the solicitation message only if the PC5 RSRP between the Remote UE (or intermediate Relay UE) and the intermediate Relay UE is above a threshold.

…

[POST127][402][Relay] Multi-Hop Relay Control Plane [2] discusses the following two approaches for multi-path U2N Relay:

- Approach 1: The network needs to directly control each of the intermediate relay UEs via Uu RRC.

- Approach 2: Only the last relay UE requires control by the network via Uu RRC.

In addition, [POST127][402][Relay] Multi-Hop Relay Control Plane [2] raises the following proposal for multi-path U2N Relay:

Proposal 7 – The scenario of the remote UE RRC_CONNECTED to one cell while an Intermediate Relay UE is RRC_CONNECTED to a different cell is supported only in approach 2.  FFS whether the scenario needs to be supported.

This document further discusses the following two cases of Relay Discovery message forwarding by a UE being authorized to act as an Intermediate Relay UE in Approach 1:

    - Whether an out-of-coverage UE being authorized to act as an Intermediate Relay UE needs to forward a Relay Discovery Solicitation message received from other UE.

    - Whether a UE being authorized to act as an Intermediate Relay UE needs to forward a Relay Discovery message received from other UE if the UE has a direct RRC connection with a serving cell.

2	Discussion

2.1	Whether an out-of-coverage UE being authorized to act as an Intermediate Relay UE needs to forward a Relay Discovery Solicitation message received from other UE

RAN2#127 agreed that some of the intermediate relay UEs on a multi-hop indirect path can be outside the network coverage. And, the email discussion on multi-hop relay discovery [1] further proposes that the intermediate Relay UE may forward the solicitation message if the PC5 RSRP between the Remote UE (or intermediate Relay UE) and the intermediate Relay UE is above a threshold.

In our understanding, after a UE is powered on, the UE would try continuously or periodically to connect to the network so as to initiate a registration procedure and then listen to paging from the network. If the UE cannot connect to the network directly, it would try to discover a U2N Relay UE so as to connect to the network indirectly via the discovered U2N Relay UE(s), assuming the UE is capable of U2N Relay operation.

Observation 1: After a UE is powered on, the UE would try continuously or periodically to connect to the network directly or indirectly (if the UE is capable of U2N Relay operation).

However, it is possible that the UE may not be able to connect to the network directly or indirectly (e.g. UE6 and UE7 in Figure 1) and may stay out of coverage (OOC) for some time, after power-on. In this situation, the UE is not capable of acting as an Intermediate U2N Relay UE because no neighboring Remote UE can connect to the network via this UE.

Cell

Lower bound

Higher bound

UE2

(Last Relay)

UE1

(Last Relay)

UE3

(Intermediate

Relay)

Cell coverage

Path – A

Path – B

Path – B

Path – A

Path – A

Path – B

UE5 5

(Remote)

UE6

UE7

Cell

Lower bound

Higher bound

UE2

(Last Relay)

UE1

(Last Relay)

UE3

(Intermediate

Relay)

Cell coverage

Path – A

Path – B

Path – B

Path – A

Path – A

Path – B

UE5 5

(Remote)

UE6

UE7

Figure 1. Potential relay paths with multiple hops

Besides, it just wastes the device battery power unnecessarily for the UE to forward the received Relay Discovery Solicitation message because no other relay UE in proximity of this UE could respond to this solicitation. Thus, in our opinion, there is no need for the UE (UE6/UE7 in Figure 1) to forward the Relay Discovery Solicitation message received from the other remote UE (UE7/UE6 in Figure 1).

Observation 2: It just wastes the device battery power unnecessarily for an out-of-coverage UE to forward a Relay Discovery Solicitation message received from other remote UE.

Proposal 1:  In Approach 1, a UE being authorized to act as an Intermediate U2N Relay UE does not forward a Relay Discovery Solicitation message received from other remote UE if the UE is out of coverage (OOC).

2.2	Whether a UE being authorized to act as an Intermediate Relay UE needs to forward a Relay Discovery message received from other UE if the UE has a direct RRC connection with a serving cell

The email discussion on multi-hop control plane [2] proposes the scenario of the remote UE RRC_CONNECTED to one cell while an Intermediate Relay UE is RRC_CONNECTED to a different cell is supported only in approach 2. In other words, the remote UE and the Intermediate Relay UE should connect to the same serving cell in Approach 1.

Observation 3: In Approach 1, the remote UE and the Intermediate Relay UE should connect to the same serving cell.

In view of the above observation, there is no need for a UE being authorized to act as an Intermediate U2N Relay UE to forward a Relay Discovery message (either a Relay Discovery Solicitation message or a Relay Discovery Announcement message) received from other UE if the UE is connecting to a serving cell directly. Otherwise, it may result in a situation where the remote UE and the Intermediate Relay UE may connect to different serving cells. We think a UE qualified for acting as a Last U2N Relay UE would just reply a Relay Discovery Response message (in respond to reception of a Relay Discovery Solicitation message) or transmit its own Relay Discovery Announcement message instead of forwarding the received Relay Discovery Solicitation message or the received Relay Discovery Announcement message.

Observation 4: If a UE, being authorized to act as an Intermediate U2N Relay UE and connecting to a serving cell directly, is allowed to forward a Relay Discovery message (either a Relay Discovery Solicitation message or a Relay Discovery Announcement message) received from other UE, it may result in a situation where the remote UE and the Intermediate Relay UE may connect to different serving cells.

Proposal 2:  In Approach 1, a UE being authorized to act as an Intermediate U2N Relay UE does not forward a Relay Discovery message (either a Relay Discovery Solicitation message or a Relay Discovery Announcement message) received from other UE if the UE is connecting to a serving cell directly.

3 	Conclusion

In this contribution, we raise the following observations and proposals:

Observation 1:  After a UE is powered on, the UE would try continuously or periodically to connect to the network directly or indirectly (if the UE is capable of U2N Relay operation).

Observation 2: It just wastes the device battery power unnecessarily for an out-of-coverage UE to forward a Relay Discovery Solicitation message received from other remote UE.

Observation 3: In Approach 1, the remote UE and the Intermediate Relay UE should connect to the same serving cell.

Observation 4: If a UE, being authorized to act as an Intermediate U2N Relay UE and connecting to a serving cell directly, is allowed to forward a Relay Discovery message (either a Relay Discovery Solicitation message or a Relay Discovery Announcement message) received from other UE, it may result in a situation where the remote UE and the Intermediate Relay UE may connect to different serving cells.

Proposal 1:  In Approach 1, a UE being authorized to act as an Intermediate U2N Relay UE does not forward a Relay Discovery Solicitation message received from other remote UE if the UE is out of coverage (OOC).

Proposal 2:  In Approach 1, a UE being authorized to act as an Intermediate U2N Relay UE does not forward a Relay Discovery message (either a Relay Discovery Solicitation message or a Relay Discovery Announcement message) received from other UE if the UE is connecting to a serving cell directly.

- References

[1] R2-240xxxx, Report of [POST127][401][Relay] Multi-hop relay discovery and (re)selection

[2] R2-240xxxx, Report of [POST127][402][Relay] Multi-Hop Relay Control Plane