3GPP TSG-RAN2 Meeting # 128	R2-2410218

# Orlando, USA,  18th – 22nd November, 2024

Agenda item:	7.1.3

Source: 	Huawei, HiSilicon

Title: 	Discussion on the issues in GAD in SLPP

Document for:	Discussion and Decision

1. Introduction

With the SLPP message ProvideLocationInformation, the UE is allowed to provide location estimate, range and/or direction, velocity estimate, and relative location estimate. The ASN1 encoding has been provided for these location results. In this paper, we discuss on the potential issues with these fields and their descriptions.

2. Discussion

During the previous RAN2 discussion, the following CR has been agreed regarding the GAD descriptions within the SLPP spec

R2-2407146	Correction to SLPP PDU Common Contents	Qualcomm Incorporated	draftCR	Rel-18	38.355	18.2.0	F	NR_pos_enh2-Core

- To be merged into rapporteur CR (subject to offline review)

- Email discussion to confirm acceptability of NBC changes

- Merged into R2-2407719

Within the CR, various issues have been clarified for which the discussion, according to our opinion, should happen in SA2/CT1 but RAN2 has already agreed.

Within the QoS definition in requestLocationInformation, for horizontal/vertical/range accuracy, to the best of our knowledge, there is no prior discussion on this that which types of uncertainty should be used. For ranging accuracy, according to the CR rapp, it is considered that high accuracy extended uncertainty is more suitable for ranging services. For this we think we should inform SA2.

2.1	Alignment with other WGs

For the accuracy indication for azimuth and elevation, the issue with the current SA2 spec is that there is no definition for the coding for azimuth/elevation, or more generally, even for any angle. Since we have already agreed on them in our spec, we should tell CT4/CT1 our agreement.  And ask them if they find any issue with the current agreement.

Proposal1: Send an LS to SA2/CT1/CT4 on the agreement for rangeAndDirection, and ask if there are any issues with them:

- Adopt high accuracy extended uncertainty as defined in clause 6.2b in TS 23032 for horizontal/vertical/range accuracy

- Adopt scale factor of accuracy is 1 degree; range 0 to 127 degrees for azimuth accuracy (angle uncertainty)

- Adopt scale factor of accuracy is 1 degree; range 0 to 63 degrees for elevation accuracy (angle uncertainty)

Then, with the shape definitions in provideLocationInformation, first, the coding for relative velocity with uncertainty has not been defined yet, while, within the SLPP spec, the exact coding for these shapes have already been agreed and defined. We propose the following

Proposal2: Send an LS to SA2/CT1/CT4 on the agreement for relative velocity with uncertainty, and ask if there are any issues with them:

Radical velocity in the units of meters per second or centi-meters per second and transverse velocity in the units of degree per second or per 0.1 degree per second

Radical velocity in the range of (-2049..2047) with negative sign indicating the opposite direction between point A and B; elevation and azimuth rate of change in the range of (0..1023)

Uncertainty for Azimuth and Elevation Rate Of Change in the units of transverse velocity and range in (0..255)

2.2	Potential issues with GAD in SLPP

For horizontal/vertical accuracy, they are already supported in the LPP spec for absolute positioning while for LPP, normal accuracy in clause in section 6.2 and high accuracy uncertainty in section 6.2a are supported for request Location Information. We think for absolute positioning, LPP and SLPP should be aligned.

In addition, currently, uncertainty has been defined for relative velocity, but within the request location information, there is no uncertainty indication within, but only a single yes/no request. The uncertainty should be added to the request location information for relative velocity.

Hence, we propose the following

Proposal3a: Support normal uncertainty in clause 6.2 and High Accuracy Uncertainty in Section 6.2a in TS 23.032 for the horizontal/vertical accuracy in the QoS within requestLocationInformation.

Proposal3b: Support uncertainty indication for the velocity and relative velocity in the QoS within requestLocationInformation.

With the above discussion, we can see that the RAN2 and SA2 spec 23032 are not completely aligned. For the legacy LPP spec and the current locationEstimate in the SLPP spec, we simply give the following description with a reference to the SA2 spec 23032 added. While for the current SLPP spec, descriptions for the GADs including the rangeAndOrDirection, relativeLocationEstimate, and relativeVelocityWithUncertainty, are given instead of just referencing the SA2 spec.

With the above discussion, we propose the following to be discussed in RAN2

Proposal4: Down-select from the following WFs from RAN2’s perspective:

- Option1: Remove the coding format description for rangeAndOrDirection, relativeLocationEstimate, and relativeVelocityWithUncertainty while only cite the SA2 spec TS 23.032

- Option2: Keep the current description in RAN2 while correct the issues for the description of uncertainty.

3. Conclusion

In this paper, we discuss the current issues within the SLPP spec for the description of GAD. We propose the following:

Alignment with other WGs

Proposal1: Send an LS to SA2/CT1/CT4 on the agreement for rangeAndDirection within provideLocationInformation and QoS within requestLocationInformation, and ask if there are any issues with them:

- Adopt high accuracy extended uncertainty as defined in clause 6.2b in TS 23032 for horizontal/vertical/range accuracy

- Adopt scale factor of accuracy is 1 degree; range 0 to 127 degrees for azimuth accuracy (angle uncertainty)

- Adopt scale factor of accuracy is 1 degree; range 0 to 63 degrees for elevation accuracy (angle uncertainty)

Proposal2: Send an LS to SA2/CT1/CT4 on the agreement for relative velocity with uncertainty, and ask if there are any issues with them:

Radical velocity in the units of meters per second or centi-meters per second and transverse velocity in the units of degree per second or per 0.1 degree per second

Radical velocity in the range of (-2049..2047) with negative sign indicating the opposite direction between point A and B; elevation and azimuth rate of change in the range of (0..1023)

Uncertainty for Azimuth and Elevation Rate Of Change in the units of transverse velocity and range in (0..255)

Potential issues with GAD in SLPP

Proposal3a: Support normal uncertainty in clause 6.2 and High Accuracy Uncertainty in Section 6.2a in TS 23.032 for the horizontal/vertical accuracy in the QoS within requestLocationInformation.

Proposal3b: Support uncertainty indication for the velocity and relative velocity in the QoS within requestLocationInformation.

Proposal4: Down-select from the following WFs from RAN2’s perspective:

- Option1: Remove the coding format description for rangeAndOrDirection, relativeLocationEstimate, and relativeVelocityWithUncertainty while only cite the SA2 spec TS 23.032

- Option2: Keep the current description in RAN2.