3GPP TSG-RAN WG2 Meeting #128					           R2-2410540

Orlando, USA, November 18th – 22th, 2024

Agenda Item:		8.3.5

Source:			Huawei, HiSilicon

Title:			Discussion on simulations assumption and generalization

Document for:		Discussion and Decision

# 1   Introduction

In this contribution, we will further discussion the simulations for measurement event prediction.

# 2   Discussion

## 2.1 Simulation assumption

In post email discussion [POST127bis][022][AI mobility], companies have disagreements about the HO model. The following has been proposed in the e-mail discussion:

Figure 2.3.1-2 Example timeline for FR2 temporal domain case A based on indirect prediction (…) Figure 2.3.1-3 Example timeline for FR2 temporal domain case A based on direct prediction (…) The HO model for FR2 temporal domain case A is defined as following: If a predicted A3 event at future t1 is reported at t0 and one potential real A3 event is triggered at t2, then HO command is transmitted at t3, where t3=t0+ max(HO prep time, min(t1-t0,t2-t0)). After that one fixed execution time is assumed. For direct prediction, t1 is the middle of the event occurrence window. The HO model for FR1 temporal domain case B is defined as following: If a predicted A3 event at t1 is reported at t0 (t0<=t1) then HO command is transmitted at t3, where t3=t0+max(HO prep time, t1-t0). After that one fixed execution time is assumed.

In our understanding, there are couple of issues with the proposed HO modelling:

- RAN2 made the following agreement in the previous meeting:

A3 event prediction should follow legacy rules (i.e. the “predicted” conditions have to persist for the duration of TTT)

It is unclear where the TTT is in the figures provided by the rapporteur, but we assume an event is assumed to be met when TTT expires, as per the agreement. TTT is started when event’s entering conditions are met and the purpose of TTT is to make sure that an event is stable and it helps to avoid hasty handover decisions. However, in case of predicted events, it makes no sense to wait with the HO command till TTT expires. If the prediction indicates that the event is stable and it will be met for the duration of TTT, then the measurement report should be sent to the network without waiting for TTT to truly expire. This in consequence allows to send the HO command to the UE earlier. It should be noted that this will not lead to too early handovers as the event is still required to be met for the duration of TTT, but based on prediction, not actual TTT expiry.

- We do not think we need to consider “real event triggering” in the HO determination for the case where event predictions are used. It is an optimization while it seems unlikely this will happen for our simulated cases, so for simplicity we can just agree to rely on predicted measurement event and ignore the real one in the simulations.

Based on the above, we think there are tow main advantages of using predicted events for the HO determination:

- HO preparation time can be started earlier, i.e. the network can initiate HO preparation procedure at the time when a predicted event is reported from UE.

- Measurement report can be sent earlier, i.e. already at the time when its entering conditions are met (according to actual/real measurements) and the prediction says it will be met for the duration of TTT.

Observation 1: there are two main advantages of using predicted events for the HO determination:

- HO preparation time can be started earlier, i.e. the network can initiate HO preparation procedure at the time when a predicted event is reported from UE.

- Measurement report can be sent earlier, i.e. already at the time when its entering conditions are met (according to actual/real measurements) and the prediction says it will be met for the duration of TTT

Based on the above points, we propose the following HO modelling. For non-AI case, HO preparation starts when A3 is met for TTT duration and HO command is sent when HO preparation expires. For AI case, HO preparation starts when an event is predicted to happen (i.e., t0), and HO command is sent when A3 entering conditions are met based on actual measurement, i.e., HO command is sent when HO preparation time expires if t0+HO preparation>t1-TTT and HO command is sent at t1-TTT if t0+HO preparation<t1-TTT.

Proposal 1: For AI mobility, HO preparation starts when an event is predicted to happen (i.e., t0), and HO command is sent when A3 entering conditions are met based on actual/real measurement and an event is predicted to be met for the duration of TTT. Using the timing from the figures: HO command is sent at t3, where t3=t0+max(HO prep time, t1-t0-TTT), provided that entering conditions of the event are met based on real/actual measurement at t3.

## 2.2 Model generalization

In TR 38.744 [1], there is an editor note that AI/ML model generalization could be studied after sufficient evaluation of different use cases.

Editor Note 1: This section intends to capture evaluation metrics, methodology and simulation assumptions common for all use cases Editor Note 2: AI/ML model generalization could be studied after sufficient evaluation of different use cases. Editor Note 3: No explicit result calibration (e.g., as in TR 36.839) is expected. Evaluation KPI(s) with detailed evaluation descriptions can be reported for cross-checking purposes.

Until RAN2#127bis meeting, RAN already made some progress on the evaluation of RRM measurement prediction. Furthermore, the model generalization is very important for the deployment of the feature of AI/ML aided mobility.

Therefore, it is suggested to start to discuss on model generalization. For example, if the model is not well generalized, a complex LCM procedure may be needed, and it may cost a lot of resources to train and store the models.

Observation 2: Model generalization has huge impacts on feature deployment and LCM.

The first thing we need to discuss is what use cases should be studied for model generalization. For now, we have RRM measurement prediction, measurement event prediction and RLF prediction. In our understanding, the RRM measurement prediction is the most basic use case, as the output of RRM measurement prediction can be used to further predict measurement event and RLF. To limit the extra simulation workload, it is suggested that RAN2 focuses on the study of model generalization for RRM measurement prediction.

Observation 3: Compared to measurement event prediction and RLF prediction, RRM measurement prediction is the most basic use case, whereas the output of RRM measurement prediction can be used to further predict measurement event and RLF.

Proposal 2: It is suggested that RAN2 focuses on the study of model generalization for RRM measurement prediction.

In RAN2#126 meeting [2], the priority of each use case of RRM measurement prediction was agreed as follows:

| Case number | Prioritization | Evaluation scenario combination | target study goal | Methodology |
| --- | --- | --- | --- | --- |
| 1 | Low | FR1 to FR1 intra-frequency temporal domain case A | 2nd goal | TBD |
| 2 | High | FR1 to FR1 intra-frequency temporal domain case B | 1st goal | Intra-cell |
| 3 | High | FR1 to FR1 inter-frequency (frequency domain) | 1st goal | Inter-cell |
| 4 | High | FR2 to FR2 intra-frequency temporal domain case A | 2nd goal | Intra-cell |
| 5 | Low | FR2 to FR2 intra-frequency temporal domain case B | 1st goal | TBD |
| 6 | Middle | FR2 to FR2 intra-frequency spatial domain | 1st goal | Intra-cell |

Similarly, in order to minimize the extra workload needed for model generalization simulations, we think we should select only a part of use cases. In our view, use case#2 and use case#4 can be the representative cases, whereas the purposes are for measurement overhead reduction and HO performance improvement respectively.

Proposal 3: To limit the extra simulation workload, for model generalization, we suggest RAN2 to study the following use cases as high priority:

- study use case#2 for 1st goal

- study use case#4 for 2nd goal

Regarding how to perform the model generalization, in our understanding, considering the actual deployment, the first issue that needs to be generalized is whether the model can be used in cells with different configurations. If the model is cell specific, this would mean it is not practical to deploy the AI/ML aided mobility feature as it would require a complex LCM to support the deployment. One the other hand, if it is feasible to train a generalized model for many cells with good performance, it would be easier to deploy and manage such feature. Among different ways of verifying how well the AIML model can be generalized for different cell configurations, the following can be considered to be checked:

- how well the model trained in one frequency (e.g. 2GHz) performs in another one (e.g. 4GHz)

- how well the model trained in a certain deployment scenario performs on another one, e.g. UMa, UMi deployments, different ISDs

- how well a model trained in a certain cell configuration perform in another one, e.g. gNB/UE antenna heights, different UE/gNB port settings, different number of beams etc.

- how well the same model works for different prediction window lengths

All these aspects seem to be interesting, but RAN2 will have to most likely select several aspects as highest priority in order for the exercise to be doable within the SI timeline.

Proposal 4: To verify how well the AIML model can be generalized, RAN2 should check the following (a prioritization discussion is expected):

- how well the model trained in one frequency (e.g. 2GHz) performs in another one (e.g. 4GHz)

- how well the model trained in a certain deployment scenario performs on another one, e.g. UMa, UMi deployments, different ISDs

- how well a model trained in a certain cell configuration perform in another one, e.g. gNB/UE antenna heights, different UE/gNB port settings, different number of beams etc.

- how well the same model works for different prediction window lengths

Lastly, the UE speed is also very important for model generalization, as the UE may move with different speeds in real life scenarios. Therefore, it is suggested that the UE speed should also be considered in model generalization, i.e., whether the model trained with one UE speed can be applied for inference for the UE with a different speed. This can be achieved with two methods:

- Train model for one specific UE speed (e.g. 30 km/h) and apply it to UE’s moving with another speed (e.g. 60 km/h)

- Train model using data from the UEs with different speeds (e.g. all of 30, 60, 90, 120 km/h) and check the accuracy for specific UE speeds (i.e. separately for 30, 60, 90, 120 km/h).

Proposal 5: RAN2 should study whether the model can be generalized for different UE speeds, e.g using the following methods:

- Train model for one specific UE speed (e.g. 30 km/h) and apply it to UE’s moving with another speed (e.g. 60 km/h)

- Train model using data from the UEs with different speeds (e.g. all of 30, 60, 90, 120 km/h) and check the accuracy for specific UE speeds (i.e. separately for 30, 60, 90, 120 km/h).

# 3   Conclusions

In this contribution, we discussed simulation assumption and model generalization, and provide following observations and proposals:

Handover modelling for SLS simulations

Observation 1: there are two main advantages of using predicted events for the HO determination:

- HO preparation time can be started earlier, i.e. the network can initiate HO preparation procedure at the time when a predicted event is reported from UE.

- Measurement report can be sent earlier, i.e. already at the time when its entering conditions are met (according to actual/real measurements) and the prediction says it will be met for the duration of TTT

Proposal 1: For AI mobility, HO preparation starts when an event is predicted to happen (i.e., t0), and HO command is sent when A3 entering conditions are met based on actual/real measurement and an event is predicted to be met for the duration of TTT. Using the timing from the figures: HO command is sent at t3, where t3=t0+max(HO prep time, t1-t0-TTT), provided that entering conditions of the event are met based on real/actual measurement at t3.

Model generalization

Observation 2: Model generalization has huge impacts on feature deployment and LCM.

Observation 3: Compared to measurement event prediction and RLF prediction, RRM measurement prediction is the most basic use case, whereas the output of RRM measurement prediction can be used to further predict measurement event and RLF.

Proposal 2: It is suggested that RAN2 focuses on the study of model generalization for RRM measurement prediction.

Proposal 3: To limit the extra simulation workload, for model generalization, we suggest RAN2 to study the following use cases as high priority:

- study use case#2 for 1st goal

- study use case#4 for 2nd goal

Proposal 4: To verify how well the AIML model can be generalized, RAN2 should check the following (a prioritization discussion is expected):

- how well the model trained in one frequency (e.g. 2GHz) performs in another one (e.g. 4GHz)

- how well the model trained in a certain deployment scenario performs on another one, e.g. UMa, UMi deployments, different ISDs

- how well a model trained in a certain cell configuration perform in another one, e.g. gNB/UE antenna heights, different UE/gNB port settings, different number of beams etc.

- how well the same model works for different prediction window lengths

Proposal 5: RAN2 should study whether the model can be generalized for different UE speeds, e.g using the following methods:

- Train model for one specific UE speed (e.g. 30 km/h) and apply it to UE’s moving with another speed (e.g. 60 km/h)

- Train model using data from the UEs with different speeds (e.g. all of 30, 60, 90, 120 km/h) and check the accuracy for specific UE speeds (i.e. separately for 30, 60, 90, 120 km/h).

# References

- TR 38.744 v0.0.4

- Draft Report of 3GPP TSG RAN WG2 meeting #126, Fukuoka, Japan, 20 - 24 May, 2024