3GPP TSG-RAN WG2 Meeting #128	R2-2410573

Orlando, USA, Nov 18th – 22nd, 2024

Agenda item:		8.2.2

Source:		Futurewei

Title:		Further discussions on AS ID for Ambient IoT devices

Document for:		Discussion and Decision

# Introduction

In meeting #127-bis, RAN2 reached the following agreements regarding the AS ID [1]:

Agreements on AS ID RAN2 assumes that if “AS ID” is defined it is used at least for purpose of D2R scheduling and R2D reception. Up to RAN1 to decide whether a “AS ID” is defined. RAN2 assumes this “AS ID” should be a short AS layer ID, rather than the full upper layer device ID. FFS on the length. FFS if AS ID can be based on partial upper layer device ID. From RAN2 perspective, following options are possible for “AS ID”: - Option 1: a random ID if used in Msg1 can be reused; - Option 2: reader assigns this “AS ID”. FFS via which R2D message. RAN2 will aim to define one common design for all RA procedures (if technically possible)

In one of our papers submitted to the previous meeting [2], we have discussed and concluded that AS ID is not needed within 3-step CBRA procedure and 2-step CBRA procedure, and that AS ID can be used for reducing signaling overhead, if there are needs for additional communications after 3-step CBRA or 2-step CBRA procedure, e.g., to perform NAS procedures (such as authentication, authorization, NAS temporary ID assignment, security key management, etc.) or to transfer data due to read or write command(s).

In this contribution, we further discuss additional purpose of AS ID for Ambient IoT devices.

# Discussions

## Discussion on additional purpose of AS ID

In meeting #127-bis, RAN2 reached the following agreements regarding the energy status indication [1]:

Agreements on energy bit indication

Capture in the TR the option: the device may include energy status indication in D2R messages (e.g. MSG1,MSG3 and Command Response message), if the device can.  1 bit indication can be captured in the TR.    We will capture the use case in the TR.   FFS whether it is reader controlled.

In one of our companion papers submitted to this meeting, we describe how the energy status indication (ESI) is used [3]. To monitor the energy status of a device based on the ESI received from the device, the reader needs to store the ESI value in association with an identifier of the device for a long period, e.g., for the duration of a paging round or for the duration of an (estimated) unavailability period of the device. It has been assumed in the RAN plenary discussion that the charging time for an Ambient IoT device can be up to several tens of seconds [4]. And to predict a next availability time of a device for communications, the reader also needs to have an identifier of the device that will remain valid and unique after a long unavailability period of the device.

The random ID that the device sends in Msg1 may not remain valid for such a long period. Otherwise, the same random ID may be selected by another device for its random access and therefore become no longer unique for identification purpose nor for ESI tracking purpose.

An alternative ID to be associated with the ESI and the predicted next availability time of the device is the upper layer device ID of the device. An interim conclusion made by SA2 on the device ID is shown in the Annex, in accordance with [5]. First, it is unclear if the upper layer device ID will be provided or fully visible to the reader. Although the device may include such device ID in Msg3, the length and structure of the device ID may be variable, depending on whether the part 1 of the ID is operator assigned or third-party assigned and also depending on what is in the part 2 (e.g., there are different versions of EPC with different lengths), and therefore it may be beyond the knowledge of the reader as to how to parse the ID from Msg 3 and how to use it properly.

Therefore, a new need for an AS ID, which can remain valid and unique for a longer duration (than the valid duration of random ID), may have emerged due to the study on the ESI and the study on controlling the device’s availability/unavailability time.

Observation: A new need for a separate AS ID, which can remain valid and unique for a longer duration, may have emerged due to the study on the ESI and the study on controlling the device’s availability/unavailability time.

Proposal 1. RAN2 consider that AS ID is required if the energy status indication is to be supported.

## Discussion on a need for differentiating between AS ID and random ID in the AS signaling

Regardless how the AS ID is generated, there is a need for determining whether the AS ID or the random ID is used in a particular message. In a first option, we can mandate which type of ID is used in which type(s) of messages, e.g., we can specify that the random ID is used on Msg1, Msg2, and Msg3, and the AS ID, if assigned, is used on subsequent messages in CBRA procedures and on messages in CFRA procedures. In a second option, a 1-bit indicator may be sent along with the ID to indicate whether the ID is the random ID or the AS ID.

Proposal 2. RAN2 study a way for differentiating between the AS ID and the random ID in the AS signaling.

Proposal 3. As a starting point, RAN2 can consider the following options:

- Option A. Specify which type of ID is used in which type(s) of messages, e.g., the random ID is used on Msg1, Msg2, and Msg3, and the AS ID, if assigned, is used on subsequent messages in CBRA procedures and on messages in CFRA procedures.

- Option B. Specify a 1-bit indicator, which can be sent along with the ID to indicate whether the ID is the random ID or the AS ID.

## Discussion on how the AS ID is generated

Between previously agreed Option 1 (reusing the random ID as the AS ID) and Option 2 (reader assigning the AS ID), we consider Option 2 as a must-have to ensure the final uniqueness in AS ID assignment. However, we also recognize that there may be signaling overhead saving for AS ID assignment if the random ID of the device can be directly converted into its AS ID whenever possible, e.g., when the random ID is still unique if assigned as an AS ID. For example, if the random ID of a device has not been taken as the AS ID of any other devices, the reader may simply use a 1-bit indicator in an R2D message to indicate that the random ID sent by the device in Msg1 is to be used as its AS ID as well. On the other hand, if the random ID of the device has been taken as the AS ID of another device, the reader provides a unique AS ID, generated by the reader, in the R2D message for the device to use as its AS ID. Therefore, we propose the following:

Proposal 4. RAN2 down-select between the following two revised options for generating AS ID:

- Option 1’: the reader indicates to the device to use the random ID sent in its Msg1 as its AS ID, if the random ID is still unique as an AS ID; otherwise, the reader assigns a unique AS ID to the device.

- Option 2’: The reader always assigns the AS ID to a device independent of the random ID sent by the device in Msg1.

For whichever option down-selected, RAN2 can determine which R2D message carries the indication/assignment during the WI phase.

# Conclusions

Observation: A new need for a separate AS ID, which can remain valid and unique for a longer duration, may have emerged due to the study on the ESI and the study on controlling the device’s availability/unavailability time.

Based on the observation, we propose the following:

Proposal 1. RAN2 consider that AS ID is required if the energy status indication is to be supported.

Proposal 2. RAN2 study a way for differentiating between the AS ID and the random ID in the AS signaling.

Proposal 3. As a starting point, RAN2 can consider the following options:

- Option A. Specify which type of ID is used in which type(s) of messages, e.g., the random ID is used on Msg1, Msg2, and Msg3, and the AS ID, if assigned, is used on subsequent messages in CBRA procedures and on messages in CFRA procedures.

- Option B. Specify a 1-bit indicator, which can be sent along with the ID to indicate whether the ID is the random ID or the AS ID.

Proposal 4. RAN2 down-select between the following two revised options for generating AS ID:

- Option 1’: the reader indicates to the device to use the random ID sent in its Msg1 as its AS ID, if the random ID is still unique as an AS ID; otherwise, the reader assigns a unique AS ID to the device.

- Option 2’: The reader always assigns the AS ID to a device independent of the random ID sent by the device in Msg1.

For whichever option down-selected, RAN2 can determine which R2D message carries the indication/assignment during the WI phase.

# References

R2_127bis_ChairNotes_24-10-18_13-00.

R2-2409172, Discussions on AS ID for Ambient IoT devices, Futurewei.

R2-2410572, Discussions on energy status indication for Ambient IoT, Futurewei.

RP-240854, Moderator summary on R19 Ambient IoT, Moderator (Huawei).

TR 23.700-13 v1.1.0

# Annex: SA2 Interim Conclusion on Device ID in [5]

## 8.2	Interim Conclusion on Key Issue #2

### 8.2.1	Identifier and Identification Management

The following aspects and principles are considered and agreed for the interim conclusion on Identifier and Identification Management.

The following principles are agreed for Identifier and Identification Management:

-	An Ambient IoT Device is configured with a permanent Ambient IoT Device Identifier which can be assigned by an operator or by a third party. The Identifier is used to identify Ambient IoT Device and locate the corresponding authentication server.

NOTE 1:	How to configure Ambient IoT Device with the permanent Ambient IoT Device Identifier is out of 3GPP scope.

The permanent Ambient IoT Device Identifier includes the following information:

1)	Part1information:

-	The ID type, which is used to indicate the identifier is assigned by an operator or by a third party.

-	The operational entity, which is the entity managing and allocating the Identifier:

-	For operator allocated ID, it is a network identifier (i.e. MCC+MNC and/or NID).

-	For third party allocated ID, it is information used to identify a 3rd party.

Editor's note:	Whether the operational entity is always needed is FFS.

2)	Part2 information:

-	The information (e.g. EPC or others) used to distinguish different Ambient IoT Devices within the scope identified by the Part1 information.

NOTE 2:	If the information used to identify the 3rd party is the scheme and the company prefix of the EPC, Part2 information is not necessary to be the whole EPC but can be the leftover part of the EPC.

NOTE 3:	The coding for the above information is left to stage 3.

Figure 8.2.1-1: The Operator allocated ID

For operator allocated Identifier, the network identifier is mandatory and can be used to index the authentication server or not.

Editor's note:	For operator allocated Identifier, it is FFS whether further information (e.g. the credential holder ID) in Part 1 is needed.

Figure 8.2.1-2 The third party allocated ID

For third party allocated Identifier, the network identifier is not needed. The third party may be the credential holder or not. If it is not the credential holder, the network should be provided with third party related context including the information used to locate the authentication server.

Editor's note:	The length of Identifier is fixed or dynamical is FFS.

Editor's note:	The uniqueness of Identifier is FFS.

Editor's note:	Whether the temporary ID in the AIoT NAS layer is required for the privacy protection is FFS and is pending SA WG3 decision.