3GPP TSG-RAN WG2 meeting #129	R2-2500002

Agenda Item:	2.2

Source: 	ETSI MCC

Title:		Report of 3GPP TSG RAN WG2 meeting #128, Orlando, USA

Report of 3GPP TSG RAN WG2 meeting #128

Orlando, USA

18 - 22 November, 2024

Document for: Approval

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

© 2024, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

# Contents

Organisation of the meeting	7

Statistics/Executive Summary	7

General	8

1	Opening of the meeting	8

1.1	Call for IPR	8

1.2	Network usage conditions	8

1.3	Other	8

2	General	8

2.1	Approval of the agenda	8

2.2	Approval of the report of the previous meeting	8

2.3	Reporting from other meetings	9

2.4	Instructions	9

2.5	Others	9

3	Incoming liaisons	11

4	EUTRA Rel-17 and earlier	11

4.1	EUTRA corrections Rel-17 and earlier	11

4.1.0	In-principle agreed CRs	12

4.1.1	Other	12

4.2	V2X and Sidelink corrections Rel-15 and earlier	13

4.3	Positioning corrections Rel-16 and earlier	14

4.3.0	In-principle agreed CRs	14

4.3.1	Other	14

5	NR Rel-15 and Rel-16	14

5.1	Common	14

5.1.1	Stage 2 and Organisational	15

5.1.1.0	In-principle agreed CRs	15

5.1.1.1	Other	16

5.1.2	User Plane corrections	16

5.1.2.0	In-principle agreed CRs	16

5.1.2.1	MAC	17

5.1.2.2	RLC PDCP SDAP BAP	17

5.1.2.3	Other	17

5.1.3	Control Plane corrections	17

5.1.3.0	In-principle agreed CRs	17

5.1.3.1	NR RRC	19

5.1.3.2	UE capabilities	20

5.1.3.3	Other	21

5.2	NR V2X	21

5.2.0	In-principle agreed CRs	21

5.2.1	Other	22

5.3	NR Positioning Support	22

5.3.0	In-principle agreed CRs	22

5.3.1	Other	23

6	NR Rel-17	26

6.1	Common	26

6.1.1	Stage 2 and Organisational	27

6.1.1.0	In-principle agreed CRs	28

6.1.1.1	Other	29

6.1.2	User Plane corrections	29

6.1.2.0	In-principle agreed CRs	30

6.1.2.1	Other	31

6.1.3	Control Plane corrections	33

6.1.3.0	In-principle agreed CRs	33

6.1.3.1	NR RRC	34

6.1.3.2	UE capabilities	37

6.1.3.3	Other	40

6.2	NR Sidelink relay	41

6.2.0	In-principle agreed CRs	41

6.2.1	Other	41

6.3	NR positioning enhancements	41

6.3.0	In-principle agreed CRs	41

6.3.1	Other	42

6.6	NR Sidelink enhancements	44

6.6.0	In-principle agreed CRs	45

6.6.1	Other	45

7	Rel-18	47

7.0	Common	47

7.0.0	In-principle agreed CR	47

7.0.1	UE Capabilities	50

7.0.2	Rel-18 corrections	51

7.0.2.1	RACH-less HO	51

7.0.2.2	NR network-controlled repeaters	52

7.0.2.3	NR support for UAV	52

7.0.2.4 	Mobile Terminated Small Data Transmission	52

7.0.2.5	IDC enhancements for NR and MR-DC	53

7.0.2.6	Mobile IAB (Integrated Access and Backhaul) for NR	53

7.0.2.7	Timing Resiliency and URLLC Enh	53

7.0.2.8	Enhanced support of reduced capability NR devices	53

7.0.2.9	Further NR coverage enhancements	53

7.0.2.10	Network energy savings for NR	55

7.0.2.11	Further enhancement of data collection for SON MDT in NR and EN-DC	56

7.0.2.12	Dual Transmission/Reception (Tx/Rx) Multi-SIM for NR	57

7.0.2.13	NR MIMO evolution	58

7.0.2.14	Enhancements of NR Multicast and Broadcast Services	61

7.0.2.15	Enhancement on NR QoE management and optimizations for diverse services	63

7.0.2.16	XR Enhancements for NR	64

7.0.2.17	Others	65

7.1	Expanded and improved NR positioning	67

7.1.0	In-principle agreed CRs	67

7.1.1	Organizational	70

7.1.2	Stage 2	70

7.1.3	SLPP corrections	70

7.1.4	LPP corrections	72

7.1.5	RRC corrections	73

7.1.6	MAC corrections	76

7.1.7	Corrections to other specifications	76

7.2	Further NR mobility enhancements	76

7.2.1	Organizational	76

7.2.2	In-principle agreed CRs	76

7.2.3	Others	77

7.3	IoT NTN enhancements	82

7.3.0	In-principle agreed CRs	82

7.3.1	Organizational	83

7.3.2	Corrections	83

7.4	NR NTN enhancements	85

7.4.0	In-principle agreed CRs	85

7.4.1	Organizational	85

7.4.2	Corrections	85

7.5	Enhanced NR Sidelink Relay	89

7.5.0	In-principle agreed CRs	89

7.5.1	Organizational	90

7.5.2	Stage 2 corrections	90

7.5.3	Control plane corrections (including UE capabilities)	91

7.5.4	User plane corrections (including SRAP)	92

7.6	NR Sidelink evolution	93

7.6.1	Organizational	93

7.6.2	In-principle agreed CRs	93

7.6.3	Others	94

7.7	TEI18	96

7.7.0	In-principle agreed CRs	96

7.7.1	TEI proposals by Other Groups	96

7.7.2	TEI proposals by RAN2	96

7.8	R18 Other	98

7.8.0	In-principle agreed CRs	98

7.8.1	RAN4 led items	99

7.8.2	RAN1 led items	101

7.8.3	Other	102

8	Rel-19	102

8.0	General	102

8.1	AI/ML for NR air interface	102

8.1.1	Organizational	102

8.1.2	Functionality based LCM	103

8.1.2.1	LCM for NW-sided model for Beam Management use case	103

8.1.2.2	LCM for UE-sided model  for Beam Management use case	103

8.1.2.3	LCM for Positioning use case	109

8.1.3	NW side data collection	113

8.1.4	UE side data collection	118

8.1.5	Model transfer/delivery	120

8.2	Ambient IoT	122

8.2.1	Organizational	122

8.2.2	Functionality aspects	123

8.2.3	A-IoT Paging	127

8.2.4	A-IoT Random Access	131

8.2.5	Topology 2 considerations	135

8.4	Low-power wake-up signal and receiver for NR (LP-WUS/WUR)	154

8.4.1	Organizational	155

8.4.2	Procedure and configuration of LP-WUS in RRC_IDLE/INACTIVE	155

8.4.3	RRM measurement relaxation and offloading in RRC_IDLE/INACTIVE	159

8.4.4	Procedures for LP-WUS in RRC_CONNECTED	161

8.5	Network Energy Saving Enh.	164

8.5.1	Organizational	164

8.5.2	On-demand SSB SCell operation	164

8.5.3	On-demand SIB1	165

8.5.4	Adaptation of common signal/channel transmissions	171

8.6	Mobility Enhancement Ph4	173

8.6.1	Organizational	173

8.6.2	Inter-CU LTM	174

8.6.3	L1 event triggered measurement reporting	180

8.6.4	Conditional intra-CU LTM	185

8.7	XR Enhancements Ph3	190

8.7.1	Organizational	190

8.7.2	Multi-modality support	191

8.7.3	RRM measurement gaps/restrictions related enhancements	191

8.7.4	Scheduling enhancements	194

8.7.4.1	LCP enhancements	194

8.7.4.2	DSR enhancements	197

8.7.5	RLC enhancements	199

8.7.6	XR rate control	205

8.8	NTN for NR Ph3	208

8.8.1	Organizational	208

8.8.2	Downlink coverage enhancements	209

8.8.3	Uplink Capacity/Throughput Enhancement	214

8.8.4	Support of Broadcast service	214

8.8.5	Support of regenerative payload	218

8.8.6	LTE to NR NTN mobility	218

8.9	IoT NTN Ph3	219

8.9.1	Organizational	220

8.9.2	Support of Store & Forward	220

8.9.3	Uplink Capacity Enhancement	225

8.9.4	Support of PWS	231

8.10	SON/MDT Ph4	234

8.10.1	Organizational	234

8.10.2	MRO enhancements for Rel-18 mobility features	234

8.10.3	SON/MDT for Slicing	237

8.10.4	SON/MDT for NTN	237

8.10.5	Leftovers from Rel-18	237

8.11	Evolution of NR duplex operation: Sub-band full duplex (SBFD)	238

8.11.1	Organizational	238

8.11.2	Random access in SBFD	238

8.11.3	Other aspects	242

8.12	NR MIMO Phase 5	243

8.12.1	Organizational	243

8.12.2	Asymmetric DL sTRP/UL mTRP	244

8.12.3	Others	246

8.13	NR sidelink multi-hop relay	247

8.13.1	Organizational	247

8.13.2	Relay discovery and (re)selection	248

8.13.3	Control Plane Procedures and SRAP impact	254

8.13.4	Service continuity	259

8.14	Additional topological enhancements	262

8.15	NavIC L1 SPS A-GNSS support	262

8.16	BDS B2b in A-GNSS	263

8.17	IoT-NTN TDD mode	264

8.18	TEI19	265

9	Breakout session reports	268

9.1	Session on V2X/SL, R19 NES and MOB	268

9.2	Session on R18 MIMOevo, R18 MUSIM, and R19 LP-WUS	268

9.3	Session on NR NTN and IoT NTN	268

9.4	Session on positioning and sidelink relay	268

9.5	Session on R18 MBS, R18 QoE and R19 XR	269

9.6	Session on maintenance and SON/MDT	269

Closing of the meeting	269

Annex A:	List of participants	270

Annex B:	List of Tdocs	270

Annex C:	Incoming liaison statements	270

Annex D:	Outgoing liaison statements	273

Annex E:	List of agreed CRs	274

Annex F:	List of email discussions during the meeting	285

Annex G:	Post-meeting email discussions	287

Guidelines for email discussions:	287

Deadlines	287

Short email discussions, Deadline Nov. 29th, 10:00 UTC	287

Long email discussions, for R2-129, Deadline Jan.  31st , 10:00 UTC (unless otherwise stated)	289

Annex H:	History	291

# Organisation of the meeting

Meeting:	3GPP TSG RAN2#128

Meeting location:	Orlando, USA

Duration:	18 - 22.11.2024

Host:	ATIS

TSG RAN WG2 Chair	Diana Pani (InterDigital) (diana.pani@interdigital.com)

TSG RAN WG2 Vice chair:	Kyeongin Jeong (Samsung) (kyeongin.j@samsung.com)

TSG RAN WG2 Vice chair:	Erlin Zeng (CATT) (erlin.zeng@catt.cn)

TSG RAN WG2 MCC Support:	Juha Korhonen (ETSI MCC) (juha.korhonen@etsi.org)

Email reflector:	3GPP_TSG_RAN_WG2@LIST.ETSI.ORG

Technical documents:	ftp://ftp.3gpp.org/tsg_ran/WG2_RL2/TSGR2_128/Docs

Next meetings:	TSG RAN2#129	17 - 21.02.2025, Athens, Greece

TSG RAN2#129bis	07 - 11.04.2025, Wuhan, China

# Statistics/Executive Summary

TSG RAN2#128 was a normal face-to-face meeting, with a possibility for two-way remote access.

There were 66 numbered email discussions during this meeting.

The topics discussed were:

-	NR Rel-15, Rel-16 and Rel-17 User Plane corrections, UE Capabilities, Rel-18 Common, XR Enhancements for NR, TEI18, Rel-18 Other, Rel-19 General, AI/ML for NR air interface, Ambient IoT, AI/ML for Mobility - Diana Pani (Chair)

-	NR V2X, NR Sidelink enhancements, NR Sidelink evolution, Further NR mobility enhancements, Network Energy Saving Enh, Mobility Enhancement Ph4 - Kyeongin Jeong (VC)

-	NR MIMO evolution, Low-power wake-up signal and receiver for NR (LP-WUS/WUR), Evolution of NR duplex operation: Sub-band full duplex (SBFD), NR MIMO Phase 5 - Erlin Zeng (VC)

-	Rel-17 NR stage 2 and RRC, IoT NTN enhancements, NR NTN enhancements, NTN for NR Ph3, IoT NTN Ph3, IoT-NTN TDD mode, NB-IoT satellite information - Sergio Parolari

-	Positioning corrections Rel-16 and earlier, Rel-15 and Rel-16 NR Positioning Support, NR sidelink relay, NR positioning enhancements, Expanded and improved NR positioning, Enhanced NR Sidelink Relay, NR sidelink multi-hop relay, NavIC L1 SPS A-GNSS support, BDS B2b in A-GNSS - Nathan Tenny

-	Enhancements of NR Multicast and Broadcast Services, Enhancement on NR QoE management and optimizations for diverse services, XR Enhancements Ph3 - Dawid Koziol

-	EUTRA corrections Rel-17 and earlier, NR Rel-15, Rel-16 and Rel-17 (except user plane), Further enhancement of data collection for SON MDT in NR and EN-DC, SON/MDT Ph4 - Mattias Bergström

The statistics from this meeting are:

-	432 f2f participants

-	1717 Tdoc numbers allocated with 1646 available contributions. (See the attached tdoc list)

-	36 incoming liaison statements, out of which 30 were noted. The remaining non-treated liaison will be treated in RAN2#129 meeting.

-	24 outgoing liaison statements

-	66 at-meeting email discussions

-	27 email approvals/discussions scheduled after the RAN2#128 meeting (16 short and 11 long), see Annex G for details.

Number of CRs submitted: 636. Out of these, 284 were agreed. See Annex E for details.

# General

This meeting was an ordinary meeting and had full decision power, i.e. full decision power to make agreements and approvals according to RAN WG2 terms of reference, without any need to ratify decisions at a later RAN2 or other meeting.

# 1	Opening of the meeting

## 1.1	Call for IPR

The attention of the delegates of this Working Group is drawn to the fact that 3GPP Individual Members have the obligation under the IPR Policies of their respective Organizational Partners to inform their respective Organizational Partners of Essential IPRs they become aware of. The delegates were asked to take note that they were hereby invited: - to investigate whether their organization or any other organization owns IPRs which were, or were likely to become Essential in respect of the work of 3GPP. - to notify their respective Organizational Partners of all potential IPRs, e.g., for ETSI, by means of the IPR Statement and the Licensing declaration forms (https://www.etsi.org/images/files/IPR/etsi-ipr-form.doc)

NOTE:	IPRs may be declared to the Director-General or Chairman of the SDO, but not to the RAN WG2 Chairman.

## 1.2	Network usage conditions

1/ 	To avoid email system overload, please don’t attach files and documents to emails e.g. for offline email discussions, but instead use files placed on the meeting server instead. Inbox/Drafts folder is used for meeting offline discussions.

## 1.3	Other

In accordance with the Working Procedures it is reaffirmed that: (i) compliance with all applicable antitrust and competition laws is required; (ii) timely submissions of work items in advance of TSG or WG meetings are important to allow for full and fair consideration of such matters; and (iii) the chair will conduct the meeting with strict impartiality and in the interests of 3GPP

Note on (i): In case of question please contact your legal counsel.

Note on (ii): WIDs don’t need to be submitted to the RAN2 meeting and will typically not be discussed here either.

# 2	General

## 2.1	Approval of the agenda

R2-2409501	Agenda for RAN2#128	Chairman	agenda

Approved

## 2.2	Approval of the report of the previous meeting

R2-2409502	RAN2#127bis Meeting Report	MCC	report

Approved

## 2.3	Reporting from other meetings

## 2.4	Instructions

CRs

- Use latest CR template version 12.3 for all CRs submitted to RAN2 meeting

Rel-18 and earlier maintenance CRs

- Only essential/critical corrections are expected

- Editorial and clarification corrections should be sent to be reviewed and approved by spec rapporteurs prior to submission.

- Editorials corrections should be collected and submitted by spec rapporteurs.

- NOTE: the tdoc limit applies to all CRs (i.e. WI spec rapporteurs are NO longer expected to submit individual contributions).  They can submit a company CR where they also include miscellaneous corrections that have been sent to them.

Rel-18 UE capabilities

-	EUTRA UE capabilities corrections are covered by separate CRs

-	RAN1/RAN4 NR UE capabilities (new) and corrections are covered in Rel-18 common MegaCRs (38306 and 38331) covering all rel-18 WIs (end outcome).

-	UE capabilities in LPP 37355 and SLPP 38355 are covered in the main CRs for the Positioning WI.

During the work on NR UE caps:

-	In a Common Rel-18 Agenda Item (AI): RAN1 and RAN4 feature corrections are handled jointly under a common AI, with some explicit exceptions. Running UE cap MegaCRs are maintained for the parts handled in the common AI.

-	In WI-specific Rel-18 Agenda Items: RAN2 features/corrections are handled per WI and agreed as individual CRs

Tdoc limitations

Tdoc limitations doesn’t apply to Rapporteur Input, i.e.

-	Assigned summary rapporteur input of the summary.

-	Email / offline discussions outcomes by discussion rapporteur,

-	Limit of 1 WI/SI  rapporteurs input for WI planning.  The work plan is not expected to be updated/submitted every meeting, unless needed.   It can include progress of other WG groups in the same Tdoc (i.e. separate Tdocs on other WG agreements are not required).

-	TS rapporteur input for TS maintenance.

-	Contact Company of a LSin that triggers RAN2 action may submit one tdoc to facilitate the LS reply. This only applies to one of the contact companies in case there are several (default the first).

Tdoc limitations doesn’t apply to Input created at the meeting, revisions, assigned documents etc.

Tdoc limitations doesn’t apply to shadow / mirror CRs (Cat A), or In-Principle Agreed CRs.

Tdoc limitations applies to all other submitted tdocs (e.g. discussion tdoc and CR tdoc are counted as two).

Postponed CRs still count towards tdoc limit unless 3 or more companies are co-sourcing it.

Tdoc request/submission for RAN2#128 deadlines:

- Tdoc Submission deadline: Nov. 8th, 1000 UTC

## 2.5	Others

R2-2409503	RAN2 Handbook	MCC	discussion

Noted

R2-2409711	Reply LS on RAN3 vs RAN2 Basketball Match	OPPO	LS out	Rel-19	To:RAN3

The LS is approved

R2-2409712	Report on RAN2 Basketball team	OPPO	discussion

Noted

R2-2410130	Clarification on providing impact analysis in CR coversheet	Lenovo	discussion

Noted

R2-2410464	Discussion on CR coversheet update 	Ericsson 	discussion

Proposal 1	Information on “Impacted 5G architecture options” is not required for a CR coversheet containing impact analysis.

Noted

Discussion

-	Samsung thinks that we should only omit if we the CR applies to all architecture options otherwise it should be there. If we omit then it is not friendly to implementation.

-	Vodafone thinks it is not useful as delegates either just click all options or don’t do it accurately.

-	Huawei thinks it is better to keep it and there are cases that we didn’t indicate NR-DC architecture only.

-	ZTE also thinks that this is useful. But we should have some improvement but not change it now.

-	Qualcomm thinks it was useful before but not much gain now. If we keep it we should review what we have included and what we should include.

-	Mediatek would support to keep it but improve it.

Can consider further improvements on architecture options on CRs.

- Agreement

Confirm that for stage 2/stage 3 CRs that are submitted towards frozen releases and have impacts to 5G architecture options, the impact analysis statement should include the “Impacted 5G architecture options”, “Impacted functionality” and “Interoperability”.

RAN3 endorsed stage-2 CRs for RAN2 review and agreement ([POST][128][999] Review of RAN3 endorsed CRs (MCC))

R2-2411233	Correction on conditional PSCell addition or change failure	R3 (CATT, CMCC, ZTE, Nokia, Nokia Shanghai Bell)	CR	Rel-18	37.340	18.3.0	0407	-	F	NR_ENDC_SON_MDT_enh2-Core

=> Agreed

R2-2411234	Correction on AI/ML Terminology	R3 (Nokia, Ericsson, ZTE, Huawei)	CR	Rel-18	38.300	18.3.0	0945	-	F	NR_AIML_NGRAN-Core

=> Agreed

R2-2411235	OAM Requirements on Energy Cost index	R3 (Nokia, Jio, Telecom Italia, Ericsson, ZTE, Samsung, NEC, Huawei, Deutsche Telekom)	CR	Rel-18	38.300	18.3.0	0946	-	F	NR_AIML_NGRAN-Core

=> Agreed

R2-2411236	Correction of IAB F1-C transfer in NR-DC	R3 (ZTE Corporation, Ericsson, Lenovo, Huawei, Qualcomm, Nokia, Nokia Shanghai Bell, CATT, Samsung)	CR	Rel-17	37.340	17.7.0	0408	-	F	NR_IAB_enh-Core

=> Agreed

R2-2411237	Correction of IAB F1-C transfer in NR-DC	R3 (ZTE Corporation, Ericsson, Lenovo, Huawei, Qualcomm, Nokia, Nokia Shanghai Bell, CATT, Samsung)	CR	Rel-18	37.340	18.3.0	0409	-	A	NR_IAB_enh-Core

=> Agreed

R2-2411238	Stage 2 correction for AI/ML for NG-RAN	R3 (Ericsson, Huawei, Nokia, InterDigital, ZTE, NEC, Deutsche Telekom)	CR	Rel-18	38.300	18.3.0	0947	-	F	NR_AIML_NGRAN-Core

=> Agreed

R2-2411239	QoE measurement handling in idle mode	R3 (CATT, Ericsson, ZTE, Xiaomi)	CR	Rel-18	38.300	18.3.0	0948	-	F	NR_QoE_enh-Core

=> Agreed

R2-2411240	Correction to area-specific SRS activation	R3 (CATT, ZTE, Ericsson, China Telecom, Nokia, Samsung, Qualcomm Incorporated, Xiaomi, Huawei)	CR	Rel-18	38.305	18.3.0	0181	-	F	NR_pos_enh2-Core

=> Agreed

R2-2411241	Coarse UE Location Information Reporting from MME to eNB for NB-IoT UEs	R3 (Qualcomm, Ericsson, Nokia, Nokia Shanghai Bell, ZTE, Huawei, CATT)	CR	Rel-18	36.300	18.3.0	1415	-	F	IoT_NTN_enh-Core

=> Agreed

R2-2411242	Corrections to Stage-2 for XR awareness	R3 (Ericsson, Qualcomm Inc., Nokia, Nokia Shanghai Bell, Huawei, Samsung, ZTE, China Telecom, Xiaomi, CATT, CMCC, Lenovo)	CR	Rel-18	38.300	18.3.0	0949	-	F	NR_XR_enh-Core

=> Agreed

R2-2411243	Stage-2 updates for intra-SN S-CPAC with MN involvement	R3 (Ericsson, Nokia, CATT, LG Electronics, Huawei, ZTE, Samsung)	CR	Rel-18	37.340	18.3.0	0410	-	F	NR_Mob_enh2-Core

=> Agreed

# 3	Incoming liaisons

Note: LSs are moved to the respective agenda items if any.

# 4	EUTRA Rel-17 and earlier

Only essential corrections. No documents should be submitted to 4. Please submit to 4.x

## 4.1	EUTRA corrections Rel-17 and earlier

(NB_IOTenh4_LTE_eMTC6-Core; leading WG: RAN1; REL-17; WID: RP-211340)

(UPIP_EN-DC_UE; leading WG: RAN3; REL-17; WID: RP-213669)

(LTE TEI17)

Essential corrections to LTE Rel-17 topics not covered by other agenda items.

(NB_IOTenh3-Core; leading WG: RAN1; REL-16; started: Jun 18; Completed: June 20; WID: RP-200293); REL-15 and Earlier NB-IoT WIs are in scope but not listed explicitly (long list).

(LTE_eMTC5-Core; LTE_eMTC5-Core; leading WG: RAN1; REL-16; started: Jun 18; Completed:  June 20; WID: RP-192875;), REL-15 and Earlier eMTC WIs are in scope but not listed explicitly (long list).

(LTE_feMob-Core; leading WG: RAN2; REL-16; started: Jun 18; Completed: June 20; WID: RP-190921);

(LTE_terr_bcast-Core, LTE_DL_MIMO_EE-Core, LTE_high_speed_enh2-Core; LTE TEI16 Non-positioning);

(LTE_NBIOT_eMTC_NTN; leading WG: RAN1; REL-17; WID: RP-211601)

REL-16 and Earlier EUTRA WIs are in scope but not listed explicitly (long list), Except V2X and Sidelink WIs and Positioning WIs, which are addressed by AIs below.

NOTE that LTE corrections related to NR WIs or Joint NR LTE WIs should be submitted to NR AIs below.

NOTE that LTE corrections which are the same as an NR correction should be submitted to the respective NR AI (so the NR CR and LTE CR can be treated together).

This Agenda Item is treated in the Maintenance Breakout session (Corrections for LTE_NBIOT_eMTC_NTN might be treated in the NTN breakout session)

Missing reference

R2-2410559	Addition of reference to 38.304 for Qoffsettemp handling	Nokia	CR	Rel-15	36.331	15.23.0	5077	-	F	TEI12

R2-2410560	Addition of reference to 38.304 for Qoffsettemp handling	Nokia	CR	Rel-16	36.331	16.17.0	5078	-	A	TEI12

R2-2410561	Addition of reference to 38.304 for Qoffsettemp handling	Nokia	CR	Rel-17	36.331	17.10.0	5079	-	A	TEI12

R2-2410562	Addition of reference to 38.304 for Qoffsettemp handling	Nokia	CR	Rel-18	36.331	18.3.1	5080	-	A	TEI12

-	Qualcomm thinks its not really needed to add this reference. Samsung (spec rapp) think this could be considered editorial but wanted to let other companies have a chance to properly review this hence this was submitted as a stand-alone CR. Lenovo think this is not editorial since the offset would not be applied to NR, but anyway it is optional so they are OK with the change. Qualcomm is not OK with this change as it is too late now, it should have been added in Rel-15 if at all, now its too late.

-	Huawei is OK with this but from Rel-18 if so. Lenovo think its optional and it clear from the capability spec.

We do this change from Rel-18 with magic sentence. CR to be produced offline.

[AT128][604][Maint] Reference to 38.304 for Qoffsettemp (Nokia)

Scope:

    - Produce agreeable CRs

Intended outcome:

    - Agreed CR in R2-2411015 (Nokia)

Deadline:

    - Wednesday 17:00. The intention is to agree the CRs over email.

R2-2411015	Addition of reference to 38.304 for Qoffsettemp handling	Nokia	CR	Rel-18	36.331	18.3.1	5080	1	A	TEI18

Agreed

### 4.1.0	In-principle agreed CRs

Revised

R2-2410891	Correction to NR Beam Reporting in LTE RRC	Apple, ZTE Corporation	CR	Rel-18	36.331	18.3.1	5069	1	F	NR_newRAT-Core	R2-2409946

Agreed unseen in R2-2411016 with adding TEI18 and add “annex G” as impacted sections. Update revision number in Annex G itself.

R2-2411016	Correction to NR Beam Reporting in LTE RRC	Apple, ZTE Corporation	CR	Rel-18	36.331	18.3.1	5069	1	F	NR_newRAT-Core, TEI18

=> Agreed

Old revisions

R2-2409946	Correction to NR Beam Reporting in LTE RRC	Apple, ZTE Corporation	CR	Rel-18	36.331	18.3.1	5069	-	F	NR_newRAT-Core

### 4.1.1	Other

System information acquisition

R2-2409878	Minor Corrections on TS36.331	vivo	CR	Rel-18	36.331	18.3.1	5066	-	A	LTE_eMTC5-Core

R2-2409879	Minor Corrections on TS36.331	vivo	CR	Rel-17	36.331	17.10.0	5067	-	A	LTE_eMTC5-Core

R2-2409880	Minor Corrections on TS36.331	vivo	CR	Rel-16	36.331	16.17.0	5068	-	F	LTE_eMTC5-Core

-	Qualcomm think this is agreeable

R2-2410026	Correction on SI Reception in IoT-NTN	vivo	CR	Rel-17	36.331	17.10.0	5070	-	F	LTE_NBIOT_eMTC_NTN

R2-2410027	Correction on SI Reception in IoT-NTN	vivo	CR	Rel-18	36.331	18.3.1	5071	-	A	LTE_NBIOT_eMTC_NTN

-	Qualcomm wants to add SIB33 (for R18).

CRs need to be updated and merged. This is to be handled offline.

[AT128][605][Maint] Corrections on SI Reception (vivo)

Scope:

    - Produce agreeable CRs. Discuss if we can find a more future proof way of writing this part of the spec so we don’t need to update all the time in the future.

Intended outcome:

    - Agreed CRs in R2-2411017, R2-2411018, R2-2411019 (vivo)

Deadline:

    - Wednesday 17:00. The intention is to agree the CRs over email.

R2-2411017	Correction on SI Reception in RRC CONNECTED	vivo	CR	Rel-16	36.331	16.17.0	5068	1	F	LTE_eMTC5-Core

R2-2411018	Correction on SI Reception in RRC CONNECTED	vivo 	CR	Rel-17	36.331	17.10.0	5067	1	F	LTE_eMTC5-Core, LTE_NBIOT_eMTC_NTN

R2-2411204	Correction on SI Reception in RRC CONNECTED	vivo 	CR	Rel-168	36.331	18.3.1	5066	1	F	LTE_eMTC5-Core, LTE_NBIOT_eMTC_NTN

-	Qualcomm want to make the wording more generic and refer to other parts of the spec, rather than referring to explicit SIBs. Samsung prefers this explicit proposal from vivo.

All three are agreed

highProrityAccess

R2-2410547	Alignment of highProrityAccess cause in RRC Establishment and Resume	Peraton Labs	CR	Rel-18	36.300	18.3.0	1413	-	F	NR_newRAT-Core

-	Peraton labs indicate that there is a type “ng-gNB”.

-	Lenovo wonders why we need to address this now (in R18)? No one complained so far, why is this critical now? Peraton indicates that there is a misalignment. Huawei think that the current first paragraph of the impacte section is clear enough since it refers to 38.300. Peraton think that 36.300 must capture ng-eNB behaviour. Samsung think that the high prio access was there since Rel-15 so why do we need this change now?

-	Lenovo think that this is stage-2 so there might be misalignment, but there is stage-2 so how come it is critical?

Agreed unseen in R2-2411035, but add impact analysis and TEI18 WI code. And “gNB” typo is fixed.

R2-2411035	Alignment of highProrityAccess cause in RRC Establishment and Resume	Peraton Labs	CR	Rel-18	36.300	18.3.0	1413	1	F	NR_newRAT-Core, TEI18

=> Agreed

Old revisions

R2-2411019	Correction on SI Reception in RRC CONNECTED	vivo

## 4.2	V2X and Sidelink corrections Rel-15 and earlier

REL-15 and Earlier WIs related to V2x and Sidelink are in scope but not listed explicitly (long list).

This Agenda Item is treated in the V2X and Sidelink Breakout session

Tdoc Limitation: 1 tdocs

## 4.3	Positioning corrections Rel-16 and earlier

(LTE_NavIC-Core, LTE TEI16 Positioning), REL-15 and Earlier WIs related to positioning are in scope but not listed explicitly (long list).

Tdoc Limitation: 1 tdoc

### 4.3.0	In-principle agreed CRs

### 4.3.1	Other

R2-2410232	Correction on broadcast of assistance data-r15	Huawei, HiSilicon	CR	Rel-15	36.331	15.23.0	5072	-	F	LCS_LTE_acc_enh-Core

Not pursued

Discussion:

Lenovo have some sympathy for the need code changes but are not sure if the NOTE clarifies.  They think we should follow what we previously did with SBAS ID in NR.

Nokia think the usage of the fields is clear, and they would prefer not to make ASN.1 changes.

Qualcomm think Need OP is correct and OR is wrong; there is nothing to release, and they understand that the behaviour on absence is clear already in the field description; e.g., for encryption, present means yes and implicitly absent means no.

CATT agree with Qualcomm about the need code changes, and they think OR would be wrong because it implies that stored assistance data could be deleted or misapplied.  They think we could add some description for absence.  On the second change, they think the limitation to generic assistance data is not right.

Huawei think Need ON would be all right but OP is not.  They think CATT’s comment about the second change is confusing as it suggests that the common assistance data would apply to only a certain GNSS.  CATT indicate that for any GNSS, the AD should be included and there are both generic and common types.

Qualcomm understand that the GNSS ID is only for the generic assistance data and the common AD are GNSS-independent.

Ericsson think there would be inconsistency between NR and LTE if we took the GNSS ID change in this form, and maybe we don’t need to do anything; they see no practical confusion.

R2-2410233	Correction on broadcast of assistance data-r16	Huawei, HiSilicon	CR	Rel-16	36.331	16.17.0	5073	-	A	LCS_LTE_acc_enh-Core

Not pursued

R2-2410234	Correction on broadcast of assistance data-r17	Huawei, HiSilicon	CR	Rel-17	36.331	17.10.0	5074	-	A	LCS_LTE_acc_enh-Core

Not pursued

R2-2410235	Correction on broadcast of assistance data-r18	Huawei, HiSilicon	CR	Rel-18	36.331	18.3.1	5075	-	A	LCS_LTE_acc_enh-Core

Not pursued

# 5	NR Rel-15 and Rel-16

Essential corrections only.

Tdoc Limitation: 2 tdocs in total for all sub agenda items NOTE: some agenda items have additional Tdoc limits.

In case a correction need to be reflected in both NR TS and LTE TS, the corrections should be submitted under one single AI (so the NR and LTE correction can be treated together), the sub-Ais below this

## 5.1	Common

Includes the following WIs and input that doesn’t fit elsewhere.

(NR_newRAT-Core; leading WG: RAN1; REL-15; started: Mar. 17; closed: Jun. 19: WID: RP-191971)

(NR_IAB-Core; leading WG: RAN2; REL-16; started: Dec 18; target Aug 20; WID: RP-200840)

(NR_unlic-Core; leading WG: RAN1; REL-16; started: Dec 18; Closed June 20; WID: RP-192926).

(NR_IIOT-Core; leading WG: RAN2; REL-16; started: Mar 19; Completed: Jun 20; WID: RP-200797)

(NR_UE_pow_sav-Core; leading WG: RAN1; REL-16; started: Mar 19; Completed Jun 20; WID: RP-200494).

(NR_2step_RACH-Core; leading WG: RAN1; REL-16; started: Dec 18; Completed: June 20; WID: RP-200085).

(SRVCC_NR_to_UMTS-Core; leading WG: RAN2; REL-16; started: Dec 18; Completed; Mar 20; WID: RP-190713)

(RACS-RAN-Core, leading WG: RAN2; REL-16; started: Mar 19; completed: Jun 20; WID: RP-191088)

(NG_RAN_PRN-Core; leading WG: RAN3; REL-16; started: Mar 19; completed: June 20; WID: RP-200122)

(NR_eMIMO-Core, leading WG: RAN1; REL-16; started: Jun 18; target; Aug 20; WID: RP-200474😉

(NR_CLI_RIM; leading WG: RAN1; REL-16; started: Dec 18; Completed: Jun 20; WID: RP-191997;)

(NR_L1enh_URLLC-Core, leading WG: RAN1; REL-16; Completed: June 20; WID: RP-191584)

(LTE_NR_DC_CA_enh-Core; leading WG: RAN2; REL-16; started: Jun 18; Target Aug 20; WI RP-200791)

(NR_Mob_enh-Core; leading WG: RAN2; REL-16; started: Jun 18; Completed June 20; WID: RP-192277).

(NR_SON_MDT-Core; leading WG: RAN3; REL-16; started: Jun 19; Completed June 20; WID: RP-191776)

(NR_HST, NR_RRM_enh-Core, NR_RF_FR1, NR_RF_FR2_req_enh, NR_n66_BW, LTE_NR_B41_Bn41_PC29dBm-Core, NR_CSIRS_L3meas,)

(NR TEI16)

LTE mob enh corrections that are common with NR mobility enhancements should be submitted to this AI.

### 5.1.1	Stage 2 and Organisational

Incoming LSs, etc. You should discuss your stage 2 CRs with the specification rapporteurs before submission. Includes impact to 38.300, 36.300, 37.340

Max nrof CCs

R2-2409814	Maximum number of configured CCs	Nokia, Nokia Shanghai Bell	CR	Rel-15	38.300	15.18.0	0926	-	F	NR_newRAT-Core

R2-2409815	Maximum number of configured CCs	Nokia, Nokia Shanghai Bell	CR	Rel-16	38.300	16.17.0	0927	-	A	NR_newRAT-Core

R2-2409816	Maximum number of configured CCs	Nokia, Nokia Shanghai Bell	CR	Rel-17	38.300	17.10.0	0928	-	A	NR_newRAT-Core

R2-2409817	Maximum number of configured CCs	Nokia, Nokia Shanghai Bell	CR	Rel-18	38.300	18.3.0	0929	-	A	NR_newRAT-Core

4 are agreed

BFD-BFR on PSCell

R2-2410451	Clarification on support of BFD-BFR on PSCell	Ericsson	CR	Rel-15	38.300	15.18.0	0933	-	F	NR_newRAT-Core

R2-2410452	Clarification on support of BFD-BFR on PSCell	Ericsson	CR	Rel-16	38.300	16.17.0	0934	-	A	NR_newRAT-Core

R2-2410453	Clarification on support of BFD-BFR on PSCell	Ericsson	CR	Rel-17	38.300	17.10.0	0935	-	A	NR_newRAT-Core

R2-2410454	Clarification on support of BFD-BFR on PSCell	Ericsson	CR	Rel-18	38.300	18.3.0	0936	-	A	NR_newRAT-Core

-	Huawei think there may be other places that need to be changed if we do this change. ZTE agrees that there are more than this place that would be impacted if we do this change. Ericsson think.

Postponed

#### 5.1.1.0	In-principle agreed CRs

Updated

R2-2410900	Gap requirement for CSI-RS based measurements and inter-RAT measurements  Huawei, HiSilicon, ZTE Corporation, Ericsson, Samsung, CATT, Apple, Nokia, Nokia Shanghai Bell, LG Electronics Inc., OPPO	CR	Rel-16	38.300	16.17.0	0906	3	F	NR_newRAT-Core, TEI16, SRVCC_NR_to_UMTS-Core

R2-2410901	Gap requirement for CSI-RS based measurements and inter-RAT measurements  Huawei, HiSilicon, ZTE Corporation, Ericsson, Samsung, CATT, Apple, Nokia, Nokia Shanghai Bell, LG Electronics Inc., OPPO	CR	Rel-17	38.300	17.10.0	0907	2	A	NR_newRAT-Core, TEI16, SRVCC_NR_to_UMTS-Core

R2-2410902	Gap requirement for CSI-RS based measurements and inter-RAT measurements  Huawei, HiSilicon, ZTE Corporation, Ericsson, Samsung, CATT, Apple, Nokia, Nokia Shanghai Bell, LG Electronics Inc., OPPO	CR	Rel-18	38.300	18.3.0	0908	2	A	NR_newRAT-Core, TEI16, SRVCC_NR_to_UMTS-Core

All agreed

Old revisions

R2-2410419	Gap requirement for CSI-RS based measurements and inter-RAT measurements	Huawei, HiSilicon, ZTE Corporation, Ericsson, Samsung, CATT, Apple, Nokia, Nokia Shanghai Bell, LG Electronics Inc., OPPO	CR	Rel-16	38.300	16.17.0	0906	1	F	NR_newRAT-Core	R2-2408234

R2-2410420	Gap requirement for CSI-RS based measurements and inter-RAT measurements	Huawei, HiSilicon, ZTE Corporation, Ericsson, Samsung, CATT, Apple, Nokia, Nokia Shanghai Bell, LG Electronics Inc., OPPO	CR	Rel-17	38.300	17.10.0	0907	1	A	NR_newRAT-Core	R2-2408235

R2-2410421	Gap requirement for CSI-RS based measurements and inter-RAT measurements (R18)	Huawei, HiSilicon, ZTE Corporation, Ericsson, Samsung, CATT, Apple, Nokia, Nokia Shanghai Bell, LG Electronics Inc., OPPO	CR	Rel-18	38.300	18.3.0	0906	2	A	NR_newRAT-Core	R2-2408234	Withdrawn

R2-2410431	Gap requirement for CSI-RS based measurements and inter-RAT measurements	Huawei, HiSilicon, ZTE Corporation, Ericsson, Samsung, CATT, Apple, Nokia, Nokia Shanghai Bell, LG Electronics Inc., OPPO	CR	Rel-18	38.300	18.3.0	0908	1	A	NR_newRAT-Core	R2-2408236

#### 5.1.1.1	Other

Misc

R2-2409997	Miscellaneous non-controversial corrections Set XXIII	Ericsson	CR	Rel-15	38.331	15.27.0	5127	-	F	NR_newRAT-Core

R2-2409998	Miscellaneous non-controversial corrections Set XXIII	Ericsson	CR	Rel-16	38.331	16.18.0	5128	-	F	NR_newRAT-Core, TEI16

To be handled in a post meeting email [POST128][004]

### 5.1.2	User Plane corrections

User Plane corrections will be handled in the User Plane break out session

#### 5.1.2.0	In-principle agreed CRs

R2-2409552	Correction to available UL-SCH resources in SR triggering	Samsung (Rapporteur), Ericsson, Spreadtrum	CR	Rel-16	38.321	16.17.0	1962	1	F	NR_2step_RACH-Core	R2-2409079

CR is updated with architecture option

CR is agreed with the changes above in R2-2411086

R2-2409553	Correction to available UL-SCH resources in SR triggering	Samsung (Rapporteur), Ericsson, Spreadtrum	CR	Rel-17	38.321	17.10.0	1963	1	A	NR_2step_RACH-Core	R2-2409080

CR is updated with architecture option

CR is agreed with the changes above in R2-2411087

R2-2409554	Correction to available UL-SCH resources in SR triggering	Samsung (Rapporteur), Ericsson, Spreadtrum	CR	Rel-18	38.321	18.3.0	1964	1	A	NR_2step_RACH-Core	R2-2409081

CR is updated with architecture option

CR is agreed with the changes above in R2-2411088

#### 5.1.2.1	MAC

#### 5.1.2.2	RLC PDCP SDAP BAP

#### 5.1.2.3	Other

User plane related corrections that should be handled in User plane break out session.

### 5.1.3	Control Plane corrections

deprioritisationTimer

R2-2410563	Correction on deprioritisationTimer	Nokia, MediaTek Inc., LG Electronics Inc.	CR	Rel-15	38.331	15.27.0	5167	-	F	NR_newRAT-Core

R2-2410564	Correction on deprioritisationTimer	Nokia, MediaTek Inc., LG Electronics Inc.	CR	Rel-16	38.331	16.18.0	5168	-	A	NR_newRAT-Core

R2-2410565	Correction on deprioritisationTimer	Nokia	CR	Rel-17	38.331	17.10.0	5169	-	A	NR_newRAT-Core

R2-2410566	Correction on deprioritisationTimer	Nokia	CR	Rel-18	38.331	18.3.0	5170	-	A	NR_newRAT-Core

-	Lenovo think none of these changes are needed. vivo think that this is aligning to 304 and are fine with the intention but the last change (timer table) needs updating as the stopping should be in response to PLMN selection. Lenovo disagrees with all of vivos comments, 304 talks about deleting a timer so it doesn’t matter about stopping of the timer. Qualcomm supports all changes, in particular the change to the timer-table and want the CR from R18. Samsung agrees with the intention but think the timer table doesn’t need updating.

Try to produce agreeable CR for R18 NR offline.

[AT128][606][Maint] Example (Nokia)

Scope:

    - Produce agreeable CR

Intended outcome:

    - Agreed CR in R2-2411020

Deadline:

    - Wednesday 17:00. The intention is to agree the CRs over email.

R2-2411020	Correction on deprioritisationTimer	Nokia	CR	Rel-18	38.331	18.3.0	5170	2	A	NR_newRAT-Core

Agreed unseen in R2-2411039, but the RAN-tick box should be ticked.

#### 5.1.3.0	In-principle agreed CRs

R2-2409642	Correction on IE SRS-CarrierSwitching	CATT, Vivo, China Telecom, China Unicom, Nokia, ZTE Corporation, CMCC, Ericsson	CR	Rel-15	38.331	15.27.0	4893	3	F	NR_newRAT-Core	R2-2409285

R2-2409643	Correction on IE SRS-CarrierSwitching	CATT, Vivo, China Telecom, China Unicom, Nokia, ZTE Corporation, Ericsson	CR	Rel-16	38.331	16.18.0	4894	3	A	NR_newRAT-Core	R2-2409286

R2-2409644	Correction on IE SRS-CarrierSwitching	CATT, Vivo, China Telecom, China Unicom, Nokia, ZTE Corporation, Ericsson	CR	Rel-17	38.331	17.10.0	4895	3	A	NR_newRAT-Core	R2-2409287

R2-2409645	Correction on IE SRS-CarrierSwitching	CATT, Vivo, China Telecom, China Unicom, Nokia, ZTE Corporation, Ericsson	CR	Rel-18	38.331	18.3.0	4896	3	A	NR_newRAT-Core	R2-2409288

R2-2410203	Correction to missing NR-DC parameters branches	Ericsson	CR	Rel-16	38.331	16.18.0	5089	1	F	NR_newRAT-Core, TEI16	R2-2409292

R2-2410204	Correction to missing NR-DC parameters branches	Ericsson	CR	Rel-17	38.331	17.10.0	5090	1	A	NR_newRAT-Core, TEI16	R2-2409293

R2-2410205	Correction to missing NR-DC parameters branches	Ericsson	CR	Rel-18	38.331	18.3.0	5091	1	A	NR_newRAT-Core, TEI16	R2-2409294

R2-2410829	Corrections on parallelTx capabilities for inter-band and intra-band	Huawei, HiSilicon, Ericsson, Nokia, Nokia Shanghai Bell	CR	Rel-15	38.306	15.26.0	1170	1	F	NR_newRAT-Core	R2-2408469

R2-2410830	Corrections on parallelTx capabilities for inter-band and intra-band	Huawei, HiSilicon, Ericsson, Nokia, Nokia Shanghai Bell	CR	Rel-16	38.306	16.18.0	1171	1	A	NR_newRAT-Core	R2-2408470

R2-2410831	Corrections on parallelTx capabilities for inter-band and intra-band	Huawei, HiSilicon, Ericsson, Nokia, Nokia Shanghai Bell	CR	Rel-17	38.306	17.10.0	1172	1	A	NR_newRAT-Core	R2-2408471

R2-2410832	Corrections on parallelTx capabilities for inter-band and intra-band	Huawei, HiSilicon, Ericsson, Nokia, Nokia Shanghai Bell	CR	Rel-18	38.306	18.3.0	1173	1	A	NR_newRAT-Core	R2-2408472

All above are agreed

Updated

R2-2411101	Correction to Relaxed measurement	LG Electronics, Nokia, Samsung, Ericsson	CR	Rel-16	38.304	16.10.0	0412	4	F	NR_UE_pow_sav-Core	R2-2410885

R2-2411102	Correction to Relaxed measurement	LG Electronics, Nokia, Samsung, Ericsson	CR	Rel-17	38.304	17.9.0	0413	4	A	NR_UE_pow_sav-Core	R2-2410886

R2-2411103	Correction to Relaxed measurement	LG Electronics, Nokia, Samsung, Ericsson	CR	Rel-18	38.304	18.3.0	0414	4	A	NR_UE_pow_sav-Core	R2-2410887

Agreed unseen in R2-2411021, R2-2411022, R2-2411023. Fix the revision number

R2-2411021	Correction to Relaxed measurement	LG Electronics, Nokia, Samsung, Ericsson	CR	Rel-16	38.304	16.10.0	0412	6	F	NR_UE_pow_sav-Core

R2-2411022	Correction to Relaxed measurement	LG Electronics, Nokia, Samsung, Ericsson	CR	Rel-17	38.304	17.9.0	0413	6	A	NR_UE_pow_sav-Core

R2-2411023	Correction to Relaxed measurement	LG Electronics, Nokia, Samsung, Ericsson	CR	Rel-18	38.304	18.3.0	0414	6	A	NR_UE_pow_sav-Core

=> All above are agreed

Old revisions

R2-2410885	Correction to Relaxed measurement	LG Electronics, Nokia, Samsung, Ericsson	CR	Rel-16	38.304	16.10.0	0412	4	F	NR_UE_pow_sav-Core	R2-2410058

R2-2410886	Correction to Relaxed measurement	LG Electronics, Nokia, Samsung, Ericsson	CR	Rel-17	38.304	17.9.0	0413	4	A	NR_UE_pow_sav-Core	R2-2410059

R2-2410887	Correction to Relaxed measurement	LG Electronics, Nokia, Samsung, Ericsson	CR	Rel-18	38.304	18.3.0	0414	4	A	NR_UE_pow_sav-Core	R2-2410060

R2-2410058	Correction to Relaxed measurement	LG Electronics, Nokia, Samsung, Ericsson	CR	Rel-16	38.304	16.10.0	0412	3	F	NR_UE_pow_sav-Core	R2-2409282	Revised

R2-2410059	Correction to Relaxed measurement	LG Electronics, Nokia, Samsung, Ericsson	CR	Rel-17	38.304	17.9.0	0413	3	A	NR_UE_pow_sav-Core	R2-2409283	Revised

R2-2410060	Correction to Relaxed measurement	LG Electronics, Nokia, Samsung, Ericsson	CR	Rel-18	38.304	18.3.0	0414	3	A	NR_UE_pow_sav-Core	R2-2409284	Revised

#### 5.1.3.1	NR RRC

Corrections to 38331, and related change to other TS if applicable, e.g. 36331, Stage-2 etc.

SI request

R2-2409928	Correction on SI request in RRC_CONNECTED	LG Electronics Inc.	CR	Rel-16	38.331	16.18.0	5122	-	F	TEI16

R2-2409929	Correction on SI request in RRC_CONNECTED	LG Electronics Inc.	CR	Rel-17	38.331	17.10.0	5123	-	A	TEI16

R2-2409930	Correction on SI request in RRC_CONNECTED	LG Electronics Inc.	CR	Rel-18	38.331	18.3.0	5124	-	A	TEI16

-	Apple thinks that even if the UE re-reads SIB1 the UE stores SIB1, “stored” is not an issue. MediaTek think the current text is clear and think there is no critical issue. Lenovo is not convinced. Samsung think the requirement is clear on SIB1 acquisition and this “stored” can be kept. Vivo agrees with the previous speakers. ZTE think this CR causes issues.

Not pursued

Protection of RRC messages

R2-2410074	Correction on protection of RRC messages	ZTE Corporation, Sanechips	CR	Rel-16	38.331	16.18.0	5141	-	F	NR_IAB-Core, NR_SON_MDT-Core

R2-2410075	Correction on protection of RRC messages	ZTE Corporation, Sanechips	CR	Rel-17	38.331	17.10.0	5142	-	A	NR_IAB-Core, NR_SON_MDT-Core

R2-2410076	Correction on protection of RRC messages	ZTE Corporation, Sanechips	CR	Rel-18	38.331	18.3.0	5143	-	A	NR_IAB-Core, NR_SON_MDT-Core

-	Huawei think this is informative text and can be merged to the rapp CR. Ericsson agrees and they have tried themselves earlier.

To be merged in the RRC rapporteurs CR

smtc

R2-2410356	Corrections to smtc configuration	ZTE Corporation, Sanechips	CR	Rel-15	38.331	15.27.0	5155	-	F	NR_newRAT-Core

R2-2410357	Corrections to smtc configuration	ZTE Corporation, Sanechips	CR	Rel-16	38.331	16.18.0	5156	-	A	NR_newRAT-Core

R2-2410358	Corrections to smtc configuration	ZTE Corporation, Sanechips	CR	Rel-17	38.331	17.10.0	5157	-	A	NR_newRAT-Core

R2-2410359	Corrections to smtc configuration	ZTE Corporation, Sanechips	CR	Rel-18	38.331	18.3.0	5158	-	A	NR_newRAT-Core

-	MediaTek wonders if the purpose is to clarify or to add new requirements? ZTE think its just clarifying. Nokia think the intention is OK but think its strange to refer to an IE-name, instead they can refer to fields of a certain IE-type. ZTE points out in LTE RRC they use this IE name in procedural text. Ericsson think that if we refer to the IE name there may be an ambiguity? ZTE think that the IE-name is only used for smtc1 and smtc. Ericsson think that in this case we should spell out smtc1 and smtc and prefer to refer to field names.

Produce CRs saying “in the smtc or smtc1 configuration” instead.

[AT128][607][Maint] Corrections to smtc configuration (ZTE)

Scope:

    - Produce agreeable CRs

Intended outcome:

    - Agreed CRs in R2-2411024, R2-2411025, R2-2411026, R2-2411027

Deadline:

    - Wednesday 17:00. The intention is to agree the CRs over email.

-	ZTE reports after offline that companies are fine to use the original proposed corrections, which is more maintainable.

R2-2411024	Corrections to smtc configuration	ZTE Corporation, Sanechips	CR	Rel-15	38.331	15.27.0	5155	1	F	NR_newRAT-Core

R2-2411025	Corrections to smtc configuration	ZTE Corporation, Sanechips	CR	Rel-16	38.331	16.18.0	5156	1	A	NR_newRAT-Core

R2-2411026	Corrections to smtc configuration	ZTE Corporation, Sanechips	CR	Rel-17	38.331	17.10.0	5157	1	A	NR_newRAT-Core

R2-2411027	Corrections to smtc configuration	ZTE Corporation, Sanechips	CR	Rel-18	38.331	18.3.0	5158	1	A	NR_newRAT-Core

All 4 are agreed

PNI-NPN

R2-2410746	Correction to PNI-NPN identity	Huawei, HiSilicon, CATT	CR	Rel-16	38.331	16.18.0	5182	-	F	NG_RAN_PRN-Core

R2-2410747	Correction to PNI-NPN identity	Huawei, HiSilicon, CATT	CR	Rel-17	38.331	17.10.0	5183	-	A	NG_RAN_PRN-Core

R2-2410748	Correction to PNI-NPN identity	Huawei, HiSilicon, CATT	CR	Rel-18	38.331	18.3.0	5184	-	A	NG_RAN_PRN-Core

-	Nokia think this is editorial and want to do it from Rel-18 only. Ericsson think the current spec is fine. China Telecom want to leave the spec as is. CATT think its important to align with SA specs. ZTE think the current definition is correct. Samsung agrees with ZTE, in RAN2 specs there is no issue to keep current definitions. Qualcomm think that the last change should be using field names instead. MediaTek think that for PLMN its difficult to refer to field names. Ericsson think that the last change causes “or.. or… or… and” issues. Ericsson think this is not critical.

Not pursued.

#### 5.1.3.2	UE capabilities

UE cap corrections 38306, 38331

Inter-band capabilities

R2-2410052	Clarification on inter-band handover enhancements capabilities	Qualcomm Inc., OPPO	CR	Rel-16	38.306	16.18.0	1186	3	F	NR_Mob_enh-Core	R2-2409289

R2-2410053	Clarification on inter-band handover enhancements capabilities	Qualcomm Inc., OPPO	CR	Rel-17	38.306	17.10.0	1187	4	A	NR_Mob_enh-Core, NR_NTN_solutions-Core	R2-2409290

R2-2410054	Clarification on inter-band handover enhancements capabilities	Qualcomm Inc., OPPO	CR	Rel-18	38.306	18.3.0	1188	4	A	NR_NTN_enh-Core, NR_ATG-Core, Netw_Energy_NR-Core	R2-2409291

-	MediaTek think that the R18 CR doesnt mention the R17 fields on the cover page. Qualcomm indicates that for some Rel-17 features it was not possible to have inter-band procedures. Huawei think that conditions are incomplete or inconsistent "condHandover".

Attempt to fix the CRs offline

[AT128][608][Maint] Inter-band handover capabilities (Qualcomm)

Scope:

    - Produce agreeable CRs

Intended outcome:

    - Agreed CRs in R2-2411028, R2-2411029, R2-2411030

Deadline:

    - Wednesday 17:00. The intention is to agree the CRs over email.

R2-2411028	Clarification on inter-band handover enhancements capabilities	Qualcomm Inc., OPPO	CR	Rel-16	38.306	16.18.0	1186	4	F	NR_Mob_enh-Core	R2-2410052

R2-2411029	Clarification on inter-band handover enhancements capabilities	Qualcomm Inc., OPPO	CR	Rel-17	38.306	17.10.0	1187	5	A	NR_Mob_enh-Core	R2-2410053

R2-2411030	Clarification on inter-band handover enhancements capabilities	Qualcomm Inc., OPPO	CR	Rel-18	38.306	18.3.0	1188	5	A	NR_NTN_enh-Core, NR_ATG-Core, Netw_Energy_NR-Core	R2-2410054

The three above are agreed, but then R2-2411030 was coversheet revised by MCC in R2-2411259: Cat F is a more suitable category for this CR. It has a different set of WI codes from the Rel-16 cat F CR.

R2-2411259	Clarification on inter-band handover enhancements capabilities	Qualcomm Inc., OPPO	CR	Rel-18	38.306	18.3.0	1188	6	F	NR_Mob_enh-Core, NR_NTN_enh-Core, NR_ATG-Core, Netw_Energy_NR-Core

=> Agreed

Max data rate

R2-2410666	Correction to maximum data rate calculation	Nokia	CR	Rel-15	38.306	15.26.0	1213	-	F	NR_newRAT-Core

R2-2410667	Correction to maximum data rate calculation	Nokia	CR	Rel-16	38.306	16.18.0	1214	-	A	NR_newRAT-Core

R2-2410668	Correction to maximum data rate calculation	Nokia	CR	Rel-17	38.306	17.10.0	1215	-	A	NR_newRAT-Core, NR_DL1024QAM_FR1-Core

R2-2410669	Correction to maximum data rate calculation	Nokia	CR	Rel-18	38.306	18.3.0	1216	-	A	NR_newRAT-Core, NR_DL1024QAM_FR1-Core

-	Huawei think that the field descriptions for the referred fields already clarify what it means it he fields are not present, this CR on its own is not correct. MediaTek and OPPO agrees. Nokia is concerned about that the section for the data rate calculation should refer to the fields even though the field descriptions explain the fields. Ericsson agrees with Huawei and others.

Not pursued.

#### 5.1.3.3	Other

This agenda item addresses the idle and inactive behaviour specified in 38.304 or 36.304, LTE-specific changes for the applicable WIs, Other parts not covered elsewhere.

## 5.2	NR V2X

(5G_V2X_NRSL-Core; leading WG: RAN1; REL-16; started: Mar 19; target; Aug 20; WID: RP-200129).

CR rapporteurs will take care of miscellaneous CRs to collect small changes. Please first contact / coordinate with CR rapporteur company for small changes (e.g. non-controversial clarification/correction, editorial correction, etc.).

Tdoc Limitation: 1 tdocs

### 5.2.0	In-principle agreed CRs

R2-2409684	RRC correction on sl-X-Overhead field description of SL-ResourcePool	Philips International B.V.	CR	Rel-16	38.331	16.18.0	5058	2	F	5G_V2X_NRSL-Core	R2-2409366

R2-2409685	RRC correction on sl-X-Overhead field description of SL-ResourcePool	Philips International B.V.	CR	Rel-17	38.331	17.10.0	5059	2	A	5G_V2X_NRSL-Core	R2-2409367

R2-2409686	RRC correction on sl-X-Overhead field description of SL-ResourcePool	Philips International B.V.	CR	Rel-18	38.331	18.3.0	5060	2	A	5G_V2X_NRSL-Core	R2-2409368

In summary of change, “In clause 6.3.2,” should be corrected to “In clause 6.3.5”.

With the CR cover page update as above, CRs are agreed in R2-2410931, R2-2410932 and R2-2410933.

[Apple]: Cover page should be updated. The actual changed section is 6.3.5 (not 6.3.2)

R2-2410077	Correction on intra-UE prioritization	ZTE Corporation, Sanechips	CR	Rel-16	38.321	16.17.0	1974	1	F	5G_V2X_NRSL-Core	R2-2409363

R2-2410078	Correction on intra-UE prioritization	ZTE Corporation, Sanechips	CR	Rel-17	38.321	17.10.0	1975	1	A	5G_V2X_NRSL-Core	R2-2409364

R2-2410079	Correction on intra-UE prioritization	ZTE Corporation, Sanechips	CR	Rel-18	38.321	18.3.0	1976	1	A	5G_V2X_NRSL-Core	R2-2409365

Adding missed conditional “and the MAC entity is not able to perform this SR transmission simultaneously with the transmission of the SL-SCH resource” in the note.

With the addition above, CRs are agreed in R2-2410934, R2-2410935 and R2-2410936.

[Apple]: Compared to similar contents of the MAC, one additional condition (“and the MAC entity is not able to perform this SR transmission simultaneously with the transmission of the SL-SCH resource”) is missed in the note.

### 5.2.1	Other

## 5.3	NR Positioning Support

(NR_newRAT-Core; leading WG: RAN1; REL-15; started: Mar. 17; closed: Jun. 19: WID: RP-191971)

(NR_pos-Core; leading WG: RAN1; REL-16; started: Mar 19; target; Jun 20; WID: RP-200218).

(NR TEI16 Positioning)

Stage 2 corrections shall be discussed with the specification rapporteur (Sven Fischer sfischer@qti.qualcomm.com) before submission. Stage 2 CRs not discussed with the specification rapporteur will not be treated.

Tdoc Limitation: 1 tdoc

### 5.3.0	In-principle agreed CRs

R2-2409562	Correction on GNSS-AlmanacSupport and GNSS-UTC-ModelSupport in A-GNSS positioning	ZTE Corporation	CR	Rel-16	37.355	16.13.0	0516	1	F	NR_pos-Core	R2-2408213

Agreed as R2-2410906 (coversheet fixes)

R2-2410906	Correction on GNSS-AlmanacSupport and GNSS-UTC-ModelSupport in A-GNSS positioning	ZTE Corporation	CR	Rel-16	37.355	16.13.0	0516	2	F	LCS_NAVIC-Core

Agreed

R2-2409563	Correction on GNSS-AlmanacSupport and GNSS-UTC-ModelSupport in A-GNSS positioning	ZTE Corporation	CR	Rel-17	37.355	17.8.0	0517	1	A	NR_pos-Core	R2-2408214

Agreed as R2-2410907 (coversheet fixes)

R2-2410907	Correction on GNSS-AlmanacSupport and GNSS-UTC-ModelSupport in A-GNSS positioning	ZTE Corporation	CR	Rel-17	37.355	17.8.0	0517	2	A	LCS_NAVIC-Core

Agreed

R2-2409564	Correction on GNSS-AlmanacSupport and GNSS-UTC-ModelSupport in A-GNSS positioning	ZTE Corporation	CR	Rel-18	37.355	18.3.0	0518	1	A	NR_pos-Core	R2-2408215

Agreed as R2-2410908 (coversheet fixes)

R2-2410908	Correction on GNSS-AlmanacSupport and GNSS-UTC-ModelSupport in A-GNSS positioning	ZTE Corporation	CR	Rel-18	37.355	18.3.0	0518	2	A	LCS_NAVIC-Core

Agreed

### 5.3.1	Other

R2-2410236	Corection to high accuracy extended uncerntainty in QoS-r16	Huawei, HiSilicon	CR	Rel-16	37.355	16.13.0	0537	-	F	TEI16	Revised

Revised in R2-2410817

R2-2410817	Correction to high accuracy extended uncertainty in LCS QoS	Huawei, HiSilicon	CR	Rel-16	37.355	16.13.0	0537	1	F	TEI16	R2-2410236

Not pursued

Discussion:

vivo think the addition should be an NCE rather than in the parent IE, and it can be covered in the parent field description.

CATT think the CR is not needed because the QoS already requires the accuracy, and they understand that uncertainty information is not required in the request.

Qualcomm think this is NBC and based on a misunderstanding.  They see no relation between QoS and the GAD shapes; they understand that the extension becomes mandatory to avoid a new capability.

Huawei note that the location estimate is based on the uncertainty level, and with the HA enhancements, they do not see how the LMF can take the uncertainty into account properly.  Qualcomm think it is not related to the GAD shapes; we use the same encoding for convenience.

Huawei indicate that this is for alignment with SA2 spec, which in turn is aligned with OMA.

CATT wonder if Huawei see that the accuracy in the location already meets the requirement from SA2.  Huawei understand that it depends on the LCS service level, and our accuracy structure matches SA2.

Qualcomm indicate that the LMF cannot request the UE to report a particular GAD shape; the UE just fulfills the QoS as well as it can.

vivo support the CR in principle as an alignment with SA2; they understand that the SA2 spec distinguishes the two mapping tables for uncertainty.  Qualcomm think in this case there is no ambiguity because we only support the 7-bit shape.

R2-2410237	Correction to high accuracy extended uncertainty in LCS QoS	Huawei, HiSilicon	CR	Rel-17	37.355	17.8.0	0538	-	A	TEI16

Not pursued

R2-2410238	Correction to high accuracy extended uncertainty in LCS QoS	Huawei, HiSilicon	CR	Rel-18	37.355	18.3.0	0539	-	A	TEI16

Not pursued

R2-2409714	Correction on NavIC almanac set IE, and field descriptions under KlobucharModelParamater, UTC-ModelSet2, and GNSS-SystemTime.	Reliance Jio, MediaTek, Ericsson, Qualcomm Incorporated, CEWiT	CR	Rel-16	37.355	16.13.0	0531	-	F	LCS_NAVIC-Core

Remove the change to the UTC field descriptions

Coversheet to clarify that the changes are to align with the ICD

Check offline on the suffix of the extension field

Revised in R2-2410982

Discussion:

Nokia think the coversheet is not completely informative about the L5 inclination field that is added, and they do not understand the motivation for this part.  The rest seems somewhat editorial and fixing errors.

CATT think the UTC parameter field description change is not necessary because the references already indicate this dependency.

Lenovo indicate that there is a mistake on the coversheet: 6.5.2.10 is not changed; and in the ASN.1, for the new L5 field, the suffix should be -r16 (it is already correct in Rel-18).  Qualcomm understand that for a NCE, -v16xy would be correct.

Qualcomm agree with CATT’s comment.

Ericsson think on the coversheet we could clarify that the changes are to align with the ICD.

CEWiT are OK to take a revision and update the coversheet.  On the UTC field, they are OK with omitting the change.

CATT wonder if the ICD file is updated or the added IE was overlooked.  Reliance Jio indicate that it was just an oversight in implementing the existing ICD.

[AT128][401][POS] Revision of NavIC assistance data upates (CEWiT)

Scope: Check and update the CRs in R2-2409714 / R2-2410024 / R2-2410025 in accordance with online discussion.

Intended outcome: Agreed CRs (without CB if possible) in R2-2410982 / R2-2410983 / R2-2410984

Deadline: Wednesday 2024-11-20 1600 EST

R2-2410024	Correction on NavIC L5 almanac set IE, and field descriptions under KlobucharModelParamater, UTC-ModelSet2, and GNSS-SystemTime	Reliance Jio, MediaTek, Ericsson, Qualcomm Incorporated, CEWiT	CR	Rel-17	37.355	17.8.0	0533	-	A	LCS_NAVIC-Core

Revised in R2-2410983

R2-2410025	Correction on NavIC almanac set IE, and field descriptions under KlobucharModelParamater, UTC-ModelSet2, and GNSS-SystemTime.	Reliance Jio, MediaTek, Ericsson, Qualcomm Incorporated, CEWiT	CR	Rel-18	37.355	18.3.0	0534	-	A	LCS_NAVIC-Core

Revised in R2-2410984

R2-2410982	Correction on NavIC almanac set IE, and field descriptions under KlobucharModelParamater, UTC-ModelSet2, and GNSS-SystemTime.	Reliance Jio, MediaTek, Ericsson, Qualcomm Incorporated, CEWiT	CR	Rel-16	37.355	16.13.0	0531	1	F	LCS_NAVIC-Core

Revised in R2-2411186

R2-2410983	Correction on NavIC L5 almanac set IE, and field descriptions under KlobucharModelParamater, UTC-ModelSet2, and GNSS-SystemTime	Reliance Jio, MediaTek, Ericsson, Qualcomm Incorporated, CEWiT	CR	Rel-17	37.355	17.8.0	0533	1	A	LCS_NAVIC-Core

Revised in R2-2411187

R2-2410984	Correction on NavIC almanac set IE, and field descriptions under KlobucharModelParamater, UTC-ModelSet2, and GNSS-SystemTime.	Reliance Jio, MediaTek, Ericsson, Qualcomm Incorporated, CEWiT	CR	Rel-18	37.355	18.3.0	0534	1	A	LCS_NAVIC-Core

Revised in R2-2411188

R2-2411186	Correction on NavIC almanac set IE, and field descriptions under KlobucharModelParamater, UTC-ModelSet2, and GNSS-SystemTime.	Reliance Jio, MediaTek, Ericsson, Qualcomm Incorporated, CEWiT	CR	Rel-16	37.355	16.13.0	0531	2	F	LCS_NAVIC-Core

Revised in R2-2411206 (ASN.1 details and style issues identified in email discussion [AT128][401])

R2-2411187	Correction on NavIC L5 almanac set IE, and field descriptions under KlobucharModelParamater, UTC-ModelSet2, and GNSS-SystemTime	Reliance Jio, MediaTek, Ericsson, Qualcomm Incorporated, CEWiT	CR	Rel-17	37.355	17.8.0	0533	2	A	LCS_NAVIC-Core

Revised in R2-2411207 (ASN.1 details and style issues identified in email discussion [AT128][401])

R2-2411188	Correction on NavIC almanac set IE, and field descriptions under KlobucharModelParamater, UTC-ModelSet2, and GNSS-SystemTime.	Reliance Jio, MediaTek, Ericsson, Qualcomm Incorporated, CEWiT	CR	Rel-18	37.355	18.3.0	0534	2	A	LCS_NAVIC-Core

Revised in R2-2411208 (ASN.1 details and style issues identified in email discussion [AT128][401])

R2-2411206	Correction on NavIC almanac set IE, and field descriptions under KlobucharModelParamater, UTC-ModelSet2, and GNSS-SystemTime.	Reliance Jio, CEWiT, MediaTek, Ericsson, Qualcomm, CATT, Nokia	CR	Rel-16	37.355	16.13.0	0531	3	F	LCS_NAVIC-Core

Agreed (email discussion [AT128][401])

R2-2411207	Correction on NavIC L5 almanac set IE, and field descriptions under KlobucharModelParamater, UTC-ModelSet2, and GNSS-SystemTime	Reliance Jio, CEWiT, MediaTek, Ericsson, Qualcomm, CATT, Nokia	CR	Rel-17	37.355	17.8.0	0533	3	A	LCS_NAVIC-Core

Agreed (email discussion [AT128][401])

R2-2411208	Correction on NavIC almanac set IE, and field descriptions under KlobucharModelParamater, UTC-ModelSet2, and GNSS-SystemTime.	Reliance Jio, CEWiT, MediaTek, Ericsson, Qualcomm, CATT, Nokia	CR	Rel-18	37.355	18.3.0	0534	3	A	LCS_NAVIC-Core

Agreed (email discussion [AT128][401])

R2-2410821	Presence of ValueTag and ExpirationTime when posSIBs are segmented	Ericsson	CR	Rel-15	37.355	15.3.0	0541	-	F	NR_pos-Core	Revised

Revised in R2-2410873

R2-2410873	Presence of ValueTag and ExpirationTime when posSIBs are segmented	Ericsson	CR	Rel-15	37.355	15.3.0	0541	1	F	NR_newRAT-Core	R2-2410821

Revised in R2-2411065

R2-2411065	Presence of ValueTag and ExpirationTime when posSIBs are segmented	Ericsson, MediaTek Inc.	CR	Rel-15	37.355	15.3.0	0541	1	F	NR_newRAT-Core	R2-2410821

Not pursued

Discussion:

CATT think the second and third corrections are not needed; they understand that the valueTag will be there in every segment unless the content changes frequently, and the third change (field description) is gNB implementation.

ZTE think the added note is correct as far as describing UE behaviour, but if the valueTag is not present, it should also be understood to mean frequent change, and the UE should not accumulate segments with no valueTag at all.

Huawei think the first change (description) is correct but not essential, and the second change (note) is just one possible implementation; they understand the issue, but they think the expirationTime can also be used to avoid the same issue from an implementation pov, so they understand that it is up to the gNB implementation.

Nokia agree with Huawei; segmentation and reassembly is up to network implementation, and they think we could address any specific interoperability issues but we do not need a general guideline.

Qualcomm think the CR is not consistent; the note somewhat implies that the valueTag must be present in each segment, and the field description only says it needs to be there in the first segment.  They think the expirationTime has been lost in the evolution of the CR, and that may be the field that is only needed in the first segment.  They do not see a need to fix it from Rel-15, especially since the UE will fail decoding if it assembles segments with different valueTags.

Ericsson agree with CATT that we have guidance that for rapidly changing posSIBs, the valueTag and expirationTime are not applicable, but from UE perspective they think it can see segments with different or missing valueTags, irrespective of how fast the content changes.  They agree that expirationTime is another indication of mismatched segments, but it should not be expected to be there in every segment.

MediaTek wonder what the space of network implementations is: Can the network segment a posSIB with no value tag, and the UE just has to rely on decoding failure?

vivo think the UE can detect the change based on the segment number.

Ericsson think the segment number is not enough, but it may be necessary to show an example.

CATT think there are two situations: the posSIB does not change frequently, and the posSIB changes frequently with each broadcast interval.  For the first situation, they understand that the description is clear enough that the gNB should set the valueTag always, and the valueTag should be increased when the content changes; the UE should not mix segments from different valueTags.  For the second situation, if the posSIB changes in each broadcast interval, segmentation does not work.

Qualcomm think segmentation with rapid change is possible if the valueTag is included, and they do not see where the specs are broken.

Nokia think adding more implementation detail into the spec opens a can of worms.

Huawei wonder if the proposal is the only method to avoid the issue; they think the network can also use the expirationTime, e.g., for RTK where the delivery of AD is highly predictable.

CATT understand that according to the existing mechanism, the valueTag will already prevent the UE from having this problem, but for the expirationTime they do not see a connection.

R2-2410822	Presence of ValueTag and ExpirationTime when posSIBs are segmented	Ericsson	CR	Rel-16	37.355	16.13.0	0542	-	A	NR_newRAT-Core

Revised in R2-2411062

R2-2411062	Presence of ValueTag and ExpirationTime when posSIBs are segmented	Ericsson, MediaTek Inc.	CR	Rel-16	37.355	16.13.0	0542	-	A	NR_newRAT-Core

Not pursued

R2-2410823	Presence of ValueTag and ExpirationTime when posSIBs are segmented	Ericsson	CR	Rel-17	37.355	17.8.0	0543	-	A	NR_newRAT-Core

Revised in R2-2411063

R2-2411063	Presence of ValueTag and ExpirationTime when posSIBs are segmented	Ericsson, MediaTek Inc.	CR	Rel-17	37.355	17.8.0	0543	-	A	NR_newRAT-Core

Not pursued

R2-2410824	Presence of ValueTag and ExpirationTime when posSIBs are segmented	Ericsson	CR	Rel-18	37.355	18.3.0	0544	-	F	NR_newRAT-Core	Revised

Revised in R2-2410869

R2-2410869	Presence of ValueTag and ExpirationTime when posSIBs are segmented	Ericsson	CR	Rel-18	37.355	18.3.0	0544	1	A	NR_newRAT-Core	R2-2410824

Revised in R2-2411064

R2-2411064	Presence of ValueTag and ExpirationTime when posSIBs are segmented	Ericsson, MediaTek Inc.	CR	Rel-18	37.355	18.3.0	0544	1	A	NR_newRAT-Core	R2-2410824

Not pursued

# 6	NR Rel-17

Essential corrections only.  Editorial/clarifications should be sent to be reviewed and approved by spec rapporteurs prior to submission.  Editorials should only be submitted by spec rapporteurs.

Tdoc limitation: 4 Tdocs

## 6.1	Common

(NR_MG_enh-Core; leading WG: RAN4; REL-17; WID: RP-211591)

(NR_UDC_enh-Core; leading WG: RAN2; REL-17; WID: RP-211203)

(NG_RAN_PRN_enh-Core; leading WG: RAN3; REL-17; WID: RP-202363)

(NR_IAB_enh-Core; leading WG: RAN2; REL-17; WID: RP-211548)

(NR_UE_pow_sav_enh-Core; leading WG: RAN2; REL-17; WID: RP-212630)

(LTE_NR_DC_enh2-Core; leading WG: RAN2; REL-17; WID: RP-201040)

(LTE_NR_MUSIM-Core; leading WG: RAN2; REL-17; WID: RP-212610)

(NR_Slice -Core; leading WG: RAN2; REL-17; WID: RP-212534)

(NR_QoE-Core; leading WG: RAN3; REL-17; WID: RP-211406)

(NR_ext_to_71GHz-Core; leading WG: RAN1; REL-17; WID: RP-212637)

(NR_cov_enh-Core; leading WG: RAN1; REL-17; WID: RP-211566): non-RACH-indication parts

(NR_redcap-Core; leading WG: RAN1; REL-17; WID: RP-211574)

(NR_feMIMO-Core; leading WG: RAN1; REL-17; WID: RP-212535)

(NR_SmallData_INACTIVE-Core, leading WG: RAN2; REL-17; WID: RP-212594)

(NR_IIOT_URLLC_enh-Core; leading WG: RAN2; REL-17; WID: RP-210854)

(NR_MBS-Core; leading WG: RAN2; REL-17; WID: RP-201038)

(NR_ENDC_SON_MDT_enh-Core; leading WG: RAN3; REL-17; WID: RP-201281)

(NR_NTN_solutions-Core; leading WG: RAN2; REL-17; WID: RP-211557)

PRACH partitioning items

(NR TEI17)

Includes Rel-17 Work Items without specific R2 Agenda Item, e.g. RAN1 and RAN4 led items, SA2 and CT1 led items (was previously “Rel-17 Other”)

Includes aspects that does not fit under the more specific AIs, e.g. multi-WI aspects.

Corrections for NR_NTN_solutions-Core might be treated in the NTN breakout session.

### 6.1.1	Stage 2 and Organisational

Incoming LSs, etc. You should discuss your stage 2 CRs with the specification rapporteurs before submission. Includes impact to 38.300, 37.340, (36.300 if applicable)

msg3-TransformPrecoding

R2-2409515	LS on waveform determination for PUSCH scheduled by fallbackRAR UL grant and MsgA PUSCH (R1-2409301; contact: CATT)	RAN1	LS in	Rel-17	NR_redcap-Core, NR_SmallData_INACTIVE-Core, NR_cov_enh-Core, NR_slice-Core	To:RAN2

Alternatives listed in the LS:

  - Alt 1: It is always msg3-transformPrecoder in rach-ConfigCommon included directly within BWP configuration.

  - Alt 2:

    - For msgA-ConfigCommon included directly within BWP configuration, it is msg3-transformPrecoder in rach-ConfigCommon included directly within BWP configuration.

    - For msgA-ConfigCommon included in additionalRACH-ConfigList, it is

msg3-transformPrecoder included in the same AdditionalRACH-Config.

R2-2409602	Contact company’s input on RAN1 LS in R2-2409515	CATT	discussion	NR_SmallData_INACTIVE-Core, NR_cov_enh-Core, NR_redcap-Core, NR_slice-Core

Proposal 1a: RAN2 confirms that Alt.2 in RAN1 LS [1] is the intended UE behavior on which msg3-TRanformPrecoder to apply for “MsgA PUSCH transmission when msgA-TransformPrecoder is not configured”.

Proposal 1b: RAN2 confirms that Alt.2 in RAN1 LS [1] is the intended UE behavior on which msg3-tranformPrecoder to apply for “PUSCH scheduled by fallbackRAR UL grant”.

Proposal 2a: Add the intended UE behavior in Proposal 1a/1b into the field descriptions of msgA-transformPrecoder/msg3-transformPrecoder.

Proposal 2b: Agree the CRs in [3][4] for TS 38.331.

Proposal 3: RAN2 approves the LS response to RAN1 with below answers provided:

	For Q1: inform RAN1 that from RAN2 perspective Alt.2 reflects the intended UE behavior on which msg3-transformPrecoder to apply, both for PUSCH scheduled by fallbackRAR UL grant and for MsgA PUSCH when msgA-TransformPrecoder is not configured.

	For Q2: inform RAN1 that RAN2 agrees the RRC CRs to capture the intended UE behavior in the field descriptions.

-	Ericsson agrees with Alt2 but think that we should only reply this to RAN1 and RAN1 will tell us if/when we should do something. LG want to only reply that RAN2 understanding is Alt2. CATT think we need to update RRC. Vivo agrees with CATT that we should update the specs now. Qualcomm does not want to do changes to the RAN2 specs. Ericsson highlights that the RAN1 LS indicates that the discussion is still ongoing in RAN1.

Reply to RAN1 as follows:

For Q1: RAN2 confirms that Alt.2 in RAN1 LS [1] is the intended UE behavior on which msg3-TRanformPrecoder to apply for “MsgA PUSCH transmission when msgA-TransformPrecoder is not configured” and for "PUSCH transmission scheduled by fallback RAR UL grant".

For Q2: Reply that current RAN2 specification already suggests Alt2. RAN2 does not preclude doing clarifications later if deemed necessary.

[AT128][609][Maint] Reply LS on waveform determination (CATT)

Scope:

    - Produce approvable LS

Intended outcome:

    - Approved LS in R2-2411031

Deadline:

    - Wednesday 17:00. The intention is to approve the LS over email.

R2-2411031	LS response on waveform determination for PUSCH scheduled by fallbackRAR UL grant and MsgA PUSCH	RAN2	LS out	Rel-17	NR_redcap-Core, NR_SmallData_INACTIVE-Core, NR_cov_enh-Core, NR_slice-Core	To:RAN1

Approved

R2-2409603	CR on the usage of msg3-transmitPrecoder	CATT	CR	Rel-17	38.331	17.10.0	5099	-	F	NR_redcap-Core, NR_SmallData_INACTIVE-Core, NR_cov_enh-Core, NR_slice-Core

R2-2409604	CR on the usage of msg3-transmitPrecoder	CATT	CR	Rel-18	38.331	18.3.0	5100	-	A	NR_redcap-Core, NR_SmallData_INACTIVE-Core, NR_cov_enh-Core, NR_slice-Core

R2-2410874	Discussion on RAN1 LS on waveform determination for PUSCH scheduled by fallbackRAR UL grant and MsgA PUSCH	Ericsson	discussion	Rel-17	NR_cov_enh-Core

Moved from 6.1

R2-2410818	Discussion on RAN1 LS on waveform determination	LG Electronics Inc.	discussion	Rel-17	NR_redcap-Core, NR_SmallData_INACTIVE-Core, NR_cov_enh-Core, NR_slice-Core

Moved from 6.1.1.1

#### 6.1.1.0	In-principle agreed CRs

R2-2409600	Correction on location based measurements in NR NTN	CATT, Nokia, Nokia Shanghai Bell	CR	Rel-17	38.300	17.10.0	0909	1	F	NR_NTN_solutions-Core	R2-2408366

R2-2409601	Correction on location based measurements in NR NTN	CATT, Nokia, Nokia Shanghai Bell	CR	Rel-18	38.300	18.3.0	0910	1	A	NR_NTN_solutions-Core	R2-2408367

R2-2409806	Clarification on MII usage	Samsung, Nokia	CR	Rel-17	38.300	17.10.0	0924	1	F	NR_MBS-Core	R2-2409276

R2-2409807	Clarification on MII usage	Samsung, Nokia	CR	Rel-18	38.300	18.3.0	0925	1	A	NR_MBS-Core	R2-2409277

All 4 agreed

#### 6.1.1.1	Other

R2-2410807	Correction on TDD support in NTN	Huawei, HiSilicon	CR	Rel-17	38.300	17.10.0	0942	-	F	NR_NTN_solutions-Core

- QC thinks we should also clarify that CA/DC is not supported

- Ericsson thinks this is already clear as there are no TDD bands for this so far. Nokia agrees

- NEC thinks it makes sense to align to the description for IoT NTN

- CATT supports the CR

Agreed

R2-2410808	Correction on TDD support in NTN	Huawei, HiSilicon	CR	Rel-18	38.300	18.3.0	0943	-	A	NR_NTN_solutions-Core

Agreed

QoE

R2-2410131	Miscellaneous corrections on stage 2 description of QMC	Lenovo	CR	Rel-17	38.300	17.10.0	0930	-	F	NR_QoE-Core

-	Ericsson has a bunch of minor comments.

To be updated offline

[AT128][610][Maint] Misc QoE corrections (Lenovo)

Scope:

    - Produce agreeable CR

Intended outcome:

    - Agreed CR in R2-2411032

Deadline:

    - Wednesday 17:00. The intention is to agree the CRs over email.

R2-2411032	Miscellaneous corrections on stage 2 description of QMC	Lenovo	CR	Rel-17	38.300	17.10.0	0930	-	F	NR_QoE-Core	R2-2410131

Agreed

FeMIMO

R2-2410195	CR on TCI state indication of CORESET#0	OPPO	CR	Rel-17	38.321	17.10.0	1994	-	F	NR_FeMIMO-Core

R2-2410196	CR on TCI state indication of CORESET#0	OPPO	CR	Rel-18	38.321	18.3.0	1995	-	A	NR_FeMIMO-Core

R2-2410511	PHR for serving cells configured with multiple TRP PUSCH repetition in NR-DC	Huawei, HiSilicon	CR	Rel-17	38.321	17.10.0	2005	-	F	NR_FeMIMO-Core

R2-2410512	PHR for serving cells configured with multiple TRP PUSCH repetition in NR-DC	Huawei, HiSilicon	CR	Rel-18	38.321	18.3.0	2006	-	A	NR_FeMIMO-Core

The 4 above were already treated in the main session

Misc

R2-2409999	Miscellaneous non-controversial corrections Set XXIII	Ericsson	CR	Rel-17	38.331	17.10.0	5082	1	F	NR_newRAT-Core, TEI17	R2-2409124

To be further checked in post meeting email disc [POST128][004]

### 6.1.2	User Plane corrections

User Plane Related aspects will be handled in the User Plane break out session. (exception: TEI new proposals if any).

#### 6.1.2.0	In-principle agreed CRs

R2-2409609	Correction on PHR for mTRP PUSCH repetition	LG Electronics Inc., MediaTek Inc., CATT, Ericsson	CR	Rel-17	38.321	17.10.0	1892	4	F	NR_FeMIMO-Core	R2-2409043

R2-2411068	Correction on PHR for mTRP PUSCH repetition	LG Electronics Inc., MediaTek Inc., CATT, Ericsson	CR	Rel-17	38.321	17.10.0	1892	5	F	NR_FeMIMO-Core

Add architecture option

The CR is agreed in R2-2411095

R2-2409610	Correction on PHR for mTRP PUSCH repetition (R17)	LG Electronics Inc., MediaTek Inc., CATT, Ericsson	CR	Rel-17	38.331	17.10.0	5068	1	F	NR_FeMIMO-Core	R2-2409044

R2-2411069	Correction on PHR for mTRP PUSCH repetition (R17)	LG Electronics Inc., MediaTek Inc., CATT, Ericsson	CR	Rel-17	38.331	17.10.0	5068	2	F	NR_FeMIMO-Core

Add architecture option

The CR is agreed in R2-2411096

R2-2409611	Correction on PHR for mTRP PUSCH repetition	LG Electronics Inc., MediaTek Inc., CATT, Ericsson	CR	Rel-17	38.306	17.10.0	1190	1	F	NR_FeMIMO-Core	R2-2409045

R2-2411070	Correction on PHR for mTRP PUSCH repetition	LG Electronics Inc., MediaTek Inc., CATT, Ericsson	CR	Rel-17	38.306	17.10.0	1190	2	F	NR_FeMIMO-Core

Add architecture option

The CR is agreed in R2-2411097

R2-2409612	Correction on PHR for mTRP PUSCH repetition (R18)	LG Electronics Inc., MediaTek Inc., CATT, Ericsson	CR	Rel-18	38.321	18.3.0	1893	3	A	NR_FeMIMO-Core	R2-2409046

R2-2411071	Correction on PHR for mTRP PUSCH repetition (R18)	LG Electronics Inc., MediaTek Inc., CATT, Ericsson	CR	Rel-18	38.321	18.3.0	1893	4	A	NR_FeMIMO-Core

Add architecture option

The CR is agreed in R2-2411098

R2-2409613	Correction on PHR for mTRP PUSCH repetition (R18)	LG Electronics Inc., MediaTek Inc., CATT, Ericsson	CR	Rel-18	38.331	18.3.0	5069	1	A	NR_FeMIMO-Core	R2-2409047

R2-2411072	Correction on PHR for mTRP PUSCH repetition (R18)	LG Electronics Inc., MediaTek Inc., CATT, Ericsson	CR	Rel-18	38.331	18.3.0	5069	2	A	NR_FeMIMO-Core

Add architecture option

The CR is agreed in R2-2411099

R2-2409614	Correction on PHR for mTRP PUSCH repetition (R18)	LG Electronics Inc., MediaTek Inc., CATT, Ericsson	CR	Rel-18	38.306	18.3.0	1191	1	A	NR_FeMIMO-Core	R2-2409048

R2-2411073	Correction on PHR for mTRP PUSCH repetition (R18)	LG Electronics Inc., MediaTek Inc., CATT, Ericsson	CR	Rel-18	38.306	18.3.0	1191	2	A	NR_FeMIMO-Core

Add architecture option

The CR is agreed in R2-2411100

R2-2409698	Corrections to validity of configured uplink grant for SDT	Samsung, LG Electronics, Sony	CR	Rel-17	38.321	17.10.0	1956	2	F	NR_SmallData_INACTIVE-Core	R2-2409220

The CR is agreed

R2-2409804	Correction on HARQ process	Samsung, Ericsson, Nokia, LG Electronics Inc., Qualcomm Incorporated, Apple, ZTE, CATT	CR	Rel-17	38.321	17.10.0	1937	1	F	NR_NTN_solutions-Core, NR_MBS-Core	R2-2408467

The CR is agreed

R2-2409805	Correction on HARQ process	Samsung, Ericsson, Nokia, LG Electronics Inc., Qualcomm Incorporated, Apple, ZTE, CATT	CR	Rel-18	38.321	18.3.0	1938	1	A	NR_NTN_solutions-Core, NR_MBS-Core	R2-2408468

The CR is agreed

R2-2410455	Correction on use of recommended of IAB-MT beam indication	Ericsson, Samsung	CR	Rel-17	38.321	17.10.0	1901	3	F	NR_IAB_enh-Core	R2-2409219

The CR is agreed

R2-2410456	Correction on use of recommended of IAB-MT beam indication	Ericsson, Samsung	CR	Rel-18	38.321	18.3.0	1902	2	A	NR_IAB_enh-Core	R2-2408814

The CR is agreed

R2-2410511	PHR for serving cells configured with multiple TRP PUSCH repetition in NR-DC	Huawei, HiSilicon	CR	Rel-17	38.321	17.10.0	2005	-	F	NR_FeMIMO-Core

-	Ericsson and Qualcomm support the CR

Remove “is configured” from text

The CR is agreed in R2-2411091 with change above

R2-2410512	PHR for serving cells configured with multiple TRP PUSCH repetition in NR-DC	Huawei, HiSilicon	CR	Rel-18	38.321	18.3.0	2006	-	A	NR_FeMIMO-Core

Remove “is configured” from text

The CR is agreed in R2-2411092 with change above

#### 6.1.2.1	Other

R2-2409605	Discussion on remaining issues for one-shot HARQ Feedback in NR NTN	CATT	discussion

Proposal 1: RAN2 confirms that one-shot HARQ feedback feature can be supported in NR NTN since Rel-17 (based on RAN1 specification and related RAN1-introduced UE capabilities).

R2-2410879	NTN DRX Timer Handling and URLLC One-Shot HARQ Feedback	Sharp	discussion	Rel-17	NR_NTN_solutions-Core, NR_IIOT_URLLC_enh-Core

Proposal 1	RAN2 to down-select one option for this issue:

a.	Specify that one-shot HARQ feedback is not supported when downlinkHARQ-FeedbackDisabled is configured.

b.	Fix the problem with TP in Section 5.

Discussion

-	Qualcomm indicates that RAN1 didn’t agree and are concerned with the changes as it shouldn’t introduce NBC changes.  Can only agree if we make it clear that only PDCCH indicates one shot feedback.

-	CATT indicates that RAN1 explicitly agreed.  Sharp agrees with the observation but whether we change the RAN2 spec is another discussion, so it may be late to make any change.

-	Vivo thinks that we never had a discussion whether there is a motivation for this and if we do any changes it will cause some backward compatibility issue.  We can do this in TEI if needed.

-	LG and Ericsson agrees with CATT.  We can fix the problem from Rel-18.   There is currently no restriction.

-	CATT asks what is the consequence if we don’t support it in RAN2 but RAN1 accepts it.

Discuss offline if there is a way to support this in a BC way and if it is acceptable

[AT128][006][UP] NTN and on shot feedback  (CATT)

Intended outcome: discuss how to capture it in a BC way and agree to CR (if agreable)

Deadline:  Thursday

R2-2411117	Report of [AT128][006][UP] NTN and one shot feedback (CATT)	CATT	discussion	NR_NTN_solutions-Core

No new UE capability is introduced

Noted

R2-2411118	Correction on HARQ-RTT-TimerDL-NTN in NR NTN	CATT, LG Electronics Inc., Sharp	CR	Rel-17	38.321	17.10.0	2019	-	F	NR_NTN_solutions-Core

[NBC CR]

The CR is agreed

R2-2411119	Correction on HARQ-RTT-TimerDL-NTN in NR NTN	CATT, LG Electronics Inc., Sharp	CR	Rel-18	38.321	18.3.0	2020	-	A	NR_NTN_solutions-Core

The CR is agreed

R2-2409699	Corrections to validity of configured uplink grant for SDT	Samsung, LG Electronics, Sony	CR	Rel-18	38.321	18.3.0	1979	-	A	NR_SmallData_INACTIVE-Core

The CR is agreed

R2-2409832	Correction on the field name of candidate beam list in BFR MAC CE	Samsung	CR	Rel-17	38.321	17.10.0	1980	-	F	NR_FeMIMO-Core

-	Huawei and Nokia think that this is more of an editorial so we should soften the impact analysis.

Update impact analysis to soften it given it is editorial

The CR is agreed in R2-2411089 with change above

R2-2409833	Correction on the field name of candidate beam list in BFR MAC CE	Samsung	CR	Rel-18	38.321	18.3.0	1981	-	A	NR_FeMIMO-Core

Update impact analysis to soften it given it is editorial

The CR is agreed in R2-2411090 with change above

R2-2410195	CR on TCI state indication of CORESET#0	OPPO	CR	Rel-17	38.321	17.10.0	1994	-	F	NR_FeMIMO-Core

-	Nokia, Huawei agrees with intention but he change is incomplete and doesn’t include the unified TCI framework. Oppo thinks this is not applicable to it so it was intentionally omitted.

Update variables in italics font

=> Revised in R2-2411173

R2-2411173	CR on TCI state indication of CORESET#0	OPPO	CR	Rel-17	38.321	17.10.0	1994	1	F	NR_FeMIMO-Core

Companies should have a quick chat between RAN1 and RAN2 delegates

The CR is revised R2-2411214

R2-2411214	CR on TCI state indication of CORESET#0	OPPO	CR	Rel-17	38.321	17.10.0	1994	2	F	NR_FeMIMO-Core

Remove tracked changes from cover page

The CR is agreed with changes above in R2-2411216

R2-2411216	CR on TCI state indication of CORESET#0	OPPO	CR	Rel-17	38.321	17.10.0	1994	3	F	NR_FeMIMO-Core

=> Agreed

R2-2410196	CR on TCI state indication of CORESET#0	OPPO	CR	Rel-18	38.321	18.3.0	1995	-	A	NR_FeMIMO-Core

=> Revised in R2-2411174

R2-2411174	CR on TCI state indication of CORESET#0	OPPO	CR	Rel-18	38.321	18.3.0	1995	1	A	NR_FeMIMO-Core

The CR is revised R2-2411215

R2-2411215	CR on TCI state indication of CORESET#0	OPPO	CR	Rel-18	38.321	18.3.0	1995	2	A	NR_FeMIMO-Core

Remove tracked changes from cover page

The CR is agreed with changes above in R2-2411217

R2-2411217	CR on TCI state indication of CORESET#0	OPPO	CR	Rel-18	38.321	18.3.0	1995	3	A	NR_FeMIMO-Core

=> Agreed

R2-2410720	Correction on drx HARQ RTT timer in NTN	LG Electronics Inc.	CR	Rel-17	38.321	17.10.0	1967	1	F	NR_NTN_solutions-Core	R2-2409123

Not treated

R2-2410722	Correction on drx HARQ RTT timer in NTN	LG Electronics Inc.	CR	Rel-18	38.321	18.3.0	1966	1	A	NR_NTN_solutions-Core	R2-2409121

### 6.1.3	Control Plane corrections

#### 6.1.3.0	In-principle agreed CRs

R2-2409808	Clarification on determining about to start sessions	Samsung, Nokia	CR	Rel-17	38.331	17.10.0	5092	1	F	NR_MBS-Core	R2-2409278

R2-2409809	Clarification on determining about to start sessions	Samsung, Nokia	CR	Rel-18	38.331	18.3.0	5093	1	A	NR_MBS-Core	R2-2409279

R2-2409914	Correction to unified TCI signalling	MediaTek Inc.	CR	Rel-17	38.331	17.10.0	4997	1	F	NR_FeMIMO-Core	R2-2408323

R2-2409915	Correction to unified TCI signalling	MediaTek Inc.	CR	Rel-18	38.331	18.3.0	4998	1	A	NR_FeMIMO-Core	R2-2408324

R2-2409919	Correction on UE behavior of setting failedPSCellId	NTTDOCOMO, INC.	CR	Rel-17	38.331	17.10.0	5009	1	F	NR_ENDC_SON_MDT_enh-Core	R2-2408440

R2-2409920	Correction on UE behavior of setting failedPSCellId	NTTDOCOMO, INC.	CR	Rel-18	38.331	18.3.0	5010	1	A	NR_ENDC_SON_MDT_enh-Core	R2-2408441

R2-2410417	Corrections to NR NTN (R17)	Huawei, HiSilicon, CMCC, Sequans Communications	CR	Rel-17	38.331	17.10.0	5075	2	F	NR_NTN_solutions-Core	R2-2409238

R2-2410418	Corrections to NR NTN (R18)	Huawei, HiSilicon, CMCC, Sequans Communications	CR	Rel-18	38.331	18.3.0	5076	2	A	NR_NTN_solutions-Core	R2-2409240

R2-2410533	Correction for CFRA configuration due to PRACH partitioning	Huawei, HiSilicon, ZTE Corporation, Ericsson, Mediatek	CR	Rel-17	38.331	17.10.0	4899	2	F	NR_redcap-Core, NR_slice-Core	R2-2408730

R2-2410534	Correction for CFRA configuration due to PRACH partitioning	Huawei, HiSilicon, ZTE Corporation, Ericsson, Mediatek	CR	Rel-18	38.331	18.3.0	4900	2	A	NR_redcap-Core, NR_redcap_enh-Core, NR_slice-Core	R2-2408731

R2-2410650	Correction on IE perRAInfoList for SCGFailureInformation	Huawei, HiSilicon	CR	Rel-17	38.331	17.10.0	5044	2	F	NR_ENDC_SON_MDT_enh-Core	R2-2409298

R2-2410651	Correction on IE perRAInfoList for SCGFailureInformation	Huawei, HiSilicon	CR	Rel-18	38.331	18.3.0	5045	2	A	NR_ENDC_SON_MDT_enh-Core	R2-2409299

R2-2410835	Correction on UE capabilities for TCI state indication	Huawei, HiSilicon	CR	Rel-17	38.331	17.10.0	5014	2	F	NR_FeMIMO-Core	R2-2409309

R2-2410836	Correction on UE capabilities for TCI state indication	Huawei, HiSilicon	CR	Rel-18	38.331	18.3.0	5015	2	A	NR_FeMIMO-Core	R2-2409310

R2-2410837	Correction on UE capabilities for TCI state indication	Huawei, HiSilicon	CR	Rel-17	38.306	17.10.0	1175	2	F	NR_FeMIMO-Core	R2-2409311

R2-2410909	Correction on UE capabilities for TCI state indication	Huawei, HiSilicon	CR	Rel-18	38.306	18.3.0	1176	3	A	NR_FeMIMO-Core	R2-2410838

All above are agreed, but then R2-2410534 was coversheet revised by MCC in R2-2411264: cat A CR must have the same WIs as the corresponding cat F CR.

R2-2411264	Correction for CFRA configuration due to PRACH partitioning	Huawei, HiSilicon, ZTE Corporation, Ericsson, Mediatek	CR	Rel-18	38.331	18.3.0	4900	3	A	NR_redcap-Core, NR_slice-Core

=> Agreed

Updated compared to the IPA

R2-2411037	Corrections on measurement gap configuration	ZTE Corporation, Sanechips	CR	Rel-17	38.331	17.10.0	5024	3	F	NR_NTN_solutions-Core, NR_redcap-Core	R2-2410917

R2-2411038	Corrections on measurement gap configuration	ZTE Corporation, Sanechips	CR	Rel-18	38.331	18.3.0	5025	2	A	NR_NTN_solutions-Core, NR_redcap-Core	R2-2410361

R2-2410893	Clarification of MeasurementTimingConfiguration use	Vodafone,Ericsson,Nokia	CR	Rel-18	38.331	18.3.0	5016	2	F	NR_BWP_wor-Core	R2-2409932

The 3 above are agreed

Old revisions

R2-2410360	Corrections on measurement gap configuration	ZTE Corporation, Sanechips	CR	Rel-17	38.331	17.10.0	5024	1	F	NR_NTN_solutions-Core, NR_redcap-Core	R2-2408651

R2-2410838	Correction on UE capabilities for TCI state indication	Huawei, HiSilicon	CR	Rel-18	38.306	18.3.0	1176	2	A	NR_FeMIMO-Core	R2-2409312

R2-2409932	Clarification of MeasurementTimingConfiguration use	Vodafone,Ericsson,Nokia	CR	Rel-18	38.331	18.3.0	5016	1	F	NR_BWP_wor-Core	R2-2408506

Moved from 6.1.3

R2-2410917	Corrections on measurement gap configuration	ZTE Corporation, Sanechips	CR	Rel-17	38.331	17.10.0	5024	2	F	NR_NTN_solutions-Core, NR_redcap-Core	R2-2410360

R2-2410361	Corrections on measurement gap configuration	ZTE Corporation, Sanechips	CR	Rel-18	38.331	18.3.0	5025	1	A	NR_NTN_solutions-Core, NR_redcap-Core	R2-2408652

#### 6.1.3.1	NR RRC

Corrections to 38331, and related change to other TS if applicable, except UE caps.

R2-2409781	Corrections on UL polarization parameters in NR NTN	CATT	CR	Rel-17	38.331	17.10.0	5117	-	F	NR_NTN_solutions-Core

- LG supports the CR

- Ericsson does not support the CR and thinks this is due to some RAN1 agreement. CATT thinks the RAN1 agreement is for DL polarization, but this CR is about UL. ZTE agrees with Ericsson

- QC agrees with the intention but thinks we can add some text about NW behaviour

- Samsung also agrees with the intention but thinks nothing is really broken even if we don’t change anything

- HW thinks we need to change at least the need code. Ericsson agrees but thinks this could go to the rapporteur CR

Not pursued

RAN2 understands that UL polarization is not needed for neighbour cells for RRM measurements

We change the need code on ntn-PolarizationUL-r17 in MeasObjectNR from Need R to Need S (to be captured in the RRC rapporteur CR)

R2-2409782	Corrections on UL polarization parameters in NR NTN	CATT	CR	Rel-18	38.331	18.3.0	5118	-	A	NR_NTN_solutions-Core

Not pursued

R2-2410028	Correction on NTN in FR1-NTN Bands	vivo	CR	Rel-17	38.331	17.10.0	5132	-	F	NR_NTN_solutions-Core

- Ericsson thinks this should also go into the RRC rapporteur CR. Nokia agrees but also thinks that additional details should be added

- Vivo thinks we could also remove the reference to the specific table

Not pursued

Remove the reference to the specific table (correction to be included in the RRC rapporteur CR)

R2-2410029	Correction on NTN in FR1-NTN Bands	vivo	CR	Rel-18	38.331	18.3.0	5133	-	A	NR_NTN_solutions-Core

Not pursued

R2-2410362	Corrections to NTN SMTC configuration	ZTE Corporation, Sanechips	CR	Rel-17	38.331	17.10.0	5159	-	F	NR_NTN_solutions-Core

- LG supports the CR

- Vivo agrees with the intention but thinks we should revise the actual change

- QC thinks this is not needed

- CATT agrees with the CR but thinks that in idle mode is up to UE implementation

- Ericsson thinks that if we need to clarify something for idle mode it should go into the idle mode spec

- ZTE reports that different companies have different understanding on the UE behaviour regarding RAN4 requirements and suggests to either check internally with RAN4 colleagues first or send an LS to RAN4

- QC thinks we can do an internal check first. Ericsson agrees an if needed we can send an LS from the next meeting

Postponed

R2-2410363	Corrections to NTN SMTC configuration	ZTE Corporation, Sanechips	CR	Rel-18	38.331	18.3.0	5160	-	A	NR_NTN_solutions-Core

Postponed

Max nrof SI messages

R2-2409662	Correction of field description si-broadcastStatus and posSI-broadcast status to reflect maxSI message (Rel-17)	Xiaomi, Ericsson, LG electronics, Philips International B.V., Qualcomm Incorporated, ZTE Corporation	CR	Rel-17	38.331	17.10.0	5107	-	F	TEI17

R2-2409663	Correction of field description si-broadcastStatus and posSI-broadcast status to reflect maxSI message (Rel-18)	Xiaomi, Ericsson, LG electronics, Philips International B.V., Qualcomm Incorporated, ZTE Corporation	CR	Rel-18	38.331	18.3.0	5108	-	A	TEI18

-	Lenovo think the change is not complete because it only applies to Msg1-based but should apply also to Msg3-based SI request and this needs to be fixed. Docomo think that if we remove “when posSI-RequestConfig or posSI-RequestConfigRedCap or posSI-RequestConfigSUL is configured”, which is suggested by Lenovo then the whole field description may need updating, and also other FDs need updating.

We will update the CRs offline

[AT128][611][Maint] Description of si-broadcastStatus (Xiaomi)

Scope:

    - Produce agreeable CRs

Intended outcome:

    - Agreed CRs in R2-2411033 and R2-2411034

Deadline:

    - Wednesday 17:00. The intention is to agree the CRs over email.

R2-2411033	Correction of maxSI messages [SI-SCHEDULING]	Xiaomi, Ericsson, LG electronics, Philips International B.V., Qualcomm Incorporated, ZTE Corporation	CR	Rel-17	38.331	17.10.0	5107	1	F	TEI17

R2-2411034	Correction of maxSI messages [SI-SCHEDULING]	Xiaomi, Ericsson, LG electronics, Philips International B.V., Qualcomm Incorporated, ZTE Corporation	CR	Rel-18	38.331	18.3.0	5108	1	A	TEI17

Both are agreed

R2-2410253	Maximum number of SI messages	Nokia, Nokia Shanghai Bell	discussion	Rel-17	TEI17

Noted

IDC

R2-2409664	Clarification for IDC solutions of logged MDT (Rel-17)	Xiaomi, CMCC	CR	Rel-17	38.331	17.10.0	5109	-	F	NR_ENDC_SON_MDT_enh-Core

R2-2409665	Clarification for IDC solutions of logged MDT (Rel-18)	Xiaomi, CMCC	CR	Rel-18	38.331	18.3.0	5110	-	A	NR_ENDC_SON_MDT_enh-Core

-	LG agrees with the intention but want to change in procedural text rather than capturing this with a NOTE. Huawei think this CR is an optimization and no change is needed. Qualcomm agrees with Huawei and think this is not testable and do not see a need for this. Samsung think this can be left to UE implementation and there is no need to overspecify here

Postponed

CPA

R2-2409775	Correction on the pre-condition for conditional reconfiguration for CPA	vivo	CR	Rel-17	38.331	17.10.0	5115	-	F	LTE_NR_DC_enh2-Core

R2-2409776	Correction on the pre-condition for conditional reconfiguration for CPA, S-CPAC and LTM configuration	vivo	CR	Rel-18	38.331	18.3.0	5116	-	F	LTE_NR_DC_enh2-Core, NR_Mob_enh2-Core

-	Apple thinks that the CR suggests that CPA can only be added if there is already an SCG,but that is not correct. Huawei disagrees with all changes. Ericsson think that there is text in the same section which clarifies that security needs to be activated (“the addition of Secondary Cell Group and SCells is performed only when AS security has been activated”). Qualcomm think the changes are not essential.

Not pursued, can be revisited if essential issues are found.

SON/MDT

R2-2410043	Correction on handling successHO-Config during full configuration	Samsung, Ericsson	CR	Rel-17	38.331	17.10.0	5136	-	F	NR_ENDC_SON_MDT_enh-Core

Agreed

R2-2410157	Correction for RA resource information in RA-report	Sharp, Ericsson, Lenovo	CR	Rel-17	38.331	17.10.0	5144	-	F	NR_ENDC_SON_MDT_enh-Core

R2-2410162	Correction for RA resource information in RA-report	Sharp, Ericsson, Lenovo	CR	Rel-18	38.331	18.3.0	5145	-	A	NR_ENDC_SON_MDT_enh2-Core

Both above are agreed

Coversheet revision by MCC (for R2-2410162): A cat A CR must have the same WI code as the cat F CR.R2-2411254	Correction for RA resource information in RA-report	Sharp, Ericsson, Lenovo	CR	Rel-18	38.331	18.3.0	5145	1	A	NR_ENDC_SON_MDT_enh-Core

- Agreed

R2-2410767	Correction on PSCell mobility history information	Samsung, ZTE Corporation, Sanechips, Ericsson	CR	Rel-17	38.331	17.10.0	5186	-	F	NR_ENDC_SON_MDT_enh-Core

R2-2410775	Correction on PSCell mobility history information	Samsung, ZTE Corporation, Sanechips, Ericsson	CR	Rel-18	38.331	18.3.0	5188	-	A	NR_ENDC_SON_MDT_enh-Core

-	Samsung indicates that after offline discussion there is need to update the wording for example “current PCell” can be different than the current camped cell, so some updates are needed. LG also want to discuss more.

[AT128][601][SONMDT] Correction on PSCell mobility history information (Samsung)

Scope:

    - Produce agreeable CRs

Intended outcome:

    - Agreed CRs in R2-2411012 and R2-2411013

Deadline:

    - Wednesday 17:00. The intention is to agree the CRs over email.

R2-2411012	Correction on PSCell mobility history information	Samsung, ZTE Corporation, Sanechips, Ericsson, Nokia	CR	Rel-17	38.331	17.10.0	5186	1	F	NR_ENDC_SON_MDT_enh-Core

R2-2411013	Correction on PSCell mobility history information	Samsung, ZTE Corporation, Sanechips, Ericsson, Nokia	CR	Rel-18	38.331	18.3.0	5188	1	A	NR_ENDC_SON_MDT_enh-Core

Both above are agreed

#### 6.1.3.2	UE capabilities

UE cap corrections 38306, 38331.

R2-2410030	Correction on Capability Reporting in FR1-NTN Bands	vivo	CR	Rel-17	38.306	17.10.0	1207	-	F	NR_NTN_solutions-Core

- Vivo thinks we can remove the last change on the specific sub-clause

Revised in R2-2410961 to remove the last change

R2-2410961	Correction on Capability Reporting in FR1-NTN Bands	vivo	CR	Rel-17	38.306	17.10.0	1207	1	F	NR_NTN_solutions-Core

Agreed

R2-2410031	Correction on Capability Reporting in FR1-NTN Bands	vivo	CR	Rel-18	38.306	18.3.0	1208	-	A	NR_NTN_solutions-Core

Revised in R2-2410970

R2-2410970	Correction on Capability Reporting in FR1-NTN Bands	vivo	CR	Rel-18	38.306	18.3.0	1208	1	A	NR_NTN_solutions-Core

Agreed

Max nrof segments

R2-2409749	Network signalling of maximum number of UL segments - RRC processing delay	Qualcomm Incorporated	discussion	Rel-17	TEI17

Proposal:	Specify 640ms as the RRC processing delay requirement for the case UECapabilityEnquiry message includes the field rrc-MaxCapaSegAllowed.

R2-2410348	Discussion on segmented UE capability	Huawei, HiSilicon	discussion	Rel-17	TEI17

Proposal 1: RAN2 confirms that the RRC processing delay for UE capability refers to the time from when the UE receives the Capability Enquiry message until it is ready to receive the grant to transmit the first segment.

Proposal 2: The mechanism on network indication of maximum number of UL segments applies to both NR and LTE.

Proposal 3: RAN2 considers to extend the processing time to 800ms for segmented UE capability transfer procedure.

Proposal 4: The proposed extended processing delay is introduced for legacy capability segmentation mechanism for NR and LTE from Release-16.

On P2:

-	MediaTek wants it both for LTE and NR. Apple wants this from Rel-17. MediaTek wants from Rel-16. Qualcomm wants Rel-17.

On P3:

-	Apple is OK with 800 ms. MediaTek wants 1280 ms since this is 16*80. ZTE is concerned by this large number and think that it is not very rare that the UE sends UE capabilities and based on feedback from their product team it seems that it happens rather frequent that the UE needs to send UE capabilities, and it happens that CN releases UE capabilities and this happens in terms of every 30 minutes. Qualcomm understand that the NW vendors want a low number but also the issue is due to NW problems and we need to find a middle ground. Nokia think that we should define a fixed value.

On P4:

-	MediaTek agrees to extend. Samsung also want to extend the existing number. Ericsson think this is NBC. Qualcomm think we can discuss the legacy part separately. Nokia agrees with Qualcomm.

Agreements - The mechanism on network indication of maximum number of UL segments applies to both NR and LTE for Rel-17. - We can discuss if the legacy processing time should be extended later, based on company contributions

[AT128][612][Maint] UE processing delay time for max nrof segments (Qualcomm)

Scope:

    - Come up with a number for the UE processing delay .

Intended outcome:

    - Decided on an agreeable UE processing delay number

Deadline:

    - In time for the Thursday session

-	Qualcomm reports from the offline that the below agreements are likely agreeable and we can have a post meeting email disc to finalize the CRs.

Agreements - RRC processing delay requirement is defined as follows: - 560 + max(0, n-7)*80 [ms], where n is the number of segments. - For both LTE and NR, CRs are from Rel-17, and made early implementable from Rel-16. - Decouple Rel-16 UL segmentation procedure and the new Rel-17 segmentation procedure, i.e. it is possible for the UE to indicate the support of Rel-17 procedure without supporting Rel-16 procedure. - Add “16” as one of the values for the network-requested number of segments.

[POST128][601][Maint] CRs for maximum number of UL segments (Qualcomm)

Scope:

    - Produce agreeable CR for NR and LTE

Intended outcome:

    - Agreed CRs

Deadline:

    - Short

R2-2409750	Introduction of network signalling of maximum number of UL segments [Max-RRC-SegUL]	Qualcomm Incorporated	CR	Rel-17	38.331	17.10.0	5004	2	B	TEI17	R2-2409300

R2-2411247	Introduction of network signalling of maximum number of UL segments [Max-RRC-SegUL]	Qualcomm Incorporated	CR	Rel-17	38.331	17.10.0	5004	3	B	TEI17

=> Agreed

R2-2409751	Introduction of network signalling of maximum number of UL segments [Max-RRC-SegUL]	Qualcomm Incorporated	CR	Rel-18	38.331	18.3.0	5113	-	A	TEI17

R2-2411248	Introduction of network signalling of maximum number of UL segments [Max-RRC-SegUL]	Qualcomm Incorporated	CR	Rel-18	38.331	18.3.0	5113	1	A	TEI17

=> Agreed

R2-2409752	Introduction of network signalling of maximum number of UL segments [Max-RRC-SegUL]	Qualcomm Incorporated	CR	Rel-17	38.306	17.10.0	1202	-	B	TEI17	R2-2409301

R2-2411249	Introduction of network signalling of maximum number of UL segments [Max-RRC-SegUL]	Qualcomm Incorporated	CR	Rel-17	38.306	17.10.0	1202	1	B	TEI17

=> Agreed

R2-2409753	Introduction of network signalling of maximum number of UL segments [Max-RRC-SegUL]	Qualcomm Incorporated	CR	Rel-18	38.306	18.3.0	1203	-	A	TEI17

R2-2411250	Introduction of network signalling of maximum number of UL segments [Max-RRC-SegUL]	Qualcomm Incorporated	CR	Rel-18	38.306	18.3.0	1203	1	A	TEI17

=> Agreed

R2-2411228	Introduction of network signalling of maximum number of UL segments [Max-RRC-SegUL]	Huawei, HiSilicon	CR	Rel-17	36.331	17.10.0	5083		B	TEI17

R2-2411252	Introduction of network signalling of maximum number of UL segments [Max-RRC-SegUL]	Huawei, HiSilicon	CR	Rel-17	36.331	17.10.0	5083	1	B	TEI17

=> Agreed

R2-2411229	Introduction of network signalling of maximum number of UL segments [Max-RRC-SegUL]	Huawei, HiSilicon	CR	Rel-18	36.331	18.3.1	5084	-	A	TEI17

R2-2411253	Introduction of network signalling of maximum number of UL segments [Max-RRC-SegUL]	Huawei, HiSilicon	CR	Rel-18	36.331	18.3.1	5084	1	A	TEI17

=> Agreed

R2-2411230	Introduction of network signalling of maximum number of UL segments [Max-RRC-SegUL]	Huawei, HiSilicon	CR	Rel-17	36.306	17.10.0	1904	-	B	TEI17

=> Agreed

R2-2411231	Introduction of network signalling of maximum number of UL segments [Max-RRC-SegUL]	Huawei, HiSilicon	CR	Rel-18	36.306	18.3.0	1905	-	A	TEI17

=> Agreed

R2-2409862	RRC processing time for NW signalled UL segments	Samsung	discussion	Rel-17	TEI17

Proposal 1: RAN2 agree to increase RRC processing delay for NW requested UL segment feature.

Proposal 2: RAN2 agree that the total RRC processing delay for UE capability enquiry with the new NW requested UL segment is 640msec.

Proposal 3: RAN2 agree to support 16 UL segments in the new NW requested UL segment feature.

Enhanced channel raster

R2-2409992	Mandatory support of Enhanced channel raster by RedCap UE	Ericsson	CR	Rel-17	38.306	17.10.0	1157	2	F	NR_redcap-Core	R2-2409295

Agreed

R2-2410905	Mandatory support of Enhanced channel raster by (e)RedCap UE	Ericsson	CR	Rel-18	38.306	18.3.0	1158	3	F	NR_redcap-Core	R2-2409993

Agreed unseen in R2-2411040, fix meeting number and clarify the “from Rel-18” change clearer on the cover page.

Old revisions

R2-2409993	Mandatory support of Enhanced channel raster by (e)RedCap UE	Ericsson	CR	Rel-18	38.306	18.3.0	1158	2	F	NR_redcap-Core	R2-2409296

#### 6.1.3.3	Other

Including idle and inactive behaviour specified in 38.304 or 36.304.

Emergency

R2-2410772	On eDRX Operation with Emergency Services	ZTE Corporation	discussion	Rel-17	NR_redcap-Core

Proposal 1: To capture in specification that UE shall not use INACTIVE eDRX if UE has PDU session associated with emergency services.

Proposal 2: RAN2 to adopt the text proposals provided in Annex A and B for TS 38.304 Rel-17 and Rel-18 respectively.

Proposal 3: RAN2 to decide whether a standard solution is needed to get RAN aware of whether UE has PDU session associated with emergency services, or leave to network implementation to prevent RAN from using eDRX for the UE.

-	Nokia think that the NW is not aware of emergency services today but it is needed and hence SA2 would need to be contacted. Huawei also want to send an LS to SA2, SA2 needs to decide when the CN should indicate to the RAN about ongoing emergency services. Qualcomm also wants to send an LS and think that if the RAN gets the info from the CN the RAN can configure the UE properly and no autonomous UE action is needed. Nokia think that if the RAN is aware of emerngegy services it can avoid configuring eDRX. ZTE agrees to send an LS but does not want to ask SA2 if CN can indicate to the RAN about emergency services but also want to ask if CN can deconfigure IDLE eDRX and that solves everything. Samsung wants to send an LS but are not sure if IDLE eDRX should really be released by CN when emergency services is ongoing. CATT thinks that this is a NW-issue and could be left to NW implementation or if a standard solution is needed it should be handled by RAN3/SA2. Qualcomm wonders why releasing IDLE eDRX would help. ZTE wants to ask open questions to SA2 about if/how RAN needs to be aware of ongoing emergency services. Samsung are OK to ask open questions. Nokia think that we can just say that based on RAN2 undersatnding the gNB is not aware of emergencv and then highlight the issue.

-	Ericsson think that ARP values can be used by the CN to indicate to the RAN that emergency is ongoing. Ericsson are unsure what is the intention of the LS would be, instead Ericsson prefers to leave this whole problem to NW implementation.

Send an LS to SA2 and CT1, cc RAN3, explaining the RAN2 understanding of the issue and see if and how SA2 think this should be addressed.

R2-2410557	Correction on emergency call back when eDRX is configured in RRC_INACTIVE	Nokia	CR	Rel-17	38.304	17.9.0	0423	-	F	NR_redcap-Core

R2-2410558	Correction on emergency call back when eDRX is configured in RRC_INACTIVE	Nokia	CR	Rel-18	38.304	18.3.0	0424	-	A	NR_redcap-Core

Both are noted

[POST128][602][Maint] LS on emergency services and eDRX (ZTE)

Scope:

    - Produce approvable LS to SA2 and CT1, cc RAN3 as per the agreement

Intended outcome:

    - Approved LS in R2-2411041 (ZTE)

Deadline:

    - Short

R2-2411041	LS on emergency services and eDRX	RAN2	LS out	Rel-17	NR_newRAT-Core, NR_redcap-Core	To:SA2, CT1	Cc:RAN3

=> Approved

## 6.2	NR Sidelink relay

(NR_SL_Relay-Core; leading WG: RAN2; REL-17; WID: RP-212601)

### 6.2.0	In-principle agreed CRs

R2-2409757	Miscellaneous CR for Rel-17 SL relay	Huawei, HiSilicon, Philips International B.V., OPPO	CR	Rel-17	38.331	17.10.0	5086	1	F	NR_SL_relay-Core	R2-2409260

Agreed

R2-2409758	Miscellaneous CR for Rel-17 SL relay	Huawei, HiSilicon, Philips International B.V., OPPO	CR	Rel-18	38.331	18.3.0	5087	1	A	NR_SL_relay-Core	R2-2409261

Agreed

R2-2409850	Clarification on the L2 U2N Remote UE Measurement	CATT	CR	Rel-17	38.331	17.10.0	4977	1	F	NR_SL_relay-Core	R2-2407996

Revised in R2-2410903 (formatting fixes)

R2-2410903	Clarification on the L2 U2N Remote UE Measurement	CATT	CR	Rel-17	38.331	17.10.0	4977	2	F	NR_SL_relay-Core	R2-2407996

Agreed

R2-2409851	Clarification on the L2 U2N Remote UE Measurement	CATT	CR	Rel-18	38.331	18.3.0	4978	1	A	NR_SL_relay-Core	R2-2407997

Revised in R2-2410904 (formatting fixes)

R2-2410904	Clarification on the L2 U2N Remote UE Measurement	CATT	CR	Rel-18	38.331	18.3.0	4978	2	A	NR_SL_relay-Core	R2-2407997

Agreed

### 6.2.1	Other

R2-2410579	RRC correction on SidelinkUEInformationNR for NR sidelink relay communication transmission	Philips International B.V., NEC	CR	Rel-17	38.331	17.10.0	5171	-	F	NR_SL_relay-Core

Not pursued

Discussion:

Huawei think the change is not correct because the following level 4 bullet covers relay behaviour

ZTE agree with Huawei, and they note that they submitted a similar CR in RAN2#126 that was considered not necessary.

Nokia tend to agree with Huawei.

Huawei think we have seen this issue before and decided not to change it.

R2-2410580	RRC correction on SidelinkUEInformationNR for NR sidelink relay communication transmission	Philips International B.V., NEC	CR	Rel-18	38.331	18.3.0	5172	-	A	NR_SL_relay-Core

Not pursued

## 6.3	NR positioning enhancements

(NR_pos_enh-Core; leading WG: RAN1; REL-17; WID: RP-210903)

### 6.3.0	In-principle agreed CRs

R2-2410220	Correction to MAC for R17 POS-r17	Huawei, HiSilicon	CR	Rel-17	38.321	17.10.0	1998	-	F	NR_pos_enh-Core

Agreed, but then coversheet revised by MCC: wrong tdoc number on the coversheet.

R2-2411255	Correction to MAC for R17 POS-r17	Huawei, HiSilicon	CR	Rel-17	38.321	17.10.0	1998	1	F	NR_pos_enh-Core

=> Agreed

R2-2410221	Correction to MAC for R17 POS	Huawei, HiSilicon	CR	Rel-18	38.321	18.3.0	1999	-	A	NR_pos_enh-Core

Agreed

### 6.3.1	Other

Related to Rel-18 LS in R2-2409508 (see AI 7.1.1)

R2-2409565	Correction on spatial relation info in SP SRS activation deactivation MAC CE (R17)	ZTE Corporation, Ericsson	CR	Rel-17	38.321	17.10.0	1977	-	F	NR_pos_enh-Core

Revised in R2-2410985

Discussion:

Huawei think the CR is not needed in this form, because it addresses only the SP-SRS case in RRC_INACTIVE and may be misleading.  They think we could address it with an RRC clarification and not touch the MAC spec.

ZTE understand that RAN1’s LS did not indicate that SSB and PRS cannot be used; only the CSI-RS and SRS configured in RRC_CONNECTED were excluded.

CATT agree with the intention of the CR but think the wording can be polished in the related RRC CR.

Qualcomm think the second sentence needs a bit of improvement; they think it should say “configured in RRC_INACTIVE” rather than “configured in RRCRelease”.

Huawei want to avoid discussion of whether the spatial relation can be based on SSB or PRS, and they think an RRC clarification can eliminate the problematic setting of the MAC CE.

ZTE reiterate that RAN1 did not say anything about SSB or PRS; they are not sure why the SSB cannot be used.

CATT understand that signals from connected mode do not apply in RRC_INACTIVE, hence the exclusion of CSI-RS and SRS from connected; but for SSB and PRS, they think it should work.  In the RRC CR, they think the wording could include the relevant IE names to clarify which IEs are/are not configured for this SRS.

Huawei think when the gNB or UE wants to use other sources for the spatial relation, it needs to be within the configuration.  They think the MAC CE will never be set wrongly if the RRC configuration is correct.

Qualcomm think it makes sense to document in the MAC spec, because the origin of this discussion was a question about whether a configuration from RRC_CONNECTED can be used in RRC_INACTIVE, and the CR clearly answers this.

Ericsson note that the MAC spec covers SP-SRS; they understand that the CR aligns with RAN1 specs.

Huawei think it is clear that in RRC_INACTIVE, the UE only uses the configuration from SuspendConfig.  Qualcomm think this may be right in principle, but the positioning setting is potentially different from other features and this is where the confusion originally came from.

CATT understand that RAN1 wanted to clarify Rel-17.  Huawei think the main conclusion from the RAN1 LS is that we do not need to add CSI-RS configuration to SuspendConfig, and so nothing needs to be changed.

ZTE think we should take the CR to follow the RAN1 LS.

[AT128][402][POS] Spatial relation info source for positioning in RRC_INACTIVE (ZTE)

Scope: Polish the RRC and MAC CRs in R2-2409565 and R2-2409607 and their shadows, and discuss to converge on what level of changes to the MAC spec are acceptable.

Intended outcome: Agreeable CRs (with CB) in R2-2410985 / R2-2410986 / R2-2410987 / R2-2410988

Deadline: Wednesday 2024-11-20 1600 EST

R2-2409566	Correction on spatial relation info in SP SRS activation deactivation MAC CE (R18)	ZTE Corporation, Ericsson	CR	Rel-18	38.321	18.3.0	1978	-	A	NR_pos_enh-Core

Revised in R2-2410986

R2-2409607	Correction on spatial relation info in SRS configuration (R17)	ZTE Corporation, Ericsson	CR	Rel-17	38.331	17.10.0	5101	-	F	NR_pos_enh-Core

Revised in R2-2410987

R2-2409608	Correction on spatial relation info in SRS configuration (R18)	ZTE Corporation, Ericsson	CR	Rel-18	38.331	18.3.0	5102	-	A	NR_pos_enh-Core

Revised in R2-2410988

R2-2410985	Correction on spatial relation info in SP SRS activation deactivation MAC CE (R17)	ZTE Corporation, Ericsson	CR	Rel-17	38.321	17.10.0	1977	-	F	NR_pos_enh-Core

R2-2410986	Correction on spatial relation info in SP SRS activation deactivation MAC CE (R18)	ZTE Corporation, Ericsson	CR	Rel-18	38.321	18.3.0	1978	-	A	NR_pos_enh-Core

Come back in main session on the MAC CRs above

R2-2410987	Correction on spatial relation info in SRS configuration (R17)	ZTE Corporation, Ericsson, Qualcomm, CATT, Samsung, vivo, Nokia, Xiaomi	CR	Rel-17	38.331	17.10.0	5101	1	F	NR_pos_enh-Core

Agreed

R2-2410988	Correction on spatial relation info in SRS configuration (R18)	ZTE Corporation, Ericsson, Qualcomm, CATT, Samsung, vivo, Nokia, Xiaomi	CR	Rel-18	38.331	18.3.0	5102	1	A	NR_pos_enh-Core

Agreed

Discussion:

Huawei think not even the RRC change is needed, but they can accept the RRC CR without the MAC CR.  They understand that the RRC procedure is already clear since Rel-15.

ZTE think the procedure is not so clear and it is helpful for implementation teams to have it explicit.

Ericsson agree that the RRC constraint is there, but for the MAC, we already have specifications on which signals are applicable and which are not, and they find no harm in clarifying this case.  They agree it is helpful for implementation.

CATT think it is clear that SRS is configured in separate IEs in the two states, and which signals cannot be used is clear from the RAN1 LS, so they see some connection between the connected and inactive configurations.  They think we need to capture the RAN1 agreement.

Nokia think some explicit clarification is good, especially since we needed the exchange of LSs with RAN1 to get the clarification from them.  If there is a technical issue they are open to discuss it, but in principle they think we should have the change.

Qualcomm consider the MAC CR more important; we have one MAC CE for both connected and inactive, and this CR is the only place that indicates it.

Huawei think if there is a clarification in RRC, it should start from Rel-15.  They understand that it is clear already that the UE only applies the configuration from SuspendConfig.

ZTE think we are only considering the positioning-specific features, and a general clarification from Rel-15 would affect other features and should not be considered here.  They think it would be correct if such a clarification were made, but not in the positioning session.

Huawei still think the MAC CR is redundant with the restriction in the RRC configuration and there should be no confusion about the applicability of the MAC CE.

ZTE think the MAC CE description could be misread regarding what is applicable in which state, leading to a wrong implementation.

Qualcomm note that SRS for positioning in inactive was introduced in Rel-17, and in Rel-18 we had the discussion of the MAC CE applicability, so it was clearly not obvious to everyone.

CATT observe that we had no conclusion before RAN1 sent the LS, which argues that we need some clarification.

Huawei think a clarification from the reader’s point of view is up to the reader’s interpretation.

Ericsson would like to postpone if we cannot reach agreement.  Nokia think postponing or going to the main session would not help and we should decide here.

Ericsson think we could consult with the MAC spec rapporteur.

R2-2409628	Corrections on the NOTE in the description of dl-PRS-MeasRRC-Inactive	CATT	CR	Rel-17	37.355	17.8.0	0529	-	F	NR_pos_enh-Core

Not pursued

Discussion:

Lenovo understand we captured text from the RAN1 feature list; they agree it looks a bit confusing, but they understood it meant that all these capabilities are consistent between connected and inactive.

Huawei wonder if the CR changes the meaning.

Ericsson think the original sentence says that all these capabilities, as already defined for connected mode, are also applicable in inactive mode.

Agreement:

RAN2 understand that for the capabilities enumerated in NOTE 2 under the field description of dl-PRS-MeasRRC-Inactive, each of the capabilities is the same in RRC_CONNECTED and RRC_INACTIVE (i.e., there is no implication that the different capabilities are the same as each other).

R2-2409629	Corrections on the NOTE in the description of dl-PRS-MeasRRC-Inactive	CATT	CR	Rel-18	37.355	18.3.0	0530	-	A	NR_pos_enh-Core

Not pursued

R2-2410222	Correction to PRS priority subset for DL-AoD-r17	Huawei, HiSilicon	CR	Rel-17	37.355	17.8.0	0535	-	F	NR_pos_enh-Core

Postponed

Discussion:

Qualcomm wonder if it is really forbidden to use this for UE-based.  Huawei agree it could be used, but our usual usage would say this field is only applicable for UE-assisted.

CATT understand that it applies to UE-based.

Samsung agree with the CR; after checking the RAN1 feature list, they found that this is for the UE-assisted case.

vivo also support the CR and confirmed with RAN1 colleagues the understanding that it is only needed for measurement reports.

CATT note that the UE can report the measurement also in UE-based.

Huawei think normally if an AD field is used for both, we label it as being for UE-assisted.

Nokia looked in the stage 3, and the field description there says it is for “reporting”, which suggests both UE-based and UE-assisted, with a reference to 38.214.

Chair wonders what the operational impact of the CR is.  Huawei indicate that it is just for the sake of the spec, and an implementation will infer from other related specs what needs to be done.

Samsung checked the RAN1 feature list and found that the feature name includes “UE-assisted”.

R2-2410223	Correction to PRS priority subset for DL-AoD-r18	Huawei, HiSilicon	CR	Rel-18	37.355	18.3.0	0536	-	A	NR_pos_enh-Core

Postponed

R2-2410825	Correction of SRS type for TA alignment	Ericsson	CR	Rel-17	38.321	17.10.0	2014	-	F	NR_pos_enh-Core

Agreed

Discussion:

Huawei think the change is fine but not critical.

R2-2410826	Correction of SRS type for TA alignment	Ericsson	CR	Rel-18	38.321	18.3.0	2015	-	A	NR_pos_enh-Core

Agreed

## 6.6	NR Sidelink enhancements

(NR_SL_enh-Core; leading WG: RAN1; REL-17; WID: RP-202846)

CR rapporteurs will take care of miscellaneous CRs to collect small changes. Please first contact / coordinate with CR rapporteur company for small changes (e.g. non-controversial clarification/correction, editorial correction, etc.).

### 6.6.0	In-principle agreed CRs

R2-2409743	Misc RRC corrections for SL enhancements	Huawei, HiSilicon	CR	Rel-17	38.331	17.10.0	5001	1	F	NR_SL_enh-Core	R2-2408361

R2-2409744	Misc RRC corrections for SL enhancements	Huawei, HiSilicon	CR	Rel-18	38.331	18.3.0	5002	1	A	NR_SL_enh-Core	R2-2408362

Agreed.

R2-2410126	Correction to random resource selection for sidelink	Ericsson	CR	Rel-17	38.321	17.10.0	1944	2	F	NR_SL_enh-Core	R2-2409369

R2-2410127	Correction to random resource selection for sidelink	Ericsson	CR	Rel-18	38.321	18.3.0	1945	2	A	NR_SL_enh-Core	R2-2409370

Agreed.

[Ericsson]: Indicates the cover page has been updated compared to in-principle agreed CR last meeting, to correct change affect and to add CR’s revision history.

### 6.6.1	Other

R2-2409509	Reply LS on IUC Scheme-2 and Random Selection (R1-2409174; contact: OPPO)	RAN1	LS in	Rel-17	NR_SL_enh-Core	To:RAN2

Noted.

R2-2409551	Discussion on R1 LS Reply (R1-2409174)	OPPO	discussion	Rel-17	NR_SL_enh-Core

Proposal 1: R2 decide either to allow IUC scheme-2 and random-selection co-configuration or not, and adopt CR correspondingly (TP1 if allow, TP2 if disallow).

TP2 is agreed.

CRs will be revised in R2-2410937 and R2-2410938

[LG, Qualcomm]: TP2 is preferred. TP1 seems to introduce new function, which is not acceptable in this late phase.

R2-2410937	Co-configuration of IUC scheme-2 and random selection	OPPO	CR	Rel-17	38.321	17.10.0	2016	-	F	NR_SL_enh-Core

R2-2410938	Co-configuration of IUC scheme-2 and random selection	OPPO	CR	Rel-18	38.321	18.3.0	2017	-	A	NR_SL_enh-Core

R2-2411139	Co-configuration of IUC scheme-2 and random selection	OPPO	CR	Rel-18	38.321	18.3.0	2017	1	A	NR_SL_enh-Core

Rel-17 CR in R2-2410937 and Rel-18 CR in R2-2411139 are agreed.

R2-2410070	Correction on SL DRX parameters	ZTE Corporation, Sanechips, Huawei, HiSilicon, Ericsson, Nokia, ASUSTeK, Apple, OPPO, LG	CR	Rel-17	38.331	17.10.0	5139	-	F	NR_SL_enh-Core

R2-2410071	Correction on SL DRX parameters	ZTE Corporation, Sanechips, Huawei, HiSilicon, Ericsson, Nokia, ASUSTeK, Apple, OPPO, LG	CR	Rel-18	38.331	18.3.0	5140	-	A	NR_SL_enh-Core

Both Agreed.

R2-2409961	Correction on SL DRX procedure	Apple	CR	Rel-17	38.321	17.10.0	1982	-	F	NR_SL_enh-Core

R2-2409962	Correction on SL DRX procedure	Apple	CR	Rel-18	38.321	18.3.0	1983	-	A	NR_SL_enh-Core

Noted. Companies would like to have more time to think of.

[OPPO]: Change 1 and 4 are ok. [Huawei]: For change 1, we can add the correct RRC configured IE names instead of removing it. [Apple]: Considering sl-drx-HARQ-RTT-Timer is not real RRC configured IE name and it is commonly applied to all cases (both UC and GC/BC), it should be ok without explicitly adding names. [Samsung]: All should be RRC configured IE names because of the sentence “by configuring the following parameters” in the above bullet. [OPPO]: It is not an urgent issue, would like to have more time to think of.

[OPPO]: For change 2 and 3, do not understand what is real difference between the current text and the proposed text. [Qualcomm]: The association between QoS and destination id is specified in the upper layer spec. We do not need to specify it in exact. [CATT]: Understand the intention is to make the procedure being performed for each destination Layer-2 ID associated to groupcast or broadcast. [ZTE]: For change 2, isn’t it better to remove “and the associated cast type is groupcast or broadcast” in the sub-bullets since it is already covered in the upper bullet? [OPPO]: It is hard to understand what changes correspond to issue 2 and issue 3. Cover page is also note clear.

R2-2410080	Correction on unit of SL DRX timer	ZTE Corporation, Sanechips, Huawei, HiSilicon, Ericsson, Nokia, ASUSTeK, Apple, OPPO, LG	CR	Rel-17	38.321	17.10.0	1940	1	F	NR_SL_enh-Core	R2-2408539

R2-2410081	Correction on unit of SL DRX timer	ZTE Corporation, Sanechips, Huawei, HiSilicon, Ericsson, Nokia, ASUSTeK, Apple, OPPO, LG	CR	Rel-18	38.321	18.3.0	1941	1	A	NR_SL_enh-Core	R2-2408540

The same wording that is used for Uu DRX will be used.

CRs will be revised in R2-2410939 and R2-2410940

[LG]: Suggest to use the same wording that is used for Uu DRX.

R2-2410939	Correction on unit of SL DRX timer	ZTE Corporation, Sanechips, Huawei, HiSilicon, Ericsson, Nokia, ASUSTeK, Apple, OPPO, LG	CR	Rel-17	38.321	17.10.0	1940	2	F	NR_SL_enh-Core

R2-2410940	Correction on unit of SL DRX timer	ZTE Corporation, Sanechips, Huawei, HiSilicon, Ericsson, Nokia, ASUSTeK, Apple, OPPO, LG	CR	Rel-18	38.321	18.3.0	1941	2	A	NR_SL_enh-Core

Agreed.

R2-2410128	Correction to MAC on resource selection	Ericsson	CR	Rel-17	38.321	17.10.0	1986	-	F	NR_SL_enh-Core

R2-2410129	Correction to MAC on resource selection	Ericsson	CR	Rel-18	38.321	18.3.0	1987	-	A	NR_SL_enh-Core

Not pursued.

[LG]: No real difference between the current text and the proposed text. Prefer keeping the current text since it seems not essential correction. [OPPO]: Agree with LG. In addition, do not understand why it is NBC change in the CR cover page. [ZTE]: Agree with the CR, but it should be considered as BC change. [Huawei]: Agree with LG.

R2-2410171	Clarification on terminologies for SL IUC	ASUSTeK	CR	Rel-17	38.321	17.10.0	1988	-	F	NR_SL_enh-Core

R2-2410172	Clarification on terminologies for SL IUC	ASUSTeK	CR	Rel-18	38.321	18.3.0	1989	-	A	NR_SL_enh-Core

For the first change, change different names to “Sidelink Inter-UE Coordination Information MAC CE” and “Sidelink Inter-UE Coordination Request MAC CE”.

With the change above, CRs in R2-2410941 and R2-2410942 are agreed.

[Huawei]: Prefer to have unified name as “Sidelink Inter-UE Coordination Information MAC CE” [LG]: Second change is aligned with other parts in MAC so it should be ok.

R2-2410711	Correction on IUC		LG Electronics Inc.	CR	Rel-17	38.321	17.10.0	2010	-	F	NR_SL_enh-Core

R2-2410712	Correction on IUC		LG Electronics Inc.	CR	Rel-18	38.321	18.3.0	2011	-	A	NR_SL_enh-Core

For the first change, offline discussion for the better wordings.

For the second change, it will change to “by Sidelink Inter-UE Coordination Request MAC CE only, both Sidelink Inter-UE Coordination Request MAC CE and SCI (See TS 38.214 [7]), or condition (See TS 38.331 [5] and TS 38.214 [7]).” and “by an SL-IUC Request MAC CE alone, or by both an Sidelink Inter-UE Coordination Request MAC CE and an SCI, and not cancelled:”

CRs will be revised in R2-2410950 and R2-2411131

[Apple]: The current text already can cover both cases. [ZTE]: Agree with the change 2, however wonders if the change under the bullet 1> is required. [OPPO]: Agree with the correction. [Nokia, Huawei]: Suggest rewordings as follow. [Apple, Qualcomm, ZTE]: Do not think any change is needed under the bullet 1>. [OPPO]: Prefer to have clear clarification also for bullet 1>.

To change the proposed first sentence to “by Sidelink Inter-UE Coordination Request MAC CE only, both Sidelink Inter-UE Coordination Request MAC CE and SCI (See TS 38.214 [7]), or condition (See TS 38.331 [5] and TS 38.214 [7]).”

To change the proposed second sentence to “by an SL-IUC Request MAC CE alone, or by both an Sidelink Inter-UE Coordination Request MAC CE and an SCI, and not cancelled:”

R2-2410950	Correction on IUC		LG Electronics Inc.	CR	Rel-17	38.321	17.10.0	2010	1	F	NR_SL_enh-Core

R2-2411131	Correction on IUC		LG Electronics Inc.	CR	Rel-18	38.321	18.3.0	2011	1	A	NR_SL_enh-Core

Agreed.

# 7	Rel-18

## 7.0	Common

Rel-18 WIs not covered under an explicit AI in 7.x.  Multi-WI Rel-18 items, e.g. cross-WI-issues not handled under another WI. UE capabilities.

### 7.0.0	In-principle agreed CR

Only in-principle agreed CRs that haven’t been modified should be submitted in this AI.   If an in-principle agreed CR has been modified it should be submitted under corresponding sub-AI.

SON/MTD

R2-2409646	Miscellaneous corrections for Rel-18 SON/MDT	CATT, Samsung, Ericsson	CR	Rel-18	38.331	18.3.0	4986	2	F	NR_ENDC_SON_MDT_enh2-Core	R2-2409281

The CR is agreed

R2-2410652	Miscellaneous corrections on R18 SONMDT for 36.331	Huawei, HiSilicon	CR	Rel-18	38.331	18.3.0	5177	-	F	NR_ENDC_SON_MDT_enh2-Core	R2-2408842	Withdrawn

R2-2410698	Miscellaneous corrections on R18 SONMDT for 36.331	Huawei, HiSilicon	CR	Rel-18	36.331	18.3.1	5063	1	F	NR_ENDC_SON_MDT_enh2-Core	R2-2408842

The CR is agreed

XR

R2-2409810	Correction for stored SDUs handling when a t-Reordering expires	Samsung	CR	Rel-18	38.323	18.3.0	0142	1	F	NR_XR_enh-Core	R2-2408481

The CR is agreed

R2-2410396	stage 2 Correction on additional buffer size table	NEC, Nokia (Rapporteur)	CR	Rel-18	38.300	18.3.0	0932	-	F	NR_XR_enh-Core	Withdrawn

R2-2410532	Clarification for UE capability on UL traffic information	Huawei, HiSilicon	CR	Rel-18	38.306	18.3.0	1182	1	F	NR_XR_enh-Core	R2-2408729

The CR is agreed

R2-2410859	Stage 2 correction on additional buffer size table	NEC, Nokia (Rapporteur)	CR	Rel-18	38.300	18.3.0	0917	2	F	NR_XR_enh-Core	R2-2409381

The CR is agreed

CG-SDT

R2-2410226	Rapporteur CR for MT-SDT and CG-SDT enhancement [CG-SDTenh]	Huawei, HiSilicon	CR	Rel-18	38.321	18.3.0	2001	-	F	TEI18, NR_MT_SDT-Core

The CR is agreed

QoE

R2-2410262	Correction of Enhancement on NR QoE management and optimizations for diverse services	Ericsson, ZTE	CR	Rel-18	38.331	18.3.0	5154	-	F	NR_QoE_enh-Core	Withdrawn

R2-2410307	Correction on QoE measurements release at successful handover from LTE/5GC to NR	Nokia, Nokia Shanghai Bell, Ericsson	CR	Rel-18	36.331	18.3.1	5062	1	F	NR_QoE_enh-Core	R2-2408833

=> Revised in R2-2410919

R2-2410919	Correction on QoE measurements release at successful handover from LTE/5GC to NR	Nokia, Nokia Shanghai Bell, Ericsson	CR	Rel-18	36.331	18.3.1	5062	2	F	NR_QoE_enh-Core

The CR is agreed

R2-2410424	Correction of Enhancement on NR QoE management and optimizations for diverse services	Ericsson, ZTE	CR	Rel-18	38.331	18.3.0	5030	1	F	NR_QoE_enh-Core	R2-2408745

=> Revised in R2-2410920

R2-2410920	Correction of Enhancement on NR QoE management and optimizations for diverse services	Ericsson, ZTE	CR	Rel-18	38.331	18.3.0	5030	2	F	NR_QoE_enh-Core

The CR is agreed

R2-2410653	Correction on priority-based QoE measurements in TS 38.300	Huawei, HiSilicon, China Unicom, Nokia, Ericsson, ZTE, Apple	CR	Rel-18	38.300	18.3.0	0919	2	F	NR_QoE_enh-Core	R2-2409274

-	The sentence should be placed at the end and should be improved

Move sentence at the end and work offline for wording improvement

[AT128][003][QoE]  Update to R2-2410653 (Huawei)

Intended outcome: agree by email

Deadline:  Thursday

R2-2411161	Correction on priority-based QoE measurements in TS 38.300	Huawei, HiSilicon, China Unicom, Nokia, Ericsson, ZTE, Apple	CR	Rel-18	38.300	18.3.0	0919	3	F	NR_QoE_enh-Core

The CR is agreed

eRedCap

R2-2410349	Corrections on capabilities for eRedCap	Huawei, HiSilicon, LG Electronics Inc., Intel Corporation	CR	Rel-18	38.306	18.3.0	1166	2	F	NR_redcap_enh-Core	R2-2409395

The CR is agreed

FRx/xDD differentiation

R2-2410368	Guidelines on implementing FRx/xDD differentiation in per UE capability	Huawei, HiSilicon	CR	Rel-18	38.306	18.3.0	1211	-	F	NR_newRAT-Core, TEI18

R2-2410369	Guidelines on implementing FRx/xDD differentiation in per UE capability	Huawei, HiSilicon	CR	Rel-18	38.331	18.3.0	5162	-	F	NR_newRAT-Core, TEI18

=> Revised in R2-2410899

R2-2410899	Guidelines on implementing FRx/xDD differentiation in per UE capability	Huawei, HiSilicon	CR	Rel-18	38.331	18.3.0	5162	1	F	NR_newRAT-Core, TEI18

Start it from Rel-17 and Rel-18 becomes a shadow CR

R2-2411081	Guidelines on implementing FRx/xDD differentiation in per UE capability	Huawei, HiSilicon	CR	Rel-17	38.306	17.10.0	1219	-	F	NR_newRAT-Core, TEI17

The CR is agreed

R2-2411082	Guidelines on implementing FRx/xDD differentiation in per UE capability	Huawei, HiSilicon	CR	Rel-18	38.306	18.3.0	1211	1	A	NR_newRAT-Core, TEI17

The CR is agreed

R2-2411083	Guidelines on implementing FRx/xDD differentiation in per UE capability	Huawei, HiSilicon	CR	Rel-17	38.331	17.10.0	5195	-	F	NR_newRAT-Core, TEI17

The CR is agreed

R2-2411084	Guidelines on implementing FRx/xDD differentiation in per UE capability	Huawei, HiSilicon	CR	Rel-18	38.331	18.3.0	5162	2	A	NR_newRAT-Core, TEI17

The CR is agreed

IDC

R2-2410585	Correction on the idc-TDM-AssistanceConfig	Huawei, HiSilicon, Xiaomi, Nokia, Intel Corporation, Ericsson, Apple, Samsung	CR	Rel-18	38.331	18.3.0	5062	1	F	NR_IDC_enh-Core	R2-2408937

The CR is NBC and the NBC sentence needs to be added

The CR is agreed with the functional NBC sentence added in R2-2411080

MBS

R2-2410615	Paging clarification for MBS	SHARP Corporation	CR	Rel-18	38.331	18.3.0	5097	1	F	NR_MBS_enh-Core	R2-2409275

The CR is agreed

R2-2410811	Rapporteur correction on multicast MCCH	Huawei, HiSilicon, Samsung, CATT	CR	Rel-18	38.331	18.3.0	5190	-	F	NR_MBS_enh-Core

=> Revised in R2-2410896

R2-2410896	Rapporteur correction on multicast MCCH	Huawei, HiSilicon, Samsung, CATT	CR	Rel-18	38.331	18.3.0	5190	1	F	NR_MBS_enh-Core

The CR is agreed

R2-2410812	Correction on multicast reception in RRC_INACTIVE upon paging	Huawei, HiSilicon, Nokia, CATT, Ericsson, Samsung, Apple, ZTE	CR	Rel-18	38.331	18.3.0	5191	-	F	NR_MBS_enh-Core

The CR is agreed

MUSIM

R2-2410773	Correction to the musim-AffectedBandsList and musim-AvoidedBandsList	ZTE Corporation	CR	Rel-18	38.331	18.3.0	5006	2	F	NR_DualTxRx_MUSIM-Core	R2-2409223

=> Revised in R2-2410910

R2-2410910	Correction to the musim-AffectedBandsList and musim-AvoidedBandsList	ZTE Corporation	CR	Rel-18	38.331	18.3.0	5006	3	F	NR_DualTxRx_MUSIM-Core

The CR is agreed

CE

R2-2409811	Clarifications on DPC field in PHR MAC CE	Samsung	CR	Rel-18	38.321	18.3.0	1957	1	F	NR_cov_enh2-Core	R2-2408972

The CR is agreed

### 7.0.1	UE Capabilities

Multi-WI handling of Rel-18 feature lists and UE capability Mega CRs.

R2-2409506	LS on higher layer parameters for DCI format 2_3 (R1-2407534; contact: CATT)	RAN1	LS in	Rel-16	NR_newRAT-Core, TEI16	To:RAN2

Noted

UE feature list

R2-2409507	LS on Rel-18 RAN1 UE features list for NR after RAN1#118bis (R1-2409045; contact: NTT DOCOMO, AT&T)	RAN1	LS in	Rel-18	TEI18, NR_MIMO_evo_DL_UL, NR_pos_enh2, Netw_Energy_NR, NR_netcon_repeater, NR_NTN_enh, NR_Mob_enh2, NR_SL_enh2, NR_redcap_enh, NR_MC_enh, NR_XR_enh, NR_FR1_lessthan_5MHz_BW, NR_DSS_enh, NR_BWP_wor, NR_cov_enh2	To:RAN2	Cc:RAN4

Noted

R2-2409917	Miscellaneous updates and Introduction of DPC UE capabilities for Rel-18 WIs	Intel Corporation	CR	Rel-18	38.306	18.3.0	1205	-	B	NR_MIMO_evo_DL_UL-Core, Netw_Energy_NR, NR_Mob_enh2, NR_MC_enh, NR_netcon_repeater-Core, NR_cov_enh2

=> Revised in R2-2410894

R2-2410894	Miscellaneous updates for Rel-18 WIs	Intel Corporation	CR	Rel-18	38.306	18.3.0	1205	1	F	NR_MIMO_evo_DL_UL-Core, Netw_Energy_NR, NR_Mob_enh2, NR_MC_enh, NR_netcon_repeater-Core

-	Lenovo would prefer to not have the NOTE2 as note but rather same level as previous text.

-	Intel indicates that we should wait for positioning

Wait for positioning and mobility session to agree and update accordingly

[POST128][002][UE Cap] UE capability CR  (Intel)

Intended outcome: agree to CR

Deadline:  short

R2-2409918	Miscellaneous updates and Introduction of DPC UE capabilities for Rel-18 WIs	Intel Corporation	CR	Rel-18	38.331	18.3.0	5120	-	B	NR_Mob_enh2, NR_cov_enh2

=> Revised in R2-2410895

R2-2410895	Miscellaneous updates for Rel-18 WIs	Intel Corporation	CR	Rel-18	38.331	18.3.0	5120	1	F	NR_Mob_enh2

Need to add impact analysis

Review the CR in the email discussion [002]

R2-2411245	Miscellaneous updates for Rel-18 WIs	Intel Corporation	CR	Rel-18	38.306	18.3.0	1205	2	B	NR_MIMO_evo_DL_UL-Core, Netw_Energy_NR, NR_Mob_enh2, NR_MC_enh, NR_netcon_repeater-Core

=> Agreed

R2-2411246	Miscellaneous updates for Rel-18 WIs	Intel Corporation	CR	Rel-18	38.331	18.3.0	5120	2	B	NR_Mob_enh2

=> Agreed

### 7.0.2	Rel-18 corrections

Essential corrections only. For smaller corrections please contact CR editor / Rapporteur directly.  Coordinate with rapporteurs and chair if input above limit is required

Tdoc limitation: 6

#### 7.0.2.1	RACH-less HO

Corrections to generalized RACH-less HO procedure, including NTN, mIAB, and overlapping sections of the LTM cell switch procedure

Rapporteur CR

R2-2410225	Rapporteur MAC CR for RACH-less HO and LTM [RACH-lessHO]	Huawei, HiSilicon, Qualcomm	CR	Rel-18	38.321	18.3.0	2000	-	F	TEI18, NR_Mob_enh2-Core, NR_NTN_enh-Core, NR_mobile_IAB-Core	Late

Withdrawn

Other CRs

R2-2410415	Correction on RACH-less handover procedure in TS38.321 [RACH-lessHO]	CATT, Samsung, InterDigital	CR	Rel-18	38.321	18.3.0	2003	-	F	TEI18, NR_NTN_enh-Core, NR_mobile_IAB-Core

-	LG agrees with the understanding but there is no change needed as if the first if is met it will not go to else if.  Qualcomm agrees with LG.

-	CATT thinks that there may be a ambiguity how to handle subsequent transmission.    Interdigital was convinced by CATT and if there is confusion we should clarify.

The CR is revised in R2-24111178

RAN2 confirms the understanding that if the initial transmission of RACH-less handover has not been performed according to clause 5.4.1 and 5.33 (i.e. the "else" case w.r.t. the "after the initial transmission of RACH-less handover has been performed according to clause 5.4.1 and 5.33" part in the first "1> if" condition).

R2-2411178	Correction on RACH-less handover procedure in TS38.321 [RACH-lessHO]	CATT, Samsung, InterDigital, Ericsson, Qualcomm Incorporated, LG Electronics Inc., Huawei, HiSilicon, vivo	CR	Rel-18	38.321	18.3.0	2003	1	F	TEI18, NR_NTN_enh-Core, NR_mobile_IAB-Core

The CR is agreed

R2-2410855	Correction on RACH-less handover fallback [RACH-lessHO]	Huawei, HiSilicon, Samsung, CATT, LG Electronics, Nokia, Nokia Shanghai Bell, Ericsson	CR	Rel-18	38.321	18.3.0	1972	2	F	NR_mobile_IAB-Core, NR_NTN_enh-Core, TEI18	R2-2409406

Update with architecture option

The CR is agreed in R2-2411093 with change above

R2-2410745	Correction on RACH-less handover fallback [RACH-lessHO]	Huawei, HiSilicon, Samsung, CATT, LG Electronics, Nokia, Nokia Shanghai Bell, Ericsson	CR	Rel-18	38.331	18.3.0	5181	-	F	NR_mobile_IAB-Core, NR_NTN_enh-Core, TEI18	R2-2409406	Withdrawn

#### 7.0.2.2	NR network-controlled repeaters

(NR_NetConRepeater; leading WG: RAN1; REL-18; WID: RP-230175)

R2-2410726	Clarification on NCR-MT Mandatory features	Google	CR	Rel-18	38.306	18.3.0	1217	-	F	NR_Mob_enh2-Core, NR_netcon_repeater-Core

-	Huawei and ZTE Agree with intention but since handover is listed, it would cover LTM.  Samsung agrees and we should stop these types of CRs.

-	Google asks if it is clear that LTM is handover and should be clarified in stage 2.  ZTE clarifies it is a method for handover.   LG is not sure if LTM covers handover across the specifications.   Xiaomi indicates that in 9.2.3.1 of 38.300 it mentions it is a handover.

The CR is not pursued

R2-2410809	Correction on the NCR-MT capability	Huawei, HiSilicon	CR	Rel-18	38.306	18.3.0	1218	-	F	NR_netcon_repeater-Core

-	ZTE disagrees, this capability was introduced so it knows whether NAS is done in SRB1 or SRB2 and we have same IAB capability without this restriction.   If none capabilities are signalled then SRB1 is used for NAS.

-	Huawei thinks that SRB2 is mandatory.  ZTE explains that SRB2 is not mandatory and that’s why we introduced this capability.

-	Qualcomm agrees with ZTE and we followed the IAB framework.

-	ZTE thinks that we need to check if we need to do similar change for IAB

The CR is postponed

#### 7.0.2.3	NR support for UAV

(NR_UAV -Core; leading WG: RAN2; REL-18; WID: RP-230782 and LTE WID: RP-230783 )

R2-2410207	Correction on Band Selection and Cell Barring for Aerial UEs	Ericsson	CR	Rel-18	38.331	18.3.0	5149	-	F	NR_UAV-Core

-	LG agrees with first change but second change can cause a problem and is not needed.  Nokia agrees.

-	Nokia and qualcomm agree with intention but further check is needed

-	Samsung ask if it is possible for the UE to not support any bands in the aerial list.  Qualcomm explains that the intention is that the UE has to support one of them if they are included.   But if it doesn’t include a list you can depend on legacy list.    Nokia thinks that Ns values were optional.

-	Samsung asks what happens if the Ns value of one frequency is not supported by the UE but there is a second frequency that doesn’t have Ns value list.    Qualcomm thinks that if the UE supports that frequency but there is no Ns the UE would pick that frequency.    Samsung then indicates that the proposed text is not capturing properly.

[AT128][007][UAV] Cell barring (Ericsson)

Intended outcome: Agree to CR

Deadline:  Thursday

R2-2411120	Correction on Band Selection and Cell Barring for Aerial UEs	Ericsson	CR	Rel-18	38.331	18.3.0	5149	1	F	NR_UAV-Core

Update CR to remove changes on changes

The CR is agreed in R2-2411202 with changes above

#### 7.0.2.4 	Mobile Terminated Small Data Transmission

(NR_MT_SDT-Core; leading WG: RAN2; REL-18; WID: RP-222993)

R2-2409545	Correction on MT-SDT Capability for NR-NTN Case	vivo, ZTE Corporation, Sanechips, Huawei, HiSilicon, Ericsson	CR	Rel-18	38.306	18.3.0	1201	-	F	NR_MT_SDT-Core, NR_NTN_solutions-Core

The CR is agreed

R2-2410227	Correction to RRC for CG-SDT enhancement [CG-SDTenh]	Huawei, HiSilicon	CR	Rel-18	38.331	18.3.0	5152	-	F	TEI18, NR_MT_SDT-Core

Update the CR with some ASN.1 comments and agree by email

[POST128][011][SDT] CG-SDT CR (Huawei)

Intended outcome: review updates and agree by email

Deadline:  Thursday

R2-2411223	Correction to RRC for CG-SDT enhancement [CG-SDTenh]	Huawei, HiSilicon	CR	Rel-18	38.331	18.3.0	5152	1	F	TEI18, NR_MT_SDT-Core

=> Agreed

#### 7.0.2.5	IDC enhancements for NR and MR-DC

(NR_IDC_enh-Core; leading WG: RAN2; REL-18; WID: RP-221281)

#### 7.0.2.6	Mobile IAB (Integrated Access and Backhaul) for NR

( NR_mobile_IAB -Core; leading WG: RAN3; REL-18; WID: RP-232669)

R2-2409813	Miscellaneous corrections on BAP for IAB	Huawei, HiSilicon (Rapporteur)	CR	Rel-18	38.340	18.1.0	0040	-	D	NR_mobile_IAB-Core

The CR is agreed

#### 7.0.2.7	Timing Resiliency and URLLC Enh

(NR_TRS_URLLC; leading WG: RAN3; REL-18; WID: RP-230754)

#### 7.0.2.8	Enhanced support of reduced capability NR devices

(NR_redcap_enh-Core; leading WG: RAN1; REL-18; WID: RP-232671)

R2-2410611	Clarification on the definition of eRedCap UEs	Ericsson, Sequans Communications	CR	Rel-18	38.306	18.3.0	1197	1	F	NR_redcap_enh-Core	R2-2409205

-	Huawei thinks that we can just capture - TR 38.822 [24] as well as capabilities specified in this specification remain applicable for eRedCap UEs same as redcap other UEs, unless indicated otherwise.  Ericsson is concerned that the aspect related to MIMO part is not exactly the same.

-	Vivo agree with the CR and if we make Huawei’s change it implies that Rel-18 UE will have to support all the Rel-17 features.  Apple agrees

-	LG would like to follow original intention of CR.

The CR is agreed

#### 7.0.2.9	Further NR coverage enhancements

(NR_cov_enh2-Core; leading WG: RAN1; REL-18; WID: RP-221858)

LS

R2-2409513	LS on the time period for R18 preamble repetition (R1-2409262; contct: Huawei)	RAN1	LS in	Rel-18	NR_cov_enh2-Core	To:RAN2

Noted

R2-2410617	Discussion on LS from RAN1 on the time period for R18 preamble repetition	Huawei, HiSilicon	discussion	Rel-18	NR_cov_enh2-Core

Noted

FFS the level of details and how we provide it in the LS

Discussion

-	ZTE would like to provide more details in the LS.  Huawei thinks that the details in ZTE LS is not relevant to RAN1 LS.   LG thinks we can provide some level of details for some use case.   Ericsson and Vivo thinks we should just answer RAN1 question at high level.  Vivo thinks that RAN2 guys can internally share with RAN1 colleagues.

Proposal 4: For Q3, RAN2 confirms that in Case #2, a UE capable of msg1-repetition but incapable of RedCap will not consider the RACH resource and parameters indicated by feature combination msg1-repetition-r18+redCap-r17 according to the current RAN2 specifications.

Proposal 5: For Q4, RAN2 confirms that in Case #2, a UE capable of msg1-repetition and RedCap will not consider the RACH resource and parameters indicated by feature combination msg1-repetition-r18 according to the current RAN2 specifications.

Proposal 6: For Q5, RAN2 confirms that only the RACH resource and parameters corresponding to repetition numbers indicated for the sets of PRACH resources selected by the UE are considered as “configured” to the UE while all parameter repetition numbers indicated for all the other sets of PRACH resources as “not configured”.

R2-2410447	Discussion on RAN1 LS on time period in Msg1 repetition	ZTE Corporation	discussion	Rel-18	NR_cov_enh2-Core

Noted

R2-2410543	Discussion on RAN1 LS on time period	Ericsson	discussion	Rel-18	NR_cov_enh2-Core

Noted

R2-2410819	Discussion on RAN1 LS on the time period for R18 preamble repetition	LG Electronics Inc.	discussion	Rel-18	NR_cov_enh2-Core

Noted

Agreements

- For Q0, RAN2 confirms that Case#1 is not precluded, i.e. Repetition number n2 and n4 are configured in one RACH-ConfigCommon, while n8 is configured in another RACH-ConfigCommon.

- For Q1, RAN2 confirms that Case#2 is not precluded. Repetition number n2 and n4 are associated to one feature combination(i.e. msg1-repetition-r18), n8 is associated to another feature combination(i.e. msg1-repetition-r18+redCap-r17).

3	For Q2, RAN2 confirms that Case#1-Rev is not precluded

[AT128][008][CE] Response LS to RAN1 (Huawei)

Intended outcome: Agreed answers and LS

Deadline:  10-17-24

R2-2411167	Summary of [AT128][008][CE] Response LS to RAN1 (Huawei)	Huawei	discussion	Rel-18	NR_cov_enh2-Core

For Q3-Q5, from RAN2 perspective, the text “all the configured number of preamble repetitions” in Case#2 can be interpreted as Interpretation 1-2 in the RAN1 LS.

Noted

R2-2411168	Reply LS on the time period for R18 preamble repetition	Huawei	LS out	Rel-18	NR_cov_enh2-Core	To:RAN1

The LS is approved

#### 7.0.2.10	Network energy savings for NR

(Netw_Energy_NR-Core; leading WG: RAN1; REL-18; WID: RP-223540)

Rapporteur CR

R2-2410301	Network energy savings for NR rapporteur RRC CR	Huawei, HiSilicon	CR	Rel-18	38.331	18.3.0	5011	1	F	Netw_Energy_NR-Core	R2-2408443

Add architecture options

The CR is agreed in R2-2411094 with change above

Correction CRs

R2-2410531	Cell DRX MPS exemption in RA	Peraton Labs	CR	Rel-18	38.321	18.3.0	2007	-	F	Netw_Energy_NR-Core

-	ZTE explains that we clarified in stage 2 so the change is not needed.

-	Interdigital explains that in WI we didn’t make exceptions for high priority scheduling requests which is in the same category so we should follow what we agreed.

-	Huawei thinks that in stage 2 we explain network behaviour and in stage 3 we explain UE behavior.   We had this discussion in the past and we deliberately left out MPS as it is a priority service.

-	Apple also doesn’t think we need stage 3.

-	Peraton would like to have a discussion on this as they think that this can be categorized as public safety. Huawei explains that in SA1 it is not defined as public safety and MPS is not in this category. Peraton agrees it is not public safety but it is a public safety related service.    ZTE explains that it is not RAN2 job to clarify, and it should be first specified in SA1.

-	Qualcomm has some sympathy since it is related to government services but the problem is that it is not a correction but rather a new feature.

-	Xiaomi also thinks that this is a new function and if we consider Priority ID 1 then we should consider Priority ID 2

-	LG agrees that we don’t need the change as we agreed to not handle priority services.

-	Interdigital thinks that we have some tools as you can send RACH even if you are in DRX.

After CB

-	ZTE thinks that we can use same wording as in stage 2, for example if UE initiates public related services.   Apple thinks that we shouldn’t use public service wording as it is not clear in stage 3.

RAN2 assumes that MPS and MCS are public safety related services.

Update the CR to capture both MPS and MCS

R2-2411209	Cell DRX MPS exemption in RA	Peraton Labs	CR	Rel-18	38.321	18.3.0	2007	1	F	Netw_Energy_NR-Core

The CR is agreed

R2-2410169	Miscellaneous corrections for NES	 ZTE Corporation, Sanechips	CR	Rel-18	38.331	18.3.0	5146	-	F	Netw_Energy_NR-Core

-	Huawei thinks that the definition of scrambling ID is already captured in RAN1 so we don’t need that part.   The rest is good.    LG agrees with Huawei.

-	Xiaomi thinks that RE to EPRE is not related to Rel-18 NES, this is from Rel-15.  ZTE thinks that the power offset is related to NES, but if we want to make the RE to EPRE change for rel-15 it can be considered.   Huawei thinks that at least for Rel-18 RAN1 indicates that the is EPRE, so we can agree to that.

Remove definition of definition scrambling ID

Add architecture option

The CR is agreed with the changes above in R2-2411104, but then the CR was coversheet revised by MCC in R2-2411260: wrong rev number

R2-2411260	Miscellaneous corrections for NES	 ZTE Corporation, Sanechips	CR	Rel-18	38.331	18.3.0	5146	2	F	Netw_Energy_NR-Core

=> Agreed

SSB-Less

R2-2409693	Measurements on the carrier of SSB-less SCell	NEC	discussion	Rel-18

Proposal-1: Adopt the same handling of servingCellMO configuration for both R18 inter-band SSB-less and R15 intra-band contiguous SSB-less SCell operation.

Proposal-2: Capture “For both inter-band and intra-band SSB-less SCell(s), this field is not present” in the conditional presence of servingCellMO of the IE structure BWP-DownlinkDedicated and ServingCellConfig. (see Text proposal in Annex).

Proposal-3: P1 and P2 can be checked according to concurrent RAN4 discussions during the upcoming meeting.

Noted

R2-2410259	SSBLess handling	Nokia	discussion	Rel-18	Netw_Energy_NR-Core

Proposal 1:In the MO indicated by ServingCellMO, network may configure ssbFrequency to indicate a carrier frequency for the SSB-less SCell so that UE can determine measurement as intra-frequency or inter-frequency measurement based on the ssbFrequency.

Noted

Wait for next meeting to make decision

#### 7.0.2.11	Further enhancement of data collection for SON MDT in NR and EN-DC

(NR_ENDC_SON_MDT_enh2-Core; leading WG: RAN3; REL-18; WID: RP-221825)

R2-2409647	Corrections on SPR configuration release and logging PSCell identity	CATT	CR	Rel-18	38.331	18.3.0	5105	-	F	NR_ENDC_SON_MDT_enh2-Core

On first change:

-	Samsung thinks this reflects an earlier agreement and sees no issue with current spec. Ericsson think the spec is not broken but are OK to do the change to make the spec clearer. Huawei think this change is not needed, it is OK that there is a duplication. LG agrees with the other companies that nothing is broken and want to leave the spec as is. Nokia agrees with LG.

On the second change:

-	Samsung think this is a new scenario and it is too late now. CATT think that the field description (shown on the cover page) dictates that we should do this change. Ericsson agrees with CATT and are OK with this change. Huawei is OK on this.

-	LG wonders if the current spec text is not intentional? Nokia think that this is a functional change and are not OK with it. Samsung think it is correct but its too late and this is not an essential correction. CATT think that a previous RAN3 LS suggests that the behaviour is as described by this CR. ZTE agrees with CATT that we should do this based on what RAN3 suggested earlier. Samsung need more time to check this.

The first change (release of successPSCell-Config) is not pursued as RAN2 understands that this duplicating of release is not causing any issues.

Discuss the second change (setting of the pSCellId) offline.

[AT128][602][SONMDT] Corrections on SPR configuration (CATT)

Scope:

    - Produce agreeable CRs if possible

Intended outcome:

    - Agreed CR R2-2411036 (Company)

Deadline:

    - Wednesday 17:00. The intention is to agree the CRs over email.

    - In time for the Thursday session

R2-2411036	Correction on logging PSCell identity CATT CR Rel-18 38.331 18.3.0 5105 1 F NR_ENDC_SON_MDT_enh2-Core

Agreed

R2-2409648	Corrections on RA Report retrieval	CATT	CR	Rel-18	37.340	18.3.0	0405	-	F	NR_ENDC_SON_MDT_enh2-Core

-	Samsung and ZTE think this text comes from RAN3 and should be discussed there if needed. Huawei does not agree with the changes and agree that RAN3 should do changes to this part of the spec if needed.

Not pursued by RAN2, RAN3 may discuss if they want

R2-2409986	NR-U related RA information in SCGFailureInformation	Nokia	CR	Rel-18	38.331	18.3.0	5126	-	F	NR_ENDC_SON_MDT_enh2-Core

-	Samsung are OK with the intention but the ASN.1 is wrong, it should be “v18” instead of “r18xx” and impacted arch options are missing.

-	Ericsson are OK to add perRAInfoList but not numberOfLBT-Failures since that is new behaviour.

-	ZTE think we intentionally agreed to not support this, but if we do something then only adding the perRAInfoList is enough. Nokia think the numberOfLBT-Failures is also useful and if we don’t add it we need clarifying the procedural text. ZTE are open to clarify in the procedural text.

We aim to add perRAInfoList to ASN.1 and update the procedural text to clarify the behaviour for numberOfLBT-Failures. Also add impacted arch options and fix ASN.1 (v18). CRs to be produced offline

[AT128][603][SONMDT] NR-U related RA information in SCGFailureInformation (Nokia)

Scope:

    - Produce agreeable CRs

Intended outcome:

    - Agreed in principle CR in R2-2411014

Deadline:

    - Wednesday 17:00. The intention is to agree the CRs over email.

R2-2411014	NR-U related RA information in SCGFailureInformation	Nokia	CR	Rel-18	38.331	18.3.0	5126	1	F	NR_ENDC_SON_MDT_enh2-Core

R2-2411165	Correction in the optionality of NR-U related information	Nokia	CR	Rel-18	38.306	18.3.0	1221	-	F	NR_ENDC_SON_MDT_enh2-Core

Both are agreed

R2-2410045	Correction on Inter-RAT SHR	Samsung	CR	Rel-18	38.331	18.3.0	5137	-	F	NR_ENDC_SON_MDT_enh2-Core

-	CATT, Ericsson and Nokia think that the last bullet 1> in 5.7.10.6 can be removed instead. Nokia think that the proposed change in this CR is a functional enhancement. Samsung wants to postpone. Ericsson also want to postpone.

Postponed

R2-2410057	Correction on logging RSSI measurements in RLF report and SHR	Samsung	CR	Rel-18	38.331	18.3.0	5138	-	F	NR_ENDC_SON_MDT_enh2-Core

NE-DC should be added as impacted architecture options. With that the CR is agreed unseen in R2-2411011.

#### 7.0.2.12	Dual Transmission/Reception (Tx/Rx) Multi-SIM for NR

(NR_DualTxRx_MUSIM-Core; leading WG: RAN2; REL-18; WID: RP-233071)

R2-2409994	Clarification of UE capability restrictions in MUSIM	Ericsson, Samsung, Xiaomi, LG Electronics Inc., Vivo	CR	Rel-18	38.300	18.3.0	0920	2	F	NR_DualTxRx_MUSIM-Core	R2-2409224

Include NE-DC and NR-DC in architecture option

Update CR number to correct one

The CR will be update to merge content of R2-2410350

R2-2411105	Clarification of UE capability restrictions in MUSIM	Ericsson, Huawei, HiSilicon, Samsung, Xiaomi, LG Electronics Inc., Vivo	CR	Rel-18	38.300	18.3.0	0920	3	F	NR_DualTxRx_MUSIM-Core

The CR is agreed

R2-2410350	Corrections for MUSIM in 38.300	Huawei, HiSilicon, Samsung, vivo	CR	Rel-18	38.300	18.3.0	0931	-	F	NR_DualTxRx_MUSIM-Core

The CR will be merged with R2-2409994

[AT128][009][MUSIM] 38.300 CR (Ericsson)

Intended outcome: merge two CRs, review and agree by email

Deadline:  Thursday

R2-2411105	Clarification of UE capability restrictions in MUSIM	Ericsson, Huawei, HiSilicon, Samsung, Xiaomi, LG Electronics Inc., vivo	CR	Rel-18	38.300	18.3.0	0920	3	F	NR_DualTxRx_MUSIM-Core

The CR is agreed

#### 7.0.2.13	NR MIMO evolution

(NR_MIMO_evo_DL_UL-Core; leading WG: RAN1; REL-18; WID: RP-233028)

LS on two TA

R2-2409510	LS on TDD UL/DL Configuration for Two TA (R1-2409179; contact: Ericsson)	RAN1	LS in	Rel-18	NR_MIMO_evo_DL_UL-Core	To:RAN2

Noted

R2-2410397	On LS on TDD UL/DL Configuration for Two TA	Ericsson	discussion	Rel-18	NR_MIMO_evo_DL_UL-Core

Noted

Proposal 1	Agree to add configuration for TDD-UL-DL-ConfigCommon in IE BWP-UplinkCommon s shown in TP1.

Proposal 2	Clarify in 38.306 that a UE supporting capability multiDCI-InterCellMultiTRP-TwoTA-r18 also supports TDD UL/DL configuration.

DISCUSSION

P1

- ZTE agree with the intention but think TP has some problems in filed description.

- CATT think it is better to ext the existing additionalRACH-perPCI-ToAddModList-r18. Ericsson think it may be misleading to have tdd ul dl config parameters in RACH configuration.

- CATT think if we use Ericsson proposal then maxNrofAdditionalTDDConfigs is not defined.

P2

- ZTE think it is not needed and think this is detail.  QC, LG E agree.

Intention of P1 is agreeable, maxNrofAdditionalTDDConfigs needs to be added. CR will be discussed offline and checked in CB session.

P2 is not pursued.

[AT128][204][MIMOevo] RRC CR on TDD UL/DL Configuration for Two TA (Ericsson)

Intended outcome: Agreeable CR on TDD UL/DL Configuration for Two TA in R2-2410954.

Deadline: Before CB

R2-2410954	Correction on TDD UL/DL Configuration for Two TA	Ericsson CR	Rel-18	38.331	18.3.0	5196	-	F	NR_MIMO_evo_DL_UL-Core

DISCUSSION

- Ericsson think this is in general agreeable, but companies can further check the details due to late uploading.

[POST128][008][MIMOevo] RRC CR on TDD UL/DL Configuration for Two TA (Ericsson)

Intended outcome: Agree the CR

Deadline: short

=> Postponed

PHR

Chair: plan is to discuss PHR related contributions in offline

R2-2410528	Correction on PHR for MIMO STx2P multi-panel scheme	Samsung, LG, Huawei, CATT, Ericsson	CR	Rel-18	38.321	18.3.0	1959	1	F	NR_MIMO_evo_DL_UL-Core	R2-2409024

The CR is agreed.

R2-2410175	Correction on PHR for MIMO	ASUSTeK	CR	Rel-18	38.321	18.3.0	1991	-	F	NR_MIMO_evo_DL_UL-Core

The CR is not pursued.

R2-2410625	Clarification to 38.321 on R17 PHR MAC CE for mTRP PUSCH Repetition	ZTE Corporation	CR	Rel-18	38.321	18.3.0	2008	-	F	NR_MIMO_evo_DL_UL-Core

The CR is postponed.

[AT128][201][MIMOevo] Proposals/CRs for PHR (Samsung)

Scope: Discuss PHR related proposals for MIMOevo

Intended outcome: Proposals in R2-2410951 for CB, updated CR(s) in R2-24xxxxx.

Deadline: Before CB.

R2-2410951 Report of [AT128][201][MIMOevo] Proposals/CRs for PHR	Samsung	discussion	Rel-18	NR_MIMO_evo_DL_UL-Core

Noted

Proposal 1: Change 1 in R2-2410175 is not needed. Discuss whether Change 2 is needed.

Proposal 2: RAN2 understands a MAC entity is not be configured with twoPHRmode if in this MAC entity no serving cell is configured with Rel-17 mTRP PUSCH repetition or Rel-18 mTRP STx2P multi-panel scheme.

Proposal 3: Agree CR R2-2410528.

DISCUSSION

P1

- LG E think change 2 is right but not essential.

P2

- CATT think ZTE has a point and think we can postpone. LG E think it is meaningful to hear more views form other NW vendors. Samsung think we can CB next meeting.

- QC do not think this is valid issue. Ericsson do not think this is an important issue.

8Tx

R2-2410173	Discussion on supporting 8Tx in MAC specification	ASUSTeK	discussion	Rel-18	NR_MIMO_evo_DL_UL-Core

Noted

R2-2410174	Correction on supporting 8Tx in MAC specification	ASUSTeK	CR	Rel-18	38.321	18.3.0	1990	-	F	NR_MIMO_evo_DL_UL-Core

Not treated

R2-2410624	Harmonization of 8Tx in MAC specification	ZTE Corporation	discussion	Rel-18	NR_MIMO_evo_DL_UL-Core

=> Revised in R2-2411076

R2-2411076	Harmonization of 8Tx in MAC specification ZTE Corporation	discussion	Rel-18	NR_MIMO_evo_DL_UL-Core

Noted

- Modelling method#1: For 8Tx, One uplink grant and One HARQ process for 2TB transmission (keep the agreements achieved in RAN2#127bis meeting as it is )

- Modelling method#2: For 8Tx, Two uplink grants Two HARQ processes with one HARQ process Id for 2TB transmission.

DISCUSSION

- Ericsson support ASUSTeK proposal and think it is similar in DL.

- LG E think there is one more option, 2 grant, 1 harq process and 2 harq buffer, and think if 2 TB is generated and then 2 HARQ buffer are needed. Xiaomi agree, and think for separate HARQ retransmission we need 2 HARQ process.

- QC think R1 want to have 2 TB and we need 2 HARQ buffer and think in this case it is better to have 2 HARQ process (so agree with ZTE method #2). QC think in R1, DCI only contains 1 HARQ ID and it is up to UE implementation how to associate the ID with the 2 HARQ process.

- CATT think 2 HARQ buffer is needed, and regarding modelling CATT think both methods work.

- CATT think the problem is from R1 point of view there is only one HARQ ID and this needs careful review (300, RRC, etc.). Ericsson wonders what is the RRC change. CATT think in RRC we now have # of UL HARQ processes so there may be impact. HW think there is no RRC impact.

- HW think it is important to make it as simple as possible, and think method 2 is better. Apple agree. Nokia agree.

- MAC rapp think method 2 is better to minimize the MAC spec impact.

- Ericsson think currently it is one DCI scheduling the PUSCH so it is one grant.

- ?? For 8Tx 2TB transmission, we assume the following from MAC point of view: two uplink grants for two HARQ processes, one HARQ process ID, 2 HARQ buffer. Detailed changes to MAC will be discussed in offline.

[AT128][202][MIMOevo] Discuss on the modelling and review the MAC CR for 8Tx 2TB if the modelling is agreeable (ASUSTeK, ZTE)

Scope: Discuss the modelling, and if there is an agreeable modelling, prepare and review the MAC CR. Can also check if any change to RRC is needed.

Intended outcome: Summary in R2-2410952 and agreeable CR in R2-24xxxxx.

Deadline: Before CB

R2-2410952	Report of [AT127bis][202][MIMOevo] Discuss on the modelling and review the MAC CR for 8Tx 2TB if the modelling is agreeable	ASUSTeK, ZTE	discussion	Rel-18	NR_MIMO_evo_DL_UL-Core

Noted

The following is used as baseline for drafting an LS to R1

- Q1

- RAN2 made the following assumption regarding how to model UL 8Tx 2TB in MAC:

- For 8Tx 2TB transmission, we assume the following from MAC point of view: two uplink grants scheduled by one DCI for two HARQ processes, one HARQ process ID, 2 HARQ buffer.

- Ask for RAN1’s feedback on the above assumption

- Q2

- Ask RAN1 whether ensuring a 2-TB generation is needed in NR 8Tx

[POST128][006][MIMOevo] LS on 8Tx 2TB  (Samsung)

Intended outcome: Approve the LS

Deadline: short

R2-2411220	LS on UL 8Tx	RAN2	LS out	Rel-18	NR_MIMO_evo_DL_UL-Core	To:RAN1

=> Approved

On simultaneousU-TCI-UpdateListx

R2-2409715	Correction on simultaneousU-TCI-UpdateListx	CATT, Ericsson	CR	Rel-18	38.331	18.3.0	5111	-	F	NR_MIMO_evo_DL_UL-Core

Send LS to R1 to ask how to differentiate the different cases of sDCI mTRP, mDCI mTRP and sTRP

CR is postponed

[AT128][203][MIMOevo] LS on differentiation of sDCI mTRP, mDCI mTRP and sTRP (CATT)

Intended outcome: Draft LS in R2-2410953.

Deadline: Before CB

R2-2410953	[Draft] LS on differentiation of sDCI mTRP, mDCI mTRP and sTRP	CATT	discussion	Rel-18	NR_MIMO_evo_DL_UL-Core

Noted

DISCUSSION

- HW, ZTE think Q2 part is not so clear.

- Samsung think we can further discuss the Q2 and we can have a short email disc.

[POST128][007][MIMOevo] LS on differentiation of sDCI mTRP, mDCI mTRP and sTRP  (CATT)

Intended outcome: Approve the LS

Deadline: short

R2-2411244	LS on differentiation of sDCI mTRP, mDCI mTRP and sTRP	RAN2	LS out	Rel-18	NR_MIMO_evo_DL_UL-Core	To:RAN1

=> Approved

#### 7.0.2.14	Enhancements of NR Multicast and Broadcast Services

(NR_MBS_enh-Core; leading WG: RAN2; REL-18; WID: RP-231829)

R2-2409599	Correction on Group Paging Handling	CATT, CBN	CR	Rel-18	38.331	18.3.0	5129	-	F	NR_MBS_enh-Core	Withdrawn

R2-2409756	Misc correction on NR MBS enhancement	ZTE Corporation, Sanechips	CR	Rel-18	38.331	18.3.0	5114	-	F	NR_MBS_enh-Core

- ZTE suggests to focus on the first change.

- Ericsson does not see a strong need to clarify as this is a corner case. Current spec should be clear enough.

- Samsung agrees with the change, perhaps it would be better to say “configured to receive”.

- CATT agrees with the change, it seems a valid case.

- QCM can accept the change, but we need to understand where exactly we need to change.

The intention of the change 1 from the coversheet in R2-2409756 is agreeable

ZTE to bring the CR to the next meeting, covering all the places where this change needs to be applied

The other changes are not pursued

R2-2409939	Clarification on group paging procedure	Samsung	CR	Rel-18	38.331	18.3.0	5134	-	F	NR_MBS_enh-Core

Revised in R2-2411008

- Ericsson does not see a strong need for this clarification. Ericsson thinks the current text is clear enough.

- Samsung clarifies that this condition is only relevant for sessions which are in group paging, not all sessions the UE is configured with.

- Huawei understands the intention, but it seems it can be derived from the context.

- ZTE, Nokia is OK with the CR, it is better to make it clear.

- Ericsson thinks in other places we do not specify this way and it is not needed for tis case either. If we do it here, then we need to change other places

Offline

R2-2411008	Clarification on group paging procedure	  Samsung, Ericsson, Huawei, HiSilicon, ZTE, Nokia	CR	Rel-18	38.331	18.3.0	5134	1	F	NR_MBS_enh-Core

CR is agreed

R2-2410254	Multicast reception after reselection to cell with MCCH	Nokia, Samsung, Ericsson, ZTE	CR	Rel-18	38.331	18.3.0	5153	-	F	NR_MBS_enh-Core

Update the CR by adding “and for which the UE was indicated to stop monitoring G-RNTI;” at the end of the current change

Clarify it refers to the new cell

Add impacted architecture options

[AT128][505][MBS] Update R2-2410254  (Nokia)

Scope: Update R2-2410254 and make final review

Intended outcome: Agreeable CR in R2-2411004

Deadline: CR ready for offline agreement: Friday 2024-11-22 09:00

R2-2411004 Multicast reception after reselection to cell with MCCH	Nokia, Samsung, Ericsson, ZTE	CR	Rel-18	38.331	18.3.0	5153	1	F	NR_MBS_enh-Core

Revised in R2-2411009

R2-2411009 Multicast reception after reselection to cell with MCCH	Nokia, Samsung, Ericsson, ZTE	CR	Rel-18	38.331	18.3.0	5153	2	F	NR_MBS_enh-Core

CR is agreed, but then coversheet revised by MCC in R2-2411258: clauses affected empty

R2-2411258 Multicast reception after reselection to cell with MCCH	Nokia, Samsung, Ericsson, ZTE	CR	Rel-18	38.331	18.3.0	5153	3	F	NR_MBS_enh-Core

=> Agreed

R2-2410630	Correction on applying of PTM configuration in Paging procedure	SHARP Corporation	CR	Rel-18	38.331	18.3.0	5176	-	F	NR_MBS_enh-Core

Not pursued

- Ericsson indicates we did not specify when exactly the UE stores the configuration, so we do not have to specify in this case either.

- Samsung, CATT thinks the change is not needed as it is already clear.

R2-2410876	Correction on Group Paging Handling	CATT, CBN, China Broadnet	CR	Rel-18	38.331	18.3.0	5194	-	F	NR_MBS_enh-Core

Change the reference before the removed text from “5.3.13” to “5.3.13.1d”. Otherwise, the change number 1 from R2-2410876 is agreeable.

With the change above, the CR is agreed in R2-2411005

For the first change:

- Samsung also indicates that in case the UE receives both group Paging and MT-SDT, the UE will also set resume cause twice. Samsung suggests that in this case we could also refer to a common section to avoid duplicity.

- Chair: Companies are encouraged to check whether for MT-SDT and MBS we also have a duplicity issue and whether it needs to be corrected.

For the second change:

- CATT clarifies that the config received for deactivated session will not be applied when received, so it needs to be applied after Paging.

- Huawei asks what happens if there is no configuration to be applied.

- Nokia agrees with the intention and thinks we need to check other parts of specs.

- Ericsson asks for clarification about the intention. Wonders about usefulness of applying the configuration which will be overwritten by the one in MCCH anyway.

- CATT thinks there are other cases, e.g. the configuration is not present in MCCH.

#### 7.0.2.15	Enhancement on NR QoE management and optimizations for diverse services

(NR_QoE_enh-Core; leading WG: RAN3; REL-18; WID: RP-223488)

R2-2410654	Correction on UE behaviours when reporting SRB is modified	Huawei, HiSilicon	CR	Rel-18	38.331	18.3.0	5178	-	F	NR_QoE_enh-Core

- Ericsson thinks HO case is different because the UE disconnects from the source node. With SRB change case the UE can still report. Ericsson does not think the change is needed

- QCM agrees with the intention of the CR. QCM wonders if the already received segments can be forwarded between MN and SN?

- Huawei think MN and SN need to coordinate. Huawei asks if Ericsson assumes the UE will finalize the reporting over the old SRB? Ericsson confirms.

- Samsung thinks there is no forwarding of segments between MN and SN. Samsung thinks the reporting SRB should be changed if there is no reporting ongoing.

- Ericsson think there can be some collision cases.

- QCM does not think the UE can send something over the non-existing configuration. Supports Samsung suggestion that the command is not sent from the network in the middle of reporting.

- Nokia thinks that when SRB is changed, the old SRB can be removed. Supports suggestion from Samsung.

- Huawei is also OK with Samsung suggestion.

- QCM is OK with suggestion from Samsung and thinks it should be captured in stage-2.

This agreement was superseded by the conclusion from offline [503]:

“1. The issue described in R2-2410654 can be avoided by network implementation, i.e. the network may avoid changing the reporting SRB for QoE if there is ongoing QoE reporting. FFS whether the network may keep the previous SRB even when changing the reporting SRB to let the UE finish the ongoing reporting over the old SRB

2. Capture this in stage-2.”

[AT128][503][QoE] QoE reporting SRB change issue (Huawei)

Scope: Discuss the FFS and stage-2 CR

Intended outcome: Agreeable proposal and stage-2 CR

Deadline:  Report and CR ready for discussion during CB session on Thursday

R2-2411006 Report of [AT128][503][QoE] QoE reporting SRB change issue (Huawei), Huawei, HiSilicon

The following replaces previous agreement:

    - If there is an ongoing transmission of a MeasurementReportAppLayer and the network wants to change the reporting SRB, the network may keep or release the previous SRB by implementation. This has no specifications impact.

R2-2410659	Miscellaneous correction on QoE measurement	Samsung	CR	Rel-18	38.331	18.3.0	5179	-	F	NR_QoE_enh-Core

Add impacted architecture options.

With this the CR is agreed in R2-2411003

- QCM thinks the 1st change is not needed as there is no need to report measIDs in the case where UE goes to IDLE.

- Ericsson thinks it is useful to have the indication to upper layers anyway.

- Nokia, Huawei, Ericsson agrees with all changes.

#### 7.0.2.16	XR Enhancements for NR

(NR_XR_enh-Core; leading WG: RAN2; REL-18; WID: RP-230786)

R2-2410249	Correction to multi-PUSCH configured grant	Huawei, HiSilicon, Qualcomm, Apple, vivo, Nokia, Nokia Shanghai Bell	CR	Rel-18	38.321	18.3.0	2002	-	F	NR_XR_enh-Core

Move impact analysis to summary section

The CR is agreed in R2-2411106

R2-2411067	Correction on DSR cancellation	CMCC	draftCR	Rel-18	38.321	18.3.0	F	NR_XR_enh-Core

-	Lenovo doesn’t think the network dynamically changes the threshold, so this is more of an optimization.   LG also thinks this is an optimization and not needed.

-	Qualcomm agrees with the intention but the change should be restricted to case when triggering threshold becomes larger.

-	Huawei doesn’t think it is need, if it is changed then then network will miss a DSR.   Nokia, Ericsson agrees with Huawei and LG and think it is a corner cases.

-	Lenovo explains that only concequence of not agreeing to this CR is an additional MAC CE DSR so it is not a bit problem.

Missing impact analysis

The CR is not pursued

R2-2410729	Correction for Delay Critical Indication from PDCP to RLC	Samsung, LG Electronics Inc., Nokia, Huawei, HiSilicon, Ericsson	CR	Rel-18	38.323	18.3.0	0144	-	F	NR_XR_enh-Core

-	Huawei thinks that ‘if’ should be ‘when’.   Qualcomm agrees as these triggers are event triggered.  Samsung indicates that the intention was to align with legacy text as much as possible.

Update CR from ‘if’ to ‘when’ and add architecture

The CR is agreed in R2-2411107 with changes above

R2-2409852	Further Discussion on the SN Gap Report	CATT	discussion	Rel-18	NR_XR_enh-Core

Proposal 1: RAN2 to confirm which of the following two options is the correct understanding:

-	Option 1: The discarded PDCP SDU included in the PDCP SN gap report should not include the PDCP SDU(s) which have been delivered to lower layer by RLC.

-	Option 2: The discarded PDCP SDU included in the PDCP SN gap report can include in the PDCP SDU(s) which have been delivered to lower layer by RLC.

Proposal 2: If Option 1 is selected in Proposal 1, suggest RAN2 to discuss whether specification modification is needed or not.

Proposal 3: If Option 2 is selected in Proposal 1, no specification effort is needed.

-	CATT thinks that we should confirm option 2

-	Nokia thinks that the understanding should be option 1.   Vivo thinks that option 1 the receiver would have to wait so it will incur delays.

-	LG thinks its option 2 as it was the original intention.   Huawei thinks it is 2.

Come back next meeting

Noted

#### 7.0.2.17	Others

Including Multi-WI Rel-18 items, e.g. cross-WI-issues not handled under another WI

Corrections CRs

R2-2410000	Miscellaneous non-controversial corrections Set XXIII	Ericsson	CR	Rel-18	38.331	18.3.0	5083	1	F	NR_newRAT-Core, TEI18	R2-2409125

Update based on further corrections from this meeting

[POST128][004][RRC] Rapporteur correction (Ericsson)

Intended outcome: Review and agree to all CRs from R15 (if applicable), R16, R17, and 18

Deadline:  short

R2-2411224	Miscellaneous non-controversial corrections Set XXIII	Ericsson	CR	Rel-15	38.331	15.27.0	5127	1	F	NR_newRAT-Core	R2-2409997

=> Agreed

R2-2411225	Miscellaneous non-controversial corrections Set XXIII	Ericsson	CR	Rel-16	38.331	17.18.0	5128	1	F	NR_newRAT-Core, TEI16	R2-2409998

=> Agreed

R2-2411226	Miscellaneous non-controversial corrections Set XXIII	Ericsson	CR	Rel-17	38.331	17.10.0	5082	2	F	NR_newRAT-Core, TEI17	R2-2409999

=> Agreed

R2-2411227	Miscellaneous non-controversial corrections Set XXIII	Ericsson	CR	Rel-18	38.331	18.3.0	5083	2	F	NR_newRAT-Core, TEI18	R2-2410000

=> Agreed

R2-2410116	Correction on UE receiver features	China Telecom	draftCR	Rel-18	38.306	18.3.0	F	NR_demod_enh3-Core

-	Huawei indicates that current wording is from RAN4 feature list and they don’t use that wording on type 1 and type 2.

Need a CR number and increase rev. 0 (CR 1223) and architecture option

The CR is agreed in R2-2411203 with changes above

R2-2410457	Coexistence of NCR and CPAC with (e)RedCap	Ericsson, ZTE Corporation	CR	Rel-18	38.300	18.3.0	0937	-	F	NR_Mob_enh2-Core, NR_netcon_repeater, NR_redcap_enh-Core

-	Huawei doesn’t think this is needed as it is clear in 306.  Ericsson thinks that we need be consistent.

-	ZTE thinks that we should be aligned with IAB and since we have the description for IAB we should add NCR.    Samsung thinks we should add NCR.

-	Samsung and ZTE Think we should remove the CPC and CPA so we don’t get further clarification CR

-	Nokia asks what about LTM.  Ericsson has another CR

Further alignment on this restriction for (e)redcap capability is not encouraged for Rel-18

The CR is agreed

R2-2410696	Transmit operation of SR and SN gap report	Nokia, Nokia Shanghai Bell	CR	Rel-18	38.323	18.3.0	0143	-	F	NR_SL_enh2-Core, NR_XR_enh-Core

-	Oppo and Huawei thinks the change is not needed

-	LG thinks the first change is not agreeable as it causes a problem

-	Second change may be need if we have gap reporting for SL.

The Tdoc number and architecture options are missing

The CR is postponed

R2-2410758	Clarification on SIBs validity	Google Korea LLC	CR	Rel-18	38.331	18.3.0	5185	-	F	NR_SL_enh-Core, NR_UE_pow_sav_enh-Core, TEI18, NR_pos_enh2

-	Mediatek explains that the added SIBs are segmented and those segments are discarded during cell reselections.  But it doesn’t mean that the SIBs can’t be used again.   Ericsson agrees and that sentence is for the SIBs that are not under value tag control and the new ones added are.  Samsung also agrees with Ericsson and mediatek

The CR is not pursued

R2-2410219	Correction to co-existence of multi-PUSCH CG and CG-SDTenh [CG-SDTenh]	Huawei, HiSilicon	CR	Rel-18	38.321	18.3.0	1997	-	F	NR_XR_enh-Core, TEI18	Revised

R2-2410820	Correction to co-existence of multi-PUSCH CG and CG-SDTenh [CG-SDTenh]	Huawei, HiSIlicon	CR	Rel-18	38.321	18.3.0	1997	1	F	TEI18, NR_XR_enh-Core	R2-2410219

-	ZTE, Nokia and Ericsson thinks nothing is broken but if we make a change we should indicate in RRC that these two shouldn’t be configured together.

-	Samsung supports the CR and the two features shouldn’t be configured together.   LG agreed and we copied the text and this is the root cause of the problem.

-	Huawei thinks that if we don’t expect them to be configured together it should be clear.

-	Nokia is not sure what is the issue if they are configured together.

-	ZTE agrees but there are other features that shouldn’t be configured together so don’t want to see other CRs.

-	Apple has no strong view but if it is clear that they shouldn’t be configured together.

The CR is agreed

RAN2 confirms that CG-SDT periodicity extension and multi-PUSCH CG are not be configured together.

LTM + (e)RedCap

R2-2410446	On supporting LTM for (e)RedCap UEs	ZTE Corporation	discussion	Rel-18	NR_Mob_enh2-Core, NR_redcap-Core, NR_redcap_enh-Core

Proposal 1	RAN2 confirms that LTM can be supported for (e)RedCap UEs, no spec change is foreseen.

Noted

R2-2410458	Coexistence of LTM with RedCap	Ericsson	discussion	Rel-18	NR_Mob_enh2-Core, NR_redcap_enh-Core

Proposal 1	RAN2 to confirm that LTM can be used for (e)RedCap UEs by clarify the following:

a.	If the SSB frequency within the LTM-SSB-Config IE is the same as the SSB frequency within the NonCellDefiningSSB IE, this means the SSB is a NCD-SSB.

b.	If an additional RACH resources are configured, the UE shall use such resources only when performing an LTM cell switch procedure, and if CFRA resources are not provided within the LTM cell switch MAC CE.

Noted

Discussion

-	Ericsson indicates that from RAN2 point of view it can work even if RAN4 doesn’t have any requirement.  Apple agrees.

-	Vivo agrees but we can agree with Huawei’s CR.

-	Xiaomi is not sure this can be supported if there is no RAN4 requirement.   ZTE thinks this is not an issue as RAN4 can have some requirement in the future.

-	Qualcomm asks if we checked that it works given the per band capability.  ZTE thinks that for this case all existing capabilities are applicable for this use case.

- Agreement

RAN2 confirms that MCG LTM can be supported for (e)RedCap UEs in current specification, no spec change is foreseen in order to support the scenario.  It is up to RAN4 when and whether requirements for this use case are ever introduced.

R2-2410516	Correction to LTM MAC CE based CFRA with MSG1 repetition	Huawei, HiSilicon	CR	Rel-18	38.321	18.3.0	1969	1	F	NR_Mob_enh2-Core, NR_cov_enh2-Core, NR_redcap-Core, NR_redcap_enh-Core	R2-2409139

-	ZTE thinks that nothing is broken with current specification. ZTE thinks that if we were to do a change the first change in NOTE1 from Vivo is better.

-	Ericsson thinks that we can just update NOTE1 to include all these considerations.

Revised and reviewed by email

R2-2409777	Coexistence of (e)RedCap UEs and LTM and other MAC corrections	vivo	discussion	Rel-18	NR_Mob_enh2-Core, NR_redcap_enh-Core, NR_redcap-Core

Merged with R2-2410516

Merge the two CRs and identify wording improvements by combining the content of NOTE2 and 3 in NOTE 1.

[AT128][005][RedCapLTM] CR  (Huawei)

Intended outcome: Agree to CR by email

Deadline:  Thursday

R2-24211212	Correction to LTM MAC CE based CFRA with MSG1 repetition	Huawei, HiSilicon	CR	Rel-18	38.321	18.3.0	1969	2	F	NR_Mob_enh2-Core, NR_cov_enh2-Core, NR_redcap-Core, NR_redcap_enh-Core

The CR is agreed

R2-2409923	Clarification on LTM capabilities	Google	CR	Rel-18	38.306	18.3.0	1183	1	F	NR_redcap-Core, NR_Mob_enh2-Core, NR_redcap_enh-Core	R2-2408756

-	ZTE indicates that we agree to not add more clarification and MR DC covers this case.

-	Qualcomm thinks that in the context of MR DC it is not clear whether it cover NR DCs in our specifications.   So for 306 we should include NR DC.  ZTE thinks that this is clear in the stage 2 but if we want to make this clear we should add this clarification at the higher lever of the 306 to cover all cases.   Vivo thinks that we need to check one by one.   Samsung thinks in RRC specification it is quite clear.

The CR is not pursued

## 7.1	Expanded and improved NR positioning

(NR_pos_enh2; leading WG: RAN1; REL-18; WID: RP-232670)

Time budget: 0 TU

Tdoc Limitation: 2 tdocs

Minor and editorial issues should be coordinated with the appropriate spec rapporteur and submitted by rapporteur company together with any additional corrections the rapporteur company may have.    Larger issues can be discussed based on contributions/individual CRs.

### 7.1.0	In-principle agreed CRs

Contributions agreed in principle at RAN2#127bis.

Unchanged (as far as the chair is aware) from AIP version

R2-2409567	Correction on assistance data transfer in SL positioning for stage-2	ZTE Corporation	CR	Rel-18	38.305	18.3.0	0175	2	F	NR_pos_enh2	R2-2409259

CN to be ticked instead of RAN

Impact only to NR SA

Agreed with these changes as R2-2410989

Discussion:

Lenovo think CN should be ticked instead of RAN, and the impacted architecture options should only be NR SA for all SL positioning CRs.  Huawei wonder about LTE connected to 5GC.

Huawei understand that a UE camped on LTE cannot send NR SL-PRS.  ZTE understand that we excluded SL positioning in MR-DC based on 37.340.

R2-2409618	Corrections of location time stamp, RSTD and RTOA report	CATT	CR	Rel-18	38.355	18.3.0	0008	3	F	NR_pos_enh2-Core	R2-2409268

Revised in R2-2401990

Discussion:

Lenovo think the new field should have no -r18 or -v18xy suffix.  ZTE think it should be -v18xy.

[AT128][403][POS] ASN.1 formalities in R2-2409618 (CATT)

Scope: Check the format of the new field name in R2-2409618 and align with the guidance of the RRC rapporteur.  Also address the coversheet (should be impact only to NR SA).

Intended outcome: Agreed CR (without CB if possible) in R2-2410990

Deadline: Wednesday 2024-11-20 1600 EST

R2-2410990	Corrections of location time stamp, RSTD and RTOA report	CATT	CR	Rel-18	38.355	18.3.0	0008	4	F	NR_pos_enh2-Core	R2-2409268

Agreed

R2-2409683	RRC correction on NR sidelink positioning	Philips International B.V., Ericsson	CR	Rel-18	38.331	18.3.0	4940	2	F	NR_pos_enh2-Core	R2-2408864

Agreed

R2-2409916	Miscellaneous corrections to SLPP specification	Intel Corporation	CR	Rel-18	38.355	18.3.0	0011	1	D	NR_pos_enh2-Core	R2-2409254

Impact only to NR SA

Agreed with this change as R2-2410991

R2-2410215	Rapporteur CR to IDLE mode procedure for R18 Positioning	Huawei, HiSilicon, Phillips	CR	Rel-18	38.304	18.3.0	0422	-	F	NR_pos_enh2

Impact only to NR SA

Spelling of Philips to be corrected

Agreed with this change as R2-2410992

R2-2410992	Rapporteur CR to IDLE mode procedure for R18 Positioning	Huawei, HiSilicon, Phillips	CR	Rel-18	38.304	18.3.0	0422	1	F	NR_pos_enh2

Revised due to tdoc clash

Agreed as R2-2411112

R2-2411112	Rapporteur CR to IDLE mode procedure for R18 Positioning	Huawei, HiSilicon, Phillips	CR	Rel-18	38.304	18.3.0	0422	2	F	NR_pos_enh2

Agreed

R2-2410494	Miscellaneous RRC Positioning Correction	Ericsson	CR	Rel-18	38.331	18.3.0	5061	1	F	NR_pos_enh2-Core	R2-2408935

Revised in R2-2411061

R2-2411061	Miscellaneous RRC Positioning Correction	Ericsson	CR	Rel-18	38.331	18.3.0	5061	1	F	NR_pos_enh2-Core	R2-2408935

Agreed

R2-2410495	Correction of misplaced else condition of SL Positioning clause	Ericsson	CR	Rel-18	38.321	18.3.0	1971	1	F	NR_pos_enh2-Core	R2-2409158

Agreed

R2-2410644	Clarification on the maximum number of other UEs in sidelink positioning	vivo	CR	Rel-18	38.305	18.3.0	0178	1	F	NR_pos_enh2-Core	R2-2409251

Impact only to NR SA

Agreed with this change as R2-2410993

Coversheet revision

R2-2410217	Correction on SLPP	Huawei, HiSilicon	CR	Rel-18	38.355	18.3.0	0014	-	F	NR_pos_enh2

Revised in R2-2411079 (remove changemark on coversheet)

R2-2411079	Correction on SLPP	Huawei, HiSilicon	CR	Rel-18	38.355	18.3.0	0014	1	F	NR_pos_enh2

Impact only to NR SA

Mention the consequences if not approved in interoperability analysis

Clauses affected should be 6.8, not the IE name

Agreed with these changes as R2-2410994

R2-2410994	Correction on SLPP	Huawei, HiSilicon	CR	Rel-18	38.355	18.3.0	0014	2	F	NR_pos_enh2

Revised from AIP version

R2-2410214	Rapporteur CR to MAC spec for R18 Positioning	Huawei, HiSilicon, ASUSTek	CR	Rel-18	38.321	18.3.0	1996	-	F	NR_pos_enh2

[AT128][404][POS] Rel-18 positioning MAC CR update (Huawei)

Scope: Check the CR in R2-2410214 and update if necessary.

Intended outcome: Agreed CR (without CB if possible) in R2-2411127 and summary in R2-2411126

Deadline: Wednesday 2024-11-20 1600 EST

R2-2411127	Rapporteur CR to MAC spec for R18 Positioning	Huawei, HiSilicon, ASUSTek	CR	Rel-18	38.321	18.3.0	1996	1	F	NR_pos_enh2

Sentence on “the UE will be unable to comply” to be removed

Formula to be deleted and inserted with revision marks in 5.8.3

Revision number to be fixed

Agreed with these changes as R2-2411181

Discussion:

Lenovo think “the UE will be unable to comply” should say the behaviour is unspecified.  Ericsson think we could remove that sentence from the NOTE entirely.

Huawei agree with Ericsson.  Lenovo think this would be OK.

R2-2411181	Rapporteur CR to MAC spec for R18 Positioning	Huawei, HiSilicon, ASUSTek	CR	Rel-18	38.321	18.3.0	1996	2	F	NR_pos_enh2

Agreed

R2-2411126	Summary of email discussion [AT128][404]	Huawei, HiSilicon	discussion	Rel-18	NR_pos_enh2

Noted (email discussion [AT128][404])

### 7.1.1	Organizational

Including incoming LSs and rapporteur inputs.

Incoming LS with “take into account” action and no related document

R2-2409508	Reply LS on CSI-RS and SRS for spatial relation (R1-2409097; contact: ZTE)	RAN1	LS in	Rel-18	NR_pos_enh2-Core	To:RAN2

Noted

### 7.1.2	Stage 2

Impact to 38.300, 37.340, and 38.305.

This agenda item may be handled at lower priority.

R2-2410497	Miscellaneous corrections for Positioning	Ericsson	CR	Rel-18	38.300	18.3.0	0938	-	F	NR_pos_enh2-Core

Change to 9.2.6 to be removed

Agreed with this change as R2-2411171

Discussion:

CATT agree with the principle, but they think there are spelling and format issues.

Huawei think the change to 9.2.6 is wrong: The RACH procedure here is just for legacy reasons to send the RRCResume.  Ericsson think it should be captured because there is a new cause code with a new corresponding procedure.

Qualcomm see Ericsson’s point, because the generation of the resume request is different from the normal case where the UE really wants to resume.

vivo agree with Huawei that there is a high-level description and we do not need to clarify it here.

CATT wonder if the UE moves to a new cell and wants to send SRS, if it will RACH to sync with the new cell and get a new TA; in such a case, should we specify the RACH trigger here?  They consider that this is just following RRCResume.

### 7.1.3	SLPP corrections

Impact to 38.355.

R2-2409568	Correction on tx timestamp request in SL-RTT	ZTE Corporation	CR	Rel-18	38.355	18.3.0	0012	-	F	NR_pos_enh2

“dummy” should be lowercase

Description of associatedSL-PRS-TxTimeStampRequest to include the name of the response field

Agreed with these changes as R2-2410995

Discussion:

Chair notes that “dummy” should be lowercase in the field description.

Qualcomm understand that we should also clarify what field is the response to the associatedSL-PRS-TxTimeStampRequest.

Lenovo think the field description should refer to the “receiving endpoint” rather than the UE.  ZTE understand that it is only sent to the UE.

R2-2409826	Clarification on SLPP session ID existence in SLPP messages between target UE and LMF	vivo	discussion	Rel-18	NR_pos_enh2-Core

Noted

Proposal:	RAN2 to Clarify that SLPP session ID is not included in the SLPP message between the target UE and the LMF, including the SLPP message embedded in the Supplementary RSPP message and the SLPP message directly exchanged in between.

Discussion:

Ericsson think we discussed this previously and decided to leave the stage 2 description as it is.  They think there are cases where the session ID would be beneficial to the LMF.  Qualcomm have the same understanding; they think vivo are right that for UE1, the LMF only needs the correlation ID, but identifying one of multiple sessions involving other UEs might be useful for the LMF.

Nokia agree with Ericsson and think we do not need to reopen the discussion, and they see the ID as needed for user identification.

vivo think this is not just a stage 2 description issue, and it is not clear how the UE and network will implement the feature with the current description.  They understand that the LMF only needs the correlation ID and it should never need to know the session ID.

Ericsson agree that it is up to implementation whether the ID is included or used, but they think there are scenarios where it could be needed involving multiple sessions with the same target.

ZTE wonder if UE1 allocates the session ID to map to the correlation ID, if it eliminates the issue.  vivo want to clarify that the UE and network may have different expectations about behaviour from the current wording.

ZTE understand that the CR would forbid the UE and LMF from using the session ID, and there may be cases where it makes sense, depending on the implementation of ID allocation.

R2-2410132	Corrections on capabilities for FG R1 41-1-19a and 41-1-19b in IE CommonSL-PRS-MethodsIEsProvideCapabilities	Lenovo	CR	Rel-18	38.355	18.3.0	0013	-	F	NR_pos_enh2

Revised in R2-2411121

Discussion:

Qualcomm indicate that we have this capability in individual methods as well, so we have to dummify it and implement the change there as well as in the common IEs.

[AT128][408][POS] SLPP capability corrections (Lenovo)

Scope: Extend the changes of R2-2410132 to apply to the individual positioning methods.

Intended outcome: Agreed CR (without CB if possible) in R2-2411121

Deadline: Wednesday 2024-11-20 1600 EST

R2-2411121	Corrections on capabilities for FG R1 41-1-19a and 41-1-19b in IE CommonSL-PRS-MethodsIEsProvideCapabilities	Lenovo	CR	Rel-18	38.355	18.3.0	0013	1	F	NR_pos_enh2

Agreed

R2-2410218	Discussion on the issues in GAD in SLPP	Huawei, HiSilicon	discussion	Rel-18	NR_pos_enh2

Noted

Proposal1: Send an LS to SA2/CT1/CT4 on the agreement for rangeAndDirection within provideLocationInformation and QoS within requestLocationInformation, and ask if there are any issues with them:

	Adopt high accuracy extended uncertainty as defined in clause 6.2b in TS 23032 for horizontal/vertical/range accuracy

	Adopt scale factor of accuracy is 1 degree; range 0 to 127 degrees for azimuth accuracy (angle uncertainty)

	Adopt scale factor of accuracy is 1 degree; range 0 to 63 degrees for elevation accuracy (angle uncertainty)

Proposal2: Send an LS to SA2/CT1/CT4 on the agreement for relative velocity with uncertainty, and ask if there are any issues with them:

	Radical velocity in the units of meters per second or centi-meters per second and transverse velocity in the units of degree per second or per 0.1 degree per second

	Radical velocity in the range of (-2049..2047) with negative sign indicating the opposite direction between point A and B; elevation and azimuth rate of change in the range of (0..1023)

	Uncertainty for Azimuth and Elevation Rate Of Change in the units of transverse velocity and range in (0..255)

Proposal3a: Support normal uncertainty in clause 6.2 and High Accuracy Uncertainty in Section 6.2a in TS 23.032 for the horizontal/vertical accuracy in the QoS within requestLocationInformation.

Proposal3b: Support uncertainty indication for the velocity and relative velocity in the QoS within requestLocationInformation.

Proposal4: Down-select from the following WFs from RAN2’s perspective:

	Option1: Remove the coding format description for rangeAndOrDirection, relativeLocationEstimate, and relativeVelocityWithUncertainty while only cite the SA2 spec TS 23.032

	Option2: Keep the current description in RAN2.

Discussion:

Qualcomm are uncertain what involvement SA2 and CT groups would have.  They understand that there are no CN specs for range, relative location, direction, etc.

Huawei note that the current SLPP points to an SA2 spec.  Qualcomm indicate this is just to avoid code duplication in SLPP.

Ericsson wonder if this could be directly brought up in SA2.

Huawei think the LMF can request a relative location in the LMF-involved case.  Qualcomm think this is allowed for the LMF, but the AMF will never request it.

Huawei understand that the LCS client can request relative location.  Qualcomm think there is nothing to support it in the LocationRequest.

Ericsson think this is more of an SA2 discussion.

### 7.1.4	LPP corrections

Impact to 37.355.

R2-2409619	Correction of nr-DL-PRS-RSCPD-ReportingRRC-Inactive in NR-DL-TDOA-MeasurementCapability	CATT	CR	Rel-18	37.355	18.3.0	0528	-	F	NR_pos_enh2-Core

Revised in R2-2411122

Discussion:

Qualcomm think the suffix should be -v18xy and there are editorial/formatting issues with the ASN.1.  They also think the field description needs to move to the end of the table.

vivo agree with the CR, but they think the field description may be misleading.

CATT wonder if RAN1 specified that the measurement will be reported in connected mode.  vivo indicate that RAN1 just said the measurement is taken in RRC_INACTIVE, not that it is necessarily reported in RRC_INACTIVE.

Xiaomi think we could check the feature list.

Qualcomm understand that the reporting does not matter for LPP; we just have the capability of the UE to measure in RRC_INACTIVE.  They checked 38.215 and they understand the intention is for measuring in inactive and reporting in connected.

ZTE agree with vivo and Qualcomm that it should be measuring rather than reporting, and they think the field name should be changed accordingly.

CATT think the field description was copied from the parameter list.

[AT128][409][POS] Missing ASN.1 field for RSCPD measurement capability in RRC_INACTIVE (CATT)

Scope: Check and revise the CR in R2-2409619, taking the feature list into account.

Intended outcome: Agreed CR (without CB if possible) in R2-2411122

Deadline: Wednesday 2024-11-20 1600 EST

R2-2411122	Correction of NR-DL-TDOA-MeasurementCapability and NR-Multi-RTT-MeasurementCapability	CATT	CR	Rel-18	37.355	18.3.0	0528	1	F	NR_pos_enh2-Core

Agreed (email discussion [AT128][409])

R2-2410401	Correction for the UE capability on PosSRS-BWA-RRC-Inactive	Xiaomi	CR	Rel-18	37.355	18.3.0	0540	-	F	NR_pos_enh2-Core

Revised in R2-2411123

Discussion:

Lenovo indicate that this issue was discussed in the common session for the RRC/38.306 CRs, but here they think that we do not need to adopt exactly what RAN1 concluded.  They think this can be addressed more clearly as in their TP in R2-2410133.

Nokia prefer the TP from Lenovo.

[AT128][410][POS] UE capability on PosSRS-BWA-RRC-Inactive (Xiaomi)

Scope: Revise the CRs in R2-2410401 and R2-2410402 in line with the TPs from R2-2410133.

Intended outcome: Agreed CRs (without CB if possible) in R2-2411123 / R2-2411124

Deadline: Wednesday 2024-11-20 1600 EST

R2-2411123	Correction for the UE capability on PosSRS-BWA-RRC-Inactive	Xiaomi, Lenovo	CR	Rel-18	37.355	18.3.0	0540	1	F	NR_pos_enh2-Core

Agreed (email discussion [AT128][410])

R2-2411124	Correction for the UE capability on PosSRS-BWA-RRC-Inactive	Xiaomi, Lenovo	CR	Rel-18	38.306	18.3.0	1212	1	F	NR_pos_enh2-Core

Agreed (email discussion [AT128][410])

### 7.1.5	RRC corrections

Impact to 38.331 and 38.306.

R2-2409569	Correction on the dedicated pool interest frequency request in SUI	ZTE Corporation	CR	Rel-18	38.331	18.3.0	5098	-	F	NR_pos_enh2

Merged into R2-2411170

Discussion:

ZTE indicate that only the second change is needed.

Ericsson think this is somewhat editorial and could be merged into R2-2410498 or R2-2409639.

R2-2409620	Clarification on the activation mechanism of srs-PosConfigOrActivationReq	CATT, Ericsson	CR	Rel-18	38.331	18.3.0	5103	-	F	NR_pos_enh2-Core

Postponed

Discussion:

Samsung understand that the CR is to allow the UE to use the resume procedure to request activation, but they think it is not an essential correction but more of a new feature.  They understand that activation can be triggered through the LMF by an Event Report and NRPPa, and they see nothing broken.

Qualcomm indicate that Samsung describe the legacy Rel-17 behaviour, and this is for the validity area, where we agreed previously to use the same cause code for activation requests.  However, they think there are mistakes in the change: The condition “no stored srs-PosRRC-InactiveValidityAreaPreConfigList for the camped cell exists” is only met for the configuration case, not activation.

Samsung consider that the activation we discussed up to now is activation of a preconfiguration, and if we add this functionality there is no way for the gNB to report the activation of SP-SRS to LMF, so the LMF cannot ask neighbouring TRPs to measure it.

CATT indicate that RAN3 just agreed a CR allowing the gNB to indicate the activation, and they see this as an alignment CR.

Ericsson think there are three cases: non-preconfigured area periodic or SP-SRS, and preconfigured area.  For SP-SRS they thought the existing text is correct.

vivo agree with the intention of the CR, but they agree with Qualcomm that the new condition is in the wrong paragraph and a new paragraph is needed.

CATT think we can reach a common understanding that SP-SRS can be activated both in preconfigured and non-preconfigured cases.  For the preconfigured case, the activation request is in the paragraph before the change in the CR.

vivo wonder if there should be a Rel-17 change for the non-preconfigured case.  CATT indicate non-preconfigured with validity area is introduced in Rel-18.

Samsung would like time to check the RAN3 CR.

R2-2409639	Correction on NW restriction for dedicated SL-PRS resource pool	vivo, Ericsson	CR	Rel-18	38.331	18.3.0	5104	-	F	NR_pos_enh2-Core

Postponed

Discussion:

InterDigital think “colliding” may not be the right word; they would prefer “overlapping”.  vivo indicate that the language came from RAN1.

Qualcomm note that the impact analysis is missing.

ZTE ask if “S-SSB” is correct.  vivo indicate this also came from RAN1.

ZTE think “is not expected to be configured” should be “is not configured”.  vivo understand that the network can configure it, but the UE will ignore the problematic configurations.  ZTE think in this case it should only be captured in the RAN1 spec.

Ericsson also thought it should be captured in RAN1, but apparently it has not been, and they understand from RAN1 colleagues that it would be OK to capture in RAN2 as network guidance.  They are OK with the CR as it is.

vivo indicate “S-SSB” is used elsewhere in RRC, and they do not think we should restrict network behaviour too strictly.

Chair wonders if the language is clear for implementation.  Ericsson think we could put something in the coversheet.

Chair suggests “if dedicated SL-PRS resource pool is configured in the slots colliding with the slots (pre)configured for any other resource pool(s) or S-SSB resource(s) in other SL carriers, the UE does not use the colliding slots”.  InterDigital think we should not specify both sides.

Ericsson think this would be a wrong network configuration under the RAN1 agreement.

Huawei think this should be captured in RAN1.  vivo indicate that RAN1 agreed to have no related specification work.

[Post128][404][POS] LS to RAN1 on SL-PRS resource pool configuration (vivo)

Scope: Draft an LS to RAN1 inquiring as to the intention of the agreement quoted in R2-2409639, to clarify whether the restriction should be documented as a network or UE behaviour.

Intended outcome: Approved LS

Deadline: Short (not for RP)

R2-2411251	LS on co-existence of SL-CA and SL PRS transmission/reception for Dedicated SL-PRS resource pool	RAN2	LS out	Rel-18	NR_pos_enh2-Core	To:RAN1

=> Approved in R2-2411251

R2-2409656	Correction for UE indicating its preference TX/RX frequencies for sidelink positioning in SUI	vivo	CR	Rel-18	38.331	18.3.0	5106	-	F	NR_pos_enh2-Core

Styles in the procedure text to be fixed

Agreed with this change as R2-2411130

R2-2410216	Correction for positioning SRS CA in RRC_INACTIVE	Huawei, HiSilicon, Ericsson, Samsung	CR	Rel-18	38.331	18.3.0	5150	-	F	NR_pos_enh2

CR to be indicated as mandatory on coversheet

AggregatedSRS-PosAdditionalCarrier-r18 to be replaced with a single SRS-PosConfigPerULCarrier

Names with “RRC-AggBW-Inactive” to be changed to “RRC-InactiveAggBW”

Agreed with these changes as R2-2411129

Discussion:

ZTE agree with the principle but think SUL should not be included; they understand that it cannot perform SRS aggregation.  Huawei intend to align with the configuration in the main CA part.

CATT think SRS-PosConfigPerULCarrier-r18 includes two SCSs and two frequencies, one explicitly and one in the BWP.  Huawei indicate that the BWP is only used for point A, not to configure SCS, and they understand that this is the same as legacy.  CATT think there should be some description.

vivo wonder if there should be a UE capability.  Huawei would be ok with documenting the CR as mandatory.

CATT agree with ZTE about the SUL configuration.  Ericsson are also fine with excluding SUL.

Huawei think we do not need to say if it is NUL or SUL, just a frequency within a band, and the additional carrier frequency should be the same as the main carrier.

ZTE understand that the Rel-18 additional carrier is an addition over Rel-17, but they do not think there should be SRS aggregation on the SUL, so it only applies when the Rel-17 carrier is NUL.

Huawei agree that you cannot perform CA across different carriers and there should only be one carrier, but they still think it does not need to say NUL/SUL.

CATT think the CR explicitly includes a SEQUENCE of NUL and SUL.  Huawei can update to fix this.

R2-2411129	Correction for positioning SRS CA in RRC_INACTIVE	Huawei, HiSilicon, Ericsson, Samsung	CR	Rel-18	38.331	18.3.0	5150	1	F	NR_pos_enh2

Revised in R2-2411189 (formalities of mandatory description on coversheet)

R2-2411189	Correction for positioning SRS CA in RRC_INACTIVE	Huawei, HiSilicon, Ericsson, Samsung	CR	Rel-18	38.331	18.3.0	5150	2	F	NR_pos_enh2

Agreed

R2-2410224	Correction to sidelinkUEInformation for SL POS	Huawei, HiSilicon	CR	Rel-18	38.331	18.3.0	5151	-	F	NR_pos_enh2

Not pursued

R2-2410498	Sidelink RRC Positioning Correction	Ericsson, vivo	CR	Rel-18	38.331	18.3.0	5165	-	F	NR_pos_enh2-Core

Second change to be removed

R2-2409569 to be merged in

Agreed with these changes as R2-2411170

Discussion:

vivo are not sure of the exact intention of the second change.  They think carrier selection is more of a MAC issue.  Huawei have the same view and think it is already captured in MAC.

Ericsson intended for the carrier to be aligned between upper and lower layers; they think the RRC layer should do the actual selection.

Huawei indicate carrier selection is already in the MAC layer.

R2-2411170	Sidelink RRC Positioning Correction	Ericsson, vivo, ZTE Corporation	CR	Rel-18	38.331	18.3.0	5165	1	F	NR_pos_enh2-Core

Agreed

R2-2410584	RRC correction on NR sidelink positioning	Philips International B.V.	CR	Rel-18	38.331	18.3.0	5174	-	F	NR_pos_enh2-Core

First change to be removed (can be checked in the sidelink session)

Agreed with this change as R2-2411172

R2-2411172	RRC correction on NR sidelink positioning	Philips International B.V.	CR	Rel-18	38.331	18.3.0	5174	1	F	NR_pos_enh2-Core

Agreed

### 7.1.6	MAC corrections

Impact to 38.321.

R2-2410176	Correction on prioritization between SR and SL-PRS transmission	ASUSTeK	CR	Rel-18	38.321	18.3.0	1992	-	F	NR_pos_enh2

Agreed

Discussion:

Huawei think the change is correct but would suggest a different formulation, outside the sidelink communication paragraph; they do not have a strong view.

InterDigital agree with Huawei and think a separate description would be good.

vivo find the comments confusing; this is a general requirement and it is not clear why we would give SL-PRS a separate paragraph.

### 7.1.7	Corrections to other specifications

Impact to any specifications not identified above.

R2-2410133	Correction on the capability description for posSRS-BWA-RRC-Inactive-r18 (FG R1 41-4-8)	Lenovo	discussion	Rel-18	NR_pos_enh2

Noted

R2-2410402	Correction for the UE capability on PosSRS-BWA-RRC-Inactive	Xiaomi	CR	Rel-18	38.306	18.3.0	1212	-	F	NR_pos_enh2-Core

Handled in discussion [AT128][410]

Revised in R2-2411124

## 7.2	Further NR mobility enhancements

(NR_Mob_enh2-Core; leading WG: RAN2; REL-18; WID:RP-233970)

Time budget: 0 TU)

Tdoc Limitation: 2 tdocs.

### 7.2.1	Organizational

Including incoming LSs and rapporteur inputs.

### 7.2.2	In-principle agreed CRs

R2-2409936	Misc state 2 corrections for LTM mobility	Vodafone, Ericsson, Nokia, ZTE	CR	Rel-18	38.300	18.3.0	0911	2	F	NR_Mob_enh2-Core	R2-2409355

Will be further discussed as part of offline/email discussion on stage 2 CR.

[Huawei]: Cover page should be updated, to correct “Consequences if not approved”. Understand it has nothing to do with activated TCI states. [Huawei]: Also have some comments on wordings. [Nokia]: We will have another offline/email discussion on stage 2 CR. We can further consider better wordings as part of the offline/email discussion. [Session chair]: Agree with Nokia suggestion.

[AT128][103][MOB] (Nokia)

Scope: Discuss R2-2410442 and R2-2410449 and update 38.300 if needed. Note 38.300 CR will be updated based on the endorsed version.

Intended outcome: 38.300 CR in R2-2410925. Email approval is applied.

Deadline: Until Thursday CB session.

R2-2410925	Misc Stage-2 corrections for LTM mobility	Vodafone, Ericsson, Nokia, ZTE	CR	Rel-18	38.300	18.3.0	0911	3	F	NR_Mob_enh2-Core	R2-2409355

Agreed.

R2-2410459	Misc RRC corrections for feMob	Ericsson	CR	Rel-18	38.331	18.3.0	5096	1	F	NR_Mob_enh2-Core	R2-2409353

Will be baseline and further revised in R2-2411137 to reflect the agreements this meeting.

R2-2410922	Misc RRC corrections for feMob	Ericsson	CR	Rel-18	38.331	18.3.0	5096	2	F	NR_Mob_enh2-Core	R2-2409353

R2-2411137	Misc RRC corrections for feMob	Ericsson	CR	Rel-18	38.331	18.3.0	5096	3	F	NR_Mob_enh2-Core	R2-2409353

Agreed.

R2-2410514	Miscellaneous corrections for LTM	Huawei, HiSilicon	CR	Rel-18	38.321	18.3.0	1968	2	F	NR_Mob_enh2-Core	R2-2409361

Will be baseline and further revised in R2-2410924 to reflect the agreements this meeting.

[Huawei]: Cover page needs to be further updated. [Session chair]: Cover page can be updated as part of offline/email discussion on MAC CR, if ok with the proposed changes, we still can endorse it.

[AT128][102][MOB] (Huawei)

Scope: Capture the agreements in MAC CR. Note MAC CR will be updated based on the endorsed version.

Intended outcome: MAC CR in R2-24010924. Email approval is applied.

Deadline: Until Thursday CB session.

R2-2410924	Miscellaneous corrections for LTM		Huawei	CR	Rel-18	38.321	18.3.0	1968	3	F	NR_Mob_enh2-Core

Agreed.

R2-2410801	Corrections for mobility enhancements in stage-2	ZTE Corporation, Ericsson	CR	Rel-18	37.340	18.3.0	0404	2	F	NR_Mob_enh2-Core	R2-2409357

Will be baseline and further revised in R2-2410926 to reflect the agreements this meeting.

[AT128][104][MOB] (ZTE)

Scope: Discuss R2-2410802 and update 37.340 if needed. Note 38.340 CR will be updated based on the endorsed version.

Intended outcome: 37.340 CR in R2-24010926. Email approval is applied.

Deadline: Until Thursday CB session.

R2-2410926	Correction for mobility enhancements in stage-2		ZTE Corporation, Ericsson	CR	Rel-18	37.340	18.3.0	0404	1	F	NR_Mob_enh2-Core

Agreed, but then coversheet revised by MCC in R2-2411257: wrong rev number.

R2-2411257	Correction for mobility enhancements in stage-2		ZTE Corporation, Ericsson	CR	Rel-18	37.340	18.3.0	0404	4	F	NR_Mob_enh2-Core

=> Agreed

### 7.2.3	Others

Including all corrections.  Minor and editorial issues should be coordinated with the CR rapporteur. A contribution can include multiple TPs. Note RRC CR and MAC CR rapporteurs’ summary and suggestion (based on the submitted contributions) may be provided. Agreed changes may be merged into a single or multiple CRs containing similar issues.

CP Corrections:

R2-2410462	Summary of RRC proposals for feMob	Ericsson	discussion	Rel-18	NR_Mob_enh2-Core	Late

[AT128][101][MOB] (Ericsson)

Scope: 1) Discuss and capture C1, C2 in R2-2409900, P1,2 and P5,6 in R2-2410461, R2-2410460 and R2-2410117, 2) Discuss C3 in R2-2409900, P2 in R2-2410707, 3) Further discussion from the discussion on R2-2409922/R2-2410737.

Intended outcome: Discussion summary in R2-2410921, RRC CR in R2-2410922, UE Capability CR in R2-24010923. Note RRC CR will be updated based on the endorsed version. -> Revised RRC CR in R2-2411137 and UE capability CR in R2-2411138 (if needed)

Deadline: Come back to Thursday CB session. F2F offline discussion schedule will be announced by offline discussion rapporteur. => For CR, email approval until 8:00am Friday.

R2-2410921	Summary of [AT128][101][MOB]	Ericsson	discussion	Rel-18	NR_Mob_enh2-Core	Late

Proposal 1: Clarify in RRC that the conditional reconfiguration cannot include the ltm-config.

Proposal 2: Clarify in daps-config that if the field is configured, any RRCReconfiguration message contained in ltm-config cannot contain the field daps-Config. Similar clarification is also done with the field description of conditionalReconfiguration.

Proposal 3: Modify the existing restriction in nrofHARQ-Processes field description as following: “The network sets the value of this field to 1 when cg-LTM-Configuration is configured”.

Proposal 4: The changes captured in R2-2410894 are agreed.

Proposal 5: RAN2 confirm that if UE supports ltm-InterFreq-r18, the support of FG45-3a/4a, 45-5/5a/6, and FG45-3/4 implies the support for inter-frequency scenarios in addition to intra-frequency scenarios. If UE only supports ltm-MCG-IntraFreq-r18 or ltm-SCG-IntraFreq-r18, the support of FG45-3a/4a, 45-5/5a/6, and FG45-3/4 implies the support for intra-frequency scenarios. RAN2 does not see the need to add additional clarifications in current spec.

Proposal 6: RAN2 understanding is that network should always include pdcp-config and sdap-config within an LTM candidate configuration or reference configuration.

Proposal 7: About the handling of logical channels during the execution of an LTM cell switch, RAN2 has the following understanding:

a.	If the network provides in an applied LTM candidate configuration a logical channel identity which is not part of current configuration, the UE adds automatically the logical channel.

b.	If the UE has a logical channel identify which is not part of the applied LTM candidate configuration, the UE will release the logical channel identity.

Proposal 8 (modified): Modify existing restriction in s-MeasureConfig as follows: “This field is also applicable to L1 measurements configured with ltm-CSI-ResourceConfigToAddModList-r18.”

All proposals are agreed.

[Ericsson]: For P4 and P5, Intel will capture the actual changes in their CR according to P4 and P5. We don’t need to capture in our CRs. R2-2410894 is Intel CR.

R2-2409922	Clarification on conditional, DAPS and LTM candidate configurations	Google	CR	Rel-18	38.331	18.3.0	5121	-	F	NR_Mob_enh-Core, NR_Mob_enh2-Core

R2-2410737	Restrictions on simultaneous conditional, DAPS and LTM candidate configurations	Google	CR	Rel-18	38.300	18.3.0	0941	-	F	NR_Mob_enh-Core, NR_Mob_enh2-Core

RAN2 confirms conditional, DAPS cannot be included into a LTM reference/candidate configuration.

Whether the current spec specifies it enough or not will be further checked in the offline discussion [101].

[Google]: Could not find out any restriction on the restriction for DAPS and LTM in 38.300. [Ericsson]: We have a specified restriction in RRC. [MediaTek]: CR is not entirely correct. CR only indicates about candidate configuration and does not include reference configuration. [Vivo]: Intention is correct, but concerns on the wordings. Current RRC specifies when to release a configuration, which is not crystal clear on the restriction.

R2-2410018	Correction on the field  description of ltm-UE-MeasuredTA-ID	CATT	CR	Rel-18	38.331	18.3.0	5130	-	F	NR_Mob_enh2-Core

RAN2 understands if any candidate cell is configured with 2 tag, no UE measurement TA id is configured for any of candidate cells. No spec change is needed.

Not pursued.

[MediaTek]: Current text seems ok. Original intention is not to support UE based TA measurement with 2 tag. With this change, it seems a kind of mixed case for UE based TA measurement and it brings more confusion. [Vivo]: Agree with correction. [Ericsson, Apple]: According to the current spec, if any candidate cell is configured with 2 tag, then according to the restriction, UE measurement TA ID will not be configured for any of candidate cells. Nothing is broken.

R2-2410019	Correction on the field description of ltm-NoResetID	CATT	CR	Rel-18	38.331	18.3.0	5131	-	F	NR_Mob_enh2-Core

“and ensures that the UE has stored a valid value for ltm-ServingCellNoResetID within VarLTM-ServingCellNoResetID” will be added into the end of current text.

The change will be merged into RRC CR rapporteur CR.

[Ericsson]: Think it is not needed. The UE behaviour is already specified in the procedure, e.g. the UE behaviour when the UE does not have ltm-NoResetID. [MediaTek, Xiaomi, LG]: Agree with the correction considering we already have similar text for ltm-UE-MeasuredTA-ID. [Ericsson]: This change can be merged into rapporteur CR.

R2-2410063	Remaining issues on L2 reset for LTM	NEC	discussion	Rel-18	NR_Mob_enh2-Core

Proposal: In case of LTM cell switch without L2 reset, UE triggers PDCP recovery for an AM DRB, if one of its associated RLC bearer is released. The draft CR in Annex is adopted.

Not pursued.

[Apple]: Wonders if the scenario is really valid one. [Ericsson]: Considering it is NBC change, prefer not to have it. We still can survive w/o this change. [Xiaomi]: If companies are not favourable with this change, it’s good to capture that NW ensures this situation doesn’t happen in the meeting minutes. [Samsung, Qualcomm]: Although we agree in technical point of view, also share the concern with Ericsson. It may be too late to specify it now.

[Vivo]: It’s ok not to agree with this correction, but considering it is valid scenario since it was already considered in SCPAC and specified it, it would be better to capture that NW ensures this situation doesn’t happen. [Ericsson]: Prefer to do nothing. Reasonable NW implementation takes care of it. [MediaTek]: Agree with Ericsson. [Google]: Agree with Ericsson. We may consider for Rel-19.

R2-2410442	Miscellaneous Rel-18 LTM Aspects and Corrections	Nokia	discussion	Rel-18	NR_Mob_enh2-Core

Proposal 2: Adopt the TP in Annex B.

Noted.

[MediaTek]: Intention is ok, but not happy with wording. Instead of “same RLC entity”, it would be better to associate with master key or secondary key. [LG]: In the current PDCP spec, it is specified that PDCP recovery is applied to the released DRB or re-established DRB, so thinks it can be already handled by PDCP. [Nokia]: PDCP does not know this situation, RRC should inform PDCP. [Samsung]: Understand with this proposal, it is to avoid redundant PDCP re-establishment. We still survive w/o this change. Do we really need this optimization in the late phase? [Huawei]: Agree with Samsung. For DC case, we do not handle all cases for optimization. [Xiaomi]: Agree with LG.

R2-2410461	Remaining issues on LTM	Ericsson	discussion	Rel-18	NR_Mob_enh2-Core

Proposal 3: RAN2 to confirm that, upon an LTM cell switch procedure with a L2 reset, the logicalChannelIdentity and logicalChannelIdentityExt of the radio bearer in current UE configuration can be reconfigured.

Proposal 4: RAN2 to agree to the TP in Annex 4.B.

[Huawei]: Understand if no logical id is assigned, the logical channel id previously assigned should be released. Do not think any additional thing is needed. [Samsung]: For intra-CU case, there is no reason why logical channel id needs to be changed. Do not understand if the proposed scenario is valid. [CATT]: Understand if new logical channel id is assigned, it should replace the previous one. [MediaTek]: Understand to the current spec, direct change of logical channel id is not allowed. It can be achieved by release and addition, but that is not what proposes. [Xiaomi]: Consider it is also a kind of optimization. [Huawei]: Again, to the current spec, it is clear the DRB and logical channels is released before LTM cell switch. [Xiaomi]: Understand Ericsson intention is to re-establish RLC entity in this case. [Session chair]: Suggest to have offline discussion to check what is RAN2 common understanding on what is supported by the current spec and whether to achieve the intention.

CB session:

[Ericsson]: It was discussed as part of offline discussion on RRC. No need of separate handling.

R2-2410707	Miscellaneous corrections for SCPAC	Nokia	discussion

Proposal 1: The field description for SK-counter configuration includes note that NW includes new values for SK-counter configuration on master-key change. Refer Annexure A.

Not pursued.

[Huawei]: It is natural NW behaviour, nothing needs to be captured. [CATT]: It is already clear by SA3 spec. Nothing is needed. [OPPO]: Intention is agreeable, however share the view with Huawei.

R2-2410055	Clarification on inter-band LTM capabilities	Qualcomm Inc., OPPO	CR	Rel-18	38.306	18.3.0	1209	-	F	NR_Mob_enh2-Core

We can ask RAN1 opinion, i.e. whether the UE supports the related feature(s) based on source band or target band or consideration of both bands when the target candidate cell is on the different band.

[Nokia]: Do not understand the proposal. This capability is defined per band, with this change, it seems not per band UE capability any more. [MediaTek]: The issue is valid, but with this change, it seems now we define this capability per BC. Think it is too late to introduce this capability per BC. Instead, we can simply define whether it will be done based on source band or target band. [Session chair]: Understand it is for cross band operation (target candidate cell is on the different band), then the question is whether it is based on source band or target band capability. What about asking RAN1 since it was introduced by RAN1?

[AT128][113][MOB] (Qualcomm)

Scope: Prepare the LS to RAN1 to explain the scenario and their opinion.

Intended outcome: LS in R2-2410947, to be approved.

Deadline: Until Thursday CB session.

R2-2410947	LS on LTM UE capabilities for cross-band operation	LSout	Rel-18	NR_Mob_enh2-Core	To:RAN1

Approved in R2-2411135

=> Due to wrong upload, number R2-2411135 was already on the server

=> LS out is revised and approved in R2-2411219.

R2-2410513	LTM UE capabilities for inter-frequency L1 measurements	Huawei, HiSilicon	discussion	Rel-18	NR_Mob_enh2-Core

Proposal: agree the draft TS 38.306 and TS 38.331 CRs in annex.

Change ltm-interFreqL1-OnlyInBC-r18 as ENUMERATED {Supported} and update the field description for 38.306 accordingly.

Add clarification to the existing capability accordingly

RRC CR in R2-2410948 and UE capability CR in R2-2410949 (Come back in CB session)

[Ericsson]: For ltm-interFreqL1-OnlyInBC-r18, it is better to define as ENUMERATED {Supported}. In addition, it will be good to clarify the existing capability accordingly.

[AT128][114][MOB] (Huawei)

Scope: Prepare CRs on UE capability proposed in R2-24010513.

Intended outcome: RRC CR in R2-2410948 and UE capability CR in R2-2410949. Note f2f offline discussion in the reserved room is not required.

Deadline: Until Thursday CB session.

R2-2410948	Scope of interFreqL1-MeasConfig-r18		Huawei	CR	Rel-18	38.331	18.3.0	5198	-	F	NR_Mob_enh2-Core

Agreed.

R2-2410949	Scope of interFreqL1-MeasConfig-r18		Huawei	CR	Rel-18	38.306	18.3.0	1222	-	F	NR_Mob_enh2-Core

Agreed.

R2-2409900	Corrections on remaining LTM RRC issues	MediaTek Inc.	CR	Rel-18	38.331	18.3.0	5119	-	F	NR_Mob_enh2-Core

R2-2410460	Issues on capabilities for LTM	Ericsson	discussion	Rel-18	NR_Mob_enh2-Core

R2-2410117	Correction on LTM UE capability based on the LS from RAN1	MediaTek Inc.	discussion	Rel-18	NR_Mob_enh2-Core

UP Corrections:

R2-2410517	MAC CR rapporteur summary	Huawei, HiSilicon	discussion	Rel-18	NR_Mob_enh2-Core	Late

R2-2410515	Further MAC corrections	Huawei, HiSilicon	discussion	NR_Mob_enh2-Core

Proposal 1: In TS 38.321, remove the text in 5.8.2 to select an SSB and indicate it to lower layers (highlighted in yellow before).

Proposal 2: In TS 38.321 clause 5.8.2, align the wording on valid grant with the wording in TS 38.213 clause 21.1 (add "provided by or" before "associated with") and add a reference to TS 38.213 clause 21.1.

Not pursued for P1.

For P2, reference spec and section will be updated to the CR, no more change is pursued.

Will be merged into MAC rapporteur CR.

For P1:

[LG]: Although it may be redundant, nothing is broken with the current spec. [Vivo]: If TCI state information is indicated to the lower layer every time LTM cell switch command is received, prefer keeping the current text. [CATT]: Support the proposal.

For P2:

[LG]: Seems not essential correction. In addition, we may need to update the other related parts that are scattered in the spec once we agree with this change. [Samsung]: If we really need to consider it, we can only change the reference spec and section.

R2-2410461	Remaining issues on LTM	Ericsson	discussion	Rel-18	NR_Mob_enh2-Core

Proposal 7: The text related to LTM in section 5.4.4 is deleted as the UE will never execute those actions.

Proposal 8: RAN2 to agree to the TP in Annex 4.D

Not pursued.

[LG]: The paragraph covers for pending SR for all cases. [Samsung]: The indicated text was added because of RACH-less LTM. No issue with the current text. [MediaTek]: Agree with Samsung.

R2-2409777	Coexistence of (e)RedCap UEs and LTM and other MAC corrections	vivo	discussion	Rel-18	NR_Mob_enh2-Core, NR_redcap_enh-Core, NR_redcap-Core

Proposal 1: RAN2 confirms that LTM is supported by (e)RedCap UEs.

Not treated. It was already handled in the main session.

[Vivo]: Indicates this proposal was already handled in the main session.

R2-2410628	Clarification to 38.321 on RACH Initiation for RACH based LTM	ZTE Corporation	CR	Rel-18	38.321	18.3.0	2009	-	F	NR_Mob_enh2-Core

Proposal 2: Adopt the TP in Annex B.

Not pursued.

[LG, Huawei]: It was already discussed last meeting and not agreed. [CATT]: Consider it is optimization, which is not essential correction.

R2-2410442	Miscellaneous Rel-18 LTM Aspects and Corrections	Nokia	discussion	Rel-18	NR_Mob_enh2-Core

Proposal 1: RAN2 is asked to adopt the changes to TS 38.300, depicted in Annex A.

R2-2410449	Stage 2 TP for simultaneous execution of CHO and SCG LTM	Lenovo	discussion	Rel-18	NR_Mob_enh2-Core

R2-2410802	Correction for mobility enhancements in stage-2	ZTE Corporation, Ericsson	CR	Rel-18	37.340	18.3.0	0406	-	F	NR_Mob_enh2-Core

## 7.3	IoT NTN enhancements

(IoT_NTN_enh-Core; leading WG: RAN1; REL-18; WID: RP-223519)

Time budget: 0 TU

Tdoc Limitation: 1 tdocs

### 7.3.0	In-principle agreed CRs

Contributions agreed in principle at RAN2#127bis.

36.300

R2-2410884	IoT NTN Stage 2 correction to eMTC CHO	Ericsson (Rapporteur)	CR	Rel-18	36.300	18.3.0	1409	1	F	IoT_NTN_enh-Core	R2-2409178

Agreed

R2-2410810	Correction on measurement	Huawei, HiSilicon	CR	Rel-18	36.300	18.3.0	1414	-	F	IoT_NTN_enh-Core

Revised in R2-2410962 to reflect the change suggested in R2-2409590

R2-2410962	Correction on measurement	Huawei, HiSilicon	CR	Rel-18	36.300	18.3.0	1414	1	F	IoT_NTN_enh-Core

Agreed

36.331

R2-2410422	Miscellaneous corrections to TS 36.331 for IoT NTN	Huawei, HiSilicon	CR	Rel-18	36.331	18.3.0	5054	1	F	IoT_NTN_enh-Core	R2-2408342

Agreed

R2-2409589	Corrections on location based measurements and need code for IoT NTN	CATT	CR	Rel-18	36.331	18.3.1	5052	2	F	IoT_NTN_enh-Core	R2-2409233

- CATT indicates that the coversheet needs to be updated to remove the reference to the impacted 5G architecture

Revised in R2-2410963

R2-2410963	Corrections on location based measurements and need code for IoT NTN	CATT	CR	Rel-18	36.331	18.3.1	5052	3	F	IoT_NTN_enh-Core

Agreed

36.306

R2-2410056	Applicability of optional UE Capabilities for NB-IoT	Qualcomm Inc.	CR	Rel-18	36.306	18.3.0	1894	2	F	IoT_NTN_enh-Core	R2-2409232

- Lenovo thinks 	that in clause 4 the capability ntn-UplinkHarq-ModeB-MultiTB-r18 (4.3.38.35) is missing.

Revised in R2-2411113 to add the missing capability

R2-2411113	Applicability of optional UE Capabilities for NB-IoT	Qualcomm Inc.	CR	Rel-18	36.306	18.3.0	1894	3	F	IoT_NTN_enh-Core

Agreed

R2-2410867	IoT NTN UE capabilities correction for GNSS and HARQ enhancements	Ericsson	CR	Rel-18	36.306	18.3.0	1902	-	F	IoT_NTN_enh-Core

Agreed

36.304

R2-2410860	Corrections on distance-based measurements during T-Service for IoT NTN	Samsung	CR	Rel-18	36.304	18.2.0	0876	1	F	IoT_NTN_enh-Core	R2-2409235

Agreed

Withdrawn

R2-2410488	Corrections on distance-based measurements during T-service for IoT NTN	Samsung	CR	Rel-18	36.304	18.2.0	0877	-	F	IoT_NTN_enh-Core

Withdrawn

### 7.3.1	Organizational

LSs, rapporteur inputs.

Editorials/clarifications should not be included in any tdoc but sent to the WI spec rapporteurs

R2-2409518	Reply LS on UE Location Information for NB-IoT NTN (R3-245819; contact: Nokia)	RAN3	LS in	Rel-18	IoT_NTN_enh	To:SA2	Cc:RAN2, CT1, SA1, SA3-LI

Noted

### 7.3.2	Corrections

Corrections for all specifications.

RRC

- SIB33

R2-2410308	Correction on SIB33(-NB) for IoT NTN	Nokia, Nokia Shanghai Bell	CR	Rel-18	36.331	18.3.1	5076	-	F	IoT_NTN_enh-Core

-	Xiaomi and vivo support the CR but the cover page should be fixed to add the impact analysis

Add impact analysis and clauses affected

Add the agreed change (p2) from R2-2410481

Revised in R2-2410964

R2-2410964	Correction on SIB33(-NB) for IoT NTN	Nokia, Nokia Shanghai Bell, Samsung	CR	Rel-18	36.331	18.3.1	5076	1	F	IoT_NTN_enh-Core

Agreed

R2-2410481	Various corrections for IoT NTN Rel-18	Samsung	discussion	Rel-18	IoT_NTN_enh-Core

Proposal 1: Introduce capability for eMTC to indicate that the UE is capable of acquiring and maintaining SIB33 in connected mode.

-	Xiaomi thinks this is not needed. ZTE agrees. Nokia also agrees

-	Nokia is still not convinced about the need for this. ZTE has a similar view

We don’t introduce a capability for eMTC to indicate that the UE is capable of acquiring and maintaining SIB33 in connected mode.

Proposal 2: Add SystemInformationBlockType33(-NB) to NOTE1 of 5.3.18.

Add SystemInformationBlockType33(-NB) to NOTE1 of 5.3.18 (To be included in CR 5076)

- Satellite ID

R2-2410866	Correction to satellite ID in system infromation	Ericsson, Samsung	CR	Rel-18	36.331	18.3.0	5081	-	F	IoT_NTN_enh-Core

-	HW thinks that this is not essential, after agreeing the Stage2 CR

-	Ericsson thinks this is still useful

-	vivo supports the CR. Apple agrees

Agreed

R2-2410857	Scope and uniqueness of satelliteId	Nordic Semiconductor, Samsung	discussion	Rel-18

Revised in R2-2410892

R2-2410892	Scope and uniqueness of satelliteId	Nordic Semiconductor, Samsung	discussion	Rel-18

Proposal 1	Clarify in the specifications that the satelliteId-r17 and the satelliteId-r18 share the same number space.

-	QC has the same understanding and thinks this is obvious. ZTE agrees that no further clarification is needed. Vivo also agrees

Proposal 2	A satelliteId is unique within a PLMN, in other words satelliteID+PLMN code may be considered a global identifier for a satellite.

-	QC agrees with the intention and we should fix this, for instance sending an LS to SA2/CT4

-	ZTE wonders if we can leave this to NW implementation

-	vivo thinks the current space is sufficient for R18 and we might need to discuss this issue only for R19 S&F

-	Nordic thinks this is an issue also in Rel18 and linking the satelliteID and PLMN would be a solution to fix this

-	Xiaomi thinks we need to address this explicitly only for R19

We first continue the discussion on satellite ID for Rel19 S&F and then check whether we need to send any LS to any group

Proposal 3	Multiple satelliteId+PLMN codes can identify the same satellite as long as the satelliteId is the same.

R2-2410480	Clarification on TN-NTN mobility for IoT NTN	ZTE Corporation, Sanechips	discussion	Rel-18	IoT_NTN_enh-Core

Proposal 2: For SIB3, it can be clarified that, when satellite ID is absent, the cell type of the intra-frequency neighbor cells are same as that of the serving cell. The example change is as below:

-	Samsung wonders if anything is broken if we don’t have this. ZTE thinks the UE would then follow R17 behaviour

-	Samsung thinks that in case the NW should include a satellite ID in this case. MTK agrees

-	ZTE thinks it’s unreasonable to include satellite IDs for intra-frequency neighbour cells on the same satellite as the serving cell

Change is not pursued

36.306

R2-2409947	UE feature for SIB33(-NB) reception in RRC_IDLE state in a TN cell	Apple, Qualcomm Incorporated, Samsung, Huawei, HiSilicon, Ericsson, CATT, MediaTek Inc.	CR	Rel-18	36.306	18.3.0	1901	-	F	IoT_NTN_enh-Core

-	ZTE thinks this is not needed

-	vivo thinks we should have a general capability, also for when the UE is in a NTN cell. ZTE thinks we already discussed this. Apple thinks this is a new case

-	ZTE can compromise and have this

Revised in R2-2410965 to add the impact analysis

R2-2410965	UE feature for SIB33(-NB) reception in RRC_IDLE state in a TN cell	Apple, Qualcomm Incorporated, Samsung, Huawei, HiSilicon, Ericsson, CATT, MediaTek Inc.	CR	Rel-18	36.306	18.3.0	1901	1	F	IoT_NTN_enh-Core

Agreed

Stage 2

R2-2409543	Correction on UE Location Information for IoT-NTN	vivo, Ericsson, Nokia, Nokia Shanghai Bell	CR	Rel-18	36.300	18.3.0	1410	-	F	IoT_NTN_enh-Core

-	Xiaomi wonders if we need to cover the RAN3 aspects. Nokia thinks this is not clashing with the RAN3 agreed CR and the two CRs can be merged during the CR implementation phase

Agreed

R2-2409590	Corrections on the reference of satellite ID	Huawei, HiSilicon, Apple	CR	Rel-18	36.300	18.3.0	1411	-	F	IoT_NTN_enh-Core

-	Xiaomi agrees with the CR and thinks a Stage 2 change is sufficient

-	Samsung supports the CR but thinks this can be merged with the previous IPA CR

Change is agreed and merged into CR 1414

CR is not pursued

## 7.4	NR NTN enhancements

(NR_NTN_enh-Core; leading WG: RAN1; REL-18; WID: RP-232669)

Time budget: 0 TU

Tdoc Limitation: 1 tdocs

### 7.4.0	In-principle agreed CRs

Contributions agreed in principle at RAN2#127bis.

38.300

R2-2410541	Miscellaneous corrections to NR NTN	Samsung	CR	Rel-18	38.300	18.3.0	0922	2	F	NR_NTN_enh-Core	R2-2409244

Revised in R2-2410966 to add the impact analysis

R2-2410966	Miscellaneous corrections to NR NTN	Samsung	CR	Rel-18	38.300	18.3.0	0922	3	F	NR_NTN_enh-Core

Agreed

### 7.4.1	Organizational

LSs, rapporteur inputs.

Editorials/clarifications should not be included in any tdoc but sent to the WI spec rapporteurs

R2-2409505	LS on FR2-NTN inclusion to specifications (R1-2407406; contact: vivo)	RAN1	LS in	Rel-18	NR_NTN_enh-Core	To:RAN2, RAN4

Noted

### 7.4.2	Corrections

Corrections for all specifications.

RRC related

- FR2 related

R2-2410364	Miscellaneous corrections on NTN in FR2 bands	ZTE Corporation, Vivo, Sanechips	CR	Rel-18	38.331	18.3.0	5161	-	F	NR_NTN_enh-Core

-	QC would like to check change 4. Ericsson also

Changes other than change 4 are agreed

Continue the discussion on change 4 in the next meeting

Revised in R2-2410977 to remove the 4th change

R2-2410977	Miscellaneous corrections on NTN in FR2 bands	ZTE Corporation, Vivo, Sanechips	CR	Rel-18	38.331	18.3.0	5161	1	F	NR_NTN_enh-Core

Agreed

- Reference location

R2-2410865	Clarification of reference location within the MO for NR NTN Rel-18	Ericsson	CR	Rel-18	38.331	18.3.0	5193	-	F	NR_NTN_enh-Core

-	LG is ok to move the field description to the right place but thinks the change on the reference location is not needed

-	Huawei thinks the behaviour is already clear and the CR is not needed. Nokia agrees

-	vivo is ok but thinks this should go to the rapporteur CR

Not pursued

Changes are agreed but will be captured in the RRC rapporteur CR

- Satellite switch with resync

R2-2410878	Remaining open issues of satellite switch with resync	Sequans Communications	discussion	Rel-18	NR_NTN_enh-Core

Observation 1: Service link PDD simultaneous reports at satellite switch seems avoidable by NW implementation

-	Ericsson agrees with the observation, in the considered scenario (LEO 600km), but wonders if this would be different in other scenarios (higher altitudes) and whether the feature would work in that case

Proposal 1: Consider soft switch where SSB-timeOffset is applied only between t-serviceStart and t-service

R2-2410750	Remaining issues on SMTC	Huawei, HiSilicon	discussion	Rel-18	NR_NTN_enh-Core

Observation 1: Network can configure suitable threshPropDelayDiff to avoid PDD report during satellite switch with resync.

Proposal 1: No further enhancement to PDD reporting during satellite switch with resync.

-	QC wonder if we need to clarify the UE behaviour for PDD reporting during the transition phase. CMCC thinks this is a similar case to the HO scenario in Rel-17.

Can further check whether we need to clarify the UE behaviour for PDD reporting during the transition phase

Proposal 2: The target satellite uses different SSB transmission pattern with source satellite during [t-Start, t-Service], and then change back to the source satellite SSB pattern after t-Service.

-	QC thinks the UE should not resync another time to the target cell and does not support this solution

-	LG thinks this can be done without the need to resync

-	CMCC also supports this

Continue in offline 301

Proposal 3: RAN2 to determine whether PCI list in SMTC4 is mandatory, and choose from the following:

•	Option 1: Clarify in the field description of pci-List of SSB-MTC4 that “The network shall always configure this field when SSB-MTC4-r17 is configured”;

•	Option 2: Add the following description in 38.331 clause 5.5.2: “If pci-List parameter is absent in an SSB-MTC4 in SIB2/SIB4, it is up to UE implementation to detect neighbour cells associated with the SMTC, based on the ephemeris information in SIB19.”

Continue in offline 301

[AT128][301][R18 NR NTN] SMTC issues (Huawei)

Scope: Continue the discussion on p2 and p3 from R2-2410750

Intended outcome: Report of the offline discussion

F2F offline time and location:  FFS

Deadline for rapporteur's summary (in R2-2410971):  Thursday 2024-11-22 08:00

R2-2410971	Report of [AT128][301][NR NTN] SMTC issues	Huawei	discussion	Rel-18	NR_NTN_enh-Core

Proposal 1: RAN2 to further discuss whether to specify the following for soft satellite switching:

Option 2b: After t-Service, UE applies ssb-TimeOffet to the SMTC configured by source satellite.

Option 3: Temporarily shifted SSB pattern (determined by ssb-TimeOffset) is used during [t-Start, t-Service] for early synchronization with target satellite, FFS whether RAN4 input is needed.

-	vivo is fine to consider option 3 and send a LS to RAN4/RAN1 asking for feedback

-	QC thinks it’s late to introduce a change and prefers not to send the LS

-	Google/Sequans are fine to send the LS. Sequans thinks option 2b is not complete

Send a LS to RAN1/RAN4 on the feasibility of option 3. RAN2 will work on solution 2b or solution 3 depending on the LS response

Proposal 2: Clarify in the field description of pci-List of SSB-MTC4 that “The network configures this field when SSB-MTC4-r17 is configured”.

-	Ericsson thinks neither option 1 nor 2 are needed

-	HW thinks the spec needs to be fixed in any case, at least for idle mode

-	Sequans wonders what is the case of using smtc4 without providing the PCI list

RAN2 understands that if pci-List parameter is absent in an SSB-MTC4 in SIB2/SIB4, the UE behaviour regarding how to determine the relationship between SMTC and different PCIs is unspecified.

R2-2410973	Draft LS soft satellite switch with resync (Huawei)	LSout	To:RAN4, RAN1	NR_NTN_enh-Core

Modify as: “RAN2 has discussed one possible solution to introduce …”

Add RAN1 (in To)

Remove Draft

Revised in R2-2410978

R2-2410978	LS on soft satellite switch with resync	Huawei	LSout	Rel-18	NR_NTN_enh-Core 	To:RAN4, RAN1

Approved

- RACH-less HO and SIB19 reception

R2-2410527	Discussion on some corrections	Samsung	discussion	Rel-18	NR_NTN_enh-Core

Observation 1: RRCReconfiguration message for RACH-less HO can be prepared by the target gNB using the existing mechanism and/or by implementation. Inter-gNB RACH-less HO in NTN can be supported without RAN3 impact.

Proposal 1: Add the following sentence in TS 38.300 clause 9.2.3.6 RACH-less handover.

“In NTN, RACH-less handover can be supported for intra/inter-satellite handover with same gateway/gNB and intra/inter-satellite handover with gateway/gNB switch.”

-	Ericsson thinks this is not for RAN2 to decide and should be discussed in RAN3

-	Nokia understands this is doable with the existing RAN3 specs

RAN2 assumes that, in NTN, RACH-less handover can be supported for intra/inter-satellite handover with same gateway/gNB and intra/inter-satellite handover with gateway/gNB switch. We send a LS to RAN3 (cc RAN1) asking if there is any problem with this

Draft an LS to RAN3 in offline 302

Observation 2: In TS 38.133 clause 4.2.2.12, RAN4 has already specified measurement requirements for TN-NTN cell reselection in both directions.

Proposal 2: RAN2 understands that there are RAN4 requirements regarding measurements on a NTN cell while the UE is camping or being connected to a TN cell. This reverts the agreement in the last meeting.

-	QC thinks there are still no requirement on the GNSS turn-on times

RAN2 understands that there are RAN4 requirements regarding measurements on a NTN cell while the UE is camping in a TN cell. This corrects the agreement in the last meeting

Agreements:

1.	RAN2 assumes that, in NTN, RACH-less handover can be supported for intra/inter-satellite handover with same gateway/gNB and intra/inter-satellite handover with gateway/gNB switch. We send a LS to RAN3 (cc RAN1) asking if there is any problem with this

2.	RAN2 understands that there are RAN4 requirements regarding measurements on a NTN cell while the UE is camping in a TN cell. This corrects the agreement in the last meeting

R2-2410438	Various Corrections and Open Points for Rel-18 NTN	Nokia, Nokia Shanghai Bell	discussion	Rel-18	NR_NTN_enh-Core

Proposal 1: RAN2 assumes the requirements and parameters for FR2-1 are generally applicable to FR2-NTN, unless specified otherwise.

Proposal 2: Consider a general statement in RAN2 Technical Specifications or a pointer to TS 38.101 to reflect which requirements apply to FR2-NTN.

Proposal 3: If Proposal 2 is not agreeable, RAN2 agrees a single CR fixing the FR2-NTN requirements for all affected fields/IEs.

Proposal 4: Support RACH-less HO also in inter-gNB scenarios for NTN.

Proposal 5: RAN2 reopens the discussion on whether the UE can measure an NTN cell while camping or being connected to a TN cell. RAN2 reflects RAN4 requirements in the appropriate specifications.

Proposal 6: Align with RAN4 regarding the requirements for SIB19 acquisition in TN cell.

[AT128][302][R18 NR NTN] LS to RAN3 on RACH-less HO (Nokia)

Scope: Draft an LS to RAN3 on RACH-less HO in NTN

Intended outcome: Draft LS

Deadline for draft LS (in R2-2410967):  Thursday 2024-11-22 08:00

R2-2410967	Draft LS on RACH-less HO in NTN	 (Nokia)		LSout	To:RAN3, Cc:RAN1	NR_NTN_enh-Core

Remove “Based on the analysis provided in several TDocs (e.g. R2-2410527 or R2-24010438)”

Change “conclusion” to “assumption”

Remove Draft

Revised in R2-2411191

R2-2411191	LS on inter-gNB RACH-less HO in NTN	RAN2	LSout	Rel-18	NR_NTN_enh-Core 	To:RAN3, Cc:RAN1

Approved

38.306

R2-2409544	Correction on NTN in FR2 bands	vivo, ZTE Corporation, Sanechips	CR	Rel-18	38.306	18.3.0	1200	-	F	NR_NTN_enh-Core

-	Regarding the ChannelBW, QC thinks that 200MHz bandwidth is not mandatory for FR2-NTN

-	HW thinks there is no agreement that MIMO layer can only be set to 1

-	QC thinks sending an LS to RAN4 would be good

Postponed

Send a LS to RAN4 asking about UE capabilities for 200MHz ChannelBW for FR2-NTN and about 2 MIMO layers

R2-2411192	Draft LS on UE capabilities for FR2-NTN	vivo	LSout	Rel-18	NR_NTN_enh-Core	To:RAN4

Remove Draft

Revised in R2-2411195

R2-2411195	LS on UE capabilities for FR2-NTN	vivo	LSout	Rel-18	NR_NTN_enh-Core	To:RAN4

Approved

R2-2409948	UE feature for SIB19 reception in RRC_IDLE/RRC_INACTIVE state in a TN cell	Apple, Qualcomm Incorporated, Samsung, Huawei, HiSilicon, CATT, MediaTek Inc.	CR	Rel-18	38.306	18.3.0	1206	-	F	NR_NTN_enh-Core

-	Ericsson/Vivo/ZTE/Nokia think the capability should be about measurements for cell reselection and then it should go in 5.6

Update the name of the capability and the description

Move the change to subclause 5.6 and fix the coversheet (impact analysis)

Revised in R2-2410979

R2-2410979	Clarification on the UE feature for cell reselection from TN to NTN	Apple, Qualcomm Incorporated, Samsung, Huawei, HiSilicon, CATT, MediaTek Inc.	CR	Rel-18	38.306	18.3.0	1206	1	F	NR_NTN_enh-Core

Revise as: “It is optional for the UE in RRC_IDLE or in RRC_INACTIVE in a TN cell to support the measurement of NTN neighbor cells for cell reselection based on information acquired in SIB19 as specified in TS 38.304 [21] and in TS 38.133 [5]. This feature is only applicable if the UE supports nonTerrestrialNetwork-r17.”

Revised in R2-2411193

R2-2411193	Clarification on the UE feature for cell reselection from TN to NTN	Apple, Qualcomm Incorporated, Samsung, Huawei, HiSilicon, CATT, MediaTek Inc.	CR	Rel-18	38.306	18.3.0	1206	2	F	NR_NTN_enh-Core

Agreed

Stage 2

R2-2409606	Correction on coexistence between CHO and satellite switching with re-synchronization	CATT, Nokia, Nokia Shanghai Bell, Ericsson	CR	Rel-18	38.300	18.3.0	0903	1	F	NR_NTN_enh-Core	R2-2407968

Add the impact analysis

Revised in R2-2410980

R2-2410980	Correction on coexistence between CHO and satellite switching with re-synchronization	CATT, Nokia, Nokia Shanghai Bell, Ericsson	CR	Rel-18	38.300	18.3.0	0903	2	F	NR_NTN_enh-Core

Agreed

R2-2410642	Correction on RACH-less HO in NR-NTN	vivo, Samsung, Nokia, Nokia Shanghai Bell, THALES, Huawei, HiSilicon, CATT	CR	Rel-18	38.300	18.3.0	0904	1	F	NR_NTN_enh-Core	R2-2408013

Postponed

## 7.5	Enhanced NR Sidelink Relay

(NR_SL_relay_enh-Core; leading WG: RAN2; REL-18; WID: RP-223501)

Time budget: 0TU

Tdoc Limitation: 1 tdoc

1 additional tdoc on top of the limit is allowed for co-sourced contribution with 3 or more companies.

Minor and editorial issues should be coordinated with the appropriate spec rapporteur and submitted by rapporteur company together with any additional corrections the rapporteur company may have.    Larger issues can be discussed based on contributions/individual CRs.

### 7.5.0	In-principle agreed CRs

Contributions agreed in principle at RAN2#127bis.

R2-2409631	Corrections on security for L2 U2U relay	vivo	CR	Rel-18	38.323	18.3.0	0141	1	F	NR_SL_relay_enh-Core	R2-2408374

Revised in R2-2410918

R2-2410918	Corrections on security for L2 U2U relay	vivo	CR	Rel-18	38.323	18.3.0	0141	2	F	NR_SL_relay_enh-Core	R2-2408374

Agreed

R2-2409682	RRC correction on NR SL U2U relay operation	Philips International B.V.	CR	Rel-18	38.331	18.3.0	5048	2	F	NR_SL_relay_enh-Core	R2-2409263

Agreed

R2-2409735	Clarification for ul-DataSplitThreshold setting in multi-path relay	OPPO	CR	Rel-18	38.331	18.3.0	5081	1	F	NR_SL_relay_enh-Core	R2-2409118

Agreed

R2-2409759	Miscellaneous CR for Rel-18 SL relay enhancement	Huawei, HiSilicon	CR	Rel-18	38.331	18.3.0	4994	2	F	NR_SL_relay_enh-Core	R2-2409262

Agreed

R2-2409760	Correction to error handling for U2U operation	Huawei, HiSilicon	CR	Rel-18	38.351	18.2.0	0037	2	F	NR_SL_relay_enh-Core	R2-2409264

Agreed

Late AIP CR submitted at end of meeting

R2-2411185 Miscellaneous and editorial changes for sidelink relay enhancement	LG Electronics Inc., ZTE Corporation, Sanechips, Philips International B.V.	CR	Rel-18	38.300	18.3.0	0923	1	F	NR_SL_relay_enh-Core

[Post128][405][Relay] Check of late AIP stage 2 CR on Rel-18 relay (LG)

Scope: Check the AIP CR in R2-2411185 and confirm that it can be agreed.

Intended outcome: Agreed CR

Deadline: Short (for RP)

=> Agreed in R2-2411185

### 7.5.1	Organizational

Including incoming LSs and rapporteur inputs.

### 7.5.2	Stage 2 corrections

Impact to 38.300.

R2-2410197	U2U Relays, Local ID Assignment	Ericsson	discussion	Rel-18	R2-2408879

Noted

Discussion:

Huawei think it is already clear that this is UE implementation.

LG note that there is a similar note in the QoS split.

ZTE think we normally only specify what the UE should do, and what is not captured is automatically up to UE implementation; we cannot capture all implementation behaviour.

Ericsson note that we had an agreement that it was up to UE implementation.

Samsung think the NOTE is useful and we have similar notes in other places.

MediaTek think there is room for confusion and the NOTE is helpful.

Apple think we concluded that there would be no specification impact, which in hindsight was not quite right because there is indirect impact to SRAP.

[AT128][407][Relay] NOTE on U2U local ID allocation (Ericsson)

Scope: Draft a CR based on the TP in R2-2410197.

Intended outcome: Agreed CR (without CB if possible) in R2-2411000

Deadline: Wednesday 2024-11-20 1600 EST

R2-2411000	U2U Relays, Local ID Assignment	Ericsson	CR	Rel-19	38.300	18.3.0	0944	-	F	NR_SL_relay_enh-Core

Revised in R2-2411176 (email discussion [AT128][407])

R2-2411176	U2U Relays, Local ID Assignment	Ericsson	CR	Rel-19	38.300	18.3.0	0944	1	F	NR_SL_relay_enh-Core

Agreed (email discussion [AT128][407])

### 7.5.3	Control plane corrections (including UE capabilities)

Impact to 38.331, 38.304, and 38.306.

R2-2409853	Clarification on the Terminology of Peer UE	CATT	discussion	Rel-18	NR_SL_relay_enh-Core

Noted

Proposal 2: Suggest RAN2 to discuss how to handle the issue that the same terminology of peer UE refers to different types of UEs in same/different sections:

-	Option 1: The peer UE only refers to Rel-16 V2X target UE and Rel-17 U2N Relay UE (same principle as in the Rel-17 specification);

-	Option 2: The peer UE refers to the target UE which has direct PC5 or indirect PC5 connection with the UE which depends on the detailed procedure description. No need to identify it in the general description;

-	Option 3: The peer UE refers to the target UE which has direct PC5 or indirect PC5 connection with the UE which depends on the detailed procedure description. A definition of peer UE or a NOTE can be added in the specification.

Discussion:

OPPO are not sure that there is a problem with the current text.

Samsung think option 2 is agreeable.

ZTE think “peer UE” should refer only to direct PC5 interface connection; otherwise they understand that we would have unwanted effects on other features like CA.

NEC note that we have not discussed whether relays would support CA, but we deleted the text that said it would not, so they think we do not need to indicate it explicitly here or change the terminology.

Huawei do not see a relation to CA.

[AT128][405][Relay] Peer UE terminology (CATT)

Scope: Start from the TP in R2-2409853 and develop a CR in line with the principle of using “peer UE” in a consistent way for both direct and indirect connections.

Intended outcome: Agreeable CR in R2-2410996

Deadline: Wednesday 2024-11-20 1600 EST

R2-2410996	Corrections on the Terminology of Peer UE	CATT	CR	Rel-18	38.331	18.3.0	5197	-	D	NR_SL_relay_enh-Core

Change to category F

Affected architecture to be added

Agreed with these changes as R2-2411182

R2-2411182	Corrections on the Terminology of Peer UE	CATT	CR	Rel-18	38.331	18.3.0	5197	1	F	NR_SL_relay_enh-Core

Agreed

R2-2409960	Corrections on RRC SRAP configuration for L2 U2U	Apple, ZTE	CR	Rel-18	38.331	18.3.0	5125	-	F	NR_SL_relay_enh-Core

Revised in R2-2410997

Discussion:

Apple indicate that some offline comments were received and changes are expected.

Nokia note that the DRB release operation is removed at the beginning of the changes, and they are not sure it is covered elsewhere.

Apple understand that when the DRB is added or removed, it will trigger the update to the PC5 RLC channel configuration, but they understand that there may be some aspects that need to be added still.

Huawei are not sure there is a problem with the existing text apart from some repetitive parts.  They think we could live with no changes.

Nokia think something does need to change.

ZTE think this interacts with the peer UE terminology discussion.  Samsung thought the terminology discussion was not directly related to how we organise the bearer configuration management.

OPPO think the CRs may have some relationship because Apple’s CR would remove some of the U2U sections that cause the terminological ambiguity; they understand that if Apple’s CR is agreed then we have aligned with CATT’s option 2.

Apple think the discussions could be kept separate and we look at any interactions when we come back.

[AT128][406][Relay] U2U SRAP configuration (Apple)

Scope: Check and update the CR in R2-2409960.

Intended outcome: Agreeable CR in R2-2410997

Deadline: Wednesday 2024-11-20 1600 EST

R2-2410997	Corrections on RRC SRAP configuration for L2 U2U	Apple, ZTE	CR	Rel-18	38.331	18.3.0	5125	1	F	NR_SL_relay_enh-Core

References to 5.8.9.7.x to be removed from coversheet

Impacted architecture to be added (NR SA)

Agreed with these changes as R2-2411183

R2-2411183	Corrections on RRC SRAP configuration for L2 U2U	Apple, ZTE	CR	Rel-18	38.331	18.3.0	5125	2	F	NR_SL_relay_enh-Core

Agreed

R2-2410614	Corrections for U2U relay measurements	ZTE Corporation, Sanechips, Huawei, Hisilicon, OPPO, CATT, Apple	CR	Rel-18	38.331	18.3.0	5175	-	F	NR_SL_relay_enh-Core

“if provided” to be added to the four instances of UE requirements to apply the coefficients

Agreed with this change as R2-2410998

Discussion:

Nokia have a concern about backward compatibility: What will a UE do if it implements the CR and the network does not?  ZTE understand that the new parameters only influence the result of the measurement, and without them the UE can still get a measurement result.

### 7.5.4	User plane corrections (including SRAP)

Impact to 38.351, 38.321, 38.322, and 38.323.

R2-2410586	RLC correction for multi-path relay with N3C	Huawei, HiSilicon	CR	Rel-18	38.322	18.1.0	0063	-	F	NR_SL_relay_enh-Core

Keep the first change only (first instance of “upper layers or N3C”)

Add N3C to the acronym list

Agreed with these changes as R2-2410999

Discussion:

Toyota are fine with the intention but think there is a typo, “SDU” that should be “SDUs” in NOTE 3.

OPPO are not sure if the change is correct; the description somehow implies a 1:1 mapping between RLC channels and logical channels, and may suggest that all packets from N3C will be delivered to a single logical channel.

Huawei indicate that the intention is that there is a connection between the RLC entity and N3C, not implying anything specific about channelisation.

Samsung understand that the CR does not change the mapping to logical channels.

Ericsson think the NOTE is not necessary from a configuration perspective; it is clear how to map the layers.  They think the NOTE actually may not be correct as written in terms of how the configuration is structured.

ZTE suggest creating a new paragraph for the N3C case separately.

OPPO think it is hard to describe the relationship between N3C and the parts in 3GPP scope, and they would rather leave this part to stage 2.

Apple somewhat agree with Ericsson and think the NOTE does not really add anything.

Huawei see that in the current RLC spec, there is no connection between the N3C entity and the RLC entity, and they think this needs to be clarified for implementation teams.

Ericsson understand the implementation concern but reiterate that N3C is outside our scope and cannot be recognised in our specs as a layer; it is just an identifier.

Samsung generally agree with Ericsson that we do not specify non-3GPP behaviour, but here we have some interaction with our layers; however, they do not think the issue is serious enough to merit a CR.

Huawei do not intend for the CR to imply anything about configuration, just to define the flow of SDUs between the two entities.  OPPO think stage 2 does this adequately.

Qualcomm think the CR is useful over the description in stage 2 for clarity.  ZTE understand that the reason we left the mapping in stage 2 to UE implementation was that there was no way to agree on a fixed behaviour; they are afraid that putting more detail in can open a can of worms.

Huawei think we could leave out the note, but have some indication that the RLC entities exchange the SDUs via N3C.

Xiaomi think we could remove the NOTE and keep the normative changes.

OPPO’s concern is mainly for the normative text.

Huawei suggest a separate sentence saying “SDUs can also be exchanged between the RLC entities via N3C”.  Nokia think this is not a good solution and the reason we had a NOTE now was that companies could not agree to normative text.  Samsung understand that the intention of the NOTE was to avoid interfering with the meaning of the channel, and this would be a different scenario.

Ericsson think we have discussed this issue a lot and it is not very critical.  They think the stage 2 is clear enough.

Xiaomi think we normally implement from the stage 3 spec.

## 7.6	NR Sidelink evolution

(NR_SL_enh2; leading WG: RAN1; REL-18; WID: RP-230077)

Time budget: 0 TU

Tdoc Limitation: 1 tdocs

1 additional tdoc on top of limited can be allowed for co-sourced contribution with 3 or more companies

### 7.6.1	Organizational

Including incoming LSs and rapporteur inputs.

### 7.6.2	In-principle agreed CRs

R2-2409549	Co-configuration of random/partial-sensing resource selection and Co-Ex	OPPO	CR	Rel-18	38.331	18.3.0	4976	1	F	NR_SL_enh2	R2-2407972

Agreed.

R2-2410723	Correction on MCSt	LG Electronics Inc.	CR	Rel-18	38.321	18.3.0	1943	2	F	NR_SL_enh2	R2-2409371

Agreed.

[LG]: Indicates revision history was added compared to in-principle agreed CR last meeting.

R2-2410724	MAC correction for resource selection of MCSt	LG	CR	Rel-18	38.321	18.3.0	1973	1	F	NR_SL_enh2	R2-2409351

Agreed.

[LG]: Indicates revision history was added compared to in-principle agreed CR last meeting.

R2-2410727	Correction on Co-channel coexistence for LTE sidelink and NR sidelink	LG Electronics Inc.	CR	Rel-18	38.321	18.3.0	1942	1	F	NR_SL_enh2	R2-2408637

R2-2411115	Correction on Co-channel coexistence for LTE sidelink and NR sidelink	LG Electronics Inc.	CR	Rel-18	38.321	18.3.0	1942	2	F	NR_SL_enh2	R2-2408637

Agreed.

[LG]: Indicates revision history was added compared to in-principle agreed CR last meeting.

R2-2410721	Correction on Co-channel coexistence for LTE sidelink and NR sidelink	LG Electronics Inc.	CR	Rel-18	38.321	18.3.0	2013	-	F	NR_SL_enh2	Withdrawn

### 7.6.3	Others

Including corrections to all specifications.Minor and editorial issues should be coordinated with the CR rapporteur.

R2-2410612	Stage 2 Corrections on MCSt	InterDigital Inc, Apple	CR	Rel-18	38.300	18.3.0	0939	-	F	NR_SL_enh2

Check what was agreed/implemented in RAN2, whether both approach 1 and approach 2 (regarding MCSt and PCFCH) are enough covered, offline.

CR will be revised in R2-2410943

[LG, Vivo]: To be more accurate, it should be restricted to single TB case. [OPPO]: Not sure if it is clear whether there is the restriction in MAC spec. If we don’t have such restriction in MAC, this CR should be ok. [Huawei]: In MAC, there is a case of MCSt where ACK is used and assume that it is for the multiple TB case. [Qualcomm]: Have different understanding. We agreed with no MCSt for both single TB case and multiple TB case, with PSFCH. [Qualcomm]: In addition, the current MAC is not clear on the differentiation of approach 1 and approach 2.

[AT128][112][V2X/SL] (ZTE)

Scope: Check what was agreed/implemented in RAN2, whether both approach 1 and approach 2 (regarding MCSt and PSFCH) are enough covered, whether we need to update MAC, actual MAC changes if needed, and attempt to reach RAN2 consensus.

Intended outcome: Discussion summary in R2-2410945. F2F offline discussion schedule will be announced by offline discussion rapporteur.

Deadline: Comeback in CB session.

R2-2410945	Discussion on MCSt	ZTE Corporation, Sanechips	Discussion	NR_SL_enh2

Proposal1: Case 1c is supported, that is N=1 or no value N is indicated to PHY layer for sensing, for a resource pool configured with PSFCH resource, MAC layer may select consecutive slots for transmissions of multiple TBs, if minimum time gap for HAQR feedback is ensured.

Postponed.

[Qualcomm]; PSFCH needs 4 symbols. Gap should be less than 16us/25us for MCSt. [Session Chair]: Question is if a resource pool with PSFCH is supported in RAN1? [Huawei]: Understand it is clear PSFCH with 4 symbols between two TBs will not impact MCSt in RAN1. Should be allowed. [OPPO]: Think it is good to postpone the discussion and it will be good to see the corresponding spec impacts if we allow case 1c. [Ericsson]: If 1c is related only to approach 1 then since approach 1 is done based on best effort manner, if the UE receives ACK in the middle of MCSt, then another solution is that the UE can stop MCSt operation for the remaining slots.

R2-2410943	Stage 2 Corrections on MCSt	InterDigital Inc, Apple	CR	Rel-18	38.300	18.3.0	0939	1	F	NR_SL_enh2

Postponed.

R2-2410583	RRC correction on SidelinkUEInformationNR for NR sidelink transmission	Philips International B.V., NEC	CR	Rel-18	38.331	18.3.0	5173	-	F	NR_SL_enh2-Core

Not pursued.

[OPPO, Apple, Qualcomm]: The reason why we didn’t include sl-FreqInfoListSizeExt was this procedure is applied to mode 1 and CA is not applicable to mode 1.

R2-2410714	MAC corrections on Release-18 Sidelink evolution	LG Electronics Inc.	CR	Rel-18	38.321	18.3.0	2012	-	F	NR_SL_enh2

Agreed.

R2-2410001	Correction on carrier selection for SL evolution	Huawei, HiSilicon, LG Electronics Inc., Apple, Nokia	CR	Rel-18	38.321	18.3.0	1936	1	F	NR_SL_enh2-Core	R2-2408363

The condition “if there is no SL grant on the carrier where the SL-CSI Request was received.” will be removed.

The cover page’s impacted functionality will be “NR Sidelink carrier aggregation”

With the changes above, CR in R2-2410944 is agreed.

[Huawei]: For the second change (adding “if there is no SL grant on the carrier where the SL-CSI Request was received.”), if it’s clear that it was already covered by TX resource check procedure, it can be removed. [Ericsson]: Cover page should be updated to correct impacted functionality to “NR Sidelink carrier aggregation”. [OPPO]: Prefer removing the sentence “if there is no SL grant on the carrier where the SL-CSI Request was received.” since it is already covered by TX resource check.

R2-2410072	Correction on carrier selection for IUC	ZTE Corporation, Sanechips, Ericsson, Apple, Nokia	CR	Rel-18	38.321	18.3.0	1984	-	F	NR_SL_enh2-Core

Agreed.

R2-2410073	Correction on MCSt	ZTE Corporation, Sanechips	CR	Rel-18	38.321	18.3.0	1985	-	F	NR_SL_enh2-Core

Postponed.

R2-2410147	discussion on remaining issue of resource selection for MCSt	Ericsson	discussion	Rel-18	NR_SL_enh2

New note will be placed between NOTE 3Al and NOTE 3A1.

Change to NOTE 3A1 is not needed.

With the change above, CR will be revised in R2-2410946

[Apple]: New note should be moved to between NOTE 3Al and NOTE 3A1.

R2-2410946	Correction to MCSt for sidelink	Ericsson	CR	Rel-18	38.321	18.3.0	2018	-	F	NR_SL_enh2

Agreed.

R2-2410177	Correction on SL IUC timer for SL CA	ASUSTeK	CR	Rel-18	38.321	18.3.0	1993	-	F	NR_SL_enh2

Not pursued.

[Ericsson]: We have never discussed per carrier SL IUC timer, so before agreeing with the CR, we need discussion on the need first. [Qualcomm]: What we need to discuss is whether to support IUC in multiple carriers at the same time. [OPPO]: Prefer to keep the current text. Even without consideration of SL CA, even in a single carrier case, we already have similar situation cross multiple resource pools. [Apple]: Considering we didn’t have any mechanism to cover multiple IUCs in multiple resource pools at the same time, agree with OPPO. [ZTE]: Although it is similar to CL-CSI, assume we still can survive without multiple SL IUCs. It is too late for this kind of discussion.

R2-2409729	TP for SL enhancemen in TS 38.321	NEC Corporation	discussion	NR_SL_enh2-Core

[Qualcomm]: Understand it is not correct. The UE does not need to check keep condition for all carriers that have grants. [ZTE]: Wording is confusing due to “allowed”.

CB session:

[NEC]: Based on offline discussion, companies should be ok with the following simple change.

R2-2411162	Correction on carrier selection for SL evolution	NEC	CR	Rel-18	38.321	18.3.0	2021	-	F	NR_SL_enh2

Remove “(s)” in pool(s).

Agreed in R2-2411140

[ZTE]: With this change, it seems the UE can select more than one resources in a given carrier? [NEC]: Understand selection of multiple resource pools is already allowed in legacy release, so it is not the issue. [NEC]: If companies are not sure, we can remove “(s)” in pools now.

## 7.7	TEI18

Specific items may be allocated to a breakout session for treatment.   Essential corrections only.  No new proposals will be treated.

Time budget: 1 TU

Tdoc limitation: 1

### 7.7.0	In-principle agreed CRs

R2-2410536	Correction for cell barring for 2Rx XR UE [2Rx_XR_Device]	Huawei, HiSilicon, Xiaomi, Ericsson, LGE, ZTE Corporation	CR	Rel-18	38.331	18.3.0	4851	2	F	TEI18	R2-2408733

=> Revised in R2-2411075

R2-2411075	Correction for cell barring for 2Rx XR UE [2Rx_XR_Device]	Huawei, HiSilicon, Xiaomi, Ericsson, LGE, ZTE Corporation	CR	Rel-18	38.331	18.3.0	4851	3	F	TEI18

The CR is agreed

R2-2410833	Introduction of new capability for intra-band EN-DC channel spacing [Intra-Band_EN-DC_Channelspacing]	Huawei, HiSilicon	CR	Rel-18	38.331	18.3.0	5013	1	B	TEI18	R2-2408474

The CR is agreed

R2-2410834	Introduction of new capability for intra-band EN-DC channel spacing [Intra-Band_EN-DC_Channelspacing]	Huawei, HiSilicon	CR	Rel-18	38.306	18.3.0	1174	2	B	TEI18	R2-2409399

The CR is agreed

### 7.7.1	TEI proposals by Other Groups

Items initiated by other groups that is/has been communicated by LS, where the other group indicate this is TEI18. (Specific other-group-WIs should use the R18 Other Agenda Item below).

### 7.7.2	TEI proposals by RAN2

Items initiated in RAN2 for NR and LTE.

Contributions should focus only critical issues/corrections for already agreed TEI-18 topics.

CG-SDT

R2-2410535	Correction for Paging monitoring during SDT [CG-SDT-Enh]	Huawei, HiSilicon, LG Electronics Inc., ZTE Corporation, Sanechips, MediaTek Inc.	CR	Rel-18	38.331	18.3.0	4901	2	F	TEI18	R2-2408732

Update with architecture option

The CR is agreed in R2-2411108 with change above

Cell barring

R2-2409747	Clarifications on cell barring behaviour	Qualcomm Incorporated, Huawei, ZTE, Ericsson, LG Electronics Inc., Nokia, Samsung, Vivo	CR	Rel-18	38.331	18.3.0	5112	-	F	TEI18

Highlight network side functional change related to 2RX

Missing TEI identifiers in title

Add TEI18 in WI code

The CR is agreed in R2-2411109 with changes above

R2-2409748	Clarifications on cell barring behaviour	Qualcomm Incorporated, Huawei, ZTE, Ericsson, LG Electronics Inc., Nokia, Samsung, Vivo	CR	Rel-18	38.304	18.3.0	0421	-	F	TEI18

Highlight network side functional change related to 2RX

Missing TEI identifiers

Add TEI18 in WI code

The CR is agreed in R2-2411110 with changes above

R2-2410537	Corrections on the conditional presence of barring exemption for emergency call [EM_Call_Exemption]	Huawei, HiSilicon	CR	Rel-18	38.331	18.3.0	5166	-	F	TEI18

-	Ericsson has been supporting this for a long time, but brings up a new issue for the case of home PLMN.

-	ZTE doesn’t think this change is needed, we are overspecifying and the network can handle it.

-	Huawei didn’t want to go into all the PLMN cases and just specified a general thing.

-	Qualcomm also doesn’t agree to CR expect adding 2RX in front of XR.

-	Samsung, Apple agrees with ZTE and Qualcomm and we can leave it to network implementation.

-	Apple indicates that this parameter was added with something else in mind so overloading may bring backward compatible issues.  Huawei thinks that for this feature to be implementable there should be reassurance that the network behavior is clearly defined, if network supports this new field should be broadcast.

-	Companies think it quite obvious and no further CRs are expected.

Add the 2RX infront of XR in the em barring field in R2-2411109

The CR is not pursued

2XR

R2-2410206	Correction on the 2RxXR and Aerial UEs [2Rx_XR_Device]	Ericsson, Samsung, Qualcomm Inc.	CR	Rel-18	38.331	18.3.0	5148	-	F	NR_UAV-Core, NR_XR_enh-Core

-	Qualcomm wonders if this should be combined with UAV CR.   Samsung wonders if it makes sense to merge as the other one doesn’t impact UAV.    Huawei agrees with Qualcomm.

-	Huawei agrees with CR.

-	ZTE explains that we have previously covered different changes for different WI by clarifying in cover page and also using different tracked changes.

The CR is merged in R2-2411202

Measurement report

R2-2410463	Correction on reporting the best cell change [meas_report_enh]	Ericsson	CR	Rel-18	38.331	18.3.0	5163	-	F	TEI18

-	Huawei understands that according to sorting order it means from highest to lowest.

-	Samsung thinks that we can just change from first to best for the first bullet point.

The CR is not pursued

R2-2410192	Enhancement to measurement report	OPPO	CR	Rel-18	38.331	18.3.0	5147	-	F	TEI18

Missing TEI identifier

-	Samsung thinks that if the network is smart it will set it properly only if report on leave is set to true.

The CR is not pursued

## 7.8	R18 Other

Specific items may be allocated to a breakout session for treatment.

Impacts from Other RAN WGs and TSGs that has no separate TU budget in RAN2. LS ins for Rel-18 specific WIs/SIs that has no RAN WI.

Clarification CRs should be discussed with spec rapporteurs of the topic prior to submission.

Time budget: 1 TU

Tdoc Limitation: 2

### 7.8.0	In-principle agreed CRs

R2-2410041	Correction on SPR content determination	Google	CR	Rel-18	38.331	18.3.0	5135	-	F	NR_ENDC_SON_MDT_enh2-Core

The CR is agreed

R2-2410487	Clarification on SCS of Timing Advance Report MAC CE for ATG	Samsung, Qualcomm Inc., Huawei, HiSilicon, CMCC, CATT	CR	Rel-18	38.321	18.3.0	2004	-	F	NR_ATG-Core	Withdrawn

R2-2410664	Supporting R17 early implementation of R18 measurement gap enhancements	Nokia, Nokia Shanghai Bell, Ericsson, ZTE Corporation, BT Plc., Telecom Italia, CATT, Samsung	CR	Rel-18	36.331	18.3.1	5064	1	F	NR_MG_enh2-Core	R2-2408976

The CR is agreed

R2-2410851	Correction on UE capability for multi-carrier enhancements	NTT DOCOMO, INC., Huawei, HiSilicon	CR	Rel-18	38.306	18.3.0	1199	1	F	NR_MC_enh-Core	R2-2409403

=> Revised in R2-2410897

R2-2410897	Correction on UE capability for multi-carrier enhancements	NTT DOCOMO, INC., Huawei, HiSilicon	CR	Rel-18	38.306	18.3.0	1199	2	F	NR_MC_enh-Core

The CR is agreed

R2-2410852	Correction on UE capability for multi-carrier enhancements	NTT DOCOMO, INC., Huawei, HiSilicon	CR	Rel-18	38.331	18.3.0	5095	1	F	NR_MC_enh-Core	R2-2409404

=> Revised in R2-2410898

R2-2410898	Correction on UE capability for multi-carrier enhancements	NTT DOCOMO, INC., Huawei, HiSilicon	CR	Rel-18	38.331	18.3.0	5095	2	F	NR_MC_enh-Core

=> Revised in R2-2411074

R2-2411074	Correction on UE capability for multi-carrier enhancements	NTT DOCOMO, INC., Huawei, HiSilicon	CR	Rel-18	38.331	18.3.0	5095	3	F	NR_MC_enh-Core

The CR is agreed

R2-2410872	Clarification on SCS for Timing Advance Report MAC CE for ATG	Samsung, Qualcomm Inc., Huawei, HiSilicon, CMCC, CATT	CR	Rel-18	38.321	18.3.0	1954	1	F	NR_ATG-Core	R2-2408807

Update with architecture option

The CR is agreed in R2-2411111 with changes above, but then the CR was coversheet revised by MCC in R2-2411261: wrong rev number

R2-2411261	Clarification on SCS for Timing Advance Report MAC CE for ATG	Samsung, Qualcomm Inc., Huawei, HiSilicon, CMCC, CATT	CR	Rel-18	38.321	18.3.0	1954	3	F	NR_ATG-Core

=> Agreed

### 7.8.1	RAN4 led items

Including outcome of [POST127bis][011][less5MHz] 331 CR (ZTE)

Including incoming LS from RAN4 R4-2417119.   Input can be provided and will count towards tdoc limit.

Less than 5MHz

R2-2410768	Report of [Post127bis][011][less5MHz] 331 CR (ZTE)	ZTE Corporation	discussion	Rel-18	NR_FR1_lessthan_5MHz_BW-Core

Noted

R2-2409523	LS on Rel-19 NR channel BW less than 5MHz for FR1 (R4-2417119; contact: Intel)	RAN4	LS in	Rel-19	NR_FR1_lessthan_5MHz_BW_Ph2	To:RAN2

Noted

R2-2409654	Discussion on UE capability on less than 5MHz (LS R4-2417119)	CATT	discussion	Rel-19	NR_FR1_lessthan_5MHz_BW_Ph2

Noted

R2-2410260	Less than 5Mhz and carrier configuration	Nokia	discussion	Rel-18	NR_FR1_lessthan_5MHz_BW

Wait for RAN4 response

R2-2410769	Correction on the Less than 5M Bandwidth	ZTE Corporation, vivo	CR	Rel-18	38.331	18.3.0	5187	-	F	NR_FR1_lessthan_5MHz_BW-Core

The CR is updated and reviewed over email.

Discussion

ZTE Extend the supportedBandwidthDL/UL to include 3MHz.

CATT To declare the support of less than 5 MHz bandwidth operation in the CA/DC band combination,  make the following updates for UE capability signaling:

Add support3MHz-ChannelBW-Symmetric-r18 and support12PRB-CORESET0-r18 in FeatureSetDownlinkPerCC.

Add support3MHz-ChannelBW-Asymmetric-r18 in FeatureSetUplinkPerCC.

-	Qualcomm agrees with CATTs proposals, as it is cleaner and easier.   ZTE wants to clarify that for the other Asymetric case we use the legacy mechanism.

-	Ericsson thinks that this now messy as we have legacy signaling and supports ZTE.    Nokia, Huawei, Samsung has the same view as Ericsson and ZTE and doesn’t see the reason to introduce new mechanism.

-	Tmobile has two other channel BWs (6 and 7MHz BWs proposed) in RAN4 so wants to make sure we are consistent with how we add channel BWs.   Is the approach forward compatible?  Qualcomm thinks it is forward compatible for less than 5, but not sure about between 5 and 10.    Samsung thinks this is also dependent on RAN4, if requirements are similar than we can use same framework.   ZTE thinks that in current CR we now we only have 1 spare bits, so we may want to consider having 3 spare bits if we want to make it forward compatible.  .

Agreements

- Extend the supportedBandwidthDL/UL to include 3MHz.

- On the per band capabilities, do not indicate the 3M in the channelBWs-DL/UL but keep the support3MHz-ChannelBW-Asymmetric-r18/ support3MHz-ChannelBW-Symmetric-r18;

- Extend supportedMinBandwidthDL/UL-r17 to include 3MHz

- Remove the single carrier restriction in the field description of support5MHz-ChannelBW-20PRB-CORESET0-r18 and support12PRB-CORESET0-GSCN-41637-r18.

[AT128][012][less5MHz] UE capabilities (ZTE)

Intended outcome: Capture agreements in CR, look at forward compatibility and agree to CR

Deadline:  Thursday

R2-2411163	Correction on the Less than 5M Bandwidth	ZTE Corporation, vivo	CR	Rel-18	38.331	18.3.0	5187	1	F	NR_FR1_lessthan_5MHz_BW-Core

The CR is agreed

R2-2411164	Correction on the Less than 5M Bandwidth	ZTE Corporation	CR	Rel-18	38.306	18.3.0	1220	-	F	NR_FR1_lessthan_5MHz_BW-Core

The CR is agreed

NR_BWP_wor-Core

R2-2409778	Correction on UE capability on ncd-SSB-BWP-Wor-r18	vivo, Qualcomm Incorporated, Guangdong Genius	CR	Rel-18	38.306	18.3.0	1204	-	F	NR_BWP_wor-Core

R2-2411153	Correction on UE capability on ncd-SSB-BWP-Wor-r18	vivo, Qualcomm Incorporated, Guangdong Genius	CR	Rel-18	38.306	18.3.0	1204	1	F	NR_BWP_wor-Core

Agree to address the issue but need to discuss further how to solve it in the best way

The CR is postponed

[POST128][023][NR Other] NR_BWP (Vivo)

Intended outcome: Agree how to capture the intention of the CR

Deadline:  long email

R2-2410839	Correction on BWP operation without bandwidth restriction	Huawei, HiSilicon, ZTE Corporation, Ericsson	CR	Rel-18	38.306	18.3.0	1177	1	F	NR_BWP_wor	R2-2408480

The CR is agreed

ATG

R2-2410302	Clarification of offsetThresholdTA-r18 for NR ATG	Huawei, HiSilicon, CATT, Nokia, Nokia Shanghai Bell, LG Electronics Inc.	CR	Rel-18	38.331	18.3.0	4882	2	F	NR_ATG-Core	R2-2408444

The CR is not pursued

R2-2410486	Correction on SCS applied for TAR offset threshold for ATG	Samsung, ZTE, Qualcomm Inc., Ericsson	CR	Rel-18	38.331	18.3.0	5164	-	F	NR_ATG-Core

The CR is not pursued

R2-2410803	Alternative solution for clarification on the unit of offsetThresholdTA-r18 for ATG	ZTE Corporation	CR	Rel-18	38.331	18.3.0	5189	-	F	NR_ATG-Core

-	Huawei is not fine with this as it changes network behaviour.   Samsung prefers this one over Huawei’s CR.

-	Huawei and Qualcomm would like to send an LS to ask a question, that we are deciding between active and initial BWP and what was the intention from RAN4.   ZTE and Samsung don’t think an LS helps much.

[AT128][013][ATG] offline  (ZTE)

Intended outcome: WF

Deadline:  Thursday

R2-2411180	Clarification on the unit of offsetThresholdTA-r18 for ATG	ZTE Corporation	CR	Rel-18	38.331	18.3.0	5189	1	F	NR_ATG-Core

-	Qualcomm sees a risk of mismatch if network doesn’t include active bandwidth part in the same message.

The CR is agreed, but then coversheet revised by MCC in R2-2411262: the spec number on the coversheet is faulty.

R2-2411262	Clarification on the unit of offsetThresholdTA-r18 for ATG	ZTE Corporation	CR	Rel-18	38.331	18.3.0	5189	2	F	NR_ATG-Core

=> Agreed

R2-2410148	Correction on HST FR2	Ericsson	CR	Rel-18	38.306	18.3.0	1210	-	F	NR_HST_FR2_enh

-	Huawei thinks the spec is clear if you look at the definition of PC6 this is already clear.  Nokia agrees

The CR is not pursued

NR_MG_enh2

R2-2410665	Supporting R17 early implementation of R18 measurement gap enhancements	Nokia, Nokia Shanghai Bell, Ericsson, ZTE Corporation, BT Plc., Telecom Italia, CATT, Samsung	CR	Rel-18	38.331	18.3.0	5065	1	F	NR_MG_enh2-Core	R2-2408977

The CR is agreed

R2-2410749	Simultaneous reception of EUTRAN data and NR SSB with different numerology	Huawei, HiSilicon	discussion	Rel-18	NR_MG_enh2

Verify that the terminology use is consistent with other sections

The CRs will be submitted and agreed by email discussion

Noted

[POST128][024][NR others] NR_MG_enh (Huawei)

Intended outcome: agree to CRs by email

Deadline:  short

R2-2411210	Capability on measurement gap enhancements	Huawei, HiSilicon	CR	Rel-18	36.331	18.3.1	5082	-	F	NR_MG_enh2-Core

=> Agreed

R2-2411211	Capability on measurement gap enhancements	Huawei, HiSilicon	CR	Rel-18	36.306	18.3.0	1903	-	F	NR_MG_enh2-Core

=> Agreed

### 7.8.2	RAN1 led items

R2-2410845	Correction on applicable BWP for multi-cell scheduling	NTT DOCOMO, INC., Nokia, Samsung, Huawei	CR	Rel-18	38.331	18.3.0	5077	1	F	NR_MC_enh-Core	R2-2409100

The CR is agreed, but then coversheet revised by MCC in R2-24011256: wrong CR number on the coversheet

R2-2411256	Correction on applicable BWP for multi-cell scheduling	NTT DOCOMO, INC., Nokia, Samsung, Huawei	CR	Rel-18	38.331	18.3.0	5077	2	F	NR_MC_enh-Core

=> Agreed

R2-2410848	Discussion on applicable BWP for Multi-cell scheduling	NTT DOCOMO, INC., Nokia, Samsung	discussion	Rel-18

RAN2 understands the entries for non-scheduled cells in a row of TCI-DCI-1-3 are interpreted based on the respective current active BWPs on non-scheduled cell.

### 7.8.3	Other

RAN3, SA2, SA3, CT1 led items and others, e.g. eNPN, Slicing, NTN self evaluation issues, etc.

R2-2410496	Introduction of LCS User Plane	Ericsson, Intel Corporation, Huawei, HiSilicon, ZTE Corporation, vivo, Qualcomm Incorporated, Samsung, CATT	CR	Rel-18	38.305	18.3.0	0159	5	F	TEI18	R2-2403538

Revised in R2-2411066

R2-2411066	Introduction of LCS User Plane	Ericsson, Intel Corporation, Huawei, HiSilicon, ZTE Corporation, vivo, Qualcomm Incorporated, Samsung, CATT, Nokia	CR	Rel-18	38.305	18.3.0	0159	6	F	5G_eLCS_Ph3	R2-2410496

Agreed

# 8	Rel-19

## 8.0	General

This AI is reserved for Rel-19 LSs from other WGs.  No contributions are expected on these LSs for this meeting

## 8.1	AI/ML for NR air interface

(NR_AIML_air-Core; leading WG: RAN1; REL-19; WID: RP-242399)

Time budget: 2.5 TU

Tdoc Limitation: 3 tdocs

### 8.1.1	Organizational

LS, Rapporteur input, including workplan, etc.

R2-2409527	Reply LS on AIML data collection (S2-2411191; contact: InterDigital)	SA2	LS in	Rel-19	NR_AIML_air	To:RAN2, RAN3	Cc:SA, RAN, SA3, SA5

Noted

R2-2409531	LS on AI/ML Model transfer/delivery to UE (S5-246292; contact: NEC)	SA5	LS in	Rel-19	FS_AIML_MGT_Ph2	To:RAN2, RAN1	Cc:RAN, SA, RAN3

Noted

R2-2409532	Reply LS on AIML Data Collection (S5-246299; contact Nokia)	SA5	LS in	Rel-18	FS_AIML_MGT_Ph2	To:RAN2	Cc:RAN, RAN1, RAN3, SA, SA2, SA3

Noted

Not treated

R2-2411077	Reply LS on AIML data collection (S3-245138; contact: Samsung)	SA3	LS in	Rel-19	NR_AIML_air	To:RAN	Cc:RAN2, SA, SA1, SA2, SA5

R2-2411177	Reply LS on AIML data collection (R3-247801; contact: ZTE)	RAN3	LS in	Rel-19	NR_AIML_air, NR_AIML_air-Core	To:SA2	Cc:RAN2, SA, RAN, SA3, SA5

### 8.1.2	Functionality based LCM

Contributions should focus on general understanding of LCM procedure (except for data collection and model transfer/delivery), what is required to enable the UE to perform different steps of the LCM procedure, what is the granularity of functionality, dependencies with RAN1 and what is needed from RAN1 to progress in RAN2

Contributions should be submitted in 8.1.2.x and aspects related to data collections should be submitted in data collection section

Two-sided model discussions are out of scope of this AI

Model identification is out of scope of this AI and will not be discussed in RAN2#128 given further RAN1 progress is required.

#### 8.1.2.1	LCM for NW-sided model for Beam Management use case

LCM related to NW-sided model for beam management use case only

No contributions expected for this meeting, waiting for further RAN1 progress

#### 8.1.2.2	LCM for UE-sided model  for Beam Management use case

Including functionality identification, additional conditions and further reporting of applicable functionalities.  Contributions should focus on issues not dependent on RAN1 (i.e. on questions we sent to RAN1) and issues we haven’t yet discussed (e.g. necessary signalling/protocols to configure the UE for training, etc)

Functionality Applicability

Applicability reporting triggers

R2-2409864	Discussion on LCM for UE-sided model for BM	Xiaomi	discussion

Proposal 5: UE shall trigger the report upon candidate functionality applicability change, i.e. applicable to non-applicable or non-applicable to applicable. The candidate functionality is configured by NW.

Noted

R2-2410578	LCM for UE-side models for beam management	Ericsson	discussion

Proposal 14: When a change in applicability of an AI functionality occurs, the UE report includes both active and inactive functionalities, according to RAN1 definition of “active”.

Noted

Discussion

-	Qualcomm would like to refer to CSI report config and the UE should report CSI report config.

-	Apple thinks it should be a “may” and not a shall as this is a UAI.  Nokia thinks for this UAI it should be a shall as the network needs to know.  Huawei has been reading UAI carefully and there are cases it is a may and there are cases of shall.   Qualcomm thinks that going from non-applicable to applicable is not essential so it can be a may, but the other way around it can be a shall.

-	Apple thinks RAN1 is still discussingthe different option, either CSI report config or parameters.   Vivo agrees that we wait for RAN1.

-	LG thinks we need to wait further RAN1 input and not introduce new terminology and agree with Ericssons proposal.

-	CATT asks what is active/inactive definition and we should use applicable/non-applicable.

-	Samsung thinks that RAN1 doesn’t have this definition of functionality configured by the network, so we should just capture functionality configured by the network.

-	Samsung thinks that we should have the same format for UAI whether it is after step 3 or step 5.

When a functionality configured by the network to be reported via UAI, becomes from non-applicable to applicable, the UE can reports it to the network.

FFS detailed design

Handling of nonapplicable functionalities

R2-2410040	Discussion on LCM for UE-sided model for BM	Google	discussion	Rel-19	NR_AIML_air-Core

Proposal 10: The UE autonomously deactivates the activated functionality when it becomes non-applicable and reports the non-applicability of the activated functionality to NW.

Noted

R2-2409736	LCM for UE-sided model for BM	LG Electronics	discussion	Rel-19	NR_AIML_air-Core

Proposal 3. UE is allowed to report applicable functionality when active functionality becomes non-applicable. NW is responsible for deactivating activated functionality when it recognizes that it is non-applicable

Noted

Discussion

-	Nokia agrees with LG as it is not clear what the network would do if the UE would just stop.  There would be sufficient time for the network to react.   Ericsson agrees

-	Samsung thinks that UAI shouldn’t impact activation/deactivation status.

-	Huawei thikns that we should follow the CSI reporting mechanism, no autonomous behavior.

-	Lenovo thinks that it is wasteful for the UE to continue and report bad results.

-	ZTE Thinks it is not acceptable for the UE to stop the model as the network doesn’t know how to schedule the UE.

-	Qualcomm would prefer the autonomous but if it is not then the network should deactivate.

-	Apple thinks that there are cases where it won’t work at all (i.e. the UE is still downloading the model).   Qualcomm indicates that if it doesn’t have a model it shouldn’t have reported as applicable at all.

-	Vivo asks if this is applicable to monitoring.

-	Intergitital thinks that it is important that the UE shall send the report.  Nokia agrees and the UE should it ahead of time.    Ericsson agrees and it is important for the network to get it.  Qualcomm doesn’t want to make it a shall, the UE will do it anyways as it impacts performance.

When a functionality becomes non-applicable the UE doesn’t autonomously deactivate. NW is expected to deactivate active functionality when it receives report from UE that it is non-applicable.

R2-2410276	LCM for UE sided model in beam management	Lenovo	discussion	Rel-19

Proposal 7: When receiving an AI functionality activation command from gNB, if the concerned AI functionality is currently non-applicable, UE will not activate the AI functionality.

-	Huawei thinks this is a very rare change.

Implicit vs. explicity signaling of non-applicability

R2-2409943	Remaining issues on LCM procedure of UE-sided model for AI/ML based beam management	Apple	discussion	Rel-19	NR_AIML_air-Core

Proposal 10: No need to introduce explicit non-applicable functionality reporting, i.e. rely on NW to identify the functionality which becomes non-applicable by comparing UE’s latest reporting with its last reporting.

Noted

R2-2410550	LCM for UE-sided model for Beam Management use case	InterDigital	discussion	Rel-19	NR_AIML_air-Core

Proposal 4: UE can explicitly report which AI/ML functionality(ies) are “non-applicable”.

Proposal 6: UE indicates the reason an AI/ML functionality is non-applicable.

Noted

R2-2410604	Considerations on LCM for UE-sided model in Beam Management use case 	Kyocera	discussion

Proposal 5: RAN2 should agree that if non-applicable reporting includes supplementary information, explicit reporting can be used; otherwise, implicit reporting can be used.

Noted

Discussions on implicit vs. explicit

-	Interdigital thinks that we can achieve a unified framework with explicit.

-	Kyocera thinks that this should be stage 3 discussion and explicit.

-	Xiaomi supports interdigital proposal and it should report the report the reason so the network knows.

-	Vivo doesn’t thinks it is necessary as the network becomes aware that the network has changes.

-	CATT, LG share the apple view, implicit and only report applicable.   The network can activate the applicable functionality.

-	Oppo thinks what is important is how the network will use the information and this will depend on RAN1 LS.

-	Ericsson thinks it is important to report explicitly non-applicable functionalities and it should know the reason.  ZTE would like to know how the network will use this information.

-	Nokia thinks that we should report both and we can revisit if later we have a problem with size.

-	Samsung thinks that whats important is to know what is applicable and non-applicable, how it is signalled is stage 3.  For inference we don’t see the need for the reason but for training we can discuss.

-	Huawei thinks that number of applicable of functionalities is not very large so it is not a big issue with overhead.  Reason don’t see a motivation yet.

-	CMCC doesn’t see a need to report explicit non-applicable functionalities.

-	Interdigital thinks that it is very important that the network knows which are non-applicable so it can deactivate.

-	Qualcomm thinks we can discuss explicit, but we don’t need to know the reason, as they promise they will deactivate it.   Ericsson thinks it can be used for training.

-	Docomo supports explicit indication.   The reason is valid for network, if it knows the reason it can send a new configuration.

-	Lenovo thinks that this depends also on RAN1, if it is parameters then the meaning will be a bit different.

-	Ericsson thinks explicit is very useful as this provides a clear indication on the functionalities that changed.  Interdigital thinks that upon change of functionality we can explicitly indicate which functionality change and the new state.

-	ZTE thinks that the explicit indication is only useful if we indicate the reason.

FFS whether the UE reports explicitly “non-applicable” functionality when there is a change of applicability.

Content of applicability reporting message

R2-2410142	Discussion on LCM for UE-sided model for Beam Management	Spreadtrum, UNISOC	discussion	Rel-19

Proposal 1: RAN2 waits for RAN1 conclusion on applicability reporting content.

Functionality Management during mobility

Suppport for LCM during mobility

R2-2410592	LCM for UE-side Beam Management	Nokia Corporation	discussion	Rel-19	NR_AIML_air-Core

Proposal 12: RAN2 to discuss whether applicable configuration reporting to a target cell is supported during handover.

Noted

Applicability reporting and inference configuration during mobility

R2-2410578	LCM for UE-side models for beam management	Ericsson	discussion

Proposal 6: Applicable functionality reporting at handover is supported with the same RRC procedure that will be specified within a cell, as a baseline, i.e. the NW-side additional conditions and/or the inference configuration related to the target gNB are transmitted by the target gNB as part of the HO command, and the UE in response transmits the applicability report (either in RRCReconfigurationComplete or in UAI) to the target gNB after completing the handover.

Noted

R2-2410626	On LCM for UE-sided model for Beam Management	ZTE Corporation	discussion	Rel-19	NR_AIML_air-Core

Proposal 8: RAN2 to consider following options regarding the applicability reporting and inference configuration during mobility:

	Option 1: Target cell requests applicability reporting during or after handover.

	Option 2: Source cell transfers source cell’s applicability information to target cell to assist inference configuration in target cell.

	Option 3: Source cell requests applicability reporting for target cell with target cell’s network-side additional conditions.

Proposal 9: If option 3 is adopted, RAN2 to decide whether UAI message can be used to convey applicability reporting for neighbor cell.

Discussion

-	LG thinks that we can consider the option 1 as baseline, and there is no motivation for option 3 and no advantage compared to Ericsson’s proposal.

-	Xiaomi also thinks that option 1 should be baseline, but option 2 is quite straight forward, and option 3 don’t support.

-	Apple thinks that option 2 is already supported as we transfer the UAI to target cell and it would be useful for target cell to understand.  Oppo also agrees that option 1 is the baseline.

-	Qualcomm would like to ensure that none of this impacts the HO message and we don’t need to report this in RRCReconfiguration complete.

-	Ericsson thinks that option 2 is not necessarily supported already.   And there is no motivation to do option 2 or 3.

-	Samsung explains that with the existing framework we can support mobility without needed anything new.

-	Lenovo thinks there is motivation to have option 2.

-	Mediatek thinks that for option 2 to work we need very strong assumptions and option 3 would incur huge overhead to exchange information

-	Huawei thinks that with internode messaging we can do option 3 without RAN3.

-	Ericsson explains that as long as UAI of source cell without new information is forwarded to target cell it is ok.   Qualcomm thinks that certain information like associated ID are cell specific in addition to some other parameters and this brings complexity.    Oppo thinks that the ID is maintained by OAM so the target cell can identify the associated ID.  Qualcomm thinks that it is not possible to generalize the associated ID.     ZTE thinks that this can be guaranteed by network implementation.

-	Mediatek that further we need to consider also whether CSI configuration and inference configuration would be common.

-	Huawei thinks that UAI to target cell comes for free and whether the network can understand the configuration and UAI information it’s up to the network.

Applicable functionality reporting at handover is supported with the same RRC procedure that will be specified within a cell, as a baseline, i.e. the NW-side additional conditions and/or the inference configuration related to the target gNB are transmitted by the target gNB as part of the HO command, and the UE in response transmits the applicability report (either in RRCReconfigurationComplete or in UAI) to the target gNB after completing the handover.

Source cell UAI (as is) can be sent from source cell to target cell using existing signaling.   No further optimizations will be considered in RAN2 related to UAI.

Data Collection and Training

Details on data collection configuration

R2-2409716	Discussion on LCM for UE-sided model for BM use case	CATT	discussion	Rel-19	NR_AIML_air-Core

Proposal 9: For BM use case for UE-side model, data collection related configuration(s) (e.g., measurement resources configuration) and associated ID can be included in training data collection configuration.

-	Nokia wants to avoid having a limitation that a measurement is only associated with one ID.  It can be associated to multiple ID.   Apple thinks that it can provide multiple associated ID.

-	Nokia doesn’t understand why the associated ID Is needed.  Oppo explains that the UE needs to know for applicability purposes.

Noted

For BM use case for UE-side model, data collection related configuration(s) (e.g., measurement resources configuration) and associated ID(s) can be included in training data collection configuration.

Data collection configuration and initiation

R2-2409870	Further Discussion on LCM for UE-side Model	MediaTek Inc.	discussion

Proposal 3: RAN2 consider the following two alternatives for data collection configuration and initiation for UE-side model training:

•	Alternative 1-UE request over the air interface: The UE sends a request for data collection configuration and initiation. The network then provides the data collection configuration and initiates the data collection procedure based on the UE's request.

•	Alternative 2- Server request over network interfaces: The server for data collection for UE-side model training server sends the request and coordinates with the network on the data collection policy. The network then provides the data collection configuration and initiates the data collection procedure towards the UE based on the server's coordination.

Noted

R2-2409546	LCM for UE-sided model  for Beam Management use case	OPPO	discussion	Rel-19	NR_AIML_air-Core

Proposal 7: Do not consider data collection initiation based on UE request in R19, i.e. it’s up to NW to decide when to initiate data collection for training purpose in R19.

Noted

Discussion

-	LG doesn’t understand why alternative 2 is needed.    Samsung thinks it is reasonable to look at both options, but if we want to minimize alt. 1 is preferable and Alt. 2 would depend SA2 discussion.

-	Lenovo thinks that alternative 1 is preferable as we are discussing UE sided model.

-	Ericsson agrees with Mediatek, Samsung, Lenovo.

-	Vivo thinks alternative 1 baseline but we shouldn’t exclude alternative 2.

-	Qualcomm thinks that alternative 1 needs to be adapted as the OTT server is not aware of UE conditions like power etc.

-	Tmobile thinks that this is dependent on architecture.  Samsung and qualcomm explain that this is a UE.

Noted

For data collection configuration UE-side model training, the UE can send a request for data collection.   FFS what the request contains.    The network can provide the data collection configuration.

Network control of data collection

R2-2410626	On LCM for UE-sided model for Beam Management	ZTE Corporation	discussion	Rel-19	NR_AIML_air-Core

Proposal 7: RAN 2 to consider following methods for network control of the initiation and configuration for data collection:

	The network can decide when to start/stop the data collection.

	The network can configure whether UE is allowed to initiate request for data collection.

	The network can decide whether to accept UE’s request for data collection.

-	Samsung would like to ensure that there should be a way that the UE tell the network that it can’t do it even if it receives the configuration.   Qualcomm thinks that the UE will ultimately chose if it can do the data collection based on available resources and power.   There is no need for additional confirmation.    Nokia thinks this is not acceptable that the UE ignores.  Verizon thinks that it would be useful to get an indication from the UE when it can’t collect data based on received configuration.

The following methods for network control of the initiation and configuration for data collection:

    - The network can decide when to start/stop the data collection and send configuration.

    - The network can configure whether UE is allowed to initiate request for data collection.

    - The network can decide whether to accept UE’s request for data collection.

Agreements

1.	When a functionality configured by the network to be reported via UAI, becomes from non-applicable to applicable, the UE can reports it to the network.   FFS detailed design

2.	When a functionality becomes non-applicable the UE doesn’t autonomously deactivate. NW is expected to deactivate active functionality when it receives report from UE that it is non-applicable.

3.	FFS whether the UE reports explicitly “non-applicable” functionality when there is a change of applicability.   Verify this aligns with RAN1 configuration design

4.	Applicable functionality reporting at handover is supported with the same RRC procedure that will be specified within a cell, as a baseline, i.e. the NW-side additional conditions and/or the inference configuration related to the target gNB are transmitted by the target gNB as part of the HO command, and the UE in response transmits the applicability report (either in RRCReconfigurationComplete or in UAI) to the target gNB after completing the handover.

5.	Source cell UAI (as is) can be sent from source cell to target cell using existing signaling.   No further optimizations will be considered in RAN2 related to UAI.

6.	For BM use case for UE-side model, data collection related configuration(s) (e.g., measurement resources configuration) and associated ID(s) can be included in training data collection configuration.

7.	For data collection configuration UE-side model training, the UE can send a request for data collection.   FFS what the request contains.

8.	The network can provide the data collection configuration (at any point in time), with or without UE request.

9.	The following methods for network control of the initiation and configuration for data collection:

- The network can decide when to start/stop the data collection and send configuration.

- The network can configure whether UE is allowed to initiate request for data collection.

10.	FFS whether an indication from UE to network is needed when UE can’t perform data collection based on received configuration

Performance Monitoring

R2-2410492	Discussion on LCM for UE-sided model for Beam Management use case	Huawei, HiSilicon	discussion	Rel-19	NR_AIML_air-Core

Proposal 11: RAN2 waits for RAN1 detail progress related to performance monitoring for the UE-side AI/ML model for BM-Case1 and BM-Case2.

R2-2409834	Discussion on LCM for UE-sided Model for Beam Management Use Case	Fujitsu	discussion	Rel-19	NR_AIML_air-Core

Proposal 1 RAN2 starts at least the following discussions while waiting for further RAN1 input.

	Mechanism to trigger the performance monitoring procedure.

	Potential signaling to complete the performance monitoring procedure.

Not treated

R2-2409704	Discussion on LCM for UE-sided model for Beam Management	vivo	discussion	NR_AIML_air-Core

R2-2409727	Discussion on LCM for UE sided model	NEC Corporation	discussion	NR_AIML_air-Core

R2-2409831	Discussion on LCM for UE-sided model for beam management	Samsung	discussion	Rel-19	NR_AIML_air-Core

R2-2409908	On LCM for UE-sided Models for Beam Management	Qualcomm Incorporated 	discussion	Rel-19

R2-2410101	LCM for UE-sided model for BM	China Telecom	discussion	Rel-19	NR_AIML_air-Core

R2-2410342	Discussion on LCM for UE-sided model for BM	CMCC	discussion	Rel-19	NR_AIML_air-Core

R2-2410450	On LCM for UE-sided model for Beam Management Use Case	SHARP Corporation	discussion

R2-2410554	Discussion on UE-sided model for Beam Management	Jio	discussion	Rel-19	NR_AIML_air-Core	Late

R2-2410581	LCM for UE-side model (beam management)	Futurewei Technologies	discussion	Rel-19

R2-2410751	Discussion on LCM for UE-sided model for beam management use case	TCL	discussion

#### 8.1.2.3	LCM for Positioning use case

Contributions should focus on LCM for UE-sided model, but can discuss NW-sided model and should focus on 1st priority positioning use cases.  Aspects related to data collection should be covered in 8.1.3

LCM procedure for AI/ML positioning

Support for Proactive and Reactive applicability reporting

R2-2409944	Further discussion on LCM procedure of AI/ML based positioning	Apple	discussion	Rel-19	NR_AIML_air-Core

Proposal 4: On applicable functionality reporting in step 4, support both reactive reporting (i.e. solicited information transfer as response to LPP RequestCapabilities) and proactive reporting (i.e. unsolicited information transfer).

Noted

R2-2409824	Discussion on LCM for POS use case	Samsung	discussion	Rel-19	NR_AIML_air-Core

Proposal. 5: For POS Case 1, RAN2 confirm that the existing unsolicited UE capability report mechanism in LPP can support UE to report the applicable functionality in both “proactive” and “reactive” manner without further enhancement.

- Proactive case: When the applicability change due to UE’s internal condition, UE can send an unsolicited LPP ProvideCapabilities message to LMF .

- Reactive case: If the applicability changes based on the configuration in LPP ProvideAssistanceData message in step 3, UE can send an unsolicited LPP ProvideCapabilities message to LMF upon reception of ProvidAssistanceData message.

Noted

Discussion

-	Qualcomm and Vivo is not sure why we need the proposal from apple as this is already the case.

-	Qualcomm thinks Samsung’s proposal is ok but we shouldn’t cause dependencies.

-	Nokia as what is configuration provideassistancedata

Agreements 1 For POS Case 1, RAN2 confirm that the existing unsolicited UE capability report mechanism in LPP can support UE to report the applicable functionality in both “proactive” and “reactive” as a baseline. - Proactive case: When the applicability change, UE can send an unsolicited LPP ProvideCapabilities message to LMF . - Reactive case: If the applicability changes based on the configuration in LPP ProvideAssistanceData message in step 3, UE can send an unsolicited LPP ProvideCapabilities message to LMF. Configuration details are FFS 2 As a baseline, If the AIML based positioning method becomes non-applicable when LMF requests UE location estimation, UE cannot perform the AIML based positioning, and reply with LPP Providelocationinformation message with error cause. FFS if other fallback options are considered

NW control of unsolicited LPP provide capability message (for applicability reporting)

R2-2410475	LCM for positioning use case	Qualcomm Incorporated	discussion

Proposal 3:	There is no need to explicitly control the UE sending unsolicited LPP provide capabilities (e.g., at Step 4 in Figure 3). This NW control is implicit in a LPP positioning session.

Noted

R2-2410553	LCM for positioning use case	Nokia	discussion	Rel-19	NR_AIML_air-Core

Proposal 2: RAN2 to agree LMF shall be able to control whether UE can send unsolicited LPP Provide Capabilities for reporting of applicable functionalities. A flag in Step 1 LPP Request Capabilities controls whether UE is allowed to send unsolicited LPP Provide Capabilities for reporting of applicable functionalities. Signalling details FFS.

Noted

Discussion

-	Apple, Xiaomi, Huawei, CATT supports Qualcomm and this is similar to legacy positions.

-	Ericsson thinks we need to understand the reason for this applicability reporting.  The LMF needs to understand so it can decided whether it uses LMF positioning.  IF the UE reports it is just overhead.   We also don’t know what the applicability reporting is and the signalling overhead.    CeWit agrees with Nokia and Ericsson and there should be some control for applicability report.

-	Interdigital thinks that we can put the control on proactive reporting but no control when it can use unsolicited reporting if it is reactive reporting.   Nokia clarifies that the intention was to only control it for applicable reporting not for all UEs.

-	Vivo doesn’t think explicit is needed.

-	Qualcomm asks what is the use case to not allow the UE to send.  Nokia thinks that the network can chose to only to the reactive case and it is an implementation.   Interdigital agrees with Qualcomm that doesn’t make sense to disable the unsolicited.

-	Ericsson thinks that the network could have decided to not go ahead with AI ML.   But once it has decided to do AI ML then it should configure it.

-	Apple thinks we should follow legacy way.

Details of inference measurement reporting

R2-2409717	Consideration on LCM for Positioning use case	CATT	discussion	Rel-19	NR_AIML_air-Core

Proposal 5: For PoS case 1, RAN2 to discuss whether an explicit indicator or a new field of predicted UE location is needed to assist the LMF to identify the predicted/measured result.

-	Nokia, Ericsson, thinks there is no need to differentiate.

-	Samsung supports the proposal.  In legacy it can indicate which positioning method is used and we should have something similar.

-	Vivo thinks we should postpone the discussion.

Noted

R2-2410277	LCM for AIML based positioning with UE-sided model	Lenovo	discussion	Rel-19

Proposal 6	If the AIML based positioning method becomes non-applicable when LMF requests UE location estimation, RAN2 discusses the following two options:

a.	Option 1: UE cannot perform the AIML based positioning, and reply an LPP Error message to LMF indicating the AIML functionality is currently not applicable, or

b.	Option 2: UE can be configured with a default non-AIML based positioning method, and UE will fallback to the default non-AIML based positioning and still provide the estimated UE location.

-	Samsung thinks that LPP Error message should be LPP Providelocationinformation with error cause.

-	Qualcomm thinks that this is discussing fallback and this is what we do today and we can do the same here.   Option 1 is the baseline.  Nokia agrees and it depends on AI positioning methods.

-	Ericsson thinks that both Option 1 and 2 can co-exist.  Interdigital agrees.

-	Huawei asks if option 2 the UE is allowed to fall back.

-	ZTE supports both option and it should be under network control.

As a baseline, If the AIML based positioning method becomes non-applicable when LMF requests UE location estimation, UE cannot perform the AIML based positioning, and reply with LPP Providelocationinformation message with error cause.  FFS if other fallback options are considered

Noted

[POST128][026][AIML] LCM Procedure for Positioning Case1 (Ericsson)

Intended outcome:

•		Discuss the FFS from the existing RAN2 Agreements

- Attempt to resolve the FFS which does not need RAN1 input

- Identify questions to ask to RAN1 for resolving FFS waiting on RAN1 progress

•		Collect general guidelines on the criteria for deciding whether to enhance legacy method(s) or introduce new method

Deadline: long

Not treated

Functionality management

Functionality (De)activation

R2-2409705	Discussion on RAN2 issues of AI/ML enhanced positioning	vivo	discussion	NR_AIML_air-Core

Proposal 5. Functionality management (e.g., activation/deactivation) is realized implicitly by whether LMF requesting UE for location information with the positioning method related to the AI functionality, i.e., UE considers the functionality is activated when the corresponding method is requested for UE to provide location estimate.

Who (de)activates positioning functionality?

R2-2409570	Discussion on LCM for positioning use case	ZTE Corporation	discussion	Rel-19	NR_AIML_air-Core

Proposal 1: For AI/ML positioning use case 1 and 2a, RAN2 to consider the following approaches for functionality-LCM:

	LMF activates/deactivates the functionality

	LMF allows UE to activate/deactivate the functionality

Data Collection and Training

Consistency between training and inference

R2-2409835	Discussion on LCM for Positioning Use Case	Fujitsu	discussion	Rel-19	NR_AIML_air-Core

Proposal 3 For the NW-side models of positioning use cases, no extra specification effort needed for NW-side additional condition used at least for consistency between model training and inference.

Ground truth

R2-2410673	Discussion on LCM for positioning	CMCC	discussion	Rel-19	NR_AIML_air-Core

Proposal 2: At least ground truth label and its related data (e.g. time stamp) should be provided by LMF via signaling ProvideAssistanceData.

Request for DL PRS for training data collection

R2-2410277	LCM for AIML based positioning with UE-sided model	Lenovo	discussion	Rel-19

Proposal 1	The legacy LPP Assistance Data Transfer procedure (via RequestAssistanceData/ ProvideAssistanceData messages) is taken as the baseline for UE to request DL PRS configuration from LMF for training data collection.

Proposal 2	The legacy LPP Assistance Data Delivery procedure (via ProvideAssistanceData messages) is taken as the baseline for LMF to proactively provide DL PRS configuration to UE for training data collection.

Request for assistance data for model training (second priority)

R2-2409570	Discussion on LCM for positioning use case	ZTE Corporation	discussion	Rel-19	NR_AIML_air-Core

Proposal 9: UE can request some specific assistance data that will have impact on model training, e.g.:

	UE requests the number of TRPs that the UE wants to receive DL-PRS from;

	UE requests the area of TRPs that the UE wants to receive DL-PRS from;

	UE requests the number of PRUs that the UE wants to receive the corresponding measurement and labels from.

Topics with possible RAN1 impact

New vs. existing AI/ML positioning method

R2-2409705	Discussion on RAN2 issues of AI/ML enhanced positioning	vivo	discussion	NR_AIML_air-Core

Proposal 1: Introduce AI/ML based positioning as a new positioning method for a clear signaling structure and forward compatibility.

R2-2409717	Consideration on LCM for Positioning use case	CATT	discussion	Rel-19	NR_AIML_air-Core

Proposal 2: Consider AI/ML positioning of UE-side model as enhancements based on the existing positioning methods, e.g. DL-TDOA, DL-AoD, Multi-RTT.

R2-2410499	LCM For Positioning	Ericsson	discussion	Rel-19	NR_AIML_air-Core

Proposal 2: Wait for RAN1 to provide the Assistance data parameter list before deciding on new method or existing method.

Use of associated ID in AI/ML positioning

R2-2410475	LCM for positioning use case	Qualcomm Incorporated	discussion

Proposal 6: RAN2 assumes that any "Network-side Additional Conditions" for the positioning use case are the LPP Assistance Data elements, possibly with some enhancements, if needed. An "Associated ID" to identify "Network-side Additional Conditions" is not needed.

Granularity of AI/ML functionality

R2-2410658	Discussion on LCM for Positioning use case	Huawei, HiSilicon	discussion	Rel-19	NR_AIML_air-Core

Proposal 1: For the UE-sided model for positioning, it is proposed that RAN2 discuss the following two options for the supported functionality category:

(1) sub-use cases, e.g. Case 1

(2) a combination of sub-use cases and the input/output-related information

Details related to performance monitoring

R2-2410475	LCM for positioning use case	Qualcomm Incorporated	discussion

Proposal 10:	For UE-sided models (Case 1/2a) the model performance monitoring (and associated model performance monitoring metric calculation in label-based model monitoring) is performed at the UE and/or "server for data collection for UE-side model training".

R2-2409835	Discussion on LCM for Positioning Use Case	Fujitsu	discussion	Rel-19	NR_AIML_air-Core

Proposal 8 For positioning case 1, even if UE can calculate the monitoring metrics and make preliminary decisions of the monitoring results, final confirmation from LMF is preferred.

R2-2409791	LCM for Positioning use case	NEC	discussion

Proposal-4: For model performance monitoring for Position use case for UE sided model, wait for RAN1 and RAN3 progress.

R2-2410553	LCM for positioning use case	Nokia	discussion	Rel-19	NR_AIML_air-Core

Proposal 6: Send LS to RAN1 to ask RAN1 to:

a) Prioritize the selection of monitoring metric calculation entity or entities before discussing various LPP signalling options for support of metric calculation,

b) Clarify requirements for performance monitoring for UE and LMF,

c) Clarify whether the new measurements like CIR, PDP and DP are applicable for any of the existing legacy positioning methods and whether any of the existing legacy measurements not reported as intermediate output of model are applicable for an AI/ML model-based positioning method.

Not treated

R2-2409907	Association of measurements and ground truth labels for positioning use-cases	Fraunhofer IIS, Fraunhofer HHI	discussion

R2-2410502	LCM for Positioning use case	Interdigital Inc.	discussion	Rel-19	NR_AIML_air-Core

R2-2410637	Discussion on the LCM for AI positioning	Xiaomi	discussion

R2-2410778	Discussion on Functionality-based LCM for Positioning Use Case	CEWiT	discussion	Rel-19	NR_AIML_air-Core

### 8.1.3	NW side data collection

Contributions should focus on the mechanisms and principles identified for data collection for network side model training during rel-18.   Contributions should discusss type of data required to be collected for NW sided model and UE sided model (common to NW sided and different).  Question to RAN1 should also be identified.

Logging conditions/events:

R2-2410538	Discussion on NW-sided data collection for training	Huawei, HiSilicon	discussion			Rel-19	NR_AIML_air-Core

Proposal 6: Event-triggered data logging based on radio condition is NOT supported

Noted

R2-2409945	Remaining issues on NW-sided data collection	Apple	discussion	Rel-19	NR_AIML_air-Core

Proposal 8: Support the following two radio condition based event triggered logging:

•	Event X1: when L3 serving cell measurement becomes better than absolute threshold (similar to A1).

•	Event X2: when L3 serving cell measurement becomes worse than absolute threshold (similar to A2).

Noted

R2-2409547	Data Collection for Network Side BM Model Training	OPPO	discussion	Rel-19	NR_AIML_air-Core

Proposal 5: During the period that radio condition-based logging event fulfills, UE performs data logging periodically.

Proposal 6: For periodic logging or event-triggered periodic logging, data logging interval is configured by the network, the value range of data logging interval is pending on RAN1 inputs.

Noted

R2-2409909	On Network Side Data Collection 	Qualcomm Incorporated 	discussion	Rel-19

Proposal 4: RAN2 is requested to consider at least the following events for training data collection for network-side model training,

- Event 1.	Based on the number of samples to be collected and reported across different beams, UE triggers the measurement collection and logging if a beam becomes the top-1 beam and the logged number of measurements is less than the configured value.

- Event 2.	Based on the change of the top-1 beam: UE is configured to log the measurement when the top-1 beam changes. UE can additionally be configured with the number of samples to be logged and its periodicity

Noted

Discussion

Options

- Event-triggered data logging based on radio condition is NOT supported

- L3 serving cell measurement based (e.g. X1/X2 similar to A1/A2)

- Beam based events (e.g. beam becomes top-1 beam and number of measurements is less than configured value)

- L1 beam level measurement

-	Huawei explains that there is not good motivation or any good event that makes sense for AI/ML, the models have to generalize well for the whole cell.

-	Apple indicates that L3 measurement allows the UE to log event based on cell edge/cell center.   Huawei indicates that this is for network based, so do we expect the network to know whether the UE is in cell edge or cell center.   Ericsson thinks that it doesn’t imply that the network has to have different models, it is just to allow the network to get more measurements in cell edge or cell center to allow for better generalization.   We anyways have the periodic ones for general measurements.

-	ZTE thinks that both L3 and L1 beam measurements can be useful.

-	Nokia doesn’t think that L3 measurements will serve the purpose as we are collecting L1 measurements.   The reason for these event based is to have a complete data set.    Apple thinks that the network can do the post-prossesing.

-	Nokia asks how the UE knows and how it triggers it.  Apple thinks the beam proposal is too complex.

-	Xiaomi thinks that this avoids the UE collecting too much data if the network is only interested in a subset of data.

-	Qualcomm thinks 3 and 4 are the same.  Qualcomm is concerned that there may not be a good correlation between l1 and l3 event.

-	LG thinks that the UE is in connected state so it is aware of the UE measurements.

-	CATT asks if the UE is only going to do logging in certain area.  Ericsson explains that this co-exists with periodic logging so the UE can log in more than the specific area.

-	ZTE explains that this discussion is about who does the filtering of the data.  It is better for the UE to do it so we don’t waste air interface resources.

-	CMCC thought that this would save UE power and the operator should be able to chose whether we configure the event or not.

-	Huawei thinks the network can filter so not sure how much we save vs. the complexity

-	Ericsson is also suggesting to based on RLF timers.  Oppo is not sure how RLF is linked to beam management.  Lenovo thinks that we consider the cell edge scenario (e.g. based on L3 measurements) already covers RLF.   LG agrees that option 1 covers RLF.

Focus on the following three radio condition event based logging

- L3 serving cell measurement based (e.g. X1/X2 similar to A1/A2)

- Beam based events (e.g. beam becomes top-1 beam and number of measurements is less than configured value)

- L1 beam level measurement

[POST128][019][AI PHY] NW side data collection (Nokia)

Intended outcome: Discuss the motivation and specification complexity for the three radio conditions.

Deadline:  Long

Starting/Stopping of logging:

R2-2410343	Discussion on NW side data collection	CMCC	discussion	Rel-19	NR_AIML_air-Core

Proposal 6: The UE starts the data collection immediately upon reception of the measurement configuration.

Discussion on the dynamic activation/deactivation is not supported.

-	LG thinks that this depends on the type of CSI reporting and can only be restricted to periodic CSI measurement.   ZTE thinks that we agreed that RRC configuration has the enablement flag.    Apple doesn’t think it makes sense to have dynamic as it is not time critical.

-	Ericsson for periodic we shouldn’t have dynamic, but for semi-persistent there is already for a mechanism to activate the measurements.   Huawei thinks that for training we don’t need aperiodic.

-	Qualcomm and Lenovo think that dynamic is necessary.

-	LG thinks that RAN1 has considered all types of CS-RS for training.

Measurements on aperiodic CSI resources are not reported for NW sided data collection.

Noted

R2-2409547	Data Collection for Network Side BM Model Training	OPPO	discussion	Rel-19	NR_AIML_air-Core

Proposal 7: Data logging duration parameter is introduced in data collection configuration; UE performs data logging during the configured logging duration.

Proposal 8: If data logging duration is introduced, UE shall stop data logging when one of the following conditions fulfills:

- after the expiry of the logging duration timer; or

- logging duration timer is still running but data logging memory is full.

Proposal 9: Power state issue is not considered for training data collection procedure in R19

-	Qualcomm asks what is meant by this, that the UE doesn’t stop when there is no power or that it doesn’t report to the network.  Oppo thinks that this refers to triggers and reporting.

Noted

R2-2409836	Discussion on NW side Data Collection	Fujitsu	discussion	Rel-19	NR_AIML_air-Core

Proposal 8 UE can stop data logging automatically if low power state is detected.

Noted

R2-2410627	Further Considerations on NW side data collection	ZTE Corporation	discussion	Rel-19	NR_AIML_air-Core

Proposal 7: UE resumes the data logging if the event (e.g. buffer limitation, power state, etc) that causes the suspending of data logging is no longer existed.

Noted

Discussion on duration

-	Xiaomi, Qualcomm, LG, ZTE, etc thinks that duration is not needed

Duration is not supported

Discussion on power state

-	Xiaomi thinks this is necessary and important.  Nokia thinks that it is difficult to define threshold and we shouldn’t allow the UE to take any actions, it should all be network configuration.

-	LG doesn’t think this is an AI ML issues.

-	ZTE also thinks that it is difficult to determine UE power state.  The network should configure a threshold (e.g. 20%/40%).   Apple explains that different UEs have different powers.

-	Apple thinks that there is a high impact to UE in terms of power, so it should be prioritized, and the UE should suspend the buffer.

-	Ericsson thinks that the procedures don’t last for long time so it is unlikely that there are impacts.

-	CATT thinks it is valid, but we shouldn’t configure anything, but the network should be made aware of the UE stopping.

-	Huawei understands UEs concerns and it’s ok to stop collection when in power state, but the UE should follow the configuration.  The UE should at least report the low power state and the network should de-configure.   Samsung also supports the proposal from Huawei and it should be up to the network to release the configuration.   Lenovo thinks that the UE will stop when the memory is full but it should at least tell the network.   We don’t need to specify the reason.   Huawei agrees that memory full it is important for the network to know.   Maybe the UE should tell the network a bit earlier before the buffer is full.

-	AT&T thinks that there can other reasons why the UE stops.   Qualcomm agrees.  Apple thinks the UE should stop is out of coverage

-	Ericsson thinks another reason is that the events are not fulfilled.

-	Mediatek is concerned that if the UE battery is drained it should stop.   Qualcomm agrees.  Oppo thinks that the UE can report low power before it runs out of power.

Agreements on NW side data collection - Focus on the following three radio condition event based logging - L3 serving cell measurement based (e.g. X1/X2 similar to A1/A2) - Beam based events (e.g. beam becomes top-1 beam and number of measurements is less than configured value) - L1 beam level measurement - Measurements on aperiodic CSI resources are not reported for NW sided data collection. - Data collection is controlled by the network. The UE will not autonomously stop when low power state is detected. - The UE reports to the network when the power state is low. We will not specify how the UE determines low power state. The network should de-configure the data collection (this can be captured in stage 2). - The UE reports to the network when buffer is or may become full. FFS when it reports (before and/or after). - The UE can report the reason for triggering of indication for the status (e.g. low power state, low memory). FFS how this is signalled and if the reporting can be part of availability indication.

Not treated

Availability indication (signaling):

R2-2409706	Discussion on NW side data collection	vivo	discussion	NR_AIML_air-Core

Proposal 4.	The availability indication of logged data can be sent to network via the following messages:

-	UEAssistanceInformation, if configured to provide availability indication of logged data

-	RRCReconfigurationComplete

-	UEInformationResponse

R2-2409737	NW side data collection	LG Electronics	discussion	Rel-19	NR_AIML_air-Core

Proposal 10. Available indication is transmitted via UAI. No need additional reason for triggering of availability indication

Availability indication (triggering):

R2-2410343	Discussion on NW side data collection	CMCC	discussion	Rel-19	NR_AIML_air-Core

Proposal 2: Availability indication can be triggered when the AS buffer is full or the power state of the UE is low.

R2-2409945	Remaining issues on NW-sided data collection	Apple	discussion	Rel-19	NR_AIML_air-Core

Proposal 11: Support both NW triggered availability reporting and UE initiated availability reporting:

1)	NW triggered: reporting as response to NW request via various RRC complete messages.

2)	UE initiated: it is up to UE implementation when to report via UAI message.

R2-2409830	Disuccsion on NW side data collection	Samsung	discussion	Rel-19	NR_AIML_air-Core

Proposal 3. UE indicates the availability indication via UAI, when the size of logged data exceeds a threshold which is configured by NW.

Availability indication (content):

R2-2409706	Discussion on NW side data collection	vivo	discussion	NR_AIML_air-Core

Proposal 3: A single bit indication would be sufficient for UE to indicate to NW that UE has available logged data. No additional information is needed.

R2-2410278	Data Collection for NW-sided model training	Lenovo	discussion	Rel-19

Proposal 8: Upon the trigger of the data report, UE will send an availability indication together with data size information for gNB to fetch the logged data with enough radio resource reserved.

R2-2409865	Discussion on NW side data collection	Xiaomi	discussion

Proposal 11: UE indicates the data power state low or reaching buffer limit in the data availability indication.

Sending of large data:

R2-2410503	NW side data collection	Interdigital Inc.	discussion	Rel-19	NR_AIML_air-Core

Proposal 8: If the UE cannot fit all the logged data into one UEInformationResponse message, it will include an indication that more logged data is available in the UEInformationResponse message. The name/type of the IE/field is FFS.

R2-2410846	NW-side data collection for beam management and positioning	Ericsson	discussion

Proposal 14: In case all the collected data do not fit into a single RRC message, RAN2 to discuss the following options:

- In each transmitted RRC message, the UE indicates to the gNB that there are still collected data available for transmission, and then the gNB further requests the UE to transmit the next RRC message (via UEInformationRequest/Response) (similar to logged MDT).

- The gNB indicates (i.e. in the UEInformationRequest) the amount of collected data the UE should transmit in the following RRC UEInformationResponse messages, or the amount of RRC UEInformationResponse messages the UE should transmit, without any further request from the gNB.

- The gNB indicates (i.e. in the UEInformationRequest) that all the remaining collected data should be transmitted by the UE in the following RRC UEInformationResponse messages without any further request from the gNB.

Proposal 15: The UE indicates in the availability indication how many data or RRC samples are available for transmission.

Data collection configuration:

R2-2409881	Further Discussion on Data collection for Network Side Model Training	MediaTek Inc.	discussion

Proposal 1: In RRC_CONNECTED state (incl. handover), the UE maintains only one logged measurement configuration for data collection for network-side model training. When the network provides a new logged measurement configuration, it completely replaces any previously configured logged measurement configuration.

R2-2410538	Discussion on NW-sided data collection for training	Huawei, HiSilicon	discussion	Rel-19	NR_AIML_air-Core

Proposal 3: The NW can configure one data collection configuration per use case, i.e. multiple configurations for the same use case are not supported.

Proposal 4: Dynamic activation/deactivation of the data collection configuration is not supported (i.e. it is sufficient to rely on RRC signaling).

R2-2409909	On Network Side Data Collection 	Qualcomm Incorporated 	discussion	Rel-19

Proposal 1: The training data collection configuration consists of

-	Measurement targets (i.e., CSI resources or resource sets for measurements), and

-	Logging configuration (e.g., periodicity, events for logging measurements).

Proposal 2: The CSI framework is used for configuring measurement targets (i.e., CSI resource or resource sets) for training data collections.

Proposal 5: The UE can be configured with multiple CSI resources or resource sets for training data collection.

Proposal 6: Dynamic activation/deactivation of training data collection and logging should be used to randomize the training data collection and logging, following semi-persistent and aperiodic CSI reporting as the reference.

R2-2410627	Further Considerations on NW side data collection	ZTE Corporation	discussion	Rel-19	NR_AIML_air-Core

Proposal 1: As same as immediate MDT, The OAM/gNB Centric data collection for beam management is based on the L3 measurement framework (e.g. MeasObjectNR and reportConfigNR).

Proposal 2: For NW side data collection for beam management, the RS resources for data collection are configured in a MeasObjectNR, and the reporting related configuration for data collection are configured in the reportConfigNR associated with a same MeasId that is linked to the MeasobjectNR.

Proposal 3: For NW side data collection for beam management, at most one measurement configuration can be configured to the UE for each serving cell. Multiple configurations can be provided to the UE for multiple serving cells.

Proposal 4: For NW side data collection for beam management, the dynamic activation/deactivation of logging data/measurement is not supported in Rel-19.

Handling during HO and state transition:

R2-2410846   NW-side data collection for beam management and positioning               Ericsson             discussion

Proposal 18         For Rel.19, as baseline, RAN2 assumes that the UE discards/clears the AIML collected data at mobility events (e.g. handover to a different gNB than the source gNB, RLF/HOF, RRC state transitions).

R2-2409737   NW side data collection             LG Electronics  discussion         Rel-19               NR_AIML_air-Core

Proposal 12. UE retains logged data during HO

Proposal 13. UE indicates that logged data is available during HO (i.e., in RRCReconfigurationComplete message)

Proposal 14.  UE retains logged data when transitioning to RRC idle/inactive

Proposal 15. UE indicates that logged data is available and when transitioning to RRC connected state (i.e., in RRCSetupComplete /RRCResumeComplete message)

R2-2409548	Data Collection for Positioning Model Training	OPPO	discussion	Rel-19	NR_AIML_air-Core

R2-2409653	Consideration on NW side data collection	CATT	discussion	Rel-19	NR_AIML_air-Core

R2-2409905	AIML PHY NW-side data collection 	NEC	discussion	Rel-19	NR_AIML_air-Core

R2-2410069	Discussion on NW-side Data Collection	SHARP Corporation	discussion

R2-2410102	Discussion on NW-side data collection for AI/ML based beam management	China Telecom	discussion	Rel-19	NR_AIML_air-Core

R2-2410143	Discussion on NW-side data collection	Spreadtrum, UNISOC	discussion	Rel-19

R2-2410425	Data Collection for Training of NW-side ML Models	Nokia	discussion	Rel-19	NR_AIML_air-Core

R2-2410489	Some aspects for NW side data collection	Sony	discussion	Rel-19	NR_AIML_air-Core

R2-2410582	NW-side data collection	Futurewei Technologies	discussion	Rel-19

### 8.1.4	UE side data collection

Including outcome of [POST127bis][020][AI PHY] Reply LS to SA2/SA5 (InterDigital/Nokia)

No other contributions are expected for this AI.  Waiting for response from SA WGs.  Type of data required to be collected for UE sided model can be discussed in contributions in 8.1.3

R2-2410504	Summary of [POST127bis][020][AI PHY] Reply LS to SA2_SA5 (InterDigital_Nokia)	Interdigital Inc., Nokia	discussion	Rel-19	NR_AIML_air-Core	Late

Proposal 1: Respond to SA2’s Q1 as below:

- There is no conclusion/agreement in RAN2 regarding NG-RAN’s involvement in enabling UE data collection controllability.

-	Tmobile, Mediatek think that there is some NG-RAN to be done for 1b, 2, and 3, but the extent of involvement is FFS.  1a is not in the scope of the discussion.  Ericsson thinks not matter if we do UP or CP we will have NG-RAN – so for user plane it would involve UP establishment and CP is control plane establishment.

-	Qualcomm thinks that we can’t just say NG-RAN involvement but we have to be more concrete with what the involvement is.

-	LG asks whether the involvement is for data transfer or data collection.   Oppo thinks that both data collection and transfer should be in scope, so we can for sure explain that configuration for data collection will always have NG-RAN.  And we should differentiate the UL and DL for the involvement.  Interdigital explains that from the LS the question is referring to both transfer and data collection.  They don’t need to know all the details.   Nokia has similar understanding.

-	Samsung, and Apple thinks that the question is only for controllability of data transfer so would like to keep the original agreement.

-	Samsung thinks that we are talking about controllability.   Nokia explains that for CP there is not solution that would involve NG-RAN.   But UP we are not sure as we don’t know how it would work.    Huawei thinks that for NAS approach this would be transparent for NG-RAN.

-	Qualcomm we should add that RAN2 hasn’t agreed whether it is NG-RAN is the network entity that enforces controllability.    Interdigital thinks that they are not asking whether NG-RAN is the main controlling entity.   Qualcomm thinks it would be good to indicate this to them as they need this information for their design.

-	Vivo thinks that only for option 3 there is NG-RAN

-	China Mobile think that NG-RAN is always involved but whether it is transparent or not is not concluded.

-	Samsung would like to explain the different steps between data collection and transfer and they are not linked.  Qualcomm thinks that there are use cases where we don’t need a configuration from the network.   Lenovo thinks we should explain the difference between the two, UE measurements are part of data collections.

Confirm the answers for SA2 are related to 1b, 2, and 3.

For data collection, we can copy our agreements that there is NG-RAN involvement for configuration between UE and NG-RAN

For data transfer, RAN hasn’t studied/concluded NG-RAN involvement extensively as those discussions depend on SA2 potential solutions

Explain what is meant by data collection and data transfer. Try to explain that data transfer can happen at any point in time as long as there are collected data available in the UE or explain that configuration may not always be needed.

Respond to SA2’s Q2 as below:

- SA2 can assume that for data collection the gNB is involved in providing radio measurement configuration (if needed) for beam management use case and LMF is involved in providing radio PRS measurement configuration (if needed). However, RAN2 has not agreed that the gNB/LMF is in charge of “initiating, terminating and fully managing data transfer”.

Respond to SA2’s Q3 as below:

- RAN2 has not evaluated/analyzed the impact on UE’s normal operation due to the full controllability of the data collection process.

Respond to SA2’s Q4 as below:

- Standardized data refers to data whose format/content is explicitly defined in 3GPP specifications, allowing the network to understand its content and meaning.

Respond to SA2’s Q5 as below:

- Roaming considerations are outside the scope of RAN2.

Respond to SA2’s Q6 as below:

- As stated in the LS sent from RAN, visibility of data content signifies that the MNO will be able to be aware of, access, and comprehend the content of the collected/reported data without the need of SLA. Thus, full visibility allows the MNO to verify/match the data specified/configured to be collected and the data that is being reported.

Respond to SA5’s question “Is the “Server for data collection for UE-side model training” controlled by operators?” as below:

- The controllability requirement is referring to the controlling of the data collection/transfer process. Whether the server for UE side model training is controlled by the MNO or not is outside the scope of RAN2.

Respond to SA5’s question “What standardized data is to be collected?” as below:

- No final agreement has been made in RAN WGs regarding the standardized data to be collected. Some examples can be found in R1-2310681.

R2-2410505	[Draft] Reply LS to SA2 on AIML data collection,	Interdigital	LS out	Rel-19	NR_AIML_air-Core	To:SA2	Cc:RAN3, SA3, SA5	Late

[AT128][014][AIML] LS response to SA2 (Interdigital)

Intended outcome: Approve LS response

Deadline:  Tuesday

R2-2411116	[Draft] Reply LS to SA2 on AIML data collection	Interdigital	LS out	Rel-19	NR_AIML_air-Core 	To:SA2	Cc: RAN3, SA3, SA5

RAN2 wants to clarify the following for the discussion of the UE side data collection for UE side model training:

-        Data collection refers to the UE collecting data that involves the UE performing measurements (e.g., radio measurements).

-        Data transfer refers to the sending/transfer of the collected data from the UE to Server for data collection for UE-side model training/OTT server (as capture in Table 7.2.1.3.2-1) in TR38.843.

SA2 can assume that the NG-RAN is involved in providing radio measurement configuration (if needed) for the UE side data collection at least for the beam management use case, as captured in the following agreement in RAN2-127bis:

Data collection initiation and configuration for data collection is under network control.  FFS how the NW determines whether data collection should be initiated (e.g., via UE requests (UE directly or UE server))

Data transfer can happen at any point in time under network control if there is collected data available in the UE. Furthermore, for data transfer, RAN2 hasn’t studied/concluded on level of NG-RAN involvement.

RAN2 will continue the discussion on data collection configuration and RAN2 will appreciate SA2/SA5 input on the data transfer process.

These agreements will be captured in the final draft LS R2-2411151 and uploaded in email discussion. Final LS will be approved in R2-2411152

R2-2411151	[Draft] Reply LS to SA2 on AIML data collection	Interdigital	LS out	Rel-19	NR_AIML_air-Core	To:SA2	Cc: RAN3, SA3, SA5

SA2 can assume that the gNB is involved in providing radio measurement configuration (if needed) for beam management use case and LMF is involved in providing radio PRS measurement configuration (if needed). However, RAN2 has not agreed that the gNB/LMF is in charge of “initiating, terminating and fully managing data transfer”.

The LS is approved in R2-2411152 with the change above “removal of radio”

R2-2410506	[Draft] Reply LS to SA5 on AIML data collection,	Nokia	LS out	Rel-19	NR_AIML_air-Core	To:SA5	Cc:RAN3, SA2, SA3	Late

-	Tmobile and AT&T would like to say that the entity is controlled by MNO.   Nokia thinks that this is more of an SA2 decision.  Interdigital explains that the answer is only saying it is out of RAN2 scope, but we are not saying that it is not under control.    Ericsson thought that it was under MNO control that’s why we came up with this new terminology, it was to differentiate from OTT.   Qualcomm thinks that this is something that SA2 can decide.

-	Mediatek agrees with Ericsson.   Oppo explains that in our RAN2 table the server is within the MNO.   If it is not in the MNO than why is not controlled by MNO.

Update question 1: The controllability requirement is referring to the controlling of the data collection/transfer process without an SLA. RAN2 has assumed when discussing the solutions captured in the TR that the server for data collection for UE-side model training is within the MNO.  RAN2 assumes that if it is in the MNO it will also be controlled by the MNO.

The LS is approved in R2-2411114 with the change above

### 8.1.5	Model transfer/delivery

Only contributions originating from operators on requirements for 1-sided and 2-sided models are expected for RAN2#128.  Non-operator companies are not expected to submit contributions (but are encouraged to collaborate with operators).

(reference: RAN1 identified collaboration levels from the TR)

| y | model delivery (if needed) over-the-top. | Outside 3GPP Network | UE-side / NW-side / neutral site |
| --- | --- | --- | --- |
| z1 | model transfer in proprietary format. | 3GPP Network | UE-side / neutral site |
| z2 | model transfer in proprietary format. | 3GPP Network | NW-side |
| z3 | model transfer in open format. | 3GPP Network | UE-side / neutral site |
| z4 | model transfer in open format of a known model structure at UE, i.e., an exact model structure as has been previously identified between NW and UE and for which the UE has explicitly indicated its support. | 3GPP Network | NW-side |

Not treated

R2-2410344	Discussion on AIML model transfer delivery	CMCC, NTT Docomo, China Unicom, CATT, Apple	discussion	Rel-19	NR_AIML_air-Core

Discussion on one-sided model

Proposal 1: There is no requirements on controllability or visibility for UE-sided model transfer/delivery case y, since the model trained in OTT server can be sent to the UE directly from OTT-server (transparent to 3GPP).

Observation 2: RAN2 focuses on functionality-based LCM during R19 discussion, the network doesn’t be aware of which model is used at UE side.

Proposal 3: It is proposed to de-prioritize case z1 for one-sided model transfer/delivery, since there is no strong requirement on visibility and controllability for UE-sided model transfer/delivery from the perspective of operator.

Discussion on two-sided model

Observation 3: RAN1 focuses on case z4 for two-sided model transfer/delivery, where NW transfers the parameters to the UE for the supported known model structure.

Proposal 4: For two-sided model, controllability and visibility are required for model transfer/delivery case z4, considering the following aspects:

  - The CSI generation part at UE side and the CSI reconstruction part at NW side should be aligned for good model performance

  - The whole model is trained at NW and the CSI generation part is sent to UE

  - The model is open format and the structure is known at UE side, NW transfers the parameters of the model to UE

R2-2410601	Requirements for Model Transfer/Delivery	T-Mobile USA Inc., Nokia	discussion	Rel-19	NR_AIML_air-Core

Proposal 1: Adopt the following list of requirements for model transfer/delivery.

- Model transfer/delivery traffic should be differentiated from other user traffic.

- Model transfer/delivery traffic should be transferred at a different priority than user traffic.

- There should be a guarantee that models are transferred securely, in a NW-aware manner, such that untrusted models cannot be downloaded.

- Model transfer/delivery is initiated by the UE.

- Models need to be addressable such that the UE can request the transfer/delivery of a specific one.

- The NW is in control of if and when to transfer / deliver a model to the UE.

Proposal 2: Adopt the following assumptions for discussion about model transfer/delivery.

- UE’s can only store a very limited set of trained models.

- Trained models could be per gNB, per UE hardware/software, per morphology.

- For beam management trained models are applicable to a small number of sites, e.g., cell, gNB, or area, etc.

- gNB can evaluate current conditions to determine whether to transfer a model, e.g., if a UE is at the cell edge, based on cell load, etc.

R2-2410103	Discussion on model transfer/delivery	China Telecom	discussion	Rel-19	NR_AIML_air-Core

Proposal 1: RAN2 discussion on model transfer/delivery shall focus on a two-sided model with a known model structure.

Proposal 2: RAN2 should wait for further input from RAN1 on operation of model transfer/delivery for case y and z1.

Proposal 3: RAN2 should wait for further progress from RAN1 on model transfer/delivery for one-sided model.

R2-2410853	Operators views on the AI_ML model delivery options	BT plc, Turkcell, Deutsche Telekom	discussion	Rel-19	R2-2410529

Proposal 3: MNO has visibility of the model standardized data to keep the control over the network for one-sided and two-sided models

Proposal 4: Only the MNO has control over the model delivery/transfer and control over the AI/ML model activation/deactivation

Proposal 5: Data used to train models needs to be visible to MNO

Proposal 8: RAN2 to postpone the UP and/or CP donwselection to transfer/deliver the AI/ML model until it is decided the entity(ies) to transfer the model

## 8.2	Ambient IoT

(FS_Ambient_IoT_solutions,leading WG: RAN1; REL-19; SID: RP-240826)

Time budget: 2.5 TU

Tdoc Limitation: 4 tdocs

### 8.2.1	Organizational

LS, Rapporteur input, including workplan, etc.

R2-2409511	Reply LS to RAN2 on data block sizes for Ambient IoT (R1-2409250; contact: MediaTek)	RAN1	LS in	Rel-19	FS_AmbientIoT, FS_Ambient_IoT_solutions	To:RAN2	Cc:SA2	Withdrawn

R2-2409526	LS on security aspects of Ambient IoT (S2-2411049; contact: OPPO)	SA2	LS in	Rel-19	FS_AmbientIoT	To:SA3	Cc:RAN2

Noted

R2-2411078	Reply LS on security aspects of Ambient IoT (S3-245139; contact: OPPO)	SA3	LS in	Rel-19	FS_AmbientIoT	To:SA2	Cc:RAN2

Noted

R2-2409812	TP for TR 38.769 update	Huawei, CMCC, T-Mobile USA	pCR	Rel-19	38.769	1.0.0	FS_Ambient_IoT_solutions

Endorsed and will be further updated with meeting agreements

[POST128][015][AIoT] TP for TR  (Huawei)

Intended outcome: agreeable TP to send to RAN1 and accompanying LS

Deadline:  Dec. 1st

R2-2411221	TP for TR 38.769 update	Huawei, CMCC, T-Mobile USA	pCR	Rel-19	38.769	1.1.0	FS_Ambient_IoT_solutions

=> Endorsed in R2-2411221 (TP)

R2-2411263	LS on RAN2 outcome of Ambient IoT study	RAN2	LS out	Rel-19	FS_AmbientIoT	To:RAN1	Cc:RAN3, SA2, SA3

=> Approved in R2-2411263

Discussions on completions of SI from RAN2 perspective

-	Ericsson thinks that there are aspects that have high dependencies on other WGs and we should take those into account and possibly revisit assumptions. We should at least capture dependencies as guidance in the RAN plenary.

-	Nokia also thinks that we should list the open issues and dependencies that will impact future design.

-	Vodafone thinks that obviously we have some open issues and dependencies, but it doesn’t make sense to list them as we don’t know all the aspects that will change our assumptions. So from RAN2 perspective we can decide that it is finished.

-	CMCC points out that SA2 have already concluded on their architecture so we have candidate solutions for normative work. SA3 has also concluded on NAS encryption and that doesn’t impact RAN2, so no need to list open issues. We can proceed with completion from RAN2 perspective.

-	Spreadtrum thinks we should consider all the decisions of WGs.

-	ZTE agrees that from RAN2 perspective we have addresses all issues and there are no open issues.  Of course there is dependencies but this is the case for all Wis.  Interdigital agrees with ZTE.  The TR will include all the topics and what will need to be worked further in WI.

-	Huawei indicates that all agreements touch the dependencies and they just impact decision on which option and not feasibility. We can consider the study item complete.  Vivo, Apple, CATT, Samsung, Xiaomi, Docomo and Williot agrees.

-	Ericsson thinks that we should do something like RAN1 in the conclusion. Samsung thinks that it is totally different as from RAN2 point of view we addressed all issues.

-	Huawei – first part we list what we have studied and we can recommend basic functions, paging, RACH, and data transmissions.  Ericsson thinks that’s fine but we have impact from other WGs.  Huawei reminds everyone that we have had many TRs and we have never done this.

-	Vodafone wants to avoid that add new things we haven’t discussed

- Agreements

From RAN2 point of view, the SI is considered complete

The TR will capture in corresponding subsection any impact and/or dependencies from other WGs when applicable.

The TR will recommend that RAN2 will have to support paging, RACH, data transmission functionality in MAC. RAN2 has discussed topology 1 and topology 2 and has concluded it is feasible, from RAN2 perspective. RAN2 will discuss in WI phase any additional impacts from other WGs.

### 8.2.2	Functionality aspects

Contributions should focus on the functionalities required for A-IoT devices, remaining aspects of AS ID for study phase,  segmentation, information visible to reader if any other, protocol stack, etc.?

Protocol Stack

R2-2409897	A-IoT functionalities	Huawei, HiSilicon	discussion	Rel-19

Proposal 1:	For A-IoT air interface protocol stack, there is no other L2 AS layer (i.e. A-IoT MAC layer only).

-	Ericsson thinks that there are some functionalities that may need another layer to not increase complexity in MAC.

Noted

RAN2 assumes that there is no other L2 AS layer (i.e. A-IoT MAC layer only).   There is no CP/UP protocol stack differentiation on AIoT interfaces.

AS ID – which entity assigns it

R2-2409783	A-IoT functionality	ZTE Corporation, Sanechips	discussion

Proposal 7: The random number included in MSG1 (RN16) is used as the AS ID for scheduling subsequent A-IoT messages .

Noted

R2-2410311	Remaining issues on study of AIoT functionalities	NTT DOCOMO, INC.	discussion	Rel-19

Proposal 2.	“AS ID” is generated by A-IoT reader and allocated to each A-IoT device via Msg2 in 3-step CBRA, success indication in 2-step CBRA and CFRA.

Noted

Discussion

-	Apple thinks that random ID shouldn’t be kept for a long time as it increases chances of collision for other users, and it should be decided by the reader.

-	Samsung agrees to ZTE’s proposal and use 16 bits.  It should be long enough to avoid collisions and we can capture two directions in the TR.

-	CMCC supports reader assigned ID and it is reader decision.

-	Huawei thinks we need to determine whether collision needs to be handle by the reader.   WE can capture a third option, where it is up to the reader whether it continues to use the random ID or assigns a new one.

-	Vivo thinks that this is for scheduling purposes.

-	Mediatek doesn’t see any complexity with doing this, the reader assigns a new ID and it drops the previous one and stores the new.

Capture the option that it is up to the reader whether to use the random ID as AS ID or assigns a new AS ID.   FFS what message is used

Capture in the TR that to down-select we need to consider which message need to consider whether we need the reader to handle the collision.

AS ID – when is it released by the device

R2-2409707	Discussion on functionality aspects for Ambient IoT	vivo	discussion	FS_Ambient_IoT_solutions

Proposal 1.	The AS ID can be maintained at device and reader side, and released on a timer expiry or explicit indication from reader.

Not treated

R2-2409703	Ambient IoT ID management and implications for energy status indication	MediaTek Inc.	discussion	Rel-19	FS_Ambient_IoT_solutions

Proposal 4: The stored AS ID at the device may be assumed valid for the availability time implied by the device’s energy status indication.

Not treated

Segmentation – Segment Indication

R2-2410193	Functionality aspects for A-IoT	Ericsson	discussion	Rel-19	FS_Ambient_IoT_solutions

Proposal 5	The device sends a 1-bit indication indicating whether the current transmission is the last segment or not.

Noted

R2-2410672	Discussion on functionality for A-IoT	CMCC	discussion	Rel-19	FS_Ambient_IoT_solutions

Proposal 6: Simple message size indicator can be considered for last segment indicator.

Noted

Segmentation indication will be discussed in WI phase.

Segmentation – Retransmission

R2-2410672	Discussion on functionality for A-IoT	CMCC	discussion	Rel-19	FS_Ambient_IoT_solutions

Proposal 2: Support optional explicit R2D failure/success feedback indication for D2R segments.

Noted

R2-2410423	Functionality aspects of AIoT	Nokia France	discussion	Rel-19

Proposal 6: D2R segmentation operates on wait-and-stop basis in D2R with mandatory acknowledgement feedback. FFS feedback details.

Noted

R2-2409963	Functional Aspects of Ambient IoT	Apple	discussion	Rel-19	FS_Ambient_IoT_solutions

Proposal 10 		Not support ACK/NACK-based segment retransmission.

Noted

Discussion

-	Samsung is not sure how the UE knows whether it was successful or not.    Huawei thinks that it can be based on message size.   Samsung thinks that it is a flavour of explicit indication.    Apple thinks that it can solved by reader just scheduling a resource for the next segment.

-	Qualcomm thinks that there may be feedback and it can be implicit or explicit.

-	ZTE thinks that we should have the same protocol as msg3 and msg5.

If segmentation is supported, it is beneficial for the reader to be able to trigger a re-transmission of a segment.  FFS how this is done.

- Agreements

- RAN2 assumes that there is no other L2 AS layer (i.e. A-IoT MAC layer only).   There is no CP/UP protocol stack differentiation on AIoT interfaces.

- Capture the option that it is up to the reader whether to use the random ID as AS ID or assigns a new AS ID.   FFS what message is used

- Capture in the TR that to down-select we need to consider which message need to consider whether we need the reader to handle the collision.

- Segmentation indication will be discussed in WI phase.

- If segmentation is supported, it is beneficial for the reader to be able to trigger a re-transmission of a segment.  FFS how this is done.

Visibility Discussion

R2-2409707	Discussion on functionality aspects for Ambient IoT	vivo	discussion	FS_Ambient_IoT_solutions

Proposal 8.	The reader executes the AS procedure in response to the service request from CN based on the necessary information derived from the received service request, wherein the necessary information may comprise:

—	command type information, e.g. read/write/disable.

—	periodicity to execute the service request.

—	QoS requirement (e.g. e2e latency and target successful inventory ratio).

-	Docomo thinks that RAN2 can only agree if it is beneficial not necessary.   The QoS shouldn’t be sent as the reader can’t guarantee QoS.

-	Xiaomi thinks that QoS would be useful as the reader can take into account for scheduling point of view.   LG thinks QoS needs to be discussed in SA2.

-	Qualcomm thinks they are all useful but the real question is whether they can be provided to the reader.

-	Interdigital thinks that the first two will depend on the LS and we should consider QoS.

-	ZTE explains that we need to decide what is absolutely necessary and what is beneficial and ask SA2.     Thinks that command type is necessary.    If the reader doesn’t know if this is inventory or command then the reader wouldn’t know how to behave subsequently.

-	Huawei thinks that we need to wait for message size LS.   The periodicity and QoS is really based on SA2 discussion.

-	CATT is not sure if command type is necessary.  CMCC thinks it is necessary for the reader to know for resource allocation purposes.

-	Apple doesn’t think anything is absolutely necessary but they may be beneficial.

-	Samsung sees some benefits on QoS for resource allocation from reader side and ask SA2.

-	Vodafone thinks that in general we should ask for command type.

-	Nokia would like to ask SA2 if there is any requirement on time from a radio access procedure.

-	Ericsson thinks that command type is highly important and we would schedule accordingly.

-	CATT thinks

Agreements on visibility

1.	The reader needs to know whether a response is expected and expected D2R message size. FFS how the reader gets this information. Wait for SA2 LS response on expected D2R message size to resolve the FFS. FFS if command type is explicit, if it can be inferred from expected D2R message size if available.

2.	Ask SA2 if they can provide information that RAN2 agreed on some information that are useful for the reader and ask whether they can be provided (e.g. service type). Ask SA2 if there are requirements from SA2 perspective (e.g. on latency of completion time of procedure)

Noted

[POST128][020][AIoT] LS to SA2 on information usefulness (ZTE)

Intended outcome:  LS to SA2

Deadline:  4 weeks

Other Use Cases/Scenarios – Lean Inventory

R2-2410416	Lean Inventory procedure/counting of devices 	VODAFONE	discussion	Rel-19

Proposal 2: It is proposed to call the procedure for counting (and proximity) reasons a lean inventory/counting procedure and it is proposed to support A-IOT lean inventory procedure within the A-IOT study item and include it into  TP 38.769.

Proposal 1: It is proposed to confirm that MSG2 is optional message which does not need to be sent by the reader/ received by the device all the time.

Proposal 3: It is proposed to introduce a discrimination (e.g. service type or cause values) within A-IoT paging to let the device know what kind of inventory procedure is expected to be performed.

-	Lenovo thinks that this can be done by reader implementation by not sending message 2, which was already agreed last minute.

-	Apple thinks this is a new use case and we should ask SA1 and SA2 and there are some security issue, as there may unauthenticated devices that respond.   Vodafone doesn’t thinks there is a security issue as there are no personalized IDs.

-	Samsung doesn’t want to call this lean or counting, it is just an inventory command.    And it is up to reader whether it sends msg 2 and we just need to understand whether the device needs to know.

-	Qualcomm thinks it would be good to agree to proposal 1 even though implicitly agreed last meeting.

-	Ericsson thinks that we haven’t discussed decide device behavior.   One solution is that the UE sends the response and assumes it is successful, the other one is you assume you were not successful and expect msg 2.

-	ZTE explains that we agreed that the device behavior is that it listens all the time.  So the device will anyways keep monitoring for paging so the device doesn’t need to know.    Nokia thinks that if the device knows it can not maintain the context.   MEdiatek thinks that if we tell the device don’t expect msg2 then the device will no longer respond to subsequent paging.   The device will be confused.

-	LG thinks that the device should always know whether there is a msg2 is received.  Samsung thinks that the gain of knowing is very small.

Noted

- Agreements on new lean/counting scenario

We will not capture a new use case in the TR.   RAN2 confirms that this use case can be done via reader implementation, as per previous agreement that it is up to the reader whether MSG2 is transmitted.  FFS if the device needs to be aware whether msg2 will be transmitted.

Other Use Cases/Scenarios – Bistatic

R2-2410134	Considerations on functionality aspects for Ambient IoT	Lenovo	discussion	Rel-19	FS_Ambient_IoT_solutions

Proposal 6: Consider studying how to signal the expected D2R response and the frequency to an intermediate UE to support bistatic mode of operation for D2T2-A1 scenario.

-	Huawei thinks that RAN plenary agreed to not study

RAN2 did not study this scenario in the study item

Noted

Not treated

R2-2409621	Discussion on the Functionality Aspects for Ambient IoT	CATT	discussion	Rel-19	FS_Ambient_IoT_solutions

R2-2409702	Ambient IoT coverage, energy, TB size, and all sorts of other things	MediaTek Inc.	discussion	Rel-19	FS_Ambient_IoT_solutions

R2-2409837	Discussions on Functionality Aspect of Ambient IoT	Fujitsu	discussion	Rel-19	FS_Ambient_IoT_solutions

R2-2410002	Functionality for Ambient IOT	InterDigital	discussion	Rel-19	FS_Ambient_IoT_solutions

R2-2410016	Discussion on A-IOT functionality aspects	Xiaomi	discussion

R2-2410096	Discussion on functionality for Ambient IoT	China Telecom	discussion	Rel-19	FS_Ambient_IoT_solutions

R2-2410124	Discussion on AIoT functionalities	OPPO	discussion	Rel-19	FS_Ambient_IoT_solutions

R2-2410137	Discussion on the functionalities required for Ambient IoT	Spreadtrum, UNISOC	discussion	Rel-19

R2-2410300	Discussion on Ambient IoT segmentation functionality	Panasonic	discussion	Rel-19

R2-2410353	Ambient-IoT Functionality Aspects	NEC	discussion	Rel-19

R2-2410370	A-IoT functionalities	ETRI	discussion	Rel-19

R2-2410374	Considerations on functionality aspects for Ambient IoT	Sony	discussion	Rel-19	FS_Ambient_IoT_solutions

R2-2410477	Views on Functionality Aspects of Ambient IoT	Qualcomm Incorporated	discussion	FS_Ambient_IoT_solutions

R2-2410493	Discussion on remaining aspects of AS ID	Continental Automotive	discussion

R2-2410572	Discussions on Energy Status Indication for Ambient IoT	Futurewei	discussion	Rel-19	FS_Ambient_IoT_solutions

R2-2410573	Further discussions on AS ID for Ambient IoT devices	Futurewei	discussion	Rel-19	FS_Ambient_IoT_solutions

R2-2410594	Discussion on functionality aspects of ambient IoT	LG Electronics Inc.	discussion	Rel-19	FS_Ambient_IoT_solutions

R2-2410603	Use cases for one-bit energy status report in AIoT	SHARP Corporation	discussion

R2-2410620	Discussion on functionality for ambient IoT	Google Ireland Limited	discussion	FS_Ambient_IoT_solutions

R2-2410646	Remaining issues of functionalities for Ambient IoT 	Kyocera 	discussion	Rel-19

R2-2410679	Discussion on functionality aspects for A-IoT	HONOR	discussion	Rel-19	FS_Ambient_IoT_solutions

R2-2410753	Discussions on functionalities required for AIoT	Samsung	discussion	Rel-19	FS_Ambient_IoT_solutions

### 8.2.3	A-IoT Paging

Contributions should focus on paging aspects and content required for Ambient IoT for the different identified procedures (i.e. inventory, inventory + command, command only), including monitoring of DL message, device unavailability due to energy harvesting (based on RAN1 progress).

Multiple/Subsequent Paging from the same Reader

R2-2409892	Discussion on paging procedure for Ambient IoT	OPPO	discussion	Rel-19	FS_Ambient_IoT_solutions

Proposal 1: include an ID identifying the CN request in the A-IOT paging message to avoid A-IOT device making duplicated responding to A-IOT paging messages corresponding to the same CN request.

Noted

R2-2409622	Discussion on Paging for Ambient IoT	CATT	discussion	Rel-19	FS_Ambient_IoT_solutions

Proposal 1: Introduce a service ID in the paging message to assist device to distinguish the subsequent paging messages. The paging messages associated with the same service ID belong to the same service request from CN.

Proposal 2: The service ID in the paging messages over the A-IoT interface does not rely on the session ID/transaction ID between CN and reader (i.e., based on the candidate solutions of SA2). It can be independently generated by reader with a short format, e.g., 2 bits.

Noted

Discussion

-	Xiaomi thinks this is stage 3 and the agreements we made are enough for now.  Docomo agrees with the proposals on indicated ID.

-	LG thinks that maybe we can capture in the TR how many bits are needed to avoid duplication.   Interdigital agrees that we need an ID and the question is whether it is the reader or the CN that provides the ID.

An ID in paging message is beneficial to avoid duplicated response for same service request. FFS whether ID is generated by the reader or CN. FFS on the ID size.

Paging from different Readers

R2-2409708	Discussion on AIoT Paging	vivo	discussion	FS_Ambient_IoT_solutions

Proposal 2.	RAN2 to support the scenario that one A-IoT device can be required to respond to A-IoT paging messages associated with the same service from different readers.

-	CATT thinks that RAN3 has agreed that this is possible but it is up to reader and CN to resolve this issue by giving different service IDs, there is no RAN2 impact.   Lenovo thinks it should be supported

-	Xiaomi doesn’t think we should support this scenario.   SA2 and rAN3 didn’t reach a conclusion on this.  Oppo agrees that it is too early.

-	Interdigital explains that RAN3 is using it for localization of devices to understand that to which reader the device will respond to.

Proposal 3.	RAN2 to discuss the following ways to fulfil the need of responses to multiple readers:

-	Option 1: including reader ID in A-IoT paging message;

-	Option 2: including an indication in A-IoT paging message, to indicate that UE should respond to the current A-IoT paging message regardless of whether access successes;

-	Option 3: CN implementation to group the responses from different readers for the same service with no further enhancement in A-IoT paging message.

Capture the scenario that different readers may page the same device for same service request.  RAN2 will wait for further progress from WGs and discuss this in WI phase.

Noted

Service Type in the Paging Message

R2-2409898	A-IoT paging	Huawei, HiSilicon	discussion	Rel-19

Proposal 1:	There is no need to include service/command type as the explicit AS layer information in A-IoT paging message.

Noted

R2-2409838	Discussions on AIoT paging	Fujitsu	discussion	Rel-19	FS_Ambient_IoT_solutions

Proposal 3: Paging type, e.g., for inventory-only, command-only or inventory and command is included in the (subsequent) A-IoT paging message.

Noted

R2-2409687	Remaining issues of AIoT Paging	ZTE Corporation, Sanechips	discussion	Rel-19	FS_Ambient_IoT_solutions

Proposal 4a: In order to let device know that that it needs to wait for the subsequent command data after successfully completing Msg3 transmission (e.g., in command only or inventory + command cases), it’s suggested to introduce indication about “having subsequent data” in paging message.

Proposal 4b: In order to let device know that it may need to wait for more time for the subsequent command data (e.g., in inventory + command cases), another indication about “expected waiting time for subsequent data” can also be considered.

Noted

Discussion on service type/command type on paging

-	Apple, Vivo, Lenovo, Oppo, Docomo, Xiaomi, LG, Samsung doesn’t see a need to put the type in the paging as there is no need to tell the UE to wait.   CATT agrees with Apple and has a concerns as there may be subsequent data that is transparent data.

-	Vodafone that a code point in paging message would be good for future proofness.   For service type it is not useful, but there may be some reason in future.

-	CMCC think the benefit to include is for scheduling the CN can provide the information to reader but not need to provide for the device.

-	Oppo thinks that anyways the UE will monitor for a determined time.

-	Interdigital, Qualcomm thinks it is too early to exclude this. One example the device can use the information to store the AS ID.

-	Ericsson thinks that command type is ok, but service type information is not the only way to do it, so we should keep it open.

-	ZTE explains that this is useful for the device to know which ID it has to monitor.  Agree that the device listens to the ID but it may need to know whether it is paging or AS ID.   Qualcomm thinks that some will be encrypted.

-	Mediatek thinks the futureproofness has some merit but it is difficult to know what to futureproof for.   Thinks that ZTE has a point but we don’t know enough about paging.

-	Nokia agrees with ZTE and the UE should know whether more messages are coming.

Whether we include further information in paging message to indicate service type/command type or to indicate that further subsequent messages are coming can be discussed in WI phase once we understand more the paging design and AS ID.

Decision on Contention-Based vs Contention-Free

R2-2409964	Discussion on Ambient IoT Paging	Apple	discussion	Rel-19	FS_Ambient_IoT_solutions

Proposal 8	D2R resource indicated in Paging message is by default shared resource, unless explicitly indicated as contention-free resource.

Noted

R2-2410605	Discussion on DL messages for Ambient IoT UEs	Ericsson	discussion	Rel-19	FS_Ambient_IoT_solutions

Proposal 1	There is no need to indicate explicitly whether a device should respond to the paging message with contention free or contention-based RA.

Noted

R2-2410404	Paging procedures for Ambient IoT	Nokia	discussion	FS_Ambient_IoT_solutions	R2-2408698

Proposal 13: To simplify signaling, RAN2 to agrees that

-	an AIoT paging of a single-device means “Contention-free access”,

-	an AIoT paging of a device group or all devices means “Contention-based random access”.

-	An AIoT paging of multiple device IDs (if supported) means “Contention-free access”.

Noted

Discussion

-	Huawei indicates that this also depends on whether we decide to have a unified CFRA and CBRA.  MEdiatek is not sure what the device would do differently even if it receives this indication.   ZTE agrees and think we should agree that we don’t need to provide an indication and we should try to unify the scheduling.

-	Samsung thinks that we are not sure what we will include in the resource schedule so it is too early.

-	CATT agrees with apple’s proposal but we need to define what is meant by CFRA.

-	Oppo thinks implicit method is better.

-	Xiaomi supports ericsson proposals.

-	Docomo thinks that this is not needed for unified design but may be if not.

Wait for WI phase

Paging Message Contents for Contention-Free

R2-2409892	Discussion on paging procedure for Ambient IoT	OPPO	discussion	Rel-19	FS_Ambient_IoT_solutions

Proposal 7: to trigger A-IOT devices to find their correct access occasions in A-IOT CFRA procedure, three options of different kind of information can be discussed to be embedded in the A-IOT paging message:

1.	One-to-one mapping relationship between the A-IOT device ID and the access occasion index.

2.	Mask ID defining the group of targeted A-IOT devices and the ID for the initial access occasion and indication of the ascending or descending order of the IDs for the subsequent access occasions

3.	modulo operation related parameters such as the divisor and lists of targeted A-IOT devices.

Noted

Agreements on paging

- An ID in paging message is beneficial to avoid duplicated response for same service request. FFS whether ID is generated by the reader or CN. FFS on the ID size.

- Capture the scenario that different readers may page the same device for same service request.  We will discuss this in WI phase (if in scope of WI) and consider progress from all the WGs

- Whether we include further information in paging message to indicate service type/command type or to indicate that further subsequent messages are coming can be discussed in WI phase once we understand more the paging design and AS ID.

Paging Monitoring

R2-2410575	Further discussions on Ambient IoT paging	Futurewei	discussion	Rel-19	FS_Ambient_IoT_solutions

Proposal 4. The reader may further include information in the A-IoT paging message to indicate, or for unintended devices to estimate, how long the current paging round may be, e.g., the number of total access rounds to be scheduled in the current paging round or an estimated duration for the current paging round to be completed.

-	Nokia thinks it is easier to have a mechanism to release as it is very hard to predict duration.   Qualcomm thinks that this does help saving power for devices but how we do this would require further discussions.

-	LG thinks that RAN1 is discussing how to perform energy harvesting and there is no consensus.

-	Samsung indicates that RAN1 has agreed to include this in second prioritized issues.

Noted

Not treated

R2-2409578	Discussion on A-IOT paging procedure	Xiaomi	discussion	Rel-19	Withdrawn

R2-2409583	Discussion on A-IOT paging procedure	Xiaomi	discussion	Rel-19

R2-2409739	Discussion on ambient IoT paging	LG Electronics Inc.	discussion	FS_Ambient_IoT_solutions

R2-2409884	Discussion on Paging for A-IoT	Transsion Holdings	discussion	Rel-19

R2-2410003	Paging Related Aspects for Ambient IOT	InterDigital	discussion	Rel-19	FS_Ambient_IoT_solutions

R2-2410068	New and retransmitted A-IoT paging indication and device behaviors	Sharp	discussion

R2-2410125	Ambient-IoT Paging	NEC	discussion	Rel-19	FS_Ambient_IoT_solutions

R2-2410140	Discussion on paging procedure of AIoT	Spreadtrum, UNISOC	discussion	Rel-19

R2-2410178	Discussion on Ambient IoT paging message with multiple IDs	ASUSTeK	discussion	Rel-19	FS_Ambient_IoT_solutions

R2-2410264	Discussion on paging procedure for Ambient IoT	Lenovo	discussion	Rel-19

R2-2410295	Discussion on A-IoT paging	Panasonic	discussion	Rel-19

R2-2410312	Discussion on RA type indication in AIoT paging	NTT DOCOMO, INC.	discussion	Rel-19

R2-2410334	Discussion on A-IoT paging	CMCC	discussion	Rel-19	FS_Ambient_IoT_solutions

R2-2410375	Considerations on paging for Ambient IoT	Sony	discussion	Rel-19	FS_Ambient_IoT_solutions

R2-2410551	Ambient IoT Paging	Qualcomm Incorporated	discussion	Rel-19	FS_Ambient_IoT_solutions

R2-2410616	Ambient IoT device paging	TCL	discussion	Rel-19	Withdrawn

R2-2410645	Remaining issues of A-IoT paging for Ambient IoT 	Kyocera 	discussion	Rel-19

R2-2410680	Discussion on A-IoT Paging	HONOR	discussion	Rel-19	FS_Ambient_IoT_solutions

R2-2410693	Further discussions on Ambient IoT Paging	China Telecom	discussion

R2-2410754	Discussion on A-IoT paging	Samsung	discussion	Rel-19	FS_Ambient_IoT_solutions

R2-2410766	Discussion on paging procedure for ambient IoT	Google Ireland Limited	discussion	FS_Ambient_IoT_solutions

R2-2410779	Discussion on A-IOT paging procedure	CEWiT	discussion	Rel-19	FS_Ambient_IoT_solutions

### 8.2.4	A-IoT Random Access

Contributions should focus on possible design unification for RA types and/or need for down selection for RA types (2step, 3step, CFRA) in SI/WI phase, Msg3 (re)-transmission failure handling, failure/success feedback indication for following D2R data, re-access, and any additional aspects related to CFRA and CBRA procedures, etc.

Down-select 3-step CBRA or 2-step CBRA

R2-2410403	CBRA type down-selection	Vodafone, Huawei, CMCC, Telit Communications S.p.A., CATT, Nokia	discussion	Rel-19

Proposal 1:	RAN2 to down-select 3step CBRA only between 2step and 3step RACH

Noted

R2-2409738	Discussion on random access aspects for Ambient IoT	LG Electronics Inc.	discussion	FS_Ambient_IoT_solutions

Proposal 7. Only 2-step CBRA procedure should be introduced for A-IOT.

Noted

R2-2409965	Discussion on Random Access for Ambient IoT	Apple	discussion	Rel-19	FS_Ambient_IoT_solutions

Proposal 6	RAN2 agree that both 2-step RACH and 3-step RACH can be supported in a unified way.

Noted

Unified design for 3-step CBRA and 2-step CBRA

R2-2410351	A-IoT random access procedure	Huawei, HiSilicon	discussion	Rel-19	FS_Ambient_IoT_solutions

Proposal 7:	RAN2 to discuss the following down-selection:

WF1: Only support 3-step CBRA.

WF2: Agree on the unified CBRA solution, i.e., Msg1 optionally includes data.

A-IoT Msg1: random ID is mandatory, and whether to also include the optional upper layer data is controlled by reader;

A-IoT Msg2 echoes the random ID for contention resolution.

Noted

Discussion

-	Docomo supports Vodafone’s proposal as we have to careful about resources and there may be more than 1000 devices.  ZTE thinks that this is up to the reader, there are use cases for 2step and the cost of devices would be lower for device with 2-step.

-	ZTE thinks that we can unify and if it is not possible to unify.   Unification is very simple as per Huawei’s paper.

-	Lenovo, Vivo, and NEC agrees with unified approach.

-	CATT that the 2step and 3 step are different and it would introduce complexity.

-	CMCC that only if the number of devices is low 2step RA is better, and this is not time critical.   Vodafone thikns that amount of collisions is not that small.

-	Mediatek is not sure whether we can unify and it is not that easy to unify from device perspective.

-	Ericsson thinks it is better to capture both and leave it for future discussion.  Qualcomm agrees.

-	Oppo should support both.  Interdigital also thinks we should support both and unify.  There are cases where we have fixed number of devices and we would at least give benefit in terms of amount of energy wasted during the procedure.

-	LG thinks a device shouldn’t support both.  Xiaomi thinks we should downselect but can unify CBRA and CFRA.

-	ZTE, Sony, Samsung Qualcomm and Ericsson thinks we shouldn’t make a decision yet as we have use cases for both RA procedures and we can decide later.  This also depends on the WI scope and timeline.

Capture the unified solution in the TR and capture advantage and disadvantage of the solutions.  This requires that devices would support both forms.

FFS recommendation on whether we have 3step only or we have unified solution.

Unified design for CBRA and CFRA

R2-2409700	Device support of ambient IoT random access procedures	MediaTek Inc.	discussion	Rel-19	FS_Ambient_IoT_solutions

Proposal 2: From the device perspective, CBRA and CFRA procedures should be unified by defining a single resource-selection algorithm.

-	LG asks what is single resource-selection?  Mediatek explains that the device just follows the same resource selection regardless.

Noted

R2-2409623	Discussion on the Random Access for Ambient IoT	CATT	discussion	Rel-19	FS_Ambient_IoT_solutions

Proposal 1: Unified procedure for CBRA and CFRA is not considered in A-IoT.

Noted

Discussion

-	CATT Thinks unifying would require the UE to send the random ID if we unify.    ZTE agrees with mediatek and it is very easy to unify.   That means random ID is always included regardless of whether it is CBRA and CFRA.

-	Interdigital agrees for signalling perspective but from a procedural perspective it is not possible.

-	Xiaomi thinks we should also unify the procedure

-	Apple doesn’t understand why the device uses random ID and we shouldn’t introduce something unnecessary CFRA.    CMCC, Vivo  agrees with Apple

-	Qualcomm thinks it doesn’t work, as the reader has no idea that the contention free resources are being used.   The UE should only use the ID provided by the reader and the UE has to echo the ID.  ZTE thinks that CFRA resources shouldn’t collide between readers.

Capture in TR as one option for unification for CBRA and CFRA.  For this option random ID is always included in msg1 is defined.  Capture disadvantage of reader doesn’t know whether the intended device is responding on the given resources.

MSG3 retransmission

R2-2409965	Discussion on Random Access for Ambient IoT	Apple	discussion	Rel-19	FS_Ambient_IoT_solutions

Proposal 1	For reader which fails to receive Msg 3, support A-IoT reader to re-transmit Msg 2, w/o explicit providing success/failure feedback indication.

Noted

R2-2410004	Random Access Procedure for Ambient IOT	InterDigital	discussion	Rel-19	FS_Ambient_IoT_solutions

Proposal 6a:	A device re-transmits MSG3 upon receiving an explicit R2D failure indication for MSG3 failure.

Noted

Discussion

-	LG, Xiaomi, Oppo, think the reader re-transmits msg2 if it fails to receive msg3.

-	Interdigital explains that we shouldn’t re-send the same msg2 if that msg2 contains multiple ID

-	LG asks if the consequence is that the UE is expected to keep monitoring for msg2 and everytime it receives it msg2 it should send message 3.   Samsung confirms and RAN1 has defined a timer.  Vodafone asks if this would require storing random ID and why don’t we just repeat the paging.

-	Nokia thinks that we should maybe capture both options including explicit feedback.

-	Mediatek wonders how long will the device need to remember the ID.  Samsung thinks that this can be discussed in WI phase.

RAN2 assumes, A-IoT reader can send a Msg2 transmission to a specific device for the random ID, if it doesn’t receive msg3 from that ID

If msg2 contains multiple devices, re-transmitted Msg2 will only include the random IDs of failed devices (if supported).

Explicit feedback indication for D2R data

R2-2410351	A-IoT random access procedure	Huawei, HiSilicon	discussion	Rel-19	FS_Ambient_IoT_solutions

Proposal 3: The "R2D failure/success feedback indication" also applies to "following D2R data" after the random access procedure for re-access purpose.

Noted

R2-2409965	Discussion on Random Access for Ambient IoT	Apple	discussion	Rel-19	FS_Ambient_IoT_solutions

Proposal 3 Explicit failure/success feedback indication is not supported for follow-up D2R data.

Noted

Discussion

-	Vivo thinks that we should have a successful indication.

-	Mediatek, ZTE, Lenovo, Samsung, LG, CMCC, Vivo, Qualcomm, Ericsson agrees with Huawei and there is value so the device know.   ZTE thinks that only message 1 is contention based and everything else should be unified from msg3 onwards.

-	Huawei thinks that Apple’s assumption that the message will succeed after several tries is not always true.  The reader should be able to stop the transmission.   Apple thinks that we already agreed to not have RLC.  Huawei explains that this indication is for re-access purposes.

-	Samsung thinks that if we don’t have HARQ there is no way to know so support Huawei.

-	Xiaomi thinks that we need explicit for success but not for failure.

The "R2D failure/success feedback indication" also applies to "following D2R data" after the random access procedure for re-access purpose.

Implicit success consideration

R2-2410265	Discussion on random access for Ambient IoT	Lenovo	discussion	Rel-19

Proposal 11: Implicit success indication of Msg3/D2R message can be used in following case:

Case 3: Implicit ACK when Msg3/D2R message reception success and there is the subsequent R2D message target to the device.

Noted

Re-access mechanism

R2-2409582	Discussion on access procedure for ambient IOT	Xiaomi	discussion	Rel-19

Proposal 24: The following 3 options for re-access trigger are supported in SI phase and captured in TR. Down selection will be done in WI:

Option 1: The devices in the current round are triggered to re-access, i.e., re-access in same round.

Option 2: The devices in the previous round are triggered to re-access, i.e., re-access in different round.

Option 3: The devices are triggered to re-access based on paging retransmission.

Noted

R2-2409839	Discussions on AIoT Random Access	Fujitsu	discussion	Rel-19	FS_Ambient_IoT_solutions

Proposal 4: Support re-access in the same access round (Option 1). Dedicate access occasions for re-access in the end of the same access round may be used for re-access by the A-IoT devices which experienced access failure in the previous access occasions.

Noted

R2-2409623	Discussion on the Random Access for Ambient IoT	CATT	discussion	Rel-19	FS_Ambient_IoT_solutions

Proposal 7: It’s up to reader to optionally adjust the number of access occasions and provide the particular access occasions for re-access purpose by the A-IoT paging.

Noted

Discussion

-	Vivo thinks that option 2 and 3 should be considered.   Option 1 will lower the fairness to other devices.

-	Huawei thinks that the only discussion if we need option 2 as option 3 is already in the TR and assumed as baseline.  Lenovo thinks that option 1 and 2 should also be included.    Nokia is fine.

Capture the following option in TR: The devices can be triggered by R2D message to re-access before subsequent paging retransmissions.

Agreements

- For 2 step and 3step CBRA, capture the unified solution in the TR and capture advantage and disadvantage of the solutions.  This requires that devices would support both forms.

- FFS recommendation on whether we have 3step only or we have unified solution for 2step and 3step CBRA.

- Capture in TR as one option for unification for CBRA and CFRA.  For this option random ID is always included in msg1.  Capture disadvantage of reader doesn’t know whether the intended device is responding on the given resources.

- RAN2 assumes, A-IoT reader can send a Msg2 transmission to a specific device for the random ID, if it doesn’t receive msg3 from that ID

- If msg2 contains multiple devices, re-transmitted Msg2 will only include the random IDs of failed devices (if supported).

- The "R2D failure/success feedback indication" also applies to "following D2R data" after the random access procedure for re-access purpose

- Capture the following option in TR: The devices can be triggered by R2D message to re-access before subsequent paging retransmissions.

Not treated

R2-2409577	Discussion on access procedure for ambient IOT	Xiaomi	discussion	Rel-19	Withdrawn

R2-2409709	Random Access Procedure for AIoT Device	vivo	discussion	FS_Ambient_IoT_solutions

R2-2409784	Unified random-access procedure for A-IoT	ZTE Corporation, Sanechips	discussion

R2-2409792	Random Access for Ambient IoT device	NEC	discussion

R2-2409885	Discussion on Random Access for A-IoT	Transsion Holdings	discussion	Rel-19

R2-2409890	Discussion on D2R failure/success indication in A-IoT	SHARP Corporation	discussion	FS_Ambient_IoT_solutions

R2-2409895	random access for AIoT	OPPO	discussion	Rel-19	FS_Ambient_IoT_solutions

R2-2410097	Random access procedure for A-IoT	China Telecom	discussion	Rel-19	FS_Ambient_IoT_solutions

R2-2410138	Discussion on random access of Ambient IoT	Spreadtrum, UNISOC	discussion	Rel-19

R2-2410152	Discussion on UL multiple access	Ericsson	discussion	Rel-19	FS_Ambient_IoT_solutions

R2-2410261	Study the A-IoT Random Access Procedure	Tejas Network Limited	discussion

R2-2410313	Remaining issues on study of AIoT random access	NTT DOCOMO, INC.	discussion	Rel-19

R2-2410315	Further discussion on ambient IoT random access	CMCC	discussion	Rel-19	FS_Ambient_IoT_solutions

R2-2410376	Considerations on random access aspects for Ambient IoT	Sony	discussion	Rel-19	FS_Ambient_IoT_solutions

R2-2410414	Open issues on random access for AIoT	Nokia	discussion	FS_Ambient_IoT_solutions

R2-2410476	Views on Random Access Aspects of Ambient IoT	Qualcomm Incorporated	discussion	FS_Ambient_IoT_solutions

R2-2410576	Issues with previous agreement on Msg2 in 2-step CBRA	Futurewei	discussion	Rel-19	FS_Ambient_IoT_solutions

R2-2410593	Discussion on Ambient IoT random access	KT Corp.	discussion	Rel-19	FS_Ambient_IoT_solutions

R2-2410629	Discussion on random access for ambient IoT	Google Ireland Limited	discussion	FS_Ambient_IoT_solutions

R2-2410681	Discussion on A-IoT random access	HONOR	discussion	Rel-19	FS_Ambient_IoT_solutions

R2-2410719	Further discussion on Ambient IoT random access	Samsung	discussion	Rel-19	FS_Ambient_IoT_solutions

R2-2410780	Discussion on random access for Ambient IoT	CEWiT	discussion	Rel-19	FS_Ambient_IoT_solutions

R2-2410868	Handling of Msg2 for 2step CBRA	Philips International B.V.	discussion	Rel-19	FS_Ambient_IoT_solutions

R2-2410890	A-IoT Random Access - failure/success feedback indication for following D2R data	Wiliot LTD	discussion	Rel-19	FS_Ambient_IoT_solutions

### 8.2.5	Topology 2 considerations

Contributions should focus on study phase topology 2 related aspects between gNB and reader, including validity of resources and reader behavior, any impacts based on architecture discussions in SA2/RAN3, etc.

Cell(s) where resources are valid

R2-2409624	Discussion on Topology 2 for Ambient IoT	CATT	discussion	Rel-19	FS_Ambient_IoT_solutions

Proposal 3: Do not support resource validity across multiple cells in the first release and only support the configured A-IoT interface resource is valid within the serving cell which configures resource for the UE reader.

Noted

R2-2409742	Considerations on TP2 related aspects between BS and UE reader	Xiaomi	discussion	Rel-19

Proposal 4: If the resource validity can be extended to multiple cells, the intra-gNB cells can be considered.

Noted

Discussion

-	CMCC, Lenovo, ZTE agrees with CATT, it is an optimization and it not efficient way to utilizes resources.   Spreadtrum also support CATT.  ZTE thinks that this not just inefficient but it is also wasteful.

-	Qualcomm thinks that there are use cases with multiple readers across multiple cells.  Vodafone thinks that if ew had time it could make sense for a reader that is in cell edge and it has high probability of handover, but we don’t have time so let’s start simple.

Resource validity across multiple cells is not supported in the initial release.

Need for a validity timer

R2-2409710	Discussion on Topology 2 related aspects	vivo	discussion	FS_Ambient_IoT_solutions

Proposal 1.	The AIoT radio resource is applicable within a predefined or configured time period. The UE reader starts/restarts the validity timer upon reception of the (re-)configured AIoT radio resources.

Noted

R2-2409966	Remaining open issues in Topology 2	Apple	discussion	Rel-19	FS_Ambient_IoT_solutions

Proposal 1 	RAN2 to discuss whether to define a validity timer for A-IoT radio resources; the timer is started when RLF occurs, when the timer expires the UE reader releases the configured A-IoT radio resources.

Noted

R2-2410335	Discussions on topology 2 for A-IoT	CMCC	discussion	Rel-19	FS_Ambient_IoT_solutions

Proposal 10: Timer-based validity mechanism is not supported, otherwise it may cause interference to other intermediate UEs and it is difficult to deciding the length of the timer.

Noted

Discussion

-	Qualcomm agrees with configured timer in vivo’s paper and doesn’t agree with RLF only.    Ericsson thinks that this is only for out of coverage otherwise you rely on explicit signaling. For temporary out of coverage agree with CMCC

-	Interdigital supports for having a timer that can be used for temp out of coverage case.  Explicit signaling can be use to release the resource and RLF wouldn’t be a reasonable solution.

-	Oppo prefers timer based solution and doesn’t understand interference problem caused by RLF as the UE may not leave the coverage.

-	Xiaomi, Spreadtrum doesn’t want to support as we have to define when to stop and when to start.

-	MEdiatek agrees with vivo, and we have two ways, either the UE keeps requesting or having a timer the UE can use the resources during this time.  Timer is simpler mechanism and doesn’t understand complexity and interference for a relatively short timer.  Samsung thinks that whether the UE keeps requesting is not yet discussed.

-	CATT asks if the time is shared between the NW and the UE.   Vivo explains that the timer is between the UE and the NW so it is within the synchronized system.   Mediatek thinks that the device doesn’t need to know the timer.

-	ZTE thinks we are entering stage 3 details.   We configure some resource and a time the resource is available and whow the time is controlled can be stage 3.

-	Vodafone thinks thit is is a static UE reader.   Mediatek thinks that we can’t make that assumption.

-	Huawei agrees that somehow the network needs to control the time, and the one reason the timer may be needed is for temporary out of coverage.  And baseline should be the explicit network configuration

-	Samsung doesn’t think this is a critical issue and we should keep it simple.

-	Apple asks which state we are talking about.   ZTE thinks that it doesn’t matter, we just need to give a time frequency resource to use and it can be used in any state, it comes in dedicated signaling.

-	CMCC thinks that RLF are usually caused by late HO and anyways the UE can’t use those resources and this is why interference will happen.  The time varies for many cases including receiving response etc, so it is not easy to determine this time in advance.

-	LG for normal operation the network can control the frequency and time and during temporary out of failure we can use the resources for a short time.

-	Interdigital thinks that if we provide the UE with resources valid for a certain amount of time it handles all the cases like temporary out of coverage with one solution.

-	Ericsson thinks that we are discussing whether we should do something for temp out of coverage scenario, and we can do timer, resource configuration, or just suspend the resources.

-	Oppo thinks that we can make it an exceptional cases.

-	Qualcomm explains that we can go with configuration or resource and time as long as they are valid in any RRC state of the reader.

-	ETRI has similar view as CMCC.

-	Nokia thinks timer based is the simplest way to enable the reader to complete the procedure without interaction with gNB all the time.  Mediatek agrees that there seems to be no reason for the reader to go back and forth to the gNB for each message, and it is more efficient to use persistently.

-	CATT supports timer.

-	Huawei wonders if we have time to do all the scenarios and this is the last meeting to study.

-	Samsung thinks that resource validity is under network control and we don’t need to capture all options in the TR.

-	ZTE thinks we can just capture Timer is implicit or explicit in the configuration

Include in the TR the resource validity is under network control.  CB which option we include.

[AT128][016][AIoT] Resource validity (Interdigital)

Intended outcome: Which options to capture for validity, how to handle temp out of connection, and whether resources are valid across states.

Deadline:  10-17-24

R2-2411179	Conclusion of [AT128][016][AIoT] Resource validity	InterDigital	discussion	Rel-19	FS_Ambient_IoT_solutions

Noted

Agreements - Resource validity across multiple cells is not supported in the initial release. - Include in the TR the resource validity is under network control. CB which option we include. - We will capture the following options in the TR for resource validity in different cases when the resources are allocated semi-statically by the network: Reader UE performing AIOT operation in RRC_CONNECTED (downselection or support of both options will be decided in the WI phase): - Option 1: UE receives resources in dedicated signalling. The resources remain valid until the network releases them explicitly (understanding is this would be supported by RRC reconfiguration) - Option 2: UE receives resources in dedicated signalling along with a time period in which they can be used. The resources remain valid for that time period. Reader UE performing AIOT operation in RRC_INACTIVE (downselection or support of both options will be decided in the WI phase): - Option 1: UE receives resources in dedicated signalling before moving to RRC_INACTIVE. The resources remain valid until the network releases them explicitly. - Option 2: UE receives resources and a time period in dedicated signalling before moving to RRC_INACTIVE. The resources remain valid for that period of time. Reader UE performing AIOT operation in RRC_CONNECTED and upon “temporary out of connection” during the operation. (downselection between the following options will be performed in the WI phase) - Option 1: Resources remain valid at the reader for a time period (FFS whether the same time period as in RRC_CONNECTED options or a different time period. FFS whether the time period is related to the ongoing AIOT operation). - Option 2: Resources are temporarily invalid at the reader during the “temporary out of connection” condition but may become valid again if the UE recovers from the condition. If reader UE supports AIOT operation in RRC_IDLE, options of RRC_INACTIVE are candidates. – - Support of RRC_IDLE (other than in temporary out of connection) is down-prioritized. During WI can consider it if feasible with minimum spec impact For all options The resources remain valid only in the same cell for which the resources were configured.

Validity based on Explicit Signalling

R2-2410083	Validity of resources and reader behavior in Topology 2	NEC	discussion	Rel-19	FS_Ambient_IoT_solutions

Proposal 4: To properly terminate the validity duration, RAN2 agree to further study necessary assistance information (e.g., the A-IoT resource termination indication) from the UE reader to the NW.

Proposal 5: On the UE reader behavior of termination of the validity duration, RAN2 agree to capture the following option to the TR:

The UE reader terminates the validity of resources upon receiving the termination command of the resource validity via dedicated signaling/MAC CE/DCI.

“Temporary Out of Connection” Scenario

R2-2410151	On Topology 2	Ericsson	discussion	Rel-19	FS_Ambient_IoT_solutions

Proposal 8	Temporarily Out of Connection” means the connection to the serving cell becomes unstable or the intermediate UE may lose/release RRC connection to serving cell soon but still stays in RRC connected. Examples of temporarily out of connection include (but not limited to) following cases/events:

a.	The intermediate UE detects physical layer problems and the RLF timer (i.e., T310) is still running. In this case, the number of out of sync instances has reached a configured number (e.g., N310).

b.	The intermediate UE is performing (conditional) HO and the HO timer (e.g., T304) is still running.

c.	The intermediate UE is performing RRC reestablishment and the reestablishment timer (i.e., T301 or T311) is still running.

Recovery from “Temporary Out of Connection” Scenarios

R2-2410005	Topology 2 for Ambient IOT	InterDigital	discussion	Rel-19	FS_Ambient_IoT_solutions

Proposal 9:	An intermediate UE suspends an ongoing AIOT operation during cell reselection following RLF and can continue the operation if it has valid resources after recovery.

R2-2410595	Discussion on topology 2 considerations of ambient IoT	LG Electronics Inc.	discussion	Rel-19	FS_Ambient_IoT_solutions

Proposal 2. A-IoT radio resource is valid and used for the on-going A-IoT procedure when the UE-reader is in a temporary out of connection scenario (e.g., RLF and handover cases).

Support of IDLE/INACTIVE

R2-2410552	Further aspects of Topology 2	Qualcomm Incorporated	discussion	Rel-19	FS_Ambient_IoT_solutions

Proposal 5:	UE Reader can be in any RRC State while performing R2D/D2R activities. Consequently, the UE reader in any RRC state can collect AIoT data from the AIoT device(s).

R2-2410005	Topology 2 for Ambient IOT	InterDigital	discussion	Rel-19	FS_Ambient_IoT_solutions

Proposal 11:	The intermediate UE can be moved to RRC_INACTIVE after receiving initiation of an AIOT service if it has received valid resources from the gNB.

Proposal 12:	Performing or continuing an AIOT operation after the UE is released to RRC_IDLE is not supported.

Reader Selection

R2-2409688	Remaining issues of AIoT TP2	ZTE Corporation, Sanechips	discussion	Rel-19	FS_Ambient_IoT_solutions

Proposal 4a: As the base station may have more up-to-date information on the connection existence or radio situation of a UE, or even UE’s preference, it should also allow the base station to involve the UE reader selection.

Proposal 4b: The information about whether a UE has been authorized to act as reader may need to be passed from the CN to the base station or reported by the UE to the base station.

Noted

R2-2410098	Discussion on Topology 2 related aspects for Ambient IoT	China Telecom	discussion	Rel-19	FS_Ambient_IoT_solutions

Proposal 7: For the selection of UE reader, RAN2 leave it for RAN3/SA2 decision in the study phase.

Noted

Discussion

-	ZTE, Ericsson, CMCC,  thinks that there may reasons that gNB has more information on UE readers.  gNB has more interference information as well, like beam.

-	ZTE suggests that from RAN2 perspective gNB can be involved in reader selection.   Apple is not sure how it would work in practice.    Huawei explains that there are two options, the CN provides candidates and gNB decides or the CN decides.    ZTE is just saying that from radio perspective it make sense for gNB to be involved.

-	Lenovo prefers to leave it to other WGs for now.

-	Qualcomm thinks that the CN selectes the reader but the gNB can determine that it may not work.   CMCC explains that gNB has more information on the location and coverage of readers.

From RAN2 perspective, at least for RRC based architecture, there may be radio related reasons for gNB to be involved in reader selection.

Whether the gNB or CN selects the reader, and/or what information is shared between gNB and CN is up to RAN3/SA2

Resource Allocation Granularity (may depend on Architecture discussions)

R2-2410647	Remaining issues on Topology 2 aspects of Ambient IoT 	Kyocera 	discussion	Rel-19

Proposal 2	RAN2 should agree that the dynamic grant-like and configured grant-like resource allocations for Ambient IoT communications are supported.

R2-2410266	Considerations on Topology 2 for Ambient IoT	Lenovo	discussion	Rel-19

Proposal 7: RAN2 is suggested to consider following A-IoT air interface resource allocation methods by the serving gNB and option 1 is preferred.

	Option 1: the serving gNB allocates resource pool for the whole A-IoT procedure for specific I-UE.

	Option 2: the serving gNB allocates resource for specific A-IoT message/procedure for specific I-UE.

Information Obtained by the gNB for allocation (may depend on Architecture discussions)

R2-2409701	Ambient IoT resource allocation for topology 2	MediaTek Inc.	discussion	Rel-19	FS_Ambient_IoT_solutions

Proposal 2: An AIoT resource allocation from the gNB is sent in response to a request from the UE reader and consists of a set of radio resources and validity criteria that determine when the resources may be used.

Noted

R2-2409899	A-IoT Topology 2 aspects	Huawei, HiSilicon	discussion	Rel-19

Proposal 4a:	For UP/NAS based solution, the BS can allocate radio resources to UE readers based on UE request or indication from CN, which is subject to final decision on the architecture option (left to normative work).

Proposal 4b:	For RRC based solution, BS can directly configure the dedicated radio resources based on service request from CN.

Noted

Discussion

-	ZTE CN and reader directly then you need request from UE to gNB.  If it is terminated in the gNB the gNB.

-	Ericsson and Qualcomm thinks that option 2 on it’s own may not always work as the CN doesn’t have enough information on needed resource grants.

Agreements - From RAN2 perspective, at least for RRC based architecture, there may be radio related reasons for gNB to be involved in reader selection. - Whether the gNB or CN selects the reader, and/or what information is shared between gNB and CN is up to RAN3/SA2 - Capture the following options: - An AIoT resource allocation from the gNB is sent in response to a request from the UE reader and consists of a set of radio resources and validity criteria (if applicable) - BS can directly configure a set of radio resources and validity criteria (if applicable) based on service request from CN. - These two options can co-exist

Not treated

R2-2409840	Discussions on topology 2 related aspects	Fujitsu	discussion	Rel-19	FS_Ambient_IoT_solutions

R2-2409894	Discussion on topology 2 for Ambient IOT	OPPO	discussion	Rel-19	FS_Ambient_IoT_solutions

R2-2410083	Validity of resources and reader behavior in Topology 2	NEC	discussion	Rel-19	FS_Ambient_IoT_solutions

R2-2410141	Discussion on Topology 2 issues	Spreadtrum, UNISOC	discussion	Rel-19

R2-2410179	Consideration on information and scenario in Topology 2	ASUSTeK	discussion	Rel-19	FS_Ambient_IoT_solutions

R2-2410251	 Discussion on topology 2 for A-IoT	KT Corp.	discussion

R2-2410371	Discussion on ‘temporary out of connection’ in Topology 2	ETRI	discussion	Rel-19

R2-2410467	RAN2 Aspects for AIoT Operation in Topology 2	Nokia	discussion

R2-2410682	Consideration on resource configuration for Topology 2	HONOR	discussion	Rel-19	FS_Ambient_IoT_solutions

R2-2410760	Considerations on Topology 2 for Ambient IoT	TCL	discussion

R2-2410805	Discussion on Topology 2 related aspects	Fraunhofer HHI, Fraunhofer IIS	discussion

## 8.3	AI/ML for Mobility

(FS_NR_AIML_Mob; leading WG: RAN2; REL-19; SID: RP-242393)

Time budget: 2 TUs

Tdoc Limitation: 2 tdocs

### 8.3.1	Organizational

LS, Rapporteur input, including workplan, etc.

R2-2410186	Text proposal of 38.744	OPPO	pCR	Rel-19	38.744	0.0.5	FS_NR_AIML_Mob

The TR endorsed

Prioritization for February (RLF, Event prediction, System level performance (for event prediction), Generalization)

-	Companies would like to downprioritize RLF.    Apple thinks that RLF is more important than useless generalization.  Huawei thinks generalization for UE speeds is useful.

-	Ericsson thinks that event prediction, generalation, and system level can be second step.    Qualcomm agrees.

-	Huawei thinks that event prediction and system level are important, and generalization is lower priority.

For february meeting, RLF simulations are not expected.

[POST128][021][AI Mob] Templates for simulations  (Mediatek/Oppo)

Intended outcome:

1.	agree to updated and new templates.  Companies should update their simulation results in the new template (Mediatek)

2.	agree principles on how to capture observations from simulations (Oppo)

Deadline:  Jan. 15th

    - RRM measurement prediction

#### 8.3.2.1	Simulation results

Contributions should focus on simulation results and observations on the agreed on prioritized scenarios and agreed assumptions. Further input on remaining issues related to RRM measurement prediction.

Any simulation results on non-prioritized scenarios should be clearly captured in separate section indicating “new scenarios”

#### Temporal case A/B (Scenarios 2/4) simulation results:

R2-2410339	Simulation results for RRM measurement prediction	CMCC	discussion	Rel-19	FS_NR_AIML_Mob

Case B: to compare the RSRP difference at different MRRT with the same UE speed

Observation 2: With the same UE speed, the prediction accuracy for intra-frequency temporal domain case B reduces as the increase of MRRT.

Observation 3: When the MRRT is up to 2/3 and above, the average RSRP difference is more than 1dB, and even up to 2.133dB with MRRT is 4/5.

-	Ericsson doesn’t see that large difference so we can have a general observation.    Apple asks in general how do we capture the numbers given the large variance between companies.  Oppo thinks that we can follow RAN1 example.

Observation 5: With the increase of prediction window, the prediction accuracy of the last value at the end of the prediction window deteriorates seriously.

Case A: to compare the RSRP difference at different Observation window vs Prediction window with the same UE speed

Observation 7: The prediction accuracy of the last value does not deteriorate obviously compared with the Average RSRP difference when OW and PW have the same length, even if the UE speed is up to 90km/h.

Observation 8: Keeping OW and PW have the close length is helpful to ensure the prediction accuracy for intra-frequency temporal domain case A.

Noted

R2-2410799	Evaluation on RRM measurement prediction	ZTE Corporation	discussion	Rel-19	FS_NR_AIML_Mob

Observation 1: Regarding temporal domain prediction case B, with the same MRRT, different skipping pattern can provide different prediction performance.

-	Vivo thinks that we can go even further and say that skipping pattern example 2 has better performance.

Observation 2: Regarding temporal domain prediction case B, filtering option 2 and filtering option 3 provide similar prediction performance, where

Filtering option 2: L3 filtering is based on the L1 filtered result if the last L3 filtered result is from prediction;

Filtering option 3: L3 filtering is based on the L1 filtered result and the L3 filtered result from last actual measurement.

Proposal 1: For temporal domain prediction case B, suggest companies to report the adopted skipping pattern when providing simulation results.

Noted

#### Inter-frequency prediction (Scenario 3)

R2-2409823	Discussion on the simulation results for RRM measurement prediction	Samsung	discussion	Rel-19	FS_NR_AIML_Mob

Observation 11: For the inter-frequency prediction, the evaluation results show a high inter-frequency correlation between the L3 cell-level RSRP of the two co-located cells. The correlation coefficient is observed as 0.899, 0.902, 0.918 for each of UE speed 30, 60, 120km/h, respectively.

Observation 12: For the inter-frequency prediction, the average L3 RSRP difference is observed as 4.144, 3.907, 3.61 dB with the correlation coefficient of 0.899, 0.902, 0.918, respectively.

Observation 13: For the inter-frequency prediction, the evaluation results show that higher correlation coefficient between two frequency layers results in higher prediction accuracy.

Observation 14: Despite the high correlation between the L3 RSRP of the two cells in different freq., the L3 RSRP difference in inter-frequency prediction is a bit larger compared to the values in other cases (i.e., temporal domain prediction Case A&B).

-	ZTE doesn’t think that this is a common observation from companies, it varies quite a bit.  Huawei also doesn’t show the same observation.

Noted

R2-2409866	Discussion on RRM prediction simulation result	Xiaomi	discussion

Case 3 simulation result

Observation 1: Cluster as input can improve the prediction accuracy than single cell as input.

Observation 2: More cells in cluster as input can improve the prediction accuracy.

-	Ericsson thinks that for bad cells it is easy to predict that the value will be bad, and this may cause adding lots of bad cells in the clusters.    Apple agrees and not sure if it brings much and increased number of cells will increase model complexity.

-	Xiaomi explains that maximum is 7 cells in the cluster, but agree that if we add more than the gain won’t be much.

-	Mediatek thinks that the more good cells will improve performance but it will increase complexity.

-	Oppo thinks we should make it clear that while it improves performance but should capture that it comes with complexity cost.

-	Ericsson thinks that we need to follow the RAN4 requirements that the UE can only measure up to 4 cells in a different frequency.    Oppo and InterDigital thinks that we are measuring the serving cells frequency to predict another frequency.  Ericsson explains that we can take measurement on another non-frequency layer to predict another frequency.   Huawei doesn’t think this was the intention of inter-frequency prediction.

Observation 3: Addition of historic measurement results as input can provide prediction accuracy gain.

-	Oppo disagrees with this observation.   Vivo thinks that non-AI approach provides quite good results.

Observation 4: The addition of UE location can provide significant gain only when the number of cells in cluster is small.

-	Apple wonders if we should study how UE location improves the accuracy, as this is more general.   Mediatek thinks that it is tricky as in a real system UE location information is proprietary.    ZTE indicates that we never agreed to have UE location as an input.

Noted

Agreements/Observations to be captured in TR

- The prediction accuracy for intra-frequency temporal domain case B reduces as MRRT increases.

- For temporal domain case B, with the same MRRT, different skipping pattern can provide different prediction performance. Companies may report the adopted skipping pattern when providing simulation results. Companies are not required to run new simulations but can clarify in the spreadsheet. We can capture in the TR how the skipping patter affects the performance.

- For the inter-frequency prediction, the evaluation results show that the higher the correlation coefficient is between two frequency layers, the higher the prediction accuracy. FFS on observations on low correlations.

- For inter-frequency, cluster as input (i.e. measurements from different cells as inputs )can improve the prediction accuracy than single cell as input. For temporal domain, the gains are unclear. RAN2 will focus on frequency domain for cluster based approach.

- In cluster approach the model takes measurements from more than one cell as inputs.

#### Benchmarking with Non-AI ML/simple AI models:

R2-2410187	Discussion on simulation result of RRM measurement prediction	OPPO	discussion	Rel-19	FS_NR_AIML_Mob

Observation 1: For FR1 intra-frequency temporal domain case B, when PW is short, the performance between AI and sample-and-hold is not significant. However, when PW becomes larger, AI outperforms sample-and-hold. The observed AI gain ranges from 21% to 37%.

Observation 2: For FR1 intra-frequency temporal domain case B, the increase of MRRT results in a decrease in AI gain.

Observation 3: For FR1 intra-frequency temporal domain case B, the increase of UE speed results in a decrease in AI gain.

R2-2409868	Simulation Results for AIML RRM Prediction and Remaining Issues 	MediaTek Inc.	discussion

Observation 3: In frequency domain prediction cases, cell-based approaches achieve limited gain compared to the non-AI approach without the help of neighbor cell measurement. The cluster-based approach outperforms cell-based approaches across all sub-use cases.

Observation 8: For intra-frequency temporal domain case A, AI can provide ~20% gain (in terms of L3 cell RSRP difference) compared to the non-AI approach, i.e., sample and hold. The improvement is more significant in the high UE speed case, e.g., 120km/hr, than in the low UE speed cases, e.g., 60km/hr.

Discussion

-	Apple asks why only sample and hold.  Mediatek explains that sample and hold is very simple.    Apple thinks that there are very simple model like linear regression, and this may show that the gains may be small.   Docomo thinks that sample and hold is easier to align and with any other approaches we need to align the assumptions.  Apple explains that it is only 3 parameters.

-	Oppo thinks that we can capture the observations and explain what non-AI  approach is used.  Docomo explains that we can follow the RAN1 approach, define two non-AI benchmarks.   Nokia agrees that we can agree on two approaches, sample and hold.    Samsung is ok to use sample and hold but this is only applicable to temporal domain.

Agreements/observations

- Companies can compare results with non-AI approaches.   Temporal domain, sample and hold, and frequency domain, pathloss offset.   Companies can consider other simple models (e.g. ARIMA).

- For FR1 intra-frequency temporal domain case B, when PW is short, the performance between AI and sample-and-hold is not significant. However, when PW becomes larger, AI outperforms sample-and-hold.

- In frequency domain prediction cases (2GHz and 4GHz), cell-based approaches achieve limited gain compared to pathloss offset without the help of neighbor cell measurement. The cluster-based shows better performance compared to pathloss offset.

- For intra-frequency temporal domain case A, AI can provide gains (in terms of L3 cell RSRP difference) compared to sample and hold. The gain improves with UE speed.

#### Other aspects:

R2-2410020	Simulation results on the RRM measurement prediction and discussions	NTT DOCOMO, INC.	discussion

Proposal 2

Evaluate the RRM measurement prediction in serving and Top-K neighboring cells, where K=[1,2,… ].

Note: It is up to companies to choose the cell-common, cell-specific, or cluster-based approach.

RAN2 selects one of the following options for the RSRP difference report.

- Opt.1: Separately report for serving and neighboring. The RSRP is averaged across neighboring when K > 1.

- Opt.2: Averaged across all K + 1 cells.

- Opt.3: Report the RSRP differences averaged across the Top-M (M=1,2,…,K) cells at the prediction time instance.

- Other approaches.

R2-2409868	Simulation Results for AIML RRM Prediction and Remaining Issues 	MediaTek Inc.	discussion

Clarification of average RSRP difference

Proposal 2: Companies can report how to derive the average L3 cell-level RSRP difference. Specifically, do we consider neighbor cells and what is the scope of the “average”?

- Option 1: We consider neighbor cells prediction, the “average” refers to the average of all cells including the serving cell and neighboring cells.

- Option 2: We only consider serving cell prediction, the “average” refers to the average of the serving cell.

Other option

Proposal 3: RAN2 consider “average” to be the average of all cells, including the serving cell and all other neighbor cells as the baseline method to derive the average L3 cell-level RSRP difference.

R2-2410187	Discussion on simulation result of RRM measurement prediction	OPPO	discussion	Rel-19	FS_NR_AIML_Mob

Proposal 1: RAN2 can capture the performance results in the following structure:

RRM prediction

- Basic performance for scenario 2

- Basic performance for scenario 3

- Basic performance for scenario 4

- Basic performance for scenario 6

- Generalization performance

Summary of performance results

Proposal 2: Under basic performance for scenarios 2, 3, 4 and 6, using separate sub-sections for sub cases 1, 2, and 3, e.g.,

Basic performance for scenario 2

- Sub case 1

- Sub case 2

- Sub case 3

Proposal 3: Within each sub case, capture the following sub-bullets on performance results:

- For scenario 2: (A) MRRT, (B) UE speed

- For scenario 3: No sub-bullet

- For scenario 4: (A) observation window : prediction window, (B) UE speed

- For scenario 6: No sub-bullet

All contributions with simulation results in this AI are noted

R2-2409651	Simulation results of RRM Measurement Prediction	CATT, Turkcell	discussion	Rel-19	FS_NR_AIML_Mob

R2-2409667	Updated simulation results for RRM measurement prediction	vivo	discussion	Rel-19	FS_NR_AIML_Mob

R2-2409823	Discussion on the simulation results for RRM measurement prediction	Samsung	discussion	Rel-19	FS_NR_AIML_Mob

R2-2409866	Discussion on RRM prediction simulation result	Xiaomi	discussion

R2-2409868	Simulation Results for AIML RRM Prediction and Remaining Issues 	MediaTek Inc.	discussion

R2-2409971	Cluster-based approach, UE-sided vs. network-sided models, etc.	Apple	discussion	Rel-19	FS_NR_AIML_Mob

R2-2410020	Simulation results on the RRM measurement prediction and discussions	NTT DOCOMO, INC.	discussion

R2-2410144	Simulation results on RRM measurement prediction	Spreadtrum, UNISOC, BUPT	discussion	Rel-19

R2-2410187	Discussion on simulation result of RRM measurement prediction	OPPO	discussion	Rel-19	FS_NR_AIML_Mob

R2-2410339	Simulation results for RRM measurement prediction	CMCC	discussion	Rel-19	FS_NR_AIML_Mob

R2-2410474	Simulation results for temporal, inter-frequency and spatial domain RRM measurement predictions	Ericsson	discussion	FS_NR_AIML_Mob

R2-2410507	Simulation results for RRM measurement predictions	Interdigital Inc.	discussion	Rel-19	FS_NR_AIML_Mob

R2-2410539	Simulation results for RRM measurement prediction	Huawei, HiSilicon	discussion	Rel-19	FS_NR_AIML_Mob

R2-2410678	Simulation and evaluation of RRM measurement prediction	Indian Institute of Tech (M), IIT Kanpur	discussion	Rel-19

R2-2410781	Simulation results for RRM Measurement Prediction	CEWiT	discussion	Rel-19	FS_NR_AIML_Mob

R2-2410796	On the RRM measurement prediction aspects	Nokia	discussion	Rel-19	FS_NR_AIML_Mob

R2-2410799	Evaluation on RRM measurement prediction	ZTE Corporation	discussion	Rel-19	FS_NR_AIML_Mob

R2-2410037	Discussion on cluster based RRM measurement prediction	BJTU	discussion

R2-2410744	AI-ML based Inter-frequency measurement prediction	Rakuten Mobile, Inc	discussion	Rel-19

R2-2410774	Simulation results for RRM measurement prediction	Qualcomm Incorporated	discussion	Rel-19

### 8.3.5	Other

Including outcome [POST127bis][022][AI mobility] Simulation Assumption of measurement event/RLF prediction and SLS (OPPO)

Contributions on simulations assumptions, including  controversial aspects of email discussion or on aspects not covered in email discussion related to simulation assumptions for RLF, Event prediction, and system performance evaluation

Contributions on aspects and assumptions related to generalization study for RRM prediction

Model Generalization (definition/scenarios):

R2-2410023	Discussions on evaluation methodology of AI/ML for mobility	NTT DOCOMO, INC.	discussion

Proposal 1

•	Reuse the evaluation methodology in TR38.843 for generalization study, i.e., the generalization performance is evaluated with the following 3 cases,

- Generalization Case #1 (GC#1): The AI/ML model is trained using the dataset with Configuration #B and tested using the dataset with Configuration #B.

- Note: GC#1 is the baseline for the generalization evaluations.

- Generalization Case #2 (GC#2): The AI/ML model is trained using the dataset with Configuration #A but tested using the dataset with Configuration #B.

- Generalization Case #3 (GC#3): The AI/ML model is trained using mixed datasets with both configurations and tested using the dataset with Configuration #B.

-	Vivo thinks that RAN1 also adds scenario.  Oppo explains that this is a principle and configuration can be anything.   MEdiatek agrees with the principles.

-	Mediatek thinks we should start with case 1 to minimize simulation efforts.

-	Xiaomi thinks that case 1 is mandatory but we need to chose either 2 or 3.

Proposal 2

•	Use the intermediate KPIs for the generalization study. The relative performance of intermediate KPIs between GC#2/#3 and GC#1 is used.

-	Samsung thinks that case number 2 can be optional.   Apple suggests that we allow both and companies can chose whether we compare with 2 or 3.    Qualcomm thinks at least case 2 is necessary for generalization.   Nokia agrees with Qualcomm.

-	Ericsson is not sure that case 3 is generalization, so case 2 is needed.   Interdigital agrees and doing case 3 might incur a lot of simulations.

-	ZTE thinks that case 3 is easier for simulations.  Oppo thinks that we can simplify case 2 by having less combination.

Noted

R2-2409829	Discussion on Generalization Issues for AI/ML Mobility	Samsung	discussion	Rel-19	FS_NR_AIML_Mob

Proposal 1: RAN2 to prioritize generalization issues on RRM measurement prediction while deprioritizing generalization issues on measurement/RLF/HOF event prediction.

Proposal 2: RAN2 to study generalization issue on RRM measurement prediction in temporal domain with different UE speeds.

Discussion

-	Xiaomi thinks that we should evaluate other things in addition to UE speeds

-	Apple thinks UE speed is the least interesting thing to study, we should study against different cells and configuration.   Qualcomm agrees UE speed is not interesting, we should look at parameters, instead cells which are complicated.

-	Interdigital thinks that we should be careful on what we agree to generalize and we should focus on parameters we have already done simulations for it is easier, but we should be careful not to introduce too many new scenarios/parameters.

-	Vivo thinks we should consider other thinks like scenarios

-	Qualcomm thinks we should also consider inter-frequency domain.   ZTE is fine to prioritize but also interested for inter-frequency.   Huawei thinks that this is still temporal domain.

-	Huawie indicates that UE speed is still important to verify wether we can generalize

Noted

Model Generalization (parameters/settings to be generalized):

R2-2410188	Discussion on generalization study of AI mobility	OPPO	discussion	Rel-19	FS_NR_AIML_Mob

Proposal 2: RAN2 focus on generalization over UE speed

Proposal 4: The simulation assumption of FR1 temporal domain case B is reused for generalization study with 3 UE speeds i.e. 30Km/h, 60Km/h and 90Km/h

Proposal 5: The simulation assumption of FR2 temporal domain case A is reused for generalization study with 3 UE speeds i.e. 60Km/h, 90Km/h and 120Km/h

-	Apple reminds every one that there is zero standardization impact for UE speed.   Samsung thinks that if UE speeds can’t be generalized then it can be precluded from NW sided model cases.

-	ZTE thinks this simulation assumptions should also apply to frequency domain.  Xiaomi doesn’t think UE speed should be study for frequency domain.   Samsung agrees, UE speed doesn’t have much impact on the frequency domain so this study is not necessary.

-	Nokia thought that FR2 we would do lower speed.  Vivo explains that for FR2 the study goal is related to measurement reductions.

[AT128][017][AI mob]Simulation assumptions  ()

Intended outcome: Generalization combinations for UE speed

Deadline:  10-17-24

Study generalization over UE speeds

The simulation assumption of FR1 temporal domain case B is reused for generalization study with 3 UE speeds i.e. 30Km/h, 60Km/h and 90Km/h.  FFS on combinations

The simulation assumption of FR2 temporal domain case A is reused for generalization study with 3 UE speeds i.e. 60Km/h, 90Km/h and 120Km/h.  FFS on combinations

Agreements on generalization - Reuse the evaluation methodology in TR38.843 for generalization study, i.e., the generalization performance is evaluated with the following cases, - Baseline: The AI/ML model is trained using the dataset with Configuration #B and tested using the dataset with Configuration #B. - Generalization Case #1 (GC#1): The AI/ML model is trained using the dataset with Configuration #A but tested using the dataset with Configuration #B. - Generalization Case #2 (GC#2): The AI/ML model is trained using mixed datasets with both configurations and tested using the dataset with Configuration #B. 2 Companies can choose which case they compare with and should report it with simulation results. 3 Generalization issues on RRM measurement prediction are prioritized. 4 Start the study with generalization issue with RRM measurement prediction in temporal domain. Companies can chose to study frequency domain prediction cases and report what they have simulated. 5 Study generalization over UE speeds 6 The simulation assumption of FR1 temporal domain case B is reused for generalization study with 3 UE speeds i.e. 30Km/h, 60Km/h and 90Km/h. FFS on combinations 7 The simulation assumption of FR2 temporal domain case A is reused for generalization study with 3 UE speeds i.e. 60Km/h, 90Km/h and 120Km/h. FFS on combinations

R2-2411175	Summary of [AT128][017][AI mob]Simulation assumptions (OPPO)	OPPO	discussion	Rel-19	FS_NR_AIML_Mob

Noted

Agreements on generalization 1 Between GC#1 and GC#2, RAN2 focus on GC#2 2 simulation combination for FR1 generalization study: Training @Dataset: 30km/h; Training @Dataset: 60km/h; Training @Dataset: 90km/h; Inference @30km/h; Inference @60km/h; Inference @90km/h. Baseline; Yes; Yes. GC#1; Yes; Yes; Yes. GC#2; Yes; Yes; Yes; Yes. Baseline; Yes; Yes. GC#1; Yes; Yes; Yes. GC#2; Yes; Yes; Yes; Yes. Baseline; Yes; Yes. GC#1; Yes; Yes; Yes. GC#2; Yes; Yes; Yes; Yes. 2a simulation combination for FR2 generalization study: Training @Dataset: 60km/h; Training @Dataset: 90km/h; Training @Dataset: 120km/h; Inference @60km/h; Inference @90km/h; Inference @120km/h. Baseline; Yes; Yes. GC#1; Yes; Yes; Yes. GC#2; Yes; Yes; Yes; Yes. Baseline; Yes; Yes. GC#1; Yes; Yes; Yes. GC#2; Yes; Yes; Yes; Yes. Baseline; Yes; Yes. GC#1; Yes; Yes; Yes. GC#2; Yes; Yes; Yes; Yes.

Agreements on measurements events 1 Agreed values for Case A. NOTE1 indirect prediction only Parameters; baseline value; Note. A3 event offset (db); 2; Open for 3db. TTT (ms); 320; Open for one shorter value. UE speed (km/h); 90; Open for 60 and 120km/h. OW length (ms, note1); N/A; Up to implementation. PW length (ms, note1); 400; Open for more values. Max ETD (ms, note1); 80; Open for more values. 2 Agreed values for Case B. NOTE1 indirect prediction only Parameters; baseline value; Note. A3 event offset (db); 2; Open for 3db. TTT (ms); 320; Open for one shorter value. UE speed (km/h); 30; Open for 60 and 90km/h. OW length (ms,note1); N/A; Up to implementation. PW length (ms,note1); 200 (non-sliding) 40ms (sliding); Open for more values. Max ETD (ms, note1); 80; Open for more values. MRRT; 50%; Open for more values. 3 Company can report which filtering options is being used for the input L3 RSRP of sub-use case 2: option 1, option 2, option 3. 4 to capture the 3 options into TR

Agreements on RLF predictions 1 Agree to following values (NOTE1 indirect only) Parameter; Value. Qin threshold; -6db. Qout threshold; -8db. Sample rate (TIndication_interval); 20ms (FR2)/40ms(FR1). Qin evaluation period; 100ms. Qout evaluation period; 200ms. T310; 1000ms. N310; 1. N311; 1. Max ETD (ms, note1); 80ms. PW length (ms,note1); 400. OW length (ms, note1); Up to implementation. 2 The beam transmission pattern is synchronized across the site/cells i.e., at any given time the transmitted beam index is the same across the site/cells.

For System level simulation 1 For measurement event prediction for temporal domain case B: UE reports when A3 event is satisfied with actual measurements and predicted results. And handover command will be received after handover preparation. Remove T0 from the picture 2 For measurement event prediction for temporal domain case A, company focus on option 2 or option 3. Option 2: network transmit handover command purely based on actual measurement event regardless whether an actual measurement result(@t2) is earlier or later than predicted measurement event((@t1)) Option 3: For AI mobility, HO preparation starts when an event is predicted to happen (i.e., t0), and HO command is sent when A3 entering conditions are met based on actual/real measurement and an event is predicted to be met for the duration of TTT. 3 Handover Preparation time is 40ms and handover execution time: 40ms 4 RAN2 focus on indirect prediction methodology.

Agreements on direct prediction for RLF and event predictions 1 The time window for direct prediction goes for interpretation 2 2 For direct prediction, following values are agreed and company can report probability threshold for corresponding case (other values are allowed) Measurement event prediction; RLF prediction. Time window length (Interpretation 2); PW length as indirect case (FR1:200ms; FR2:400ms); PW length as indirect case FR1/FR2: 400ms.

R2-2410263	Discussion on generalization aspects	Ericsson	discussion	Rel-19	FS_NR_AIML_Mob

Noted

Proposal 7	For generalization evaluation over carrier frequencies:

- Training dataset is 0,8 GHz and 4 GHz, and inference dataset is 2 GHz.

- Training dataset is 2 GHz and inference dataset is 4 GHz.

- Training dataset is 4 GHz and inference dataset is 2 GHz.

-	Apple thinks this is even less interesting that UE speed.   Qualcomm sees some value in the case where we can see if we can generalize over different frequencies so we don’t need multiple measurement gaps.

-	Docomo indicates that for operators this would be important for frequencies in different bands.   Mediatek thinks that one issue is which bands we select for simulations.   Docomo thinks we can consider a larger one.

-	Nokia asks why we need three frequencies, as it is not aligned with agreed case 2.   Oppo agrees and if we should follow existing simulation assumptions and just focus on 2GHz and 4GHz.   Xiaomi agrees we should limit to two for temporal domain.

-	Samsung also has some interest on studying different frequencies so we would need a new frequency.

-	Huawei doesn’t think it is a big deal to have different models for two frequencies.

-	Oppo points out that we have only agreed to 2 and 4GHz, and introducing new frequencies would require more work and simulations.

Companies that would like to study inter-frequency generalization can start with input 2GHz and output 4GHz, and 4GHz to 2GHz.   FFS if we introduce a third frequency.

Proposal 8	For generalization evaluation over cell size:

- Training dataset is small cells, and inference dataset is large cells.

- Training dataset is large cells and inference dataset is small cells.

-	Mediatek asks if we are referring to Umi, or power.  Ericsson was thinking urban macro, UMa Umi

-	Xiaomi thinks that RAN1 has studying this and similar performance is expected and we should prioritize mobility parameters.

-	Vivo thinks that gNB heights and ISD should be considered.

-	Samsung asks if this is for temporal domain. Ericsson confirms.  Samung thins the simplest solution is to use ISD value.

-	Apple thinks we should study this and look at list of parameters we can play with.

-	ZTE thinks that this is the one that will require the most time and it is not the highest priority.

-	Qualcomm thinks that we can’t expect generalization between urban macro and micro as they have very different propagation models.

-	Ericsson wonders whether the UE knows the gNB height and ISD.  Huawei indicates that is the point, if the model does well across different height/ISD then the UE doesn’t need to know.   We should agree to a small subset.  Qualcomm thinks we should also include the power.

-	Mediatek thinks that we need generalization to determine what input is needed for LCM

Study model generalization across different cell configurations (e.g. ISD, gNB height, power, beam pattern, etc).   FFS which parameters we prioritize.

Further agreements on generalization

- Companies that would like to study inter-frequency generalization can start with input 2GHz and output 4GHz, and 4GHz to 2GHz.   FFS if we introduce a third frequency.

- Study model generalization across different cell configurations (e.g. ISD, gNB height, power, beam pattern, etc).   FFS which parameters we prioritize.

[Post128][018][AI Mob] generalization  (Apple)

Intended outcome: Discuss parameters for different cell configuration and attempt to prioritize 1 parameters and not more than 2 values per parameter.   Can do 2 max values if really reneed.   for

Deadline:  3 weeks

Email discussion ([POST127bis][022][AI mobility] Simulation Assumption of measurement event/RLF prediction and SLS (OPPO):

R2-2410190	Summary of [POST127bis][022][AI mobility] Simulation Assumptions (OPPO)	Hangzhou Mengyuxiang	discussion	Rel-19

Noted

Agreements on event prediction 1 Agree to listed 4 definitions of indirect measurement event prediction Indirect measurement event prediction for temporal domain case A: In indirect measurement event prediction, future measurement result(s) is predicted by a RRM measurement prediction model for intra-frequency temporal domain case A at first. Afterwards, predicted and optionally actual historical measurement result(s) are used to derive whether a measurement event at one future time instance occurs, without further involvement of an AI/ML model. Indirect measurement event prediction for temporal domain case B: In indirect measurement event prediction, measurement result(s) is predicted by a RRM measurement prediction model for intra-frequency temporal domain case B at first. Afterwards, predicted and optionally actual historical measurement result(s) are used to derive whether a measurement event at one time instance occurs, without further involvement of an AI/ML model. Indirect measurement event prediction for frequency domain: In indirect measurement event prediction, measurement result(s) is predicted by a RRM measurement prediction model in frequency domain at first. Afterwards, predicted and optional actual historical measurement result(s) of serving cell are used to derive whether a measurement event at one time instance occurs, without further involvement of an AI/ML model. Indirect measurement event prediction for spatial domain: In indirect measurement event prediction, measurement result(s) is predicted by a RRM measurement prediction model in spatial domain at first. Afterwards, predicted measurement result(s) and actual measurement results are used to derive whether a measurement event at one time instance occurs, without further involvement of an AI/ML model 2 The input of model for RRM measurement prediction can be reused as baseline for corresponding direct measurement event prediction. Additional input(s) is also allowed. 3 For intra-frequency temporal domain case B indirect measurement event prediction is taken as baseline. Direct prediction is optional. 4 To agree following definition for true event prediction, false event detection and missed event detection for indirect measurement event prediction Counter n3(true event prediction): it increases by 1 when a real event occurs around a predicted event with ETD, whose range is [0, maximum ETD] or vice versa Counter n1(false event detection): it increases by 1 when no real event occurs around a predicted event with ETD, whose range is [0, maximum ETD] Counter n2(missed event detection): it increases by 1 when no event is predicted around a real event with ETD, whose range is [0, maximum ETD] 5 To agree following definition for true event prediction, false event detection and missed event detection for direct measurement event prediction Counter n3’ (true event prediction): it increases by 1 when a real event occurs within the occurrence window of predicted event whose possibility is higher than a predefined threshold Counter n1’ (false event detection): it increases by 1 when no real event occurs within the occurrence window of predicted event whose possibility is higher than a predefined threshold Counter n2’ (missed event detection): it increases by 1 when a real event occurs, but it doesn’t fall in the occurrence window of any predicted event whose possibility is higher than a predefined threshold 6 Agree following definition of F1 score: F1 score = 2*Precision*Recall/(Precision + Recall) Formula_3 (13/14) 7 In addition to F1 score, precision and recall in following formula are optional metrics to report. (12/14) Precision = n3/(n1+n3) Formula_1 Recall =n3/(n2+n3) Formula_2 Note: for direct prediction, the counter should be n1’,n2’,n3’

Agreements on RLF prediction:

1	 To agree following indirect RLF prediction definition:

The future L1 SINR results are predicted based on actual L1 SINR results of the serving cell by following same way of intra-frequency temporal domain case A. Afterwards, RLF event at one time instance is determined based on predicted and optionally actual L1 SINR results within T310 duration, without further AI/ML models. As baseline L1 SINR refers to raw L1 SINR without L1 filtering.

2	Reuse agreed metrics for measurement event prediction for RLF prediction for both indirect and direct prediction respectively

3	Table 5.1-1 in TR 38.744 is taken as baseline simulation assumption for RLF prediction for both FR1 and FR2

4	It is assumed that all cells are fully loaded for interference modelling and no resource scheduler is needed

5	Interference in simulation comes from co-site cells and surrounding 6 sites of serving cell, i.e., interference comes from 20 cells

6	The beam with highest L1 RSRP of the serving cell is taken as serving beam, which is taken as the serving signal of RLM

Interference in simulation comes from co-site cells and surrounding 6 sites of serving cell, i.e., interference comes from 20 cells

-	Qualcomm we don’t need to model the interference as the processing gain will supress the interference.   Xiaomi thinks it is necessary.  Docomo thinks that the comments from qualcomm makes sense.   Oppo explains that we don’t model procedures and processing.   If the gain is 10dB will you ever have RLF?

Agreements on SLS : 1 As for handover model it is proposed to agree for both temporal domain case A and case B: Network start with handover preparation once a predicted measurement event is received. A handover command will be transmitted at least after preparation is completed. After that one fixed execution time is assumed. 2 To reuse HO failure model and corresponding metrics i.e. HO failure rate, total number of HO attempts per UE per second from 36.839 3 The agreed parameters in section 2.1.3 for measurement event prediction can be reused for SLS 4 The agreed parameters in table 2.2.2-1 i.e. all but last 3 parameters can be reused for both FR2 temporal domain case A and FR1 temporal domain case B in SLS 5 Interference model in section 2.2.3 is reused for SLS

As for simulation based on temporal domain case B, agree following approach:

If a predicted A3 event at t1 is reported at t0 (t0<=t1) then HO command is transmitted at t3, where t3=t0+max(HO prep time, t1-t0).

-	Samsung and Xiaomi thinks we should be able to report at t1.  Interdigital thinks that it should be when the command is assumed to be received (rather than when it is transmitted)

Agreements on inter-frequency prediction :

To use Pearson correlation coefficient for correlation coefficient calculatio

Not treated

System level simulation/evaluation:

R2-2410800	Discussion on generalization aspects and simulation assumption	ZTE Corporation	discussion	Rel-19	FS_NR_AIML_Mob

Proposal 7: Suggest to adopt the following handover model in the system level simulation for temporal domain case A: assuming a predicted A3 event at future t1 is reported at t0 and one potential real A3 event is triggered at t2:

If handover preparation time >= t1– t0, handover commend is sent at t0 + handover preparation time;

Else (if handover preparation time < t1-t0), the handover command is sent at t1 or t2 (depends on which one comes first).

R2-2410540	Discussion on simulation assumptions and generalization	Huawei, HiSilicon	discussion	Rel-19	FS_NR_AIML_Mob

Proposal 1: For AI mobility, HO preparation starts when an event is predicted to happen (i.e., t0), and HO command is sent when A3 entering conditions are met based on actual/real measurement and an event is predicted to be met for the duration of TTT. Using the timing from the figures: HO command is sent at t3, where t3=t0+max(HO prep time, t1-t0-TTT), provided that entering conditions of the event are met based on real/actual measurement at t3.

R2-2409867	Simulation assumptions on event/RLF/SLS and model generalization	Xiaomi	discussion

Proposal 6: RAN2 agree following HO modelling for case A and event prediction,

- NW can start the HO preparation after receiving the event prediction.

- After prediction, UE would still perform measurement to evaluate A3 event and eventually trigger MR if event is fulfilled.

- Upon receiving the real MR, NW sends HO command if HO preparation is finished. Otherwise, NW sends HO command after HO preparation is finished.

Proposal 7: For case B, the HO modelling in [1] can be reused. The MR event is evaluated based on both real and predicted measurement results.

Measurement event/RLF prediction:

R2-2410797	On the remaining simulation assumptions and model generalization aspects	Nokia, Nokia Shanghai Bell	discussion	Rel-19	FS_NR_AIML_Mob

Proposal 1: The selection of the probability threshold that is used in the classification evaluation is left for each company to choose.

R2-2409869	Simulation Assumptions of SLS, measurement event prediction, RLF prediction and generalzatiion study	MediaTek Inc.	discussion

Proposal 4: For measurement event prediction, RAN2 consider the following option for obtaining ground-truth with HO:

Consider the switch of the serving cell and neighbour cell during data processing to record all events and ultimately derive the metrics.

Proposal 5: For measurement event prediction, RAN2 consider the following options for obtaining the ground-truth without HO:

- Option1: Switch the serving cell to the strongest one at the event condition is met.

- Option 2: Fixed serving cell and reset TTT at the event condition is met.

- Option 3: Terminate the trajectory when the event condition is met.

Proposal 6: The time window is interpretated to include a prediction window and a tolerance window. The prediction window is the time window for model inference output.  The tolerance window is the time window used to derive the intermediate KPI (i.e., F1 score) based on the output of prediction window.

Proposal 9: For direct measurement event prediction, with the highest probability time instance higher than certain threshold in prediction window, the tolerance window can be used to determine the True event prediction”, “Missed event prediction”, and “False event prediction” to derive the F1 score

R2-2409972	Model generalization, RLF evaluation assumptions, etc.	Apple	discussion	Rel-19	FS_NR_AIML_Mob

Proposal 1: to consider F-2 score (which gives more weight to recall rather than precision and is therefore a more suitable metric for scenarios where false negative is more important than false positive).

Proposal 2: give more weight to missed event detection/false negative KPIs when evaluating measurement event and RLF prediction algorithms (compared to other metrics, i.e. F1-score and false event detection/false positive).

Proposal 3: consider a threshold lower than 50%, for example three values can be considered: 30%, 40%, 50% (this is much simpler). Alternatively, consider classification threshold as a hyper parameter for optimization and allow companies to evaluate different values (this may have non-insignificant evaluation burden).

Proposal 5: Prediction Window is defined as a time window, characterized by start time t1 and end time t2, within which an event is predicted to occur with a certain probability. The event may be RLF or A3 measurement event, or indeed any other event. This is applicable to both direct and indirect prediction methods.

R2-2410190	Summary of [POST127bis][022][AI mobility] Simulation Assumptions (OPPO)	Hangzhou Mengyuxiang	discussion	Rel-19

R2-2409652	Discussion on generalization for RRM prediction	CATT, Turkcell	discussion	Rel-19	FS_NR_AIML_Mob

R2-2409668	Discussion on generalization study for RRM prediction	vivo	discussion	Rel-19	FS_NR_AIML_Mob

R2-2409795	Simulation assumption for Measurement event prediction	NEC	discussion

R2-2409829	Discussion on Generalization Issues for AI/ML Mobility	Samsung	discussion	Rel-19	FS_NR_AIML_Mob

R2-2409867	Simulation assumptions on event/RLF/SLS and model generalization	Xiaomi	discussion

R2-2409869	Simulation Assumptions of SLS, measurement event prediction, RLF prediction and generalzatiion study	MediaTek Inc.	discussion

R2-2409972	Model generalization, RLF evaluation assumptions, etc.	Apple	discussion	Rel-19	FS_NR_AIML_Mob

R2-2409991	Simulation assumptions and methodology for Measurement Event prediction, RLF prediction, and SLS	Qualcomm Incorporated	discussion	Rel-19

R2-2410023	Discussions on evaluation methodology of AI/ML for mobility	NTT DOCOMO, INC.	discussion

R2-2410084	Other aspects for RRM measurement prediction	Lenovo	discussion

R2-2410188	Discussion on generalization study of AI mobility	OPPO	discussion	Rel-19	FS_NR_AIML_Mob

R2-2410263	Discussion on generalization aspects	Ericsson	discussion	Rel-19	FS_NR_AIML_Mob

R2-2410345	Discussion on  other aspects of simulation assumption	CMCC	discussion	Rel-19	FS_NR_AIML_Mob

R2-2410508	Generalization of AIML models for RRM measurement prediction	Interdigital Inc.	discussion	Rel-19	FS_NR_AIML_Mob

R2-2410522	Discussion on RRM measurement predictions and prediction-based mobility events	Sharp	discussion	Rel-19

R2-2410540	Discussion on simulation assumptions and generalization	Huawei, HiSilicon	discussion	Rel-19	FS_NR_AIML_Mob

R2-2410697	Discussion on simulation assumptions for RLF prediction	KDDI Corporation	discussion	Rel-19

R2-2410797	On the remaining simulation assumptions and model generalization aspects	Nokia, Nokia Shanghai Bell	discussion	Rel-19	FS_NR_AIML_Mob

R2-2410800	Discussion on generalization aspects and simulation assumption	ZTE Corporation	discussion	Rel-19	FS_NR_AIML_Mob

## 8.4	Low-power wake-up signal and receiver for NR (LP-WUS/WUR)

(NR_LPWUS-Core; leading WG: RAN1; REL-19; WID RP-241824)

Time budget: 1 TU

Tdoc Limitation: 3 tdocs

### 8.4.1	Organizational

LS, Rapporteur input, including workplan, etc.

Spec editor assignment suggested by WI Rapporteur

38.300 => Ericsson

38.321 => Apple

38.331 => vivo

38.306 => Huawei

38.304 => CATT

37.340 => ZTE

### 8.4.2	Procedure and configuration of LP-WUS in RRC_IDLE/INACTIVE

Procedure and configuration of LP-WUS indicating paging monitoring triggered by LP-WUS, including at least configuration, sub-grouping and entry/exit condition for LP-WUS monitoring

Subgrouping

Related proposals

R2-2410841	Discussion on LP-WUS for IDLE/INACTIVE state	NTT DOCOMO, INC.	discussion	Rel-19

Noted

Observation 1.	It is up to SA2 to discuss whether the UE prevents from reporting UE capability for CN subgrouping ID feature when the UE has an emergency PDU session.

Proposal 3.	Discuss whether any solution is needed to minimize an additional delay to page a UE caused by subgrouping for LP-WUS.

DISCUSSION

- Xiaomi do not think any enh is needed, and think it is similar to the case when we discuss PEI.

- Ericsson also think the same topic has been discussed in PEI, and think we should follow the same approach. Ericsson assume CT1/SA2 will not handle this. Ericsson think in R2 we can further discuss the case of UE-ID based subgroup.

- Vodafone think wonders the case for UE ID based subgroup.

- CATT share the understanding of O1, for P3, CATT think it is valid to discuss this issue since LPWUS is different in the sense that it requires a large time period to switch to MR.

- Vivo think there seems to be common understanding for CN-based subgroup in R2 we do not need to discuss, and for UE-ID based there may be some solution to discuss. NEC agree to further discuss.

- Nokia think there is no need to further discuss solutions for UE-ID based subgrouping.

FFS whether/how to handle the case for UE-ID based subgrouping when the UE has an emergency PDU session.

R2-2410085	LP-WUS in Idle and Inactive	Ericsson	discussion	Rel-19	NR_LPWUS-Core	R2-2409058

Noted

Proposal 12	The UE does not monitor PEI after LP-WUS, when the UE supports both features and both features are configured in SIB.

Proposal 13	In case the UE and NW support LP-WUS and PEI, then only LP-WUS is used for paging.

DISCUSSION

- Interdigital think R1 already decided for the case that it is left to UE implementation whether to monitor PEI. ZTE, HW agree. QC agree and think the coverage of PEI and LPWUS signal may be different.

- LG E do not agree and think we cannot prevent UE from using both features, think it is useful to reduce false alarm if NW properly assign the subgroup IDs for both features. NEC agree.

Further discussions on the formula

R2-2410606	Procedure and Configuration of LP-WUS in RRC Idle Inactive Mode	Samsung	discussion	Rel-19

Noted

Proposal 2. LP WUS subgroup ID based on UE_ID is determined as follows:

LP WUS subgroup ID = (floor (UE_ID/(N*Ns)) mod LPWUSsubgroupsNumForUEID) +               (LPWUSsubgroupsNumPerPO - LPWUSsubgroupsNumForUEID)

-	LPWUSsubgroupsNumForUEID: number of LPWUS subgroups for UE_ID based LPWUS subgrouping in a PO, which is broadcasted in system information

-	LPWUSsubgroupsNumPerPO: number of LPWUS subgroups per PO, which is broadcasted in system information

R2-2409718	LP-WUS in RRC_IDLE/INACTIVE	CATT	discussion	Rel-19	NR_LPWUS-Core

Noted

Proposal 3: RAN2 discuss which option is adopted:

-	Option 1: The subgrouping number for UE_ID based PEI subgrouping is considered in the formula for UE_ID based LP-WUS subgrouping.

-	Option 2: The formula for UE_ID based PEI subgrouping is reused.

DISCUSSION on P3 in R2-2409718

- Xiaomi prefer O1, and think false alarm reduction is possible by assigning different subgroup IDs for PEI and LPWUS. NEC, CMCC agree.

- HW prefer fixed value for K in option 1.

- ZTE also support O1, and think O2 is a special case for O1.

- Interdigital slightly prefer O2 since it has less spec impact. Lenovo agree.

- NEC think there is efficiency issue with O2.

- LG E think with O2 it is also possible for NW to assign different subgroup IDs, and think O2 is simpler. Lenovo agree.

- Ericsson think if we support this then the formular cannot be exactly the same, and think we need a new formular. Vivo agree.

- CMCC want to have the flexibility of having different subgroups for both.

- QC not sure about the gain due to lack of evaluations, so think O2 the simpler way is preferred. Samsung agree. Samsung think there are evaluations in R1.

- Samsung think the O1 has the issue of UE id length.

- NEC think we can go with CATT P3 and in the next meeting we down-select.

FFS on the following options

- - Option 1: The subgrouping number for UE_ID based PEI subgrouping is considered in the formula for UE_ID based LP-WUS subgrouping.

- - Option 2 The subgrouping number for UE_ID LPWUS subgrouping includes an offset K which is configurable or fixed.

- - Option 3: The formula for UE_ID based PEI subgrouping is reused.

Agreements on subgrouping

FFS whether/how to handle the case for UE-ID based subgrouping when the UE has an emergency PDU session. FFS on the following options - - Option 1: The subgrouping number for UE_ID based PEI subgrouping is considered in the formula for UE_ID based LP-WUS subgrouping. - - Option 2 The subgrouping number for UE_ID LPWUS subgrouping includes an offset K which is configurable or fixed. - - Option 3: The formula for UE_ID based PEI subgrouping is reused.

Further discussions on R1 LS in R2-2407921/ R1-2407559

2 contributions below are moved from 8.4.1

R2-2409989	On LR and MR operating frequencies and the answer to RAN1 LS 	VODAFONE,VIVO, Deutsche Telekom	discussion	Rel-19

Noted

R2-2409990	DRAFT Reply LS to R2-2409157/R1-2407559	VODAFONE Group Plc	LS out	Rel-19	To:RAN Plenary, RAN1	Cc:RAN4

Noted

R2-2410670	Further considerations on LP-WUS operation in IDLE INACTIVE mode	CMCC	discussion	Rel-19	NR_LPWUS-Core

Noted

Proposal 4: From RAN2 perspective, UE may not support LP-WUS reception on all the bands that supported by the UE.

Proposal 5: If UE does not support LP-WUS reception on the frequency of MR, LP-WUS reception can be performed on the carrier supported by LR, while paging monitoring/measurement/access is performed on MR.

R2-2409949	Procedure and configuration of LP-WUS in RRC_IDLE/INACTIVE	Apple	discussion	Rel-19	NR_LPWUS-Core

Noted

Proposal 8: Send LS reply to RAN1 and RAN4 on the following identified issues:

-	For intra-frequency scenario, there may be congestion issue on LP-WUS;

But it can be mitigated if only LP-WUS capable UEs prioritize to camp on the LP-WUS cell.

-	For inter-frequency scenario, the following issues are identified:

o	The different DL timing between MR and LR causes the LP-WUS/LP-SS reception to malfunctions.

o	The different radio quality between MR and LR causes the serving measurement offloading/relaxation to malfunction.

o	Whether and how to support the associate the LP-WUS transmission on one frequency to PO in multiple cells needs to study.

Proposal 9: RAN2 LP-WUS discussion focuses on intra-frequency scenario, and the support of inter-frequency scenario is deprioritized in R19.

Proposal 10: Introduce the LP-WUS specific frequency priority for the cell reselection for the LP-WUS capable UE. (To address the potential congestion issue on LP-WUS frequency in scenario 1)

Proposal 11: RAN2 to consider the draft reply LS in Annex.

DISCUSSION

- VDF think R1 want to support the inter-freq case, so has doubt on Apple proposal to prioritize intra-freq case. Apple clarifies it does not mean we exclude inter-freq, but to start intra-freq as baseline.

- Rapp suggest in R2 we focus on how to reply the LS, e.g., we can discuss what are the issues with intra- and inter-freq case, and think it is for RP/R1 to make the decision. LG E agree.

- LG E think Apple contribution listed many R1/R4 aspects and think we should focus on R2 issues. Interdigital agree.

- Xiaomi think from R2 point of view we need to think about how to model the different cells on the different frequencies. Xiaomi prefer to confirm the R4 agreement to have intra-freq as baseline. Samsung, Interdigital, Lenovo agree.

- Samsung support understanding in Apple contribution. CATT share the view.

- ZTE think for inter-freq case UE should go back to MR band to receive paging.

- Qualcomm think inter-freq case impact a lot of things, so suggest to stick to intra-case. Ericsson has different understanding, and think for PEI UE can indicate which band(s) it support PEI.

- Ericsson not sure if CMCC proposal is feasible.

- VDF think in R1 LS they don’t not mention intra-freq and inter-freq cases, just different bands.

- VDF do not think having intra-freq as baseline is acceptable.

- Ericsson think we do not need priority specifically introduced for LPWUS UEs, and can rely on other mechanism.

- Rapp propose to discuss the issues if we do not support or support inter band operation of MR and LR.

[AT128][205][LPWUS] R2 impact analysis and draft reply LS on LP-WUS supporting in different bands (VODAFONE)

Scope: Discuss and align understanding on potential R2 impact if we do not support or support inter band operation of MR and LR. Can also discuss whether the LS is sent to RP as well.

Intended outcome: Summary in R2-2410955, Draft LS if available in R2-2410956

Deadline: Before CB

R2-2410955	Minutes of offline discussion: [AT128][205][LPWUS] R2 impact analysis and draft reply LS on LP-WUS supporting in different bands	VODAFONE	discussion	Rel-19	NR_LPWUS-Core

Noted

R2-2410956	DRAFT Reply LS to R2-2409157/R1-2407559	VODAFONE	LS out	Rel-19	NR_LPWUS-Core	To:RAN1, RAN4; Cc:RAN

Noted

DISCUSSION

- Ericsson do not see a need/purpose to cc RP, as there is no action given to RP. Samsung, ZTE, Interdigital, Xiaomi, Nokia agree. Xiaomi and Ericsson think we are not sure whether there is an issue to solve. CMCC do not want to CC RP and think R4 do not CC RP.

- Vodafone think we need to cc RP, want to inform RP about R2 status. HW, vivo ok to CC RP.

We will use the following to draft the LS. The LS is not CC to RP.

State of discussions in RAN WG2:

During RAN WG2 discussions, many companies raised concerns that if LP-Receivers support only subset of the bands supported by the main receiver and if LP-WUS, Paging and RACH are executed in the LP-WUS bands it might lead to overload of these bands if the NW set the LP-WUS frequency as a higher priority. This will impact user performance and decrease the utilisation of higher frequency bands. In order to ensure the LP-WUS capable UEs are camping on the band LR implemented, the NW would need to offload LP capable UEs to LR band.

On the other hand if the NW operator does not offload the UEs to the band LP receiver supports, only a subset of UEs would profit from LP-WUS feature.

Some companies do not see that overload problem would occur as the NW implementation can control the offload of the UEs to the lower bands in dedicated way.

In regards to the overload problem, RAN WG2 has not finished evaluations and would need to study the solutions more.

Actions:

RAN2 kindly asks RAN WG1 and WG4 to take the information above into account once deciding on the support of the use case mentioned above.

The LS is approved (unseen) in R2-2410957.

R2-2410957	Reply LS to R2-2407921/R1-2407559	RAN2	LS out	Rel-19	NR_LPWUS-Core	To:RAN1, RAN4

=> Approved

R2-2409718	LP-WUS in RRC_IDLE/INACTIVE	CATT	discussion	Rel-19	NR_LPWUS-Core

R2-2409761	Discussion on LP-WUS WUR in RRC_IDLE INACTIVE	vivo	discussion	Rel-19	NR_LPWUS-Core

R2-2409871	General considerations on the procedure for RRC_IDLE_INACTIVE	Xiaomi Communications	discussion

R2-2409902	LP-WUS in RRC_IDLE INACTIVE 	NEC	discussion	Rel-19	NR_LPWUS-Core

R2-2409921	Discussion on procedure and configuration of LP-WUS in RRC_IDLE/INACTIVE	Huawei, HiSilicon	discussion	Rel-19

R2-2409924	LP-WUS operation in RRC_IDLE and RRC_INACTIVE	LG Electronics Inc.	discussion	Rel-19	NR_LPWUS-Core

R2-2409949	Procedure and configuration of LP-WUS in RRC_IDLE/INACTIVE	Apple	discussion	Rel-19	NR_LPWUS-Core

R2-2410085	LP-WUS in Idle and Inactive	Ericsson	discussion	Rel-19	NR_LPWUS-Core	R2-2409058

R2-2410119	Discussion on LP-WUS procedure and configuration	OPPO	discussion	Rel-19	NR_LPWUS-Core

R2-2410166	Procedure and configuration of LP-WUS for IDLE and INACTIVE mode	ZTE Corporation, Sanechips	discussion	Rel-19	NR_LPWUS-Core

R2-2410377	RAN2 aspects on LP-WUS/WUR in RRC Idle/Inactive mode	Sony	discussion	Rel-19	NR_LPWUS-Core

R2-2410412	Discussion on IDLE/INACTIVE procedures for LP-WUS	Tejas Network Limited	discussion	Rel-19

R2-2410509	Discussion on LP-WUS operation in RRC_IDLE/INACTIVE modes	InterDigital, Inc.	discussion	Rel-19	NR_LPWUS-Core

R2-2410555	LP-WUS in IDLE and INACTIVE	Nokia	discussion	Rel-19	NR_LPWUS-Core

R2-2410606	Procedure and Configuration of LP-WUS in RRC Idle Inactive Mode	Samsung	discussion	Rel-19

R2-2410632	Discussion on LP-WUS in RRC_IDLE and RRC_INACTIVE	Sharp	discussion	Rel-19

R2-2410670	Further considerations on LP-WUS operation in IDLE INACTIVE mode	CMCC	discussion	Rel-19	NR_LPWUS-Core

R2-2410683	Discussion on LP-WUS in RRC_IDLE/INACTIVE	HONOR	discussion	Rel-19	NR_LPWUS-Core

R2-2410730	LP-WUS operation in IDLE/Inactive state	Qualcomm Incorporated	discussion	NR_LPWUS-Core

R2-2410798	Procedure and Configuration of LP-WUS in RRC Idle/ Inactive	Lenovo	discussion	NR_LPWUS-Core

R2-2410841	Discussion on LP-WUS for IDLE/INACTIVE state	NTT DOCOMO, INC.	discussion	Rel-19

R2-2410858	On LP-WUS paging monitoring considerations	Nordic Semiconductor	discussion	Rel-19	Late

### 8.4.3	RRM measurement relaxation and offloading in RRC_IDLE/INACTIVE

RRM relaxation of UE MR for both serving and neighbor cell measurements, and UE serving cell RRM measurement offloaded from MR to LP-WUR, including the necessary conditions

Serving and neighbor cell measurements relaxation

R2-2409925	RRM relaxation and RRM offloading	LG Electronics Inc.	discussion	Rel-19	NR_LPWUS-Core

Noted

Proposal 4	[For case #3] The entry condition for RRM relaxation is ‘if serving cell quality measured by MR is higher than relaxation threshold, e.g. RSRP and/or RSRQ’. The LR measurement is not considered.

Proposal 5	[For case #3] No separate exit condition for RRM relaxation. If the entry condition for RRM relaxation is not met, UE stops the RRM relaxation, i.e. performs legacy measurements.

Proposal 6	[For case #3] If the condition for RRM relaxation is met, UE performs serving cell measurements using only MR with relaxed requirement.

Proposal 7	[For case #3] If the condition for RRM relaxation is met, UE performs neighbor cell measurements using MR with relaxed requirement according to the existing measurement rules for cell re-selection.

Proposal 8	The condition for RRM offloading/relaxation is not associated with the conditions for using LP-WUS.

DISCUSSION

P4

- Ericsson not sure what is the motivation of R4 case #3. Ericsson think LPWUS UE has the same requirements for other UEs when it comes to neighbour cell measurement, and think LP-WUS entry/exit is not related to neighbour cell measurement.

- Nokia think it is difficult for NW to configure the threshold if we only allow MR-based metric, so prefer to also have LR based metric.

- Samsung want to clarify whether this is for new case, LG E think it is not new but it is case 3 in R4. Samsung ask whether for case 3 it means same condition for both serving and neighbour relx. LG E prefer the same condition. Samsung prefer to have different conditions for them.

- Apple think we only need single condition.

- OPPO think it is up to R4 whether the entry condition for serving cell RRM measurement relaxation is the same as neighbour cell RM measurement relaxation.

- HW agree with LG E. HW see no reason to separate the entry condition for serving cell RRM measurement relaxation and neighbour cell RM measurement relaxation.

- HW wonders how could the LR measurement be used for entry condition since UE may not have LR results before entering case 3.

P5

- CATT think we only agreed MR-based condition, and if we also have LR-based condition then we do need separate exit condition. CATT suggest to wait.

- Sony think it is meaningful to consider LR based condition. QC agree and think LR-based condition should be included.

- HW also think exit condition is based on LR.

The entry condition for serving cell RRM relaxation is at least ‘if serving cell quality measured by MR is higher than relaxation threshold, e.g. RSRP and/or RSRQ’. FFS if LR measurement is needed.

FFS on exit condition for serving cell RRM relaxation, e.g., whether a separate exit condition other than ‘not fulfilling the entry condition’ is needed, or whether exit condition include MR and/or LR-based measurements

FFS if the entry condition for serving cell RRM measurement relaxation is the same as neighbour cell RRM measurement relaxation.

R2-2410732	LP-WUS RRM measurement relaxation and offloading	Qualcomm Incorporated	discussion	NR_LPWUS-Core

Noted

Proposal 5	The entry condition for partially offloading:

-	When both MR and LR measurement are above the thresholds defined for partially offloading, and,

-	When any of LR and MR measurement is below the threshold which is defined for totally offloading

Proposal 6	The condition for leaving offloading mode (fully or partially):  when any of LR or MR measurement is below a threshold defined for no offloading, UE should leave offloading mode (fully or partially).

R2-2409592	Further discussion on the criteria for RRM measurement relaxation and offloading	Huawei, HiSilicon	discussion	Rel-19	NR_LPWUS-Core

Noted

Proposal 4: Separate threshold(s) should be introduced for RRM measurement relaxation (RAN4 case#3), compared to the threshold(s) of Rel-16 neighbor cell RRM measurement relaxation, i.e., these two features should be decupled.

Proposal 5: The criteria for entry/exit condition of RRM measurement relaxation is the same as the criteria for entry/exit condition of LP-WUS monitoring.

DISCUSSION

P5

- Samsung think these are different procedures so no need to have same condition. Samsung do not support P5. Nokia, Lenovo share this view.

- Apple agree with the intention since case 3 is when LR is ON.

- Xiaomi agree with P5 since there is no PS gain if UE only monitor LPWUS but not doing RRM relx. NEC, ZTE agree.

Agreements on RRM measurement relaxation

The entry condition for serving cell RRM relaxation is at least ‘if serving cell quality measured by MR is higher than relaxation threshold, e.g. RSRP and/or RSRQ’. FFS if LR measurement is needed.

FFS on exit condition for serving cell RRM relaxation, e.g., whether a separate exit condition other than ‘not fulfilling the entry condition’ is needed, or whether exit condition include MR and/or LR-based measurements

FFS if the entry condition for serving cell RRM measurement relaxation is the same as neighbour cell RRM measurement relaxation.

R2-2409592	Further discussion on the criteria for RRM measurement relaxation and offloading	Huawei, HiSilicon	discussion	Rel-19	NR_LPWUS-Core

R2-2409719	RRM Relaxation and Offloading in RRC_IDLE/INACTIVE	CATT	discussion	Rel-19	NR_LPWUS-Core

R2-2409762	Discussion on RRM measurement relaxation and offloading in RRC_IDLE/INACTIVE	vivo	discussion	Rel-19	NR_LPWUS-Core

R2-2409872	Discussion on RRM measurement relaxation for RRC_IDLE_INACTIVE	Xiaomi Communications	discussion

R2-2409903	LP-WUS measurement relaxation and offloading 	NEC	discussion	Rel-19	NR_LPWUS-Core

R2-2409925	RRM relaxation and RRM offloading	LG Electronics Inc.	discussion	Rel-19	NR_LPWUS-Core

R2-2409950	RRM measurement relaxation and offloading in RRC_IDLE/INACTIVE	Apple	discussion	Rel-19	NR_LPWUS-Core

R2-2410086	LP-WUS and RRM measurements	Ericsson	discussion	Rel-19	NR_LPWUS-Core	R2-2409059

R2-2410120	Discussion on RRM measurement in RRC IDLE and INACTIVE	OPPO	discussion	Rel-19	NR_LPWUS-Core

R2-2410167	RRM measurement relaxation and offloading in RRC_IDLE and RRC_INACTIVE mode	ZTE Corporation, Sanechips	discussion	Rel-19	NR_LPWUS-Core

R2-2410273	RRM measurement relaxation and offloading in RRC_IDLE/INACTIVE	Lenovo	discussion	Rel-19

R2-2410341	Discussion on RRM measurement relaxation and offloading in RRC_IDLE INACTIVE	CMCC	discussion	Rel-19	NR_LPWUS-Core

R2-2410378	Discussion on RRM aspects for  LP-WUS/WUR	Sony	discussion	Rel-19	NR_LPWUS-Core

R2-2410510	Discussion on RRM measurement relaxation and offloading	InterDigital, Inc.	discussion	Rel-19	NR_LPWUS-Core

R2-2410556	RRM measurement relaxation in RRC_IDLE/INACTIVE	Nokia	discussion	Rel-19	NR_LPWUS-Core

R2-2410607	RRM measurement relaxation and offloading in RRC Idle Inactive Mode	Samsung	discussion	Rel-19

R2-2410633	Discussion on RRM measurement relaxation and offloading	Sharp	discussion	Rel-19

R2-2410694	RRM measurement relaxation and offloading in RRC_IDLE/INACTIVE	China Telecom	discussion

R2-2410732	LP-WUS RRM measurement relaxation and offloading	Qualcomm Incorporated	discussion	NR_LPWUS-Core

### 8.4.4	Procedures for LP-WUS in RRC_CONNECTED

Procedures to allow UE MR PDCCH monitoring triggered by LP-WUS including activation and deactivation procedure of LP-WUS monitoring.

Option 1-2 (Handling of drx-onDurationTimer, periodic CSI/L1-RSRP reporting, etc.)

R2-2410405	LP-WUS in CONNECTED mode	InterDigital	discussion	Rel-19	NR_LPWUS-Core

Noted

Proposal 6: The drx-onDurationTimer is not started with Option 1-2 LP-WUS.

R2-2409883	LP-WUS in RRC_CONNECTED	Nokia, Nokia Shanghai Bell	discussion

Noted

Proposal 2: In option 1-2, drx-onDurationTimer is maintained.

Proposal 3: In option 1-2, the UE resumes start of the drx-onDurationTimer only after receiving a PDCCH while a newly introduced timer (entatively named drx-lpwusInactivityTimer) is running.

DISCUSSION

- Ericsson think we do not follow legacy, and this is behaviour with a new timer. Ericsson think CSI report behaviour can be configured. LG E think in R1 they agreed to introduce such configuration.

- Xiaomi do not think drx-onDurationTimer needs to be started. HW agrees, since we use new timer to control pdcch monitoring. LG E agree. OPPO agree.

- Apple think intention of R1 is to follow DCP way.

- QC think R1 already agreed drx-onDurationTimer is not started. Lenovo agree.

- Sony think we do not need two timer running at the same time and think since we use the new timer.

R2-2409588	Discussing on LP-WUS monitoring in Connected mode	Xiaomi	discussion	Rel-19	NR_LPWUS-Core

Noted

Proposal 3	For Option 1-2, network can configure whether UE reports periodic CSI/L1-RSRP during the time given by the configured drx-onDurationTimer.

Proposal 5	For option 1-2, if UE receives DRX command MAC CE or Long DRX command MAC CE, UE stops the new timer triggered by LP-WUS.

The drx-onDurationTimer is not started with Option 1-2 LP-WUS.

For Option 1-2, network can configure whether UE reports periodic CSI/L1-RSRP during the time given by the configured drx-onDurationTimer, for the case when UE is outside C-DRX active time.

P5

- NEC, Apple in general agree with P5.

- NEC want to confirm that whether Option 1-2 can be configured in case of short DRX. Nokia think this is separate discussion.

- Nokia, Interdigital, Lenovo agree.

For option 1-2, if UE receives DRX command MAC CE or Long DRX command MAC CE, UE stops the new timer triggered by LP-WUS.

Option 1-1 and 1-2

R2-2409904	LP-WUS in RRC_CONNECTED 	NEC	discussion	Rel-19	NR_LPWUS-Core

Noted

Proposal-3: support both Option 1-1 and Option 1-2 simultaneously configured for the same UE, i.e., LP-WUS can be used to trigger both new active timer and legacy drx-onDurationTimer, when the new active timer and legacy drx-onDurationTimer is running, the UE monitors PDCCH.

R2-2410634	Discussion on LP-WUS in RRC_CONNECTED	Sharp	discussion	Rel-19

Noted

Proposal 2: Don’t support Option 1-1 and Option 1-2 simultaneously configured for the same UE.

Don’t support Option 1-1 and Option 1-2 simultaneously configured for the same UE.

LP-WUS for CA and DC

R2-2409763	Discussion on LP-WUS WUR in RRC_Connected	vivo	discussion	Rel-19	NR_LPWUS-Core

Noted

Proposal 8: For both option 1-1 and option 1-2 in NR-DC case, LP-WUS is configured separately in MCG and SCG, and LP-WUS could trigger the PDCCH monitoring of all activated serving cells within the same cell group.

- Xiaomi fine with P8, think wording can improve. NEC fine with the proposal.

- Apple do not think for DC case NW can configured LPWUS for MCG and SCG at the same time, not ready to agree for DC case.

- LG E think DCP is not used for DC case, so prefer not to have LPWUS for DC case. Vivo think it can.

- QC wonders whether UE capability needs to be defined.

- For both option 1-1 and option 1-2

    - ?? The LP-WUS can also be configured with secondary per DRX group.

    - ?? In NR-DC case, LP-WUS can be configured separately in MCG and SCG, and LP-WUS could trigger the PDCCH monitoring of all activated serving cells within the same cell group.

R2-2410168	Procedures for LP-WUS in RRC_CONNECTED	ZTE Corporation, Sanechips	discussion	Rel-19	NR_LPWUS-Core

Noted

Proposal 3a: The LP-WUS can be configured with secondary DRX group simultaneously.

Proposal 3b: When LP-WUS is configured with secondary DRX Group simultaneously, down select from the following options with an LS to RAN1 to inform our decision:

	Option 1: one LP-WUS resource is configured per DRX group.

	Option 2: one LP-WUS resource is configured per cell group, with indication in LP-WUS to indicate which DRX group is triggered

Proposal 4a: In NR-DC, LP-WUS is configured for MCG and LCG separately.

Proposal 4b: LP-WUS can be configured for MCG in NE-DC case, i.e. the MN is a gNB, and LP-WUS can be configured for SCG in EN-DC case, i.e. the SN is a gNB.

Agreements on LP-WUS in RRC_CONNECTED

The drx-onDurationTimer is not started with Option 1-2 LP-WUS.

For Option 1-2, network can configure whether UE reports periodic CSI/L1-RSRP during the time given by the configured drx-onDurationTimer, for the case when UE is outside C-DRX active time.

For option 1-2, if UE receives DRX command MAC CE or Long DRX command MAC CE, UE stops the new timer triggered by LP-WUS.

Don’t support Option 1-1 and Option 1-2 simultaneously configured for the same UE.

R2-2409588	Discussing on LP-WUS monitoring in Connected mode	Xiaomi	discussion	Rel-19	NR_LPWUS-Core

R2-2409713	LP-WUS operation for RRC_CONNECTED Mode	LG Electronics Inc.	discussion	Rel-19	NR_LPWUS-Core

R2-2409720	Analysis on LP-WUS for RRC_CONNECTED Mode	CATT	discussion	Rel-19	NR_LPWUS-Core

R2-2409763	Discussion on LP-WUS WUR in RRC_Connected	vivo	discussion	Rel-19	NR_LPWUS-Core

R2-2409883	LP-WUS in RRC_CONNECTED	Nokia, Nokia Shanghai Bell	discussion

R2-2409904	LP-WUS in RRC_CONNECTED 	NEC	discussion	Rel-19	NR_LPWUS-Core

R2-2409951	Procedures for LP-WUS in RRC_CONNECTED	Apple	discussion	Rel-19	NR_LPWUS-Core

R2-2410087	LP-WUS in Connected	Ericsson	discussion	Rel-19	NR_LPWUS-Core	R2-2409060

R2-2410099	LP-WUS in CONNECTED mode	China Telecom	discussion	Rel-19	NR_LPWUS-Core

R2-2410121	Discussion on LP-WUS in RRC_CONNECTED	OPPO	discussion	Rel-19	NR_LPWUS-Core

R2-2410168	Procedures for LP-WUS in RRC_CONNECTED	ZTE Corporation, Sanechips	discussion	Rel-19	NR_LPWUS-Core

R2-2410319	Discussion on LP-WUS operation in CONNECTED mode	CMCC	discussion	Rel-19	NR_LPWUS-Core

R2-2410352	Discussion on LP-WUS for RRC_CONNECTED mode	Huawei, HiSilicon	discussion	Rel-19	NR_LPWUS-Core

R2-2410379	Considerations on LP-WUS/WUR in RRC Connected mode	Sony	discussion	Rel-19	NR_LPWUS-Core

R2-2410405	LP-WUS in CONNECTED mode	InterDigital	discussion	Rel-19	NR_LPWUS-Core

R2-2410413	Discussion on CONNECTED mode procedures for LP-WUS	Tejas Network Limited	discussion	Rel-19

R2-2410608	Procedures for LP-WUS in RRC Connected Mode	Samsung	discussion	Rel-19

R2-2410634	Discussion on LP-WUS in RRC_CONNECTED	Sharp	discussion	Rel-19

R2-2410731	LP-WUS operation in CONNECTED state	Qualcomm Incorporated	discussion	NR_LPWUS-Core

R2-2410814	LP-WUS in RRC Connected Mode	Lenovo	discussion	NR_LPWUS-Core

## 8.5	Network Energy Saving Enh.

(Netw_Energy_NR_enh-Core; leading WG: RAN1; REL-19; WID: RP-242354)

Time budget: 1 TU

Tdoc Limitation: 3 tdocs

### 8.5.1	Organizational

Including incoming LSs and rapporteur inputs.

R2-2409520	LS on SSB adaptation (R4-2416911; contact: Apple)	RAN4	LS in	Rel-19	Netw_Energy_NR_enh-Core	To:RAN2	Cc:RAN1

Noted.

R2-2409521	LS on SSB relation in On-demand SSB and SSB adaptation on Scell (R4-2416913; contact: Ericsson)	RAN4	LS in	Rel-19	Netw_Energy_NR_enh-Core	To:RAN1	Cc:RAN2

Noted.

### 8.5.2	On-demand SSB SCell operation

Remaining open issues on L3 measurement from RAN2#127b, including L3 measurement framework, whether always-on SSB and/or OD-SSB are measured in case 2, etc. Further details of OD-SSB MAC CE (dependent on RAN1 progress).

1. MAC CE

R2-2410255	On demand SSB handling	Nokia, Nokia Shanghai Bell	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 2: The different periodicities are configured as different OD-SSB configurations in RRC and MAC indicates which OD-SSB configuration ID to be activated, which is forward compatible if other parameters also introduced in RAN1.

Proposal 3: The new MAC-CE for OD-SSB includes a bitmap for the SCell(s) with OD-SSB to be activated and the indication of the activated SSB configuration ID for each SCell.

2. L3 RRM

R2-2409940	Discussion on on-demand SSB for SCell	Apple	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 5: Keep SMTC based L3 RRM framework for OD-SSB case 1 and case 2 (i.e. not allow UE autonomous SSB measurement outside of SMTC).

Proposal 6: For case 2, introduce dynamic adaptation between different SMTC configurations depending on whether OD-SSB is activated / deactivated. FFS how to configure OD-SSB dedicated SMTC configuration (e.g. in the same MO of always-on SSB or in an OD-SSB dedicated MO).

R2-2409596	Consideration on on-demand SSB SCell operation	CATT	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 1: The RRM measurement of OD-SSB can be performed based on OD-SSB pattern while ignoring SMTC when OD-SSB is transmitted.

R2-2409550	Discussion on On-Demand SSB	OPPO	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2409596	Consideration on on-demand SSB SCell operation	CATT	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2409696	On-demand SSB SCell Operation	Samsung	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2409926	On-demand SSB SCell operation	LG Electronics Inc.	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2409940	Discussion on on-demand SSB for SCell	Apple	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2410014	Discussion on on-demand SSB SCell operation	Sharp	discussion	Rel-19

R2-2410015	Discussion on on-demand SSB	Xiaomi	discussion

R2-2410061	Further discussion on On-demand SSB for SCell	NEC	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2410145	Discussion on on-demand SSB SCell operation	Fujitsu	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2410163	On-demand SSB SCell operation in connected mode	ZTE Corporation, Sanechips	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2410255	On demand SSB handling	Nokia, Nokia Shanghai Bell	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2410284	Issues on the procedure of on-demand SSB SCell operation	Lenovo	discussion	Rel-19

R2-2410320	Discussion on On demand SSB for Scell	CMCC	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2410380	On-demand SSB Scell operation discussion	Sony	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2410398	Discussion on on-demand SSB for NES	Ericsson	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2410426	Discussion on On-demand SSB SCell Operation	Qualcomm Incorporated	discussion

R2-2410432	Discussion on on-demand SSB SCell operation for NES	Huawei, HiSilicon	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2410472	On demand SSB transmission for SCell	InterDigital	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2410600	Discussion on on-demand SSB SCell operation	NTT DOCOMO INC.	discussion	Rel-19

R2-2410788	Remaining issues of on-demand SSB SCell operation	vivo	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2410849	Discussion on OD-SSB	Rakuten Mobile, Inc	discussion	Rel-19

### 8.5.3	On-demand SIB1

Remaining open issues or further details of OD-SIB1, e.g. access restriction, UE behaviours related to OD-SIB1 request and failure case, how to allow NES UE to reselect to cells that are prevented from legacy UEs, UL WUS configuration details (if anything is missed from RAN2 point of view), etc.

1. Access restriction for legacy UEs when SIB1 is not transmitted?

R2-2409941	Discussion on on-demand SIB1	Apple	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 1: RAN2 confirm that “no SIB1 indication” in MIB of NES cell can bar legacy UEs according to latest RAN1 agreements on K_SSB value, and thereby no separate barring bit is needed.

R2-2410256	On demand SIB1 handling	Nokia, Nokia Shanghai Bell	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 1: RAN2 should wait RAN1 to progress their understanding of legacy UE handling of 23< K_SSB <31 for FR1 and 11< K_SSB <15 for FR2

Discussion:

[CATT, Qualcomm]: It will be good to wait for further RAN1 progress.

Let’s wait for more RAN1 progress on Kssb discussion.

2. How to enable legacy UEs camping when SIB1 is transmitted?

R2-2410256	On demand SIB1 handling	Nokia, Nokia Shanghai Bell	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 7: NES UE with SIB1 request configuration of a NES cell assumes that a NES cell, with SSB containing K_SSB < 24 for FR1 and K_SSB < 12 for FR2, will acquire SIB1 as in legacy

NES UE with SIB1 request configuration of a NES cell assumes that a NES cell, with SSB containing K_SSB < 24 for FR1 and K_SSB < 12 for FR2, will acquire SIB1 as in legacy.

3. Excluded-cell list and NES dedicated reselection priority

R2-2410400	Discussion on on-demand SIB1 for NES	Ericsson	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 4 (modified): New NES-specific reselection priority parameters for NES UEs are defined for the purpose of prioritizing/deprioritizing a NES frequency.

Discussion:

[Lenovo]: Ask if this priority is per frequency or per cell. [Ericsson]: Confirms it can be considered for both. [Nokia]: With cell specific control, how to stop legacy UE from performing measurements on the NES cells? [Huawei]: Currently we do not have cell specific priority. We have cell specific offset, but it does not stop the legacy UE performing measurements on the corresponding cell.

[Nokia]: To minimize legacy UE impacts, NW may not assign the frequency priority to the legacy UEs (then the UE will not consider the frequency in cell reselection), but to NES UEs, the NW should be able to assign frequency priority (so NES UEs still consider the frequency in cell reselection).

[Lenovo]: We already have a UE dedicated cell reselection priority. Can we rely on that instead of introduction of cell reselection priority for NES UEs? [Huawei]: Do not think a scenario that the whole certain frequency is reserved for NES cells is valid. [Vodafone, Xiaomi, Lenovo]: Agree with Huawei, practically it should be per cell base. [Apple]: Think it is one of valid NW deployment scenarios. [Nokia]: We cannot sure if this scenario is not valid. Then we need to support some control for this case. [ZTE]: Understand some operators consider a dedicated carrier for NES cells.

New NES-specific reselection priority parameters for NES UEs are defined for the purpose of prioritizing/deprioritizing a NES frequency.

R2-2410303	Discussion on on-demand SIB1 operation for NES	Huawei, HiSilicon	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 7: Introduce new IntraFreqExcludedCellList-NES / InterFreqExcludedCellList-NES IEs enable proper reselection behaviour of legacy and NES UEs.

R2-2410146	Discussion on on-demand SIB1 procedure for NES	Fujitsu	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 10: The Rel-19 NES UE can consider the cell in the legacy excluded cell list as candidate, if the UE has the UL WUS configuration corresponding to the cell in the excluded cell list.

Discussion:

[Vivo, CATT]: Agree with Fujitsu proposal. [Xiaomi]: Prefer a solution based on explicit signalling. [Lenovo]: Think explicit signaling based option should be safer than implicit based option. UL-WUS configuration can be changed, e.g. NES cell can be changed to a normal cell, once the UE acquired UL-WUS configuration for the NES cell, but how does UE know whether that cell is still operates as NES cell or changed to a normal cell after some time? [Vodafone]: Assume Fujitsu proposal can reduce the signalling overheads and work well enough. [Nokia, Samsung, Qualcomm]: Prefer Huawei proposal. There may be NW intention to configure excluded cell list regardless of UL WUS configuration. It would be safer with explicit signaling based option. [Qualcomm]: With Huawei proposal, the benefit is that the UE does not need to decode all UL WUS configurations for this.

Introduce new IntraFreqExcludedCellList-NES / InterFreqExcludedCellList-NES IEs enable proper reselection behaviour of legacy and NES UEs.

4. Triggering condition for OD-SIB1

R2-2409941	Discussion on on-demand SIB1	Apple	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 5: Reuse legacy cell reselection criterion as trigger condition of OD-SIB1 acquisition. No need to specify other conditions (e.g. a new RSRP threshold).

Reuse legacy cell reselection criterion as trigger condition of OD-SIB1 acquisition. No need to specify other conditions (e.g. a new RSRP threshold).

R2-2409927	On-demand transmission of SIB1	LG Electronics Inc.	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 1 (modified): The existing 1-second rule in the cell reselection criteria is still applied to the triggering condition of UL WUS transmission.

The existing 1-second rule in the cell reselection criteria is still applied to the triggering condition of UL WUS transmission.

5. RACH Failure for OD-SIB1

R2-2410239	Consideration on on-demand SIB1	OPPO	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 1: Leave it to the UE implementation to determine whether the associated OD-SIB1 cell is considered barred in the case that the RA procedure fails related to the UE’s OD-SIB1 request, i.e. the UE may or may not consider this OD-SIB1 cell is barred.

R2-2410789	Discussion on  on-demand SIB1 for RRC IDLE and INACTIVE UE	vivo	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 10 (modified): The UE considers the cell as barred upon RACH failure for the OD-SIB1 request.

Discussion:

[CATT, Xiaomi, Huawei, CMCC, Apple, IDC, Lenovo, NEC]: Support Vivo proposal, to enable the UE attempt to find another suitable cell asap. [ZTE, Nokia, Vodafone, KDDI, Sony]: Support OPPO proposal and based on UE implementation, Vivo proposal can be also achieved. [Apple]: With Vivo proposal, the UE can be better reachable by NW. [LG]: The situation is similar to legacy UE, but there was no time restriction for the legacy UE. Also wonders if Vivo proposal needs legacy UE behaviour change. [Nokia]: Ok with Vivo proposal.

The UE considers the cell as barred after MAC indicates max number of preamble transmission for the OD-SIB1 request.

If MSG2 (ACK) is received, but UE fails to receive SIB1 then the UE may consider this cell as barred, no spec impact.

6. Whether to alleviate 300s restriction after NES UE acquires UL WUS configuration

R2-2410411	Barring and SIB1-less case 2	Vodafone, Deutsche Telekom, Xiaomi	discussion	Rel-19

Proposal (modified): It is proposed to clarify that a UE bars the NES/SIB1 less cell and/or excludes it as a candidate for reselection since the UE had no corresponding UL WUS configuration, the UE would treat this cell as if cell status is “not barred” and consider it as candidate for cell reselection once it has received a UL-WUS configuration to request SIB1 for this cell.

R2-2409597	Consideration on on-demand SIB1	CATT	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 5: If a NES cell is barred due to being unable to acquire the SIB1, the legacy access restriction is applied, i.e., UE may exclude the barred cell as a candidate for cell selection/reselection for up to 300 seconds.

Discussion:

[CATT]: Our intention is for NES UE to continue barring status on the cell in the scenario that Vodafone describes. [Nokia, Lenovo]: Thinks Vodafone proposal makes sense. [Vodafone]: Prefer to have common UE behaviours rather than leaving all to UE implementation. [LG]: Think ok with CATT proposal. Do not understand what problem is given to the operator if it is left to UE implementation. [Vodafone]: To minimize service interruption is an important factor to the operator.

A UE bars the NES/SIB1 less cell and/or excludes it as a candidate for reselection since the UE had no corresponding UL WUS configuration, the UE would treat this cell as if cell status is “not barred” and consider it as candidate for cell reselection once it has received a UL-WUS configuration to request SIB1 for this cell.

7. Validity of UL WUS configuration

R2-2410256	On demand SIB1 handling	Nokia, Nokia Shanghai Bell	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 11 (modified): UE considers WUS configuration only valid in directly succeeding cell reselection from cell A (the cell where UE acquired WUS configuration). FFS on SI validity area rather than cell level.

R2-2410469	Case 2 and remaining essential issues	Lenovo	discussion	Netw_Energy_NR_enh-Core

Proposal 1: WUS configuration includes a validity timer to control validity of the WUS configuration.

Proposal 2: Network configures the “remaining validity time” of WUS configuration as part of WUS configuration in the new agreed SIB.

Discussion:

[OPPO]: Understand legacy SIB validity check mechanism can work enough and it is aligned with Nokia proposal. [ZTE]: For Nokia proposal, when cell A provides WUS configurations for NES cell 1 and NES cell 2, and the UE moves to NES cell 1, does it mean WUS configuration acquired for cell 2 is not valid anymore once the UE moved on UES cell 1? [Nokia]: Confirms yes. [Huawei]: Agree with Nokia proposal.

[LG]: We already agreed WUS configuration is included in new SIB, then we can rely on the existing SIB validity check mechanism. However, Nokia proposal is applied when the SIB validity area is defined as per cell level. [Samsung]: Agree with Nokia proposal. However, we still can discuss further SI validity area level in stage 3.

[Lenovo]: Think WUS configuration will remain valid for long time, then it would be good to apply long timer. [Qualcomm]: In general, ok with Nokia proposal. [Xiaomi]: For RRC idle/inactive state, agree with LG. However, it’s not clear how to handle RRC connected UEs. [Vivo]: With the Lenovo proposal, it may bring risk, e.g. if the UE moves out of cell A coverage already, but still timer runs, the UE may use wrong WUS configuration. [Apple]: In addition, it would be difficult to maintain/update the timer. [Ericsson]: The Nokia proposal seems not so accurate, we need some rewording.

UE considers WUS configuration only valid in directly succeeding cell reselection from cell where UE acquired WUS configuration. FFS on SI validity area rather than cell level.

8. Handling of RRC connected UEs

R2-2409695	On-demand SIB1	Samsung	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 5: RAN2 to consider supporting SIB1 request in RRC_CONNECTED.

R2-2410303	Discussion on on-demand SIB1 operation for NES	Huawei, HiSilicon	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 8: Support RRC_CONNECTED UEs to trigger on-demand SIB1 acquisition by enhancing the legacy DedicatedSIBRequest or a new RRC message.

Discussion:

[Fujitsu, LG, ZTE]: It is not clear whether handling of RRC connected UEs is included in WID. [LG]: Understand the RRC connected UE can acquire SIB1 only when search space for SIB1 is configured. [CATT]: We can rely on option 2 in Samsung contribution, i.e. UE does not send SIB1 request. UE rely on network to provide the latest SIB1 via RRC message. Then we don’t need any spec change. [Huawei]: NW does not have any idea when the UE will trigger OD-SIB request or when the UE enters RLF. If we follow option2, unnecessary signalling overhead will be brought.

[LG]: Clarify previous LG’s comment was not to block any handling on RRC connected UEs. For SIB1 request, WID clearly indicates it is only for RRC idle/active, so SIB1 request is not applicable to RRC connected UE. However other options are still can be discussed/decided in RAN2. [Fujitsu]: Agree with LG.

Wait one cycle of RAN discussion (i.e. to see whether WID is updated or not to handle RRC connected UEs)

R2-2409580	Discussion on on-demand SIB1	Xiaomi	discussion	Rel-19

Proposal 17: The R19 UE will skip on-demand SIB1 cell when perform cell selection for RRC re-establishment.

R2-2410789	Discussion on  on-demand SIB1 for RRC IDLE and INACTIVE UE	vivo	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 11: RAN2 to discuss the following options for the RRC-CONNECTED UE acquiring the OSI message:

-	Option 1: The RRC-CONNECTED UE configured with searchSpaceOtherSystemInformation and searchSpaceSIB1 can send the DedicatedSIBRequest message without acquiring the NES cell’s SIB1 to check si-BroadcastStatus;

-	Option 2: All RRC-CONNECTED UEs are not configured with searchSpaceSIB1 and obtain the NES cell’s SIB1 by the dedicated RRC message, i.e. no spec impact and the legacy UE behavior is applied.

9. UL WUS configuration by RRC Release

R2-2410239	Consideration on on-demand SIB1	OPPO	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 4: The UL WUS configuration can also be provided via the RRCRelease message.

R2-2409580	Discussion on on-demand SIB1	Xiaomi	discussion	Rel-19

Proposal 3: RRCRelease message is not used to configure UL WUS configuration.

Discussion:

[Rakuten]: RRC release will be anyway used for transition from RRC connected to RRC idle/inactive, no additional RRC message is needed. Support OPPO proposal. [Nokia, Apple]: Support OPPO proposal otherwise the UE needs first move to the cell where provides the corresponding WUS configuration. [LG]: Think WUS configuration in RRC release is not needed. [Vivo, Fujitsu]: Support Xiaomi proposal. [Apple]: Do not see much specification efforts for RRC release.

[Samsung]: It is for the same scenario where it can be already covered by OD-SIB1 REQ. Why we need additional mechanism to handle the same scenario? [Nokia]: When the UE is released and if the UE has valid corresponding WUS configuration, we don’t need RRC release based option. However not sure if there are other cases where UE has not valid configuration. [IDC]: Agree with Samsung, we can count on proper NW configuration.

Working assumption: UL-WUS configuration in RRC release is not supported.

10. Others

R2-2410842	Considerations on OD-SIB1 feature	Deutsche Telekom, Vodafone, Lenovo, Fraunhofer IIS   	discussion	Rel-19

Proposal 1: Clarify that Cell A can also transmit the WUS configuration for itself in case it intends to become a SIB1-less cell.

Noted.

Discussion:

[IDC]: Specification will not specify cell A or NES cell, but since we already agreed NES cell can include its own WUS configuration, in principle agree with proposal 1. [Huawei]: Agree with IDC. [NTTDCM]: Having the clarification is good to other WGs since there are some confusion there.

[Qualcomm]: We should make sure the proposal 1 does not mean standalone case is supported. [Ericsson]: With the proposal, cell A needs to provide its own WUS configuration. [Nokia]: In addition, NES cell also needs to provide any neighboring cells’ WUS configuration (no restriction to neighboring cell’s NES cell). [Huawei]: We do not need to agree any further thing except what we already agreed. [Qualcomm]: Concerns on agreeing with the proposal. We already finished SI based on the terms and now we’re changing the terms. [Apple]: It was discussed in RAN1 and no conclusion was made. Would like to suggest to discuss in RAN.

Agreements on OD-SIB1

- NES UE with SIB1 request configuration of a NES cell assumes that a NES cell, with SSB containing K_SSB < 24 for FR1 and K_SSB < 12 for FR2, will acquire SIB1 as in legacy.

- New NES-specific reselection priority parameters for NES UEs are defined for the purpose of prioritizing/deprioritizing a NES frequency.

- Introduce new IntraFreqExcludedCellList-NES / InterFreqExcludedCellList-NES IEs enable proper reselection behaviour of legacy and NES UEs.

- Reuse legacy cell reselection criterion as trigger condition of OD-SIB1 acquisition. No need to specify other conditions (e.g. a new RSRP threshold).

- The existing 1-second rule in the cell reselection criteria is still applied to the triggering condition of UL WUS transmission.

- The UE considers the cell as barred after MAC indicates max number of preamble transmission for the OD-SIB1 request.

- If MSG2 (ACK) is received, but UE fails to receive SIB1 then the UE may consider this cell as barred, no spec impact.

- A UE bars the NES/SIB1 less cell and/or excludes it as a candidate for reselection since the UE had no corresponding UL WUS configuration, the UE would treat this cell as if cell status is “not barred” and consider it as candidate for cell reselection once it has received a UL-WUS configuration to request SIB1 for this cell.

- UE considers WUS configuration only valid in directly succeeding cell reselection from cell where UE acquired WUS configuration. FFS on SI validity area rather than cell level.

- Working assumption: UL-WUS configuration in RRC release is not supported.

R2-2409575	Discussion on on-demand SIB1	Xiaomi	discussion	Rel-19	Withdrawn

R2-2409580	Discussion on on-demand SIB1	Xiaomi	discussion	Rel-19

R2-2409597	Consideration on on-demand SIB1	CATT	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2409695	On-demand SIB1	Samsung	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2409820	Discussion on the issues for OD-SIB1	Google Ireland Limited	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2409927	On-demand transmission of SIB1	LG Electronics Inc.	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2409941	Discussion on on-demand SIB1	Apple	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2410009	Discussion on On-demand SIB1 WUS provisioning, UE behaviour, and barring	NEC Telecom MODUS Ltd.	discussion

R2-2410042	Discussion on Ondemand-SIB1	KDDI Corporation	discussion	Rel-19

R2-2410100	Discussion on OD-SIB1	China Telecom	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2410146	Discussion on on-demand SIB1 procedure for NES	Fujitsu	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2410164	Remaining issues of on-demand SIB1 in idle and inactive mode	ZTE Corporation, Sanechips	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2410170	On-demand SIB1 for Idle/Inactive mode UEs	III	discussion	Rel-19	Netw_Energy_NR_enh

R2-2410239	Consideration on on-demand SIB1	OPPO	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2410256	On demand SIB1 handling	Nokia, Nokia Shanghai Bell	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2410303	Discussion on on-demand SIB1 operation for NES	Huawei, HiSilicon	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2410321	Discussion on on-demand SIB1	CMCC	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2410381	WUS configuration details for on-demand SIB1	Sony	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2410391	Cell barring in on-demand SIB1 cells	Sony	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2410400	Discussion on on-demand SIB1 for NES	Ericsson	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2410411	Barring and SIB1-less case 2	Vodafone, Deutsche Telekom, Xiaomi	discussion	Rel-19

R2-2410427	Discussion on On-demand SIB1	Qualcomm Incorporated	discussion

R2-2410469	Case 2 and remaining essential issues	Lenovo	discussion	Netw_Energy_NR_enh-Core

R2-2410470	On-demand SIB1 request and reception	InterDigital	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2410602	Discussion on on-demand SIB1	NTT DOCOMO INC.	discussion	Rel-19

R2-2410738	Discussion on on-demand SIB1 for NES	Rakuten Mobile, Inc	discussion	Rel-19

R2-2410776	Discussion on On-demand SIB1 for NES	Fraunhofer IIS, Fraunhofer HHI	discussion	Rel-19

R2-2410789	Discussion on  on-demand SIB1 for RRC IDLE and INACTIVE UE	vivo	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2410842	Considerations on OD-SIB1 feature	Deutsche Telekom, Vodafone, Lenovo, Fraunhofer IIS   	discussion	Rel-19

### 8.5.4	Adaptation of common signal/channel transmissions

Further details of paging adaptation option-b, high-level discussion on SSB adaptation and RACH adaptation highlighting RAN2 spec impacts and RAN1 progress, etc.

1. SSB Adaptation

R2-2409679	Discussion on adaptation of common signal/channel transmissions	OPPO	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 1: R2 confirms that SSB adaptation in time domain for RRC_IDLE and RRC_INACTIVE UEs is not supported in Rel-19.

Proposal 2: R2 adopts a unified MAC CE design for both on-demand SSB transmission in SCells (i.e., NES O1) and SSB adaptation in time domain (i.e., NES O3), and wait for R1 for the applicability to PCell.

Proposal 3: R2 adopts a unified L3 RRM measurement design for both on-demand SSB transmission in SCells (i.e., NES O1) and SSB adaptation in time domain (i.e., NES O3).

R2-2410433	Discussion on adaptation of common signals/channels transmissions	Huawei, HiSilicon	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 5: A dedicated SMTC with larger periodicity can be configured to measure neighbour cells supporting dynamic SSB adaptation.

2. Paging Adaptation

[AT128][111][NES] (OPPO)

Scope: Discuss and suggest max number of N and Ns values, which is configurable by NW. Also discuss if there is problem with the current PEI.

Intended outcome: Discussion summary in R2-2410930. F2F offline discussion schedule will be announced by offline discussion rapporteur.

Deadline: Comeback in Thursday CB session.

R2-2410930	Offline discussion summary [AT128][111][NES]	OPPO	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal-1: Introduce value for Ns=8. FFS on 16.

Proposal-2: Introduce value for N= T/32. FFS on T/64, T/128, T/256.

Proposal-3: R2 further discuss to whether 1) based on a single PEI-O configuration, and rely on network implementation to have separate PEI:s for the two types of PO:s, or 2) based on separate PEI-O configurations, and thus automatically separate PEIs for the two types of PO:s. Where the two types of POs are 1) PO:s monitored by legacy UE (may or may not shared with R19 UE, up to network configuration), and 2) PO:s monitored only by R19 UE

Discussion:

[Apple]: Separate PEI configuration for NES UEs is quite aligned with what RAN2 previously agreed on separate PO configurations for NES UEs. [ZTE]: Think it can be handled by NW implementation by using the existing PEI configuration. Do not see a real need to introduce separate PEI configuration for NES UEs. [Xiaomi]: Prefer having no agreement at all this meeting. [Huawei]: Ok with proposal 3 in general, however some wording gives confusion, we need to clearly distinguishes three case. i) single PEI configuration with separate PEI transmission, ii) single PEI configuration with single PEI transmission and iii) separate PEI configuration. [OPPO]: The intention of offline discussion was to exclude the option ii). [Nokia]: It is early to exclude case ii) now.

Proposal-4: R2 assume to leave it to network implementation to set po-NumPerPEI and maxPEI-perPF properly, in order to be compatible with Ns=8 without further spec effort

[Huawei]: Think proposal 4 is also dependent on the conclusion of the issue indicated by proposal 3. [ZTE]: Share the same understanding as Huawei.

Introduce value for Ns=8. FFS on 16.

Introduce value for N= T/32. FFS on T/64, T/128, T/256.

RAN2 will revisit PEI configuration and PEI transmission aspects with the consideration of offline discussion in the next meeting.

Agreements on paging adaptation

- Introduce value for Ns=8. FFS on 16.

- Introduce value for N= T/32. FFS on T/64, T/128, T/256.

R2-2409576	Discussion on common signal adaptation	Xiaomi	discussion	Rel-19	Withdrawn

R2-2409581	Discussion on common signal adaptation	Xiaomi	discussion	Rel-19

R2-2409598	Adaptation of Common signal channel transmissions	CATT	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2409679	Discussion on adaptation of common signal/channel transmissions	OPPO	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2409692	Discussion on common signal and channel adaptation	LG Electronics Inc.	discussion	Rel-19	Netw_Energy_NR_enh

R2-2409697	Adaptation of common signal channel transmissions	Samsung	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2409841	Adaptation of common signal or channel	Fujitsu	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2409942	Discussion on common signal transmission adaptation	Apple	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2410010	PRACH and paging adaptation for NES	NEC Telecom MODUS Ltd.	discussion

R2-2410165	Consideration on common signal/channel transmissions	ZTE Corporation, Sanechips	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2410257	Common signal aspects of NES WI	Nokia, Nokia Shanghai Bell	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2410285	Discussion on the adaptation transmissions for NES operation	Lenovo	discussion	Rel-19

R2-2410322	Discussion on SSB adaptation	CMCC	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2410428	Discussion on Adaptation of Common Signal/Channel Transmissions	Qualcomm Incorporated	discussion

R2-2410433	Discussion on adaptation of common signals/channels transmissions	Huawei, HiSilicon	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2410471	Time domain adaptation of common signalling and channels	InterDigital	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2410610	Adaptation of common signal/channel transmissions for NES	Ericsson	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2410741	Adaptation of paging signal/channel transmissions	III	discussion

R2-2410743	Discussion on RACH adaptation	SHARP Corporation	discussion	Rel-19

R2-2410790	Discussion on adaptation of common signal transmissions	vivo	discussion	Rel-19	Netw_Energy_NR_enh-Core

## 8.6	Mobility Enhancement Ph4

(NR_Mob_Ph4-Core; leading WG: RAN2; REL-19; WID: RP-242356)

Time budget: 2 TU

Tdoc Limitation: 3 tdocs

### 8.6.1	Organizational

Including incoming LSs, WI rapporteur inputs, stage 2 running CR to be endorsed, etc.

R2-2409514	LS on the support of semi-persistent CSI-RS resource for LTM candidate cells (R1-2409283; contact: Fujitsu)	RAN1	LS in	Rel-19	NR_Mob_Ph4-Core	To:RAN3	Cc:RAN2

Noted.

R2-2409534	Reply LS on security handling for inter-CU LTM in non-DC cases (S3-244316; contact: Samsung)	SA3	LS in	Rel-19	NR_Mob_Ph4-Core	To:RAN2	Cc:RAN3

Noted.

[Lenovo]: We also included some other questions in the LS, but there is no SA3 response on them. [Apple]: Understand they have not finished all discussion. This LS is to inform the current status and to ask some question to RAN2.

R2-2409535	Reply LS on security key update of inter-CU SCG LTM (S3-244317; contact: Xiaomi)	SA3	LS in	Rel-19	NR_Mob_Ph4-Core	To:RAN2	Cc:RAN3

Noted.

R2-2409979	Introduction of NR mobility enhancements Phase 4 in TS 38.300	Apple	draftCR	Rel-19	38.300	18.3.0	B	NR_Mob_Ph4-Core

[POST128][105][MOB] (Apple)

Scope: Update 38.300 running CR capturing all RAN2 agreements.

Intended outcome: 38.300 running CR in R2-24010927, to be endorsed.

Deadline: Short email discussion

R2-2410927	Introduction of NR mobility enhancements Phase 4 in TS 38.300	Apple	draftCR	Rel-19	38.300	18.3.0	B	NR_Mob_Ph4-Core

=> Endorsed

R2-2410112	Introduction of NR mobility enhancements Phase 4 in TS 37.340	China Telecom	draftCR	Rel-19	37.340	18.3.0	B	NR_Mob_Ph4-Core

[POST128][106][MOB] (China Telecom)

Scope: Update 37.340 running CR capturing all RAN2 agreements.

Intended outcome: 37.340 running CR in R2-2410928, to be endorsed.

Deadline: Short email discussion

R2-2410928	Introduction of NR mobility enhancements Phase 4 in TS 37.340	China Telecom	draftCR	Rel-19	37.340	18.3.0	B	NR_Mob_Ph4-Core

=> Endorsed

[POST128][107][MOB] (Ericsson)

Scope: For inter-CU LTM and C-LTM parts, prepare 38.331 running CR capturing all RAN2 agreements, identify stage 3 issues (possibly with rapporteur’s suggestion) that need to be discussed, and discuss them.

Intended outcome: 38.331 running CR, to be endorsed next meeting, and discussion summary.

Deadline: Long email discussion

[POST128][108][MOB] (Huawei)

Scope: For L1 event-driven MR, prepare 38.331 running CR capturing all RAN2 agreements, identify stage 3 issues (possibly with rapporteur’s suggestion) that need to be discussed, and discuss them.

Intended outcome: 38.331 running CR, to be endorsed next meeting, and discussion summary.

Deadline: Long email discussion

[POST128][109][MOB] (Vivo)

Scope: Prepare 38.321 running CR capturing all RAN2 agreements, identify stage 3 issues (possibly with rapporteur’s suggestion) that need to be discussed, and discuss them.

Intended outcome: 38.321 running CR, to be endorsed next meeting, and discussion summary.

Deadline: Long email discussion

### 8.6.2	Inter-CU LTM

Remaining open issues or details for inter-CU LTM (SA) and inter-CU LTM in DC (including wether to support SCG addition when an inter-CU MCG LTM cell switch is executed, further scenario or issue clarification on the coexistence of intra-MN/inter-MN MCG LTM and inter-SN/intra-SN SCG LTM, R19 set ID for DC, etc.)

1. Security key update in non-DC case (related to SA3 LS: R2-2409534)

R2-2409593	Discussion on Inter-CU LTM	CATT	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 1: RAN2 excludes the solution where the UE is provided with the NCC value via RRC signalling upon every inter-CU LTM cell switch execution. RAN2 sends an LS to SA3 to inform this agreement.

R2-2410466	On remaining issues for Inter-CU LTM	Nokia	discussion

Proposal 9: RRC signalling (Option 4) is used to indicate the security key information update for inter-CU LTM.

R2-2410443	Dsicussion on Inter-CU LTM	ZTE Corporation	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 8: Reply to SA3 that RAN2 think avoiding RRC configuration/signaling between cell switches for subsequent LTM is important and should be respected in solution selection.

Discussion:

[Qualcomm]: Does CATT propose if NCC never be signalled between two inter-CU LTM cell switches, or if NCC can be signalled by RRC but not all the time. [CATT]: Suggest no NCC signalling by RRC for subsequent inter-CU LTM. [Rakuten]: Avoiding RRC signalling of NCC upon every inter-CU LTM cell switch execution is important. In SA3 LS, it seems they already identified two possible candidate solutions for it. Support CATT proposal. [Ericsson]: SA3 indicates option4 is baseline now and can consider further analysis if subsequent inter-CU LTM w/o RRC signalling is preferred, so it is not reasonable to exclude option4 now.

[Nokia, Huawei]: Main intention of subsequent LTM is to avoid reconfiguration of candidate cells, to configure just NCC between inter-CU LTM cell switches should be still ok. [Huawei]: NCC can be signalled as part of path switch, no additional delay is expected. [MediaTek]: For option4, it is not clear whether the signalled NCC is for the current LTM cell switch or for the next LTM cell switch.

[Apple]: It is not reasonable to directly discuss (or down-select) options in RAN2 side, but we should indicate subsequent inter-CU LTM w/o RRC signalling is aimed according to WID, and ask further analysis if feasible from the security point of view. Note we do not have enough time to spend this issue only. [NEC, ZTE, Samsung, IDC, LG, Xiaomi, Ericsson]: Agree with Apple. [Ericsson]: Also think it will be good to clarify what the subsequent inter-CU LTM means. On the definition of subsequent inter-CU LTM, we also agree with Nokia and Huawei. [Apple]: In addition, it would be good to indicate the time plan information, e.g. RAN2 expects the feedback until when TBD.

[Qualcomm]: If we include subsequent inter-CU LTM w/o RRC signalling, it would be also good to indicate two specific candidate solutions (MAC CE to deliver the NCC, NCC per candidate cell via RRC during LTM preparation phase) that was included in SA3 LS. [Session chair]: Based on further companies’ comments, it seems not possible to down-select to those two options right now.

Attempt to have RAN2 consensus on the definition of subsequent inter-CU LTM in offline and include it to reply LS. If failed, we will copy the WID aim that is written in the WID.

Reply to SA3 that RAN2 think avoiding RRC configuration between cell switches for subsequent inter-CU LTM is important and it needs to be respected in solution selection.

Include the time plan information to indicate until when their feedback is required or until when RAN2 needs to down-select the options. We assume in RAN2#130, to complete WI.

[AT128][115][MOB] (CATT)

Scope: Prepare response LS to SA3 (including the discussion on the detailed wordings and attempt to derive RAN2 consensus on the definition of subsequent inter-CU LTM).

Intended outcome: Response LS in R2-2411132, to be approved. F2F offline discussion schedule will be announced by CATT.

Deadline: Come back in CB session.

R2-2411132	Reply LS on security handling for inter-CU LTM in non-DC cases	LSout	Rel-19	NR_Mob_Ph4-Core	To:SA3

Approved in R2-2411136.

2. Fast recovery

R2-2410279	Remaining issues for Inter-CU LTM	Lenovo	discussion	Rel-19

Proposal 1: The Rel18 handling on failure is reused in R19 if the UE selects an intra-CU LTM candidate cell after detecting RLF of MCG as legacy.

R2-2409973	Important issues in Inter-CU LTM	Apple	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 2: RAN2 to postpone the discussion on fast recovery upon inter-CU LTM failure until inter-CU LTM security solution is finalized.

Discussion:

[Fujitsu]: Agree with Lenovo, it is what we already have in Rel-18. [Ericsson]: The issue is that the UE cannot know whether the selected cell is intra-CU from the serving cell point of view. It can be intra-CU cell that belongs to the different CU. Propose to wait for inter-CU LTM security solution. [MediaTek]: There is also another case for intra-CU LTM, e.g. after inter-CU LTM cell switch, the UE enters RLF. [Nokia]: Not clear if the proposal means if fast recovery upon inter-CU LTM failure.

RAN2 to postpone the discussion on fast recovery upon inter-CU LTM failure until inter-CU LTM security solution is finalized, if fast recovery is feasible.

3. Rel-19 Set ID in DC case

R2-2409973	Important issues in Inter-CU LTM	Apple	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 6 (modified): Rel-19 Set ID is configured for a candidate configuration. For DRB:

•	Inter-CU MCG LTM: When Rel-19 Set ID of candidate cell is different from serving cell,

o	UE performs PDCP re-establishment to all radio bearers.

•	Inter-CU SCG LTM: When Rel-19 Set ID of candidate cell is different from serving cell,

o	For MN terminated bearer without change of termination point, UE does not perform PDCP re-establishment. PDCP recovery is needed if it is split bearer.

o	For SN terminated bearer without change of termination point, UE performs PDCP re-establishment.

o	If there is change of termination point for a radio bearer (the keyToUse in the RadioBearerConfig is different from the keyToUse in the current UE configuration), UE performs PDCP re-establishment.

Discussion:

[Fujitsu]: Agree with Apple. Wonder if bearer’s termination point can be changed in inter-CU SCG LTM. [Apple]: Yes, it should be supported. [ZTE]: Agree with proposal. For MN terminated bearer, PDCP recovery can be applicable if it is split bearer. [OPPO]: Ask if the proposal is also applicable SRB. [Apple]: Original intention was to apply for both, but ok to discuss SRB in separate. [Samsung]: Do we still need PDCP reestablishment for SN terminated split bearer? [ZTE]: Confirms yes.

Rel-19 Set ID is configured for a candidate configuration. For DRB:

•	Inter-CU MCG LTM: When Rel-19 Set ID of candidate cell is different from serving cell,

o	UE performs PDCP re-establishment to all radio bearers.

•	Inter-CU SCG LTM: When Rel-19 Set ID of candidate cell is different from serving cell,

o	For MN terminated bearer without change of termination point, UE does not perform PDCP re-establishment. PDCP recovery is needed if it is split bearer.

o	For SN terminated bearer without change of termination point, UE performs PDCP re-establishment.

o	If there is change of termination point for a radio bearer (the keyToUse in the RadioBearerConfig is different from the keyToUse in the current UE configuration), UE performs PDCP re-establishment.

R2-2409764	Discussion on inter-CU LTM	vivo	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 13 (modified): For SRBs in inter-CU SCG LTM, Rel-19 ID is used to determine whether PDCP re-establishment or PDCP SDU discard is performed for LTM execution for SRB3. FFS on SRB1 and SRB2.

[Apple]: Ask if the proposal13 is only applied to inter-CU SCG LTM. [Vivo]: Confirm the proposal is for inter-CU SCG LTM. [CATT]: Understand L2 reset includes PDCP re-establishment, but how NW can know whether it is intra-CU LTM or inter-CU LTM in LTM preparation phase? [Lenovo]: The easiest option would be to follow Rel-15 principle, to re-establish all SRBs. In addition, not sure if CATT comment is correct. Understand how to set Rel-19 ID should be known to NW side.

For SRBs in inter-CU SCG LTM, Rel-19 ID is used to determine whether PDCP re-establishment or PDCP SDU discard is performed for LTM execution for SRB3.

Offline on SRB1 and SRB2 cases, Come back in CB session.

CB session:

[Vivo]: Seems companies have different understanding on SRB1 and SRB2 in SCPAC.

- Understanding1: NW indication (PDCP re-establishment flag) and the UE follows NW indication

- Understanding2: There are some cases where NW indication is not configured, then it is not clear how the UE behaviour is.

However, it is also questionable whether we will mimic SCPAC ways for inter-CU SCG LTM.

[Samsung]: Why not apply same approach to both DRB and SRB?

[CATT, IDC]: PDCP re-establishment flag does not work since NW does not know, whether intra-CU or inter-CU LTM happens, in LTM preparation phase. [OPPO]: It is clearly specified that SRB1/2 operates based on PDCP re-establishment flag. [Ericsson, Huawei]: NW can handle it by NW implementation. [Apple]: Ok with the proposal, however it is not clear whether the UE needs to check PDCP re-establishment flag or not? [IDC]: For inter-CU SCG LTM, the proposal is ok. For inter-CU MCG LTM, we may follow same approach as DRBs.

For SRB1/2 in inter-CU SCG LTM, PDCP re-established is not performed based on NW configuration (PDCP re-establishment flag and SDU discard)

4. Further clarification on inter-CU MN LTM in DC case

R2-2410443	Dsicussion on Inter-CU LTM	ZTE Corporation	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 3: RAN2 confirms that inter-CU MCG LTM with SCG addition is supported.

Discussion:

[Apple]: It is for RAN discussion. With the current WID, it is not within the scope. [Ericsson]: We agreed with SCG reconfiguration. Then it would be natural to support SCG addition. [Xiaomi]: Although there is not much RAN2 impact, there may be RAN3 impacts. [Samsung]: For this proposal, understand the only RAN2 impact is the reference configuration should be always configured for SN part. Is it correct?

[CATT]: Agree with proposal to avoid configuration mismatch. [Apple]: Think there will be some further spec impact and we do not have enough time for WI completion. Assume NW can achieve it with other ways. [Ericsson]: With this proposal, NW can better handle avoiding configuration mismatch issue. Otherwise NW will have super restriction. Also, do not think there is much specification efforts to support it. [LG, Nokia]: Agree with Ericsson. [Lenovo]: Not much specification effort is seen and it is up to NW when to use it dependent a scenario.

[Xiaomi]: What about intra-CU MCG LTM? If it is applied to inter-CU MCG LTM, shouldn’t it be also applicable to intra-CU MCG LTM? [Ericsson]: Assume so, but we can have that discussion in stage3. [Apple]: Ok with the proposal, but it is good to capture if we find much specification efforts later, we will not have it in the end. [Ericsson]: Accept with that condition.

RAN2 confirms that inter-CU MCG LTM with SCG addition is supported assuming no much specification effort is required. If there are much specification efforts, we will not have it.

R2-2410752	Further Considerations to Support Inter-CU LTM	Samsung	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 2-2: RAN2 confirms that the inter-CU MCG LTM with intra-SN PSCell change is supported in Rel19.

RAN2 confirms that the inter-CU MCG LTM with intra-SN PSCell change is supported in Rel19.

5. Co-existence of intra-CU LTM and inter-CU LTM

R2-2410443	Dsicussion on Inter-CU LTM	ZTE Corporation	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 4: RAN2 confirms the following coexistence cases in NR-DC are supported:

•	Case 1: Intra-CU MCG LTM + Inter-CU MCG LTM

•	Case 2: Intra-CU SCG LTM + Inter-CU SCG LTM

Proposal 5: In coexistence cases of inter-CU MCG/SCG LTM and intra-CU MCG/SCG LTM, when inter-CU MCG or SCG LTM is executed, it’s up to the NW to ensure that maintained LTM candidate configurations are valid, e.g. reconfigure or release invalid intra-CU MCG/SCG LTM candidate configurations.

Proposal 6: RAN2 to support intra-CU SCG LTM in MN format (i.e. MN RRCReconfiguration message), in addition to SN format, e.g. in order to avoid configuration mismatch problem and frequent RRC reconfiguration in coexistence cases of inter-CU MCG/SCG LTM and intra-CU SCG LTM.

Proposal 7: RAN2 to support intra-CU MCG LTM with SCG configuration, e.g. in order to avoid configuration mismatch problem and frequent RRC reconfiguration in coexistence cases of inter-CU MCG/SCG LTM and intra-CU MCG LTM.

6. Inter-CU SN LTM

R2-2409973	Important issues in Inter-CU LTM	Apple	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 5: Introduce a new LTM-Config (e.g., ltm-ConfigSCG-r19) for inter-CU SCG LTM.

R2-2410021	Discussion on inter-CU LTM	Xiaomi	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 10: A single ltm-Config is used to configure both intra-MN MCG LTM and inter-SN SCG LTM, and the unified LTM candidate ID pool is used for both.

R2-2409593	Discussion on Inter-CU LTM	CATT	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 7: It’s up to NW to ensure that the complete configuration includes the MCG part and SCG part configuration when UE combines the reference and candidate configuration for inter-CU SCG LTM.

R2-2409764	Discussion on inter-CU LTM	vivo	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 7: Source SN sends measurement results of candidate PSCells to the MN. The MN then forwards the measurement results to the candidate SN(s), and then the candidate SN(s) determines the LTM candidate cells based on the measurement results and the upper limit for the number of PSCells that can be prepared by each candidate SN.

Proposal 10: After acquiring the TA value via the CFRA triggered by the PDCCH order of the source SN, the candidate SN sends the TA value to the MN, and the MN then forwards the TA value to the source SN.

Proposal 10: Source SN notifies the MN after LTM cell switch is triggered, and the MN forwards the cell switch notification to the target SN.

[AT128][116][MOB] (ZTE)

Scope: Discuss remaining topics 5. co-existence of intra-CU LTM and inter-CU LTM, and 6. Inter-CU SN LTM, and attempt to derive conclusions.

Intended outcome: Discussion summary in R2-2411133. F2F offline discussion schedule will be announced by ZTE.

Deadline: Come back in CB session.

R2-2411133	[AT128][116][MOB] Inter-CU LTM (ZTE)	ZTE Corporation	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 1: From RAN2 perspective, the following coexistence cases in NR-DC can be supported:

•	Case 1: Intra-CU MCG LTM + Inter-CU MCG LTM

•	Case 2: Intra-CU SCG LTM + Inter-CU SCG LTM

Proposal 2: In coexistence cases of inter-CU MCG/SCG LTM and intra-CU MCG/SCG LTM, when inter-CU MCG or SCG LTM is executed, it’s up to the NW to ensure that maintained LTM candidate configurations are valid, e.g. reconfigure or release invalid intra-CU MCG/SCG LTM candidate configurations. UE does not autonomously release invalid intra-CU candidate configurations.

Proposal 3: RAN2 to support intra-CU SCG LTM in MN RRC message (i.e. MN RRCReconfiguration message), in addition to SN RRC message.

Proposal 4: RAN2 to support intra-CU MCG LTM with SCG configuration.

Proposal 5: It’s up to NW to ensure that the complete configuration includes the MCG part and SCG part configuration when UE combines the reference and candidate configuration for inter-CU SCG LTM.

Proposal 6: RAN2 assumes that how to indicate the list of candidate PSCells from source SN to MN is up to RAN3. From RAN2 perspective, in INM, source SN may send measurement results of candidate PSCells to the MN. The MN then forwards the measurement results to the candidate SN(s), and then the candidate SN(s) determines the LTM candidate cells based on the measurement results and the upper limit for the number of PSCells that can be prepared by each candidate SN. The existing IEs defined in INM can be reused as a baseline.

All proposals above are agreed.

Agreements on inter-CU LTM:

- Rel-19 Set ID is configured for a candidate configuration. For DRB:

- Inter-CU MCG LTM: When Rel-19 Set ID of candidate cell is different from serving cell,

> UE performs PDCP re-establishment to all radio bearers.

- Inter-CU SCG LTM: When Rel-19 Set ID of candidate cell is different from serving cell,

> For MN terminated bearer without change of termination point, UE does not perform PDCP re-establishment. PDCP recovery is needed if it is split bearer.

> For SN terminated bearer without change of termination point, UE performs PDCP re-establishment.

> If there is change of termination point for a radio bearer (the keyToUse in the RadioBearerConfig is different from the keyToUse in the current UE configuration), UE performs PDCP re-establishment.

- For SRBs in inter-CU SCG LTM, Rel-19 ID is used to determine whether PDCP re-establishment or PDCP SDU discard is performed for LTM execution for SRB3.

- For SRB1/2 in inter-CU SCG LTM, PDCP re-established is not performed based on NW configuration (PDCP re-establishment flag and SDU discard).

- RAN2 confirms that inter-CU MCG LTM with SCG addition is supported assuming no much specification effort is required. If there are much specification efforts, we will not have it.

- RAN2 confirms that the inter-CU MCG LTM with intra-SN PSCell change is supported in Rel19.

- From RAN2 perspective, the following coexistence cases in NR-DC can be supported:

- Case 1: Intra-CU MCG LTM + Inter-CU MCG LTM

- Case 2: Intra-CU SCG LTM + Inter-CU SCG LTM

7.	In coexistence cases of inter-CU MCG/SCG LTM and intra-CU MCG/SCG LTM, when inter-CU MCG or SCG LTM is executed, it’s up to the NW to ensure that maintained LTM candidate configurations are valid, e.g. reconfigure or release invalid intra-CU MCG/SCG LTM candidate configurations. UE does not autonomously release invalid intra-CU candidate configurations.

8.	RAN2 to support intra-CU SCG LTM in MN RRC message (i.e. MN RRCReconfiguration message), in addition to SN RRC message.

9.	RAN2 to support intra-CU MCG LTM with SCG configuration.

10.	It’s up to NW to ensure that the complete configuration includes the MCG part and SCG part configuration when UE combines the reference and candidate configuration for inter-CU SCG LTM.

11.	RAN2 assumes that how to indicate the list of candidate PSCells from source SN to MN is up to RAN3. From RAN2 perspective, in INM, source SN may send measurement results of candidate PSCells to the MN. The MN then forwards the measurement results to the candidate SN(s), and then the candidate SN(s) determines the LTM candidate cells based on the measurement results and the upper limit for the number of PSCells that can be prepared by each candidate SN. The existing IEs defined in INM can be reused as a baseline.

R2-2409593	Discussion on Inter-CU LTM	CATT	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2409616	Discussion on remaining issues of inter-CU LTM	LG Electronics Inc.	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2409764	Discussion on inter-CU LTM	vivo	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2409863	Discussion on Inter CU LTM 	Lekha Wireless Solutions	discussion	Rel-19

R2-2409873	Discussion on open issues for inter-CU LTM	OPPO	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2409886	Further discussion on remaining issues of inter-CU LTM cell switch	Transsion Holdings	discussion	Rel-19

R2-2409973	Important issues in Inter-CU LTM	Apple	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2409980	Important issues in Inter-CU LTM	Apple	discussion	Rel-19	NR_Mob_Ph4-Core	Withdrawn

R2-2410012	Further Discussion on inter -CU LTM	ETRI	discussion	Rel-19

R2-2410021	Discussion on inter-CU LTM	Xiaomi	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2410035	Discussion on inter-CU LTM in non-DC and DC cases	Fujitsu	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2410113	Discussion on inter-CU LTM	China Telecom	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2410118	Leftover issues on Inter-CU LTM	MediaTek Inc.	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2410228	Discussion on reference configuration for inter-CU LTM	ITRI	discussion	NR_Mob_Ph4-Core

R2-2410242	Discussion on inter-CU LTM	NEC	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2410279	Remaining issues for Inter-CU LTM	Lenovo	discussion	Rel-19

R2-2410323	Discussion on Inter-CU LTM	CMCC	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2410382	LTM for Inter-CU	Sony	discussion	Rel-19	NR_Mob_Ph4

R2-2410443	Dsicussion on Inter-CU LTM	ZTE Corporation	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2410466	On remaining issues for Inter-CU LTM	Nokia	discussion

R2-2410518	Inter-CU LTM	Huawei, HiSilicon	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2410530	Security handling for Inter-CU LTM	Qualcomm Incorporated	discussion

R2-2410544	Security handling and DC aspects for inter-CU LTM	Ericsson	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2410598	Discussion on Inter-CU LTM	InterDigital, Inc.	discussion	Rel-19

R2-2410660	Potential issue on coexistence of inter-MN LTM and intra-SN LTM 	Kyocera 	discussion	Rel-19

R2-2410690	Further discussion on inter-CU LTM	HONOR	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2410703	Discussion on issues for supporting inter-CU LTM	Sharp	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2410742	Discussion on Inter-CU LTM	Rakuten Mobile, Inc	discussion	Rel-19

R2-2410752	Further Considerations to Support Inter-CU LTM	Samsung	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2410763	LTM in DC scenarios	Rakuten Mobile, Inc	discussion

R2-2410856	Discussion on inter-CU LTM	DENSO CORPORATION	discussion	Rel-19	NR_Mob_Ph4-Core

### 8.6.3	L1 event triggered measurement reporting

Remaining open issues or details for L1 event triggered measurement reporting (including TTT operation, e.g. granularity of TTT operation for a candidate cell, whether to reset TTT on current beam changing, measurement RS type alignment, more details of MR MAC CE, e.g. whether N beams should satisfy the event or not, beam identification, etc.)

1. TTT Operation for candidate beam

R2-2409630	Event LTM triggered measurement report	LG Electronics Inc.	discussion	NR_Mob_Ph4-Core

Proposal 1: TTT is evaluated per beam, and measurement report is only triggered by beam that has satisfied the condition (entering/leaving) for the whole duration of TTT.

R2-2410444	Discussion on L1 event triggered measurement reporting	ZTE Corporation	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 3: For the LTM event evaluation, TTT is evaluated per cell, i.e. not per beam or per resource set. The TTT shall keep running as long as at least one beam of the cell meets the entering condition of the LTM event.

R2-2410153	Discussion on L1 event-triggered measurement reporting	Huawei, HiSilicon	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 3: TTT timer is maintained per report Config.

Discussion:

[Nokia]: From NW point of view, beam level knowledge is very important. Prefer beam level TTT operation, however Huawei’s solution may be acceptable. [Samsung]: Agree with Huawei solution. [Vivo]: Prefer beam level TTT operation, however also Huawei’s solution seems acceptable. [Ericsson]: Agree with ZTE but Huawei’s solution seems acceptable. [Xiaomi]: Also accept Huawei solution. [Session chair]: Then can we go to Huawei solution?

[Nokia]: After further thinking, with the Huawei’s solution, it will make UE complicated and probably for RAN4 works as well if NW configures multiple beams per cell in the same resource set. [Apple]: According to the current spec, multiple beams from the different cells can be included in the same resource set, then it would actually become TTT per cell, which is not aligned with the previous RAN2 agreement, unless we define a kind of NW restriction on resource set configuration. [Qualcomm, LG, MediaTek]: Agree with Apple. [Ericsson]: Resource set was already there in Rel-18 so NW restriction for the resource set is not acceptable. [IDC]: A NW demand can be different, per NW configuration makes sense. [Ericsson]: TTT operation per beam is acceptable, but it will be good if the UE includes more beams in MR.

TTT is evaluated per beam, and measurement report is only triggered by beam that has satisfied the condition (entering/leaving) for the whole duration of TTT.

2. TTT Operation for the current beam

R2-2410153	Discussion on L1 event-triggered measurement reporting	Huawei, HiSilicon	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 1: TTT timer is not restarted if the current beam changes and the entering condition is still met with the new current beam.

R2-2410180	Discussion on L1 measurement reporting for LTM	ASUSTeK	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 1: In response to a current beam change of a Serving Cell, TTT associated with trigger event LTM2, LTM3, and LTM5 should be is reset.

Discussion:

[LG, Nokia]: Huawei solution is acceptable if majority companies support it.

TTT timer is not restarted if the current beam changes and the entering condition is still met with the new current beam.

3. Whether TTT is applied to a leaving condition

R2-2409842	Discussions on event triggered L1 measurement reporting	Fujitsu	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 4 (modified): TTT is applied to the leaving condition.

Discussion:

[Ericsson]: Proposal doesn’t make sense. Also, TTT is not applied for L3 MR. [CATT, MediaTek]: Understand TTT is also applicable for the leaving condition in L3 MR. [Nokia]: Ok with proposal 4 regardless of whether it is supported in L3 MR. [LG, CATT, MediaTek]: Agree with the proposal 4.

TTT is applied to the leaving condition.

4. Configuration of measurement RS type

R2-2410444	Discussion on L1 event triggered measurement reporting	ZTE Corporation	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 2 (modified): Network can configure which RS type (SSB or CSI-RS) is used for LTM event evaluation.

[Apple, LG, CATT]: Support the proposal. [Ericsson]: Ok with the proposal, but the detailed location where to put this IE is the stage 3 issue.

Network can configure which RS type (SSB or CSI-RS) is used for LTM event evaluation.

5. Alignment of measurement RS type

R2-2409765	Discussion on LTM measurement event evaluation and reporting	vivo	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 3 (modified): Either CSI-RS or SSB could be configured as candidate beam and the measurement RS of the serving cell beam is determined based on the candidate beam to ensure same RS type, i.e. the RS for current beam of serving cell is same as or QCLed with the QCL RS of the indicated TCI state, according to what is agreed in RAN1.

Discussion:

[Session chair]: RAN1 had similar discussion and understand proposal 3 is aligned with RAN1 decision. [Vivo]: Confirm the session chair understanding. [CATT]: Do we need to consider a configuration both CSI-RS and SSB are allowed for candidate beam? [Apple]: Do not understand how a candidate cell is configured for both CSI-RS and SSB. [CATT]: We already have similar configuration for the serving cell. [Session chair]: That might be introduced for MIMO purpose. It is not clear whether it also needs to be configured for LTM candidate measurement purpose.

Either CSI-RS or SSB could be configured as candidate beam and the measurement RS of the serving cell beam is determined based on the candidate beam to ensure same RS type, i.e. the RS for current beam of serving cell is same as or QCLed with the QCL RS of the indicated TCI state, according to what is agreed in RAN1.

6. Beam identification in MR MAC CE

R2-2409952	LTM event triggered measurement reporting	Apple	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 10 (modified): SSBRI and CRI is used to represent the candidate beam ID in LTM MR MAC CE.

R2-2410545	Event definition and MAC CE content for L1 event-triggered measurements	Ericsson	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 9: The combination of reference signal ID plus the LTM candidate configuration ID is reported within the event-triggered L1 measurement report MAC CE report.

Discussion:

[Session chair]: Any big difference between two options in the signaling overhead point of view? Or any real technical blocking issue for one or another? [Apple]: No, both options can work. Seems a matter of taste. [Session chair]: Then better to keep what is already supported in Rel-18 unless many companies want a new one. [Xiaomi]: SSBRI and CRI is applied to a candidate beam. Wonders how to indicate the serving beam, so proposal needs to be limited to a candidate beam ID now.

SSBRI and CRI is used to represent the candidate beam ID in LTM MR MAC CE.

7. Candidate beams included in MR MAC CE

R2-2409952	LTM event triggered measurement reporting	Apple	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 9: Only the candidate beams that satisfy the event criteria are included in LTM MR MAC CE.

R2-2410114	Discussion on L1 event triggered measurement reporting	China Telecom	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 5: For event-triggered L1 LTM measurement reporting, the beam(s) not satisfying the event cloud be reported in MR MAC CE, which is configured by the NW.

Discussion:

[Nokia, CATT, MediaTek, Huawei, Vivo, Rakuten]: Do not see the need of other beams’ measured results if they do not meet event. Agree with Apple. [Vivo]: If NW want to know more beam information, NW can configure another MR event(s) with lower threshold. [Panasonic]: If NW want to know more beam information, NW can configure periodic or semi-static MRs.

[Xiaomi]: With China Telecom proposal, the NW can have better understanding on cell level radio condition and it can be used to avoid ping-pong. [Ericsson]: Agree with Xiaomi. [OPPO]: Support China Telecom proposal. We can also use the beams for deactivation purpose. [Samsung, Qualcomm]: China Telecom proposal will bring more flexibilities to NW. Otherwise NW needs to configure more events, which brings more signalling overheads. [Qualcomm]: When there is enough grant, why not report more beam information? [LG]: With China Telecom proposal, NW can be better aware of more beams in the mobility decision.

[Session chair]: China Telecom proposal is based on NW configuration. Then can Apple proposal be also achieved based on NW configuration? [Nokia]: Configurable may be acceptable, but want to avoid a situation when the UE has enough grant to include beam information that fulfils with the events, but not enough to include all N beams, which may delay the whole MR. [LG]: We need to define minimum size of MR MAC CE. [Xiaomi]: Assuming we will have a truncated MR MAC CE, it should be no problem.

For event-triggered L1 LTM measurement reporting, max N is total number of all beams included into MR MAC CE.

For event-triggered L1 LTM measurement reporting, NW controls if the beam(s) not satisfying the event could be reported according to N beams in MR MAC CE.

8. MR MAC CE Format

R2-2410545	Event definition and MAC CE content for L1 event-triggered measurements	Ericsson	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 8	A single MAC CE format for the event-triggered L1 measurement report is used for both the SSB and CSI-RS reference signals.

A single MAC CE format for the event-triggered L1 measurement report is used for both the SSB and CSI-RS reference signals.

R2-2410441	On L1 Measurement Reporting Enhancements for Rel-19 LTM	Nokia	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 18: RAN2 to study the RSRP value reporting using differential RSRP mechanism in MAC CE.

Discussion:

[Samsung, Xiaomi, Lenovo]: Since we have MR MAC CE, it would be easier and more straightforward to include all explicit values (rather than using differential values). [Apple]: It is already supported in Rel-18. [IDC]: Agree with the proposal, it would be good to optimize the MR MAC CE size. [CATT]: Agree with proposal. [Huawei]: In general, ok with the proposal. Wonder if there needs any RAN4 impact for mapping. [Lenovo]: It is an optimization and not sure how many bits can be saved compared to the case using all explicit values.

Revisit it once we can have better understanding on MR MAC CE format and how much bits actually can be saved from differential RSRP values.

R2-2410153	Discussion on L1 event-triggered measurement reporting	Huawei, HiSilicon	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 7: Support the truncated measurement report MAC CE.

Support the truncated measurement report MAC CE.

Agreements on L1 event-driven MR:

- TTT is evaluated per beam, and measurement report is only triggered by beam that has satisfied the condition (entering/leaving) for the whole duration of TTT.

- TTT timer is not restarted if the current beam changes and the entering condition is still met with the new current beam.

- TTT is applied to the leaving condition.

- Network can configure which RS type (SSB or CSI-RS) is used for LTM event evaluation.

- Either CSI-RS or SSB could be configured as candidate beam and the measurement RS of the serving cell beam is determined based on the candidate beam to ensure same RS type, i.e. the RS for current beam of serving cell is same as or QCLed with the QCL RS of the indicated TCI state, according to what is agreed in RAN1.

- SSBRI and CRI is used to represent the candidate beam ID in LTM MR MAC CE.

- For event-triggered L1 LTM measurement reporting, max N is total number of all beams included into MR MAC CE.

- For event-triggered L1 LTM measurement reporting, NW controls if the beam(s) not satisfying the event could be reported according to N beams in MR MAC CE.

- A single MAC CE format for the event-triggered L1 measurement report is used for both the SSB and CSI-RS reference signals.

- Support the truncated measurement report MAC CE.

R2-2409594	L1 event triggered measurement reporting	CATT	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2409630	Event LTM triggered measurement report	LG Electronics Inc.	discussion	NR_Mob_Ph4-Core

R2-2409657	Discussion on event triggered L1 MR	MediaTek Inc.	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2409659	Remaining issues of L1 event triggered measurement reporting	Xiaomi	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2409765	Discussion on LTM measurement event evaluation and reporting	vivo	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2409802	Event triggered L1 measurement reporting for LTM.	Interdigital, Inc.	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2409842	Discussions on event triggered L1 measurement reporting	Fujitsu	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2409874	Open issues for event triggered  L1 measurement reporting	OPPO	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2409887	Discussion on L1 event triggered measurement reporting	Transsion Holdings	discussion	Rel-19

R2-2409952	LTM event triggered measurement reporting	Apple	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2409987	L1 event-triggered measurement reporting for LTM	Qualcomm Incorporated	discussion

R2-2410062	Further details of L1 event triggered measurement reporting	NEC	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2410114	Discussion on L1 event triggered measurement reporting	China Telecom	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2410153	Discussion on L1 event-triggered measurement reporting	Huawei, HiSilicon	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2410180	Discussion on L1 measurement reporting for LTM	ASUSTeK	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2410244	Discussion on event triggered L1 report	Huawei, HiSilicon	discussion	Rel-19	NR_Mob_Ph4-Core	Withdrawn

R2-2410280	Event based L1 measurement report	Lenovo	discussion	Rel-19

R2-2410306	L1 measurement event configuration and reporting	Panasonic	discussion	Rel-19

R2-2410340	Discussion on L1 event triggered measurement reporting	CMCC	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2410399	Discussion on L1 event triggered measurement reporting 	Rakuten Mobile, Inc	discussion	Rel-19

R2-2410441	On L1 Measurement Reporting Enhancements for Rel-19 LTM	Nokia	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2410444	Discussion on L1 event triggered measurement reporting	ZTE Corporation	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2410545	Event definition and MAC CE content for L1 event-triggered measurements	Ericsson	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2410567	Discussion on L1 event triggered measurement	Jio	discussion

R2-2410571	Open issues for L1 event triggered measurement reporting	Fraunhofer HHI, Fraunhofer IIS	discussion

R2-2410621	Support of Event Triggered L1 Measurement Report	Samsung	discussion	Rel-19	NR_Mob_Ph4-Core

=> Revised in R2-2410888

R2-2410888	Support of Event Triggered L1 Measurement Report	Samsung	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2410648	Details of event-triggered L1 measurement reporting for LTM 	Kyocera 	discussion	Rel-19

R2-2410663	Discussion on LTM measurement reporting configuration	Baicells	discussion

R2-2410688	Discussion on measurement event evaluation and report	HONOR	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2410702	L1 event triggered measurement reporting	Sharp	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2410708	Discussion on L1 event triggered measurement reporting	KDDI Corporation	discussion	Rel-19

R2-2410710	Discussion on event triggered L1 measurement reporting	ITL	discussion	Rel-19	NR_Mob_Ph4

### 8.6.4	Conditional intra-CU LTM

Further details of each phase (C-LTM preparation, early sync, evaluation and execution, and completion phases), highlighting what new delta should be really required compared to LTM (e.g. why LTM way cannot be also applied, etc.)

1. Additional C-LTM execution condition(s)

R2-2409658	Further discussion on Conditional LTM	MediaTek Inc.	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 6 (modified): The triggering condition of conditional LTM can be based on L3 measurement.

Proposal 7 (modified): CondEventA3 and CondEventA5 conditions can be baseline for the conditional LTM execution.

R2-2409595	Discussion on Conditional Intra-CU LTM	CATT	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 1a: CHO conditions are not used as conditional LTM execution condition.

Proposal 1b: Conditional LTM is only based on L1 measurement (i.e., L3 measurement-based conditional LTM is not supported). RAN2 sends LS to RAN4 to inform this agreement.

Discussion:

[Lenovo]: Support both proposals from MediaTek. L3 measurement report would be useful to C-LTM case. [Samsung]: Assuming CondEventA3 and CondEventA5 is configured, what is benefit of using C-LTM instead of CHO? [Lenovo]: Understand previous RAN4 indicated that for L1 measurement configuration, the UE has been already configured for L3 measurements. [Huawei]: Agree with Lenovo. [Apple]: Also, C-LTM supports subsequent C-LTM while CHO cannot do that.

[Nokia]: Prefer keeping “CondEventA3-like” and “CondEventA5-like” otherwise it already means cell level execution evaluation is used for C-LTM. [Qualcomm]: To make simpler, prefer not to have “like” wording now. [Ericsson]: Consider existing CHO conditions can be baseline, but it may not be exactly same as the existing CHO conditions if needed.

The triggering condition of conditional LTM can be based on L3 measurement.

CondEventA3 and CondEventA5 conditions can be baseline for the conditional LTM execution.

[CATT]: Understand RAN4 waits for RAN2 decision regarding whether C-LTM can be supported based on L3 measurement or not. It would be good to send LS to RAN4 to inform the decision. [Xiaomi]: RAN4 just takes RAN2 agreements in the session minutes into account. Do not see a real need to send LS. [MediaTek]: Understand RAN4 already has discussed it and now waiting for RAN2 decision, support CATT suggestion. Also, it will be good to include RAN3 for possible RAN3 impacts.

[POST128][117][MOB] (Lenovo)

Scope: Prepare LS to RAN4 and RAN3 to take the RAN2 agreements (including C-LTM based on L3 measurements, and possible other agreements from inter-CU LTM and L1-event driven MR) into account for their jobs.

Intended outcome: LS in R2-2411134, to be approved.

Deadline: Short email discussion

R2-2411134	RAN2 agreements on Inter-CU LTM and Conditional LTM	RAN2	LS out	Rel-19	NR_Mob_Ph4-Core	To: RAN1, RAN3, RAN4

=> Approved

2. Number of triggering events for C-LTM execution

R2-2410445	Discussion on conditional intra-CU LTM	ZTE Corporation	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 3: For conditional LTM, an execution condition may consist of one or two triggering condition(s) (e.g. Event LTM3/5 like or CondEvent A3/5). If there are two triggering conditions associated with the same candidate cell, the UE shall consider the execution condition is fulfilled only when both triggering conditions are met.

Proposal 4: For conditional LTM, only single RS type is supported and at most two different trigger quantities can be configured simultaneously for the evaluation of execution condition of a single candidate cell.

R2-2409595	Discussion on Conditional Intra-CU LTM	CATT	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 2 (modified): The L1 execution condition of a candidate cell is associated to only one triggering event. It can be revisited if RAN1 decides to support more quantities (e.g., L1-SINR or L1-RSRQ) later.

The L1 execution condition of a candidate cell is associated to only one triggering event.

For L3 execution condition, it may consist of one or two triggering condition(s). If there are two triggering conditions associated with the same candidate cell, the UE shall consider the execution condition is fulfilled only when both triggering conditions are met. Only single RS type is supported and at most two different trigger quantities can be configured simultaneously for the evaluation of execution condition of a single candidate cell.

3. Node to provide C-LTM execution condition(s)

R2-2410022	Discussion on conditional LTM	Xiaomi	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 5 (modified): To support initial and subsequent conditional LTM, the following items can be considered for the configuration of execution condition:

	The CLTM configuration of each candidate cell shall include the execution condition for initial conditional LTM, which is generated by the initial source cell to trigger the CLTM for the candidate cell.

	The CLTM configuration of each candidate cell may include execution conditions for subsequent conditional LTM, which is generated by the candidate cell to trigger the CLTM for other candidate cells when the candidate cell becomes a serving cell.

Agreed.

4. Early UL/DL sync procedure: General

R2-2410519	Intra-CU conditional LTM	Huawei, HiSilicon	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 3: The network can configure measurement reports e.g., L1 periodic, semi-persistent, aperiodic and event triggered report, or L3 measurement reports for conditional LTM, e.g., to trigger PDCCH ordered early RACH.

R2-2410701	Discussion on conditional LTM	Sharp	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 7. A cell switch request from UE can be transmitted to assist gNB to trigger PDCCH order for certain candidate cells if UE can decide the target cell to switch to.

Discussion:

[Ericsson, Samsung, LG, Vivo]: Agree with Huawei and considering the MR is already supported, no new mechanism is needed. [LG]: Without L1 measurement configuration, the UE still can report L3 measurement report (including beam level measured results) to the NW. [Nokia]: For early UL/DL sync procedure, we do not need any new UE assistance information, however we may still need discussion whether it needs just before UE autonomous cell switch.

The network can configure measurement reports e.g., L1 periodic, semi-persistent, aperiodic and event triggered report, or L3 measurement reports for conditional LTM, e.g., to trigger PDCCH ordered early RACH.

R2-2410546	Further considerations for conditional LTM	Ericsson	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 7: For CLTM, the Candidate Cell TCI States Activation/Deactivation MAC CE is re-used for the early activation/deactivation of TCI state(s) of a CLTM candidate configuration.

For CLTM, the Candidate Cell TCI States Activation/Deactivation MAC CE is re-used for the early activation/deactivation of TCI state(s) of a CLTM candidate configuration.

5. TA delivery in early UL sync procedure: which cell signal TA to the UE?

R2-2410448	Conditional LTM Scenarios and remaining points	Lenovo	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 1: The Early TA is signalled to the UE from the source cell (i.e., not from the candidate cell directly to the UE).

The Early TA is signalled to the UE from the source cell (i.e., not from the candidate cell directly to the UE). This agreement will be included in the LS to RAN1/3/4.

R2-2409803	Conditional LTM.	Interdigital, Inc.	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 3: For conditional trigger to perform RACH for UL sync, RAR containing TA is received from the target.

6. TA delivery in early UL sync procedure: which message is used?

R2-2409953	Conditional Intra-CU LTM Topics	Apple	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 3: The network can inform the candidate cell’s TA information to UE via new TAC MAC CE, which is the initial TA value when UE switches to that candidate cell during CLTM.

R2-2410448	Conditional LTM Scenarios and remaining points	Lenovo	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 3: The Rel. 18 LTM Cell Switch Command MAC CE is reused to provide Early TA, TCI states etc. to the UE for C-LTM purpose without triggering a mobility execution directly.

R2-2409888	Further discussion on supporting intra-CU conditional LTM	Transsion Holdings	discussion	Rel-19

Proposal 10 The RAN2 to develop a RAR for source cell to carry TAs of candidate cells to reduce signalling overhead.

R2-2409843	Discussion on conditional Intra-CU LTM	Fujitsu	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 2: The UE obtains the TA from the serving cell. TAC MAC CE is baseline.

Discussion:

[Samsung]: Ok with Apple proposal. For simplicity, with a new MAC CE, RACH procedure should not be changed, i.e. not coupling a new MAC CE as a part of RACH procedure.

The network can inform the candidate cell’s TA information to UE via new MAC CE, which is the TA value when UE switches to that candidate cell during CLTM.

7. TA maintenance in the UE

R2-2409875	Discussion on conditional LTM	OPPO	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 4	Candidate cell TA validity is guaranteed by TA timer.

Proposal 5	RAN2 study how to perform TA maintenance for candidate cell for CLTM with following options:

- Option1: per TAG

- Option2: per candidate cell

R2-2410795	On conditional LTM	Nokia	discussion	Rel-19	NR_Mob_Ph4

Proposal 14: When CLTM is configured, the maintenance of the TA value should be performed at the UE and a maintenance method similar to CG-SDT can be followed.

Discussion:

[Apple]: To check validity with Nokia proposal, is the UE required to periodically perform measurements? [Nokia]: Yes [Apple]: That would be additional burden to the UE. [Xiaomi]: The background reason for CG-SDT was that NW cannot send the latest updated TAC since this UE is in inactive. However, for C-LTM, this restriction is not applied. And good to avoid the UE’s burden to perform measurements. [Huawei, Lenovo, Ericsson]: Using timer is sufficient.

Candidate cell TA is maintained by a new timer.

8. Which layer to perform C-LTM event evaluation?

R2-2410795	On conditional LTM	Nokia	discussion	Rel-19	NR_Mob_Ph4

Proposal 9: For L1-based conditional LTM the condition evaluation is at MAC level and for L3-based conditional LTM the condition evaluation is at RRC level.

For L1-based conditional LTM the condition evaluation is at MAC level and for L3-based conditional LTM the condition evaluation is at RRC level.

9. Beam(s) used for C-LTM event evaluation

R2-2409595	Discussion on Conditional Intra-CU LTM	CATT	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 13 (modified): UE evaluates the execution condition for candidate beam and triggers the LTM cell switch procedure when at least one beam fulfills the associated execution condition.

R2-2409617	Discussion on conditional LTM	LG Electronics Inc.	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 11: RAN2 to consider at least one of the following approaches for event evaluation based on multiple beams:

- Approach1) UE derives a consolidated quality of multiple beams of candidate cell based on a method similar to the cell quality derivation as specified in 5.5.3.3a of 38.331 and use the consolidated result for evaluation CLTM execution condition.

- Approach2) Network configures the minimum number of good candidate cell beams for CLTM event (event 3/4/5), say N. If N is configured for Event CLTM, and UE considers the CLTM execution condition to be met if at least N beams of the candidate cell satisfy the CLTM condition respectively.  The value range of N is {1, 2, 3, 4}.

Discussion:

[IDC]: If L3 measurement is used for C-LTM, do we still need anything new? [Apple]: For C-LTM, consideration of one beam should be enough. [Qualcomm]: For L3 measurement, approach 1. For L1 measurement, per beam makes sense. [CMCC]: For L1 measurement, approach 2 is preferred. [MediaTek]: Prefer CATT proposal.

Will revisit it next meeting.

10. Beam selection in RACH-less LTM with CG

R2-2410064	Discussion on conditional intra-CU LTM	NEC	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 5: For RACH-less conditional LTM, UE selects a beam fulfilling the L1 event of the execution condition.

Proposal 5a: RAN2 to further down-selects between the following options:

	Option 1. If there are multiple beams fulfils the L1 event condition, UE selects from the beams which are also associated with activated TCI state

	Option 2. UE only evaluates the beams which are associated with activated TCI states for L1 event condition fulfilment

R2-2410445	Discussion on conditional intra-CU LTM	ZTE Corporation	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 8: For CG-based RACH-less conditional LTM, it’s up to the UE to select the configured grant, e.g. based on the associated beam quality. A RSRP threshold can be configured by the NW for beam selection associated with the configured grant, e.g. similar to cg-RRC-RSRP-ThresholdSSB.

R2-2410519	Intra-CU conditional LTM	Huawei, HiSilicon	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 8: In conditional LTM, when LTM cell switch is triggered, if the target cell is configured for RACH-less LTM cell switch (with CG or DG), the UE has previously received TCI states and the quality of the RS associated with the indicated TCI states is above a threshold, the UE initiates RACH-less LTM cell switch. Otherwise, the UE initiates RA.

11. Whether to support RACH-less LTM with DG

R2-2409953	Conditional Intra-CU LTM Topics	Apple	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 11: Do not support DG-based RACH-less CLTM.

R2-2410546	Further considerations for conditional LTM	Ericsson	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 9: For RACH-less CLTM, DG-based (i.e., BSR/SR mechanism) for first UL transmission on target CLTM candidate cell is supported.

12. Co-existence between LTM and C-LTM

R2-2410013	Discussion on conditional intra-CU LTM	ETRI	discussion	Rel-19

Proposal 5: conditional LTM and intra-CU LTM can be configured simultaneously.

Proposal 6: A UE identifies an LTM candidate cell as a conditional LTM candidate cell if the LTM candidate configuration includes execution conditions for conditional LTM.

Agreements on C-LTM:

- The triggering condition of conditional LTM can be based on L3 measurement.

- CondEventA3 and CondEventA5 conditions can be baseline for the conditional LTM execution.

- The L1 execution condition of a candidate cell is associated to only one triggering event.

- For L3 execution condition, it may consist of one or two triggering condition(s). If there are two triggering conditions associated with the same candidate cell, the UE shall consider the execution condition is fulfilled only when both triggering conditions are met. Only single RS type is supported and at most two different trigger quantities can be configured simultaneously for the evaluation of execution condition of a single candidate cell.

- To support initial and subsequent conditional LTM, the following items can be considered for the configuration of execution condition:

- The CLTM configuration of each candidate cell shall include the execution condition for initial conditional LTM, which is generated by the initial source cell to trigger the CLTM for the candidate cell.

- The CLTM configuration of each candidate cell may include execution conditions for subsequent conditional LTM, which is generated by the candidate cell to trigger the CLTM for other candidate cells when the candidate cell becomes a serving cell.

- The network can configure measurement reports e.g., L1 periodic, semi-persistent, aperiodic and event triggered report, or L3 measurement reports for conditional LTM, e.g., to trigger PDCCH ordered early RACH.

- For CLTM, the Candidate Cell TCI States Activation/Deactivation MAC CE is re-used for the early activation/deactivation of TCI state(s) of a CLTM candidate configuration.

- The Early TA is signalled to the UE from the source cell (i.e., not from the candidate cell directly to the UE). This agreement will be included in the LS to RAN1/3/4.

- The network can inform the candidate cell’s TA information to UE via new MAC CE, which is the TA value when UE switches to that candidate cell during CLTM.

- Candidate cell TA is maintained by a new timer.

- For L1-based conditional LTM the condition evaluation is at MAC level and for L3-based conditional LTM the condition evaluation is at RRC level.

R2-2409595	Discussion on Conditional Intra-CU LTM	CATT	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2409617	Discussion on conditional LTM	LG Electronics Inc.	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2409658	Further discussion on Conditional LTM	MediaTek Inc.	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2409766	Discussion on conditional LTM	vivo	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2409803	Conditional LTM.	Interdigital, Inc.	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2409843	Discussion on conditional Intra-CU LTM	Fujitsu	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2409875	Discussion on conditional LTM	OPPO	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2409888	Further discussion on supporting intra-CU conditional LTM	Transsion Holdings	discussion	Rel-19

R2-2409953	Conditional Intra-CU LTM Topics	Apple	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2409988	Conditional intra-CU LTM	Qualcomm Incorporated	discussion

R2-2410013	Discussion on conditional intra-CU LTM	ETRI	discussion	Rel-19

R2-2410022	Discussion on conditional LTM	Xiaomi	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2410044	Discussion on Conditional Intra CU LTM	Lekha Wireless Solutions	discussion	Rel-19

R2-2410064	Discussion on conditional intra-CU LTM	NEC	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2410115	Discussion on conditional intra-CU LTM	China Telecom	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2410136	Discussion on conditional intra-CU LTM	Spreadtrum, UNISOC	discussion	Rel-19

R2-2410230	Discussion on early TA acquisition for conditional intra-CU LTM	ITRI	discussion	NR_Mob_Ph4-Core

R2-2410252	Discussion on Conditional LTM	KT Corp.	discussion

R2-2410324	Discussion on Conditional LTM	CMCC	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2410389	Conditional Intra-CU LTM	Sony	discussion	Rel-19	NR_Mob_Ph4

R2-2410445	Discussion on conditional intra-CU LTM	ZTE Corporation	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2410448	Conditional LTM Scenarios and remaining points	Lenovo	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2410465	Discussion on conditional intra-CU LTM	Panasonic	discussion	Rel-19

R2-2410519	Intra-CU conditional LTM	Huawei, HiSilicon	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2410546	Further considerations for conditional LTM	Ericsson	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2410622	Support of Conditional Intra-CU LTM	Samsung	discussion	Rel-19	NR_Mob_Ph4-Core

=> Revised in R2-2410889

R2-2410889	Support of Conditional Intra-CU LTM	Samsung	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2410661	Discussion on Conditional Intra-CU LTM 	Kyocera 	discussion	Rel-19

R2-2410689	Discussion on conditional LTM	HONOR	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2410695	Discussion on Conditional intra-CU LTM	NTT DOCOMO, INC.	discussion	Rel-19

R2-2410701	Discussion on conditional LTM	Sharp	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2410709	Discussion on Conditional LTM	ITL	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2410782	Discussion on Conditional LTM	CEWiT	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2410795	On conditional LTM	Nokia	discussion	Rel-19	NR_Mob_Ph4

## 8.7	XR Enhancements Ph3

(NR_XR_Ph3-Core; leading WG: RAN2; REL-19; WID: RP-241771)

Time budget: 2 TU

Tdoc Limitation: 5 tdocs

### 8.7.1	Organizational

LS, Rapporteur input, workplan, etc.

Incoming LS from SA2 in S2-2411253 will be discussed based on the input from the contact company.

Rapporteur input

R2-2409818	Rapporteur Inputs	Nokia, Qualcomm (Rapporteurs)	discussion	Rel-19	NR_XR_Ph3-Core

Noted

Incoming LS

R2-2409517	Reply LS on multi-modality awareness at RAN (R3-245682; contact: Nokia)	RAN3	LS in	Rel-19	NR_XR_Ph3-Core, XRM_Ph2	To:SA2	Cc:RAN2, SA4, RAN

Noted

R2-2409525	Reply LS on Application-Layer FEC Awareness at RAN (S2-2410999; contact: Qualcomm)	SA2	LS in	Rel-19	XRM_Ph2	To:RAN2	Cc:SA4

Noted

R2-2409533	LS for PDU Set Information Marking Support without QoS parameters (S2-2411253; contact: vivo)	SA2	LS in	Rel-19	XRM_Ph2	To:RAN2, RAN3

Q1:SA2 would like RAN2 to provide feedback (e.g. the usefulness) on the PDU Set based handling without PDU set QoS parameters.

Noted

Discussion on the reply LS on PDU Set Information

R2-2409767	Discussion on SA2 LSs on XRM and FEC	vivo	discussion	Rel-19	NR_XR_Ph3-Core

Noted

Proposal 1: RAN2 confirms that it is useful for gNB to have PDU Set Information marking without PDU Set QoS parameters, e.g. for PSI based SDU discard.

Proposal 2: RAN2 to discuss the draft reply LS in Annex A for PDU Set Information Marking Support without QoS parameters.

DISCUSSION on P1:

- OPPO thinks SA2 only asks about DL and asks if it applies for UL as well.

- Vivo thinks LS mentions DL only.

- LGE thinks both LSes from SA2 focus only on DL. LGE agrees with the proposal.

- OPPO agrees this is only for DL and then agrees with the proposal.

- Sony agrees with the proposal, it is useful.

Agreement for the reply to SA2 on PDU set information: - RAN2 confirms that it can be useful for gNB to have PDU Set Information marking without PDU Set QoS parameters.

[AT128][501][XR] Reply LS on PDU set (vivo)

Scope: Reply LS

Intended outcome: Reply LS in R2-2411001

Deadline: For offline approval on Friday 2024-11-22 09:00

R2-2411001 Reply LS to SA2 on PDU set	RAN2	LS out	Rel-19	NR_XR_Ph3-Core	To:SA2

LS is approved

R2-2410491	Discussion on LS from SA2 on PDU Set information	Sony	discussion	Rel-19	NR_XR_Ph3

Draft reply LS on AL-FEC

R2-2409560	Reply LS on Application-Layer FEC Awareness at RAN	Qualcomm Incorporated	LS out	Rel-19	NR_XR_Ph3-Core	To:SA2	Cc:SA4

### 8.7.2	Multi-modality support

No contributions are expected for this AI for RAN2#128

### 8.7.3	RRM measurement gaps/restrictions related enhancements

Objective: Specify enhancements to enable transmission/reception in gaps/restrictions that are caused by RRM measurements (from inter-frequency RRM measurement gaps, or intra-frequency measurements, or other scheduling restrictions etc).

Focus on RAN2 impacts from solutions considered by RAN1/RAN4.

Impacts on DSR

R2-2409768	Discussion on RRM measurement gaps enhancements	vivo	discussion	Rel-19	NR_XR_Ph3-Core

Noted

Observation 1	It is a reasonable assumption that a gNB can determine the cancellable MG skipping based on the DSR report received with a certain offset before the cancellable MG occasion.

Observation 2	Depending on the overlapping status between the PDCP discardTimer running duration and the cancellable MG during, the DSR transmission is delayed and the gNB cannot determine MG skipping in time in the following two cases:

-	The DSR is triggered during the MG and can only be transmitted after the MG.

-	The DSR is triggered before the MG, but the offset to the cancellable MG is too short for gNB to receive DSR or indicate the MG skipping.

Proposal 1	RAN2 to consider early DSR triggering and reporting before a cancellable MG when there is an overlapping between PDCP discardTimer running duration and MG duration.

Observation 3	Even if a triggered DSR cannot be transmitted before a cancellable MG and is transmitted to gNB after the MG, it can be still helpful somehow for delay-critical data transmission after MG.

Proposal 2	Do not support cancellation of DSR that cannot be transmitted early enough before a cancellable MG.

R2-2409912	Discussion on MG enhancement for XR	LG Electronics Inc.	discussion	Rel-19	NR_XR_Ph3-Core

Noted

Proposal 2. No MG-specific enhancements is needed on DSR operation.

DISCUSSION:

- Ericsson agrees with LGE that DSR configuration is flexible, so no enhancements are needed. We will anyway have additional granularity in reporting.

- CMCC also supports LGE’s proposal.

- Lenovo has some sympathy for vivo’s proposal for DSR triggering enhancements.

- Xiaomi thinks there is no impact on triggering, but there is impact on remaining time calculation.

- ZTE clarifies we already have measurement gaps activation/deactivation. There is nothing new now, even with MG skipping. No DSR impact is needed.

- QCM agrees no enhancements for DSR are needed. We should define remaining time based on PDB, it should not be impacted by MG or any other scheduling events.

- Vivo indicates that in some cases DSR will not be sent on time.

- OPPO thinks that it can be considered by the network when configuring DSR.

No MG-specific enhancements is needed on DSR operation.

Impacts on DRX

R2-2410209	RRM measurement gaps/restrictions related enhancements	Nokia, Nokia Shanghai Bell	discussion	Rel-19	NR_XR_Ph3-Core

Noted

Proposal 2: The UE shall consider the duration of a measurement gap or scheduling restriction period indicated to be skipped as active time without relying on configuring long enough inactivity timer.

Proposal 3: RAN2 to evaluate rules for extending the UE active time when a measurement opportunity window overlaps with (or is too close to) the DRX OnDuration, e.g. by extending the OnDuration or by waking up the UE earlier than the start of the OnDuration for potential DCI skip command monitoring.

R2-2410154	Discussion on RRM enhancements for XR	Huawei, HiSilicon	discussion	Rel-19	NR_XR_Ph3-Core

Noted

Observation 3: When DRX is configured and the DRX periodicity is longer than the MG periodicity, the gNB may be unable to indicate the UE to skip the conflicting MGs considering the limited active time.

Proposal 4: Semi-static solutions can be considered for addressing the collision between DRX and MG.

R2-2410082	Impacts on DSR and DRX from MG skipping	NEC	discussion	Rel-19	NR_XR_Ph3-Core

Noted

Observation 2	During the skipped MG period, the PDCCH monitoring activity of the UE in RRC connected mode is also governed by DRX, BA, DCP and cell DTX.

Proposal 2:  	Regrading impacts on DRX from MG skipping, RAN2 confirm that there are no impacts requiring specification changes/enhancements.

DISCUSSION on whether/what enhancements are needed for DRX due to MG skipping:

- IDT thinks that DCI for skipping should be indicated during active time. DRX and MG should be configured properly.

- Lenovo indicates DRX is configured according to traffic pattern.

- ZTE thinks nothing is needed to optimize. ZTE thinks the baseline is that any DCI activates inactivity timer.

- LGE also believes NW implementation can handle this as traffic pattern is known, NW can also extend active time by sending a DCI.

- QCM agrees no enhancement is needed. NW should not cancel a gap which is in inactive time. Longer on duration can also be configured. There will be data in each period anyway so inactivity timer will be triggered.

- Xiaomi also does not see issues.

- Nokia believes that UE should monitor during MG indicated as skipped by the NW.

- Huawei thinks that there is an issue with the current RAN1 solution, because we may not be able to indicate skipping of some MGs. We should note there are some issues. QCM agrees that RRC-based solution may be useful if the NW knowns in advance that there will be overlaps.

- Ericsson thinks there is no big issue, because the network is aware in advance.

- Vivo’s understanding is that MG skipping indication does not impact PDCCH monitoring. Vivo thinks we need to address the issue of UE not being able to receive DCI (because it is in inactive time).

- Ericsson sees no issue because RAN1 design allows the network to indicate each gap to be skipped.

- Lenovo’s and OPPO understanding is that UE follows current procedures, just assumes there is no MG.

- Ericsson thinks we should not discuss PDCCH monitoring in RAN2.

RAN2 assumes that UE follows DRX pattern as currently, even when MG is indicated as skipped

No MG-specific enhancements is needed for DRX operation.

Agreements on RRM measurement gaps impacts - No MG-specific enhancements is needed on DSR operation. - RAN2 assumes that UE follows DRX pattern as currently, even when MG is indicated as skipped - No MG-specific enhancements is needed for DRX operation.

Stage-3 details of DCI-based solution

R2-2409844	Discussions on measurement gap related enhancements	Fujitsu	discussion	Rel-19	NR_XR_Ph3-Core

Not treated

Proposal 1: Enhance MAC procedure that when receiving the indication from lower layer, consider the next measurement gap occurrence starting after a minimum time offset skipped. Whether there is any restriction on the eligibility of the next measurement gap to be skipped is subject to RAN1/RAN4 decision.

Proposal 2: To support DCI-based measurement gap skipping indication, RAN2 design necessary RRC configurations, e.g., minimum time offset between the last symbol of the PDCCH carrying the DCI format and the start of the corresponding skipped measurement gap occasion, configuration on explicit indication for each DCI format, etc.

R2-2409555	Discussion on measurement gap cancelation	Qualcomm Incorporated	discussion	Rel-19	NR_XR_Ph3-Core

R2-2409722	Discussion on RRM measurement gaps/restrictions related enhancements	Hanbat National University	discussion	Rel-19	Withdrawn

R2-2409734	Discussion on Measurement Gap enhancements	OPPO	discussion	Rel-19	NR_XR_Ph3-Core

R2-2409785	Measurement gap skipping for XR	ZTE Corporation, Sanechips	discussion

R2-2409825	Discussion on RRM measurement gaps enhancements of XR traffic	Xiaomi Communications	discussion

R2-2409854	Consideration on Enabling TX RX for XR during RRM Measurements	CATT	discussion	Rel-19	NR_XR_Ph3-Core

R2-2409955	Views on Enhancements Relating to RRM Measurement Gaps	Apple	discussion	Rel-19	NR_XR_Ph3-Core

R2-2410089	RAN2 Issues on Gap Enhancements	Sharp	discussion	Rel-19	NR_XR_Ph3-Core

R2-2410095	Enabling TX/RX for XR during measurement gaps/restrictions	Lenovo	discussion	Rel-19	NR_XR_Ph3-Core

R2-2410198	RRM Measurement Gap/Restrictions	Ericsson	discussion	Rel-19

R2-2410245	Discussion on RRM enhancements for XR	Huawei, HiSilicon	discussion	Rel-19	NR_XR_Ph3-Core	Withdrawn

R2-2410338	RRC-based MG skipping solution	CMCC, Qualcomm, Google	discussion	Rel-19	NR_XR_Ph3-Core

R2-2410387	Discussion on enabling TX/RX for XR during RRM measurements	Sony	discussion	Rel-19	NR_XR_Ph3

R2-2410406	RRM measurement gap related enhancements for XR	InterDigital	discussion	Rel-19	NR_XR_Ph3-Core

R2-2410577	RRM Measurement Gaps/Restrictions related enhancements for XR	Google Ireland Limited	discussion

R2-2410718	Discussion on RRM measurement gaps/restrictions enhancements for Rel-19 XR	Samsung	discussion	Rel-19	NR_XR_Ph3-Core

R2-2410740	Discussion on XR RRM measurement gaps/restrictions related enhancements	III	discussion	NR_XR_Ph3-Core

R2-2410761	Discussion on RRM measurement for XR enhancements	Hanbat National University	discussion	Rel-19

R2-2410783	Discussion on RRM Measurement Gaps/Restrictions Enhancements	Meta	discussion

### 8.7.4	Scheduling enhancements

R2-2410390	UL scheduling enhancements for Multi-modal traffic flows	Sony	discussion	Rel-19	NR_XR_Ph3

#### 8.7.4.1	LCP enhancements

Objective: Specify Enhancements for support of UL scheduling to enable high XR capacity while meeting delay requirements/avoiding too late PDUs, as follows [RAN2]:

- Specify additional Logical Channel priority handling using delay/deadline information of packets;

Further details of handling of the additional priority for LCH.

Priority during 2nd LCP round

R2-2410316	Consideration on fairness in delay-aware LCP for XR	CMCC, Qualcomm Incorporated	discussion	Rel-19	NR_XR_Ph3-Core

Noted

Proposal 1: RAN2 agrees the negative impact of applying additional priority to non-delay-critical data on other LCHs and delay-critical data should be avoided for fairness.

Proposal 2: Delay-critical data and non-delay-critical data should be separately handled during LCP priority adjustment. The exact method for the identification and separation of delay-critical data and non-delay-critical data is up to UE implementation

R2-2410435	Discussion on Leftover Issues for Additional LCH Priority	China Telecom	discussion

Noted

Proposal 1: If all delay-critical data is assembled during 1st round, the LCH priority is changed to legacy one before 2nd round of resource allocation to ensure fairness.

Proposal 2: How to change the priority before the 2nd round of resource allocation can be left to UE implementation.

R2-2409956	On Priority Switching during LCP Procedure	Apple	discussion	Rel-19	NR_XR_Ph3-Core

Noted

Proposal 1: RAN2 confirms that the additional LCH priority is applied to both the first round and the second round of the LCP procedure. The UE does not fallback to the default LCH priority in the second round even if there is no more LCH priority-adjusted data after the first round.

Proposal 2: If Proposal 1 is not agreeable, RAN2 leaves it to UE implementation to determine if LCH priority can be changed for the second round of the LCP procedure. RAN2 should not mandate the UE behavior of changing LCH priority during an ongoing LCP procedure by specification.

DISCUSSION:

- IDT would like to address fairness, but leaving this to UE implementation makes things unpredictable to the scheduler.

- Xiaomi thinks leaving this to implementation would probably mean UEs will not do it.

- QCM thinks companies acknowledge importance of fairness. QCM does not understand the complexity issue. UE knows how much data is adjustable, so calculating the leftover is easy. Adjusting priority order is also easy among a few LCHs. Ensuring fairness is more important. Leaving up to implementation will increase unfairness issue between UE vendors.

- MTK thinks changing LCH order can be complex. MTK indicates that fairness in the second round is not critical.

- Lenovo agrees with QCM – this is not complex. Lenovo still believes that taking all the resources in the second round violates fairness. We already agreed there is no intra LCH prioritization, so this is good compromise.

- vivo acks the fairness issue, but is concerned about the complexity. Fairness issue is not big as delay critical will be scheduled in the first round.

- OPPO would not like to impact the delay of LCP procedure and fairness is not an issue in 2nd round.

- LGE agrees with MTK, vivo, OPPO. There should be one behaviour and we should choose simpler approach.

- Huawei supports QCM/Lenovo view that there is no real complexity. The UE can check the data volume only at the beginning of the procedure. Supports adjustment in the 2nd round.

- Ericsson, ZTE also sees this as a compromise, we already excluded intra-LCH, so at least adjusting priority in the second round should be supported.

- Nokia thinks we are repeating the discussions, if we cannot agree, we should move on with the baseline.

- Samsung recognizes the issue, but does not see it as practical issue, e.g. XR and URLLC is unlikely torun together. Up to UE implementation is OK.

- Sharp does not think it can be up to UE implementation. Behaviour needs to be testable.

- QCM thinks there are two dimensions of fairness – fairness between the UEs but also fairness between LCHs within the UE.

- Huawei thinks there could also be a capability and then the NW knows the UE behavoiur.

- Apple thinks that such traffic will be high priority anyway. There will be not much to optimize.

- MTK indicates Bj will anyway control where the UE takes data from.

- ZTE thinks the NW needs to know what the UE does, how it uses the grants.

- Ericsson also clarifies that the size of grant can be adjusted based on knowledge.

Agreements on LCP prioritization - As a baseline, the additional LCH priority is applied to both the first round and the second round of the LCP procedure. The UE does not fallback to the default LCH priority in the second round even if there is no more LCH priority-adjusted data after the first round. - As an optional capability, the UE can also support to fallback to default priority in the 2nd round of LCP.

Impact on SRB and intra-UE prioritization

R2-2409677	Discussion on LCP enhancements for XR	OPPO	discussion	Rel-19	NR_XR_Ph3-Core

Proposal 3	Up to network implementation to avoid SRB impact due to LCH prioritization-based LCP enhancement, i.e., configure higher priority for LCH of SRBs than LCH with LCH priority-adjusted data.

Proposal 4	The additional LCH priority also applies to intra-UE prioritization procedure. FFS whether it applies for the cases including 1) the priority of uplink grant for new transmission, 2) the priority of uplink grant for retransmission, and 3) the priority of SR.

Impact on PBR/Bj

R2-2409845	Discussions on enhancements for LCH priority-adjusted data	Fujitsu	discussion	Rel-19	NR_XR_Ph3-Core

Proposal 2: The LCH with LCH priority-adjusted data is not restricted by PBR in the first round of resource allocation procedure.

R2-2409769	Remaining issues on LCP enhancements for XR	vivo	discussion	Rel-19	NR_XR_Ph3-Core

Observation 4	For MAC PDU construction, the LCH with delay-critical data may not be able to be allocated enough radio resource due to its buckets (Bj) exhausted.

Proposal 2	In addition to LCH priority adaptation, RAN2 should also consider temporarily allowing UE to raise the rate limit (e.g. Bj) for delay-critical LCH.

R2-2410500	LCP enhancements	Ericsson	discussion	Rel-19	NR_XR_Ph3-Core

Proposal 2	A delay-aware LCP solution uses a new counter variable to keep track of the scheduling of the LCH priority-adjusted data.

One or multiple additional priorities

R2-2410373	Discussion on additional priority based LCP enhancements in XR	Huawei, HiSilicon	discussion	Rel-19	NR_XR_Ph3-Core

Observation 1:	Depending on when the grant is provided from the network, there may be a situation where the “LCH priority-adjusted data” on an LCH with lower additional priority has lower remaining time than the data in another LCH with higher additional priority. If NW configures only one additional priority for each LCH based on the remaining time threshold, the “LCH priority-adjusted data” on the LCH with lower remaining time may be impacted, e.g. its PDB may expire.

Proposal 2:	To maximize the chances that the resource allocation goes to LCH with lower remaining time, NW can configure a list of additional priorities, UE determines the additional priority for each LCH based on the smallest remaining time of data buffered on each LCH according to the configured list of additional priorities.

R2-2409556	Discussion on LCP enhancements	Qualcomm Incorporated	discussion	Rel-19	NR_XR_Ph3-Core

R2-2409786	LCP enhancements for XR	ZTE Corporation, Sanechips	discussion

R2-2409790	Discussion on Logical channel priority	CANON Research Centre France	discussion	Rel-19	NR_XR_Ph3-Core

R2-2409828	Discussion on LCP enhancements of XR traffic	Xiaomi Communications	discussion

R2-2409855	Consideration on LCP Enhancement	CATT	discussion	Rel-19	NR_XR_Ph3-Core

R2-2409910	Discussion on LCP enhancement for XR	LG Electronics Inc.	discussion	Rel-19	NR_XR_Ph3-Core

R2-2410038	Discussion on how to apply additional priority for delay-aware LCP 	TCL	discussion

R2-2410090	Details of LCP Enhancements	Sharp	discussion	Rel-19	NR_XR_Ph3-Core

R2-2410094	Enhanced Logical channel prioritization for XR 	Lenovo	discussion	Rel-19	NR_XR_Ph3-Core

R2-2410194	Considerations on LCP enhancements for XR	NEC  Corporation	discussion	Rel-19	NR_XR_Ph3-Core

R2-2410208	Discussion on LCP enhancements for XR	DENSO CORPORATION	discussion	Rel-19	NR_XR_Ph3-Core

R2-2410210	LCP Enhancements for XR	Nokia, Nokia Shanghai Bell	discussion	Rel-19	NR_XR_Ph3-Core

R2-2410407	LCP enhancements for XR	InterDigital	discussion	Rel-19	NR_XR_Ph3-Core

R2-2410692	Remaining issue for LCP enhancement	MediaTek Inc.	discussion	Rel-19	NR_XR_Ph3-Core

R2-2410784	Discussion on LCP Enhancements for XR	Meta	discussion

R2-2410844	Further Discussion on additional LCP handling	ETRI	discussion

R2-2410850	LCP enhancements for Rel-19 XR – outstanding issues	Samsung R&D Institute UK	discussion

#### 8.7.4.2	DSR enhancements

Objective: Specify enhanced DSR (Delay Status Report) reporting with multiple pairs of remaining time and buffer size for a LCG.

Including aspects such as need of thresholds configuration constraints (including analysis of impact on DSR triggering/cancellation etc.), inclusion of non-delay critical data, MAC CE design, interworking with legacy DSR etc.

Thresholds configuration

R2-2410436	Discussion on Leftover Issues for DSR Enhancement	China Telecom	discussion

Proposal 1: The triggering threshold is set to a value equivalent to a lower reporting threshold.

R2-2409856	Consideration on DSR Enhancement	CATT	discussion	Rel-19	NR_XR_Ph3-Core

Proposal 1: The reporting remaining time thresholds need to be less than the triggering threshold in enhanced DSR to offer finer granularity about the delay status.

R2-2409797	Discussion on Delay status report	CANON Research Centre France	discussion	Rel-19	NR_XR_Ph3-Core

Noted

Observation 1: In release 18, triggering threshold and reporting threshold have the same value.

Observation 2: Each setting of reporting and triggering thresholds may be advantageous to certain scenario

Proposal 1: Let the network configure the triggering and reporting thresholds without constraints.

DISCUSSION:

- LGE agrees we do not need any restrictions. It depends on the scenario how to configure the thresholds. LGE does not observe the issues raised by CATT as triggering/cancellation depends on triggering threshold.

- OPPO, Lenovo, Samsung agrees with CANON and LGE.

- IDT thinks without restrictions we may report also non-delay critical data which was not the intention.

- Xiaomi thinks that some data may be reported again if we have no restrictions. There may be some spec impact.

- Samsung thinks that NW may want to know about data remaining time even for non-delay critical data.

- QCM can accept the proposal (no restrictions). QCM would like to ensure that there are no duplicate reporting.

- LGE shares the understanding from QCM, i.e. the reported data should not trigger DSR, but it can be reported again (if another data triggers DSR).

- CMCC thinks DSR should be retriggered if DC data has not been sent.

- Nokia thinks according to current specs it will not trigger again.

Let the network configure the triggering and reporting thresholds without constraints.

RAN2 understanding is that the data that has been already reported in the DSR should not trigger another DSR

R2-2410684	Discussion on DSR enhancements	HONOR	discussion	Rel-19	NR_XR_Ph3-Core

Noted

Proposal 1: The triggering threshold for the enhanced DSR should be configured by the network, which can be lower than largest reporting threshold.

Proposal 2: The existing cancelling and triggering of Rel-18 DSR is reused for the enhanced DSR.

The existing cancelling and triggering of Rel-18 DSR is reused for the enhanced DSR.

Non-delay critical data ahead in the queue

R2-2410408	DSR enhancements for UL scheduling	InterDigital	discussion	Rel-19	NR_XR_Ph3-Core

Proposal 2: Define the non-delay critical data ahead of delay critical data in the buffer as delay critical and include it in the buffer size calculation for DSR.

R2-2409911	Discussion on DSR enhancement for XR	LG Electronics Inc.	discussion	Rel-19	NR_XR_Ph3-Core

Proposal 3. No further enhancements in the buffer size calculation for DSR to include non-delay critical data ahead of delay critical data

DISCUSSION:

- OPPO agrees with LGE. The first use case is for congestion but this will be corner case. The second case depends on threshold configuration by the NW.

- Ericsson thinks this also happens in the interleaving case, so it is not corner case. Ericsson thinks this has not additional complexity compared to what we are already defining.

- Vivo agrees with LGE analysis. Vivo thinks it could be up to UE implementation how to place data in the buffer, but there is no need to impact DSR.

- Nokia thinks NW cannot make any assumptions on UE implementation. This information is useful for NW scheduling and there is no additional complexity for UE.

- Lenovo agrees with Nokia, Ericsson. Reordering data in the buffer is not a good solution, because it impacts reordering.

- QCM thinks this helps the network and supports the proposal from Interdigital.

- Xiaomi has sympathy with vivo’s proposal, but the main concern is with specs impact. It will impact how we specify the delay-critical data. Xiaomi has a strong concern on this.

- Samsung agrees with LGE, does not believe interleaving happens often. Samsung thinks it can be handled by UE implementation.

- LGE would not like to modify Rel-18 data-critical calculation. This causes the confusion depending on which DSR is used, i.e. with legacy DSR calculation is different than with Rel-19 DSR.

- IDT indicates that this will depend on UE release, not on which DSR is used.

- IDT indicates that it does not seem possible to handle this with UE implementation if the UE uses pre-processing.

- Ericsson indicates that the point is to fix the flaws of Rel-18.

- LGE, MTK have strong concerns on changing the DSR calculation.

- Apple indicates that UE implementations should handle this case.

- HONOR asks about interworking with Rel-18 DSR.

The UE may also support including non-delay critical data ahead of delay critical data in the buffer size calculation for DSR, which is a capability indicated to the NW.

MAC CE design

R2-2409770	Remaining issues on DSR enhancements for XR	vivo	discussion	Rel-19	NR_XR_Ph3-Core

Proposal 5	RAN2 to down-select one of the following options for R19 DSR format design:

-	Option A: One bitmap per LCG to indicate the presence of the pairs;

-	Option B: One extension bit (e.g. redefining R bit in R18 DSR) per pair to indicate the presence of the next pair of the same LCG.

Co-existence with Rel-18 DSR

R2-2410728	Further discussion on DSR enhancement	TCL	discussion	Rel-19

Proposal 2: It is suggested to use a new enhanced DSR in case there is at least one LCG configured with multiple reporting thresholds.

R2-2409557	Discussion on DSR enhancements	Qualcomm Incorporated	discussion	Rel-19	NR_XR_Ph3-Core

Proposal 1.	If there is at least one LCG configured with multiple reporting thresholds, the UE should always use Rel-19 DSR MAC CE, as long as it can be accommodated by the available PUSCH resource.

Proposal 2.	If available PUSCH resource is not large enough to accommodate a Rel-19 DSR MAC CE, a Rel-18 DSR MAC CE is sent instead, with the delay information calculated based on the trigger threshold.

R2-2409678	Discussion on DSR enhancements for XR	OPPO	discussion	Rel-19	NR_XR_Ph3-Core

R2-2409770	Remaining issues on DSR enhancements for XR	vivo	discussion	Rel-19	NR_XR_Ph3-Core

R2-2409787	DSR enhancements for XR	ZTE Corporation, Sanechips	discussion

R2-2409827	Discussion on DSR enhancements of XR traffic	Xiaomi Communications	discussion

R2-2409846	Discussions on DSR enhancements	Fujitsu	discussion	Rel-19	NR_XR_Ph3-Core

R2-2409957	Views on DSR Enhancements	Apple	discussion	Rel-19	NR_XR_Ph3-Core

R2-2410091	Details of DSR Enhancements	Sharp	discussion	Rel-19	NR_XR_Ph3-Core

R2-2410189	Considerations on DSR enhancements for XR	NEC  Corporation	discussion	Rel-19	NR_XR_Ph3-Core

R2-2410211	DSR Enhancements for XR	Nokia, Nokia Shanghai Bell	discussion	Rel-19	NR_XR_Ph3-Core

R2-2410212	Discussion on DSR enhancements for XR	DENSO CORPORATION	discussion	Rel-19	NR_XR_Ph3-Core

R2-2410229	Discussion on enhanced DSR for XR	ITRI	discussion	NR_XR_Ph3-Core

R2-2410286	Enhanced delay status reporting for XR	Lenovo	discussion	Rel-19

R2-2410317	Further consideratio on DSR enhancment	CMCC	discussion	Rel-19	NR_XR_Ph3-Core

R2-2410372	Discussion on DSR enhancements in XR	Huawei, HiSilicon	discussion	Rel-19	NR_XR_Ph3-Core

R2-2410408	DSR enhancements for UL scheduling	InterDigital	discussion	Rel-19	NR_XR_Ph3-Core

R2-2410501	DSR enhancements	Ericsson	discussion	Rel-19	NR_XR_Ph3-Core

R2-2410717	DSR enhancements for Rel-19 XR	Samsung	discussion	Rel-19	NR_XR_Ph3-Core

R2-2410728	Further discussion on DSR enhancement	TCL	discussion	Rel-19

R2-2410762	Discussion on XR DSR enhancements	III	discussion	NR_XR_Ph3-Core

R2-2410785	Discussion on DSR Enhancements for XR	Meta	discussion

R2-2410843	Further Discussion on DSR enhancements	ETRI	discussion

### 8.7.5	RLC enhancements

Objective: RLC re-transmission related enhancements for operation of RLC Acknowledged Mode (AM) with small packet delay budget.

Including aspects such as:

- how to avoid unnecessary retransmissions, e.g. details of the combined approach

- how to  ensure timely RLC retransmissions for XR, e.g.

  - what kind of enhancements are needed, e.g. autonomous retransmission, retransmission based on enhanced polling

  - details and pros and cons of different solutions (including impact on capacity and packet delay)

- discussion on the LS from SA2 in S2-2410999

Discussion on the reply to LS on AL-FEC

R2-2409561	Discussion on AL-FEC awareness at RAN	Qualcomm Incorporated, China Telecom, Huawei, HiSilicon, Lenovo, Nokia, Nokia Shanghai Bell, Xiaomi	discussion	Rel-19	NR_XR_Ph3-Core

Noted

Proposal 1.	Reply to SA2 that it is feasible to use content ratio for discarding DL PDUs during congestion for DRBs configured with RLC AM.

Proposal 2.	Reply to SA2 that it is feasible to use RLC AM for XR services, especially with the upcoming latency reduction enhancements for RLC AM (per RP-240791).

Proposal 3.	Reply to SA2 that it is feasible to use content ratio for discarding DL PDUs during congestion for DRBs configured with RLC UM.

Proposal 4.	Include an example of possible gNB implementation in the reply LS to illustrate the feasibility of using content ratio for discarding DL PDUs during congestion.

R2-2409788	RLC enhancements for XR	ZTE Corporation, Sanechips	discussion

Noted

Proposal 1: RAN2 should reply to SA2 that content ratio information is not useful for packet discard at RAN for RLC AM as AM mode of operation generally aims at transmitting all AM packets reliably to the receiver and hence data packets that are not needed at the upper layers should never to be submitted to RAN in the first place in case of AM operation.

Proposal 2: RAN2 should reply to SA2 that content ratio information is not useful for packet discard for RLC-UM since RAN cannot determine reliably whether a given PDU has been received successfully or not for RLC UM.

R2-2410337	Discussion on the RLC enhancements for XR and RAN2 impacts on the SA2 LS	CMCC	discussion	Rel-19	NR_XR_Ph3-Core

Noted

Proposal 1: It is useful for gNB to be provided the content ratio information for discarding of DL PDUs during congestion for RLC AM mode.

Proposal 2: It may not bring benefit for gNB to be provided with the content ratio information for discarding of DL PDUs during congestion for RLC UM mode.

DISCUSSION:

- Ericsson agrees with ZTE that in general we should not use AL-FEC for AM mode as it will only increase the congestion and also delays other packets.

- MTK, LGE, Meta agrees with Ericsson and ZTE. MTK also indicates that considering that we will have fake ACK, this may impact gNB judgement.

- LGE also indicates CU/DU split issues, agrees with the issue of using fake ACK to determine whether packet was truly received.

- Huawei indicates that this solution is supposed to be used during congestion, so it can tell gNB how many packets can be dropped. So it helps to mitigate the congestion. Fake ACK makes no difference.

- Meta does not believe content ratio is helpful information.

- Samsung agrees that AL-FEC should not be applied to AM mode.

- QCM reminds that AL-FEC will not increase the congestion according to SA2’s understanding. Without information about the AL-FEC, gNB discards randomly, but thanks to knowing the ratio, it can choose packets to discard.

- Vivo agrees with QCM and Huawei, this is only for congestion.

- Ericsson thinks this that it is unclear how this interacts with rate adaptation. Ericsson thinks we should have no extra packets during congestion.

- Xiaomi agrees this is useful to have. SA2 LS already clarifies that no additional packets will be generated due to intentional dropping at gNB.

- Sharp does not think this is useful for RLC AM.

- Chair: No consensus in RAN2 on whether AL-FEC ratio is useful due to concerns on e.g. impact on gNB pre-processing gain, impact of intentional dropping on app layer behaviour, lack of reliable reception ACK etc.

- Agreements on AL-FEC (related to LS from SA2) - There is no consensus in RAN2 that AL-FEC ratio information is useful for the gNB for both RLC AM and RLC UM. - RAN2 understanding is that in case this information would be provided to the gNB, it is up to gNB how/whether to consider it, i.e. no impact on RAN2 specifications

[AT128][502][XR] Reply LS on AL-FEC (QCM)

Scope: Reply LS to SA2

Intended outcome: Reply LS to SA2 in R2-2411002

Deadline:  LS for offline approval: Friday 2024-11-22 09:00

R2-2411002 Reply LS on Application-Layer FEC Awareness at RAN 	RAN2	LS out	Rel-19	NR_XR_Ph3-Core, XRM_Ph2	To:SA2	Cc:SA4

Unnecessary retransmissions avoidance – solution details

R2-2409636	RLC AM retransmission enhancements	Xiaomi	discussion	Rel-19	NR_XR_Ph3-Core

Noted

Proposal 5: The indication from Tx to Rx about the discarded RLC PDUs are not supported.

R2-2410092	Issues on RLC Enhancements	Sharp	discussion	Rel-19	NR_XR_Ph3-Core

Noted

Proposal 2	A TX-to-RX indication of obsolete SDUs can be configurable.

DISCUSSION on indication from Tx to Rx:

- LGE, Ericsson, Huawei, Nokia, QCM agrees with Xiaomi’s proposal.

- LGE thinks that the only gain is to avoid Tx window stall, but this is corner case.

- MTK, vivo does not agree this is a corner case, thinks that during congestion this can happen frequently.

- Huawei indicates that there will be delay to send indication and we agreed to have Rx local timer which addresses window stalling issue.

- Nokia thinks indication would be useful for reordering which will not be used. Window stalling is addressed by local timer.

- Vivo think with an indication we can save some time. Vivo thinks that local timer at Rx side will be long.

- OPPO agrees that we need an indication to address window stalling issue. OPPO thinks we can include it in data packet to minimize impact on specifications.

- Futurewei analyzed this in the past and window stall will almost never happen if the local timer is shorter than 1-2 seconds.

- Lenovo agrees with MTK, it can also be useful to avoid discarding of C-PDU.

- CMCC, KDDI indication is beneficial as it gives more accurate information for discarding.

- Samsung also support having an indication.

- Ericsson does not agree with the delay issue.

- QCM also does not think that some additional delay in discarding is a critical issue.

- LGE does not see clear benefit.

- OPPO thinks that local Rx timer will be set to long value to avoid losses, so then window stalling can happen.

- Ericsson thinks we are repeating the discussions we already had and we agreed a combined approach.

Agreements on unnecessary RLC retransmissions avoidance - There is no clear understanding on how the indication would look like or what problem it would solve that cannot be solved by the local timer - Unless critical issue is identified, no Tx to Rx indication will be introduced

R2-2409740	Further details of RLC enhancements for XR	LG Electronics Inc.	discussion	Rel-19	NR_XR_Ph3-Core

Proposal 2. Special handling to avoid PDCP control PDU discard is not needed.

Proposal 3. A new RLC timer at the Rx is introduced to determine obsolete RLC SDUs.

Proposal 4. The abandoned RLC SDUs determined by a new RLC timer are positively acknowledged in the STATUS report.

DISCUSSION on P2:

- Xiaomi agrees with P2.

DISCUSSION on P3:

- MTK asks when this timer is started.

- LGE explains it should be started when gap is detected. Next details are whether this is per SDU or per entity.

- Huawei, Lenovo agrees with P3, we need new timer.

- MTK think there is an issue because the gap may never happen.

- Vivo thinks we need an indication from Tx.

- Lenovo, Ericsson thinks the timer is started when there is a gap.

- OPPO asks if gap is detected in RLC or PDCP.

DISCUSSION on P4:

- Xiaomi asks whether the intention is to reuse existing SR. LGE confirms.

Special handling to avoid PDCP control PDU discard is not needed.

A new RLC timer at the Rx is introduced to determine obsolete RLC SDUs. The timer starts when the gap is detected at RLC layer.

The abandoned RLC SDUs determined by a new RLC timer are positively acknowledged in the STATUS report.

Timely RLC retransmissions – autonomous versus polling

R2-2410199	More Discussions on RLC AM Enhancements	Ericsson	discussion	Rel-19

Noted

Observation 2	It is important to establish the applicability of autonomous retransmissions in the context of the discard timers.

Observation 3	Autonomous retransmissions are only relevant for short discard timers.

Observation 4	For shorter discard timers, the Tx entity (under gNB control) can continue to perform blind HARQ retransmissions (rather than triggering RLC retransmissions) thereby obtaining a 3 dB gain from soft combining.

Observation 5	For short discard timers, the efficacy of HARQ retransmissions is higher than RLC retransmissions.

Observation 6	Gains with RLC retransmissions are only obtained for long discard timers.

Observation 7	Autonomous retransmissions will be redundant 90% of the time and would consume 2x UL resources per UE, per RLC SDU retransmission. For example, if segmented into 4 AMD PDUs, a UE would consume eight times the UL resources which would exacerbate UL congestion.

Observation 8	The current polling mechanism cannot consider the delay criticality of the AMD PDU in the buffer. It is a trade-off between transmission overhead and timely reception of the RLC status PDU.

Proposal 7	Do not pursue autonomous retransmissions to achieve timely retransmissions.

Proposal 8	Pursue enhancements to the polling mechanism where the delay criticality of the AMD PDU is considered.

R2-2410135	Discussion on timely RLC retransmission(s)	Spreadtrum, UNISOC	discussion	Rel-19

Noted

Observation 1: The too frequent status report triggered by enhanced polling will cause unnecessary RLC retransmission.

Observation 2:  The status report triggered by enhanced polling will bring long RTT.

Observation 3:  The transmission of status report is not reliable.

Proposal 1: Autonomous RLC retransmission is supported.

R2-2409857	Consideration on XR-specific RLC Enhancement	CATT	discussion	Rel-19	NR_XR_Ph3-Core

Proposal 3: Both RLC autonomous retransmission and enhanced Polling could be supported and applied to different remaining delay case if configured.

DISCUSSION on autonomous reTx vs polling:

- MTK agrees with Ericsson analysis. MTK thinks that another enhancement could be to use shorter polling values.

- QCM has different view than Ericsson. QCM thinks autonomous reTx can still be useful, even for longer discard timers. Also, in the field the NW stop HARQ retransmissions after few retransmissions, so RLC reTx could be issued then.

- Apple has a compromise suggestion – if the remaining time drops below a threshold, poll is triggered. If SR is not received within a time, then Tx can autonomously retransmit the delay critical packet.

- Huawei thinks poll is used to trigger SR and SR should trigger retransmission. This increases the delay of reTx. Autnomous reTx is more useful if HARQ retransmissions are over.

- OPPO thinks the main advantage is for short discard timers, so they prefer polling.

- LGE thinks that whether a packet is delay critical is not indicated in SR. Also the SR may not be received timely. LGE thinks both solutions are useful in different scenarios. Another compromise is to have both and NW can decide which to use.

- Lenovo whether auto reTx is useful depends on how the NW operates the link. Lenovo agrees it depends on scheduler implementation. Supports compromise solution from Apple.

- NEC has concern on overhead and delay of polling, prefers auto retx.

- ZTE has sympathy for the analysis from Ericsson.

- Samsung has some variation of the compromise solution 1, so we could leave it open.

- Nokia supports having both solutions and leaving the choice to network.

- QCM also supports option 2 as option 1 is truly a polling enhancement and does not provide gains of autonomous reTx. Option 1 will not provide much gains compared to short polling timer.

- Ericsson prefers compromise from Apple (solution 1).

- Interdigital believes option 2 covers option 1, so it’s better to use it.

- CMCC also prefers option 2 as different network implementations may use different options.

- ZTE is OK with option 2 if UE will always support both together.

- In Apple’s view option 1 covers option 2 if we set polling timer to 0, but it will have much smaller impact to specs. Ericsson agrees.

- LGE agrees with ZTE it should be a single UE capability.

- MTK thinks that option 2 means the UE needs to implement more, so prefers option 1.

- Futurewei thinks we can harmonize both options by having two remaining thresholds.

- Sharp thinks from UE point of view complexity is the same for both options. Option 2 is simply more general.

- Huawei thinks option 1 is a special case of option 2, i.e. when both auto reTx and polling are configured.

Compromise solutions:

- If the remaining time drops below a threshold, poll is triggered. If SR is not received within a time, then Tx can autonomously retransmit the delay critical packet.

- Have both autonomous and polling enhancement.

Timely RLC retransmission solution covers both autonomous retransmission and polling enhancement and NW can configure either or both of them.

Timely RLC retransmissions – polling enhancements

R2-2410777	RLC AM enhancements with small packet delay budget	MediaTek Inc.	discussion	Rel-19	38.322	NR_XR_enh-Core

Proposal 3: RAN2 to adopt enhanced polling mechanism (with smaller values in 38.331) to achieve timely retransmission.

R2-2409882	RLC Enhancements for XR	Samsung	discussion	Rel-19

Proposal 2: Dedicated polling thresholds and the associated counters are introduced for delay-critical RLC SDUs.

Timely RLC retransmissions – autonomous retransmissions

R2-2410409	Discussion on RLC enhancements	InterDigital	discussion	Rel-19	NR_XR_Ph3-Core

Observation 1:	With a proper configuration of the triggering condition for autonomous retransmission, the network can minimize the capacity loss while still guarantee QoS of a service.

Proposal 1:	The UE retransmits a RLC SDU if the remaining time of the SDU is smaller than a configured threshold or the number of HARQ retransmissions is greater than a configured threshold.

R2-2409558	Discussion on RLC enhancements	Qualcomm Incorporated	discussion	Rel-19	NR_XR_Ph3-Core

Proposal 1.	UE autonomously triggers retransmission of a RLC PDU when one of the following conditions are met:

-	after the remaining time of a PDU has dropped below a configured threshold; or

-	after a PDU has failed a configured number of HARQ transmissions; or

-	if a PDU is in the RLC retransmission buffer and there are spare PUSCH resources available after the LCP procedure.

R2-2410155	Discussion on RLC AM enhancements	Huawei, HiSilicon	discussion	Rel-19	NR_XR_Ph3-Core

Proposal 8:	RAN2 works on the autonomous RLC retransmission, where the capacity loss can be mitigated by introducing a transmission gap or considering the channel quality.

R2-2409733	Discussion on RLC re-transmission related enhancements	OPPO	discussion	Rel-19	NR_XR_Ph3-Core

R2-2409771	Discussion on RLC enhancement for XR	vivo	discussion	Rel-19	NR_XR_Ph3-Core

R2-2409798	Discussion on RLC AM Enhancements	CANON Research Centre France	discussion	Rel-19	NR_XR_Ph3-Core

R2-2409819	RLC enhancements	Nokia	discussion	Rel-19	NR_XR_Ph3-Core

R2-2409847	Discussions on RLC enhancements	Fujitsu	discussion	Rel-19	NR_XR_Ph3-Core

R2-2409958	Discussions on Fast RLC Retransmission	Apple	discussion	Rel-19	NR_XR_Ph3-Core

R2-2410036	Discussion on Reply LS to SA2 on Application-Layer FEC Awareness	TCL	discussion

R2-2410246	Discussion on RLC AM enhancements	Huawei, HiSilicon	discussion	Rel-19	NR_XR_Ph3-Core	Withdrawn

R2-2410287	AM RLC enhancement	Lenovo	discussion	Rel-19

R2-2410383	RLC AM enhancements	Sony	discussion	Rel-19	NR_XR_Ph3

R2-2410393	RLC AM enhancement	NEC	discussion	Rel-19	NR_XR_Ph3-Core

R2-2410437	Discussion on RLC AM Enhancements	China Telecom	discussion

R2-2410685	Discussion on RLC enhancements	HONOR	discussion	Rel-19	NR_XR_Ph3-Core

R2-2410786	Discussion on RLC AM Enhancements for XR	Meta	discussion

### 8.7.6	XR rate control

Objective: Specify uplink congestion signaling [RAN2]:

- Specify in MAC layer XR rate control signaling over downlink per QoS flow/per DRB to enable faster source rate adaption to uplink congestion

Including aspects such as: per DRB or per flow indication (including analysis of the impact on QoS enforcement, interworking with L4S etc.), bit rate values indication enhancements, indication/assistance from UE to gNB etc.

Per DRB or per QoS flow indication

R2-2409901	XR Rate Control	Lenovo	discussion	NR_XR_Ph3-Core

Noted

Proposal 1	RAN2 to agree that the Recommended Bitrate indication is on a per DRB level.

Proposal 2	How the UE maps the Recommended Bitrate indication to different QoS flows of a DRB can be left to UE implementation.

R2-2410787	Discussion on RAN Awareness and UL Rate Control for XR	Meta	discussion

Noted

Observation 1: A QoS flow  indication is critical for facilitating the XR application's ability to effectively adapt to network congestion on a flow level and meet QoS requirements.

Observation 2: A QoS flow level indication ensures seamless interworking with existing latency reduction and QoS enhancement techniques.

Observation 3: It is feasible for RAN to estimate the congestion information per-QoS flow.

Proposal 1: Support per QoS flow level indication of MAC layer XR rate control.

R2-2409789	Data rate control for XR applications	ZTE Corporation, Sanechips	discussion

Noted

Observation 1: When multiple-QoS flows are mapped to a single DRB, then per QoS-Flow rate control provides better control over the application rate than a per DRB control

Observation 2: Multi-Modal XR applications may require the network to map multiple (dependent) QoS flows to the same DRB to satisfy synchronization and other dependency related requirements

Observation 3: L4S and RAN based rate control are complimentary in nature and there is no issue with interoperability if they are both enabled regardless of whether RAN based rate control is per QoS flow or per DRB

Observation 4: Having no network control over how the bit rate recommendation is applied by the UE when multiple QoS flows are mapped to the same DRB is restrictive and may result in violation of QoS guarantees in some scenarios

Observation 5: If per QoS flow level granularity is agreed for XR rate control then further work may be needed in RAN3 to enable this in case of CU/DU split architecture.

Proposal 1: Support both (the existing) per DRB level granularity as well as per QoS-flow level granularity for XR rate control

DISCUSSION:

- Ericsson thinks the gNB looses visibility to QoS flow once the flow is mapped to DRB.

- ZTE clarifies that there is already a feature in Rel-18 which relies on per QoS flow level estimation and feasibility was confirmed. LGE agrees, no need to reconfirm.

- LGE would like to have a single solution, not both, prefer per QoS flow.

- Ericsson indicates RAN cannot decide which flow to control, it has no knowledge about the flows. Asks if the UE will follow. Meta suggests checking with L4S team. Ericsson clarifies that with 1:1 mapping they have no issue.

- Huawei, ZTE underlines that 1:1 mapping is not always possible. SA2 is working also on data rate exposure from gNB to CN which is also QoS flow level, so we should align in RAN2.

- ZTE also indicates that ECN marking is per QoS flow, so it does work, it is up to NW implementation.

- Nokia thinks technically per DRB will be OK, does not believe many flows will be mapped to a single DRB.

- Samsung thinks network should decide which flow is throttled as gNB is responsible for QoS enforcement. Proposes that UE can indicate its preference to gNB, but the final decision should be gNB based on both QoS requirements and UE preference.

- Xiaomi thinks per DRB is more aligned with MAC operation which focuses on LCH.

- CATT thinks that flow ID will introduce overhead as we need to indicate PDU session and flow ID.

- QCM recognizes there are different NW implementation and proposes a compromise that both can be signalled.

- Lenovo thinks application has the best knowledge about which flows should be throttled.

- Ericsson is still concerned that application will ignore the indication from the network.

- LGE thinks application layer will not deny, because otherwise the service will deteriorate, the application should follow. Per DRB is harder for app to follow.

- Ericsson asks how we can test the app layer. ZTE thinks the same applies to per DRB.

- QCM asks if we will force the UE to support per QoS flow indication.

- ZTE thinks we can discuss capability later and it should be separate capability of the UE.

- Samsung thinks that there is already QoS profile can already come from CN, so UE indication should be optional.

- Sony, Ericsson thinks that per QoS flow has impact on architecture.

- Apple observed more N:1 mapping than 1:1 mapping in the field, so per QoS flow makes more sense.

- Huawei indicates that QoS flow visibility at DU is already specified in RAN3. OK with the UE indication.

- Ericsson thinks we could configure QoS flows to be throttled in RRC and indicate per LCH.

- Ericsson indicates we should check architecture impacts, e.g. CU/DU split.

RAN2 confirms it is feasible for RAN to estimate the congestion information at both per-DRB and per-QoS flow level.

gNB can be indicated which QoS flows can be throttled. FFS whether this is indicated from UE/CN

Rate indication from gNB to the UE on a per QoS flow level is supported. FFS the details, e.g. if: 1) flows are indicated by MAC CE or 2) by RRC while MAC CE is per DRB.

DL rate control and UL MAC CE

R2-2409637	XR rate control	Xiaomi	discussion	Rel-19	NR_XR_Ph3-Core

Noted

Proposal 6: Rate control can be applicable to UL data as well as to DL data.

Proposal 7: Query indication (similar to Recommended bit rate query) from UE to gNB is supported.

DISUCSSION on P6:

- CATT asks for the motivation. Xiaomi thinks that the other end will be affected, so DL indication is useful.

- LGE thinks the WID is clear that this is out of scope.

- QCM indicates that DL rate control is supported by legacy MAC CE already and it is useful instead of using CN signalling.

- OPPO understand the motivation but it is not in WID.

- ZTE supports downlink. ZTE clarifies that in RAN plenary they thought there is no impact to RAN2.

RAN2 will not discuss/support rate indication for DL unless WID is updated to include it by RANP.

DISCUSSION on P7:

- Ericsson, LGE, OPPO, Nokia thinks the UL MAC CE is not needed. The gNB can know from CN.

- QCM supports P7, it is useful for the UE indicate.

- Vivo thinks that UE can provide assistance on which flow needs rate control.

- Samsung sees some value, UE can share its preference.

- ZTE would like the enhanced feature to be on par with the legacy feature, so prefers to have both DL indication and rate query

- CATT thinks we need to understand in which scenario this rate query is to be applied, e.g. before or after congestion

RAN2 assumes that the congestion situation can be known at the gNB without any indication from the UE

FFS whether UL MAC CE rate query/preference is supported as UE recommendation to the NW or whether legacy MAC CE can serve this already. FFS in which scenarios this is useful.

Agreements on XR rate control - RAN2 confirms it is feasible for RAN to estimate the congestion information at both per-DRB and per-QoS flow level. - gNB can be indicated which QoS flows can be throttled. FFS whether this is indicated from UE/CN - Rate indication from gNB to the UE on a per QoS flow level is supported. FFS the details, e.g. if: 1) flows are indicated by MAC CE or 2) by RRC while MAC CE is per DRB. - RAN2 will not discuss/support rate indication for DL unless WID is updated to include it by RANP. - RAN2 assumes that the congestion situation can be known at the gNB without any indication from the UE - FFS whether UL MAC CE rate query/preference is supported as UE recommendation to the NW or whether legacy MAC CE can serve this already. FFS in which scenarios this is useful.

Assistance information for the gNB

R2-2410039	XR rate control	Nokia, Nokia Shanghai Bell	discussion	NR_XR_Ph3-Core

Not treated

-	Option 1: UE based approach. For example, UE informs gNB of the QoS flow(s)/DRB(s) that are with the capability of adjusting their bit rate.

-	Option 2: CN based approach. For example, gNB gets the information from CN entity (e.g. SMF) of which QoS flow(s) are with the capability of adjusting their bitrate when provided with congestion information.

Proposal 2: For XR rate control, RAN2 discuss different options how the gNB identifies which QoS flows are subject to adaptive bit rate.

R2-2409559	Discussion on XR rate control	Qualcomm Incorporated	discussion	Rel-19	NR_XR_Ph3-Core

R2-2409741	Rate control signaling for XR	LG Electronics Inc.	discussion	Rel-19	NR_XR_Ph3-Core

R2-2409772	Discussion on XR rate control	vivo	discussion	Rel-19	NR_XR_Ph3-Core

R2-2409858	Discussion on XR Rate Control	CATT	discussion	Rel-19	NR_XR_Ph3-Core

R2-2409959	Views on MAC Signalling for XR Rate Control	Apple	discussion	Rel-19	NR_XR_Ph3-Core

R2-2410093	XR Rate Control Enhancements	Sharp	discussion	Rel-19	NR_XR_Ph3-Core

R2-2410156	Discussion on XR rate control	Huawei, HiSilicon	discussion	Rel-19	NR_XR_Ph3-Core

R2-2410191	Uplink rate control for XR	NEC  Corporation	discussion	Rel-19	NR_XR_Ph3-Core

R2-2410200	More on XR Rate Control	Ericsson	discussion	Rel-19

R2-2410240	Discussion on XR rate control	OPPO	discussion	Rel-19	NR_XR_Ph3-Core

R2-2410247	Discussion on XR rate control	Huawei, HiSilicon	discussion	Rel-19	NR_XR_Ph3-Core	Withdrawn

R2-2410318	Further consideration on XR rate control	CMCC	discussion	Rel-19	NR_XR_Ph3-Core

R2-2410410	Discussion on UL congestion signaling	InterDigital	discussion	Rel-19	NR_XR_Ph3-Core

R2-2410490	Recommended bit rate based XR rate control	Sony	discussion	Rel-19	NR_XR_Ph3

R2-2410686	Discussion on XR rate control	HONOR	discussion	Rel-19	NR_XR_Ph3-Core

R2-2410716	Discussion on UL rate control for Rel-19 XR	Samsung	discussion	Rel-19	NR_XR_Ph3-Core

R2-2410739	Discussion on Rate Control for XR	China Telecom	discussion

R2-2410877	Uplink congestion control signalling	MediaTek Inc.	discussion	Rel-19	38.321	NR_XR_enh-Core

## 8.8	NTN for NR Ph3

(NR_NTN_Ph3-Core; leading WG: RAN2; REL-19; WID: RP-241789)

LTE_TN_NR_NTN_mob, leading WG: RAN2, Rel-19 WID: RP-240924)

Time budget: 2 TU

Tdoc Limitation: 3 tdocs

### 8.8.1	Organizational

LS, Rapporteur input, including workplan, etc.

For the LTE_TN_NR_NTN_mob WI, including endorsed draft CRs from the WI spec rapporteurs.

Rapporteur inputs do not count towards the tdoc limitation.

Incoming LSs

- Common TA

R2-2409512	Reply LS on common TA in a regenerative payload scenario (R1-2409258; contact: CMCC)	RAN1	LS in	Rel-19	NR_NTN_Ph3-Core	To:RAN2	Cc:RAN4

Noted

R2-2409522	Reply LS on common TA in a regenerative payload scenario (R4-2416920; contact: CMCC)	RAN4	LS in	Rel-19	NR_NTN_Ph3-Core	To:RAN2	Cc:RAN1

Noted

We to stick to 0 as minimum possible value for TA common without introducing the negative value for ta-Common.

Agreements:

1	We to stick to 0 as minimum possible value for TA common without introducing the negative value for ta-Common.

- MBS broadcast service

R2-2409519	LS on Supporting MBS broadcast service for NR NTN (R3-245844; contact: Xiaomi)	RAN3	LS in	Rel-19	NR_NTN_Ph3-Core	To:RAN2, SA2, CT4

Noted

Draft a reply LS in R2-2410972 based on meeting agreements

[AT128][303][R19 NR NTN] reply LS to RAN3 on MBS broadcast service for NTN (Xiaomi)

Scope: Draft a reply LS to RAN3 on MBS broadcast service for NTN

Intended outcome: Draft LS

Deadline for draft LS (in R2-2410972):  Thursday 2024-11-22 08:00

R2-2410972	Draft reply on Supporting MBS broadcast service for NR NTN (Xiaomi)	LSout	To:RAN3, SA2; Cc: CT4	NR_NTN_Ph3-Core

Remove Draft

Revised in R2-2411196

R2-2411196	Reply on Supporting MBS broadcast service for NR NTN	RAN2	LSout	Rel-19	NR_NTN_Ph3-Core 	To:RAN3, SA2; Cc: CT4

Approved

Rapporteurs’ inputs for LTE_TN_NR_NTN_mob

R2-2409536	Introduction of LTE TN to NR NTN Mobility UE Capability	vivo	CR	Rel-19	36.306	18.3.0	1900	-	B	LTE_TN_NR_NTN_mob-Core

Endorsed

R2-2409669	Introduction of LTE TN to NR NTN IDLE mode mobility	CATT	CR	Rel-19	36.331	18.3.1	5065	-	B	LTE_TN_NR_NTN_mob	R2-2407963	Late

-	CATT indicates this is a clean-up, adding latest agreements

Revised in R2-2410968 to take meeting agreements into account

R2-2410968	Introduction of LTE TN to NR NTN IDLE mode mobility	CATT	CR	Rel-19	36.331	18.3.1	5065	1	B	LTE_TN_NR_NTN_mob	-Core

Endorsed

R2-2409938	Introduction of stage 2 for LTE to NR NTN idle mode mobility	Samsung	CR	Rel-19	36.300	18.3.0	1412	-	B	LTE_TN_NR_NTN_mob

Revised in R2-2410969 to take meeting agreements into account

R2-2410969	Introduction of stage 2 for LTE to NR NTN idle mode mobility	Samsung	CR	Rel-19	36.300	18.3.0	1412	1	B	LTE_TN_NR_NTN_mob

Endorsed (in a further revision we can consider to further align the text for 10.2.XX to 10.2.6)

Rapporteurs’ inputs for NR_NTN_Ph3

R2-2410861	Running RRC CR for NR NTN phase 3	Ericsson	CR	Rel-19	38.331	18.3.0	5192	-	B	NR_NTN_Ph3-Core

### 8.8.2	Downlink coverage enhancements

Contributions should focus on RAN2 aspects of DL coverage enhancements (e.g. cell level / beam level DTX/DRX mechanism, etc.).

Impact with extended SSB periodicity

- Access Control and Cell reselection enhancement

R2-2409666	Discussion on downlink coverage enhancements	LG Electronics Inc.	discussion

- Access Control

Proposal 1	If we need to bar UEs supporting DL-CE, a new bar bit should be introduced in SIB1 to control access of UEs supporting DL-CE.

-	Apple thinks we should refer to UEs not supporting extended SSB periodicity rather than supporting DL-CE. Xiaomi thinks that we should refer to DL-CE for now

-	Ericsson thinks there is no need to kick out UEs that can connect to the network

-	Sequans still thinks we did not discuss whether there is a real need to bar UEs

If we need to bar UEs not supporting DL-CE (via legacy mechanism), we introduce a new barring in SIB1 to be able to selectively bar “UEs supporting DL-CE” (FFS if we will finally refer to “UEs supporting extended SSB periodicity” instead). FFS on the details (e.g. how many bits, etc.)

Proposal 2	The new bar bit shall be present in a cell operating with DL-CE, with code points {barred, notBarred}.

Proposal 3	If the new bar bit is absent in SIB1, UE supporting DL-CE considers that the cell does not support DL-CE and follows cellBarredNTN.

- Cell reselection enhancement

Proposal 4	Network can use the existing neighbor cell list, such as ExcludedCellList in SIB3,4 and ntn-NeighCellConfigList in SIB19, to prevent UEs not supporting DL-CE from reselecting DL-CE cells, without backward compatibility issue.

Proposal 5	RAN2 should introduce a new neighbor cell list, either/both a new ExcludedCellList in SIB3,4 or/and a new ntn-NeighCellConfigList in SIB19, to allow UEs supporting DL-CE to reselect DL-CE cells.

Proposal 6	If new ExcludedCellList for DL-CE is present, UEs supporting DL-CE only consider new ExcludedCellList for DL-CE and ignore existing ExcludedCellList.

Proposal 7	If new ntn-NeighCellConfigList for DL-CE is present, UEs supporting DL-CE consider both new ntn-NeighCellConfigList for DL-CE and existing ntn-NeighCellConfigList.

- SMTC impacts

Observation 1	The decision regarding the extension of the SSB periodicity is still ongoing and there is no progress on beam hopping in RAN1.

Observation 2	If the network properly configures beam hopping for SSB transmission, the current number of SMTCs will be sufficient for neighbor cells measurement.

Proposal 8	From RAN2's perspective, there is no SMTC issue related to beam hopping or SSB periodicity extension unless further input is provided by RAN1.

- DTX

Proposal 9	RAN2 postpones the discussion on DTX operation until it receives sufficient input from RAN1 regarding the DTX operation.

R2-2410267	Cell barring and reselection for NTN DL-CE	Lenovo	discussion	Rel-19

- Access control

Proposal 1: RAN2 to discuss the following options of cell barring for an NTN-capable UE supporting DL-CE.

Option 1: [Explicit] By a new cell barring indication cellBarredNTN_DLCE with configurable value “barred” and “not barred”.

Option 2A: [Explicit] By presence of a new flag indication cellBarredNTN_DLCE.

Option 2B: [Explicit] By presence of a new flag indication cellAllowedNTN_DLCE is introduced.

Option 3: [Implicit] By presence of DL-CE configuration (e.g., SSB periodicity extension larger than 160ms).

Proposal 2: RAN2 to consider cell barring enhancement based on the validity duration of cell barring status for NTN DL-CE.

- Cell reselection enhancement

Proposal 3: If a neighboring NTN cell operating in DL-CE is considered as not barred, an NTN-capable UE supporting DL-CE prioritizes the frequency of the cell for cell reselection.

Proposal 4: If a neighboring NTN cell operating in DL-CE is considered as barred, an NTN-capable UE supporting DL-CE considers the cell as barred or deprioritizes the frequency of the cell for cell reselection.

Proposal 5: If a neighboring NTN cell operating in DL-CE is considered as barred, an NTN-capable UE NOT supporting DL-CE considers the cell as barred or deprioritizes the frequency of the cell for cell reselection.

R2-2409821	Discussion on the DL coverage enhancement at system level	Google Ireland Limited	discussion	Rel-19	NR_NTN_Ph3-Core

- The necessity of extending the SSB periodicity:

Observation 1	The increased SSB periodicity due to the beam hopping can be known to the UE based on the original SSB periodicity (i.e., the SSB periodicity before beam hopping) plus the DTX configuration.

Observation 2	The existing values for ssb-periodicityServingCell, SSB-MTC, SSB-MTC4, and mgrp can remain the same if the DTX pattern is signaled to support the beam hopping mechanism.

Observation 3	The DTX configuration offers more flexible beam hopping options, due to the configurable ON duration in the DTX configuration.

Proposal 1	RAN2 neither extends the values for ssb-periodicityServingCell (and other SSB related parameters) nor extends the SMTC list, but introduces the DTX configuration instead to support the system level DL coverage enhancement.

- The provision of the DTX configuration:

Observation 4	The beam hopping mechanism may have the impact to the UE in idle/inactive state, as when the UE wakes up at its paging occasions, it may not detect anything including the SSB burst.

Proposal 2	The DTX configuration can be provided in the system information for idle/inactive UEs, and covers the Tx state where the beam/cell is not transmitting anything.

Observation 5	Providing beam DTX configuration can support most of the mapping options between the satellite beam and the NR beam/cell, and the cell DTX configuration can be considered as a special case of the beam DTX configure.

Proposal 3	The DTX configuration can be a beam-specific configuration, and can be associated to an SSB index.

Proposal 4	If a DTX configuration is not associated with an SSB index, it is applicable for the entire cell.

- The impact on the legacy UEs:

Observation 6	It is unlikely the legacy UE can obtain a meaningful measurement result from the cell operating with DL-CE per Tmeasure,NR_Inter or Tmeasure,NR_Intra.

Proposal 5	RAN2 neither introduces a new barring mechanism to bar the Rel-19 UE supporting DL-CE, nor introduces a new method for the UE not supporting DL-CE to down-prioritize the cell reselection to cells operating with DL-CE.

- The impact on the RRC_CONNECTED UE:

Proposal 6	The connected UE is not required to monitor the PDCCH, perform the RLM/BFD, and perform the serving cell measurement during the OFF periods configured by the DTX configuration.

Proposal 7	The connected UE is expected to receive a c-DRX configuration whose the cycle length is equal to one or multiple times of the DTX cycle length.

R2-2410549	Downlink coverage enhancement for NTN	InterDigital	discussion	Rel-19	NR_NTN_Ph3-Core

- Access Control

Proposal 1a:	A new barring bit is introduced to control access of UEs supporting Rel-19 NTN DL-CE.

Proposal 1b:	Confirm that UEs supporting Rel-19 NTN DL-CE will not consider the existing NTN barring bit.

- Cell reselection enhancement

Proposal 2:	Define absolute priority information for DL-CE capable UEs.

- Dynamic power sharing

Proposal 3:	UE can be notified of changes to beam power sharing/pattern/size for both serving and neighbouring cell(s).

Proposal 4:	Satellite assistance information (e.g., within SIB19) can be used to predict future changes to beam power sharing/pattern/size.

Proposal 5:	UE can consider a future change to beam power sharing/pattern/size during mobility and cell (re)selection.

Agreements:

1.	If we need to bar UEs not supporting DL-CE (via legacy mechanism), we introduce a new barring in SIB1 to be able to selectively bar “UEs supporting DL-CE” (FFS if we will finally refer to “UEs supporting extended SSB periodicity” instead). FFS on the details (e.g. how many bits, etc.)

- SMTC impacts

R2-2410804	Downlink coverage enhancement SMTC impacts	Sequans Communications	discussion	Rel-19	NR_NTN_Ph3-Core	R2-2408970

Proposal 1: RAN2 assumes that the number of configured SMTCs may increase due to beam-hopping / larger SSB periodicity (depending on RAN1/4 solution)

-	LG thinks the discussion on additional SMTC is up to other groups and then thinks we don’t need to take this assumption for now

-	Google is not sure we need to extend the number of STMCs even if larger periodicity will be used

-	Nokia supports this proposal and thinks this is also consistent with the options being considered by RAN1

-	Sequans thinks there could be the need for multiple SMTCs per frequency layer and this is the baseline scenario that we would have to consider

-	QC is not sure why we need to increase the number of SMTCs

Proposal 2: RAN2 investigates related additional power consumption impact (including legacy UEs)

As part of the work on DL-CE, RAN2 investigates related UE power consumption impact (including legacy UEs)

Proposal 3: RAN2 considers signaling DL CE cells SMTCs separately (e.g. new SMTC5 list)

-	Sequans thinks that if we don’t update the signalling then we only have one periodicity

RAN2 will consider whether to introduce separate signalling (e.g. new SMTC5 list) for DL CE cells SMTCs, e.g. if different periodicities need to be signalled or to prevent reselection to specific cells

Observation 1: The UE should ideally consider only SMTCs corresponding to the closest neighbor cells

Proposal 4: Consider enhancements to allow location-based UE SMTC selection (broadcast of neighbor cells reference location / SSB indexes associated with SMTCs)

-	Nokia agrees but thinks we could work on a UE-centric solution

-	QC thinks there is no need to capture anything for this at this stage. LG, CATT, Samsung agree

Can come back in the next meeting

Observation 2: In connected, NW may not have UE location information needed to configure SMTCs to UE

Observation 3: R18 location-based CHO without associated measurement avoids SMTCs configuration, but adds complexity and may increase handover interruption time

Proposal 5: Consider location-based UE SMTC selection also in connected

R2-2410213	Discussion on NTN downlink coverage enhancement	Nokia	discussion	NR_NTN_Ph3-Core	R2-2408699

- RAN1 reply related

Proposal 1: RAN2 should refrain from using the term “Beam hopping” and instead focus on defining terms related to beam footprints and their activity.

Proposal 2: RAN2 to focus on defining signals and procedures that allows for the situation of a satellite not being able to have all satellite beams simultaneously active.

- MIB impact

Proposal 3: RAN2 to include in the study the impact on MIB transmission and reception due to the SSB periodicity extension e.g. allowing MIB periodicity to follow SSB even for periods lower than 80 ms, and to prevent a joint configuration of CORESET multiplexing pattern 1 together with SSB periods longer than 20 ms.

- SMTC impacts

Observation 1: SMTC configuration using the existing number of SMTC windows is not possible.

Observation 2: Configuring SMTC windows covering group of SSB measurements will be very energy inefficient.

Proposal 4: RAN2 to assume that there is no distinct pattern to be configured e.g. for SMTC windows and at least the number of SMTC windows must be extended.

Observation 3: SMTC window configuration of all possible SSB transmissions of neighbouring cells is energy inefficient.

Observation 4: Location based SMTC configuration may be a solution to optimising the energy consumption of an NR NTN UE performing neighbour cell measurements on a subset of the configured SMTC windows.

Observation 5: Location based SMTC configuration provide additional signalling overhead and requires a UE to be aware of the relative location between not only the gNB but also all neighbouring cells.

Observation 6: Optimal SMTC window configuration may be dynamic for a given UE if we account not only for SSB transmission but also data transmission.

Proposal 5: RAN2 to study at least four aspects of downlink coverage enhancements

•	Increasing the number and duration of SMTC windows;

•	Allowing a UE can do a scan for neighboring cells over a period and to limit the measurements afterwards to the relevant cells

•	Setting the SSB period to a lower value during mobility events in case of Earth Fixed Cells.

- Access Control

Observation 7: The need for a barring bit may also depend on whether RAN1 believes the MIB can be read by non DL-CE enabled UEs, but this is up to RAN1

Proposal 6: RAN2 ask RAN1 whether the MIB can be read by non DL-CE enabled UEs.

Agreements:

1.	As part of the work on DL-CE, RAN2 investigates related UE power consumption impact (including legacy UEs)

2.	RAN2 will consider whether to introduce separate signalling (e.g. new SMTC5 list) for DL CE cells SMTCs, e.g. if different periodicities need to be signalled or to prevent reselection to specific cells

Cell DTX/ Beam DTX

R2-2410066	Discussion on DL coverage enhancement	Xiaomi	discussion	Rel-19	NR_NTN_Ph3-Core

- Cell DTX/DRX

Proposal 1: for cell level DTX/DRX for NTN DL coverage enhancement, same as cell DTX/DRX for NES, cell transmission/reception state is modelled into two states: "cell DTX/DRX Active period" state and "Cell DTX/DRX inactive period" state.

-	ZTE understands the intention is not to introduce other states and thinks we’d better refer to On and Off rather than Active/Inactive. QC agrees

-	HW wonders if the suggestion is to reuse what we have for NES or to add another variant. Samsung wonders the same

-	Ericsson thinks that RAN1 has not decided yet whether to reuse NES or not

For cell level DTX for NTN DL coverage enhancement, RAN2 currently understands that there will be only two states: "cell DTX On period" and "Cell DTX Off period" (FFS if the UE behaviour will be different from the NES states or the same). FFS on Cell DRX states.

Proposal 2: If beam level DTX/DRX for NTN DL coverage enhancement is supported, beam transmission/reception state is modelled into two states: "beam DTX/DRX Active period" state and "beam DTX/DRX inactive period" state.

Continue in the next meeting

Proposal 3: During cell/beam level DTX inactive period for NTN DL coverage enhancement, UE is not expected to receive any signal from the cell/beam.

Proposal 4: During cell/beam level DRX inactive period for NTN DL coverage enhancement, UE is not expected to receive any signal from the cell/beam.

Proposal 5: The configuration for cell level DTX/DRX for NTN DL coverage enhancement should be separate from cell DTX/DRX configuration for NES.

- Access Control

Proposal 6: For intra-freq/inter-freq cell reselection, introduce a DL CE UE specific intraFreqExcludedCellList/interFreqExcludedCellList.

Proposal 7: For inter-freq cell reselection, introduce a DL CE UE specific interFreqCarrierFreqList.

- LL coverage enhancements

Proposal 8: If msg4 PDSCH repetition is agreed to support, Msg3 is used to carry the indication of msg4 PDSCH repetition capability/request.

R2-2410671	RAN2 Impact on DL coverage enhancements	CMCC	discussion	Rel-19	NR_NTN_Ph3-Core

Proposal 1: for senario1, i.e. Different PCIs for mutiple beam footprints in one satellite cell (1-to-1 mapping), the current periodic cell DTX/DRX pattern (i.e. active and non-active periods) in Rel-18 NES for reducing gNB downlink transmission/uplink reception active time seems can be utilized for the DL system level enhancement (i.e., beam hopping) use case.

Proposal 2: for senario2, i.e. Single PCI for Multi-beam footprints in one satellite cell (multiple-to-1 mapping), the current periodic cell DTX/DRX pattern (i.e. active and non-active periods) in Rel-18 NES for reducing gNB downlink transmission/uplink reception active time seems can be enhanced for the DL system level enhancement (i.e., beam hopping) use case:

Alt 1. Extend the granularity of  periodic cell DTX/DRX pattern in NES from cell to beam footprint

Alt 2. Reuse current periodic cell DTX/DRX pattern with more domain information, e.g. beam footprint or geographical information

Proposal 3: As for NES, no impact to idle UE, e.g., RACH, paging, SIBs and SSB transmission, it is proposed to define different UE behavior is different in the non-actvie status when reusing the current periodic cell DTX/DRX pattern (i.e. active and non-active periods) in Rel-18 NES.

Agreements:

1.	For cell level DTX for NTN DL coverage enhancement, RAN2 currently understands that there will be only two states: "cell DTX On period" and "Cell DTX Off period" (FFS if the UE behaviour will be different from the NES states or the same). FFS on Cell DRX states.

R2-2409671	Further discussion on downlink coverage enhancements	CATT	discussion

R2-2409848	Discussions on cell DTX during satellite dynamic power sharing	Fujitsu	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2409978	DL coverage enhancement in NTN	Apple	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2410048	Discussion on cell DTX	Qualcomm Incorporated	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2410107	NR NTN coverage enhancement	China Telecom	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2410122	Discussion on DL coverage enhancement for NTN	OPPO	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2410293	Consideration on downlink coverage enhancement	NEC Corporation	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2410365	Consideration on downlink coverage enhancements	ZTE Corporation, Sanechips	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2410386	SMTC impacts due to NTN downlink coverage enhancements	Sony	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2410525	Discussion on Downlink Coverage Enhancement	Samsung	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2410677	NTN DL coverage enhancements	NERCDTV	discussion

R2-2410691	Discussions on downlink coverage enhancement	HONOR	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2410699	Discussion on Downlink Coverage Enhancements	Sharp	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2410715	Discussion on downlink coverage enhancements in NR NTN	ETRI, Korea University	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2410806	Downlink coverage enhancement access control	Sequans Communications	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2410870	Discussion on RAN2 Aspects for system-level Downlink Coverage enhancements	THALES	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2410881	DL coverage enhancements	Ericsson	discussion	Rel-19	IoT_NTN_Ph3-Core

### 8.8.3	Uplink Capacity/Throughput Enhancement

Contributions can be submitted on the possible RAN2 aspects of the agreements reached in RAN1.

R2-2410434	Discussion on Uplink Capacity Enhancements	Huawei, HiSilicon, Turkcell	discussion	Rel-19	NR_NTN_Ph3-Core

Observation 1: If RAN1 agrees on Option 1 (inter-slot with OCC length 4 to multiplex up to 4 UEs), OCC scheme does not need to be indicated to the UE, since only one OCC scheme (i.e. inter-slot OCC) will be supported.

Observation 2: If RAN1 agrees on Option 3 (combination of Inter-slot OCC with OCC length 2 and intra-symbol pre-DFT OCC with OCC length 2 to multiplex up to 4 UEs), OCC length does not need to be indicated to the UE, since only OCC length 2 is supported.

Observation 3: Regardless of which option is adopted by RAN1, OCC index (i.e. the line index of the OCC matrix) needs to be indicated to the UE.

Proposal 1: At least OCC index (i.e., the line index of the OCC matrix) needs to be indicated to the UE, RAN2 will further consider whether OCC scheme or OCC length is also indicated to the UE depending on RAN1 progress.

Proposal 2: For configured grant Type 1, OCC configuration (i.e. at least OCC index) is provided to the UE via RRC signalling. FFS what OCC configuration should be provided for dynamic grant or configured grant type 2.

Proposal 3: RAN2 introduce UE capability for OCC support. Whether separate UE capabilities are used for different OCC length/OCC scheme is pending RAN1 agreement.

R2-2410067	Discussion on uplink capacity enhancement	Xiaomi	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2410330	Discussion on uplink capacity/throughput enhancement for NR NTN	CMCC	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2410367	Consideration on NTN miscellaneous issues	ZTE Corporation, Sanechips	discussion	Rel-19	NR_NTN_Ph3-Core

### 8.8.4	Support of Broadcast service

Contributions should address the signaling of the intended service area of a broadcast service.

Moved here from 8.8.1

R2-2410638	Discussion on the reply LS to RAN3 on supporting MBS broadcast service for NR NTN	Xiaomi	discussion

Proposal 1: The encoding of TN coverage introduced in Rel-18 in TS38.331, including tn-ReferenceLocation-r18 and tn-DistanceRadius-r18, is reused for the geographical area of the circle.

Agreed

Proposal 2: The encoding of Polygon in TS37.355 is reused for the geographical area of the Polygon.

-	LG supports this. ZTE also supports this

Agreed

Proposal 3: The IntendedServiceArea can be considered as the IE name of the geographical area.

-	LG suggests to add MBMS in front

Agreed (we can still update the name in the CR implementation if needed)

Proposal 4: If the proposals 1-3 are agreed, the draft reply LS in the annex can be considered.

Draft a reply LS to RAN3 and SA2 in offline 303

R2-2410526	Discussion on MBS Broadcast Service Intended Area	Samsung	discussion	Rel-19	NR_NTN_Ph3-Core

- Signalling of intended service area

Observation 1: The geographic area information for the intended service areas is assumed to be semi-static which does not cause frequent SIB update.

RAN2 understands that the geographic area information for the intended service areas can be semi-static and not cause frequent updates.

Proposal 1: Introduce a new SIB to include a list of intended service areas and related IDs.

-	CATT could agree on this if we no longer discuss about possible misalignment between SIB and MCCH. Samsung thinks that if the information is semi-static there should be no issues

-	IDC supports this also to limit the usage of radio resources. CATT thinks the problem will be similar with SIB

-	Oppo prefers a MCCH-based solution as the UE would only have to read MCCH

-	Xiaomi thinks that the UE could skip MCCH reading also with a MCCH based solution.

-	Samsung thinks that using MCCH also might have an impact on legacy UEs

-	LG thinks we don’t necessarily need to use IDs but can rely on bitstrings

-	CATT thinks we could use the list index rather than an ID

-	ZTE thinks we need IDs to refer to multiple intendedServiceAreas.

Introduce a new SIB to include a list of intended service areas and related pointer (FFS if we point to the intended services areas via the index in the list or with an ID or another way)

Proposal 2: The legacy SIB modification procedure is applied to update the intended service area information in the new SIB.

Agreed

Proposal 3: Circles and polygons are used to describe an intended area only in the case that the intended service area does not cover a whole cell. If any service is provided for the whole cell, the cell ID can be indicated and associated to the intended service area ID.

Proposal 4: Discuss for earth moving cell how to avoid NW frequent update and UE frequent acquisition of information on MBS broadcast service and the intended area.

- Service Continuity

Proposal 5: Discuss how an intended service area is provided.

Option 1: An intended service area for a MBS BC service includes all geographic areas across the current serving cell and neighbor cell(s), and NW ensures the current serving and all neighbor cells that provide the same service signal the same intended service area information.

Option 2: For a MBS BC service, the current serving cell provides the intended service area only within the current cell.

Option 3: Up to NW implementation, i.e., both option 1 and 2 are possible.

Option 2: For a MBS BC service, the current serving cell provides the intended service area only within the current cell.

A signalled intended service area for a MBS BC service may include geographic areas across the current serving cell and overlapping neighbor cell(s).

Proposal 6: For option 2 and 3, consider to support service continuity by signaling the mapping between intended service area and frequency/neighbor cell.

R2-2410331	Considerations on broadcast service for NR NTN	CMCC	discussion	Rel-19	NR_NTN_Ph3-Core

- Intended service areas

Proposal 1: Using MCCH based solution to include the intended service areas (and related IDs) information.

Proposal 2: A new SIB should be considered last, unless the MCCH based solution is determined unworkable.

Proposal 3: It could be also up to UE implementation to allow UE skipping MCCH re-acquisition when UE is not within intended service area of any interested broadcast service based on the intended service areas provision by network.

Proposal 4: Not to pursue the MBS service announcement solution in R19.

- MCCH information update

Proposal 5: For MCCH update due to service area information changing, we could consider to leverage the existing mechanism and just add some clarification under “The LSB in the 2-bit bitmap” part to include the service area information changing (e.g. service areas modification).

- Service continuity

Proposal 6: It is proposed to provide intended service area within each neighbor cell for each MBS broadcast service session in MBSBroadcastConfiguration.

Proposal 7: It is proposed to provide intended service area for each MBS broadcast service session in SIB21.

- MBS interest indication

Proposal 8: It is proposed not to pursue that a R19 UE supporting the feature may initiate the MBS Interest Indication procedure when UE is entering or leaving the intended area.

- Signaling format of service area

Proposal 9: For the referenceLocation + radius solution, the following new IE format could be discussed:

IntendedServiceAreaInfo-r19 ::=       SEQUENCE {

intendedServiceAreaId-r19                  IntendedServiceAreaId-r19,

intendedServiceArea-ReferenceLocation-r19       ReferenceLocation-r17,

intendedServiceArea-DistanceRadius-r19          INTEGER(0..65536)

Proposal 10: It is proposed to use “CHOICE” signaling structure to support both the service area format solutions.

R2-2410862	Support for broadcast services in NR NTN	Ericsson	discussion	Rel-19	NR_NTN_Ph3-Core

Proposal 1	MBS Service Announcement is used to indicate the intended service area for MBS broadcast in NTN cells.

Proposal 2	If the ISA is to be sent using AS-protocols, the definition of the intended service area is provided in a new SIB.

Proposal 3	If ISA is sent in SIB, the ISA ID is provided in the MBS Service Announcement to facilitate MCCH acquisition relaxation. RAN2 sends an LS to SA2.

-	Apple wonders if this means that ISA ID should become a globally unique identifier

-	ZTE sees some benefit and is fine to further discuss this

-	CATT thinks it’s late to add more work to SA2/CT1 for this

-	Samsung thinks we can address the issue to avoid reading MCCH when not needed directly in RAN2 without involving other groups

-	Apple/Xiaomi think this would require coordination between RAN and CN and then involve other groups. Ericsson thinks the coordination will be there

Continue the discussion in the next meeting

Proposal 4	A UE interested in an MBS broadcast service may skip MCCH acquisition when it is located outside the intended service area where this service is provided.

Proposal 5	A UE may use, by implementation, its location, the ISA definition in SIB, and mapping between ISA ID and MBS Service ID (TMGI) in the USD/Service Announcement to decide whether to skip MCCH acquisition.

Proposal 6	RAN2 focuses on service continuity for the scenario where the quasi-Earth-fixed cells are replaced due to satellite movement.

Proposal 7	In a quasi-Earth fixed cell, the network provides information about the MBS broadcast configuration and services of the upcoming satellite that will serve the area.

Proposal 8	Introduce geographic information in ETWS notification for geo-fencing in NTN cells.

R2-2410521	Discussion on MBS broadcast over NTN	Huawei, HiSilicon, Turkcell	discussion	Rel-19	NR_NTN_Ph3-Core

Proposal 1: Provide the intended service area ID(s) via SIB20 for the power saving purpose.

Proposal 2: Both current serving cell and neighbour cells MBS session specific intended area ID(s) should be indicated in the MCCH.

Proposal 3: Based on the neighbour cells MBS session specific intended area, the UE can request unicast reception of the service of interestif moving to a cell not providing the MBS session in the UE location area.

Proposal 4: The UE can decide to consider an MBS frequency as the highest priority frequency for cell reselection based on its location and on the intended area info/ID(s) provided per MBS frequency via SIB21 .

Proposal 5: The intended service area (composed of one or multiple circles or polygons) is provided in a new SIB, along with an area ID for each area. The area ID is used in all places requiring service area info, as mentioned in previous proposals.

R2-2410871	Discussion on MBS Broadcast service area signaling	THALES	discussion	Rel-19	NR_NTN_Ph3-Core	R2-2408488

Proposal 1		RAN2 focus on defining a RAN-level solution to indicate the intended service area for MBS broadcast service in NR NTN system

<Location-dependent MBS broadcast and Service Announcement>

Proposal 2a		The MBS Service Announcement can be used for NTN MBS broadcast but is not sufficient for service area smaller than a cell.

Proposal 2b	The MBS Service Announcement using the target location area to indicate the intended service area for MBS broadcast in NTN cells cannot be used as it.

Proposal 2c		If MBS Service Announcement is adopted for location dependent MBS broadcast in NTN, a location description based on geographical coordinates should be defined. FFS on 5G MBS architecture impacts.

Proposal 3		If a RAN-level mechanism is defined for geo-filtering MBS services, Service Announcement with location for location-dependent MBS broadcast service may be unnecessary but the information could be provided to the MBS-aware application to know the MBS service is limited to the current geographical area. FFS how to align the geographical information broadcasted in the cell and the geographical information in the Service Announcement

<Intended service area signaling in MCCH>

Proposal 4		The MBS intended service area should be restricted to NTN cells for NTN-capable UEs.

Proposal 5		The intended service area geographical coordinates are included in the MCCH.

<UE skipping MCCH (re-)acquisition enhancement>

Proposal 6a		A non-related to a MBS session unique MBS broadcast services area is included in SIB20, corresponding to the sum of MBS broadcast services area signaled in the MCCH to allow user outside this zone to skip the (re-)acquisition of the MCCH.

Proposal 6b	By default, if the SIB20 does not contain an intended service area, then the whole NTN cell is covered by at least one MBS broadcast service and the UE is needed to acquire MCCH for MBS session related intended service area.

Proposal 6c		It comes if the SIB20 is not broadcasted, then there is not MBS broadcast service in the NTN cell.

<Geographical Area coding>

Observation 7	SIB8 for CMAS signaling uses the Warning Area Coordinated IE to encode geographical region

Proposal 7		Re-use the Warning Area Coordinates coding for MBS Broadcast service intended service area

Proposal 8		Consider the MBS-IntendedServiceAreaCoordinates presented in Table 1 to describe MBS broadcast service intended area for Rel-19 NTN.

<Service continuity and UE behaviour>

Proposal 9		RAN2 will not define the UE behaviour for MBS Interest Indication and the left how to handle it to the network implementation.

Proposal 10		The intended service area ID associated to MBS FSAI in SIB21 could be beneficial for MBS service continuity.

Agreements:

1.	The encoding of TN coverage introduced in Rel-18 in TS38.331, including tn-ReferenceLocation-r18 and tn-DistanceRadius-r18, is reused for the geographical area of the circle.

2.	The encoding of Polygon in TS37.355 is reused for the geographical area of the Polygon.

3.	The IntendedServiceArea is considered as the IE name of the geographical area (we can still update the name in the CR implementation if needed)

4.	A signalled intended service area for a MBS BC service may include geographic areas across the current serving cell and overlapping neighbor cell(s).

5.	RAN2 understands that the geographic area information for the intended service areas can be semi-static and not cause frequent updates.

6.	Introduce a new SIB to include a list of intended service areas and related pointer (FFS if we point to the intended services areas via the index in the list or with an ID or another way)

7.	The legacy SIB modification procedure is applied to update the intended service area information in the new SIB.

R2-2409537	Further Discussion on MBS Broadcast Service Area Provision	vivo	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2409538	Discussion on MBS Broadcast Service Continuity in NTN	vivo	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2409615	Discussion on support of broadcast service in NTN	LG Electronics France	discussion	Rel-19	38.331	NR_NTN_Ph3

R2-2409670	Further discussion on support of broadcast service in NR NTN	CATT, CBN	discussion

R2-2409849	Discussions on supporting broadcast intended to serve partial cell	Fujitsu	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2409893	Discussion on providing MBS service area in NTN network	OPPO	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2409977	Intended broadcast service area provision over NTN	Apple	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2410011	Discussion on intended service area signalling	NEC	discussion

R2-2410046	Signaling of MBS broadcast service area information	Qualcomm Incorporated	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2410108	Broadcast service area information in NR NTN	China Telecom	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2410231	Discussions on configuration of intended service areas	ITRI	discussion	NR_NTN_Ph3-Core

R2-2410268	NTN broadcast service area indication and service continuity	Lenovo	discussion	Rel-19

R2-2410366	Consideration on broadcast service enhancements	ZTE Corporation, Sanechips	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2410440	On the Open Points for Supporting MBS in Rel-19 NR NTN	Nokia, Nokia Shanghai Bell	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2410597	Support for broadcast service in non-terrestrial networks	InterDigital, Inc.	discussion	Rel-19

R2-2410639	Discussion on the support of broadcast service	Xiaomi	discussion

R2-2410662	Further Discussion on Support of MBS Broadcasting over NTN 	TCL	discussion

R2-2410700	Discussion on UE behaviours within intended service area	Sharp	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2410713	Discussion on the support of broadcast service in NR-NTN	ETRI	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2410764	Further considerations on intended broadcast service area provision	Continental Automotive	discussion	Rel-19

### 8.8.5	Support of regenerative payload

Contributions, if any, should focus on the needed updates for Stage 2 description and on whether any other existing essential features (not considered so far) would be affected - and potentially need any modifications - in a regenerative payload architecture.

R2-2410880	Regenerative payload	Ericsson	discussion	Rel-19	IoT_NTN_Ph3-Core

Observation 1	The timing and synchronization in 38.300 section 16.14.2.1 is out of RAN3’s scope.

Observation 2	RAN3 has endorsed a baseline CR for 38.300 that includes almost all changes needed.

Proposal 1	Clarify that the 38.300 figure in 16.14.2.1 is for transparent payload.

Proposal 2	Consider the text proposal in section 4.

Proposal 3	Specific configurations of common TA and Kmac in regenerative architecture are not captured in Stage 2.

R2-2410590	Discussion on regenerative payload	Huawei, HiSilicon, Turkcell	discussion	Rel-19	NR_NTN_Ph3-Core

Observation 1: Neither RAN1 nor RAN4 reached any consensus on introducing negative values for common TA parameters.

Proposal 1: Capture in TS 38.300 for regenerative payload, the RP is located at the satellite gNB and the value of common TA and Kmac can be configured as zero.

R2-2409981	Regenerative payload for NTN for NR Ph3	TOYOTA Info Technology Center	discussion

R2-2410123	Discussion on satellite switch with resynch for regenerative payload	OPPO	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2410269	UE location verification in NTN regenerative architecture	Lenovo	discussion	Rel-19

R2-2410384	Satellite switch with re-sync in regenerative payload	Sony	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2410649	Discussion on regenerative payload	Fujitsu Limited	discussion	Rel-19	NR_NTN_Ph3-Core

### 8.8.6	LTE to NR NTN mobility

Contributions, if any, should focus on any possible missing aspects for the support of idle mode mobility between LTE and NR NTN.

R2-2410047	Frequency priorities and redirection from LTE to NR NTN	Qualcomm Incorporated	discussion	Rel-19	LTE_TN_NR_NTN_mob

Proposal 1	Clarify that the UE ignores the dedicated frequency priority for NR NTN frequency that is associated with satellite ID.

Proposal 2	Clarify that the EUTRA network does not include NR NTN frequency in redirectedCarrierInfo in the RRCConnectionRelease message.

-	QC thinks that at least for legacy UEs (R17 and R18) we cannot have any different behaviour, but we can discuss for R19

-	Samsung thinks basic redirection should be supported without any capability. But we can discuss capability for enhancements. Xiaomi agrees.

-	HW thinks that redirection is not part of this WI. LG also agrees

-	ZTE wonders about the use case.

-	HW wonders where we would put the UE capability in case we introduce it. Samsung thinks this would be conveyed in the NR capability. QC/CATT do not agree.

RAN2 understands that the UE behaviour is currently not clear if the network includes a NR NTN frequency in redirectedCarrierInfo in the RRCConnectionRelease message in a TN cell. We can further discuss whether we want to clarify the UE behaviour and in case whether we need to introduce a new UE capability.

R2-2410640	Discussion on the NR NTN frequency configured by the RRC connection release message	Xiaomi	discussion

Proposal 1: If the NR NTN frequencies configured by the RRC connection release message are also included in the SIB24, the corresponding satellite information in SIB33 is used for the redirection and cell reselection (UE specific frequency priority).

Proposal 2: If the NR NTN frequencies configured by the RRC connection release message is not included in the SIB24, the satellite information (e.g., satellite ID) is configured by the RRC connection release message for the redirection and cell reselection (UE specific frequency priority).

Proposal 3: Introduce a UE capability to support receiving the NR NTN frequency and satellite information (e.g., satellite ID) by the RRC connection release message.

R2-2410485	Further issues on E-UTRAN to NR NTN idle mode mobility	Samsung	discussion	Rel-19	NR_NTN_Ph3-Core

Proposal 1: E-UTRAN to NR NTN re-direction via RRCConnectionRelease is considered supported if UE supports Rel-17 NTN and no new UE capability is required for this.

Proposal 2: RAN2 can discuss new assistance information for E-UTRAN to NR NTN redirection, for which a new capability can be considered to be required.

Proposal 3: Polarization should not be signalled per satellite in satelliteInfoList-r19.

-	vivo thinks the current model is ok

Proposal 4: RAN2 to discuss how the polarization mode can be signalled per cell or whether it is needed at all.

Proposal 5: In regenerative payload scenario, RAN2 confirms common TA value is 0.

R2-2410394	Clarification on NR Satellite Info Provision	NEC	discussion	Rel-19	NR_NTN_Ph3-Core

Proposal 1: RAN2 confirm the idle mode mobility from IOT NTN to NR NTN is not supported

Proposal 2: Clarify NR satellite info provision (IE NeighSatelliteInfoNR-r19) is only signalled in a LTE TN cell but not NTN cell

Add a clarification in Stage 2 that only LTE to NR NTN is supported (and not eMTC NTN to NR NTN). Also add in the field description that NeighSatelliteInfoNR-r19 is only signalled in a LTE TN cell but not IoT NTN cell

R2-2409539	Discussion on Redirection from E-UTRA TN to NR-NTN	vivo	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2409672	Discussion on remaing issue for LTE TN to NR NTN IDLE mode mobility	CATT	discussion

R2-2409982	Improvements_to_LTE-TN-to-NTN_mobility	PANASONIC	discussion	Rel-19

R2-2410109	Remaining issue for NTN mobility redirection	China Telecom	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2410439	On LTE TN to NR NTN reselection using redirection in Release message	Nokia, Nokia Shanghai Bell	discussion	Rel-19	NR_NTN_Ph3-Core

Agreements:

1.	RAN2 understands that the UE behaviour is currently not clear if the network includes a NR NTN frequency in redirectedCarrierInfo in the RRCConnectionRelease message in a TN cell. We can further discuss whether we want to clarify the UE behaviour and in case whether we need to introduce a new UE capability.

2.	Add a clarification in Stage 2 that only LTE to NR NTN is supported (and not eMTC NTN to NR NTN). Also add in the field description that NeighSatelliteInfoNR-r19 is only signalled in a LTE TN cell but not IoT NTN cell

## 8.9	IoT NTN Ph3

(IoT_NTN_Ph3-Core; leading WG: RAN2; REL-19; WID: RP-242397)

Time budget: 1 TU

Tdoc Limitation: 3 tdocs

### 8.9.1	Organizational

LS, Rapporteur input, including workplan, etc.

Rapporteur inputs do not count towards the tdoc limitation.

Incoming LSs

R2-2409529	Reply LS on FS_5GSAT_Ph3_ARCH conclusions (S2-2411250; contact: Sateliot)	SA2	LS in	Rel-19	5GSAT_Ph3_ARCH	To:SA3	Cc:RAN2, SA3-LI

Noted

R2-2409530	Reply LS on FS_5GSAT_Ph3_ARCH conclusions (s3i240703; contact: Tridea Works)	SA3-LI	LS in	Rel-18	5GSAT_Ph3_ARCH	To:SA2	Cc:RAN2, SA3

Noted

R2-2409524	Reply LS on Support of Regenerative-based Satellite Access (S2-2410918; contact: Qualcomm)	SA2	LS in	Rel-19	NR_NTN_Ph3-Core, 5GSAT_Ph3_ARCH	To:RAN3, RAN	Cc:RAN2

Noted

Rapporteurs’ inputs

R2-2410813	RRC Runing CR for IoT NTN	Huawei, HiSilicon	draftCR	Rel-19	36.331	18.3.1	B	IoT_NTN_Ph3-Core

R2-2410883	Draft Introduction of IoT NTN phase 3	Ericsson	draftCR	Rel-19	36.300	18.3.0	B	IoT_NTN_Ph3-Core

### 8.9.2	Support of Store & Forward

Contributions should focus on possible impacts to the radio interface.

R2-2409822	Discussion on the S&F operation indication	Google Ireland Limited	discussion	Rel-19	IoT_NTN_Ph3-Core

- Access Control

Proposal 1	A Rel-19 UE can inform eNB of its S&F capability in details (e.g., full S&F support including the NAS-layer support, or only limited S&F support in the RRC layer) via MSG3.

Proposal 2	A remaining duration associated with the current feeder link status can be broadcast by the cell supporting the S&F operation.

-	Xiaomi agrees some additional time indication is beneficial but we need to discuss the details

-	Samsung is not so convinced this is needed

-	QC thinks that we could use Tservice

-	ZTE thinks some time related information is useful but prefers to have some absolute time and doesn’t think that Tservice would do.

-	IDC thinks this might be useful but indeed we need to clarify the UE behaviour in case. Vivo agrees

-	Nokia supports this and thinks that Tservice is not sufficient. Sequans agrees. Telit also agrees

-	HW supports this and agrees we need to clarify UE behaviour.

-	Samsung thinks that this would trigger SI modification

-	Ericsson thinks we need to discuss the UE behaviour

Time information can be broadcasted to indicate when the current satellite operation mode will transit from “S&F operation” mode to real-time/normal mode. RAN2 assumes that a R19 UE could use this information at least to delay some NAS procedures until the feeder link is resumed (FFS what we specify in RAN2 specs for this and if we differentiate the behaviour for R19 UEs supporting and not supporting S&F mode, also considering the relation to the Wait Timer). FFS which SIB is used (SIB1 or SIB31). FFS if absolute or relative time is used. FFS on the opposite transition (i.e. whether to indicate when the “S&F operation” will start)

Proposal 3	When the S&F operation indication is present, the remaining duration indicates when the feeder link will become available.

-	Google thinks this could help to freeze some AS/NAS functions

-	HW thinks a R19 UE supporting S&F could use this information to delay some NAS procedures until the feeder link is resumed

Proposal 4	When the S&F operation indication is absent, the remaining duration indicates when the feeder link will become unavailable.

-	Xiaomi thinks this could be useful for R19 UEs not supporting the S&F mode to trigger cell reselection.

- Impact on cell reselection

Proposal 5	While camping on the cell indicating when the feeder link will become unavailable, the Rel-19 UE can trigger the intra-/inter-frequency measurement before the feeder link becomes unavailable.

Proposal 6	Rel-19 UE can be provided with a list of cells that are not operating in the S&F mode.

Proposal 7	If being provided with the list of cells not operating in the S&F mode, the Rel-19 UE supporting the S&F operation can trigger the intra-/inter-frequency measurement, upon detecting the presence of the S&F operation indication.

- RRC release

Proposal 8	When the feeder-link is not available, eNB can suspend UE’s RRC and NAS by transmitting an RRC Connection Release message to the UE with full S&F support. FFS whether a new release cause is needed.

-	ZTE is fine with this and thinks we could have a new case

-	Ericsson wonders what is meant by suspend RRC

-	vivo thinks there is no need to suspend RRC

-	CATT agrees on this under the assumption that we are not introducing a new RRC state.

-	Samsung thinks this is not needed

It is FFS whether we need to introduce a new release cause when the NW wants to release a R19 UE to RRC idle because the feeder link becomes unavailable

R2-2409585	Discussion on Store and Forward operation	Xiaomi	discussion	Rel-19	IoT_NTN_Ph3-Core

Proposal 1: “S&F operation” indication is provided in SIB1.

“S&F operation” indication is provided in SIB1

Proposal 2: Time information can be broadcasted in SIB1 to indicate when the current satellite operation mode will transit, e.g. from “S&F operation” mode to real-time/normal mode, or vice versa. Absolute time is used.

Proposal 3: Existing intraFreqExcludedCellList in SIB4 and interFreqExcludedCellList in SIB5 can be used if network want to prevent legacy UEs and Rel-19 UEs not supporting S&F from reselecting to an S&F NTN neighbor cell.

-	IDC and MTK think this is not needed. CATT also agree

Proposal 4: Introduce S&F specific intraFreqAllowedCellList in SIB4 and interFreqAllowedCellList in SIB5 to include S&F NTN neighbor cells for Rel-19 UEs supporting S&F to perform measurement and cell reselection.

-	Samsung thinks this would imply frequent updates

-	MTK is ok to consider this but agrees it would imply frequent changes

-	CATT thinks we need to wait before discussing this

Can come back in the next meeting

R2-2410468	Radio Interface Impacts of SF operation	Nokia, Nokia Shanghai Bell	discussion

- Remaining issues related to SF Operation Indication and Transition

Observation 1: Use of system Information modification procedure for switching between SF operation and normal operation is not signaling efficient considering the short duration of normal operation window for SF mode cell.

Proposal 1: New parameter T-Service-SF is introduced to support the transition of SF operation from SF mode to Normal and vice-versa.

Proposal 2: On expiry of the T-Service-SF the UE reads the new value of T-Service-SF and also the new SF operation related parameters to start the validity timer for the new mode.

Proposal 3: FFS need for system information value tag in addition to T-Service-SF parameter.

Proposal 4: Additional information related to maximum delay impact of SF operation is not broadcasted in system information. UE can deduce this information from other parameters related SF operation provided in NAS level procedures.

- RAN2 Impacts for CN Architecture Options

Observation 2: For CIoT-CP operation the RAN2 signalling procedure for CIOT data transmission is not impacted for both MME-Split and Full-CN Architecture options.

Observation 3: RAN2 assumes Store-forward PLMN operates with same architecture option across all NTN cells.

Proposal 5: RAN2 signalling is not needed to indicate the architecture option (Split MME or Full CN).

RAN2 assumes that AS signalling is not needed to indicate the architecture option (Split MME or Full CN).

Proposal 6: RAN2 checks with SA2 to address the unknown UE mobility scenario in which the UE does not find the target SAT for completing the remaining network procedure after the waiting time but another SAT instead.

- CIoT User Plane for SF Operation

Observation 4: CIoT-UP solution support will require additional RAN impacts for the user plane data forwarding for MME split option.

Observation 5: SA2 Technical report only focus on CIoT-CP solution as the basis for the store-forward study.

Proposal 7:  RAN2 to discuss and conclude on the need for support of CIOT-UP solution for SF operation.

-	Ericsson does not want to exclude the possibility to support CIoT-UP solution for SF operation

We wait for further progress in SA2, if any, before working on RAN2 aspects of a CIoT-UP solution for SF operation

- Access control and Paging procedure for SF Operation

Proposal 8:  RAN2 to consider inclusion of additional information for access control for MO or MT in SF operation

Proposal 9:  RAN2 to consider paging enhancements in SF mode for delivering ACK for MO Traffic and MT Traffic towards IoT-NTN UE.

Proposal 10: RAN2 to consider idle mode procedure enhancements, when there is no MT data to be delivered by a store & forward satellite.

R2-2410049	Discussion on S&F mode operation	Qualcomm Incorporated	discussion	Rel-19	IoT_NTN_Ph3-Core

- Switching S&F mode

Proposal 1	RAN2 assume that the cell operating in S&F mode uses MME on board and cell operating in normal mode uses MME on the ground and switching of the operation mode is supported.

Proposal 2	The existing cell stop time is used as switching time between S&F mode and normal mode operation.

Proposal 3	For new UEs, additional indication is provided to associate the existing cell stop time with the switching time between S&F mode and normal mode operation.

- S&F indication and cell barring

Proposal 4	The cell operating in S&F mode sets the cellBarred-NTN-r17 to “barred”.

- Cell reselection

Proposal 5	When connected to S&F mode cell, it can be left to UE in RRC_IDLE to periodically search for better and normal mode cell (i.e., cells with feeder link connection available).

Proposal 6	When in RRC_IDLE and waiting to connect to a S&F mode satellite, it can be left to UE to prioritize the search for better and normal mode cell (i.e., cells with feeder link connection available).

R2-2410591	Further consideration on Store and Forward	Huawei, HiSilicon, Turkcell	discussion	Rel-19	IoT_NTN_Ph3-Core

- Normal mode based on S&F architecture

Proposal 1: Clarify that the Normal mode agreed in the S&F context refers to the S&F architecture with eNB and partial MME/whole CN on board.

- Remaining time of current operating mode

Proposal 2: RAN2 discusses the usage and impact of the indication of the remaining time of current operating mode before introducing it, e.g., UE may delay some procedure until NW operates in the normal mode and legacy s-service may need to be set to this remaining time

- UE’s RRC state when service link is not available

Proposal 3: UE can be released to RRC_IDLE when service link is not available. FFS on whether UE can be released to RRC_INACTIVE or kept in RRC_CONNECTED with the AS operation suspended.

- Paging in S&F satellite operation

Proposal 4: If the NW doesn’t have any MT data buffered for an area, it informs the UE so that the UE doesn’t need to monitor paging or initiate RRC connection.

- Access control

Proposal 5: S&F NW should prioritize the access of the UE, if it is subsequently re-attempting the access to the NW. The re-attempt from the UE can be indicated to the network while performing RRC Connection Establishment procedure.

R2-2409674	Discussion on RAN2 impacts due to the satellite ID list from MME in S&F operation	CATT	discussion

Observation 1: For both split MME architecture and full CN on-board architecture, SA2 agreed that MME can provide a list of Satellite IDs for UE to monitor and access, and the format of the Satellite ID aligns with the satellite ID indicated in the system information.

Observation 2: The reason for CN to signal a satellite ID list for the UE is that the CN can only transfer UE context and/or MT data/singling on these indicated satellites; only if the UE connects with these satellites, can it finish the Attach/TAU procedure and avoid missing MT data/singling.

Proposal 1a: If the UE determines that it is out of coverage of all target satellite(s) indicated by MME, the UE operating in S&F mode need not perform idle mode tasks related to S&F operation (e.g. cell (re)selection, paging monitoring, etc.). The determination of "in coverage/out of coverage" of a target satellite is up to UE implementation.

-	Apple wonders if this indication from the MME implies that there are no other satellites, providing normal mode operation, then this makes sense; but is this the case? Samsung agrees

-	MTK thinks we should ask SA2 to clarify the usage of this information

-	Samsung this looks like discontinuous coverage

-	QC thinks this makes sense as long as the UE decides to stay in S&F mode, but the UE should still be allowed to consider other satellites providing normal mode operation

Discuss in offline 304 whether we can confirm the RAN2 assumption that:

- if the R19 UE supporting S&F determines that it is out of coverage of all target satellite(s) indicated by MME, a UE operating in S&F mode may not need to perform NTN idle mode tasks related to S&F operation (e.g. cell (re)selection, paging monitoring, etc.). The determination of "in coverage/out of coverage" of a target satellite is up to UE implementation. This does not prevent a UE to perform a network selection

Also discuss a possible LS to SA2 to check RAN2 assumption

Proposal 1b: Once the UE determines that it is in coverage of a target satellite indicated by MME, the UE shall perform idle mode tasks related to S&F operation (e.g. cell (re)selection, paging monitoring, etc.).

Observation 3: If the SatelliteInfo-r17 in SIB32 were directly reused for S&F operation, the number of the satellites may be limited, and the configured satellites might not meet the S&F requirement of the UE, thus bringing unnecessary UE power consumption.

Proposal 2: Introduce a separate list in system information to provide the satellite assistance information for S&F operation, i.e., used for UE to predict the coverage time of the satellites indicated by MME.

Proposal 3: Take signalling format of SatelliteInfo-r17 in SIB32 as the baseline for assistance information of the satellite(s) indicated by MME for S&F operation.

Observation 4: To avoid missing MT data and signalling, UE should camp on the cells belonging to the satellites indicated by MME (if detected). However, existing cell (re-)selection mechanism based on RSRP/RSRQ cannot ensure that.

Proposal 4: Enhance cell (re-)selection mechanism to ensure that UE operating in S&F mode (re-)selects a cell belonging to the satellites indicated by MME to camp on (e.g., prioritizing the frequencies served by the indicated satellites, selecting the certain cell which belongs to the indicated satellite to camp on, etc.).

[AT128][304][R19 IoT NTN] Satellite IDs for S&F (CATT)

Scope: Discuss RAN2 assumption on use of satellite IDs for S&F and draft LS to SA2

Intended outcome: report of offline discussion and draft LS

Deadline for rapporteur's summary (in R2-2410974):  Friday 2024-11-23 08:00

R2-2410974	Report of [AT128][304][R19 IoT NTN] Satellite IDs for S&F	CATT	discussion	Rel-19	IoT_NTN_Ph3-Core

Proposal 1: RAN2 understands that the UE configured with a satellite ID list by MME is still allowed to camp on a satellite operating in normal IoT NTN mode (i.e. with feeder-link connection), and perform subsequent access and data/signalling communication with that satellite FFS whether RAN2 asks SA2 for confirmation.

RAN2 understands that the UE configured with a satellite ID list by MME is not prevented to camp on a satellite operating in normal IoT NTN mode (i.e. with feeder-link connection), and perform subsequent access and data/signalling communication with that satellite

Include this understanding in the LS to SA2

Proposal 2: RAN2 understands the UE configured with a satellite ID list by MME is still allowed to camp on, access to and communicate with a satellite which is not included in the MME-configured satellite list. Ask SA2 for confirmation.

-	Google thinks we don’t need to ask for confirmation of the understanding but about the consequence

-	Nokia wonders what we need to do based on a possible response from SA2

RAN2 understands the UE configured with a satellite ID list by MME is not prevented to camp on, attempt to access to and communicate with a satellite which is not included in the MME-configured satellite list.

Include this understanding in the LS to SA2 and ask for feedback

Proposal 3: RAN2 asks SA2 to provide an answer for the below question:

Can the MME-configured Satellite ID list include both the satellites operating in S&F mode and the satellites operating in normal IoT NTN mode (i.e. with feederlink connection like in Rel-17/18)?

-	Nokia thinks this is not needed

-	Apple thinks the list provides a list of satellites that have the UE context

-	Ericsson thinks the list is the list of satellites which have the UE context

-	MTK thinks we should ask about the intended UE behaviour.

In the LS to SA2 add a question on what it means if a satellite is listed, e.g.

- the satellite has the UE context but not necessarily supports S&F, or

- the satellite has the UE context and the Satellite definitely supports S&F mode (but may be currently operating in normal mode)

Proposal 4: RAN2 approves the LS in R2-2411205 to SA2 on MME-configured satellite list.

Draft a LS to SA2 based on agreements above

R2-2411205	Draft LS on Satellite IDs for S&F	CATT	LSout	To:SA2	IoT_NTN_Ph3-Core

Revised in R2-2411197

To be discussed in [Post128][301]

[Post128][301][R19 IoT NTN] LS to SA2 on Satellite IDs for S&F (CATT)

Scope: Draft a LS to SA2 on satellite IDs based on the outcome of the discussion on R2-2410974

Intended outcome: Approved LS

Deadline: short

R2-2411197	LS on Satellite IDs for store-and-forward operation	RAN2	LSout	Rel-19	IoT_NTN_Ph3-Core	To:SA2

=> Approved

Agreements:

1.	Time information can be broadcasted to indicate when the current satellite operation mode will transit from “S&F operation” mode to real-time/normal mode. RAN2 assumes that a R19 UE could use this information at least to delay some NAS procedures until the feeder link is resumed (FFS what we specify in RAN2 specs for this and if we differentiate the behaviour for R19 UEs supporting and not supporting S&F mode, also considering the relation to the Wait Timer). FFS which SIB is used (SIB1 or SIB31). FFS if absolute or relative time is used. FFS on the opposite transition (i.e. whether to indicate when the “S&F operation” will start)

2.	It is FFS whether we need to introduce a new release cause when the NW wants to release a R19 UE to RRC idle because the feeder link becomes unavailable

3.	“S&F operation” indication is provided in SIB1

4.	RAN2 assumes that AS signalling is not needed to indicate the architecture option (Split MME or Full CN).

5.	RAN2 waits for further progress in SA2, if any, before working on RAN2 aspects of a CIoT-UP solution for SF operation

6.	RAN2 understands that the UE configured with a satellite ID list by MME is not prevented to camp on a satellite operating in normal IoT NTN mode (i.e. with feeder-link connection), and perform subsequent access and data/signalling communication with that satellite

7.	RAN2 understands the UE configured with a satellite ID list by MME is not prevented to camp on, attempt to access to and communicate with a satellite which is not included in the MME-configured satellite list.

R2-2409540	Further Discussion on S&F Operation	vivo	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2409676	Considerations on S&F operation from device perspective	Telit Communications S.p.A., Novamint, Sateliot, Thales	discussion	Rel-19	R2-2408244

R2-2409689	Further consideration on S&F operation in IoT NTN	ZTE Corporation, Sanechips	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2409799	Support of Store and Forward.	Interdigital, Inc.	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2409876	Discussion on Store & Forward satellite operation	OPPO	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2409889	Discussion on support of Store&Forward	Transsion Holdings	discussion	Rel-19

R2-2409935	Discussion on Store & Forward operation	DENSO CORPORATION	discussion	IoT_NTN_Ph3-Core

R2-2409976	Support of S&F operation in IoT NTN	Apple	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2410110	Further discussion of IoT NTN Store & Forward	China Telecom	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2410181	Discussion on assistance information for S&F	ASUSTeK	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2410270	Further considerations on S&F operation	Lenovo	discussion	Rel-19

R2-2410314	Discussion on Support of Store & Forward	TOYOTA Info Technology Center	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2410346	Discussion on IoT NTN Store and Forward	CMCC	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2410395	Radio Interface Aspect of S&F	NEC	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2410482	Discussion on Store and Forward operation	Samsung	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2410596	Considerations on multi-satellite for S&F Satellite operation	NOVAMINT, Sateliot, Thales	discussion

R2-2410599	Suspend/resume procedure for Store and Forward satellite operation	SHARP Corporation	discussion

R2-2410636	RAN2 impact on S&F mode	MediaTek Inc.	discussion	IoT_NTN_Ph3-Core	R2-2408622

R2-2410687	Discussion on the Store and Forward satellite operation	HONOR	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2410765	Further considerations on S&F operations	Continental Automotive	discussion	Rel-19

R2-2410854	Support of Store & Forward	Sequans Communications	discussion	Rel-19	IoT_NTN_Ph3-Core	R2-2408971

R2-2410863	Support for store and forward in IoT NTN	Ericsson	discussion	Rel-19	IoT_NTN_Ph3-Core

### 8.9.3	Uplink Capacity Enhancement

Contributions should focus on the possible enhancements to reduce the necessary uplink and downlink signaling to complete an EDT transaction (Msg3 transmission without msg1/RAR; efficient delivery of msg4 / RRCEarlyDataComplete).

R2-2409541	Discussion on CB-Msg3 Mechanism	vivo	discussion	Rel-19	IoT_NTN_Ph3-Core

CB-Msg3 configuration:

Proposal 1: Only system Information is used to provide cell-specific CB-Msg3 PUSCH resources.

-	Xiaomi supports this

-	HW wonders about the TA validation parameters

Only system Information is used to provide cell-specific CB-Msg3 PUSCH resources (FFS if anything is needed in dedicated signalling for the TA validation parameters, if needed, for the case of 15kHz SCS NB-IoT and eMTC CE mode B)

Proposal 2: Taking the UL/DL resources related configuration fields in PUR-Config-r16 as a baseline reference for CB-Msg3resources configuration.

Proposal 3: Send an LS to RAN1 to check RAN2’s assumption that the existing TBS size for PUR and CE level/RSRP threshold configuration applicable to EDT can be reused for CB-Msg3.

CB-Msg3 triggering:

Proposal 4: Reuse the existing CE level selection procedure for CB-Msg3.

Reuse the existing CE level selection procedure for CB-Msg3, at least for the initial selection. FFS whether we can reuse the same thresholds. FFS on the number of levels

Proposal 5: The UE triggers CB-Msg3 only if the size of pending UL data is less than the configured TBS for the selected CE level.

-	QC thinks we should further think about this to make it more useful

-	Samsung thinks we should have the same TBS size for the different CE levels

-	ESA thinks the maximum TBS should be the same for different CE levels

The UE triggers CB-Msg3 only if the size of pending UL data is less than the configured maximum TBS (FFS if the maximum TBS is same or different for different CE levels)

Proposal 6: Except for the RSRP thresholds corresponding to CE levels, do not introduce an RSRP threshold that determines whether CB-Msg3 can be used.

-	Xiaomi thinks that the combination with OCC might lead to the need for an additional threshold

-	QC thinks we need a threshold also for the last CE level

There will be a RSRP threshold that determines whether CB-Msg3 can be used (if the RSRP is below such threshold, PRACH will have to be used) (FFS if we need a separate set of thresholds, including a different minimum threshold, in case CB-msg3 EDT is combined with OCC)

Proposal 7: It is up to network implementation to associate CB-Msg3 resources with a specific CE level.

Proposal 8: If there are no CB-Msg3 resources available for the selected CE level, the UE cannot use CB-Msg3 regardless of whether there are other CB-Msg3 resources available other CE levels.

DSA modelling:

Proposal 9: DSA with at most 2 replicas is considered in Rel-19 IoT-NTN.

-	QC is not sure we need to restrict to 2

-	ZTE thinks we could have some configurable value but we should have a limited maximum value

-	IDC wonders if the number is related to the CE level. ZTE thinks it should be general. ESA thinks it should be per CE-level. Vivo thinks the level of UEs can be different in different levels so it makes sense to have this per CE-level to control the load

The number of replicas for DSA will be configured by the NW: 1 (SA), 2, 3, 4. The configuration of the number of replicas is CE-level specific

Proposal 10: DSA can be enabled on a per CE level basis.

Proposal 11: RAN2 to define a configurable or pre-defined time window for DSA CB-Msg3 occasion selection.

-	Xiaomi thinks something like this is needed

-	CATT and HW are not convinced this is needed

-	ESA agrees we need a time window. QC agrees

There will be a configurable time window for DSA CB-Msg3 occasion selection. FFS on the details (e.g. when the time window starts)

Proposal 12: MAC considers each replica transmission as a new transmission.

Proposal 13: HARQ entity obtains the MAC PDU from the HARQ process buffer for the secondary replica.

Proposal 14: HARQ process id 0 and RV 0 are used for CB-Msg3.

Response reception:

Proposal 15: For SA case, after the end of all repetition of CB-Msg3 PUSCH transmission, UE starts a window for response reception taking UE-eNB RTT into account.

For SA case (single replica), after the end of all repetition of CB-Msg3 PUSCH transmission, UE starts a window for response reception taking UE-eNB RTT into account. FFS if we need to consider additional delay e.g. for the processing time

Proposal 16: For DSA case, only one PDCCH monitoring window is started for response reception.

-	IDC wonders if this one or multiple windows per each replica

-	Apple thinks the UE might still need to monitor multiple RNTI. Samsung agrees

-	ESA thinks we should avoid that the UE waits for additional responses if it receives a response for one replica

-	Ericsson thinks the UE could have multiple windows

-	MTK thinks we should discuss msg4 before agreeing on this

For DSA case, FFS if we only have one or multiple PDCCH monitoring window(s) (i.e. one window per each replica) for response reception. FFS when the window(s) is/are started (or restarted) and stopped. FFS on the window length. FFS if the UE needs to monitor only one RNTI or multiple RNTIs)

The UE stops the PDCCH monitoring window(s) once it receives a CB-msg4 containing a matching Contention Resolution Identity (FFS if there is no RRC message together with the CB-msg4)

Assuming that there will be scenarios where it’s possible to receive a CB-msg4 before the UE transmits some replicas, a UE stops transmitting the remaining replicas if it has received a CB-msg4 containing a matching Contention Resolution Identity (FFS if there is no RRC message together with the CB-msg4)

Response reception:

Proposal 17: Support re-attempt after backoff time (like PRACH) for CB-Msg3.

R2-2410882	UL capacity enhancements for IoT NTN	Ericsson	discussion	Rel-19	IoT_NTN_Ph3-Core	Late

Proposal 10	The UE may skip monitoring CB-msgB (Msg4) once it receives a CB-msgB containing a matching Contention Resolution Identity.

Proposal 11	A UE may stop transmitting the remaining replicas if it has received a CB-msgB (msg4) containing a matching Contention Resolution Identity.

R2-2409690	Further consideration on UL capacity enhancements in IoT NTN	ZTE Corporation, Sanechips	discussion	Rel-19	IoT_NTN_Ph3-Core

CB-Msg3 configuration:

Proposal 1a: RAN2 can discuss the following resources types for shared resources configuration for CB-msg3 EDT:

- Time domain resources for (N)PUSCH occasions: periodicity and start time (e.g., start subframe, start SFN)

- Frequency domain resources for (N)PUSCH occasions: PRB index and PRB number for eMTC; Carrier index, a list of subcarrier set index and RU number for NB-IoT

- OCC resources

- repetition number

- (N)PDCCH resource

Proposal 1b: The periodicity of the time domain resources for (N)PUSCH occasions could be with the unit of SFN, the detailed value range can be FFS.

Efficient delivery of Msg4

Proposal 2a: For CB-msg3 EDT procedure, a contention resolution window needs to be introduced, which starts at the subframe of Msg3 transmission plus 4 subframes plus UE-eNB RTT.

Proposal 2b: The same value range as pur-ResponseWindowTimer can be used to configure the length of contention resolution window for CB-msg3 EDT procedure.

Proposal 3a: The following parameters can be taken into account in the RNTI calculation for CB-msg3 EDT:

- Time domain resource: SFN index, no need of subframe index

- Frequency domain resource: PRB index for eMTC; subcarrier set index and carrier index for NB-IoT

- Code domain resource: OCC index

Proposal 3b: For the RNTI calculation for CB-msg3 EDT, to avoid a too large RNTI, it can be considered to compress the part where SFN index is involved, for example, the SFN index can be divided by the minimum periodicity, or modulo by the length of contention resolution window.

Proposal 4: The contention resolution scheme in legacy random access procedure can be reused for CB-msg3 EDT procedure, e.g., a UE Contention Resolution Identity can be included in the DL MAC PDU and UE checks whether the UE Contention Resolution Identity matches the 48 first bits of the CCCH SDU transmitted in Msg3.

Proposal 5a: RAN2 is suggested to support backoff scheme for CB-msg3 EDT (e.g., when CB-msg3 EDT fails, after a backoff time, UE can choose an occasion to perform another Msg3 transmission), FFS details.

Proposal 5b: The fallback mechanism can also be considered for CB-msg3 EDT (e.g., fallback to EDT or legacy RACH when CB-msg3 EDT fails), FFS details.

DSA

Proposal 6: For DSA, a replica selection window for selecting time domain transmission occasions needs to be introduced. UE could select randomly multiple transmission occasions within this replica select window for multiple replicas transmission. And in each time domain transmission occasions, UE could select randomly a frequency domain resource.

Within the configured time window, the UE shall select randomly different time domain occasions for transmitting different replicas. And for each time domain occasion, the UE shall select randomly a frequency domain resource.

Proposal 7a: For DSA, once the eNB receives any one of the multiple replicas, it can respond it immediately without waiting for the remaining replica(s).

RAN2 understands that, for DSA, once the eNB successfully decodes one of the multiple replicas, it may respond without waiting for the remaining replica(s) (FFS when the response window(s) is/are started)

Proposal 7b: For DSA, UE needs to start monitoring the possible Msg4 after sending the first replica.

Proposal 8a: For DSA, if eNB receives remaining replica, whether or not to respond it can be left to eNB’s implementation. It’s expected for network not to respond Msg4 for duplicated replicas.

Proposal 8b: The knowledge of replica selection window could give assistance to the eNB to determine whether a replica is duplicated replica, so it’s suggested that the replica selection window can be configured by eNB.

Proposal 9a: The DSA could be enabled/disabled by eNB per CE level.

Proposal 9b: For DSA, the number of replicas (e.g., 2 by default) can be configured by eNB.

Proposal 9c: In DSA, RAN2 is suggested to discuss the following alternatives for applying the replica selection window:

•	Alt1: a sliding replica selection window, its start time is determined by the first selected transmission occasion.

•	Alt2: a fix replica selection window, its start time is determined by the configured start time of transmission occasion.

Proposal 10: For DSA, the RNTI used to schedule Msg4 transmission is derived only based on the resource associated to the transmission occasion used for the corresponding replica.

-	Xiaomi thinks we have to accept the fact that the UE needs to monitor different RNTIs

-	HW and Nokia think we should have a common RNTI for all replicas

-	QC is fine with this and the RNTI could be linked to the transmission occasion, to limit the RNTI space. Ericsson agrees the RNTI could be just linked to the transmission occasion

-	MTK supports this and thinks it would be difficult to go for a common RNTI

-	Samsung thinks we could have a common RNTI

-	Skylo thinks we can discuss this after discussion on CB-msg4 formatting. CATT agrees

-	ZTE thinks we need both the RNTI and contention resolution identity

Continue in the next meeting

Alternative proposal:

Proposal 10alt: In DSA, one common RNTI is used for monitoring multiple MSG4s which correspond to multiple MSG3 copies. FFS on how the common RNTI is decided.

Proposal 11: For DSA, RAN2 is suggested to discuss the following alternatives for applying contention resolution window:

•	Alt1: multiple contention resolution window: within each window, UE monitors a RNTI which is derived based on transmission occasion of a replica.

•	Alt2: one contention resolution window: within this window, UE monitors all RNTIs, one of which is derived based on transmission occasion of a replica.

R2-2409975	Uplink capacity enhancement in IoT NTN	Apple	discussion	Rel-19	IoT_NTN_Ph3-Core

Msg3 transmission:

Proposal 1: RAN2 to discuss whether to exclude UE specific CB-msg3 EDT PUSCH resource by RRCConnectionRelease message.

Proposal 2: Support fallback mechanism when EDT PUSCH transmission on contention based Msg3 fails. UE can fallback to RACH based EDT procedure upon:

- Reaching to a number of failed CB-Msg3 transmission attempts

- Condition for using contention based Msg3 resource is not met

Msg4 delivery:

Observation 1: If Msg4 reception windows for two different Msg3 resources with the same Msg3-RNTI are overlapped, extra indication of its associated Msg3 resource is needed in Msg4.

Observation 2: In 2-step RACH, at most one 'MAC subPDU for successRAR' indicating presence of 'MAC subPDU(s) for MAC SDU' (containing RRC message response to RRC message in Msg3) is included in MsgB MAC PDU.

Proposal 3: Aim to guarantee that the Msg4 reception windows for two different Msg3 resources with the same Msg3-RNTI are not overlapped.

Proposal 4: Msg4 should be able to comprise of the following parts:

1) Contention resolution indication on successfully received Msg3 from a certain UE;

2) RRC message part in response to the RRC message contained in received Msg3;

3) MAC subheader with Backoff indicator to other UE(s) whose Msg3 transmission do not succeed.

Proposal 5: Contention resolution is done by echoing back UE Contention Resolution Identity (48 first bits of the CCCH SDU transmitted in Msg3).

Proposal 6: Support multiplexing of multiple contention resolution(s) for multiple UE(s) in Msg4.

Proposal 7: RAN2 to discuss whether to support the following operation within Msg4 MAC PDU:

-	Multiplexing of contention resolution indication and Msg4-RRC message part

-	Multiplexing of multiple Msg4-RRC messages for more than one UE(s)

DSA:

Proposal 8: In DSA scheme, UE randomly selects multiple resources based on the configured replica number within the resource pool.

Proposal 9: RAN2 to discuss whether to initiate one or multiple Msg4 reception windows in DSA for multiple Msg3 replica(s). Potential enhancement should be considered to allow UE skip Msg4 monitoring.

R2-2409591	Discussion on the RAN2 solutions for uplink capacity enhancement	Huawei, HiSilicon	discussion	Rel-19	IoT_NTN_Ph3-Core

SA

Observation 1: There is no RAN4 requirement for configuring RSRP-based validation parameter in IoT NTN.

Proposal 1: The RSRP-based validation parameter is not needed for CB-msg3 EDT in IoT NTN.

Proposal 2: For 15kHz SCS NB-IoT and eMTC CE mode B, at least TA validation parameters need to be acquired via RRC dedicated signalling.

Proposal 3: A MAC CE including UE ID information is used for contention resolution for CB-msg3 EDT. FFS on the detailed design of the MAC CE.

Proposal 4: The backoff parameters are provided to UE via system information and UE randomly selects a backoff time based on the backoff parameters before another attempt.

Proposal 5: After a number of failed CB-msg3 EDT attempts, the UE falls back to 4-step random access EDT.

Proposal 6: RNTI used for Msg4 in CB-MSG3 EDT can take the formula for RA-RNTI calculation as baseline. FFS on the exact parameters.

Proposal 7: NW should allocate one C-RNTI in Msg4 for the UE to receive the scheduling after MSG4.

DSA

Proposal 8: PUSCH resource for CB-msg3 EDT can be reused for DSA.

Proposal 9: The network indicates how many copies the UE sends for DSA and whether DSA is enabled or not.

Proposal 10: In DSA, one common RNTI is used for monitoring multiple MSG4s which correspond to multiple MSG3 copies. FFS on how the common RNTI is decided.

OCC

Proposal 11: Postpone RAN2 discussion on OCC before receiving feedback from RAN1.

R2-2410050	CB-Msg3 and Msg4 enhancements	Qualcomm Incorporated	discussion	Rel-19	IoT_NTN_Ph3-Core

CB-Msg3 transmission procedure

Proposal 1	For DSA solution, a separate dedicated resource is configured via system information.

Proposal 2	Grouping of Msg3 transmission occasions in time domain is defined.

Proposal 3	Number of occasions for CB-Msg3 transmission group, i.e., N and optionally K occasions out of N allowed for replica transmission, is configured via system information.

Proposal 4	The number of occasions, i.e., N > 0 indicates the size of group of Msg3 transmission occasions in time domain.

Proposal 5	The starting point of the first transmission occasion of the group is configured. The transmission occasions for replicas within the group are defined w.r.t. the first transmission occasion.

Proposal 6	When a UE transmits a copy of the CB-Msg3 in a transmission occasion, a mechanism is defined to help network determine the location of the other copy(ies). FFS on details of how to do it.

Proposal 7	For DSA, RNTI(s) is also derived based on the time/frequency/occ resource associated with CB-msg3 transmission occasion(s). FFS on details.

Proposal 8	For Msg4, existing contention resolution procedure can be used, i.e., contention resolution timer and contention resolution MAC CE.

Proposal 9	The contention resolution timer is started or restarted after the last repetition of the transmission of a msg3 transmission plus UE-eNB RTT.

Proposal 10	Based on the configuration of DSA resource UE knows the network’s capability. Based on the use of the DSA resource, network knows the UE’s capability to support DSA.

Proposal 11	Allow the CB-Msg3 establishment cause to be mo-Data or mo-Signalling or mo-ExceptionData or delayTolerantAccess or mo-VoiceCall.

Proposal 12	Send LS to RAN1 informing potential alignment work associated with the CB-Msg3 transmission for DSA and non-DSA.

CB-Msg3 transmission attempt

Proposal 13	A new RSRP threshold to define the CE level for CB-Msg3 transmission is defined.

Proposal 14	A lowest RSRP threshold is defined for each CE level.

Proposal 15	The UE uses the CE level with the highest RSRP threshold among those CE levels for which the measured RSRP is higher that threshold. If none of the RSRP thresholds are met, the UE does not use CB-msg3.

Proposal 16	If CB-Msg3 transmission fails, network configures via system information the maximum number of re-attempts allowed for CB-Msg3 transmission before falling back to 4 step RACH-based procedure.

Proposal 17	If the total UL data volume is greater than the data volume threshold defined for CB-Msg3 transmissions, the UE does not use CB-Msg3 procedure.

Proposal 18	After CB-Msg3 transmission, if a UE is moved to RRC_CONNECTED, a new C-RNTI is provided to the UE.

Proposal 19	Support network indication of backoff or fallback to 4 step RACH procedure.

Msg4 enhancements

Proposal 20	For the second objective, a PUR-like L1 ACK concept, as a response to the EDT transmission, can be considered.

Proposal 21	For the second objective, multicast Msg4 (multi-user Msg4 multiplexing, similar to RAR or NR 2-step RACH response) or multi-user Msg4 scheduled by a single DCI (as in multi-TB Msg4 scheduling) can be studied as a solution.

Proposal 22	The enhancements done for Msg4 transmission can be applicable to normal 4 step RACH or CB-Msg3 or DSA transmission.

Proposal 23	RAN2 study the mechanism that upon detection of collision, e.g., based on signal level, the network may respond with Msg4 for the UE whose Msg3 is decoded and scheduling Msg4 retransmission or fallback indication for the other UEs whose Msg3 transmissions were not successfully decoded.

R2-2410875	Implicit pointer for locating DSA replicas for EDT of CB-Msg3	DLR, ESA, Inmarsat, Viasat	discussion

Observation 1: When DSA is supported for CB-msg3 transmissions, UEs shall pick the EDT occasions over which to send their replicas randomly.

Observation 2: Allowing the gNB to determine the positions of the DSA replicas sent by a UE upon decoding one of them can lead to resource savings in the downlink (e.g., avoid multiple unnecessary Msg4 acknowledgements).

Observation 3: The use of an implicit pointer can make the gNB aware of the positions of the DSA replicas sent by the UE as soon as one of them is decoded. The solution requires no modification to the CB-msg3 and entails no cost in terms of overhead.

Proposal 1: RAN2 to consider the use of an implicit pointer to allow the network infer the positions of the DSA replicas sent by a UE over the available set of EDT occasions.

-	Xiaomi thinks this would be useful for CRDSA but not necessarily for DSA. CATT agrees and thinks this might also have RAN1 impacts.

-	vivo also doesn’t think this is needed.

-	Toyota thinks this could be useful to have some testable UE behaviour

-	QC thinks this could also be useful for DSA as well

-	Ericsson thinks this could be useful, at least for the testing reasons and maybe also at the NW side. Nokia also sees some benefits and thinks that proponents can bring a more detailed solution that can be checked

-	HW is not convinced this is needed. ZTE shares the same view.

Can continue the discussion in the next meeting

Agreements:

1.	Only system Information is used to provide cell-specific CB-Msg3 PUSCH resources (FFS if anything is needed in dedicated signalling for the TA validation parameters, if needed, for the case of 15kHz SCS NB-IoT and eMTC CE mode B)

2.	Reuse the existing CE level selection procedure for CB-Msg3, at least for the initial selection. FFS whether we can reuse the same thresholds. FFS on the number of levels

3.	The UE triggers CB-Msg3 only if the size of pending UL data is less than the configured maximum TBS (FFS if the maximum TBS is same or different for different CE levels)

4.	There will be a RSRP threshold that determines whether CB-Msg3 can be used (if the RSRP is below such threshold, PRACH will have to be used) (FFS if we need a separate set of thresholds, including a different minimum threshold, in case CB-msg3 EDT is combined with OCC)

5.	The number of replicas for DSA will be configured by the NW: 1 (SA), 2, 3, 4. The configuration of the number of replicas is CE-level specific

6.	There will be a configurable time window for DSA CB-Msg3 occasion selection. FFS on the details (e.g. when the time window starts)

7.	For SA case (single replica), after the end of all repetition of CB-Msg3 PUSCH transmission, UE starts a window for response reception taking UE-eNB RTT into account. FFS if we need to consider additional delay e.g. for the processing time

8.	For DSA case, FFS if we only have one or multiple PDCCH monitoring window(s) (i.e. one window per each replica) for response reception. FFS when the window(s) is/are started (or restarted) and stopped. FFS on the window length. FFS if the UE needs to monitor only one RNTI or multiple RNTIs)

9.	The UE stops the PDCCH monitoring window(s) once it receives a CB-msg4 containing a matching Contention Resolution Identity (FFS if there is no RRC message together with the CB-msg4)

10.	Assuming that there will be scenarios where it’s possible to receive a CB-msg4 before the UE transmits some replicas, a UE stops transmitting the remaining replicas if it has received a CB-msg4 containing a matching Contention Resolution Identity (FFS if there is no RRC message together with the CB-msg4)

11.	Within the configured time window, the UE shall select randomly different time domain occasions for transmitting different replicas. And for each time domain occasion, the UE shall select randomly a frequency domain resource.

12.	RAN2 understands that, for DSA, once the eNB successfully decodes one of the multiple replicas, it may respond without waiting for the remaining replica(s) (FFS when the response window(s) is/are started)

R2-2409586	Discussion on uplink capacity enhancements for IoT NTN	Xiaomi	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2409673	Further consideration on UL capacity enhancements	CATT	discussion

R2-2409800	EDT/PUR enhancements.	Interdigital, Inc.	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2409877	Discussion on CB-msg3 EDT and msg4 enhancement	OPPO	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2409937	Further considerations on retransmission and finite buffer for DSA	NTU	discussion	Rel-19

R2-2410111	Contention-based Msg3-EDT in IoT NTN	China Telecom	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2410271	EDT for uplink capacity enhancement in NTN	Lenovo	discussion	Rel-19

R2-2410291	Consideration on UL capacity enhancement for IoT-NTN	NEC Corporation	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2410309	Further discussion on UL capacity enhancement for IoT NTN	Nokia, Nokia Shanghai Bell	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2410332	Considerations on uplink capacity enhancement for IoT-NTN	CMCC	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2410347	Discussion on Uplink Capacity Enhancement	TOYOTA Info Technology Center	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2410483	Procedures for contention-based Msg3	Samsung	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2410641	Discussion on enhanced EDT	MediaTek Inc.	discussion	IoT_NTN_Ph3-Core	R2-2408623

R2-2410725	Discussion on EDT Enhancements for IOT NTN	Skylo Technologies	discussion	Rel-19

Withdrawn

R2-2410296	Discussion on the use of an implicit pointer for locating DSA replicas for EDT of CB-Msg3	DLR, ESA	discussion	Withdrawn

Withdrawn

### 8.9.4	Support of PWS

Contributions should focus on the introduction of support for broadcast of PWS messages for NB-IoT, re-using the LTE mechanisms.

R2-2409801	Support of PWS.	Interdigital, Inc.	discussion	Rel-19	IoT_NTN_Ph3-Core

Proposal 1: Clarify in 36.300 section 4.10 that in general a NB-IoT UE may not support all PWS requirements.

-	IDC thinks that similar discussion is happening in other groups and similar statements will be included in other specs

-	ZTE thinks we could also have more detailed explanation of the cases

We will clarify in 36.300 section 4.10 that in general a NB-IoT UE may not support all PWS requirements.

Proposal 2: Extend the existing ETWS/CMAS notification RRC procedures to NB-IoT. No further clarification to “immediate” is therefore required.

-	QC wonders if we can have other optimizations to reduce the delay

-	HW is mostly fine with this but in any case we need to extend the procedures (e.g. refer to SIB1-NB)

-	Xiaomi wonders if we need any optimizations, also based on the understanding that in any case the requirements will be relaxed. Telit agrees that it’s not worth it to make it more complex

-	Ericsson agrees we could at least explore some possible enhancements

We will extend the existing ETWS/CMAS notification RRC procedures for eMTC to NB-IoT. FFS if SIB1-NB acquisition is needed

Proposal 3: Add the following PWS indication in Paging-NB:

- etws-Indication;

- cmas-Indication.

-	IDC explains the reason for suggesting this is to simplify the specification for this

-	vivo supports the proposal and is aligned to the intention to reuse what we have

-	Xiaomi also supports this and the reason comes for the RAN1 design for DCI. Nokia agrees

Add the following PWS indication in Paging-NB:

- - etws-Indication;

- - cmas-Indication.

Proposal 4: PWS reception in RRC_CONNECTED is not supported for NB-IoT. It is up to the network implementation to release RRC_CONNECTED UEs to RRC_IDLE if a PWS message needs to be broadcast.

-	Skylo thinks it’s not easy for the NW to send the UE to idle and PWS reception should be supported in Connected. Viasat agrees

-	CMCC supports the proposal

-	Samsung thinks we would need some indication in connected mode and releasing the UE to idle is not a reliable solution; maybe there are other ways than requiring to monitor another RNTI

-	QC thinks this requires RAN1. Additionally, cat-M UE does not support this and we should extend to Cat-M UE as well

-	ZTE thinks that the feature will likely be used by smartphones with IoT NTN capability and then it’s useful to support connected mode. Xiaomi agrees on the usefulness and agrees we should ask RAN1

Send a LS to RAN1 asking whether it’s possible to specify support for PWS reception in RRC_CONNECTED for NB-IoT in Rel-19.

[AT128][305][R19 IoT NTN] PWS reception in connected (Interdigital)

Scope: Draft LS to RAN1 on PWS reception in RRC_CONNECTED

Intended outcome: Draft LS

Deadline for draft LS (in R2-2410976):  Friday 2024-11-23 08:00

R2-2410976	Draft LS on PWS reception in RRC_CONNECTED (Interdigital)	LSout	To:RAN1	IoT_NTN_Ph3-Core

-	QC and ZTE think we can remove eMTC as it’s not in the scope. Vivo agrees. Xiaomi agrees

Remove the reference to eMTC

Remove Draft

Revised in R2-2411194

R2-2411194	LS on PWS support in RRC_CONNECTED for NB-IoT NTN	RAN2	LSout	Rel-19	IoT_NTN_Ph3-Core	To:RAN1

Approved

Agreements:

1.	We will clarify in 36.300 section 4.10 that in general a NB-IoT UE may not support all PWS requirements.

2.	We will extend the existing ETWS/CMAS notification RRC procedures for eMTC to NB-IoT. FFS if SIB1-NB acquisition is needed

3.	Add the following PWS indication in Paging-NB:

- etws-Indication;

- cmas-Indication.

R2-2409691	Further consideration on PWS support for NB-IoT over NTN	ZTE Corporation, Sanechips	discussion	Rel-19	IoT_NTN_Ph3-Core

- PWS acquisition for UE in idle mode

Proposal 1: It’s suggested to delete the dummy IE for SystemInformationBlockType10-NB.

Proposal 2: It’s suggested not to introduce etws-Indication and cmas-Indication in Paging message.

Proposal 3: Upon receiving the PWS notification from NB-IoT cell, the PWS-capable NB-IoT UE firstly needs to re-acquire schedulingInfoList contained in SIB1 immediately without waiting until the next system information modification period boundary, and then acquire the corresponding PWS messages.

Proposal 4: For NB-IoT UE, the ETWS, CMAS, PWS requirement may not be met in some scenarios, e.g. when the UE is in eDRX, or when the UE is configured with a large DRX cycle (e.g., rf512, rf1024), or when PSM is enabled.

- PWS acquisition for UE in connected mode

Proposal 5: If RAN2 agrees to support ETWS/CMAS acquisition in connected mode for NB-IoT, RAN2 check with RAN1 whether it’s feasible for NB-IoT UE in connected mode to monitor NPDCCH CSS to receive etws-indication and/or cmas-indication.

R2-2410643	Discussion on supporting PWS for NB-IoT	MediaTek Inc.	discussion	IoT_NTN_Ph3-Core	R2-2408624

Proposal 1: Introduce new ETWS indication and CMAS indication in the Paging-NB message.

Proposal 2: Send an LS to RAN1 for new Direct Indication Information that includes ETWS indication and CMAS indication in RRC Connected mode.

Proposal 3: Introduce a new dedicated RRC configuration to receive new Direct Indication Information in RRC Connected mode.

Proposal 4: Introduce a new UE capability to receive new Direct Indication Information in RRC Connected mode.

R2-2409587	PWS support for NB-IoT over NTN	Xiaomi	discussion	Rel-19	IoT_NTN_Ph3-Core

Proposal 1: Add PWS indication (i.e. etws-Indication and cmas-Indication) in Paging-NB.

Proposal 2: RAN2 intend to support PWS acquisition for NB-IoT NTN UEs in Connected mode.

Proposal 3: RAN2 assume that NPDCCH can include the PWS indication for NB-IoT UEs in connected mode. RAN2 send LS to RAN1 to confirm whether this is feasible and can be supported in Rel-19.

Proposal 4: RAN2 further discuss UE behaviors due to PWS acquisition in Connected mode, e.g. whether UL transmission needs to be suspended or stopped.

Proposal 5: Introduce two separate capabilities for NB-IoT UE to support ETWS reception and CMAS reception. These two capabilities do not differentiate NTN and TN.

Proposal 6: RAN2 wait for RAN1’s design on NPDCCH before discussing whether to introduce separate UE capabilities for PWS reception in Connected mode, or to have common UE capabilities for Idle mode and Connected mode.

R2-2409542	Further Discussion on PWS Support for NB-IoT	vivo	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2409675	Support of PWS for NB-IoT NTN UE	CATT	discussion

R2-2409896	Discussion on supporting PWS in IOT-NTN network	OPPO	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2410051	Discussion on PWS in NB-IoT NTN	Qualcomm Incorporated	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2410272	PWS broadcast support for NB-IoT in NTN	Lenovo	discussion	Rel-19

R2-2410289	Discussion on PWS for NB-IoT	Google	discussion	Rel-19

R2-2410292	Consideration on PWS broadcast for NB-IoT	NEC Corporation	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2410304	Further considerations on PWS support for NB-IoT	Huawei, HiSilicon, Turkcell	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2410310	Further discussion on support of PWS	Nokia, Nokia Shanghai Bell	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2410333	Support of PWS messages for NB-IoT	CMCC	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2410484	Impact of PWS signalling for NB-IoT	Samsung	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2410864	Support for PWS in NB-IoT NTN	Ericsson	discussion	Rel-19	IoT_NTN_Ph3-Core

## 8.10	SON/MDT Ph4

(NR_ENDC_SON_MDT_Ph4-Core; leading WG: RAN3; REL-19; WID: RP-234038)

Time budget: 0.5 TU

Tdoc Limitation: 2 tdocs

### 8.10.1	Organizational

LS, Rapporteur input, including workplan, etc.

### 8.10.2	MRO enhancements for Rel-18 mobility features

LTM has 1st priority. CHO with candidate SCGs has 2nd priority

Subsequent CPAC is paused until if/when we get a RAN3 LS on the subject

LTM

R2-2410656	Discussion on MRO enhancements for Rel-18 mobility	Huawei, HiSilicon	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

Focus on P1-P7

Proposal 1: RAN2 include the specific access type in the RLF report explicitly.

Proposal 2: RAN2 include in the RLF report whether the UE receives the TA from the cell switch command.

Proposal 3: RAN2 reuse the existing ra-InformationCommon for the RA-based LTM failure

Proposal 4: RAN2 discuss whether to include early sync info to existing RACH information. The detailed early sync info is FFS.

Proposal 5: RAN2 agree to enhance the RLF report with the beam information contained in the LTM cell switch command.

Proposal 6: Introduce a flag of the LTM candidate within the MeasResultNR explicitly for each neighbour cell.

Proposal 7: Introduce a flag of whether it is RACH-less for both SHR report and RLF report.

On P1:

-	vivo think P1 is not needed since the NW will know this even without an explicit indication. Ericsson that the existing RLF report indicates if the RA is due to HOF, and RA info common and existing flags the NW knows this. Huawei asks if the last HO type is what Ericsson have in mind? Ericsson confirms. Huawei seem to agree now.

On P2:

-	CMCC think this is not needed since the UE will report if it got the TA from the NW or if it was determined by the UE. Huawei think its easier to intropduce this rather than wait for RAN3. Qualcomm think the NW-based solution should work. Samsung think we need to wait.

On P4:

-	vivo think that the previous indication is sufficient which says whether the UE or the NW gave the TA. Ericsson understands that the proposal is that the UE shall report the preambles sent due to early sync and this is not really a full RA and it is not clear what the gNB should do with this info.

-	ZTE think that the point of SONMDT is to help the NW to give more info so they are supportive in general but not sure exactly which info Huawei has in mind. Huawei are more thinking on a high level about the scenario and want to discuss further if there is anything useful. CATT think that the early sync procedure is the core part of LTM and think that the NW can use this info to optimize early sync resources (e.g. RA attempts). Ericsson does not understand what can be optimized since this early UL sync for LTM is just a one-shot preamble transmission.

On P5:

-	Samsung think that L1 measurements has already been agreed to include in the RLF report and that should be enough as they carry beam info. Nokia agrees. ZTE think that the L1 measurements are not necessarily carrying sufficient info. Vivo agrees with Samsung that L1 measurements is sufficient.

-	CATT understand that the TCI state indicated in the MAC CE should be logged. Qualcomm think that the NW should know which TCI state is indicated in the LTM MAC CE. Ericsson think that this is for the RLF report and the NW cannot know afterwards. Ericsson think that it is useful to get the TCI state. Qualcomm think that even for RLF the NW can know this. CMCC think that the NW cannot know all configurations, e.g. the TCI state after a failure. Huawei agrees with CMCC that the LTM MAC CE is not a configuration and the NW cannot remember this.

On P6:

-	Ericsson think that CHO we did a list in the RLF report where the candidates were indicated and the point was that some measurements were perhaps not available. Ericsson prefer to align with CHO. CMCC agrees with Ericsson.

On P7:

-	Ericsson think that P1 addresses this.

Agreements - No need to include the specific access type (i.e. RACH-based or RACH-less) in the RLF report explicitly. It can be known implicitly. This can be revisited if RAN2 concludes that there are issues. - Reuse the existing ra-InformationCommon for the RA-based LTM failure. - If RAN3 does not address this meaning that we need a RAN2 solution, add a list indicating which LTM candidates the UE had at RLF.

R2-2409754	Further considerations on MRO	ZTE Corporation, Sanechips	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

Focus on P2-P3

Proposal 2	RAN2 introduces a new near failure case where BFR happens soon after a successful LTM cell switch, and UE re-selects to another beam in the same target cell.

Proposal 3	Introduce a new SHR trigger: a beam failure together with a BFR happens soon after an LTM cell switch is successfully executed.

On P2:

-	vivo think that there is not need for new triggers. ZTE think that T304 can reused, but that does not exclude other failure causes. Samsung think this causes a lot of additional complexity and need more time to look in to this if we should consider it. LG also want more time.

-	Nokia think that MRO is per cell not per beam and are not fine with this proposal.

On P3:

-	LG think that the issue is different since if BFR happens then its not really a failure since the UE recovered. Qualcomm think it is a very rare case that the LTM succeeded but the beam failed. Ericsson want to keep this open and want to look more in to this.

Postponed

R2-2410065	MRO for Rel-18 mobility	NEC	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

Focus on P1 and P2

Proposal 1: Include the following information to be included in RLF-report and SHR:

	TA value measured by the UE

	Difference between the TA value received from the LTM cell switch command and TA value measured by the UE

Proposal 2: Include the following information to be included in RLF-report and SHR:

	TCI state information included in the LTM cell switch command

	Beam with the TCI state information in the LTM cell switch command

R2-2409984	MRO for LTM	Nokia	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

Focus on P1-P4

Proposal 1: RAN2 to log additional information regarding TAs that were unnecessarily acquisitioned.

Proposal 2: RAN2 to discuss adding the following information into RLF report: TA difference between TA provided by network in Cell Switch Command for which the RACH-less LTM failed, and TA received in RAR for recovery/re-establishment cell.

Proposal 3: RAN2 to enhance SHR with new information and triggering criteria that allow the network to identify unnecessary LTM configurations.

Proposal 4:  RAN2 to define a threshold related to number of RLC retransmissions of the first UL transmission in RLC AM for RACH-less LTM as new SHR trigger.

CHO with candidate SCGs

R2-2409635	MRO enhancements for Rel-18 mobility features	Samsung	discussion

Focus on 2-1, 2-2, 2-3

Proposal 2-1: UE reports the time gap between the first met condition and the second met condition as follows:

a.	When at least one PCell condition is fulfilled before all the conditions for PSCell, UE logs the time gap between the first triggered event for PCell and the first triggered event for PSCell.

b.	When at least one PSCell condition is fulfilled before all the conditions for PCell, UE logs the time gap between the first triggered event for PSCell and the first triggered event for PCell.

Proposal 2-2:	Elapsed time between the first fulfilled condition and RLF is included in RLF report for all the PCells.

Proposal 2-3:	Measurement results of PCells and PSCells and the time information (as agreed for RLF) are included in SHR and SCGFailureInformation also.

On P2-1:

-	vivo think that we should consider the last triggered event for the PCell (or PSCell) to the last triggered event for the PSCell (or PCell). Ericsson think we have already agreed what vivo suggests. Ericsson want, in addition to this, have the first triggered PSCell condition to the last triggered PSCell condition. LG does not agree with with the Ericsson assumption.

-	Nokia think we don’t need more granularity. Ericsson is OK with not adding more granularity but points out that current spec already makes the UE log, for the CHO, the time from the first trigger for the PCell and the second trigger for the PCell and does not want to remove this possibility.

On P2-2

-	 vivo wonders if “first fulfilled condition” refers to either CHO or CPAC. Samsung wonder what is the intention of logging this, hence not supportive.

On P2-3:

-	ZTE has questions about stage-3 details. The session chair think they can be disucssed later.

Agreements - RAN2 understands that current agreements is that the UE shall logs (in RLF report, SHR report and SCGFailureInformation) time from the last triggered event for the PCell (or PSCell) to the time to the last triggered event for the PSCell (or PCell). We don’t intend to do further or more granular enhancements. - Measurement results of PCells and PSCells and the time information (as agreed for RLF) are included in SHR and SCGFailureInformation also. We will check what the spec impact of this is, e.g. something in the spec today may already make the UE log this.

R2-2409650	Discussion on MRO Enhancements for Rel-18 Mobility	CATT	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

Focus on P8 – P12

Proposal 8: Enhance RLF report for CHO with candidate SCGs to include the CPAC configuration information, i.e., CPAC candidate cells and CPAC execution conditions.

Proposal 9: Enhance RLF report for CHO with candidate SCGs to include the information for each candidate PSCell, i.e., first fulfilled event and time duration between two events fulfilled, if any.

Proposal 10: Enhance RLF report for CHO with candidate SCGs to include the association between CHO and CPAC.

Proposal 11: A new trigger condition that “the time difference between fulfilled CHO execution condition and fulfilled CPAC execution condition exceeds a configured threshold” is introduced in SHR for successful CHO with candidate SCGs execution.

Proposal 12: RAN2 to consider how to associate the SHR and SPR for CHO with candidate SCG(s) execution:

-	Option 1: one report to include both SHR and SPR information;

-	Option 2: two reports for SHR and SPR separately, and the association information is added in the two reports.

R2-2409779	MRO for MOB DCCA	LG Electronics	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core	R2-2409041

R2-2409780	MRO for MOB LTM	LG Electronics	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

R2-2409970	SON for LTM	Apple	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

R2-2409933	Discussion on MRO enhancement for LTM	China Unicom	discussion	NR_ENDC_SON_MDT_Ph4-Core

R2-2409934	Discussion on MRO enhancement for CHO with candidate SCGs	China Unicom	discussion	NR_ENDC_SON_MDT_Ph4-Core

R2-2409983	MRO for CHO with candidate SCG	Nokia	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

R2-2410182	Discussion on random access report for LTM	ASUSTeK	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

R2-2410274	Discussion on MRO for R18 mobility	Lenovo	discussion	Rel-19

R2-2410328	MRO enhancements for CHO with candidate SCGs	CMCC	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

R2-2410329	MRO enhancements for LTM	CMCC	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

R2-2410473	SON support for MRO	Ericsson	discussion	NR_ENDC_SON_MDT_Ph4-Core

R2-2410635	SON enhancement for CHO with candidate SCG	SHARP Corporation	discussion

R2-2410733	MRO enhancement for SON and MDT	Qualcomm Incorporated	discussion	NR_ENDC_SON_MDT_Ph4-Core

R2-2410815	MRO for Rel-18 mobility features	vivo	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

### 8.10.3	SON/MDT for Slicing

No contributions are expected and this AI will not be treated in RAN2#128 unless we get an LS from RAN3 on the subject

### 8.10.4	SON/MDT for NTN

No contributions are expected and this AI will not be treated in RAN2#128 unless we get an LS from RAN3 on the subject

### 8.10.5	Leftovers from Rel-18

RACH optimization for SDT focus on RSRP and data volume in SON reports, and existing failure causes.

MHI Enhancement for SCG Deactivation/Activation will not be treated in RAN2#128 unless we get an LS from RAN3 on the subject

SDT

R2-2409649	Considerations on R18 leftovers for SDT	CATT	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

Proposal 1: In the case of UE falling back from RA-SDT, UE logs the DL RSRP and UL data volume for the failed triggering of RA-SDT in RA report.

Proposal 2: The following failure causes can be logged for failed SDT in RA report:

⁻	upon receiving indication from the MCG RLC that the maximum number of retransmissions has been reached while SDT procedure is ongoing;

⁻	upon random access problem indication is received from MCG MAC while SDT procedure is ongoing;

⁻	upon T319a expires;

⁻	upon the lower layers indicate that cg-SDT-TimeAlignmentTimer or the configuredGrantTimer expired before receiving network response for the UL CG-SDT transmission with CCCH message while SDT procedure is ongoing;

⁻	unsuccessfully completed upon cell re-selection.

Proposal 3: To introduce an indication to differentiate whether it’s CG-SDT failure or RA-SDT failure in RA report, which could help network to know the failure is happened in CG-SDT or RA-SDT procedure and determine which parameters need to be adjusted.

On P1:

-	Ericsson agrees and think this is implicit from earlier agreements. Nokia does not understand why RSRP is needed to be logged. Ericsson think that the NW can adjust the threshold based on the UE reported measured RSRP. Nokia are not convinced that it is useful since the RSRP threshold cannot be adjusted based on the UE reported values.

On P2:

-	vivo are not sure about “unsuccessfully completed upon cell re-selection.” and wonders whats the use to the NW? Samsung think this cause is not useful and want to have it as FFS. Ericsson think that the spec and work becomes more complex if we cherry-pick, its better that the UE simply logs the reason for the failure.

On P3:

-	Qualcomm think that the NW should know this since it is configured by the NW. Ericsson think that the NW cannot know that the UE tried by failed to initiate CG-SDT, or if it fell back from CG-SDT to RA-SDT. Qualcomm think that CG-SDT is not in scope. Nokia agrees with Qualcomm

Agreements - The UE logs the DL RSRP and UL data volume for the failed to initiate of RA-SDT in RA report. - The following failure causes can be logged for failed SDT in RA report: ⁻ upon receiving indication from the MCG RLC that the maximum number of retransmissions has been reached while SDT procedure is ongoing; ⁻ upon random access problem indication is received from MCG MAC while SDT procedure is ongoing; ⁻ upon T319a expires; ⁻ upon the lower layers indicate that cg-SDT-TimeAlignmentTimer or the configuredGrantTimer expired before receiving network response for the UL CG-SDT transmission with CCCH message while SDT procedure is ongoing; ⁻ FFS: unsuccessfully completed upon cell re-selection.

R2-2409655	Reporting failure cause for SDT	Samsung	discussion

R2-2409755	Consideration on RAN3 agreed RA report content	ZTE Corporation, Sanechips	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

R2-2410275	Discussion on RACH optimization for SDT	Lenovo	discussion	Rel-19

R2-2410657	Discussion on support of the Rel-18 leftovers	Huawei, HiSilicon	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

R2-2410816	RACH optimization for SDT	vivo	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

R2-2410847	On Rel.18 leftovers	Ericsson	discussion

## 8.11	Evolution of NR duplex operation: Sub-band full duplex (SBFD)

(NR_duplex_evo-Core; leading WG: RAN1; REL-19; WID: RP‑241614)

Time budget: 0.5 TU

Tdoc Limitation: 2 tdocs

### 8.11.1	Organizational

Incoming LS, Rapporteur input, including workplan, etc..

### 8.11.2	Random access in SBFD

RAN2 impacts to support SBFD operation to support random access in SBFD symbols by UEs in RRC _CONNECTED mode and RRC_IDLE/INACTIVE mode.

Procedure and configuration related to RA resource selection

General

R2-2409680	SBFD RACH configuration for initial random access	Charter Communications, Inc	discussion

Noted

Proposal 5: RAN2 to discuss if UE selects additional-ROs first based on some SBFD UE capability indicator.

Proposal 6: RAN2 to discuss the maximum number of retries allowed on additional-ROs before triggering fallback to legacy-ROs.

DISCUSSION

P5

- LG E wonders what is the ue cap indicator? Interdigital think this is not very clear. Charter explains it is implemented in UE.

- Nokia think this is related to whether SBFD UEs always prioritize SBFD ROs.

P6

- LG E think the number is configurable.

CFRA

R2-2409625	Consideration on Random Access in SBFD symbols	CATT	discussion	Rel-19	NR_duplex_evo-Core

Noted

Proposal 8: The RO type should be indicated by NW for CFRA.

DISCUSSION

- QC think R1 already agreed. ZTE think R1 only agreed for the case of PDCCH-order CFRA.

- Nokia support P8, as this is NW’s control. Ericsson agree.

- Apple not sure about the SI request case.

- LG E think it is just 1 bit indication.

- Sony think this mandate all UEs to use SBFD.

The RO type is indicated by NW for CFRA. FFS on signaling (can FFS for the SI request case if needed).

CBRA

R2-2409745	Impacts on the random access by the evolution of duplex operation	Huawei, HiSilicon	discussion	Rel-19	NR_duplex_evo-Core

Noted

Proposal 3: No need to have one separate/additional indication from network on UE RACH resource type selection.

Proposal 4: Support the SBFD-aware UE to select PRACH resource in non-SBFD symbols when the SSB RSRP smaller than a configured threshold, otherwise to select PRACH resource in SBFD symbols.

Proposal 5: If the network doesn't configure the condition, the SBFD-aware UE prioritizes to select the SBFD RACH resource.

DISCUSSION

P4

- QC do not think RSRQ in P4 is a good metric, and think this prevent the cell edge UEs to use SBFD resources. Nokia agree. QC think if there is no indication then UE just select legacy RO. Interdigital agree.

- LG E has different view than QC since CLI impact will be large. LG E think P4 reasonable. LG E think the proposal work together with QC proposal 6. NEC, Interdigital share this view.

R2-2410609	Random access in SBFD	Samsung	discussion	Rel-19

Noted

Proposal 3. Upon initiation of CBRA RACH procedure for a SBFD-aware UE, network can indicate the UE to utilize the nearest RO for low latency.

R2-2410478	Views on random access for SBFD	Qualcomm Incorporated	discussion	NR_duplex_evo-Core

Noted

Proposal 6: Upon initiation of RACH procedure for a SBFD-aware UE, if network provides the indication on the prioritization of the legacy ROs and additional ROs, UE selects the type of ROs based on network indication.

Proposal 7: Upon initiation of RACH procedure for a SBFD-aware UE, if no additional indication is from network and PRACH transmission with preamble repetitions is not configured in either type of ROs, the initial attempt in the RACH procedure is determined based on the prioritization of the additional-ROs over legacy-ROs. FFS on that case that PRACH transmission with preamble repetitions is configured.

DISCUSSION

P6 in R2-2410478

Do we need additional indication from NW (nearest RO, legacy or SBFD ROs)

- LG E do not support indication for nearest RO, since it complicates the MAC procedure. Ericsson , ZTE agree. Sharp agree and think it is important to consider interference.  LG E open to discuss other indication.

- NEC agree to have indication for legacy or SBFD ROs. ZTE agree and think NW should be able to balance the load in different ROs. Sony agree. Sony, Interdigital agree with P6 in QC paper.

- CATT do not agree with P6 since some UE may see worse channel so not be able to use SBFD even if NW indicates so, and think UE knows better e.g. the RSRP. Nokia agree. Sony has different understanding and think one benefit for using SBFD resource is to improve coverage.

- Nokia wonders if this should for UE-initiated CBRA.

FFS on the following options

Option 1

- Upon initiation of RACH procedure for a SBFD-aware UE, network provides the indication on the prioritization of the additional ROs over legacy RO.

If there is no such indication from the NW, FFS on the following mechanism

- UE select legacy RO or SBFD RO based on SSB RSRP, or

- UE select the legacy RO, or

- UE select the SBFD RO, or

- Other metrics than SSB RSRP.

Option 2

UE select legacy RO or SBFD RO based on SSB RSRP if such condition is configured, and if not configured, then UE can prioritize one type of the ROs, FFS which one.

Fallback

R2-2409794	Random Access for SBFD Operation	NEC	discussion

Noted

Proposal-1: the UE should be allowed to switch to additional SBFD RACH occasions after certain (configured) number of times of RACH attempt in legacy RACH occasions, if the UE selects legacy ROs first.

R2-2409913	Discussion on Random Access in SBFD	LG Electronics Inc.	discussion	Rel-19	NR_duplex_evo-Core

Noted

Proposal 5. Do not support switching from legacy RACH occasions to SBFD RACH occasions for SBFD-aware UE.

DISCUSSION

- Nokia, Sharp, QC, Ericsson agree with NEC proposal.

- Samsung support LG E proposal.

Early indication via msg3

R2-2410385	Random access for SBFD Operation	Sony	discussion	Rel-19	NR_duplex_evo-Core

Noted

Proposal 2: Early identification of SBFD-aware UEs in message 3 is not needed during RA procedure.

R2-2410088	RA Aspects for SBFD	Sharp	discussion	Rel-19	NR_duplex_evo-Core

Noted

Proposal 1	Msg3-based early indication in legacy ROs during RA procedure is supported to indicate UE’s SBFD capability.

DISCUSSION

- Sony do not think this is critical and think we should not spend too much time on this.

- Apple think today we have many features that use msg3 based EI and if we do so here it requires more time to discuss.

RACH configuration

R2-2409974	Detailed design for RACH in SBFD	Apple	discussion	Rel-19	NR_duplex_evo-Core

Noted

Proposal 3: Only one type of configuration from Option 1 and Option 2 is provided in SIB at the same time.

Proposal 4: For Option 1, SBFD specific power control parameters are provided into RACH-ConfigCommon. FFS on SBFD specific SSB-RO mapping configuration.

Proposal 5: For Option 2, SBFD specific RACH resources (e.g., rach-SBFD-ConfigCommon) are provided into BWP-UplinkCommon and AdditionalRACH-Config.

R2-2409995	SBFD RA aspects	Ericsson	discussion	Rel-19	NR_duplex_evo-Core

Noted

Proposal 1	Only one RACH configuration option (i.e., either RACH configuration Option 1 with Alt 1-1 or RACH configuration Option 2) is supported in a cell.

Proposal 2	To aim for a unified RACH design/framework between RACH configuration Option 1 with Alt 1-1 and RACH configuration Option 2.

Proposal 3	As in the legacy, for RACH configuration option 2, the additional SBFD RACH configuration is not visible in the MAC spec. In other words, the introduction of selection of RACH configuration should be avoided in the MAC spec.

DISCUSSION

- ZTE think NW can configured O1 and O2 together in order for all UEs to enjoy this feature. NEC, Charter and Sharp share this view.

- Ericsson think in practice NW will not use these two at the same time.

- CATT think R1 is not ready to do this and if we agree to have both in a cell then we need to inform R1.

Only one RACH configuration option (i.e., either RACH configuration Option 1 with Alt 1-1 or RACH configuration Option 2) is supported in a cell.

Agreements on RACH with SBFD

CFRA The RO type is indicated by NW for CFRA. FFS on signaling (can FFS for the SI request case if needed). CBRA FFS on the following options Option 1 - Upon initiation of RACH procedure for a SBFD-aware UE, network provides the indication on the prioritization of the additional ROs over legacy RO. If there is no such indication from the NW, FFS on the following mechanism - UE select legacy RO or SBFD RO based on SSB RSRP, or - UE select the legacy RO, or - UE select the SBFD RO, or - Other metrics than SSB RSRP. Option 2 UE select legacy RO or SBFD RO based on SSB RSRP if such condition is configured, and if not configured, then UE can prioritize one type of the ROs, FFS which one. RACH configuration Only one RACH configuration option (i.e., either RACH configuration Option 1 with Alt 1-1 or RACH configuration Option 2) is supported in a cell.

R2-2409571	Discussion on random access procedure in SBFD	ZTE Corporation	discussion	Rel-19	NR_duplex_evo-Core

R2-2409579	Discussion on RACH in SBFD	Xiaomi	discussion	Rel-19	Withdrawn

R2-2409584	Discussion on RACH in SBFD	Xiaomi	discussion	Rel-19

R2-2409625	Consideration on Random Access in SBFD symbols	CATT	discussion	Rel-19	NR_duplex_evo-Core

R2-2409680	SBFD RACH configuration for initial random access	Charter Communications, Inc	discussion

R2-2409745	Impacts on the random access by the evolution of duplex operation	Huawei, HiSilicon	discussion	Rel-19	NR_duplex_evo-Core

R2-2409794	Random Access for SBFD Operation	NEC	discussion

R2-2409913	Discussion on Random Access in SBFD	LG Electronics Inc.	discussion	Rel-19	NR_duplex_evo-Core

R2-2409974	Detailed design for RACH in SBFD	Apple	discussion	Rel-19	NR_duplex_evo-Core

R2-2409995	SBFD RA aspects	Ericsson	discussion	Rel-19	NR_duplex_evo-Core

R2-2410088	RA Aspects for SBFD	Sharp	discussion	Rel-19	NR_duplex_evo-Core

R2-2410241	Random Access Operation of SBFD	Nokia Corporation	discussion	Rel-19	NR_duplex_evo-Core

R2-2410336	Discussion on random access in SBFD	CMCC	discussion	Rel-19	NR_duplex_evo-Core

R2-2410385	Random access for SBFD Operation	Sony	discussion	Rel-19	NR_duplex_evo-Core

R2-2410478	Views on random access for SBFD	Qualcomm Incorporated	discussion	NR_duplex_evo-Core

R2-2410574	Random Access in Sub-Band Full Duplex	Google Ireland Limited	discussion

R2-2410609	Random access in SBFD	Samsung	discussion	Rel-19

R2-2410791	Discussion on random access procedure in SBFD	vivo	discussion	Rel-19	NR_duplex_evo-Core

R2-2410794	Discussion on random access in SBFD	Fujitsu Limited	discussion	Rel-19	NR_duplex_evo-Core

### 8.11.3	Other aspects

Other RAN2 impacts with SBFD if not covered by the previous agenda items.

R2-2409638	Other aspects of SBFD	Xiaomi	discussion	Rel-19	NR_duplex_evo-Core

Noted

Proposal 2: Prioritization of SBFD cells / frequencies during cell reselection is not considered.

Prioritization of SBFD cells / frequencies during cell reselection is not considered.

R2-2410258	Other aspects of SBFD	Nokia	discussion	Rel-19	NR_duplex_evo-Core

Noted

Proposal 4: For inter-cell CSI-RS measurements, UE is provided with information of the SBFD configuration of neighbouring cells.

DISCUSSION

- Ericsson think this is not only R2 topic but it also impact R4. Ericsson think we need to wait.  ZTE, Samsung agree.

- HW support P4. HW think R1 already agree UE need to exclude the SBFD symbols in the CSI-RS measurement. LG E  think R1 agreement is for intra cell not inter cell.

- Charter think this might also involve R3.

RAN2 wait for input from the other WGs regarding whether for inter-cell CSI-RS measurements, UE needs to be provided with information of the SBFD configuration of neighbouring cells.

R2-2409572	Discussion on CLI measurement in SBFD	ZTE Corporation	discussion	Rel-19	NR_duplex_evo-Core

Noted

UE-to-UE CLI measurement, MR-DC:

Proposal 3: RAN2 to support that:

	In EN-DC and NGEN-DC, only the SN can configure L1 CLI measurements;

	In NE-DC, only the MN can configure L1 CLI measurements;

	In NR-DC, both the MN and the SN can configure L1 CLI measurements.

Proposal 4: In NR-DC, MN and SN should coordinate the L1 CLI related configuration restriction, in order to not exceed the UE’s capability.

gNB-to-gNB CLI measurement

Proposal 5: For SBFD gNB-to-gNB CLI measurement, support to include the set of one or more periodic NZP CSI-RS resources configuration into RAN2 INM MeasurementTimingConfiguration.

Agreements on other aspects for SBFD

Prioritization of SBFD cells / frequencies during cell reselection is not considered.

RAN2 wait for input from the other WGs regarding whether for inter-cell CSI-RS measurements, UE needs to be provided with information of the SBFD configuration of neighbouring cells.

R2-2409572	Discussion on CLI measurement in SBFD	ZTE Corporation	discussion	Rel-19	NR_duplex_evo-Core

R2-2409626	Discussion on other aspects for SBFD	CATT	discussion	Rel-19	NR_duplex_evo-Core

R2-2409638	Other aspects of SBFD	Xiaomi	discussion	Rel-19	NR_duplex_evo-Core

R2-2409681	SBFD-aware UE capability indication	Charter Communications, Inc	discussion

R2-2409746	Other impacts by the evolution of duplex operation	Huawei, HiSilicon	discussion	Rel-19	NR_duplex_evo-Core

R2-2409793	Selection of SBFD Cell	NEC	discussion

R2-2409996	Non-RA aspects for subband full duplex (SBFD) operation	Ericsson	discussion	Rel-19	NR_duplex_evo-Core

R2-2410258	Other aspects of SBFD	Nokia	discussion	Rel-19	NR_duplex_evo-Core

R2-2410479	Other aspects of SBFD	Qualcomm Incorporated	discussion	NR_duplex_evo-Core

R2-2410623	Support of Cross Link Interference in SBFD	Samsung	discussion	Rel-19	NR_duplex_evo-Core

R2-2410792	Discussion on other aspects in SBFD	vivo	discussion	Rel-19	NR_duplex_evo-Core

## 8.12	NR MIMO Phase 5

(NR_MIMO_Ph5-Core; leading WG: RAN1; REL-19; WID: RP-242394)

Time budget: 0.5 TU

Tdoc Limitation: 2 tdocs

### 8.12.1	Organizational

LSs and rapporteur input, including workplan, etc.

R2-2410325	Work Plan for Rel-19 on NR MIMO Phase 5	CMCC	Work Plan	Rel-19	NR_MIMO_Ph5-Core

Noted

### 8.12.2	Asymmetric DL sTRP/UL mTRP

To identify RRC/MAC aspects that need to be discussed for asymmetric DL sTRP/UL mTRP

MAC CE for PL offset update

R2-2410326	Discussion on asymmetric DL sTRP and UL mTRP	CMCC	discussion	Rel-19	NR_MIMO_Ph5-Core

Noted

Proposal 1: PL offset should not be set to ‘delta’ value, which represents the difference in uplink UL RSRP to help UE estimate the path loss between itself and UL-only TRP.

Proposal 2:  We propose two options to address the dispute between MAC CE and RRC when updating PL offset:

-	Option1: For the same joint/UL TCI state, the UE should apply the latest PL offset value, whether it is represented by RRC configuration or MAC CE update.

-	Option2: For the same joint/UL TCI state, UE needs to determine which signaling update is the latest between RRC and MAC CE, and the criteria for this determination are to be discussed (taking latency as an example)

Proposal 4:  The new MAC CE is proposed to be consisted of the following fields, and the below MAC CE format can be adopted as baseline, with updated with further RAN1 input:

-	Serving Cell ID: This field indicates the identity of the Serving Cell for which the MAC CE applies. The length of the field is 5 bits;

-	DL/UL BWP ID:  Since the joint TCI state configured in DL BWP and the UL TCI state configured in UL BWP are equally applicable, the DL BWP ID should be indicated for the joint TCI state and the UL BWP ID should be indicated for the UL TCI state.The length of the field is 2 bits;

-       TCI state IDi: This field indicates the TCI state identified by TCI-StateId as specified in TS 38.331 [5] . The length of the field is 7 bits.

-	D/U: This field indicate whether the TCI state ID in the same octet is for joint/downlink or uplink TCI state. If this field is set to 1, the TCI state ID in the same octet is for joint/downlink. If this field is set to 0, the TCI state ID in the same octet is for uplink;

-	R: Reserved bit, set to 0.

R2-2409640	Consideration on Asymmetric DL sTRP/UL mTRP 	LG Electronics Inc.	discussion	Rel-19	NR_MIMO_Ph5-Core

Noted

Proposal 1: RAN2 introduce PL offset parameter in TCI-State IE and TCI-UL-State IE respectively, where the value range id [-12, 60] dB and the step size is 4dB.

Proposal 2. To update PL offset associated with a joint/UL TCI state, RAN2 introduce a new MAC CE with new (e)LCID.

Proposal 3. New MAC CE is designed to consider only PL offset update and to not consider TCI state activation.

Proposal 4. Absolute value of PL offset is indicated in the new MAC CE.

Proposal 5. Up to 8 PL offset are indicated in the new MAC CE.

Proposal 6. Only PL offset corresponding to active TCI state is updated by the new MAC CE.

Proposal 7. RAN2 design the new MAC CE considering follows.

-	One Serving Cell field and one BWP field are included

-	16-bit codepoint bit map is included

-	CORESET Pool ID field and D/U field are not included

DISCUSSION

P3 in R2-2409640

- Ericsson wonders whether this means TCI state activation is done separately or part of this MAC CE. LG E clarify that they are sperate.

- Nokia agree with the proposal.

P4 in R2-2409640

- Sony clarify that value is the absolute value of PL offset.

New MAC CE is introduced for PL offset update for asymmetric DL sTRP/UL mTRP. This new MAC CE is identified by new eLCID.

Absolute value of PL offset is indicated in the new MAC CE. For the offset value, the value range is [-12, 60] dB and the step size is 4dB.

P6 in R2-2409640

- Apple do not think we need this restriction.

- CATT think R1 discussed also SRS and RACH, and think this restriction is not aligned with what R1 discussed.

- Vivo support this proposal and think NW provide this offset value after TCI activation.

- Xiaomi think NW may need to update the offset value even for deactivated TCIs due to UE’s movement. ZTE share this view.

- Nokia agree with the proposal.

- Samsung think some other UL transmission use the RRC configured TCI and do not require TCI activation, so think this new MAC CE should be applied to all configured TCI states.

- LG E think RRC reconfiguration can be used to update the PL offset values for the deactivated TCI states. LG E think we need to check with R1 whether PL offset applies to SRS transmission.

- QC, Apple agree with Samsung that we need to cover all the cases.

In the MAC CE, PL offset value can be updated for any configured TCI states with RRC configured PL offset, i.e., not limited to the activated TCI states.

Agreements on asymmetric DL sTRP and UL mTRP

New MAC CE is introduced for PL offset update for asymmetric DL sTRP/UL mTRP. This new MAC CE is identified by new eLCID.

Absolute value of PL offset is indicated in the new MAC CE. For the offset value, the value range is [-12, 60] dB and the step size is 4dB.

In the MAC CE, PL offset value can be updated for any configured TCI states with RRC configured PL offset, i.e., not limited to the activated TCI states.

PHR triggering

R2-2409661	Discussion on asymmetric DL sTRP and UL mTRP	Xiaomi	discussion	Rel-19	NR_MIMO_Ph5-Core

Not treated

Proposal 2: The PHR is triggered, when the path loss for the UL-only TRP has changed more than phr-Tx-PowerFactorChange due to:

- The RRC configuration of pathloss offset

- The MAC CE update of pathloss offset

R2-2410248	RAN2 Aspects of Asymmetric DL sTRP/UL mTRP	Nokia Corporation	discussion	Rel-19	NR_MIMO_Ph5-Core

Not treated

Proposal 2: RAN2 to confirm that Path loss offset has no impact on PHR triggering condition

2TA for single-DCI mTRP

R2-2410388	Enhancement for Asymmetric DL sTRP/UL mTRP	Sony	discussion	Rel-19	NR_MIMO_Ph5

Not treated

Proposal 5: RAN2 follows/confirms RAN1 agreements to support 2TA for the asymmetric DL sTRP/UL mTRP deployment scenarios.

R2-2410523	Discussion on Asymmetric DL sTRP/UL mTRP	Samsung	discussion	Rel-19	NR_MIMO_Ph5

Not treated

Proposal 11: For 2TA in asymmetric DL sTRP/UL mTRP scenario, Rel-17 2TA operation is applied with the following RRC change:

•	remove the restriction that coresetPoolIndex needs to be configured for the 2TA feature in relevant RRC fields (e.g., tag2);

•	a single n-TimingAdvanceoffset is configured.

R2-2409640	Consideration on Asymmetric DL sTRP/UL mTRP 	LG Electronics Inc.	discussion	Rel-19	NR_MIMO_Ph5-Core

R2-2409661	Discussion on asymmetric DL sTRP and UL mTRP	Xiaomi	discussion	Rel-19	NR_MIMO_Ph5-Core

R2-2409721	Discussion on RRC and MAC Impacts for Asymmetric DL sTRP/UL mTRP	CATT	discussion	Rel-19	NR_MIMO_Ph5-Core

R2-2409773	Discussion on MAC CE impact for asymmetric DL sTRP/UL mTRP scenarios	vivo	discussion	Rel-19	NR_MIMO_Ph5-Core

R2-2409954	RAN2 Impacts for Rel-19 NR MIMO	Apple	discussion	Rel-19	NR_MIMO_Ph5-Core

R2-2410248	RAN2 Aspects of Asymmetric DL sTRP/UL mTRP	Nokia Corporation	discussion	Rel-19	NR_MIMO_Ph5-Core

R2-2410294	Asymmetric DL/UL mTRP user plane impact from MIMO ph. 5	Ericsson	discussion	Rel-19	NR_MIMO_Ph5-Core

R2-2410326	Discussion on asymmetric DL sTRP and UL mTRP	CMCC	discussion	Rel-19	NR_MIMO_Ph5-Core

R2-2410388	Enhancement for Asymmetric DL sTRP/UL mTRP	Sony	discussion	Rel-19	NR_MIMO_Ph5

R2-2410429	Discussion on UL only mTRP 	Qualcomm Incorporated	discussion

R2-2410520	Discussion on Asymmetric DL sTRP/UL mTRP	Huawei, HiSilicon	discussion	Rel-19	NR_MIMO_Ph5-Core

R2-2410523	Discussion on Asymmetric DL sTRP/UL mTRP	Samsung	discussion	Rel-19	NR_MIMO_Ph5

R2-2410770	Consideration on the PL Offset MAC CE for R19 MIMO	ZTE Corporation	discussion	Rel-19	NR_MIMO_Ph5-Core

### 8.12.3	Others

To identify R2 impact on other objectives

UE-initiated Beam Reporting

Modelling

R2-2410430	Discussion on UE initiated beam reporting	Qualcomm Incorporated	discussion

Noted

Proposal 1. RAN2 supports that the event evaluation is handled at MAC layer.

Consider an LS to RAN1 if it is agreed to support event evaluation at MAC layer.

Proposal 2. RAN2 waits for RAN1’s decision on the first PUCCH design.

R2-2410355	Discussion on MAC CE impact of Rel-19 MIMO	NEC	discussion

Noted

Proposal 1:	It is up to RAN1 to capture the agreement on triggering event determination for Event 2.

DISCUSSION

- ZTE think it is meaningful to have some conclusion considering the time frame of this WI.

- Samsung think from technical perspective we cannot conclude to have it in MAC, since R1 already agreed to send in PUCCH the step 1 notification, there may not be a need to involve MAC.

- HW think there is no difference in MAC whatever R1 agreed on the step 1 PUCCH notification. Vivo, MediaTek, CMCC, Ericsson, Nokia, InterDigital agree.

- CATT think this case is different from LTM or RLF and prefer to leave it to R1 spec.

- vivo think LTM already agreed to capture the trigger in MAC and it is good to have common way.

- Apple prefer to wait for R1 and think we can progress in the next meeting. LG E agree.

- Xiaomi think it is quite difficult to have common way for LTM and UE initiated BM since the event evaluation is different.

Others

R2-2410524	Discussion on UE-initiated Beam Reporting and CSI enhancement	Samsung	discussion	Rel-19	NR_MIMO_Ph5

Not treated

Proposal 1: Introduce a new IE under CSI-ReportConfig to include the UEI beam reporting configuration.

R2-2410202	Impacts from other NR MIMO Phase 5 objectives	Ericsson	discussion

Not treated

Proposal 3	enabledCurrentBeamReport-r19 is defined as an optional need R field.

R2-2409641	Impact from UEI beam reporting	LG Electronics Inc.	discussion	Rel-19	NR_MIMO_Ph5-Core

R2-2409660	Discussion on the modelling of the UE-initiated beam report	Xiaomi	discussion	Rel-19	NR_MIMO_Ph5-Core

R2-2409774	Discussion on UE-initiated/event-driven beam management	vivo	discussion	Rel-19	NR_MIMO_Ph5-Core

R2-2409891	Discussion on UE-initiated/event-driven beam management	SHARP Corporation	discussion	NR_MIMO_evo_DL_UL-Core

R2-2410202	Impacts from other NR MIMO Phase 5 objectives	Ericsson	discussion

R2-2410250	RAN2 Aspects of the NR MIMO	Nokia Corporation	discussion	Rel-19	NR_MIMO_Ph5-Core

R2-2410327	Discussion on UE-initiated/event-driven beam management	CMCC	discussion	Rel-19	NR_MIMO_Ph5-Core

R2-2410355	Discussion on MAC CE impact of Rel-19 MIMO	NEC	discussion

R2-2410430	Discussion on UE initiated beam reporting	Qualcomm Incorporated	discussion

R2-2410524	Discussion on UE-initiated Beam Reporting and CSI enhancement	Samsung	discussion	Rel-19	NR_MIMO_Ph5

R2-2410618	Enhancements for UE-initiated/event-driven beam management	Huawei, HiSilicon	discussion	Rel-19	NR_MIMO_Ph5-Core

R2-2410771	Consideration on the UEIBM for R19 MIMO	ZTE Corporation	discussion	Rel-19	NR_MIMO_Ph5-Core

## 8.13	NR sidelink multi-hop relay

(NR_SL_relay_enh2; leading WG: RAN2; REL-19; WID: RP-242349)

Time budget: 1 TU

Tdoc Limitation: 3 tdocs

### 8.13.1	Organizational

LSs and rapporteur input, including workplan, etc.

### 8.13.2	Relay discovery and (re)selection

Enhancements to relay dscovery and (re)selection to support one additional hop relay (remote UE  first relay UE  last relay UE  gNB). Extensibility to a second additional hop in this WI is considered as a design criterion.

Email discussion summary

R2-2410305	Report of [POST127][401][Relay] MH relay discovery and (re)selection	LG Electronics Inc.	discussion	Rel-19

Revised in R2-2410891

R2-2410981	Report of [POST127][401][Relay] MH relay discovery and (re)selection	LG Electronics Inc.	discussion	Rel-19

Noted

[Conditions for discovery]

[EASY] Proposal 1a: A lower bound of Uu RSRP is not required for the intermediate Relay UE.

[DISCUSSION] Proposal 1b: For the intermediate Relay UE, RAN2 needs to discuss whether upper bound of Uu RSRP is required .

Discussion:

AT&T wonder if this relates to whether the UE could also be a last relay UE.

ZTE think no upper bound is needed; they understand that the UE cannot act as an intermediate and last relay UE at the same time, and as long as it cannot act as a last it can act as an intermediate.

NEC want to clarify that the Uu RSRP refers to the serving cell of the last relay UE.

Huawei think the intermediate relay UE should first connect to the last relay UE, then stop Uu monitoring.

vivo would like to have the upper bound to avoid being too close to cell centre (and for the last relay UE as well).

OPPO wonder if the intermediate relay UE will check the Uu RSRP threshold for the remote UE and how to capture the possible dependency on acting as a last relay UE.  They think having a threshold defined is needed to clarify this dependency.

Apple think the lower bound of the last relay UE criterion can be the upper bound of the intermediate relay UE, so no need for a separate upper bound.

Samsung would like to have the upper bound to avoid selecting an intermediate node which is closer than the last relay node, which seems strange.

Xiaomi think the two relay UEs could be on different cells, so the intermediate could have high RSRP with a different cell from the last relay UE.

LG think in the inter-cell case, if the other cell does not support multihop, the intermediate UE should not engage in multihop.

InterDigital see value to the bound for some cases, but the network could choose not to configure it and it does no harm to have the option.

Fraunhofer think we should consider a relay UE with no Uu capability that gets all information through the relay link.  Huawei understand that the relay must be authorised by the network first, which seems to require Uu operation.

Qualcomm think the intermediate UE’s serving cell may not support relaying, otherwise the UE could be the last relay UE.

LG understand that all relay UEs have to have the capability for Uu operations.

Huawei think the upper bound might be the same as for remote UE operation, since it has to operate as a remote UE first.

LG think for the inter-cell case, if the sidelink frequency is the same as Uu, a UE in a non-supporting cell cannot do sidelink/relay, so we have to exclude that case.

Kyocera think there would be overlap between the last relay UE and the intermediate relay UE bounds.  They think we could have a requirement to check for operation as a last relay UE first.

Qualcomm think the in-coverage intermediate relay UE is largely for the case of non-supporting cells.

NEC think the scenario can be simplified to say that we do not support inter-cell cases at all.

Samsung wonder what the specification impact for the inter-cell cases would be.

ZTE wonder why we need to discuss whether the sidelink and Uu frequencies are the same; they think we have not developed enough background on this issue.  If there is support they think it could be a WA.

Ericsson have somewhat the same concern as Qualcomm and think there is relation to downselection between approaches 1 and 2, e.g., if the intermediate UE is in idle/inactive.

OPPO understand that the proposed agreement is a basic principle and does not relate to approaches 1 and 2, for the case where the sidelink and Uu frequencies are the same; the UE should follow the cell configuration.

Qualcomm think the cell could have discovery/communication resources but not support of multihop, and then the intermediate relay UE could operate.

Apple think the intra-frequency sidelink/Uu case is legacy behaviour, and they wonder if we are looking at the case that the cell does not support multihop but provides the thresholds for single-hop.  LG are concerned with a cell that does not support any relaying, and they think we have to follow legacy operations.

LG think is the cell supports multihop, it will presumably support legacy relaying.

Apple think in LG’s scenario, the sidelink frequency would have to be from preconfiguration.

Huawei think the main aim is coverage extension and we should not spend time on the inter-cell case.  Qualcomm think inter-cell is the main use case and think there is no spec impact to support it.

OPPO have a similar understanding to Huawei, based on the Rel-17/18 principle, so they think the inter-cell case should be excluded.

AT&T agree with Huawei that the main use case is coverage extension, and inter-cell is an example of that but not the only one.

Ericsson think we should discuss the co-channel case first in the presence of relay support.

Apple think there is a bit of tension between the upper bound and the last proposed agreement on operating as a last relay UE; it seems to follow that the upper bound for the intermediate is the same as the lower bound of the last.

Agreements:

A lower bound of Uu RSRP is not required for the UE to operate as an intermediate Relay UE.

If a UE can operate as a last relay UE according to the restrictions configured by the network, it does not operate as an intermediate relay UE towards the same cell for any remote UE (i.e., minimise number of hops when possible).  FFS if operation as an intermediate relay UE towards a different cell would be supported.

The network can configure an upper bound of Uu RSRP for the UE to operate as an intermediate relay UE.  If the upper bound is not configured, there is no threshold, but this does not override the previous agreement.  FFS if there would be additional restrictions in the case of no upper bound.

[EASY] Proposal 2: RAN2 understands that the intermediate Relay UE can initiate the announcement message when receiving announcement message or, at least, when the intermediate Relay UE has the information for the announcement message provided by the last Relay UE.

Discussion:

CATT think we need to clarify SA2 conclusions related to this issue on the establishment of PC5-S connection.

Qualcomm think P2 is in SA2 scope and they already have a procedure.

[EASY] Proposal 3: The discovery announcement message can be forwarded at the intermediate Relay UE when the SD-RSRP or SL-RSRP between the last/parent intermediate Relay UE and itself is above a configured threshold.

Further clarification can be discussed in RAN2.

Discussion:

Qualcomm agree with CATT’s understanding that a PC5-S connection is required, and therefore this proposal is not needed.

InterDigital think P2 is harmless and the wording is agnostic to the status of the PC5-S connection, but receiving the announcement message should take place on a PC5-S connection.

OPPO think the intermediate relay will act as a remote UE first and decide if it will do multihop.

Kyocera have the same understanding that there needs to be a PC5-S connection.

Huawei have a strong concern on P3; they see that U2N relay is very different from this scenario and we should not use sidelink RSRP as a criterion for forwarding the messages, and first responder cases may need to work even when the link is marginal.

ZTE think establishing the PC5-S connection first is unreasonable before sending a broadcast message; they think we need to send an LS to SA2 for clarification.  LG agree with ZTE and think the AS layer needs to check the link quality.

InterDigital understand the UE relies on link quality to avoid setting up a link with a bad SL connection, even for public safety use cases.  Qualcomm understand that having the PC5-S connection is enough.

Qualcomm understand that in SA2, the PC5-S connection is supposed to guarantee that the link is OK.

ZTE think when establishing the PC5-S connection, the link quality is not checked.

Lenovo think similar issues were discussed in the previous release and the additional link quality checks were not agreed.

vivo think we should check with SA2.

[EASY] Proposal 4: The following model B discovery forwarding condition can be baseline for further discussion:

- For Model B, the intermediate Relay UE forwards the solicitation message only if the PC5 RSRP between the Remote UE (or intermediate Relay UE) and the intermediate Relay UE is above a threshold.

- For Model B discovery, upon discovery response messages reception, the Remote UE considers an intermediate Relay UE(s) as a candidate first relay UE(s) along the path to the last Relay UE if the SD-RSRP towards the first intermediate Relay UE is above a configured threshold. (modified by Ericson’s comment)

- For Model B, no AS criterion is needed for the intermediate Relay UE(s) to forward the response message to the Remote UE.

[EASY] Proposal 5a: For the discovery announcement message initiation (in the case of model A), the last Relay UE doesn’t need to check PC5 AS condition.

[DISCUSSION] Proposal 5b: In the case of discovery model B, the following two options can be further discussed

(option 1) For the discovery response message transmission (in the case of model B), the last Relay UE needs to check the PC5 AS condition before sending discovery response message to the (first) intermediate Relay UE.

(option 2) As the legacy Rel-17 U2N relay, the U2N Relay UE doesn’t have any PC5 AS conditions for transmitting the announcement/response discovery message. The same principle can be applied to the last Relay UE.

[(Re)selection triggering conditions]

[EASY] Proposal 6a: The following relay selection triggering conditions for Remote UE are supported for the multi-hop relay operation.

[Relay selection triggering conditions for Remote UE]

q)	If the Remote UE has no serving cell;

j) If the Remote UE does not have a selected intermediate Relay UE;

[ESAY] Proposal 6b: The following relay re-selection triggering conditions for Remote UE are supported for the multi-hop relay operation at least when there is only one intermediate Relay UE.

[Relay re-selection triggering conditions for Remote UE]

q)	If the SL-RSRP of the currently selection intermediate Relay UE is below a configured threshold;

r)	If the SD-RSRP of the currently selected intermediate Relay UE is below a configured threshold;

s)	If the upper layer indicates not to use the currently selected intermediate Relay UE;

t)	If the upper layer of the selected intermediate Relay UE requests the release of the PC5-RRC connection;

u)	If the RLF is detected on the PC5-RRC connection with the current intermediate Relay UE;

v)	If the Remote UE receives a notification message from the intermediate Relay UE caused by one of the following:

h-1) if intermediate Relay UE detects PC5 RLF between intermediate Relay UE and last Relay UE (or serving intermediate Relay UE)

h-2) if intermediate Relay UE receives RRCReconfiguration message for HO (if the intermediate Relay UE is in RRC_CONNECTED)

h-3) if intermediate Relay UE performs cell reselection

h-4) if intermediate Relay UE fails Uu RRC connection establishment/Resume via last Relay UE

h-5) if intermediate Relay UE receives upper layers request the release of the PC5-RRC connection between intermediate Relay UE and last Relay UE (or serving intermediate Relay UE)

[EASY] Proposal 6C: Following condition can be additionally considered for relay selection

-	  If direct Uu signal strength of current serving cell of the Remote UE is below a configured signal strength threshold.

[EASY] Proposal 6D: The following solution is not considered in this release:

-	  The last Relay UE generates notification message when the Uu RSRP is decreased under a configured threshold

[EASY] Proposal 6E: The following solution is not considered in this release:

-	  The intermediate Relay UE generates notification message when the PC5 RSRP is decreased under a configured threshold.

[Quite EASY] Proposal 7: The notification message generated by last Relay UE or by (not first) intermediate Relay UE can be [forwarded/delivered] toward the Remote UE via the first intermediate Relay UE.

[EASY] Proposal 8: Any relay (re)selection triggering condition is not specified for intermediate Relay UE.

[(Re)selection criteria]

[EASY] Proposal 9: Rel-18 relay (re)selection criteria can be reused for relay (re)selection criteria for the Rel-19 multi-hop.

- SL-RSRP and SD-RSRP can be used for relay selection/reselection criteria.

- In both cases, it is left to remote UE implementation whether to use SL-RSRP or SD-RSRP for relay (re)selection trigger evaluation in case of no data transmission.

- Besides the PC5 link quality, RAN2 does not pursue other AS criteria for relay (re)selection.

[Scenarios and UE roles]

[EASY] Proposal 10: The last Relay UE of a multi-hop relay operation can also act as U2N Relay UE of a single-hop relay operation simultaneously.

[Quite EASY] Proposal 11: An intermediate Relay UE in multi-hop relay operation can also act as a Remote UE in single-hop relay operation as long as there is only single PC5 connection between Last Relay UE and intermediate Relay UE for both multi-hop and single-hop relay operation.

[Quite EASY] Proposal 12: An Intermediate Relay UE can serve multiple multi-hop indirect paths of different Remote UEs. The intermediate Relay UE(also can act as an Remote UE) cannot support multiple PC5 unicast links with same/different last/U2N/parent intermediate Relay UE(s) for support of different indirect paths. One intermediate Relay UE should have only one single PC5 unicast link for one physical intermediate/last Relay UE and only single PC5 unicast link for one physical Remote UE.

Discussion:

Qualcomm think the intermediate relay UE could support multi-path for different remote UEs.  They see some dependency on the control plane approach.

InterDigital think we can agree for the remote UE.

OPPO think Qualcomm’s concern should be OK since the second sentence already calls out that the intermediate relay UE can also act as a remote UE.

[Supported and unsupported cases of multiple connections, edited by chair for clarity]

[EASY] Proposal 13: following cases are supported:

- One last Relay UE can have two connections with one intermediate Relay UE and one Remote UE (the intermediate Relay UE and Remote UE are physically different UE).

- Two physically different Remote UE(s) can have each indirect path via the same intermediate Relay UE and the same last Relay UE.

FFS if the last relay UE can use the same L2ID for both of the connections in either case.

[EASY] Proposal 14: following cases are not supported:

-	Physically one intermediate Relay UE has two PC5 unicast link with the physically one last Relay UE.

-	  Physically different two Remote UEs have each indirect connection via physically one intermediate Relay UE. The intermediate Relay UE has two PC5 unicast link with the physically one last Relay UE

-	  One intermediate Relay UE, which also acts as Remote UE, can have two different indirect connection via physically different last Relay UE

-	 Two physically different Remote UE can have each indirect path via the physically same intermediate Relay UE and physically different last Relay UE(s). i.e., physically one intermediate Relay UE has connections with physically two different last Relay UE(s) to support physically different Remote UE(s).

- Two different indirect links at the physically same Remote UE can have connections with the gNB via the physically same intermediate Relay UE and physically same last Relay UE.

- Two different indirect links at the physically same Remote UE can have connections with the gNB via the physically same intermediate Relay UE and physically different last Relay UE.

- (2.5D-1) One intermediate Relay UE, which also acts as Remote UE, can have two different indirect connection via physically different last Relay UE. Each last Relay UE has a connection with different cells in the same gNB.

- (2.5D-2) Two physically different Remote UE can have each indirect path via the physically same intermediate Relay UE and physically different last Relay UE(s) i.e., physically one intermediate Relay UE has connections with physically two different last Relay UE(s) to support physically different Remote UE(s). Each last Relay UE has a connection with different cells in the same gNB.

- (2.5D-3) Two different indirect links at the physically same Remote UE can have connections with the gNB via the physically same intermediate Relay UE and physically different last Relay UE. Each last Relay UE has a connection with different cells in the same gNB.

Agreements:

The following cases are supported for L2 multihop relay:

- One last Relay UE can have two connections with one intermediate Relay UE and one Remote UE (the intermediate Relay UE and Remote UE are physically different UE).

- Two physically different Remote UE(s) can have each indirect path via the same intermediate Relay UE and the same last Relay UE.

FFS if the last relay UE can use the same L2ID for both of the connections in either case.

Cases with two indirect paths to the gNB for the same remote UE are not supported.

An Intermediate Relay UE can serve multiple multi-hop indirect paths of different Remote UEs.

If the intermediate Relay UE also is acting as a Remote UE, it cannot support different indirect paths to the gNB with same/different last/U2N/parent intermediate Relay UE(s) based on different PC5 unicast links.

Other contributions

R2-2410032	Discussion on multi-hop Relay discovery and (re)selection	ZTE Corporation, Sanechips	discussion	Rel-19	NR_SL_relay_multihop-Core

Noted

Proposal 2: RAN2 sends LS to ask SA2 to clarify the following questions:

- whether an intermediate relay needs to establish PC5 link with the relay from which the discovery announcement message is received before sending/forwarding the discovery announcement message.

- whether an intermediate relay can send/forward discovery announcement messages for different RSCs/last relay UEs(if Root relay info is included) or need to select a parent relay with a RSC and only sending/forwarding the discovery announcement message for the selected RSC.

- whether an intermediate relay can have two parent relays towards different last relay UEs/gNBs for different remote UEs.

Discussion:

Qualcomm think the SA2 spec is clear and companies can check internally.

Huawei think we could skip the third bullet on cross-path topologies.

OPPO think SA2 will not understand the second question without some background.

LG think we could skip the second question as well; they understand that this case was excluded at plenary.  ZTE think it relates to the architecture and they would like to check whether it is possible.

Agreement:

RAN2 sends LS to ask SA2 to clarify whether an intermediate relay needs to establish PC5 link with the relay from which the discovery announcement message is received before sending/forwarding the discovery announcement message.

[AT128][411][Relay] LS to SA2 on relay discovery announcement (ZTE)

Scope: Draft an LS to ask SA2 to clarify whether an L2 intermediate relay needs to establish PC5 link with the relay from which the discovery announcement message is received before sending/forwarding the discovery announcement message.

Intended outcome: Approved LS (without CB if possible) in R2-2411125

Deadline: Wednesday 2024-11-20 1600 EST

R2-2411125	LS to SA2 on relay discovery announcement	ZTE Corporation	LS out	Rel-19	NR_SL_relay_multihop-Core	To:SA2

WI code to be changed to NR_SL_relay_multihop-Core

Approved with this change as R2-2411184

R2-2411184	LS to SA2 on relay discovery announcement	ZTE Corporation	LS out	Rel-19	NR_SL_relay_multihop-Core	To:SA2

Approved

R2-2410827	Considerations on relay discovery and (re)selection	Samsung	discussion	Rel-19	NR_SL_relay_multihop-Core

Proposal 4: RAN2 is kindly asked to discuss whether the hop count information is included in the discovery message.

Proposal 5: RAN2 is kindly asked to discuss whether there is need to limit the number of hops.

R2-2410587	Relay discovery and (re)selection for multi-hop Relay	Huawei, HiSilicon	discussion	Rel-19	NR_SL_relay_multihop-Core

Proposal 2: Reuse the Rel-17 direct discovery procedures for the intermediate relay UE to find a parent relay UE first and then for the remote UEs to find the first relay UE in multi-hop scenario.

Proposal 3: The intermediate relay UE performs discovery procedure as a relay UE after establishing the PC5 connection with its parent relay UE. In this case it does not need to check the AS condition.

R2-2409967	Relay discovery and selection for Multi-hop UE-to-NW Relay	Apple	discussion	Rel-19	DUMMY

Proposal 1 	An in-coverage relay UE shall always determine whether it can be the “Last relay UE” first and may become intermediate relay UE only if it cannot satisfy the conditions of last relay UE.

Proposal 3 	Only mode 2 RA is supported for remote UE and intermediate relay UE.

R2-2409632	Discussion on topology and intermediate relay UE (re)selection	vivo	discussion	Rel-19

R2-2409728	Discussion on multi-hop U2N relay discovery and relay selection	NEC Corporation	discussion	NR_SL_relay_multihop-Core

R2-2409730	Discovery and relay (re)selection for multi-hop U2N relay	OPPO	discussion	Rel-19	NR_SL_relay_multihop-Core

R2-2409859	Discussion on Multi-hop Discovery and (Re)selection	CATT	discussion	Rel-19	NR_SL_relay_multihop-Core

R2-2409906	Discussion on relay discovery and (re)selection for NR sidelink multi-hop relay	TOYOTA InfoTechnology Center	discussion	Rel-19	NR_SL_relay_multihop-Core

R2-2410007	Discovery and Relay (Re)Selection for Multi-hop U2N Relays	InterDigital	discussion	Rel-19

R2-2410104	Multi-hop relay discovery and reselection	China Telecom	discussion	Rel-19	DUMMY

R2-2410150	discussion on discovery and relay (re)selection	Ericsson	discussion	Rel-19

R2-2410183	Remaining issues on multi-hop U2N Relay Discovery message forwarding	ASUSTeK	discussion	Rel-19	NR_SL_relay_multihop

R2-2410281	Relay (re)selection in Multi-hop relay	Lenovo	discussion	Rel-19

R2-2410288	Relay discovery aspects for multi-hop relay	Nokia	discussion	NR_SL_relay_multihop

R2-2410298	Discussion on the discovery and relay (re)selection for multi-hop U2N relay	LG Electronics Inc.	discussion	Rel-19

R2-2410392	Multi-hop relay selection/re-selection	Sony	discussion	Rel-19	NR_SL_relay_multihop-Core

R2-2410570	Discovery and (re)selection under multihop relay 	Kyocera	discussion

R2-2410619	Relay discovery and (re)selection	TCL	discussion	Rel-19

R2-2410840	Relay discovery and (re)selection	TCL	discussion	Rel-19

R2-2410704	discussion on Relay discovery and (re)selection for multi-hop relay	Sharp	discussion	Rel-19	NR_SL_relay_multihop-Core

R2-2410734	Discovery and Relay (re)selection for multi-hop U2N relay	Qualcomm Incorporated	discussion	NR_SL_relay_multihop-Core

### 8.13.3	Control Plane Procedures and SRAP impact

Contributions should focus on control plane procedures and can include SRAP impact and QoS handling to support additional hops.

Email discussion summary

R2-2410006	Report of [Post127][402][Relay] Multi-hop relay control plane	InterDigital	discussion	Rel-19

Noted

[Connection establishment, two approaches, edited by chair for clarity]

Proposal 1 – In one approach (“approach 1”) of U2N relays, each of the Intermediate Relay UEs must be in RRC_CONNECTED when the U2N remote UE is in RRC_CONNECTED.  Connection establishment in the U2N remote UE first requires that each Intermediate Relay UE which is in RRC_IDLE/RRC_INACTIVE first enters RRC_CONNECTED.  FFS whether connection establishment of an Intermediate Relay UE (other than the Last Relay UE) is captured in specification as connection establishment of a remote UE or a relay UE.

Proposal 2 – The figure and description under P1 of R2-2410006 serves as a baseline connection establishment procedure for multi-hop U2N Relays if Approach 1 (all relay UEs must be in RRC_CONNECTED when the remote UE is in RRC_CONNECTED) is adopted.

Discussion:

OPPO think P2 could say “for discussion”.

Kyocera think P1 implies more delay than may be necessary for multiple connection setups.

Samsung think we might not mandate staying connected after initial access.  InterDigital think this is different from what was proposed in the email discussion.

Proposal 3 – In one approach (“approach 2”) of U2N relays, Intermediate Relay UEs (other than the Last Relay UE) can be in RRC_IDLE/RRC_INACTIVE when the U2N remote UE is in RRC_CONNECTED.

Proposal 4 – In approach 2, any relay UE which happens to be in RRC_CONNECTED can obtain its relaying RLC channel configuration in dedicated signalling.

Proposal 5 – The figure and description under P4 of R2-2410006 serves as a baseline connection establishment procedure for multi-hop U2N Relays if Approach 2 (relays other than the Last Relay may/may not remain in RRC_IDLE/RRC_INACTIVE when the remote UE is in RRC_CONNECTED) is adopted.

Discussion:

OPPO think the RRC_CONNECTED case can be added in P3.

LG wonder how P4 works; how does the dedicated signalling get to the intermediate relay UE?  InterDigital understand that being in RRC_CONNECTED means it can communicate with the network.

LG think in the RRC_CONNECTED case, there is no difference between approaches 1 and 2.  Qualcomm understand that there may be an intermediate UE in between with its own serving cell (not in RRC_CONNECTED).

Huawei wonder how the intermediate relay UE that is not in RRC_CONNECTED state can be authorised.  InterDigital indicate that this was discussed by email, and the understanding is that the authorisation comes from upper layers while the UE is connected, and then it can transit to idle/inactive.  Huawei think the RAN needs to know if the relay UE is authorised.

Apple understand that P4 assumes the relay UE also acts as a remote UE.

Agreements:

In one approach (“approach 1”) of U2N relays, each of the Intermediate Relay UEs must be in RRC_CONNECTED when the U2N remote UE is in RRC_CONNECTED.  Connection establishment in the U2N remote UE first requires that each Intermediate Relay UE which is in RRC_IDLE/RRC_INACTIVE first enters RRC_CONNECTED.  FFS whether connection establishment of an Intermediate Relay UE (other than the Last Relay UE) is captured in specification as connection establishment of a remote UE or a relay UE.  FFS if SRB0 would be forwarded before connection establishment of each intermediate relay UE.

The figure and description under P1 of R2-2410006 serves as a baseline connection establishment procedure for discussion for multi-hop U2N Relays if Approach 1 (all relay UEs must be in RRC_CONNECTED when the remote UE is in RRC_CONNECTED) is adopted.

In one approach (“approach 2”) of U2N relays, Intermediate Relay UEs (other than the Last Relay UE) can be in any RRC state when the U2N remote UE is in RRC_CONNECTED.

In approach 2, any intermediate relay UE which happens to be in RRC_CONNECTED towards the last relay UE’s serving gNB and is operating as a remote UE is assumed to obtain its relaying configuration (RLC channel configuration, SRAP configuration, etc.) in dedicated signalling from the last relay UE’s serving gNB via the U2N connection.

The figure and description under P4 of R2-2410006 serves as a baseline connection establishment procedure for discussion for multi-hop U2N Relays if Approach 2 (relays other than the Last Relay may/may not remain in RRC_IDLE/RRC_INACTIVE when the remote UE is in RRC_CONNECTED) is adopted.

Proposal 7 – The scenario of the remote UE RRC_CONNECTED to one cell while an Intermediate Relay UE is RRC_CONNECTED to a different cell is supported only in approach 2.  FFS whether the scenario needs to be supported.

Proposal 8 – For approach 1, QoS split for each hop is performed by the network.

Proposal 9 – For approach 2, QoS split between the Uu hop and all remaining hops is performed by the network.  FFS how to split the QoS over each of the individual remaining hops.

Discussion:

Samsung understand that P8 refers to the baseline procedure.

Qualcomm understand that there is no per-hop QoS split as such.  They see it as just gNB implementation behaviour.  OPPO think “QoS split” is what we used in Rel-17 and makes sense.  ZTE also think “QoS split” is correct, and something like “QoS management” would be a much larger scope.

AT&T think we should not say that the network is “managing” QoS since it is not performing direct procedures on the UEs; the UE has to manage however the QoS is allocated.

Samsung think it is the gNB’s task to ensure the e2e QoS, and if there is concern about the term “QoS split”, we could say “the gNB guarantees the e2e QoS”.

Xiaomi think there may be assistance information from the UE side, so it is not purely gNB implementation.

Agreements:

For the baseline procedure, the PDB for each hop is provided by the network according to gNB implementation.   This does not preclude the possibility of assistance information from the UE(s).

If enhancements allow some intermediate relay UEs to remain in idle/inactive, the PDB split between the Uu hop and all remaining hops is performed by the network.  FFS how to split the PDB over each of the individual remaining hops.

[System information]

Proposal 6 – In multi-hop, the U2N Remote UE uses the SI of the cell of the Last Relay UE, which is forwarded via the Intermediate Relay UE(s).  FFS on how to perform the forwarding.

Discussion:

ZTE wonder if it implies anything about how to use the SI.

Lenovo wonder if the intermediate relay UEs could receive the SI directly when in coverage, vs. receiving it from the last relay UE.

Qualcomm think the intermediate relay UEs in RRC_CONNECTED could acquire the SI on demand directly.  ZTE wonder if the intermediate relay UE would have to store the SI.  Qualcomm are thinking of the case where the intermediate relay UE already has it.

Agreement:

In multi-hop, the U2N Remote UE acquires the SI of the cell of the Last Relay UE, which is forwarded via the Intermediate Relay UE(s).  FFS how to perform the forwarding and whether an intermediate relay UE can forward available SI directly (rather than retrieving it from the last relay UE).

[Use case and scenario support]

Proposal 10: The scenario of two remote UEs connected to different cells via a single relay is supported only by approach 2.  RAN2 discusses if this is a valid use case to support in this release or future releases.

Proposal 11: If the scenario of the relay and remote UE connected to different cells is supported, service continuity for this scenario is outside of Rel19 scope.

[Additional details]

Proposal 12: Local UE ID of the remote UE is provided by the gNB.  FFS for approach 2, how it is provided to relay UEs in RRC_IDLE/RRC_INACTIVE and which cell/gNB provides it.

Proposal 13: RAN2 discuss, in approach 2, whether RLC channel configuration is provided by the gNB or is obtained by SIB/Preconfiguration.

[Pros and cons of approaches]

Proposal 14: Approach 1 adds signalling and latency associated with connection (re)establishment of the relay UEs.  RAN2 discuss whether this is a concern.

Proposal 15: Approach 1 makes connection establishment at the remote UE dependant on successful connection establishment by each relay. RAN2 discuss whether this is a concern.

Proposal 16: RAN2 discusses which approach has a higher signalling overhead.

Report of unofficial offline

R2-2411128	Way forward on SL multi-hop relay	LG Electronics Inc.	discussion	Rel-19

Noted

Proposal:

Design baseline procedure based on the case in which all intermediate relay UEs for a remote UE are/become in RRC_CONNECTED.

1. Support this case based on existing U2N framework

2. Continue to discuss whether/how to support the case that intermediate relay UEs are not in RRC_CONNECTED.

3. Other solutions for optimization are not precluded e.g. in terms of signaling and latency

4. Support of other cases are not precluded

Discussion:

Apple have some concerns about the wording as related to e2e connection setup.  They understand the baseline procedure should be based on the existing U2N framework, and each intermediate relay UE should initiate connection setup after receiving a trigger from the remote UE.

Qualcomm would like to say “for the cases” rather than “based on the cases”.

Samsung think the spirit of the proposal is that first we should set up the baseline, and this is easier to do with approach 1; after we have this baseline, we can look at whether there is some optimisation space to reduce latency/overhead.

InterDigital are OK with bullet 1 but would like to change the wording of bullet 2.  They do not agree with bullet 3 because of the shortage of TUs.

Nokia also think we should remove bullet 3.

Qualcomm are concerned that the approach 2 procedure could be quite different from approach 1.

AT&T note that we have to accommodate the case with all UEs in RRC_CONNECTED.  Nokia think both approaches cover this case and we cannot decide one approach based on this common case.

Samsung think we can focus on the baseline procedure design, or we can do some enhancements on top of it, and after another meeting or two we can evaluate our progress.

Agreement:

Support a baseline procedure for the case in which the intermediate relay UEs for a remote UE all transition to RRC_CONNECTED (if not already there) when the remote UE goes to RRC_CONNECTED.

1. Support this case based on existing U2N framework, with all the UEs RRC_CONNECTED to the last relay UE’s serving cell.

2. Continue to discuss whether/how to support the case that intermediate relay UEs do not all move to RRC_CONNECTED when the remote UE triggers connection establishment.

All agreements in this WI apply to both these cases unless otherwise specified.

[Post128][401][Relay] Control plane baseline solution (InterDigital)

Scope: Develop the baseline control plane procedures for connection establishment, and paging/system information forwarding, towards a stage 3 development stage.

Intended outcome: Report to RAN2#129

Deadline: Long

Other contributions

Approach 1

R2-2410139	Discussion on control plane and QoS handling for NR sidelink multi-hop relay	Spreadtrum, UNISOC	discussion	Rel-19

Proposal 1: Support multi-hop U2N relay Approach 1 which the NW needs to directly control each of intermediate relay UEs via Uu RRC signalling and all relay UEs must be in RRC_CONNECTED when the remote UE is in RRC_CONNECTED.

Proposal 2: The connection establishment procedure of single-hop U2N relays can be as a baseline for multi-hop U2N relays if Approach 1 is adopted.

Proposal 3: For remote UE end-to-end QoS split in multi-hop L2 U2N relay, the mechanism in single-hop L2 U2N relay is as baseline, i.e. gNB implementation can handle the end-to-end QoS split over multi-hop.

Approach 2

R2-2410149	discussion on control plane procedure	Ericsson	discussion	Rel-19

Proposal 1	RAN2 to work on two scenarios including:

a.	Scenario 1: Intermediate Relay UE(s) and Last  Relay UE are in the same serving cell.

b.	Scenario 2: Relay UEs can be in different serving cells of the same gNB.

Proposal 4	Same as in legacy, when L2 U2N Remote UE is in RRC_CONNECTED, L2 U2N Last Relay UE also needs to be in RRC_CONNECTED.

Proposal 5	To down-select Approach 2 between Approach 1 and Approach 2, i.e., L2 U2N Intermediate Relay UEs are not mandated to be in RRC_CONNECTED.

SRAP aspects

R2-2410290	SRAP impacts on MH relay	Nokia	discussion	NR_SL_relay_multihop

Noted

Proposal 2: An SRAP header can be added per hop to map ingress and egress RLC channels.

Discussion:

Nokia clarify that they think it is also possible to have this solution with a single SRAP header.

Apple understand the SRAP header maps e2e bearer to egress PC5 RLC channel, and we should keep this principle.  They think the exact management of the header is more of a stage 3 detail.

Samsung think the baseline is that we use the information in the SRAP header for the mapping, so maybe it’s better not to add multiple headers.

Apple understand that there is no per-hop information in the SRAP header.

Nokia are not sure that their proposal requires multiple SRAP entities.  From one relay UE’s perspective, it only needs to look at the next hop.

LG think only one SRAP header is needed; the local ID can indicate the remote UE ID.

Ericsson wonder if we are going to reuse the Rel-17 SRAP header format.  They wonder if this works if the same relay serves both a Rel-17 remote UE and a Rel-19 remote UE and needs to determine which is the next hop.

Nokia think if we do not reuse the Rel-17 header, we cannot reuse a Rel-17 UE as the last relay UE.  Chair thinks this would mean two SRAP entities at the intermediate relay UE.

OPPO are not sure if Nokia’s case is valid; they understand from SA2 colleagues that Rel-17 and Rel-19 relaying will use different RSCs.

LG understand that the header only needs one UE ID even for multihop.

R2-2409969	Discussion on SRAP for Multi-hop Layer-2 U2N Relay	Apple	discussion	Rel-19	DUMMY

Noted

Proposal 1	Each Layer-2 Intermediate Relay UE has a single PC5 SRAP entity.

Proposal 2	Remote UE ID and BEARER ID are included in SRAP header for multi-hop L2 U2N relay.

Proposal 3	For SRAP mapping in intermediate relay, a single set of PC5 Relay RLC channel configurations is used for both directions, whereas different SRAP mapping could be used in each direction.

Proposal 4	A “direction” bit to discern UL/DL is included in SRAP header.

Proposal 5	Support SRAP Control PDU including remote UE L2 ID to be transported together with the first RRC message .

Agreements:

Maintain the principle that the SRAP header enables mapping from the e2e bearer ID to the egress PC5 RLC channel.  This does not preclude changes/rewrites/addition of the SRAP header by the intermediate UEs.

Remote UE ID and BEARER ID are included in Rel-19 SRAP header for multi-hop L2 U2N relay.

Bearer mapping

R2-2410033	Discussion on control plane procedures for multi-hop SL relay	ZTE Corporation, Sanechips	discussion	Rel-19	NR_SL_relay_multihop-Core

Proposal 9: For the bearer mapping configured to intermediate relays, it is the mapping of remote UE’s RB to egress PC5 RLC channel.

Proposal 10: RAN2 to discuss how intermediate relays to determine the egress link in the downlink.

[Alternatives from text of paper related to P10:]

- Way 1: the gNB indicates the downlink routing information of each remote UE to the intermediate relay, i.e. the next node in the downstream lead to the remote UE.

- Way 2: the intermediate relay records from which node the remote UE’s packets are received in the uplink. Then the intermediate relay forwards the remote UE’s packets to the same node in the downlink.

QoS split

R2-2409860	Discussion on the Control Plane Procedures	CATT	discussion	Rel-19	NR_SL_relay_multihop-Core

Proposal 9: The network is responsible for the QoS split for each hop.

Proposal 10: Same as Rel-17 L2 U2N relay, the parameter needs to be split is PDB.

Paging/SI

R2-2409796	CP and SRAP for Multi-hop Relay	NEC	discussion

Proposal 6:	For paging and SI forwarding, the similar principles of Rel-17 U2N relay mechanism can apply to multi-hop case.

Proposal 7:	For paging and SI forwarding, there are multiple PC5-RRC connections along with the multi-hop path (i.e., among the remote UE and multiple intermediate U2N relay UEs).

Discussion:

Apple think P6 is not sufficiently clear on what the “principle” is.  ZTE have the same concern and think the proposal is too general.

R2-2409633	Discussion on CP and SRAP impact for Approach 1	vivo	discussion	Rel-19

R2-2409732	Control plane procedures of multi-hop U2N relay	OPPO	discussion	Rel-19	NR_SL_relay_multihop-Core

R2-2409968	Discussion on End-to-End Connection Setup Approaches for Multi-hop UE-to-NW Relay	Apple	discussion	Rel-19	DUMMY

R2-2410008	Control Plane Aspects for Multi-hop U2N Relays	InterDigital	discussion	Rel-19

R2-2410105	Discussion on control plane aspects for NR sidelink multi-hop relay	China Telecom	discussion	Rel-19	DUMMY

R2-2410184	Remaining issues on SRB0 message forwarding in multi-hop U2N Relay	ASUSTeK	discussion	Rel-19	NR_SL_relay_multihop

R2-2410282	Control plane in Multi-hop relay	Lenovo	discussion	Rel-19

R2-2410297	Discussion on the control plane procedure for multi-hop U2N relay	LG Electronics Inc.	discussion	Rel-19

R2-2410569	Control Plane under multihop L2 U2N relaying 	Kyocera	discussion

R2-2410588	Control plane procedures for multi-hop relay	Huawei, HiSilicon	discussion	Rel-19	NR_SL_relay_multihop-Core

R2-2410631	On approach 1	Nokia	discussion	NR_SL_relay_multihop-Core

R2-2410705	discussion on C-plane procedure for multi-hop relay	Sharp	discussion	Rel-19	NR_SL_relay_multihop-Core

R2-2410735	Control procedure for multi-hop L2 based U2N relay	Qualcomm Incorporated	discussion	NR_SL_relay_multihop-Core

R2-2410756	Consideration on CP and UP issues for multi-hop SL relay	Samsung	discussion	Rel-19	NR_SL_relay_multihop-Core

### 8.13.4	Service continuity

First priority scenarios: (A) intra-gNB multi-hop indirect to direct path switch, (B) intra-gNB multi-hop indirect to single-hop indirect path switch.  Second priority scenarios: (C) intra-gNB direct to multi-hop indirect path switch, (D) intra-gNB single-hop indirect to multi-hop indirect path switch.

R2-2410736	Consideration on multi-hop U2N relay service continuity	Qualcomm Incorporated	discussion	NR_SL_relay_multihop-Core

Noted

Proposal 2	For scenario B, RAN2 supports the case that the target U2N relay UE is a new relay UE which is not on the source relay path, and existing Rel-17 indirect path to indirect path switching can be reused.

Proposal 3	For scenario B, RAN2 does not support the case that that the target U2N relay UE is an intermediate relay UE which is on the source relay path.

Proposal 4	For scenario B, RAN2 supports the case that last relay UE is unchanged, the solution should be postponed until the control plane solution is determined.

Discussion:

CATT think P2 should say Rel-18.

OPPO understand there is no e2e PC5 connection between the remote UE and the last relay UE, so we do not need specific enhancements for this case.

Kyocera think there are cases where the remote UE can only see an intermediate relay UE that is on the path, so they have concerns about P3.  They wonder if we really need to prevent this group handover-like case.

Samsung think P3 is not meaningful because the target path would not be single-hop.

vivo think P3 is possible if the intermediate relay UE can also serve as a last relay UE.

Qualcomm explain that P3 is directed to the role change case, where the intermediate relay UE switches to a direct connection to the network.

Samsung think we do not need to consider the case of simultaneous path switches at the remote UE and the intermediate relay UE.

Ericsson think it would be difficult for the network to trigger the case in P3.

OPPO think there should be no UE impact from these decisions.

LG support the second agreement (last relay UE unchanged), but think it is too soon to say there is no UE impact.

Agreements:

For scenario B, RAN2 supports the case that the target U2N relay UE is a new relay UE which is not on the source relay path, and existing Rel-18 indirect path to indirect path switching can be reused.

For scenario B, RAN2 will support the case that the target single-hop relay UE is the last relay UE on the source path using the existing Rel-18 i2i path switching.  Spec impact will be minimised.

For scenario B, RAN2 does not specify anything to support the case that that the target U2N relay UE is an intermediate relay UE which is on the source relay path using a single procedure.  This case can be handled in the baseline control plane model, if necessary and subject to network implementation, by sequential path switches for the remote UE and the target intermediate relay UE.

R2-2409731	Service continuity of multi-hop U2N relay	OPPO	discussion	Rel-19	NR_SL_relay_multihop-Core

Noted

[Scenario A]

Proposal 1	RAN2 to confirm that the intra-gNB switching from indirect to direct path procedure defined in TS 38.300 for single hop case can be used as the baseline for multi-hop indirect to direct path switching.

Proposal 2	RAN2 to discuss event X1 /X2 can be reused in multi-hop indirect to direct path switching with the understanding that the “first relay UE” in multi-hop relay link is “serving L2 U2N Relay UE” to be reported.

[Scenario B]

Proposal 3	RAN2 to confirm the intra-gNB switching from indirect path to indirect path procedure defined in TS 38.300 in single hop case can be used as baseline for multi-hop indirect to single hop indirect path switching procedure.

[Chair’s note: “multi-hop indirect to direct” in P4 seems to be a typo for “multi-hop indirect to single-hop indirect”, i.e., scenario B]

Proposal 4	RAN2 to discuss the following measurement event can be reused in multi-hop indirect to direct path switching:

-	Event Y2

-	Event Z1 with the understanding that the “first relay UE” in multi-hop relay link is “serving L2 U2N Relay UE” to be reported.

Discussion:

Qualcomm think there might be a difference between the control plane approaches, because the intermediate relay UE may not be visible to the gNB.  LG are not sure why this makes a difference, because the intermediate relay UEs will be triggered to enter connected mode for the procedure.  Huawei agree with LG.

Qualcomm think the intermediate relay UEs will not need to enter connected mode even during path switch.

Agreements for baseline control plane model:

The intra-gNB switching from indirect to direct path procedure defined in TS 38.300 for single hop case can be used as the baseline for multi-hop indirect to direct path switching.

Event X1 /X2 can be reused in multi-hop indirect to direct path switching with the understanding that the “first relay UE” in multi-hop relay link is “serving L2 U2N Relay UE” to be reported.

The intra-gNB switching from indirect path to indirect path procedure defined in TS 38.300 in single hop case can be used as baseline for multi-hop indirect to single hop indirect path switching procedure.

The following measurement events can be reused in multi-hop indirect to single-hop indirect path switching:

-	Event Y2

-	Event Z1 with the understanding that the “first relay UE” in multi-hop relay link is “serving L2 U2N Relay UE” to be reported.

R2-2410589	Discussion on service continuity for Multi-hop Relay	Huawei, HiSilicon	discussion	Rel-19	NR_SL_relay_multihop-Core

Noted

[Figures in proposals refer to R2-2410589]

Proposal 2: Agree the above procedure in Figure 1 as a baseline for intra-gNB multi-hop indirect to direct path switching procedure to be captured in TS 38.300.

Proposal 4: Agree the above procedure in Figure 2 as a baseline for intra-gNB multi-hop indirect to single-hop indirect path switching to be captured in TS 38.300.

Discussion:

Samsung want to clarify if it means we can reuse the existing flow to derive the stage 2 procedures.  Huawei think it would be difficult to reuse the existing clause and we would probably create a new one.

Agreements:

Figure 1 of R2-2410589 can be used as a baseline for intra-gNB multi-hop indirect to direct path switching procedure to be captured in TS 38.300.

Figure 2 of R2-2410589 can be used as a baseline for intra-gNB multi-hop indirect to single-hop indirect path switching to be captured in TS 38.300.

R2-2410034	Discussion on service continuity for multi-hop SL relay	ZTE Corporation, Sanechips	discussion	Rel-19	NR_SL_relay_multihop-Core

Noted

Proposal 3: When an intermediate relay detects PC5 link quality of an intermediate hop becoming worse, it should notify the downstream intermediate relays/remote UEs.

Proposal 4: Introduce a new measurement report event for multi-hop indirect path switching:

-  Event X3: Upon receiving a notification that an intermediate hop becomes worse than threshold.

Discussion:

NEC think P3 can only work when the intermediate relay is in RRC_IDLE/RRC_INACTIVE; otherwise it will be directly reported to the network.  They also think the intermediate relay UE can report its Uu link quality towards the remote UE.  ZTE think it also works in RRC_CONNECTED, because if only the intermediate relay UE reports link quality to the network, the network does not have enough information and also needs the measurements from the remote UE to determine what the target should be.

InterDigital agree with ZTE, and they think the main problem is that the network could receive separate events at different times; the intention is to coordinate the events so that they make sense at the network.

Lenovo think RLF at the intermediate relay UE will already result in a notification message, and the remote UE will perform re-establishment.  They wonder what the remote UE would do.

ZTE intended that the mechanism would be used before RLF to preserve QoS rather than wait for RLF.

Huawei think this is an enhancement that can be looked at in a later phase.

Samsung think the whole thing should be under network control, so informing the remote UE is an optimisation.

OPPO agree with Huawei and Samsung; they see that there are existing mechanisms to reduce latency and this is an optimisation.

LG also think it can go through the network.

CATT also see it as an optimisation with no strong motivation.

Proposal 7: For the measurements of the serving multi-hop indirect path in the measurement report, it may include at least the first intermediate relay UE ID and the PC5 link quality with the first intermediate relay. FFS the PC5 link quality status of intermediate hops.

Discussion:

OPPO note that the legacy measurement report also includes the cell ID.

Qualcomm wonder if the UE should report the source or only the target.  ZTE think this is just about the contents of the measurement report.

Qualcomm think the gNB should already know the serving intermediate relay.

R2-2409634	Discussion on Service continuity for multi-hop relay	vivo	discussion	Rel-19

R2-2409861	Intra-gNB Service Continuity for Multi-hop U2N Relay	CATT	discussion	Rel-19	NR_SL_relay_multihop-Core

R2-2410106	Discussion on service continuity for multi-hop relay	China Telecom	discussion	Rel-19	DUMMY

R2-2410185	Discussion on measurement report for multi-hop U2N Relay	ASUSTeK	discussion	Rel-19	NR_SL_relay_multihop

R2-2410201	Service Continuity for Multi-Hop Relays	Ericsson	discussion	Rel-19

R2-2410283	Service continuity for Multi-hop system	Lenovo	discussion	Rel-19

R2-2410299	Discussion on service continuity for multi-hop U2N relay	LG Electronics Inc.	discussion	Rel-19

R2-2410354	Considerations on Service Continuity of Multi-hop Relay	NEC	discussion	Rel-19

R2-2410706	discussion on service continuity for multi-hop relay	Sharp	discussion	Rel-19	NR_SL_relay_multihop-Core

R2-2410828	Initial considerations on service continuity	Samsung	discussion	Rel-19	NR_SL_relay_multihop-Core

## 8.14	Additional topological enhancements

(NR_WAB_5GFemto; leading WG: RAN3; REL-19; WID RP-242395)

Time budget: 0 TU

Tdoc Limitation: 0 tdocs

Work on this WI will only be triggered by LS from RAN3 so work on this WI is not expected to start RAN2#127bis or RAN2#128.

No contributions expected for this meeting

R2-2409516	Reply LS on Support of UE move between CAG cell of 5G Femto and CSG cell (R3-244830; contact: Ericsson)	RAN3	LS in	Rel-19	FS_NR_WAB_5GFemto	To:SA2	Cc:RAN2

R2-2409528	Reply LS on Clarification regarding definition of 5G NR femto ownership (S2-2411241; contact: LGE)	SA2	LS in	Rel-19	FS_5G_Femto_Sec, 5G_Femto	To:SA3	Cc:RAN2, RAN3

## 8.15	NavIC L1 SPS A-GNSS support

(LCS_NAVIC_L1_SPS_NR_LTE-Core; leading WG: RAN2; REL-19; WID RP-242414)

Time budget: 0.5 TU

Tdoc Limitation: 1 tdoc

R2-2409723	Discussion on Introduction of NavIC L1 SPS support to A-GNSS positioning 	Reliance Jio, CEWiT	discussion	Rel-19	LCS_NAVIC_L1_SPS_NR_LTE-Core

Noted

R2-2410243	A-GNSS support for NavIC L1 SPS	NEC	discussion	Rel-19	LCS_NAVIC_L1_SPS_NR_LTE-Core

Noted

Proposal 1: The new Klobuchar Model parameter IE and NeQuick-N Model parameter IE is introduced for ionospheric model.

Proposal 2: For GNSS-navigationModel IE, the following information is introduced to support NavIC L1 band SPS:

-	the NavIC L1 health indication should be introduced

-	the iod of NavIC L1 SPS is indicated by 11 bits of toec (seconds, scale factor 300) in reference [2]

-	a new NavIC clock Model is introduced

-	a new NavIC orbit Model is introduced

Proposal 3: For almanac model, extend the value of some parameters/fields in AlmanacNavIC-AlmanacSet IE, e.g., navic-AlmE, navic-AlmOMEGADOT, navic-Almf0, to support the NavIC L1 band SPS.

Proposal 4: Reuse the UTC-ModelSet2 for NavIC L1 band SPS.

Proposal 5: For NavIC L1 band SPS, the transmission timing of the navigation message provided by the ITOWC and TOIC corresponding to the given set of grid ionospheric parameters. The range of the ITOWC is from 0 to 83 and the range of TOIC is from 0 to 399.

Discussion:

CEWiT indicate that the CR extends the Klobuchar model rather than introducing a new one, introduces a new almanac model rather than extending fields.

Reliance Jio indicate that the extensions from P5 are in the CR.

R2-2409724	Introduction of NavIC L1 SPS A-GNSS in LTE Stage 2 specification	Reliance Jio, CEWiT, Ericsson, Huawei	CR	Rel-19	36.305	18.0.0	0120	-	B	LCS_NAVIC_L1_SPS_NR_LTE-Core

Agreed in principle

R2-2409725	Introduction of NavIC L1 SPS A-GNSS in NR Stage 2 specification	Reliance Jio, CEWiT, Ericsson, Huawei	CR	Rel-19	38.305	18.3.0	0179	-	B	LCS_NAVIC_L1_SPS_NR_LTE-Core

Agreed in principle

R2-2409726	Introduction of NavIC L1 SPS A-GNSS in LPP	Reliance Jio, ISRO, CEWiT, MediaTek, Ericsson	CR	Rel-19	37.355	18.3.0	0532	-	B	LCS_NAVIC_L1_SPS_NR_LTE-Core

[Post128][403][POS] NavIC L1 stage 3 CR check (Reliance Jio)

Scope: Check the CR in R2-2409726 and update if necessary.

Intended outcome: Agreeable CR

Deadline: Long

R2-2409573	Introduction of NavIC in A-GNSS positioning	ZTE Corporation	discussion	Rel-19	LCS_NAVIC_L1_SPS_NR_LTE-Core

R2-2410161	Discussion on the support of NavIC L1 SPS	Huawei, HiSilicon	discussion	Rel-19	LCS_NAVIC_L1_SPS_NR_LTE-Core

## 8.16	BDS B2b in A-GNSS

(BDS_B2b; leading WG: RAN2; REL-19; WID RP-242413)

Time budget: 0.25 TU

Tdoc Limitation: 1 tdoc

R2-2410160	Discussion on the remaining issues for BDS B2b	Huawei, HiSilicon	discussion	Rel-19	LCS_BDS_B2b_LTE_NR

Proposal1: For different parameters in the navigation message defined in clause 6 and 7 of ICD, support them by the following:

	Ephemeris parameters: Reuse the existing IE NavModel-BDS-KeplerianSet2 but the field bdsIODE is ignored

	BDT-UTC Time Offset Parameters: reuse the IE UTC-ModelSet2. Clarify in the field description that on the 3 bits from LSB are used. Legacy BDS may also have the same issue.

	Satellite Health Status: reuse the existing IE svHealth. But clarify that bit 1 and bit 2 are used for B2b signal

	No change is needed for all the parameters related to integrity in the nav message.

Discussion:

CATT understand on bullet 2 that the ICD does have 3 bits while LPP has 4 bits, but they do not think it is an issue, because the UE will just interpret the value in the 4 bits; the only possible issue they see is LMF implementation on how to fill the 4 bits.  Chair understood the issue was whether the value was in the LSBs or MSBs.

CATT think it is clear the high (first) bit would be 0.  Huawei think we should document the difference from the ICD and say the MSB is ignored.

Huawei indicate on bullet 3 that 1 bit is enough for the current spec, but we cannot be sure that there will be no future extension, so it makes more sense to have 2 bits for compatibility.  CATT think we do not need to reserve bits in LPP before they are used in the ICD.

Qualcomm tend to agree with CATT that we have not historically copied all the reserved bits into PP.

Agreements:

In UTC-ModelSet2, clarify in the field description that only the 3 bits from LSB are used.

Do not introduce the reserved bit for svHealth for the B2b signal.

R2-2409627	Introduction of B2b signal in BDS system in A-GNSS	CATT, CAICT, Ericsson, Huawei, HiSilicon	draftCR	Rel-19	37.355	18.3.0	B	LCS_BDS_B2b_LTE_NR-Core

Discussion:

Huawei think we could use a post-meeting discussion to check the details.

[Post128][402][POS] BDS B2b stage 3 CR check (CATT)

Scope: Check and update the CR in R2-2409627.

Intended outcome: Agreeable CR

Deadline: Long

R2-2410158	Introduction of BDS B2b in A-GNSS for TS 36305	Huawei, HiSilicon, CAICT, CATT, Ericsson	CR	Rel-19	36.305	18.0.0	0121	-	B	LCS_BDS_B2b_LTE_NR

Agreed in principle

R2-2410159	Introduction of BDS B2b in A-GNSS for TS 38305	Huawei, HiSilicon, CAICT, CATT, Ericsson	CR	Rel-19	38.305	18.3.0	0180	-	B	LCS_BDS_B2b_LTE_NR

Agreed in principle

R2-2409574	Introduction of BDS B2b in A-GNSS positioning	ZTE Corporation	discussion	Rel-19	LCS_BDS_B2b_LTE_NR-Core

## 8.17	IoT-NTN TDD mode

(IoT_NTN_TDD; leading WG: RAN1; REL-19; WID RP-242415)

Time budget: 0.5 TU

Tdoc Limitation: 0 tdoc

No contributions are expected for this meeting. The agenda is open only for possible discussions based on urgent LSs, if any.

R2-2409694	Work plan for WID: introduction of IoT-NTN TDD mode	Iridium Satellite LLC	Work Plan	Rel-19	Late

Postponed

## 8.18	TEI19

Time budget: 1 TU

Tdoc Limitation: 1 tdoc

Companies are encouraged to submit co-sourced contributions, which will have priority for discussion in RAN2#128.  Tdoc limit applies to all contributions and primary co-sourcing company (if co-sourced).

Including incoming LS from CT1 C1-245500.   No input expected in this meeting.

R2-2409504	LS on the potential impact on RAN2 specifications due to supporting ProSe in NPN (C1-245500; contact: Nokia)	CT1	LS in	Rel-19	TEI19_ProSe_NPN	To:RAN2	Cc:SA2

Noted

R2-2409985	Support of ProSe in NPN (CT1 LS R2-2409504/C1-245500)	Nokia	discussion	Rel-19	TEI19

Noted

R2-2410017	Discussion on support of NPN for ProSe	ZTE Corporation, Sanechips	discussion	Rel-19	TEI19_ProSe_NPN

Noted

Discussion

-	Chair indicates that this is a cross-TSG TEI19.  Nokia and ZTE explain that the RAN changes are only wording modifications and wouldn’t make sense to have a WI for this.

R2-2409931	Enhancement for resolving throughput degradation during handover	China Unicom, Huawei, HiSilicon, Sony, Turkcell, NTT Docomo, Meta	discussion

Observation 1: Channel State Information (CSI) of the target cannot easily be reported as soon as the necessary measurements are performed, which delays setting an efficient MCS on the target side, resulting in a DL data rate drop after cell switch, which will affect real-time services.

Observation 2: Due to the improper setting of MCS on the target side, all handover techniques will result in throughput degradation.

Proposal 1: RAN2 specify a general solution to resolve the throughput degradation issue that exists in all handovers.

Proposal 2: UE indicating that the CSI measurement results are ready via a MAC CE can be a possible solution for resolving the throughput degradation issue.

-	Qualcomm thinks that for LTM they will do it before the handover and Huawei is proposing it after the HO. Qualcomm ask if we plan to keep RAN4 requirements or relax them.

-	LG asks if this can handled by the network just sending CSI request repeatedly. Huawei thinks would waste resources.

-	Apple thinks that the problem statement can be resolved by what LTM is doing. The discussion is happening in RAN1 and they have discussed that alternative 3 may not be feasible. Huawei explains that it doesn’t cover legacy handovers.

-	Ericsson doesn’t think this is a problem and the network can just be conservate in MCS setting, but it can be useful for initial CA.    We can agree with the intention but first check what LTM is doing and whether we need to do anything.

-	Mediatek agrees with this, but it should be part of mobility objectives.

-	Samsung thinks that do this we would need to add further neighbor measurements requirements and this wouldn’t be a TEI19.

-	ZTE agrees with intention.

Wait for further progress in LTM.  This doesn’t imply that RAN1 LTM should consider this case. Understand the level of impacts and whether it could be TEI19 topic.

Noted

R2-2410542	Quality Indication in Msg3 for SDT	Ericsson	discussion	Rel-19	TEI19

Observation 1	MT-SDT operation over initial BWP restricts DL MT-SDT transmissions to the bandwidth of CORESET 0 in RRC_INACTIVE state.

Observation 2	MT-SDT BWP restriction with no DL quality feedback results in smaller, more conservative TB allocations in the downlink at low MCS values. For larger data volumes, smaller TBS translate to a higher number of subsequent transmissions.

Proposal 1	RAN2 addresses the BWP restriction in MT-SDT by introducing a Channel Quality Indication (CQI) in Msg3 or MsgA.

Option 1: Remove the CORESET 0 restriction for DL BWP for SDT from/after first DL MT-SDT data

Option 2: Introduce Channel Quality Indication (CQI) in Message 3 or MsgA

-	ZTE has sympathy on the problem statement but not sure about the solution, we should just remove the restriction.   The CQI solution is not addressing the problem identified.

-	Qualcomm thinks that removing the BWP restriction is not a good choice.   If we don’t reports CQI in msg3 it will impact RAN4 as well.

-	LG thinks that the BWP restriction is the root cause and we should remove it. Nokia agrees.

-	Sony explains that a similar issues was identified in MBS and we agreed to expand coreset 0, introduce CFR.  We can reuse that solution.

RAN2 will address this issue.  FFS the exact solution

Noted

R2-2410548	Terminating Non-MPS Subscribed UE Handling	Peraton Labs	discussion	Rel-18	38.300	TEI19

Proposal 1: The gNB remembers the priority nature of the page for a non-MPS subscribed UE and treats the page response with priority.

Proposal 2: Discuss and agree to the attached CR.

-	LG agrees with observations.  Asks if the solution in proposal 1 would work for all the cases. Peraton labs explains that the requests includes the UE ID so the gNB should be aware.

-	ZTE thinks that there are cases where only partial UE ID is provided

-	NEC thinks that this sounds more like a RAN3 discussion.   Ericsson also agrees this is a more of RAN3 issue.

-	Ericsson thinks that there are ways of solving this in RAN2, similar to what we did in MUSIM. In paging the network tells the UE the purposes of paging (e.g. MPS purposes) and when it comes back it includes the MPS indication. Peraton labs indicates that this is not an acceptable solution as they don’t want to indicate over the air this indication.   Nokia also agrees not the best idea to expose the data. Ericsson thinks that we expose this in UL and why it is not ok to expose the DL.

-	Ericsson thinks that we can’t mandate the gNB to do something as there may be other implementation dependent mechanisms to resolve this.

Discuss offline to understand whether this should be discussed in RAN2/RAN3.

Come back next meeting

Noted

R2-2410613	PDCP SN Gap reporting at mobility	Nokia	discussion	Rel-19

Proposal 1:	As part of PDCP entity re-establishment, for AM DRBs configured by upper layers to send a PDCP SN gap report in the uplink, the transmitting PDCP entity shall re-transmit any previously (prior to the PDCP entity re-establishment) transmitted PDCP SN Gap report(s) for which the successful delivery has not been confirmed by lower layers.

Proposal 2:	RAN2 adopt the Text proposal in Annex implementing Proposal 1.

-	LG reminds everything that SN gap reporting saves up to 2 reordering timer, and this would take longer to set up so there is no point to do this.

-	Nokia asks why is HO case different than normal operation.  LG explains it is because it takes time to do the HO.

-	Futurewei asks if this is a retransmission of the previous copy. Nokia thinks if more are discarged it will reflect the latest status.

-	Ericsson has same understanding as LG.

Noted

R2-2410655	Discussion on the issue of ANR reporting of HSDN cells [ANR_HSDN]	"Huawei, HiSilicon, CMCC, China Unicom, China Telecom, CATT, 	NTT DoCoMo"	discussion	Rel-19	TEI19

Proposal 1: It is proposed RAN2 to discuss the solutions for solving the issue of ANR reporting of HSDN cells:

-	allow the UE to report HSDN indication for the neighbouring cell via CGI reporting procedure. The neighbouring cells can be intra-frequency or inter-frequency cells

-	for this solution, introduce a new UE capability bit (optional with signalling)

Proposal 2: This feature is for both NR and LTE specifications.

-	Ericsson is not sure the issue is valid, and even if it is valid there are network solution.   Anyways ANR is led by RAN3 so we should check with RAN3. Huawei thinks that ANR is specified in RAN2. There are problems with network solution, if the Xn interface is not supported.

-	CMCC explains that ANR ais deployed in the network and this is a problem we should addresss.

-	ZTE thinks that we have both a UE based solution and NW based solution.  NW based solution would be deployed faster.

-	Samsung thinks in most cases the network knows whether the neighboring cell supports HSDN, and even if doesn’t know there is no problem with HO procedure. It is better to have a Network based solution if we want solve the issue.

-	Samsung asks how the serving cell creates the neighbor cell list without exchanging information with neighbour cells.

-	Qualcomm thinks that we have this information in the UE so it come for free to support it.

Continue discussion until next meeting

Noted

R2-2410674	Discussion on UE aggregation enhancement	CMCC, ZTE, Media Tek Inc., vivo, CATT	discussion	Rel-19	TEI19

Proposal 1: RAN2 is suggested to support multiple indirect paths for multi-path relay via N3C.

-	ZTE sees benefits but we have limitation for UE aggregation in Rel-18 that we would like to remove.

-	Huawei supports this.

-	Qualcomm asks what is the maximum additional relay you want to have and have we don’t a full specification analysis as there will be impact to PDCP as well.

-	Apple thinks that there a lot of impacts not captured in RRC and there are MAC impact and there is a limit and we have to design new MAC CE.

-	Samsung is concerned that there is impact to RAN3 specifications. CMCC explains that we limit the scenario to only intra gNB. Samsung thinks may be impact to F1 interface as well when considering CU and DU split.

-	Ericsson thinks that we should first clarify the scope and then do an analysis

More detailed scope and spec analysis expected for next meeting

Noted

R2-2410675	Corrections to TS 38.331 on SL Relay enhancement	CMCC, Media Tek Inc., CATT	CR	Rel-19	38.331	18.3.0	5180	-	B	TEI19

Not treated

R2-2410676	Corrections to TS 38.300 on SL Relay enhancement	CMCC, Media Tek Inc., CATT	CR	Rel-19	38.300	18.3.0	0940	-	B	TEI19

Not treated

To be treated in NTN break out session

R2-2410793	Inclusion of the NB-IoT satellite information in E-UTRAN	Google, Samsung, MediaTek Inc.	discussion	Rel-19

Revised in R2-2411085

R2-2411085	Inclusion of the NB-IoT satellite information in E-UTRAN	Google, Samsung, MediaTek Inc., Inmarsat, Viasat, Thales, Nordic	discussion	Rel-19

Proposal 1	Enable the inclusion of the satellite assistance information for NB-IoT NTN in SIB33, and create the association between the satellite assistance information in SIB33 and the NB-IoT frequency information in SIB27 (TP attached).

-	QC thinks there should be no RRM impacts. Google agrees

-	CMCC thinks we can also avoid impacts to 36.304, i.e. to cell reselection

We can work on the provision of the NB-IoT satellite information in E-UTRAN as part of TEI19 work. RAN2 assumes that this will not impact other WGs (e.g. no RRM impacts) and will only require changes to 36.331, 36.306 (to indicate this is an optional feature without capability) and Stage 2

We enable the inclusion of the satellite assistance information for NB-IoT NTN in SIB33, and create the association between the satellite assistance information in SIB33 and the NB-IoT frequency information in SIB27

TP for SIB27 in R2-2411085 is considered as a baseline

Continue the discussion in the next meeting, e.g. on the impacts to SIB33

Agreements:

1.	We can work on the provision of the NB-IoT satellite information in E-UTRAN as part of TEI19 work. RAN2 assumes that this will not impact other WGs (e.g. no RRM impacts) and will only require changes to 36.331, 36.306 (to indicate this is an optional feature without capability) and Stage 2

2.	We enable the inclusion of the satellite assistance information for NB-IoT NTN in SIB33, and create the association between the satellite assistance information in SIB33 and the NB-IoT frequency information in SIB27

3.	TP for SIB27 in R2-2411085 is considered as a baseline

# 9	Breakout session reports

No documents shall be submitted to this AI or its sub-AIs. It is only for at-meeting-generated contents.

## 9.1	Session on V2X/SL, R19 NES and MOB

R2-2410911	Report from session on V2X/SL, R19 NES and MOB	Vice Chairman (Samsung)	report

LS R2-2411135 is updated and approved in R2-2411219

The report is approved

## 9.2	Session on R18 MIMOevo, R18 MUSIM, and R19 LP-WUS

R2-2410912	Rel-18 MIMO, Rel-19 MIMO, LPWUS, and SBFD	Vice Chairman (CATT)	report

The report is approved

## 9.3	Session on NR NTN and IoT NTN

R2-2410913	Report from session on NR NTN and IoT NTN	Session chair (ZTE)	report

-	Ericsson asks if the LS on RACHless HO was Rel-18 and whether it was a correction.   Nokia explains this is Rel-18 correction for NTN only.

The report is approved

## 9.4	Session on positioning and sidelink relay

R2-2410914	Report from session on positioning and sidelink relay	Session chair (MediaTek)	report

=>	The report is approved

R2-2410985	Correction on spatial relation info in SP SRS activation deactivation MAC CE (R17)	ZTE Corporation, Ericsson	CR	Rel-17	38.321	17.10.0	1977	-	F	NR_pos_enh-Cor

The CR is postponed

R2-2410986	Correction on spatial relation info in SP SRS activation deactivation MAC CE (R18)	ZTE Corporation, Ericsson	CR	Rel-18	38.321	18.3.0	1978	-	A	NR_pos_enh-Core

The CR is postponed

## 9.5	Session on R18 MBS, R18 QoE and R19 XR

R2-2410915	Report from session on R18 MBS, R18 QoE and R19 XR	Session chair (Huawei)	report

The report is approved

- The following LS needs online discussion and approval during the RAN2 Main session:

R2-2411002 Reply LS on Application-Layer FEC Awareness at RAN 	RAN2	LS out	Rel-19	NR_XR_Ph3-Core, XRM_Ph2	To:SA2	Cc:SA4

-	Meta indicates that the LS asks about feasibility but we respond about usefulness. Qualcomm explains that we didn’t discuss feasibility.  Mediatek indicates that there was no consensus. Huawei, Lenovo, Nokia, Apple, Xiaomi, LG, Vivo, Samsung, Sharp, Mediatek thinks that the LS truly reflects the agreement of RAN2 and we didn’t conclude on feasibility.

-	Vivo indicates that from SA2 colleagues this is enough for SA2 colleagues.

The LS is approved

- The following agreement was flagged and proposed to be removed by Ericsson using [AT128][500] thread, after the online discussions on R19 XR already took place:

Rate indication from gNB to the UE on a per QoS flow level is supported. FFS the details, e.g. if: 1) flows are indicated by MAC CE or 2) by RRC while MAC CE is per DRB.

-	Ericsson indicates that after the agreement there were some impactes identified to other WGs.   So suggest to send an LS to SA2/SA4 and RAN3, provide the agreement and ask if there are any issues. Nokia agrees as usually if we find inter-WG impacts we need to ask.

-	Meta doesn’t want to send the LS.  We should keep agreement and discuss next meeting.

-	LG is ok to send the LS but not the rever the agreement.   Samsung doesn’t see a reason to delay the LS if companies would like an LS.

-	Lenovo also agrees to send the LS.

-	Qualcomm as rapporteur thinks that if we need to send nand LS it should be now as they are concerned whether it can be completed on time.

-	Meta is objecting

-	Ericsson thinks that if we don’t send an LS then we have to revert the agreement.

-	ZTE thinks that we can’t stop ourselves from working other WGs. Mediatek thinks that we are trying to solve a difficult topic so we can just ask for feedback.

-	Session chair would like to keep the agreement and this is a compromise and is happy with the conclusion. We can continue working with this agreement as an assumption and only if we get reply with concerns.

Keep the agreement as is.  Send an LS to SA4, RAN3 ccSA2 where we simply put the agreement we reached and ask if they have any feedback (if any).

[AT128][30][XR] LS to SA4  (Ericsson/Meta)

Intended outcome: agree to LS

Deadline:  Friday

R2-2411218	LS on XR UL bit rate control	RAN2	LS out	Rel-19	NR_XR_Ph3-Core	To:SA4, RAN3;	Cc:SA2

=> Approved

## 9.6	Session on maintenance and SON/MDT

R2-2410916	Report from session on maintenance and SON/MDT	Session chair (Ericsson)	report

The report is approved

# Closing of the meeting

The meeting was closed by the chair at 12:38 EST on Friday, 22nd of November.

# Annex A:	List of participants

RAN2#128 participants list is attached to this report.

Total number of face-to-face participants: 432

# Annex B:	List of Tdocs

The list of tdocs from RAN2#128 is attached to this report.

Total of 1693 tdoc numbers were allocated of which 1646 tdocs were made available.

# Annex C:	Incoming liaison statements

| TDoc | Title | Source | Status | Rel | Related WIs | To | Cc | Original LS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| R2-2409504 | LS on the potential impact on RAN2 specifications due to supporting ProSe in NPN (C1-245500; contact: Nokia) | CT1 | noted | Rel-19 | TEI19_ProSe_NPN | RAN2 | SA2 | C1-245500 |
| R2-2409505 | LS on FR2-NTN inclusion to specifications (R1-2407406; contact: vivo) | RAN1 | noted | Rel-18 | NR_NTN_enh-Core | RAN2, RAN4 |  | R1-2407406 |
| R2-2409506 | LS on higher layer parameters for DCI format 2_3 (R1-2407534; contact: CATT) | RAN1 | noted | Rel-16 | NR_newRAT-Core, TEI16 | RAN2 |  | R1-2407534 |
| R2-2409507 | LS on Rel-18 RAN1 UE features list for NR after RAN1#118bis (R1-2409045; contact: NTT DOCOMO, AT&T) | RAN1 | noted | Rel-18 | TEI18, NR_MIMO_evo_DL_UL, NR_pos_enh2, Netw_Energy_NR, NR_netcon_repeater, NR_NTN_enh, NR_Mob_enh2, NR_SL_enh2, NR_redcap_enh, NR_MC_enh, NR_XR_enh, NR_FR1_lessthan_5MHz_BW, NR_DSS_enh, NR_BWP_wor, NR_cov_enh2 | RAN2 | RAN4 | R1-2409045 |
| R2-2409508 | Reply LS on CSI-RS and SRS for spatial relation (R1-2409097; contact: ZTE) | RAN1 | noted | Rel-18 | NR_pos_enh2-Core | RAN2 |  | R1-2409097 |
| R2-2409509 | Reply LS on IUC Scheme-2 and Random Selection (R1-2409174; contact: OPPO) | RAN1 | noted | Rel-17 | NR_SL_enh-Core | RAN2 |  | R1-2409174 |
| R2-2409510 | LS on TDD UL/DL Configuration for Two TA (R1-2409179; contact: Ericsson) | RAN1 | noted | Rel-18 | NR_MIMO_evo_DL_UL-Core | RAN2 |  | R1-2409179 |
| R2-2409511 | Reply LS to RAN2 on data block sizes for Ambient IoT (R1-2409250; contact: MediaTek) | RAN1 | withdrawn | Rel-19 | FS_AmbientIoT, FS_Ambient_IoT_solutions | RAN2 | SA2 | R1-2409250 |
| R2-2409512 | Reply LS on common TA in a regenerative payload scenario (R1-2409258; contact: CMCC) | RAN1 | noted | Rel-19 | NR_NTN_Ph3-Core | RAN2 | RAN4 | R1-2409258 |
| R2-2409513 | LS on the time period for R18 preamble repetition (R1-2409262; contct: Huawei) | RAN1 | noted | Rel-18 | NR_cov_enh2-Core | RAN2 |  | R1-2409262 |
| R2-2409514 | LS on the support of semi-persistent CSI-RS resource for LTM candidate cells (R1-2409283; contact: Fujitsu) | RAN1 | noted | Rel-19 | NR_Mob_Ph4-Core | RAN3 | RAN2 | R1-2409283 |
| R2-2409515 | LS on waveform determination for PUSCH scheduled by fallbackRAR UL grant and MsgA PUSCH (R1-2409301; contact: CATT) | RAN1 | noted | Rel-17 | NR_redcap-Core, NR_SmallData_INACTIVE-Core, NR_cov_enh-Core, NR_slice-Core | RAN2 |  | R1-2409301 |
| R2-2409516 | Reply LS on Support of UE move between CAG cell of 5G Femto and CSG cell (R3-244830; contact: Ericsson) | RAN3 | available | Rel-19 | FS_NR_WAB_5GFemto | SA2 | RAN2 | R3-244830 |
| R2-2409517 | Reply LS on multi-modality awareness at RAN (R3-245682; contact: Nokia) | RAN3 | noted | Rel-19 | NR_XR_Ph3-Core, XRM_Ph2 | SA2 | RAN2, SA4, RAN | R3-245682 |
| R2-2409518 | Reply LS on UE Location Information for NB-IoT NTN (R3-245819; contact: Nokia) | RAN3 | noted | Rel-18 | IoT_NTN_enh | SA2 | RAN2, CT1, SA1, SA3-LI | R3-245819 |
| R2-2409519 | LS on Supporting MBS broadcast service for NR NTN (R3-245844; contact: Xiaomi) | RAN3 | noted | Rel-19 | NR_NTN_Ph3-Core | RAN2, SA2, CT4 |  | R3-245844 |
| R2-2409520 | LS on SSB adaptation (R4-2416911; contact: Apple) | RAN4 | noted | Rel-19 | Netw_Energy_NR_enh-Core | RAN2 | RAN1 | R4-2416911 |
| R2-2409521 | LS on SSB relation in On-demand SSB and SSB adaptation on Scell (R4-2416913; contact: Ericsson) | RAN4 | noted | Rel-19 | Netw_Energy_NR_enh-Core | RAN1 | RAN2 | R4-2416913 |
| R2-2409522 | Reply LS on common TA in a regenerative payload scenario (R4-2416920; contact: CMCC) | RAN4 | noted | Rel-19 | NR_NTN_Ph3-Core | RAN2 | RAN1 | R4-2416920 |
| R2-2409523 | LS on Rel-19 NR channel BW less than 5MHz for FR1 (R4-2417119; contact: Intel) | RAN4 | noted | Rel-19 | NR_FR1_lessthan_5MHz_BW_Ph2 | RAN2 |  | R4-2417119 |
| R2-2409524 | Reply LS on Support of Regenerative-based Satellite Access (S2-2410918; contact: Qualcomm) | SA2 | noted | Rel-19 | NR_NTN_Ph3-Core, 5GSAT_Ph3_ARCH | RAN3, RAN | RAN2 | S2-2410918 |
| R2-2409525 | Reply LS on Application-Layer FEC Awareness at RAN (S2-2410999; contact: Qualcomm) | SA2 | noted | Rel-19 | XRM_Ph2 | RAN2 | SA4 | S2-2410999 |
| R2-2409526 | LS on security aspects of Ambient IoT (S2-2411049; contact: OPPO) | SA2 | noted | Rel-19 | FS_AmbientIoT | SA3 | RAN2 | S2-2411049 |
| R2-2409527 | Reply LS on AIML data collection (S2-2411191; contact: InterDigital) | SA2 | noted | Rel-19 | NR_AIML_air | RAN2, RAN3 | SA, RAN, SA3, SA5 | S2-2411191 |
| R2-2409528 | Reply LS on Clarification regarding definition of 5G NR femto ownership (S2-2411241; contact: LGE) | SA2 | available | Rel-19 | FS_5G_Femto_Sec, 5G_Femto | SA3 | RAN2, RAN3 | S2-2411241 |
| R2-2409529 | Reply LS on FS_5GSAT_Ph3_ARCH conclusions (S2-2411250; contact: Sateliot) | SA2 | noted | Rel-19 | 5GSAT_Ph3_ARCH | SA3 | RAN2, SA3-LI | S2-2411250 |
| R2-2409530 | Reply LS on FS_5GSAT_Ph3_ARCH conclusions (s3i240703; contact: Tridea Works) | SA3-LI | noted | Rel-18 | 5GSAT_Ph3_ARCH | SA2 | RAN2, SA3 | s3i240703 |
| R2-2409531 | LS on AI/ML Model transfer/delivery to UE (S5-246292; contact: NEC) | SA5 | noted | Rel-19 | FS_AIML_MGT_Ph2 | RAN2, RAN1 | RAN, SA, RAN3 | S5-246292 |
| R2-2409532 | Reply LS on AIML Data Collection (S5-246299; contact Nokia) | SA5 | noted | Rel-18 | FS_AIML_MGT_Ph2 | RAN2 | RAN, RAN1, RAN3, SA, SA2, SA3 | S5-246299 |
| R2-2409533 | LS for PDU Set Information Marking Support without QoS parameters (S2-2411253; contact: vivo) | SA2 | noted | Rel-19 | XRM_Ph2 | RAN2, RAN3 |  | S2-2411253 |
| R2-2409534 | Reply LS on security handling for inter-CU LTM in non-DC cases (S3-244316; contact: Samsung) | SA3 | noted | Rel-19 | NR_Mob_Ph4-Core | RAN2 | RAN3 | S3-244316 |
| R2-2409535 | Reply LS on security key update of inter-CU SCG LTM (S3-244317; contact: Xiaomi) | SA3 | noted | Rel-19 | NR_Mob_Ph4-Core | RAN2 | RAN3 | S3-244317 |
| R2-2411077 | Reply LS on AIML data collection (S3-245138; contact: Samsung) | SA3 | not treated | Rel-19 | NR_AIML_air | RAN | RAN2, SA, SA1, SA2, SA5 | S3-245138 |
| R2-2411078 | Reply LS on security aspects of Ambient IoT (S3-245139; contact: OPPO) | SA3 | noted | Rel-19 | FS_AmbientIoT | SA2 | RAN2 | S3-245139 |
| R2-2411177 | Reply LS on AIML data collection (R3-247801; contact: ZTE) | RAN3 | not treated | Rel-19 | NR_AIML_air, NR_AIML_air-Core | SA2 | RAN2, SA, RAN, SA3, SA5 | R3-247801 |
| R2-2411201 | LS Reply on multi-modality awareness (S4-242223; contact: Huawei) | SA4 | available | Rel-19 | NR_XR_Ph3-Core | RAN2, RAN3, SA2 |  | S4-242223 |

36 incoming LS, of which 29 LS were noted. The remaining non-treated LSin will be treated in RAN2#129.

# Annex D:	Outgoing liaison statements

| TDoc | Title | Rel | Related WIs | To | Cc |
| --- | --- | --- | --- | --- | --- |
| R2-2409711 | Reply LS on RAN3 vs RAN2 Basketball Match | Rel-19 | TEI19 | RAN3 |  |
| R2-2410957 | Reply LS to R2-2407921/ R1-2407559 | Rel-19 | NR_LPWUS-Core | RAN1, RAN4 |  |
| R2-2410978 | LS on soft satellite switch with resync | Rel-19 | NR_NTN_enh-Core | RAN4, RAN1 |  |
| R2-2411001 | Reply LS to SA2 on PDU set | Rel-19 | NR_XR_Ph3-Core | SA2 |  |
| R2-2411002 | Reply LS on Application-Layer FEC Awareness at RAN | Rel-19 | NR_XR_Ph3-Core | SA2 | SA4 |
| R2-2411031 | LS response on waveform determination for PUSCH scheduled by fallbackRAR UL grant and MsgA PUSCH | Rel-17 | NR_redcap-Core, NR_SmallData_INACTIVE-Core, NR_cov_enh-Core, NR_slice-Core | RAN1 |  |
| R2-2411041 | LS on emergency services and eDRX | Rel-17 | NR_newRAT-Core, NR_redcap-Core | SA2, CT1 | RAN3 |
| R2-2411114 | Reply LS to SA5 on AIML data collection | Rel-19 | FS_AIML_MGT_Ph2 | SA5 | RAN, RAN1, RAN3, SA, SA2, SA3 |
| R2-2411134 | RAN2 agreements on Inter-CU LTM and Conditional LTM | Rel-19 | NR_Mob_Ph4-Core | RAN1, RAN3, RAN4 |  |
| R2-2411136 | Reply LS on security handling for inter-CU LTM in non-DC cases | Rel-19 | NR_Mob_Ph4-Core | SA3 |  |
| R2-2411152 | Reply LS to SA2 on AIML data collection | Rel-19 | NR_AIML_air-Core | SA2 | RAN, RAN1, RAN3, SA, SA5, SA3 |
| R2-2411168 | Reply LS on the time period for R18 preamble repetition | Rel-18 | NR_cov_enh2-Core | RAN1 |  |
| R2-2411184 | LS to SA2 on relay discovery announcement | Rel-19 | NR_SL_relay_multihop-Core | SA2 |  |
| R2-2411191 | LS on inter-gNB RACH-less HO in NTN | Rel-18 | NR_NTN_enh-Core | RAN3 | RAN1 |
| R2-2411194 | LS on PWS support in RRC_CONNECTED for NB-IoT NTN | Rel-19 | IoT_NTN_Ph3-Core | RAN1 |  |
| R2-2411195 | LS on UE Capabilities for FR2-NTN | Rel-18 | NR_NTN_enh-Core | RAN4 |  |
| R2-2411196 | Reply on Supporting MBS broadcast service for NR NTN | Rel-19 | NR_NTN_Ph3-Core | RAN3, SA2 | CT4 |
| R2-2411197 | LS on Satellite IDs for store-and-forward operation | Rel-19 | IoT_NTN_Ph3-Core | SA2 |  |
| R2-2411218 | LS on XR UL bit rate control | Rel-19 | NR_XR_Ph3-Core | SA4, RAN3 | SA2 |
| R2-2411219 | LS on LTM UE capabilities for cross-band operation | Rel-18 | NR_Mob_enh2-Core | RAN1 |  |
| R2-2411220 | LS on UL 8Tx | Rel-18 | NR_MIMO_evo_DL_UL-Core | RAN1 |  |
| R2-2411244 | LS on differentiation of sDCI mTRP, mDCI mTRP and sTRP | Rel-18 | NR_MIMO_evo_DL_UL-Core | RAN1 |  |
| R2-2411251 | LS on co-existence of SL-CA and SL PRS transmission/reception for Dedicated SL-PRS resource pool | Rel-18 | NR_pos_enh2-Core | RAN1 |  |
| R2-2411263 | LS on RAN2 outcome of Ambient IoT study | Rel-19 | FS_Ambient_IoT_solutions | RAN1 | RAN3, SA2, SA3 |

24 outgoing LS.

# Annex E:	List of agreed CRs

| TDoc | Title | Source | Rel | Spec | Related WIs | CR | Rev | Cat |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| R2-2409543 | Correction on UE Location Information for IoT-NTN | vivo, Ericsson, Nokia, Nokia Shanghai Bell | Rel-18 | 36.300 | IoT_NTN_enh-Core | 1410 |  | F |
| R2-2409545 | Correction on MT-SDT Capability for NR-NTN Case | vivo, ZTE Corporation, Sanechips, Huawei, HiSilicon, Ericsson | Rel-18 | 38.306 | NR_MT_SDT-Core, NR_NTN_solutions-Core | 1201 |  | F |
| R2-2409549 | Co-configuration of random/partial-sensing resource selection and Co-Ex | OPPO | Rel-18 | 38.331 | NR_SL_enh2 | 4976 | 1 | F |
| R2-2409600 | Correction on location based measurements in NR NTN | CATT, Nokia, Nokia Shanghai Bell | Rel-17 | 38.300 | NR_NTN_solutions-Core | 0909 | 1 | F |
| R2-2409601 | Correction on location based measurements in NR NTN | CATT, Nokia, Nokia Shanghai Bell | Rel-18 | 38.300 | NR_NTN_solutions-Core | 0910 | 1 | A |
| R2-2409642 | Correction on IE SRS-CarrierSwitching | CATT, Vivo, China Telecom, China Unicom, Nokia, ZTE Corporation, CMCC, Ericsson | Rel-15 | 38.331 | NR_newRAT-Core | 4893 | 3 | F |
| R2-2409643 | Correction on IE SRS-CarrierSwitching | CATT, Vivo, China Telecom, China Unicom, Nokia, ZTE Corporation, Ericsson | Rel-16 | 38.331 | NR_newRAT-Core | 4894 | 3 | A |
| R2-2409644 | Correction on IE SRS-CarrierSwitching | CATT, Vivo, China Telecom, China Unicom, Nokia, ZTE Corporation, Ericsson | Rel-17 | 38.331 | NR_newRAT-Core | 4895 | 3 | A |
| R2-2409645 | Correction on IE SRS-CarrierSwitching | CATT, Vivo, China Telecom, China Unicom, Nokia, ZTE Corporation, Ericsson | Rel-18 | 38.331 | NR_newRAT-Core | 4896 | 3 | A |
| R2-2409646 | Miscellaneous corrections for Rel-18 SON/MDT | CATT, Samsung, Ericsson | Rel-18 | 38.331 | NR_ENDC_SON_MDT_enh2-Core | 4986 | 2 | F |
| R2-2409682 | RRC correction on NR SL U2U relay operation | Philips International B.V. | Rel-18 | 38.331 | NR_SL_relay_enh-Core | 5048 | 2 | F |
| R2-2409683 | RRC correction on NR sidelink positioning | Philips International B.V., Ericsson | Rel-18 | 38.331 | NR_pos_enh2-Core | 4940 | 2 | F |
| R2-2409698 | Corrections to validity of configured uplink grant for SDT | Samsung, LG Electronics, Sony | Rel-17 | 38.321 | NR_SmallData_INACTIVE-Core | 1956 | 2 | F |
| R2-2409699 | Corrections to validity of configured uplink grant for SDT | Samsung, LG Electronics, Sony | Rel-18 | 38.321 | NR_SmallData_INACTIVE-Core | 1979 |  | A |
| R2-2409735 | Clarification for ul-DataSplitThreshold setting in multi-path relay | OPPO | Rel-18 | 38.331 | NR_SL_relay_enh-Core | 5081 | 1 | F |
| R2-2409743 | Misc RRC corrections for SL enhancements | Huawei, HiSilicon | Rel-17 | 38.331 | NR_SL_enh-Core | 5001 | 1 | F |
| R2-2409744 | Misc RRC corrections for SL enhancements | Huawei, HiSilicon | Rel-18 | 38.331 | NR_SL_enh-Core | 5002 | 1 | A |
| R2-2409757 | Miscellaneous CR for Rel-17 SL relay | Huawei, HiSilicon, Philips International B.V., OPPO | Rel-17 | 38.331 | NR_SL_relay-Core | 5086 | 1 | F |
| R2-2409758 | Miscellaneous CR for Rel-17 SL relay | Huawei, HiSilicon, Philips International B.V., OPPO | Rel-18 | 38.331 | NR_SL_relay-Core | 5087 | 1 | A |
| R2-2409759 | Miscellaneous CR for Rel-18 SL relay enhancement | Huawei, HiSilicon | Rel-18 | 38.331 | NR_SL_relay_enh-Core | 4994 | 2 | F |
| R2-2409760 | Correction to error handling for U2U operation | Huawei, HiSilicon | Rel-18 | 38.351 | NR_SL_relay_enh-Core | 0037 | 2 | F |
| R2-2409804 | Correction on HARQ process | Samsung, Ericsson, Nokia, LG Electronics Inc., Qualcomm Incorporated, Apple, ZTE, CATT | Rel-17 | 38.321 | NR_NTN_solutions-Core, NR_MBS-Core | 1937 | 1 | F |
| R2-2409805 | Correction on HARQ process | Samsung, Ericsson, Nokia, LG Electronics Inc., Qualcomm Incorporated, Apple, ZTE, CATT | Rel-18 | 38.321 | NR_NTN_solutions-Core, NR_MBS-Core | 1938 | 1 | A |
| R2-2409806 | Clarification on MII usage | Samsung, Nokia | Rel-17 | 38.300 | NR_MBS-Core | 0924 | 1 | F |
| R2-2409807 | Clarification on MII usage | Samsung, Nokia | Rel-18 | 38.300 | NR_MBS-Core | 0925 | 1 | A |
| R2-2409808 | Clarification on determining about to start sessions | Samsung, Nokia | Rel-17 | 38.331 | NR_MBS-Core | 5092 | 1 | F |
| R2-2409809 | Clarification on determining about to start sessions | Samsung, Nokia | Rel-18 | 38.331 | NR_MBS-Core | 5093 | 1 | A |
| R2-2409810 | Correction for stored SDUs handling when a t-Reordering expires | Samsung | Rel-18 | 38.323 | NR_XR_enh-Core | 0142 | 1 | F |
| R2-2409811 | Clarifications on DPC field in PHR MAC CE | Samsung | Rel-18 | 38.321 | NR_cov_enh2-Core | 1957 | 1 | F |
| R2-2409813 | Miscellaneous corrections on BAP for IAB | Huawei, HiSilicon (Rapporteur) | Rel-18 | 38.340 | NR_mobile_IAB-Core | 0040 |  | D |
| R2-2409814 | Maximum number of configured CCs | Nokia, Nokia Shanghai Bell | Rel-15 | 38.300 | NR_newRAT-Core | 0926 |  | F |
| R2-2409815 | Maximum number of configured CCs | Nokia, Nokia Shanghai Bell | Rel-16 | 38.300 | NR_newRAT-Core | 0927 |  | A |
| R2-2409816 | Maximum number of configured CCs | Nokia, Nokia Shanghai Bell | Rel-17 | 38.300 | NR_newRAT-Core | 0928 |  | A |
| R2-2409817 | Maximum number of configured CCs | Nokia, Nokia Shanghai Bell | Rel-18 | 38.300 | NR_newRAT-Core | 0929 |  | A |
| R2-2409914 | Correction to unified TCI signalling | MediaTek Inc. | Rel-17 | 38.331 | NR_FeMIMO-Core | 4997 | 1 | F |
| R2-2409915 | Correction to unified TCI signalling | MediaTek Inc. | Rel-18 | 38.331 | NR_FeMIMO-Core | 4998 | 1 | A |
| R2-2409919 | Correction on UE behavior of setting failedPSCellId | NTTDOCOMO, INC. | Rel-17 | 38.331 | NR_ENDC_SON_MDT_enh-Core | 5009 | 1 | F |
| R2-2409920 | Correction on UE behavior of setting failedPSCellId | NTTDOCOMO, INC. | Rel-18 | 38.331 | NR_ENDC_SON_MDT_enh-Core | 5010 | 1 | A |
| R2-2409992 | Mandatory support of Enhanced channel raster by RedCap UE | Ericsson | Rel-17 | 38.306 | NR_redcap-Core | 1157 | 2 | F |
| R2-2410041 | Correction on SPR content determination | Google | Rel-18 | 38.331 | NR_ENDC_SON_MDT_enh2-Core | 5135 |  | F |
| R2-2410043 | Correction on handling successHO-Config during full configuration | Samsung, Ericsson | Rel-17 | 38.331 | NR_ENDC_SON_MDT_enh-Core | 5136 |  | F |
| R2-2410070 | Correction on SL DRX parameters | ZTE Corporation, Sanechips, Huawei, HiSilicon, Ericsson, Nokia, ASUSTeK, Apple, OPPO, LG | Rel-17 | 38.331 | NR_SL_enh-Core | 5139 |  | F |
| R2-2410071 | Correction on SL DRX parameters | ZTE Corporation, Sanechips, Huawei, HiSilicon, Ericsson, Nokia, ASUSTeK, Apple, OPPO, LG | Rel-18 | 38.331 | NR_SL_enh-Core | 5140 |  | A |
| R2-2410072 | Correction on carrier selection for IUC | ZTE Corporation, Sanechips, Ericsson, Apple, Nokia | Rel-18 | 38.321 | NR_SL_enh2-Core | 1984 |  | F |
| R2-2410126 | Correction to random resource selection for sidelink | Ericsson | Rel-17 | 38.321 | NR_SL_enh-Core | 1944 | 2 | F |
| R2-2410127 | Correction to random resource selection for sidelink | Ericsson | Rel-18 | 38.321 | NR_SL_enh-Core | 1945 | 2 | A |
| R2-2410157 | Correction for RA resource information in RA-report | Sharp, Ericsson, Lenovo | Rel-17 | 38.331 | NR_ENDC_SON_MDT_enh-Core | 5144 |  | F |
| R2-2410176 | Correction on prioritization between SR and SL-PRS transmission | ASUSTeK | Rel-18 | 38.321 | NR_pos_enh2 | 1992 |  | F |
| R2-2410203 | Correction to missing NR-DC parameters branches | Ericsson | Rel-16 | 38.331 | NR_newRAT-Core, TEI16 | 5089 | 1 | F |
| R2-2410204 | Correction to missing NR-DC parameters branches | Ericsson | Rel-17 | 38.331 | NR_newRAT-Core, TEI16 | 5090 | 1 | A |
| R2-2410205 | Correction to missing NR-DC parameters branches | Ericsson | Rel-18 | 38.331 | NR_newRAT-Core, TEI16 | 5091 | 1 | A |
| R2-2410221 | Correction to MAC for R17 POS | Huawei, HiSilicon | Rel-18 | 38.321 | NR_pos_enh-Core | 1999 |  | A |
| R2-2410226 | Rapporteur CR for MT-SDT and CG-SDT enhancement [CG-SDTenh] | Huawei, HiSilicon | Rel-18 | 38.321 | TEI18, NR_MT_SDT-Core | 2001 |  | F |
| R2-2410349 | Corrections on capabilities for eRedCap | Huawei, HiSilicon, LG Electronics Inc., Intel Corporation | Rel-18 | 38.306 | NR_redcap_enh-Core | 1166 | 2 | F |
| R2-2410417 | Corrections to NR NTN (R17) | Huawei, HiSilicon, CMCC, Sequans Communications | Rel-17 | 38.331 | NR_NTN_solutions-Core | 5075 | 2 | F |
| R2-2410418 | Corrections to NR NTN (R18) | Huawei, HiSilicon, CMCC, Sequans Communications | Rel-18 | 38.331 | NR_NTN_solutions-Core | 5076 | 2 | A |
| R2-2410422 | Miscellaneous corrections to TS 36.331 for IoT NTN | Huawei, HiSilicon | Rel-18 | 36.331 | IoT_NTN_enh-Core | 5054 | 1 | F |
| R2-2410455 | Correction on use of recommended of IAB-MT beam indication | Ericsson, Samsung | Rel-17 | 38.321 | NR_IAB_enh-Core | 1901 | 3 | F |
| R2-2410456 | Correction on use of recommended of IAB-MT beam indication | Ericsson, Samsung | Rel-18 | 38.321 | NR_IAB_enh-Core | 1902 | 2 | A |
| R2-2410457 | Coexistence of NCR and CPAC with (e)RedCap | Ericsson, ZTE Corporation | Rel-18 | 38.300 | NR_Mob_enh2-Core, NR_netcon_repeater, NR_redcap_enh-Core | 0937 |  | F |
| R2-2410495 | Correction of misplaced else condition of SL Positioning clause | Ericsson | Rel-18 | 38.321 | NR_pos_enh2-Core | 1971 | 1 | F |
| R2-2410528 | Correction on PHR for MIMO STx2P multi-panel scheme | Samsung, LG, Huawei, CATT, Ericsson | Rel-18 | 38.321 | NR_MIMO_evo_DL_UL-Core | 1959 | 1 | F |
| R2-2410532 | Clarification for UE capability on UL traffic information | Huawei, HiSilicon | Rel-18 | 38.306 | NR_XR_enh-Core | 1182 | 1 | F |
| R2-2410533 | Correction for CFRA configuration due to PRACH partitioning | Huawei, HiSilicon, ZTE Corporation, Ericsson, Mediatek | Rel-17 | 38.331 | NR_redcap-Core, NR_slice-Core | 4899 | 2 | F |
| R2-2410611 | Clarification on the definition of eRedCap UEs | Ericsson, Sequans Communications | Rel-18 | 38.306 | NR_redcap_enh-Core | 1197 | 1 | F |
| R2-2410615 | Paging clarification for MBS | SHARP Corporation | Rel-18 | 38.331 | NR_MBS_enh-Core | 5097 | 1 | F |
| R2-2410650 | Correction on IE perRAInfoList for SCGFailureInformation | Huawei, HiSilicon | Rel-17 | 38.331 | NR_ENDC_SON_MDT_enh-Core | 5044 | 2 | F |
| R2-2410651 | Correction on IE perRAInfoList for SCGFailureInformation | Huawei, HiSilicon | Rel-18 | 38.331 | NR_ENDC_SON_MDT_enh-Core | 5045 | 2 | A |
| R2-2410664 | Supporting R17 early implementation of R18 measurement gap enhancements | Nokia, Nokia Shanghai Bell, Ericsson, ZTE Corporation, BT Plc., Telecom Italia, CATT, Samsung | Rel-18 | 36.331 | NR_MG_enh2-Core | 5064 | 1 | F |
| R2-2410665 | Supporting R17 early implementation of R18 measurement gap enhancements | Nokia, Nokia Shanghai Bell, Ericsson, ZTE Corporation, BT Plc., Telecom Italia, CATT, Samsung | Rel-18 | 38.331 | NR_MG_enh2-Core | 5065 | 1 | F |
| R2-2410698 | Miscellaneous corrections on R18 SONMDT for 36.331 | Huawei, HiSilicon | Rel-18 | 36.331 | NR_ENDC_SON_MDT_enh2-Core | 5063 | 1 | F |
| R2-2410714 | MAC corrections on Release-18 Sidelink evolution | LG Electronics Inc. | Rel-18 | 38.321 | NR_SL_enh2 | 2012 |  | F |
| R2-2410723 | Correction on MCSt | LG Electronics Inc. | Rel-18 | 38.321 | NR_SL_enh2 | 1943 | 2 | F |
| R2-2410724 | MAC correction for resource selection of MCSt | LG | Rel-18 | 38.321 | NR_SL_enh2 | 1973 | 1 | F |
| R2-2410807 | Correction on TDD support in NTN | Huawei, HiSilicon | Rel-17 | 38.300 | NR_NTN_solutions-Core | 0942 |  | F |
| R2-2410808 | Correction on TDD support in NTN | Huawei, HiSilicon | Rel-18 | 38.300 | NR_NTN_solutions-Core | 0943 |  | A |
| R2-2410812 | Correction on multicast reception in RRC_INACTIVE upon paging | Huawei, HiSilicon, Nokia, CATT, Ericsson, Samsung, Apple, ZTE | Rel-18 | 38.331 | NR_MBS_enh-Core | 5191 |  | F |
| R2-2410820 | Correction to co-existence of multi-PUSCH CG and CG-SDTenh [CG-SDTenh] | Huawei, HiSIlicon | Rel-18 | 38.321 | TEI18, NR_XR_enh-Core | 1997 | 1 | F |
| R2-2410825 | Correction of SRS type for TA alignment | Ericsson | Rel-17 | 38.321 | NR_pos_enh-Core | 2014 |  | F |
| R2-2410826 | Correction of SRS type for TA alignment | Ericsson | Rel-18 | 38.321 | NR_pos_enh-Core | 2015 |  | A |
| R2-2410829 | Corrections on parallelTx capabilities for inter-band and intra-band | Huawei, HiSilicon, Ericsson, Nokia, Nokia Shanghai Bell | Rel-15 | 38.306 | NR_newRAT-Core | 1170 | 1 | F |
| R2-2410830 | Corrections on parallelTx capabilities for inter-band and intra-band | Huawei, HiSilicon, Ericsson, Nokia, Nokia Shanghai Bell | Rel-16 | 38.306 | NR_newRAT-Core | 1171 | 1 | A |
| R2-2410831 | Corrections on parallelTx capabilities for inter-band and intra-band | Huawei, HiSilicon, Ericsson, Nokia, Nokia Shanghai Bell | Rel-17 | 38.306 | NR_newRAT-Core | 1172 | 1 | A |
| R2-2410832 | Corrections on parallelTx capabilities for inter-band and intra-band | Huawei, HiSilicon, Ericsson, Nokia, Nokia Shanghai Bell | Rel-18 | 38.306 | NR_newRAT-Core | 1173 | 1 | A |
| R2-2410833 | Introduction of new capability for intra-band EN-DC channel spacing [Intra-Band_EN-DC_Channelspacing] | Huawei, HiSilicon | Rel-18 | 38.331 | TEI18 | 5013 | 1 | B |
| R2-2410834 | Introduction of new capability for intra-band EN-DC channel spacing [Intra-Band_EN-DC_Channelspacing] | Huawei, HiSilicon | Rel-18 | 38.306 | TEI18 | 1174 | 2 | B |
| R2-2410835 | Correction on UE capabilities for TCI state indication | Huawei, HiSilicon | Rel-17 | 38.331 | NR_FeMIMO-Core | 5014 | 2 | F |
| R2-2410836 | Correction on UE capabilities for TCI state indication | Huawei, HiSilicon | Rel-18 | 38.331 | NR_FeMIMO-Core | 5015 | 2 | A |
| R2-2410837 | Correction on UE capabilities for TCI state indication | Huawei, HiSilicon | Rel-17 | 38.306 | NR_FeMIMO-Core | 1175 | 2 | F |
| R2-2410839 | Correction on BWP operation without bandwidth restriction | Huawei, HiSilicon, ZTE Corporation, Ericsson | Rel-18 | 38.306 | NR_BWP_wor | 1177 | 1 | F |
| R2-2410859 | Stage 2 correction on additional buffer size table | NEC, Nokia (Rapporteur) | Rel-18 | 38.300 | NR_XR_enh-Core | 0917 | 2 | F |
| R2-2410860 | Corrections on distance-based measurements during T-Service for IoT NTN | Samsung | Rel-18 | 36.304 | IoT_NTN_enh-Core | 0876 | 1 | F |
| R2-2410866 | Correction to satellite ID in system infromation | Ericsson, Samsung | Rel-18 | 36.331 | IoT_NTN_enh-Core | 5081 |  | F |
| R2-2410867 | IoT NTN UE capabilities correction for GNSS and HARQ enhancements | Ericsson | Rel-18 | 36.306 | IoT_NTN_enh-Core | 1902 |  | F |
| R2-2410884 | IoT NTN Stage 2 correction to eMTC CHO | Ericsson (Rapporteur) | Rel-18 | 36.300 | IoT_NTN_enh-Core | 1409 | 1 | F |
| R2-2410893 | Clarification of MeasurementTimingConfiguration use | Vodafone,Ericsson,Nokia | Rel-18 | 38.331 | NR_BWP_wor-Core | 5016 | 2 | F |
| R2-2410896 | Rapporteur correction on multicast MCCH | Huawei, HiSilicon, Samsung, CATT | Rel-18 | 38.331 | NR_MBS_enh-Core | 5190 | 1 | F |
| R2-2410897 | Correction on UE capability for multi-carrier enhancements | NTT DOCOMO, INC., Huawei, HiSilicon | Rel-18 | 38.306 | NR_MC_enh-Core | 1199 | 2 | F |
| R2-2410900 | Gap requirement for CSI-RS based measurements and inter-RAT measurements | Huawei, HiSilicon, ZTE Corporation, Ericsson, Samsung, CATT, Apple, Nokia, Nokia Shanghai Bell, LG Electronics Inc., OPPO | Rel-16 | 38.300 | NR_newRAT-Core, TEI16, SRVCC_NR_to_UMTS-Core | 0906 | 3 | F |
| R2-2410901 | Gap requirement for CSI-RS based measurements and inter-RAT measurements | Huawei, HiSilicon, ZTE Corporation, Ericsson, Samsung, CATT, Apple, Nokia, Nokia Shanghai Bell, LG Electronics Inc., OPPO | Rel-17 | 38.300 | NR_newRAT-Core, TEI16, SRVCC_NR_to_UMTS-Core | 0907 | 2 | A |
| R2-2410902 | Gap requirement for CSI-RS based measurements and inter-RAT measurements | Huawei, HiSilicon, ZTE Corporation, Ericsson, Samsung, CATT, Apple, Nokia, Nokia Shanghai Bell, LG Electronics Inc., OPPO | Rel-18 | 38.300 | NR_newRAT-Core, TEI16, SRVCC_NR_to_UMTS-Core | 0908 | 2 | A |
| R2-2410903 | Clarification on the L2 U2N Remote UE Measurement | CATT | Rel-17 | 38.331 | NR_SL_relay-Core | 4977 | 2 | F |
| R2-2410904 | Clarification on the L2 U2N Remote UE Measurement | CATT | Rel-18 | 38.331 | NR_SL_relay-Core | 4978 | 2 | A |
| R2-2410906 | Correction on GNSS-AlmanacSupport and GNSS-UTC-ModelSupport in A-GNSS positioning | ZTE Corporation | Rel-16 | 37.355 | LCS_NAVIC-Core | 0516 | 2 | F |
| R2-2410907 | Correction on GNSS-AlmanacSupport and GNSS-UTC-ModelSupport in A-GNSS positioning | ZTE Corporation | Rel-17 | 37.355 | LCS_NAVIC-Core | 0517 | 2 | A |
| R2-2410908 | Correction on GNSS-AlmanacSupport and GNSS-UTC-ModelSupport in A-GNSS positioning | ZTE Corporation | Rel-18 | 37.355 | LCS_NAVIC-Core | 0518 | 2 | A |
| R2-2410909 | Correction on UE capabilities for TCI state indication | Huawei, HiSilicon | Rel-18 | 38.306 | NR_FeMIMO-Core | 1176 | 3 | A |
| R2-2410910 | Correction to the musim-AffectedBandsList and musim-AvoidedBandsList | ZTE Corporation | Rel-18 | 38.331 | NR_DualTxRx_MUSIM-Core | 5006 | 3 | F |
| R2-2410918 | Corrections on security for L2 U2U relay | vivo | Rel-18 | 38.323 | NR_SL_relay_enh-Core | 0141 | 2 | F |
| R2-2410919 | Correction on QoE measurements release at successful handover from LTE/5GC to NR | Nokia, Nokia Shanghai Bell, Ericsson | Rel-18 | 36.331 | NR_QoE_enh-Core | 5062 | 2 | F |
| R2-2410920 | Correction of Enhancement on NR QoE management and optimizations for diverse services | Ericsson, ZTE | Rel-18 | 38.331 | NR_QoE_enh-Core | 5030 | 2 | F |
| R2-2410924 | Miscellaneous corrections for LTM | Huawei, HiSilicon | Rel-18 | 38.321 | NR_Mob_enh2-Core | 1968 | 3 | F |
| R2-2410925 | Misc state 2 corrections for LTM mobility | Vodafone, Ericsson, Nokia, ZTE | Rel-18 | 38.300 | NR_Mob_enh2-Core | 0911 | 3 | F |
| R2-2410931 | RRC correction on sl-X-Overhead field description of SL-ResourcePool | Philips International B.V. | Rel-16 | 38.331 | 5G_V2X_NRSL-Core | 5058 | 3 | F |
| R2-2410932 | RRC correction on sl-X-Overhead field description of SL-ResourcePool | Philips International B.V. | Rel-17 | 38.331 | 5G_V2X_NRSL-Core | 5059 | 3 | A |
| R2-2410933 | RRC correction on sl-X-Overhead field description of SL-ResourcePool | Philips International B.V. | Rel-18 | 38.331 | 5G_V2X_NRSL-Core | 5060 | 3 | A |
| R2-2410934 | Correction on intra-UE prioritization | ZTE Corporation, Sanechips | Rel-16 | 38.321 | 5G_V2X_NRSL-Core | 1974 | 2 | F |
| R2-2410935 | Correction on intra-UE prioritization | ZTE Corporation, Sanechips | Rel-17 | 38.321 | 5G_V2X_NRSL-Core | 1975 | 2 | A |
| R2-2410936 | Correction on intra-UE prioritization | ZTE Corporation, Sanechips | Rel-18 | 38.321 | 5G_V2X_NRSL-Core | 1976 | 2 | A |
| R2-2410937 | Co-configuration of IUC scheme-2 and random selection | OPPO | Rel-17 | 38.321 | NR_SL_enh-Core | 2016 |  | F |
| R2-2410939 | Correction on unit of SL DRX timer | ZTE Corporation, Sanechips, Huawei, HiSilicon, Ericsson, Nokia, ASUSTeK, Apple, OPPO, LG | Rel-17 | 38.321 | NR_SL_enh-Core | 1940 | 2 | F |
| R2-2410940 | Correction on unit of SL DRX timer | ZTE Corporation, Sanechips, Huawei, HiSilicon, Ericsson, Nokia, ASUSTeK, Apple, OPPO, LG | Rel-18 | 38.321 | NR_SL_enh-Core | 1941 | 2 | A |
| R2-2410941 | Clarification on terminologies for SL IUC | ASUSTeK | Rel-17 | 38.321 | NR_SL_enh-Core | 1988 | 1 | F |
| R2-2410942 | Clarification on terminologies for SL IUC | ASUSTeK | Rel-18 | 38.321 | NR_SL_enh-Core | 1989 | 1 | A |
| R2-2410944 | Correction on carrier selection for SL evolution | Huawei, HiSilicon, LG Electronics Inc., Apple, Nokia | Rel-18 | 38.321 | NR_SL_enh2-Core | 1936 | 2 | F |
| R2-2410946 | Correction to MCSt for sidelink | Ericsson | Rel-18 | 38.321 | NR_SL_enh2-Core | 2018 |  | F |
| R2-2410948 | Scope of interFreqL1-MeasConfig-r18 | Huawei, HiSilicon | Rel-18 | 38.331 | NR_Mob_enh2-Core | 5198 |  | F |
| R2-2410949 | Scope of interFreqL1-MeasConfig-r18 | Huawei, HiSilicon | Rel-18 | 38.306 | NR_Mob_enh2-Core | 1222 |  | F |
| R2-2410950 | Correction on IUC | LG Electronics Inc. | Rel-17 | 38.321 | NR_SL_enh-Core | 2010 | 1 | F |
| R2-2410961 | Correction on Capability Reporting in FR1-NTN Bands | vivo | Rel-17 | 38.306 | NR_NTN_solutions-Core | 1207 | 1 | F |
| R2-2410962 | Correction on measurement | Huawei, HiSilicon | Rel-18 | 36.300 | IoT_NTN_enh-Core | 1414 | 1 | F |
| R2-2410963 | Corrections on location based measurements and need code for IoT NTN | CATT | Rel-18 | 36.331 | IoT_NTN_enh-Core | 5052 | 3 | F |
| R2-2410964 | Correction on SIB33(-NB) for IoT NTN | Nokia, Nokia Shanghai Bell, Samsung | Rel-18 | 36.331 | IoT_NTN_enh-Core | 5076 | 1 | F |
| R2-2410965 | UE feature for SIB33(-NB) reception in RRC_IDLE state in a TN cell | Apple, Qualcomm Incorporated, Samsung, Huawei, HiSilicon, Ericsson, CATT, MediaTek Inc. | Rel-18 | 36.306 | IoT_NTN_enh-Core | 1901 | 1 | F |
| R2-2410966 | Miscellaneous corrections to NR NTN | Samsung | Rel-18 | 38.300 | NR_NTN_enh-Core | 0922 | 3 | F |
| R2-2410970 | Correction on Capability Reporting in FR1-NTN Bands | vivo | Rel-18 | 38.306 | NR_NTN_solutions-Core | 1208 | 1 | A |
| R2-2410977 | Miscellaneous corrections on NTN in FR2 bands | ZTE Corporation, Vivo, Sanechips | Rel-18 | 38.331 | NR_NTN_enh-Core | 5161 | 1 | F |
| R2-2410980 | Correction on coexistence between CHO and satellite switching with re-synchronization | CATT, Nokia, Nokia Shanghai Bell, Ericsson | Rel-18 | 38.300 | NR_NTN_enh-Core | 0903 | 2 | F |
| R2-2410987 | Correction on spatial relation info in SRS configuration (R17) | ZTE Corporation, Ericsson | Rel-17 | 38.331 | NR_pos_enh-Core | 5101 | 1 | F |
| R2-2410988 | Correction on spatial relation info in SRS configuration (R18) | ZTE Corporation, Ericsson | Rel-18 | 38.331 | NR_pos_enh-Core | 5102 | 1 | A |
| R2-2410989 | Correction on assistance data transfer in SL positioning for stage-2 | ZTE Corporation | Rel-18 | 38.305 | NR_pos_enh2 | 0175 | 3 | F |
| R2-2410990 | Corrections of location time stamp, RSTD and RTOA report | CATT | Rel-18 | 38.355 | NR_pos_enh2-Core | 0008 | 4 | F |
| R2-2410991 | Miscellaneous corrections to SLPP specification | Intel Corporation | Rel-18 | 38.355 | NR_pos_enh2-Core | 0011 | 2 | D |
| R2-2410993 | Clarification on the maximum number of other UEs in sidelink positioning | vivo | Rel-18 | 38.305 | NR_pos_enh2-Core | 0178 | 2 | F |
| R2-2410994 | Correction on SLPP | Huawei, HiSilicon | Rel-18 | 38.355 | NR_pos_enh2 | 0014 | 2 | F |
| R2-2410995 | Correction on tx timestamp request in SL-RTT | ZTE Corporation | Rel-18 | 38.355 | NR_pos_enh2 | 0012 | 1 | F |
| R2-2410998 | Corrections for U2U relay measurements | ZTE Corporation, Sanechips, Huawei, Hisilicon, OPPO, CATT, Apple | Rel-18 | 38.331 | NR_SL_relay_enh-Core | 5175 | 1 | F |
| R2-2410999 | RLC correction for multi-path relay with N3C | Huawei, HiSilicon | Rel-18 | 38.322 | NR_SL_relay_enh-Core | 0063 | 1 | F |
| R2-2411003 | Miscellaneous correction on QoE measurement | Samsung | Rel-18 | 38.331 | NR_QoE_enh-Core | 5179 | 1 | F |
| R2-2411005 | Correction on Group Paging Handling | CATT, CBN, China Broadnet | Rel-18 | 38.331 | NR_MBS_enh-Core | 5194 | 1 | F |
| R2-2411008 | Clarification on group paging procedure | Samsung | Rel-18 | 38.331 | NR_MBS_enh-Core | 5134 | 1 | F |
| R2-2411011 | Correction on logging RSSI measurements in RLF report and SHR | Samsung | Rel-18 | 38.331 | NR_ENDC_SON_MDT_enh2-Core | 5138 | 1 | F |
| R2-2411012 | Correction on PSCell mobility history information | Samsung, ZTE Corporation, Sanechips, Ericsson | Rel-17 | 38.331 | NR_ENDC_SON_MDT_enh-Core | 5186 | 1 | F |
| R2-2411013 | Correction on PSCell mobility history information | Samsung, ZTE Corporation, Sanechips, Ericsson | Rel-18 | 38.331 | NR_ENDC_SON_MDT_enh-Core | 5188 | 1 | A |
| R2-2411014 | NR-U related RA information in SCGFailureInformation | Nokia | Rel-18 | 38.331 | NR_ENDC_SON_MDT_enh2-Core | 5126 | 1 | F |
| R2-2411015 | Addition of reference to 38.304 for Qoffsettemp handling | Nokia | Rel-18 | 36.331 | TEI18 | 5080 | 1 | A |
| R2-2411016 | Correction to NR Beam Reporting in LTE RRC | Apple, ZTE Corporation | Rel-18 | 36.331 | NR_newRAT-Core | 5069 | 2 | F |
| R2-2411017 | Correction on SI Reception in RRC CONNECTED | vivo | Rel-16 | 36.331 | LTE_eMTC5-Core | 5068 | 1 | F |
| R2-2411018 | Correction on SI Reception in RRC CONNECTED | vivo | Rel-17 | 36.331 | LTE_eMTC5-Core, LTE_NBIOT_eMTC_NTN | 5067 | 1 | F |
| R2-2411021 | Correction to Relaxed measurement | LG Electronics, Nokia, Samsung, Ericsson | Rel-16 | 38.304 | NR_UE_pow_sav-Core | 0412 | 6 | F |
| R2-2411022 | Correction to Relaxed measurement | LG Electronics, Nokia, Samsung, Ericsson | Rel-17 | 38.304 | NR_UE_pow_sav-Core | 0413 | 6 | A |
| R2-2411023 | Correction to Relaxed measurement | LG Electronics, Nokia, Samsung, Ericsson | Rel-18 | 38.304 | NR_UE_pow_sav-Core | 0414 | 6 | A |
| R2-2411024 | Corrections to smtc configuration | ZTE Corporation, Sanechips | Rel-15 | 38.331 | NR_newRAT-Core | 5155 | 1 | F |
| R2-2411025 | Corrections to smtc configuration | ZTE Corporation, Sanechips | Rel-16 | 38.331 | NR_newRAT-Core | 5156 | 1 | A |
| R2-2411026 | Corrections to smtc configuration | ZTE Corporation, Sanechips | Rel-17 | 38.331 | NR_newRAT-Core | 5157 | 1 | A |
| R2-2411027 | Corrections to smtc configuration | ZTE Corporation, Sanechips | Rel-18 | 38.331 | NR_newRAT-Core | 5158 | 1 | A |
| R2-2411028 | Clarification on inter-band handover enhancements capabilities | Qualcomm Inc., OPPO | Rel-16 | 38.306 | NR_Mob_enh-Core | 1186 | 4 | F |
| R2-2411029 | Clarification on inter-band handover enhancements capabilities | Qualcomm Inc., OPPO | Rel-17 | 38.306 | NR_Mob_enh-Core, NR_NTN_solutions-Core | 1187 | 5 | A |
| R2-2411032 | Miscellaneous corrections on stage 2 description of QMC | Lenovo | Rel-17 | 38.300 | NR_QoE-Core | 0930 | 1 | F |
| R2-2411033 | Correction of field description si-broadcastStatus and posSI-broadcast status to reflect maxSI message (Rel-17) | Xiaomi, Ericsson, LG electronics, Philips International B.V., Qualcomm Incorporated, ZTE Corporation | Rel-17 | 38.331 | TEI17 | 5107 | 1 | F |
| R2-2411034 | Correction of field description si-broadcastStatus and posSI-broadcast status to reflect maxSI message (Rel-18) | Xiaomi, Ericsson, LG electronics, Philips International B.V., Qualcomm Incorporated, ZTE Corporation | Rel-18 | 38.331 | TEI18 | 5108 | 1 | A |
| R2-2411035 | Alignment of highProrityAccess cause in RRC Establishment and Resume | Peraton Labs | Rel-18 | 36.300 | NR_newRAT-Core, TEI18 | 1413 | 1 | F |
| R2-2411036 | Correction on logging PSCell identity | CATT | Rel-18 | 38.331 | NR_ENDC_SON_MDT_enh2-Core | 5105 | 1 | F |
| R2-2411037 | Corrections on measurement gap configuration | ZTE Corporation, Sanechips | Rel-17 | 38.331 | NR_NTN_solutions-Core, NR_redcap-Core | 5024 | 3 | F |
| R2-2411038 | Corrections on measurement gap configuration | ZTE Corporation, Sanechips | Rel-18 | 38.331 | NR_NTN_solutions-Core, NR_redcap-Core | 5025 | 2 | A |
| R2-2411039 | Correction on deprioritisationTimer | Nokia | Rel-18 | 38.331 | NR_newRAT-Core | 5170 | 2 | A |
| R2-2411040 | Mandatory support of Enhanced channel raster by (e)RedCap UE | Ericsson | Rel-18 | 38.306 | NR_redcap-Core | 1158 | 4 | F |
| R2-2411061 | Miscellaneous RRC Positioning Correction | Ericsson | Rel-18 | 38.331 | NR_pos_enh2-Core | 5061 | 2 | F |
| R2-2411066 | Introduction of LCS User Plane | Ericsson, Intel Corporation, Huawei, HiSilicon, ZTE Corporation, vivo, Qualcomm Incorporated, Samsung, CATT, Nokia | Rel-18 | 38.305 | 5G_eLCS_Ph3 | 0159 | 6 | F |
| R2-2411074 | Correction on UE capability for multi-carrier enhancements | NTT DOCOMO, INC., Huawei, HiSilicon | Rel-18 | 38.331 | NR_MC_enh-Core | 5095 | 3 | F |
| R2-2411075 | Correction for cell barring for 2Rx XR UE [2Rx_XR_Device] | Huawei, HiSilicon, Xiaomi, Ericsson, LGE, ZTE Corporation | Rel-18 | 38.331 | TEI18 | 4851 | 3 | F |
| R2-2411080 | Correction on the idc-TDM-AssistanceConfig | Huawei, HiSilicon, Xiaomi, Nokia, Intel Corporation, Ericsson, Apple, Samsung | Rel-18 | 38.331 | NR_IDC_enh-Core | 5062 | 2 | F |
| R2-2411081 | Guidelines on implementing FRx/xDD differentiation in per UE capability | Huawei, HiSilicon | Rel-17 | 38.306 | NR_newRAT-Core, TEI17 | 1219 |  | F |
| R2-2411082 | Guidelines on implementing FRx/xDD differentiation in per UE capability | Huawei, HiSilicon | Rel-18 | 38.306 | NR_newRAT-Core, TEI17 | 1211 | 1 | A |
| R2-2411083 | Guidelines on implementing FRx/xDD differentiation in per UE capability | Huawei, HiSilicon | Rel-17 | 38.331 | NR_newRAT-Core, TEI17 | 5195 |  | F |
| R2-2411084 | Guidelines on implementing FRx/xDD differentiation in per UE capability | Huawei, HiSilicon | Rel-18 | 38.331 | NR_newRAT-Core, TEI17 | 5162 | 2 | A |
| R2-2411086 | Correction to available UL-SCH resources in SR triggering | Samsung (Rapporteur), Ericsson, Spreadtrum | Rel-16 | 38.321 | NR_2step_RACH-Core | 1962 | 2 | F |
| R2-2411087 | Correction to available UL-SCH resources in SR triggering | Samsung (Rapporteur), Ericsson, Spreadtrum | Rel-17 | 38.321 | NR_2step_RACH-Core | 1963 | 2 | A |
| R2-2411088 | Correction to available UL-SCH resources in SR triggering | Samsung (Rapporteur), Ericsson, Spreadtrum | Rel-18 | 38.321 | NR_2step_RACH-Core | 1964 | 2 | A |
| R2-2411089 | Correction on the field name of candidate beam list in BFR MAC CE | Samsung | Rel-17 | 38.321 | NR_FeMIMO-Core | 1980 | 1 | F |
| R2-2411090 | Correction on the field name of candidate beam list in BFR MAC CE | Samsung | Rel-18 | 38.321 | NR_FeMIMO-Core | 1981 | 1 | A |
| R2-2411091 | PHR for serving cells configured with multiple TRP PUSCH repetition in NR-DC | Huawei, HiSilicon | Rel-17 | 38.321 | NR_FeMIMO-Core | 2005 | 1 | F |
| R2-2411092 | PHR for serving cells configured with multiple TRP PUSCH repetition in NR-DC | Huawei, HiSilicon | Rel-18 | 38.321 | NR_FeMIMO-Core | 2006 | 1 | A |
| R2-2411093 | Correction on RACH-less handover fallback [RACH-lessHO] | Huawei, HiSilicon, Samsung, CATT, LG Electronics, Nokia, Nokia Shanghai Bell, Ericsson | Rel-18 | 38.321 | NR_mobile_IAB-Core, NR_NTN_enh-Core, TEI18 | 1972 | 3 | F |
| R2-2411094 | Network energy savings for NR rapporteur RRC CR | Huawei, HiSilicon | Rel-18 | 38.331 | Netw_Energy_NR-Core | 5011 | 2 | F |
| R2-2411095 | Correction on PHR for mTRP PUSCH repetition | LG Electronics Inc., MediaTek Inc., CATT, Ericsson | Rel-17 | 38.321 | NR_FeMIMO-Core | 1892 | 6 | F |
| R2-2411096 | Correction on PHR for mTRP PUSCH repetition (R17) | LG Electronics Inc., MediaTek Inc., CATT, Ericsson | Rel-17 | 38.331 | NR_FeMIMO-Core | 5068 | 3 | F |
| R2-2411097 | Correction on PHR for mTRP PUSCH repetition | LG Electronics Inc., MediaTek Inc., CATT, Ericsson | Rel-17 | 38.306 | NR_FeMIMO-Core | 1190 | 3 | F |
| R2-2411098 | Correction on PHR for mTRP PUSCH repetition (R18) | LG Electronics Inc., MediaTek Inc., CATT, Ericsson | Rel-18 | 38.321 | NR_FeMIMO-Core | 1893 | 5 | A |
| R2-2411099 | Correction on PHR for mTRP PUSCH repetition (R18) | LG Electronics Inc., MediaTek Inc., CATT, Ericsson | Rel-18 | 38.331 | NR_FeMIMO-Core | 5069 | 3 | A |
| R2-2411100 | Correction on PHR for mTRP PUSCH repetition (R18) | LG Electronics Inc., MediaTek Inc., CATT, Ericsson | Rel-18 | 38.306 | NR_FeMIMO-Core | 1191 | 3 | A |
| R2-2411105 | Clarification of UE capability restrictions in MUSIM | Ericsson, Huawei, HiSilicon, Samsung, Xiaomi, LG Electronics Inc., Vivo | Rel-18 | 38.300 | NR_DualTxRx_MUSIM-Core | 0920 | 3 | F |
| R2-2411106 | Correction to multi-PUSCH configured grant | Huawei, HiSilicon, Qualcomm, Apple, vivo, Nokia, Nokia Shanghai Bell | Rel-18 | 38.321 | NR_XR_enh-Core | 2002 | 1 | F |
| R2-2411107 | Correction for Delay Critical Indication from PDCP to RLC | Samsung, LG Electronics Inc., Nokia, Huawei, HiSilicon, Ericsson, Qualcomm Incorporated | Rel-18 | 38.323 | NR_XR_enh-Core | 0144 | 1 | F |
| R2-2411108 | Correction for Paging monitoring during SDT [CG-SDT-Enh] | Huawei, HiSilicon, LG Electronics Inc., ZTE Corporation, Sanechips, MediaTek Inc. | Rel-18 | 38.331 | TEI18 | 4901 | 3 | F |
| R2-2411109 | Clarifications on cell barring behaviour | Qualcomm Incorporated, Huawei, ZTE, Ericsson, LG Electronics Inc., Nokia, Samsung, Vivo | Rel-18 | 38.331 | TEI18 | 5112 | 1 | F |
| R2-2411110 | Clarifications on cell barring behaviour | Qualcomm Incorporated, Huawei, ZTE, Ericsson, LG Electronics Inc., Nokia, Samsung, Vivo | Rel-18 | 38.304 | TEI18 | 0421 | 1 | F |
| R2-2411112 | Rapporteur CR to IDLE mode procedure for R18 Positioning | Huawei, HiSilicon, Phillips | Rel-18 | 38.304 | NR_pos_enh2 | 0422 | 2 | F |
| R2-2411113 | Applicability of optional UE Capabilities for NB-IoT | Qualcomm Inc. | Rel-18 | 36.306 | IoT_NTN_enh-Core | 1894 | 3 | F |
| R2-2411115 | Correction on Co-channel coexistence for LTE sidelink and NR sidelink | LG Electronics Inc. | Rel-18 | 38.321 | NR_SL_enh2 | 1942 | 2 | F |
| R2-2411118 | Correction on HARQ-RTT-TimerDL-NTN in NR NTN | CATT, LG Electronics Inc., Sharp | Rel-17 | 38.321 | NR_NTN_solutions-Core | 2019 |  | F |
| R2-2411119 | Correction on HARQ-RTT-TimerDL-NTN in NR NTN | CATT, LG Electronics Inc., Sharp | Rel-18 | 38.321 | NR_NTN_solutions-Core | 2020 |  | A |
| R2-2411121 | Corrections on capabilities for FG R1 41-1-19a and 41-1-19b in IE CommonSL-PRS-MethodsIEsProvideCapabilities | Lenovo | Rel-18 | 38.355 | NR_pos_enh2 | 0013 | 1 | F |
| R2-2411122 | Correction of NR-DL-TDOA-MeasurementCapability and NR-Multi-RTT-MeasurementCapability | CATT | Rel-18 | 37.355 | NR_pos_enh2-Core | 0528 | 1 | F |
| R2-2411123 | Correction for the UE capability on PosSRS-BWA-RRC-Inactive | Xiaomi, Lenovo | Rel-18 | 37.355 | NR_pos_enh2-Core | 0540 | 1 | F |
| R2-2411124 | Correction for the UE capability on PosSRS-BWA-RRC-Inactive | Xiaomi, Lenovo | Rel-18 | 38.306 | NR_pos_enh2-Core | 1212 | 1 | F |
| R2-2411130 | Correction for UE indicating its preference TX/RX frequencies for sidelink positioning in SUI | vivo | Rel-18 | 38.331 | NR_pos_enh2-Core | 5106 | 1 | F |
| R2-2411131 | Correction on IUC | LG Electronics Inc. | Rel-18 | 38.321 | NR_SL_enh-Core | 2011 | 1 | A |
| R2-2411137 | Misc RRC corrections for feMob | Ericsson | Rel-18 | 38.331 | NR_Mob_enh2-Core | 5096 | 3 | F |
| R2-2411139 | Co-configuration of IUC scheme-2 and random selection | OPPO | Rel-18 | 38.321 | NR_SL_enh-Core | 2017 | 1 | A |
| R2-2411140 | Correction on carrier selection for SL evolution | NEC | Rel-18 | 38.321 | NR_SL_enh2-Core | 2021 | 1 | F |
| R2-2411161 | Correction on priority-based QoE measurements in TS 38.300 | Huawei, HiSilicon, China Unicom, Nokia, Ericsson, ZTE, Apple | Rel-18 | 38.300 | NR_QoE_enh-Core | 0919 | 3 | F |
| R2-2411163 | Correction on the Less than 5M Bandwidth | ZTE Corporation, vivo | Rel-18 | 38.331 | NR_FR1_lessthan_5MHz_BW-Core | 5187 | 1 | F |
| R2-2411164 | Correction on the Less than 5M Bandwidth | ZTE Corporation | Rel-18 | 38.306 | NR_FR1_lessthan_5MHz_BW-Core | 1220 |  | F |
| R2-2411165 | Correction in the optionality of NR-U related information in SON reports | Nokia | Rel-18 | 38.306 | NR_ENDC_SON_MDT_enh2-Core | 1221 |  | F |
| R2-2411170 | Sidelink RRC Positioning Correction | Ericsson, vivo | Rel-18 | 38.331 | NR_pos_enh2-Core | 5165 | 1 | F |
| R2-2411171 | Miscellaneous corrections for Positioning | Ericsson | Rel-18 | 38.300 | NR_pos_enh2-Core | 0938 | 1 | F |
| R2-2411172 | RRC correction on NR sidelink positioning | Philips International B.V. | Rel-18 | 38.331 | NR_pos_enh2-Core | 5174 | 1 | F |
| R2-2411176 | U2U Relays, Local ID Assignment | Ericsson | Rel-18 | 38.300 | NR_SL_relay_enh-Core | 0944 | 1 | F |
| R2-2411178 | Correction on RACH-less handover procedure in TS38.321 [RACH-lessHO] | CATT, Samsung, InterDigital, Ericsson, Qualcomm Incorporated, LG Electronics Inc., Huawei, HiSilicon, vivo | Rel-18 | 38.321 | TEI18, NR_NTN_enh-Core, NR_mobile_IAB-Core | 2003 | 1 | F |
| R2-2411181 | Rapporteur CR to MAC spec for R18 Positioning | Huawei, HiSilicon, ASUSTek | Rel-18 | 38.321 | NR_pos_enh2 | 1996 | 2 | F |
| R2-2411182 | Corrections on the Terminology of Peer UE | CATT | Rel-18 | 38.331 | NR_SL_relay_enh-Core | 5197 | 1 | F |
| R2-2411183 | Corrections on RRC SRAP configuration for L2 U2U | Apple, ZTE | Rel-18 | 38.331 | NR_SL_relay_enh-Core | 5125 | 2 | F |
| R2-2411185 | Miscellaneous and editorial changes for NR sidelink relay enhancement | LG Electronics, ZTE Corporation, Sanechips, Philips International B.V. | Rel-18 | 38.300 | NR_SL_relay_enh-Core | 0923 | 2 | F |
| R2-2411189 | Correction for positioning SRS CA in RRC_INACTIVE | Huawei, HiSilicon, Ericsson, Samsung | Rel-18 | 38.331 | NR_pos_enh2 | 5150 | 2 | F |
| R2-2411193 | Clarification on the UE feature for cell reselection from TN to NTN | Apple, Qualcomm Incorporated, Samsung, Huawei, HiSilicon, CATT, MediaTek Inc. | Rel-18 | 38.306 | NR_NTN_enh-Core | 1206 | 2 | F |
| R2-2411202 | Correction on Band Selection and Cell Barring for Aerial UEs | Ericsson | Rel-18 | 38.331 | NR_UAV-Core | 5149 | 2 | F |
| R2-2411203 | Correction on UE receiver features | China Telecom | Rel-18 | 38.306 | NR_demod_enh3-Core | 1223 |  | F |
| R2-2411204 | Correction on SI Reception in RRC CONNECTED | vivo | Rel-18 | 36.331 | LTE_eMTC5-Core, LTE_NBIOT_eMTC_NTN | 5066 | 2 | F |
| R2-2411206 | Correction on NavIC almanac set IE, and field descriptions under KlobucharModelParamater, UTC-ModelSet2, and GNSS-SystemTime. | Reliance Jio, MediaTek, Ericsson, Qualcomm Incorporated, CEWiT | Rel-16 | 37.355 | LCS_NAVIC-Core | 0531 | 3 | F |
| R2-2411207 | Correction on NavIC L5 almanac set IE, and field descriptions under KlobucharModelParamater, UTC-ModelSet2, and GNSS-SystemTime | Reliance Jio, MediaTek, Ericsson, Qualcomm Incorporated, CEWiT | Rel-17 | 37.355 | LCS_NAVIC-Core | 0533 | 3 | A |
| R2-2411208 | Correction on NavIC almanac set IE, and field descriptions under KlobucharModelParamater, UTC-ModelSet2, and GNSS-SystemTime. | Reliance Jio, MediaTek, Ericsson, Qualcomm Incorporated, CEWiT | Rel-18 | 37.355 | LCS_NAVIC-Core | 0534 | 3 | A |
| R2-2411209 | Cell DRX MPS exemption in RA | Peraton Labs, CISA ECD, AT&T, Verizon, FirstNet, Qualcomm, ZTE | Rel-18 | 38.321 | Netw_Energy_NR-Core | 2007 | 1 | F |
| R2-2411210 | Capability on measurement gap enhancements | Huawei, HiSilicon | Rel-18 | 36.331 | NR_MG_enh2 | 5082 | - | F |
| R2-2411211 | Capability on measurement gap enhancements | Huawei, HiSilicon | Rel-18 | 36.306 | NR_MG_enh2 | 1903 | - | F |
| R2-2411212 | Correction to LTM MAC CE based CFRA with MSG1 repetition | Huawei, HiSilicon | Rel-18 | 38.321 | NR_Mob_enh2-Core, NR_cov_enh2-Core, NR_redcap-Core, NR_redcap_enh-Core | 1969 | 2 | F |
| R2-2411216 | CR on TCI state indication of CORESET#0 | OPPO | Rel-17 | 38.321 | NR_FeMIMO-Core | 1994 | 3 | F |
| R2-2411217 | CR on TCI state indication of CORESET#0 | OPPO | Rel-18 | 38.321 | NR_FeMIMO-Core | 1995 | 3 | A |
| R2-2411223 | Correction to RRC for CG-SDT enhancement [CG-SDTenh] | Huawei, HiSilicon | Rel-18 | 38.331 | TEI18, NR_MT_SDT-Core | 5152 | 1 | F |
| R2-2411224 | Miscellaneous non-controversial corrections Set XXIII | Ericsson | Rel-15 | 38.331 | NR_newRAT-Core | 5127 | 1 | F |
| R2-2411225 | Miscellaneous non-controversial corrections Set XXIII | Ericsson | Rel-16 | 38.331 | NR_newRAT-Core, TEI16 | 5128 | 1 | F |
| R2-2411226 | Miscellaneous non-controversial corrections Set XXIII | Ericsson | Rel-17 | 38.331 | NR_newRAT-Core, TEI17 | 5082 | 2 | F |
| R2-2411227 | Miscellaneous non-controversial corrections Set XXIII | Ericsson | Rel-18 | 38.331 | NR_newRAT-Core, TEI18 | 5083 | 2 | F |
| R2-2411230 | Introduction of network signalling of maximum number of UL segments [Max-RRC-SegUL] | Huawei, HiSilicon | Rel-17 | 36.306 | TEI17 | 1904 |  | B |
| R2-2411231 | Introduction of network signalling of maximum number of UL segments [Max-RRC-SegUL] | Huawei, HiSilicon | Rel-18 | 36.306 | TEI17 | 1905 |  | A |
| R2-2411233 | Correction on conditional PSCell addition or change failure | R3 (CATT, CMCC, ZTE, Nokia, Nokia Shanghai Bell) | Rel-18 | 37.340 | NR_ENDC_SON_MDT_enh2-Core | 0407 |  | F |
| R2-2411234 | Correction on AI/ML Terminology | R3 (Nokia, Ericsson, ZTE, Huawei) | Rel-18 | 38.300 | NR_AIML_NGRAN-Core | 0945 |  | F |
| R2-2411235 | OAM Requirements on Energy Cost index | R3 (Nokia, Jio, Telecom Italia, Ericsson, ZTE, Samsung, NEC, Huawei, Deutsche Telekom) | Rel-18 | 38.300 | NR_AIML_NGRAN-Core | 0946 |  | F |
| R2-2411236 | Correction of IAB F1-C transfer in NR-DC | R3 (ZTE Corporation, Ericsson, Lenovo, Huawei, Qualcomm, Nokia, Nokia Shanghai Bell, CATT, Samsung) | Rel-17 | 37.340 | NR_IAB_enh-Core | 0408 |  | F |
| R2-2411237 | Correction of IAB F1-C transfer in NR-DC | R3 (ZTE Corporation, Ericsson, Lenovo, Huawei, Qualcomm, Nokia, Nokia Shanghai Bell, CATT, Samsung) | Rel-18 | 37.340 | NR_IAB_enh-Core | 0409 |  | A |
| R2-2411238 | Stage 2 correction for AI/ML for NG-RAN | R3 (Ericsson, Huawei, Nokia, InterDigital, ZTE, NEC, Deutsche Telekom) | Rel-18 | 38.300 | NR_AIML_NGRAN-Core | 0947 |  | F |
| R2-2411239 | QoE measurement handling in idle mode | R3 (CATT, Ericsson, ZTE, Xiaomi) | Rel-18 | 38.300 | NR_QoE_enh-Core | 0948 |  | F |
| R2-2411240 | Correction to area-specific SRS activation | R3 (CATT, ZTE, Ericsson, China Telecom, Nokia, Samsung, Qualcomm Incorporated, Xiaomi, Huawei) | Rel-18 | 38.305 | NR_pos_enh2-Core | 0181 |  | F |
| R2-2411241 | Coarse UE Location Information Reporting from MME to eNB for NB-IoT UEs | R3 (Qualcomm, Ericsson, Nokia, Nokia Shanghai Bell, ZTE, Huawei, CATT) | Rel-18 | 36.300 | IoT_NTN_enh-Core | 1415 |  | F |
| R2-2411242 | Corrections to Stage-2 for XR awareness | R3 (Ericsson, Qualcomm Inc., Nokia, Nokia Shanghai Bell, Huawei, Samsung, ZTE, China Telecom, Xiaomi, CATT, CMCC, Lenovo) | Rel-18 | 38.300 | NR_XR_enh-Core | 0949 |  | F |
| R2-2411243 | Stage-2 updates for intra-SN S-CPAC with MN involvement | R3 (Ericsson, Nokia, CATT, LG Electronics, Huawei, ZTE, Samsung) | Rel-18 | 37.340 | NR_Mob_enh2-Core | 0410 |  | F |
| R2-2411245 | Miscellaneous updates for Rel-18 WIs | Intel Corporation | Rel-18 | 38.306 | NR_MC_enh, NR_Mob_enh2, NR_MIMO_evo_DL_UL-Core, NR_netcon_repeater-Core, Netw_Energy_NR | 1205 | 2 | F |
| R2-2411246 | Miscellaneous updates for Rel-18 WIs | Intel Corporation | Rel-18 | 38.331 | NR_Mob_enh2 | 5120 | 2 | F |
| R2-2411247 | Introduction of network signalling of maximum number of UL segments [Max-RRC-SegUL] | Qualcomm Incorporated | Rel-17 | 38.331 | TEI17 | 5004 | 3 | B |
| R2-2411248 | Introduction of network signalling of maximum number of UL segments [Max-RRC-SegUL] | Qualcomm Incorporated | Rel-18 | 38.331 | TEI17 | 5113 | 1 | A |
| R2-2411249 | Introduction of network signalling of maximum number of UL segments [Max-RRC-SegUL] | Qualcomm Incorporated | Rel-17 | 38.306 | TEI17 | 1202 | 1 | B |
| R2-2411250 | Introduction of network signalling of maximum number of UL segments [Max-RRC-SegUL] | Qualcomm Incorporated | Rel-18 | 38.306 | TEI17 | 1203 | 1 | A |
| R2-2411252 | Introduction of network signalling of maximum number of UL segments [Max-RRC-SegUL] | Huawei, HiSilicon | Rel-17 | 36.331 | TEI17 | 5083 | 1 | B |
| R2-2411253 | Introduction of network signalling of maximum number of UL segments [Max-RRC-SegUL] | Huawei, HiSilicon | Rel-18 | 36.331 | TEI17 | 5084 | 1 | A |
| R2-2411254 | Correction for RA resource information in RA-report | Sharp, Ericsson, Lenovo | Rel-18 | 38.331 | NR_ENDC_SON_MDT_enh-Core | 5145 | 1 | A |
| R2-2411255 | Correction to MAC for R17 POS-r17 | Huawei, HiSilicon | Rel-17 | 38.321 | NR_pos_enh-Core | 1998 | 1 | F |
| R2-2411256 | Correction on applicable BWP for multi-cell scheduling | NTT DOCOMO, INC., Nokia, Samsung, Huawei | Rel-18 | 38.331 | NR_MC_enh-Core | 5077 | 2 | F |
| R2-2411257 | Corrections for mobility enhancements in stage-2 | ZTE Corporation, Ericsson | Rel-18 | 37.340 | NR_Mob_enh2-Core | 0404 | 4 | F |
| R2-24111258 | Multicast reception after reselection to cell with MCCH | Nokia, Samsung, Ericsson, ZTE | Rel-18 | 38.331 | NR_MBS_enh-Core | 5153 | 3 | F |
| R2-2411259 | Clarification on inter-band handover enhancements capabilities | Qualcomm Inc., OPPO | Rel-18 | 38.306 | NR_Mob_enh-Core, NR_NTN_enh-Core, NR_ATG-Core, Netw_Energy_NR-Core | 1188 | 6 | F |
| R2-2411260 | Miscellaneous corrections for NES | ZTE Corporation, Sanechips | Rel-18 | 38.331 | Netw_Energy_NR-Core | 5146 | 2 | F |
| R2-2411261 | Clarification on SCS for Timing Advance Report MAC CE for ATG | Samsung, Qualcomm Inc., Huawei, HiSilicon, CMCC, CATT | Rel-18 | 38.321 | NR_ATG-Core | 1954 | 3 | F |
| R2-2411262 | Clarification on the unit of offsetThresholdTA-r18 for ATG | ZTE Corporation | Rel-18 | 38.331 | NR_ATG-Core | 5189 | 2 | F |
| R2-2411264 | Correction for CFRA configuration due to PRACH partitioning | Huawei, HiSilicon, ZTE Corporation, Ericsson, Mediatek | Rel-18 | 38.331 | NR_redcap-Core, NR_slice-Core | 4900 | 3 | A |

284 Agreed CRs.

# Annex F:	List of email discussions during the meeting

Discussions during the R2-128 meeting:

List of AT meeting email discussions

[AT128][003][QoE] Update to R2-2410653 (Huawei)

[AT128][005][RedCapLTM] CR  (Huawei)

[AT128][006][UP] NTN and on shot feedback  (CATT)

[AT128][007][UAV] Cell barring (Ericsson)

[AT128][008][CE] Response LS to RAN1 (Huawei)

[AT128][009][MUSIM] 38.300 CR (Ericsson)

[AT128][011][SDT] CG-SDT CR (Huawei)

[AT128][012][less5MHz] UE capabilities (ZTE)

[AT128][013][ATG] offline  (ZTE)

[AT128][014][AIML] LS response to SA2 (Interdigital)

[AT128][016][AIoT] Resource validity (Interdigital)

[AT128][017][AI mob]Simulation assumptions  ()

[AT128][024][NR others] NR_MG_enh  (Huawei)

[AT128][101][MOB] (Ericsson)

[AT128][102][MOB] (Huawei)

[AT128][103][MOB] (Nokia)

[AT128][104][MOB] (ZTE)

[AT128][111][NES] (OPPO)

[AT128][112][V2X/SL] (ZTE)

[AT128][113][MOB] (Qualcomm)

[AT128][114][MOB] (Huawei)

[AT128][115][MOB] (CATT)

[AT128][116][MOB] (ZTE)

[AT128][200] Organizational – Rel-18 MIMO, Rel-19 MIMO, LPWUS, and SBFD (RAN2 VC)

[AT128][201][MIMOevo] Proposals/CRs for PHR (Samsung)

[AT128][202][MIMOevo] Discuss on the modelling and review the MAC CR for 8Tx 2TB if the modelling is agreeable (ASUSTeK, ZTE)

[AT128][203][MIMOevo] LS on differentiation of sDCI mTRP, mDCI mTRP and sTRP (CATT)

[AT128][204][MIMOevo] RRC CR on TDD UL/DL Configuration for Two TA (Ericsson)

[AT128][205][LPWUS] R2 impact analysis and draft reply LS on LP-WUS supporting in different bands (VODAFONE)

[AT128][300] Organizational – NR-NTN and IoT-NTN session

[AT128][301][R18 NR NTN] SMTC issues (Huawei)

[AT128][302][R18 NR NTN] LS to RAN3 on RACH-less HO (Nokia)

[AT128][303][R19 NR NTN] reply LS to RAN3 on MBS broadcast service for NTN (Xiaomi)

[AT128][304][R19 IoT NTN] LS to SA2 on satellite IDs for S&F (CATT)

[AT128][305][R19 IoT NTN] PWS reception in connected (Interdigital)

[AT128][401][POS] Revision of NavIC assistance data upates (CEWiT)

[AT128][402][POS] Spatial relation info source for positioning in RRC_INACTIVE (ZTE)

[AT128][403][POS] ASN.1 formalities in R2-2409618 (CATT)

[AT128][404][POS] Rel-18 positioning MAC CR update (Huawei)

[AT128][405][Relay] Peer UE terminology (CATT)

[AT128][406][Relay] U2U SRAP configuration (Apple)

[AT128][407][Relay] NOTE on U2U local ID allocation (Ericsson)

[AT128][408][POS] SLPP capability corrections (Lenovo)

[AT128][409][POS] Missing ASN.1 field for RSCPD measurement capability in RRC_INACTIVE (CATT)

[AT128][410][POS] UE capability on PosSRS-BWA-RRC-Inactive (Xiaomi)

[AT128][411][Relay] LS to SA2 on relay discovery announcement (ZTE)

[AT128][500] Organizational – Session on R18 MBS, R18 QoE and R19 XR

[AT128][501][XR] Reply LS on PDU set (vivo)

[AT128][502][XR] Reply LS on AL-FEC (QCM)

[AT128][503][QoE] QoE reporting SRB change issue (Huawei)

[AT128][505][MBS] Update R2-2410254  (Nokia)

[AT128][600] Organizational – Maintenance and SON/MDT (Ericsson)

[AT128][601][SONMDT] Correction on PSCell mobility history information (Samsung)

[AT128][602][SONMDT] Corrections on SPR configuration (CATT)

[AT128][603][SONMDT] NR-U related RA information in SCGFailureInformation (Nokia)

[AT128][604][Maint] Reference to 38.304 for Qoffsettemp (Nokia)

[AT128][605][Maint] Corrections on SI Reception (vivo)

[AT128][606][Maint] Example (Nokia)

[AT128][607][Maint] Corrections to smtc configuration (ZTE)

[AT128][608][Maint] Inter-band handover capabilities (Qualcomm)

[AT128][609][Maint] Reply LS on waveform determination (CATT)

[AT128][610][Maint] Misc QoE corrections (Lenovo)

[AT128][611][Maint] Description of si-broadcastStatus (Xiaomi)

[AT128][612][Maint] UE processing delay time for max nrof segments (Qualcomm)

66 at-meeting offline discussions.

# Annex G:	Post-meeting email discussions

# Guidelines for email discussions:

General guidelines for SHORT email discussions, to be concluded approved endorsed at current meeting.

- Aim to have the final version of the agreed documents provided by the rapporteur at or shortly after the deadline.

- Please provide comments on the first version of the document in good time before the deadline. This allows the rapporteur to make an update addressing all companies' comments and there still be time for a quick round of comments on the update.

- If you have provided comments in the discussion then please indicate to the rapporteur if you are ok with the update provided (preferably via reflector). This avoids the rapporteur having to wait before they can conclude that their update is acceptable to you.

- Rapporteurs, if not already available, please request your tdoc number from the Secretary when you initiate your email discussion and then provide the final version as soon as you are confident that it is agreeable. You do not need to wait for a reminder from chair, session chair or Secretary before sending the final version.

- To avoid any confusion, Secretary, chair, or session chair will send an email to confirm the final status of the document.

For emails discussion to the next meeting (long):

- Rapporteurs, feel free to set an intermediate deadline for companies to provide initial comments, so that the conclusions and proposals can be prepared and distributed before the final deadline.

- Participants, please respect any intermediate deadline indicated by the rapporteur, and preferably provide your feedback as soon as possible.

# Deadlines

Nov. 29th 	1000 UTC	Deadline short email discussions

Jan.  31st 	1000 UTC	Deadline long email discussions

Feb.  7th	1000 UTC	Submission Deadline RAN2#129

Weekends are inactive periods.

It is recommended to not send emails or update files on the server during inactive periods while It is not strictly prohibited. Rapporteurs may kick-off discussions during inactive period. However, no intermediate deadlines and no interactive discussion, no decision making may occur during the inactive period.   It shall be possible for a delegate to stay away from reflector and 3GPP server during the inactive period, and still fully participate. Rapporteur announcements during the inactive period, if any, or other updates, can be taken into account after the inactive period.

# Short email discussions, Deadline Nov. 29th, 10:00 UTC

Please request R2-128 TDoc numbers for the following email discussions from MCC if not already allocated. Approval / endorsement will be declared at or shortly after the deadline.

- Short email discussions

[POST128][002][UE Cap] UE capability CR  (Intel)

Intended outcome: agree to CR

Deadline:  short

=> Agreed in:

R2-2411245 (38.306)

R2-2411246 (38.331)

[POST128][004][RRC] Rapporteur correction (Ericsson)

Intended outcome: Review and agree to all CRs from R15 (if applicable), R16, R17, and 18

Deadline:  short

=> Agreed in:

R2-2411224 (38.331 CR#5127r1)

R2-2411225 (38.331 CR#5128r1)

R2-2411226 (38.331 CR#5082r2)

R2-2411227 (38.331 CR#5083r2)

[POST128][006][MIMOevo] LS on 8Tx 2TB  (Samsung)

Intended outcome: Approve the LS

Deadline: short

=> Approved in R2-2411220.

[POST128][007][MIMOevo] LS on differentiation of sDCI mTRP, mDCI mTRP and sTRP  (CATT)

Intended outcome: Approve the LS

Deadline: short

=> Approved in R2-2411244.

[POST128][008][MIMOevo] RRC CR on TDD UL/DL Configuration for Two TA (Ericsson)

Intended outcome: Agree the CR

Deadline: short

=> Postponed in R2-2410954

[POST128][011][SDT] CG-SDT CR (Huawei)

Intended outcome: review updates and agree by email

Deadline:  short

=> Agreed in R2-2411223

[POST128][015][AIoT] TP for TR  (Huawei)

Intended outcome: agreable TP to send to RAN1 and accompanying LS

Deadline:  Dec. 1st

=> Endorsed in R2-2411221 (TP)

=> Approved in R2-2411263 (LSout)

[POST128][024][NR others] NR_MG_enh  (Huawei)

Intended outcome: agree to CRs by email

Deadline:  short

=> Agreed in:

R2-2411210 (36.331)

R2-2411211 (36.306)

[POST128][105][MOB] (Apple)

Scope: Update 38.300 running CR capturing all RAN2 agreements.

Intended outcome: 38.300 running CR in R2-24010927, to be endorsed.

Deadline: Short email discussion

=> Endorsed in R2-24010927

[POST128][106][MOB] (China Telecom)

Scope: Update 37.340 running CR capturing all RAN2 agreements.

Intended outcome: 37.340 running CR in R2-24010928, to be endorsed.

Deadline: Short email discussion

=> Endorsed in R2-24010928

[POST128][117][MOB] (Lenovo)

Scope: Prepare LS to RAN4 and RAN3 to take the RAN2 agreements (including C-LTM based on L3 measurements, and possible other agreements from inter-CU LTM and L1-event driven MR) into account for their jobs.

Intended outcome: LS in R2-2411134, to be approved.

Deadline: Short email discussion

=> Approved in R2-2411134

[Post128][301][R19 IoT NTN] LS to SA2 on Satellite IDs for S&F (CATT)

Scope: Draft a LS to SA2 on satellite IDs based on the outcome of the discussion on R2-2410974

Intended outcome: Approved LS

Deadline: short

=> Approved in R2-2411197

[Post128][404][POS] LS to RAN1 on SL-PRS resource pool configuration (vivo)

Scope: Draft an LS to RAN1 inquiring as to the intention of the agreement quoted in R2-2409639, to clarify whether the restriction should be documented as a network or UE behaviour.

Intended outcome: Approved LS

Deadline: Short (not for RP)

=> Approved in R2-2411251

[Post128][405][Relay] Check of late AIP stage 2 CR on Rel-18 relay (LG)

Scope: Check the AIP CR in R2-2411185 and confirm that it can be agreed.

Intended outcome: Agreed CR

Deadline: Short (for RP)

=> Agreed in R2-2411185

[POST128][601][Maint] CRs for maximum number of UL segments (Qualcomm)

Scope: Produce agreeable CR for NR and LTE

Intended outcome: Agreed CRs

Deadline: Short

=> Agreed in:

R2-241230 36.306 (Rel-17)

R2-241231 36.306 (Rel-18)

R2-241247 38.331 (Rel-17)

R2-241248 38.331 (Rel-18)

R2-241249 38.306 (Rel-17)

R2-241250 38.306 (Rel-18)

R2-241252 36.331 (Rel-17)

R2-241253 36.331 (Rel-18)

[POST128][602][Maint] LS on emergency services and eDRX (ZTE)

Scope: Produce approvable LS

Intended outcome: Approved LS in R2-2411041 (ZTE)

Deadline: Short

=> Approved in R2-2411041.

# Long email discussions, for R2-129, Deadline Jan.  31st , 10:00 UTC (unless otherwise stated)

Please request R2-129 TDoc numbers for the following email discussions by 3GU according to normal tdoc submission procedure.

[Post128][018][AI Mob] generalization  (Apple)

Intended outcome: Discuss parameters for different cell configuration and attempt to prioritize 1 parameters and not more than 2 values per parameter.   Can do 2 max values if really reneed.   for

Deadline:  3 weeks

[POST128][019][AI PHY] NW side data collection (Nokia)

Intended outcome: Discuss the motivation and specification complexity for the three radio conditions.

Deadline:  Long

[POST128][020][AIoT] LS to SA2 on information usefulness (ZTE)

Intended outcome:  LS to SA2

Deadline:  4 weeks

[POST128][021][AI Mob] Templates for simulations  (Mediatek/Oppo)

Intended outcome:

1.	agree to updated and new templates.  Companies should update their simulation results in the new template (Mediatek)

2.	agree principles on how to capture observations from simulations (Oppo)

Deadline:  Jan. 15th

[POST128][026][AIML] LCM Procedure for Positioning Case1 (Ericsson)

Intended outcome:

•		Discuss the FFS from the existing RAN2 Agreements

- Attempt to resolve the FFS which does not need RAN1 input

- Identify questions to ask to RAN1 for resolving FFS waiting on RAN1 progress

•		Collect general guidelines on the criteria for deciding whether to enhance legacy method(s) or introduce new method

Deadline: Long

[POST128][107][MOB] (Ericsson)

Scope: For inter-CU LTM and C-LTM parts, prepare 38.331 running CR capturing all RAN2 agreements, identify stage 3 issues (possibly with rapporteur’s suggestion) that need to be discussed, and discuss them.

Intended outcome: 38.331 running CR, to be endorsed next meeting, and discussion summary.

Deadline: Long

[POST128][108][MOB] (Huawei)

Scope: For L1 event-driven MR, prepare 38.331 running CR capturing all RAN2 agreements, identify stage 3 issues (possibly with rapporteur’s suggestion) that need to be discussed, and discuss them.

Intended outcome: 38.331 running CR, to be endorsed next meeting, and discussion summary.

Deadline: Long

[POST128][109][MOB] (Vivo)

Scope: Prepare 38.321 running CR capturing all RAN2 agreements, identify stage 3 issues (possibly with rapporteur’s suggestion) that need to be discussed, and discuss them.

Intended outcome: 38.321 running CR, to be endorsed next meeting, and discussion summary.

Deadline: Long

[Post128][401][Relay] Control plane baseline solution (InterDigital)

Scope: Develop the baseline control plane procedures for connection establishment, and paging/system information forwarding, towards a stage 3 development stage.

Intended outcome: Report to RAN2#129

Deadline: Long

[Post128][402][POS] BDS B2b stage 3 CR check (CATT)

Scope: Check and update the CR in R2-2409627.

Intended outcome: Agreeable CR

Deadline: Long

[Post128][403][POS] NavIC L1 stage 3 CR check (Reliance Jio)

Scope: Check the CR in R2-2409726 and update if necessary.

Intended outcome: Agreeable CR

Deadline: Long

# Annex H:	History

| Document history |  |  |
| --- | --- | --- |
| Date | TSG RAN WG2 Tdoc | Subject |
| 26.11.2024 |  | First draft version |
| 08.02.2025 |  | Second draft version |
| 11.02.2025 |  | Third draft version |
| 14.02.2025 |  | Fourth draft version |
| 16.02.2025 | R2-2500002 | Version submitted for approval |
| Author: Dr. Juha Korhonen ETSI Mobile Competence Centre (MCC) Tel. +33 (0)4 92 94 42 00 email: Juha.Korhonen@etsi.org |  |  |