3GPP TSG RAN WG2#129	R2-2500036

Athens, Greece, 17th - 21st February 2025

3GPP TSG-RAN WG4 Meeting #113		R4-2420165

Orlando, FL, US, 18th – 22nd November 2024

Title:	LS on CSI-RS measurement with SBFD operation

Response to:

Release:	R19

Work Item:	NR_duplex_evo-Core

Source:	RAN4

To:	RAN1

Cc:	RAN2

Contact person:	Waseem Ozan

waseem.ozan@mediatek.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

# 1	Overall description

Issue #1: RAN4 discussed the RRM requirements impact based on the following RAN1 agreement:

RAN1#116bis agreements: If link direction indication is not supported nor provided for a SBFD symbol, for collision Case 2 (semi-statically configured DL reception vs. dynamically scheduled UL transmission) in the SBFD symbol for SBFD-aware UEs, reuse the existing collision handling principles in NR for operation on flexible symbols on a single carrier in unpaired spectrum, i.e. UE does not receive DL channel/signal. - The above does not imply link direction indication is supported - FFS on dynamically scheduled UL transmission with repetition

RAN4 seeks clarification from RAN1 perspective regarding whether the collision scenario between configured DL reception and dynamic UL transmission includes CSI-RS resources used for L1-RSRP/SINR measurement and RLM/BFD/CBD measurement. Specifically, if semi-statically configured CSI-RS resources for above mentioned measurements collide with dynamically scheduled UL transmission in the SBFD slots or symbols, which action should the UE prioritize: performing measurements on the semi-statically configured CSI-RS or executing the dynamically scheduled UL transmission?

Issue #2: RAN4 discussed the RRM requirements impact based on the following RAN1 agreement:

Agreement (RAN1#117) For UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols in different slots (each transmission/reception within a slot has either all SBFD or all non-SBFD symbols) for an SBFD aware UE, the SBFD-aware UE is provided with one of the configurations. · Configuration 1: The transmissions/receptions are restricted to SBFD symbols only or non-SBFD symbols only · Configuration 2: The transmissions/receptions can be in SBFD symbols and non-SBFD symbols · FFS granularity of the configuration, e.g. per UE, per channel/signal etc. FFS whether support of configuration 2 is subject to UE capability Agreement (RAN1#118bis) For UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols in different slots (each transmission/reception within a slot has either all SBFD or all non-SBFD symbols) for an SBFD aware UE, the SBFD-aware UE is provided with Configuration 1 or Configurat 2 via RRC configuration. Select from the following options - Option 1: SBFD-aware UE is configured with Configuration 1 or Configuration 2 on a per UE per serving cell basis - Option 2: SBFD-aware UE is configured with Configuration 1 or Configuration 2 on a per UE per BWP per channel/signal basis - Separate configurations for configured and dynamic transmission/receptions for a given channel/signal are not precluded. - Option 3: Configuration 1 or Configuration 2 is on a per cell basis. - No separate UE capabilities for Configuration 1 and 2. Agreement (RAN1#118bis) For a CSI report associated with periodic/semi-persistent CSI-RS, the valid symbol type for CSI derivation for periodic/semi-persistent CSI-RS resources for the CSI report is explicitly configured. Only CSI-RS transmission occasions within the valid symbol types are used for CSI derivation.

To define the appropriate requirements, RAN4 needs to understand from RAN1 perspective whether the above agreement on resource configuration and/or reporting configuration apply to CSI-RS based L1-RSRP/SINR measurement and RLM/BFD/CBD measurement.

# 2	Actions

To RAN1

ACTION: 	RAN4 respectfully asks RAN1 to provide clarification on the above two issues.

# 3	Dates of next TSG RAN WG4 meetings

RAN4#114	Feb. 17 – 21, 2024	Athens, Greece

RAN4#114bis	April 07 – 11, 2024	China