3GPP TSG RAN WG2#129	R2-2500037

Athens, Greece, 17th - 21st February 2025

3GPP TSG-RAN WG4 Meeting #113	R4-2420198

Orlando, Florida, USA, November 18 – 22, 2024

Title:	Response LS on gaps/restrictions that are caused by RRM measurements

Response to:	R1-2407561

Release:	Rel-19

Work Item:	NR XR Phase 3 (NR_XR_Ph3-Core)

Source:	RAN4

To:	RAN1

Cc:	RAN2

Contact person:	Prashant Sharma

prasshar (at) qti.qualcomm.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

# 1	Overall description

Working Assumption

For solutions based on triggering/enabling by network signaling to enable Tx/Rx in gaps/restrictions that are caused by RRM measurements select the following option:

- Alt. 1: Dynamic indication to enable Tx/Rx in particular gap/restriction that are caused by RRM measurements.

  - Alt 1-1: Explicit indication by DCI to skip a particular gap/restriction;

    - Indication is included as part of scheduling DCI:

      - Bit-field size is one bit;

        - The bit in the DCI is used to indicate whether to skip the first gap/restriction occasion after a minimum time offset required between the last symbol of the PDCCH carrying the DCI format and the start of corresponding skipped gap/restriction occasion indicated by the DCI.

Send an LS to RAN4 to inform them of the above working assumption and ask them if there is any issue with it.

Agreement

If Alt. 1 from RAN1#117 agreement is supported, minimum time offset(s) X between indication to skip and skipped measurement occasion is up to RAN4 to discuss and decide on particular value(s).

RAN4 thanks RAN1 for the LS on gaps/restrictions that are caused by RRM measurements informing about the following working assumption.

RAN4 discussed the above RAN1 working assumption and confirms that Alt 1-1 is feasible from RAN4 perspective with X=5ms as baseline, and X=3ms with optional UE capability.

It is also RAN4 understanding that the time offset is the time from the end of the last symbol of the PDCCH carrying the DCI to the start of the corresponding skipped measurement gap indicated by the DCI, where the start of the measurement gap is determined after applying Measurement Gap Timing Advance (mgta) configured by the network according to TS 38.331.

Furthermore, from RAN4 RRM requirements perspective, Alt 1-1 is not supported for EN-DC and NE-DC scenarios.

Lastly, in addition to Alt 1-1, RAN4 is also discussing the need to introduce semi-static indication based solutions to skip measurement gap occasions.

# 2	Actions

To RAN1

ACTION:  RAN4 respectfully asks RAN1 to take the above information in consideration.

# 3	Dates of next TSG RAN WG 4 meetings

TSG-RAN WG4 Meeting #114	17 – 21 February 2025	Athens, GR

TSG-RAN WG4 Meeting #114-bis	07 – 11 April 2025	China