3GPP TSG-SA2 Meeting #166-AdHoc-e	S2-2500386

# E-meeting, 20 – 24 January 2025	(revision of S2-250xxxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.304 | CR | 0535 | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Authorization information for Layer-2 multi-hop U2N relaying to NG-RAN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe_Ph3 |  |  |  |  |  | Date: |  |  | 2025-01-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Authorization information for Layer-2 multi-hop U2N relaying needs to be provided from AMF to NG-RAN. |  |  |  |  |  |  |  |  |
| Summary of change: |  | §4.3.4, §5.7, §6.6.2 - Add Layer-2 multi-hop UE-to-Network Relay aspects. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No authorization information for Layer-2 multi-hop U2N relaying is provided to NG-RAN. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.4, 5.7, 6.6.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of 1st Change * * * *

4.3.4	AMF

In addition to the functions defined in TS 23.501 [4], the AMF performs the following functions:

-	Select a PCF supporting 5G ProSe Policy/Parameter provisioning based on indication of 5G ProSe Capability as part of the "5GMM capability" in the Registration Request.

-	Store the 5G ProSe Capability.

-	Forward the 5G ProSe Capability to PCF in Npcf_UEPolicyControl_Create Request.

-	Obtain from UDM the subscription information related to 5G ProSe and store them as part of the UE context data.

-	Obtain PC5 QoS parameters from the PCF and store them as part of the UE context data.

-	Provision the NG-RAN with indication about the UE authorization status about the following:

-	5G ProSe Direct Discovery and 5G ProSe Direct Communication (i.e. as 5G ProSe-enabled UE for ProSe Direct Discovery, as 5G ProSe-enabled UE for ProSe Direct Communication);

-	5G ProSe UE-to-Network Relay Discovery and Communication (i.e. as 5G ProSe Layer-2 Remote UE, as 5G ProSe Layer-2 UE-to-Network Relay, as 5G ProSe Layer-3 UE-to-Network Relay);

-	Multi-path communication via direct Uu path and via 5G ProSe Layer 2 UE-to-Network Relay as a 5G ProSe Layer-2 Remote UE;

-	5G ProSe UE-to-UE Relay Discovery and Communication (i.e. as 5G ProSe Layer-2 End UE, as 5G ProSe Layer-2 UE-to-UE Relay);

-	5G ProSe Layer-3 multi-hop UE-to-Network Relay Discovery and Communication (i.e. as 5G ProSe Layer-3 UE-to-Network Relay supporting 5G ProSe Layer-3 multi-hop UE-to-Network Relay);

-	5G ProSe Layer-2 multi-hop UE-to-Network Relay Discovery and Communication (i.e. as 5G ProSe Layer-2 UE-to-Network Relay supporting 5G ProSe Layer-2 multi-hop UE-to-Network Relay, as 5G ProSe Layer-2 Intermediate UE-to-Network Relay, as 5G ProSe Layer-2 Remote UE supporting 5G ProSe Layer-2 multi-hop UE-to-Network Relay).

-	Provision the NG-RAN with PC5 QoS parameters related to 5G ProSe Direct Communication.

-	Optionally support security procedures over Control Plane for 5G ProSe UE-to-Network relaying as defined in TS 33.503 [29].

* * * * Start of Next Change * * * *

5.7	Subscription to 5G ProSe

The subscription information in the UDM contains information to give the user permission to use 5G ProSe.

At any time, the operator can amend or remove the ProSe UE subscription rights from subscription information in the UDM, or to revoke the user's permission to use 5G ProSe.

The following subscription information is defined for 5G ProSe:

-	subscription for open 5G ProSe Direct Discovery for NR PC5:

-	open 5G ProSe Direct Discovery Model A.

-	subscription for restricted 5G ProSe Direct Discovery for NR PC5:

-	restricted 5G ProSe Direct Discovery Model A;

-	restricted 5G ProSe Direct Discovery Model A with application-controlled extension;

-	restricted 5G ProSe Direct Discovery Model A with "on demand" announcing;

-	restricted 5G ProSe Direct Discovery Model B.

-	subscription for Broadcast, Groupcast and Unicast mode 5G ProSe Direct Communication for NR PC5.

-	subscription for 5G ProSe UE acting as 5G ProSe Layer-2 UE-to-Network Relay.

-	subscription for 5G ProSe UE acting as 5G ProSe Layer-3 UE-to-Network Relay.

-	subscription for 5G ProSe Layer-2 Remote UE access via 5G ProSe Layer-2 UE-to-Network Relay.

-	subscription for 5G ProSe Layer-3 Remote UE access via 5G ProSe Layer-3 UE-to-Network Relay.

-	subscription for multi-path communication via direct Uu path and via 5G ProSe Layer 2 UE-to-Network Relay as a 5G ProSe Layer-2 Remote UE.

-	subscription for 5G ProSe UE acting as 5G ProSe Layer-2 UE-to-UE Relay.

-	subscription for 5G ProSe UE acting as 5G ProSe Layer-3 UE-to-UE Relay.

-	subscription for 5G ProSe Layer-2 End UE access via 5G ProSe Layer-2 UE-to-UE Relay.

-	subscription for 5G ProSe Layer-3 End UE access via 5G ProSe Layer-3 UE-to-UE Relay.

-	subscription for 5G ProSe UE acting as 5G ProSe Layer-3 Intermediate UE-to-Network Relay.

-	subscription for 5G ProSe UE acting as 5G ProSe Layer-3 Remote UE supporting Multi-hop 5G ProSe Layer-3 UE-to-Network Relay.

-	subscription for 5G ProSe UE acting as 5G ProSe Layer-3 UE-to-Network Relay supporting Multi-hop 5G ProSe Layer-3 UE-to-Network Relay.

-	subscription for 5G ProSe UE acting as 5G ProSe Layer-2 Intermediate UE-to-Network Relay.

-	subscription for 5G ProSe UE acting as 5G ProSe Layer-2 Remote UE supporting Multi-hop 5G ProSe Layer-2 UE-to-Network Relay.

-	subscription for 5G ProSe UE acting as 5G ProSe Layer-2 UE-to-Network Relay supporting Multi-hop 5G ProSe Layer-2 UE-to-Network Relay.

-	subscription for 5G ProSe UE acting as 5G ProSe Layer-3 UE-to-UE Relay supporting Multi-hop 5G ProSe Layer-3 UE-to-UE Relay.

-	subscription for 5G ProSe UE acting as 5G ProSe Layer-3 End UE supporting Multi-hop 5G ProSe Layer-3 UE-to-UE Relay.

-	UE-PC5-AMBR for NR PC5.

-	PC5 QoS parameters as defined in clause 5.6.1 used by NG-RAN.

-	the list of the PLMNs authorized for 5G ProSe services, including:

-	the list of the PLMNs where the UE is authorised for open 5G Direct Discovery Model A, i.e. to announce or monitor or both.

-	the list of the PLMNs where the UE is authorised for restricted 5G ProSe Direct Discovery Model A, i.e. to announce or monitor or both.

-	the list of the PLMNs where the UE is authorised for restricted 5G ProSe Direct Discovery Model B, i.e. to perform Discoverer operation or Discoveree operation or both.

-	the list of the PLMNs where the UE is authorised to perform Broadcast, Groupcast and Unicast mode 5G ProSe Direct Communication for NR PC5.

-	the list of the PLMNs where the UE is authorised to act as a 5G ProSe Layer-2 UE-to-Network Relay.

-	the list of the PLMNs where the UE is authorised to act as a 5G ProSe Layer-3 UE-to-Network Relay.

-	the list of the PLMNs where the UE is authorised to act as a 5G ProSe Layer-2 Remote UE.

-	the list of the PLMNs where the UE is authorised to act as a 5G ProSe Layer-2 UE-to-UE Relay.

-	the list of the PLMNs where the UE is authorised to act as a 5G ProSe Layer-3 UE-to-UE Relay.

-	the list of the PLMNs where the UE is authorised to act as a 5G ProSe Layer-2 End UE.

-	the list of the PLMNs where the UE is authorised to act as a 5G ProSe Layer-3 End UE.

-	the list of the PLMNs where the UE is authorised to act as a 5G ProSe Layer-3 Intermediate UE-to-Network Relay.

-	the list of the PLMNs where the UE is authorised to act as a 5G ProSe Layer-3 UE-to-Network Relay supporting Multi-hop 5G ProSe Layer-3 UE-to-Network Relay.

-	the list of the PLMNs where the UE is authorised to act as a 5G ProSe Layer-2 Intermediate UE-to-Network Relay.

-	the list of the PLMNs where the UE is authorised to act as a 5G ProSe Layer-2 UE-to-Network Relay supporting Multi-hop 5G ProSe Layer-2 UE-to-Network Relay.

-	the list of the PLMNs where the UE is authorised to act as a 5G ProSe Layer-2 Remote UE supporting Multi-hop 5G ProSe Layer-2 UE-to-Network Relay.

-	the list of the PLMNs where the UE is authorised to act as a 5G ProSe Layer-3 UE-to-UE Relay supporting Multi-hop 5G ProSe Layer-3 UE-to-UE Relay.

-	the list of the PLMNs where the UE is authorised to act as a 5G ProSe Layer-3 End UE supporting Multi-hop 5G ProSe Layer-3 UE-to-UE Relay.

* * * * Start of Next Change * * * *

6.6.2	Registration procedure

The Registration procedure for UE is performed as defined in TS 23.502 [5] clause 4.2.2.2 with the following additions:

-	The UE includes the 5G ProSe Capability as part of the "5GMM capability" in the Registration Request message. The AMF stores the 5G ProSe Capability for 5G ProSe operation.

-	The 5G ProSe Capability indicates whether the UE supports one or more of the following ProSe capabilities:

-	5G ProSe Direct Discovery;

-	5G ProSe Direct Communication;

-	5G ProSe Layer-2 UE-to-Network Relay;

-	5G ProSe Layer-3 UE-to-Network Relay;

-	5G ProSe Layer-2 Remote UE;

-	5G ProSe Layer-3 Remote UE;

-	5G ProSe Layer-2 UE-to-UE Relay;

-	5G ProSe Layer-3 UE-to-UE Relay;

-	5G ProSe Layer-2 End UE;

-	5G ProSe Layer-3 End UE;

-	5G ProSe Layer-3 UE-to-Network Relay supporting 5G ProSe Layer-3 multi-hop UE-to-Network Relay;

-	5G ProSe Layer-3 Remote UE supporting 5G ProSe Layer-3 multi-hop UE-to-Network Relay;

-	5G ProSe Layer-3 Intermediate UE-to-Network Relay;

-	5G ProSe Layer-2 UE-to-Network Relay supporting 5G ProSe Layer-2 multi-hop UE-to-Network Relay;

-	5G ProSe Layer-2 Remote UE supporting 5G ProSe Layer-2 multi-hop UE-to-Network Relay;

-	5G ProSe Layer-2 Intermediate UE-to-Network Relay.

-	5G ProSe Layer-3 UE-to-UE Relay supporting 5G ProSe Layer-3 multi-hop UE-to-UE Relay;

-	5G ProSe Layer-3 End UE supporting 5G ProSe Layer-3 multi-hop UE-to-UE Relay.

NOTE 1:	Encoding of these capabilities is defined in TS 24.501 [38].

-	The AMF obtains the 5G ProSe subscription data as part of the user subscription data from UDM during UE Registration procedure using Nudm_SDM service as defined in clause 4.2.2.2.2 of TS 23.502 [5].

-	The AMF determines whether the UE is authorised to use 5G ProSe services based on UE's 5G ProSe Capability and the ProSe Service Authorisation included in the subscription data received from UDM as specified in clause 5.7. ProSe NR UE-PC5-AMBR is also provided to the AMF as part of the subscription data for 5G ProSe services. The AMF stores the authorized 5G ProSe Capability.

-	The AMF sends the authorized 5G ProSe Capability for 5G ProSe operation to PCF. Based on the received 5G ProSe Capability from the AMF, the PCF provides the PC5 QoS parameters for 5G ProSe to AMF. The AMF stores such information as part of the UE context.

-	If the UE is authorised to use 5G ProSe services, then the AMF shall include in a NGAP message sent to NG-RAN:

-	"5G ProSe authorised" information, including one or more of the following:

-	whether the UE is authorized to use 5G ProSe Direct Discovery;

-	whether the UE is authorized to use 5G ProSe Direct Communication;

-	whether the UE is authorized to act as a 5G ProSe Layer-2 UE-to-Network Relay;

-	whether the UE is authorized to act as a 5G ProSe Layer-3 UE-to-Network Relay;

-	whether the UE is authorized to act as a 5G ProSe Layer-2 Remote UE;

-	whether the UE is authorized to use multi-path communication via direct Uu path and via 5G ProSe Layer-2 UE-to-Network Relay as a 5G ProSe Layer-2 Remote UE;

-	whether the UE is authorized to act as a 5G ProSe Layer-2 UE-to-UE Relay;

-	whether the UE is authorized to act as a 5G ProSe Layer-2 End UE;

-	whether the UE is authorized to act as a 5G ProSe Layer-3 UE-to-Network Relay supporting 5G ProSe Layer-3 multi-hop UE-to-Network Relay;

-	whether the UE is authorized to act as a 5G ProSe Layer-2 UE-to-Network Relay supporting 5G ProSe Layer-2 multi-hop UE-to-Network Relay;

-	whether the UE is authorized to act as a 5G ProSe Layer-2 Intermediate UE-to-Network Relay;

-	whether the UE is authorized to act as a 5G ProSe Layer-2 Remote UE supporting 5G ProSe Layer-2 multi-hop UE-to-Network Relay.

NOTE 2:	The authorization for "5G ProSe Direct discovery" and "5G ProSe Direct communication" can be used for 5G ProSe Layer-3 UE-to-UE Relay, 5G ProSe Layer-3 End UE, 5G ProSe Layer-3 UE-to-UE Relay supporting 5G ProSe Layer-3 multi-hop UE-to-UE Relay and 5G ProSe Layer-3 End UE supporting 5G ProSe Layer-3 multi-hop UE-to-UE Relay.

-	ProSe NR UE-PC5-AMBR, used by NG-RAN for the resource management of UE's PC5 transmission for 5G ProSe services in network scheduled mode.

-	the PC5 QoS parameters for 5G ProSe used by the NG-RAN for the resource management of UE's PC5 transmission for ProSe services in network scheduled mode.

-	If the UE is authorised to use 5G ProSe services, then the AMF should not initiate the release of the signalling connection after the completion of the Registration procedure. The release of the signalling connection relies on the decision of NG-RAN, as specified in TS 23.502 [5].

* * * * End of Changes * * * *