3GPP TSG-RAN WG2 Meeting #129	R2-2500075

Athens, Greece, February 17th –21th , 2025

Agenda Item:	8.5.2

Source: 	Xiaomi

Title:  	Discussion on on-demand SSB

Document for:	Discussion and Decision

1	Introduction

In RAN2#127bis meeting, we had some further discussion on OD-SSB and achieved the following agreement.

Agreements on OD-SSB SCell

- No need to restrict the OD-SSB activation/deactivation state indication in RRC to initial configuration. No special specification effort is required.

- Don’t introduce further new MAC CE that combines SCell activation/deactivation and OD-SSB indication for scenario 2A.

- NW should be able to send OD-SSB indication for multiple SCells simultaneously by a MAC CE.

In this contribution, we would like to further discuss the procedures and signalling to support on-demand SSB and have corresponding observations and proposals.

2	Discussion

2.1 Activation/deactivation of OD-SSB

2.1.1 RRC based activation/deactivation

Also according to RAN1 discussion, deactivation of OD-SSB transmission is supported and there are the following alternatives to indicate the deactivation of OD-SSB transmission explicitly or implicitly.

Agreement For a cell supporting on-demand SSB SCell operation, support at least the following options to deactivate on-demand SSB transmission from a UE perspective. - Option 1: Explicit indication of deactivation for on-demand SSB via MAC-CE for on-demand SSB transmission indication - Deactivation by RRC is up to RAN2 - FFS: Which scenario Option 1 is used - Option 2: Configuration/indication of the number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated - FFS: Whether Option 4, 4a is needed in addition to Option 2 - FFS: Whether the value of N can be implicitly determined using a timer

Regarding whether to support RRC based deactivation, with the following agreement we achieved in previous meeting, deactivation of OD-SSB transmission of the cell can be realized via reconfiguration of OD-SSB activation/deactivation state indication, with no additional specification impact.

- No need to restrict the OD-SSB activation/deactivation state indication in RRC to initial configuration. No special specification effort is required.

Proposal 1: Deactivation of the OD-SSB transmission of the SCell by RRC can be supported by reconfiguration of the OD-SSB activation/deactivation state indication. No special specification effort is required.

2.1.2 MAC CE based activation/deactivation

In addition, RAN2 agreed to include OD-SSB indication for multiple SCells simultaneously in the same MAC CE. Generally, there are two options as listed below:

- Altenative1: bitmap to indicate the activation/deactivation status of OD-SSB per SCell

One byte/four bytes bitmap can be used to indicate the activation/deactivation status of SSB transmission per SCell.

- Alternative2: SCell index to indicate the activation of OD-SSB per SCell

For this option, SCell index is explicitly indicated in the MAC CE. Only index related to an OD-SSB SCell of which the OD-SSB transmission is activated can be included.

According to RAN1 agreement, both explicit and implicit deactivation are supported. To support explicit deactivation, bitmap like indication in alternative1 can be used. While to support implicit deactivation, either bitmap indication in alternative 1 or SCell index indication in alternative 2 can be used, and configuration/indication of the number N of on-demand SSB bursts should be indicated as well. In addition, RAN1 has also agreed to indicate the applicable periodicity with MAC CE, therefore, at least periodicity and number N are included in MAC CE. Since it is not sure if any other parameters will be indicated by MAC CE, it is better to introduce an ID which can be used to associate with a set of applicable parameters.

Proposal 2: Define a configuration ID associated with at least periodicity and number N of OD-SSB bursts/timer of OD-SSB transmission.

Proposal 3: For each SCell with OD-SSB transmission activated, include the configuration ID in MAC CE.

The next issue is whether to use single bitmap to indicate both the activation and deactivation of OD-SSB. If so, when explicitly deactivating the OD-SSB of some SCells, the indications related to the OD-SSB activated SCells need to be kept as “enabled”. In addition, the additional information associated with the OD-SSB activated SCell needs to be contained as well, e.g., number N of OD-SSB bursts/timer of OD-SSB transmission. However, this additional information needs to be updated for the OD-SSB SCells which is “re-enabled”, which introduces complexity. Take an example as below.

The first OD-SSB A/D MAC CE is to activate OD-SSB transmission on SCell1, SCell2 and SCell3 with each SCell associated with a number N of bursts/duration to support implicit deactivation. The second OD-SSB A/D MAC CE is to explicitly deactivate the OD-SSB transmission on SCell1, however, the bits related to SCell2 and SCell3 should be kept as “enabled” and the number N of bursts/duration associated with SCell2 and SCell3 needs to be indicated as well. However, compared to that contained in the first OD-SSB A/D MAC CE, the number N of bursts/duration associated with SCell2 and SCell3 in the second OD-SSB A/D MAC CE may need to be updated, otherwise, if the same value as in the first OD-SSB A/D MAC CE is used, UE may misunderstand the OD-SSB transmission on SCell2 and SCell3 are extended. In this case, it requires the NW to remember the time point to transmit each OD-SSB A/D MAC CE and the transmitted number of bursts on each SCell during this interval. Or if the corresponding value is not updated, it requires the UE to remember the above information.

Based on the above analysis, it may introduce additional complexity on UE and/or NW if both the activation and deactivation status are indicated via the same MAC CE.

Observation 1: It introduces additional complexity on UE and/or NW if both the activation and deactivation status are indicated via the same MAC CE.

Therefore, we think to support explicit deactivation and implicit deactivation, it is better to indicate the activation and deactivation status separately.

Proposal 4: Introduce two MAC CEs to indicate the activation and deactivation of OD-SSB transmission separately.

- OD-SSB activation MAC CE

The most straightforward design is to include explicit SCell index, if the number of configured OD-SSB SCell is limited, the signaling overhead is not critical. According to the existing specification, the length of the SCell index is 5 bits. In addition, RAN1 has agreed the supported periodicity of OD-SSB is 5ms, 10ms, 20ms, 40ms, 80ms and 160ms, see below. Even there is still no conclusion on the value of number N. To us, 8-bit of the configuration ID is sufficient, similar as the TRS ID in enhanced SCell activation/deactivation MAC CE.

Agreement - The periodicity of on-demand SSB is one of 5 ms, 10 ms, 20 ms, 40 ms, 80 ms, or 160 ms. - New periodicity value for on-demand SSB other than the legacy values (i.e., 5 ms, 10 ms, 20 ms, 40 ms, 80 ms, or 160 ms) is NOT introduced in Rel-19.

Therefore, the detailed format of alternative 2 is shown as below.

Proposal 5: OD-SSB activation MAC CE includes explicit SCell index and the associated OD-SSB configuration ID.

- OD-SSB deactivation MAC CE

The most straightforward design is to reuse a bitmap for explicit deactivation, e.g., if the corresponding bit is set to 1, it indicates the deactivation of the OD-SSB transmission on the related SCell, otherwise, UE ignores the indication bit. Regarding the definition of each bit, to save signalling overhead, it is proposed each bit is related to an OD-SSB SCell, but to keep the MAC CE simple and unified, the bitmap should still remain as byte aligned. If the OD-SSB SCell is no more than 8, then one byte bitmap is sufficient, see below.

-	Oi: This field indicates the deactivation status of the on-demand SSB transmission of the SCell i where i is the ascending order of the SCellIndex among the SCells configured as on-demand SSB SCell. The Oi field is set to 1 to indicate that the on-demand SSB transmission of the corresponding SCell is deactivated, otherwise, the MAC entity shall ignore the Oi field.

Proposal 6: OD-SSB deactivation MAC CE includes bitmap with each bit corresponding to an OD-SSB SCell in ascending order.

2.2 RRC impact

No matter of which scenario and which trigger to be supported, UE should be informed of whether a SCell is a normal SCell or on-demand SSB SCell. This is because without this information, UE may derive incorrect measurement results with the assumption that the SCell is normal SCell and SSB is transmitted. It may cause problems on RRM procedure as the accuracy of measurement may be significantly degraded. Therefore, we think upon SCell addition, NW should indicates whether the SSB transmission on the SCell is on-demand manner.

Proposal 7: Upon SCell addition, NW indicates whether the SSB transmission on this SCell is on-demand manner via RRC signalling.

2.3 L3 RRM measurement

2.3.1 Impact on SMTC

In last RAN2 meeting, we discussed the impact of OD-SSB on L3 RRM measurement, there are two candidate solutions as listed below. In addition, RAN4 sent LS [3] to request RAN2 to discuss the impact on SMTC due to SSB adaptation, which is quite similar as case-2 of OD-SSB.

- Option1: Based on different SMTC configuration when OD-SSB is transmitted

- Option2: Based on OD-SSB pattern ignoring SMTC when OD-SSB is transmitted

For option 1, the RRM measurement is still based on SMTC, but a list of SMTC is configured/applied to OD-SSB with each SMTC associated with a periodicity.  However, considering the OD-SSB can be activated quite dynamically with different periodicities, to adjust the SMCT based on the indicated periodicity and to perform measurement based on adjusted SMTC would increase UE complexity especially if the periodicity is allowed to be updated after the activation of OD-SSB and whether there will be any delay on measurement due to this SMTC adjustment is not clear. In addition, with the introduction of the per periodicity SMTC, the measurement requirement defined in RAN4 may be impacted.

Observation 2: if OD-SSB is measured based on different SMTC, to perform measurement based on adjusted SMTC would increase UE complexity and may introduce delay on measurement as well as impact on RAN4.

Regarding option 2, considering the OD-SSB can be activated quite dynamically, it seems reasonable to not limit the measurement to a determined window, however, according to RAN4 specification, the requirement on RRM measurement is based on SMTC framework, if the measurement of OD-SSB is based on the transmission pattern, RAN4 needs to define totally new RRM measurement requirement not based on SMTC for OD-SSB, which would introduce significant impact on RAN4. Therefore, we think the feasibility/impact of this kind of fundamental change should be further checked with RAN4.

Observation 3: if OD-SSB is measured based on OD-SSB pattern ignoring SMTC, there would be significant impact on RRM measurement requirement defined in RAN4.

Based on the above analysis, if we need to down select between the two options, we slightly prefer option 2 which has less RAN2 impact and less UE complexity. In addition, we should send LS to ask RAN4 to evaluate the impact on RRM measurement requirement.

Proposal 8: For L3 measurement, RAN2 assumes the OD-SSB on the serving cell is measured based on OD-SSB pattern while ignoring SMTC. Send LS to RAN4 to ask RAN4 to evaluate the impact on RRM measurement requirement.

2.3.2 Which SSB to use for measurement

Another issue is which SSB to consider during RRM measurement for case 2 and two options were discussed last meeting.

- Option1: UE uses on-demand SSB for RRM measurements when they are transmitted, otherwise UE uses periodic SSB

- Option2: UE uses on-demand SSB for RRM measurements when they are transmitted in addition to periodic SSB for RRM measurements.

Actually, we think this issue is out of RAN2 scope and should be discussed by RAN4, actually based on the progress in RAN4, they already discussed about this issue but still has no conclusion.

Issue 1-4-5: Which type of SSB is used during SCell activation - Recommended WF - Based on moderator’s understanding, this issue can be summarized with the following options. - In Case 1, UE is required to always use OD-SSB for Scell activation. - In Case 2, - Option 1: the same as Case 1 - Option 2: UE needs to use both OD-SSB and always-on SSB for Scell activation. - Option 3: UE is assumed to use the SSB with the smaller periodicity.

Proposal 9: For case 2, it is up to RAN4 to discuss the SSB to be used for RRM measurement on SCell configured with OD-SSB.

2.3.3 Located frequency of OD-SSB

In addition, during the discussion, an issue on the frequency relationship between OD-SSB and always-on SSB was raised. RAN4 also sent LS to RAN1 and CC RAN2 to ask this question [4]. According to the RRC specification, for L3 measurement, for all SSB based measurements there is at most one measurement object with the same ssbFrequency, therefore if the OD-SSB and always-on SSB are on different frequencies, the network shall configure separate MO for the same SCell which would add complexity. In addition, this issue is more or less related to the discussion on which type of SSB is used for case 2, if both OD-SSB and always-on SSB are used but located on different frequencies, whether it is allowed and how to derive the per cell result based on measurement of SSB on different frequencies needs to be further discussed by RAN4.

The network applies the procedure as follows: - to ensure that, whenever the UE has a measConfig associated with a CG, it includes a measObject for the SpCell and for each NR SCell of the CG to be measured; - to configure at most one measurement identity across all CGs using a reporting configuration with the reportType set to reportCGI; - to configure at most one measurement identity per the node hosting PDCP entity using a reporting configuration with the ul-DelayValueConfig; - to configure at most one measurement identity per the node hosting PDCP entity using a reporting configuration with the ul-ExcessDelayConfig; - to ensure that, in the measConfig associated with a CG: - for all SSB based measurements there is at most one measurement object with the same ssbFrequency; - an smtc1 included in any measurement object with the same ssbFrequency has the same value and that an smtc2 included in any measurement object with the same ssbFrequency has the same value and that an smtc3list included in any measurement object with the same ssbFrequency has the same value and that an smtc4list included in any measurement object with the same ssbFrequency has the same value;

Considering the motivation to support OD-SSB and always-on SSB on different frequencies it not clear, we think from RAN2 perspective, we can conclude RAN2 assumes that OD-SSB and always-on SSB are on the same frequency, i.e., frequency of the associated SCell. Since RAN2 is also in the CC of RAN4 LS, we can send LS to RAN4/RAN1 to inform them this RAN2 assumption.

Proposal 10: For case2, RAN2 assumes the OD-SSB and always-on SSB are on the same frequency, i.e., frequency of associated SCell. Send LS to RAN1/RAN4 to inform this conclusion.

2.3.4 Neighbour cell measurement

For neighbour cells measurements, we think according to the WID, the impact on neighbour cell measurement with OD-SSB is out of scope. From RAN2 perspective, there is no need to discuss this aspect and we can assume the SSB status of neighbour cell is transparent to UE and how to handle the reported neighbour cell measurement is up to gNB implementation.

- Specify procedures and signaling method(s) to support on-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA. [RAN1/2/3/4] - Specify triggering method(s) (select from UE uplink wake-up-signal using an existing signal/channel, cell on/off indication via backhaul, Scell activation/deactivation signaling) - Note1: On-demand SSB transmission can be used by UE for at least SCell time/frequency synchronization, L1/L3 measurements and SCell activation, and is supported for FR1 and FR2 in non-shared spectrum.

Proposal 11: There is no impact on neighbour cell measurement with OD-SSB, e.g., the OD-SSB status of neighbour cell is transparent to UE.

3	Conclusion

In this contribution, we discussed the procedures and signalling to support on-demand SSB and have corresponding observations and proposals:

Observation 1: It introduces additional complexity on UE and/or NW if both the activation and deactivation status are indicated via the same MAC CE.

Observation 2: if OD-SSB is measured based on different SMTC, to perform measurement based on adjusted SMTC would increase UE complexity and may introduce delay on measurement as well as impact on RAN4.

Observation 3: if OD-SSB is measured based on OD-SSB pattern ignoring SMTC, there would be significant impact on RRM measurement requirement defined in RAN4.

RRC based activation/deactivation

Proposal 1: Deactivation of the OD-SSB transmission of the SCell by RRC can be supported by reconfiguration of the OD-SSB activation/deactivation state indication. No special specification effort is required.

MAC CE based activation/deactivation

Proposal 2: Define a configuration ID associated with at least periodicity and number N of OD-SSB bursts/timer of OD-SSB transmission.

Proposal 3: For each SCell with OD-SSB transmission activated, include the configuration ID in MAC CE.

Proposal 4: Introduce two MAC CEs to indicate the activation and deactivation of OD-SSB transmission separately.

Proposal 5: OD-SSB activation MAC CE includes explicit SCell index and the associated OD-SSB configuration ID.

Proposal 6: OD-SSB deactivation MAC CE includes bitmap with each bit corresponding to an OD-SSB SCell in ascending order.

RRC impact

Proposal 7: Upon SCell addition, NW indicates whether the SSB transmission on this SCell is on-demand manner via RRC signalling.

L3 RRM measurement

Proposal 8: For L3 measurement, RAN2 assumes the OD-SSB on the serving cell is measured based on OD-SSB pattern while ignoring SMTC. Send LS to RAN4 to ask RAN4 to evaluate the impact on RRM measurement requirement.

Proposal 9: For case 2, it is up to RAN4 to discuss the SSB to be used for RRM measurement on SCell configured with OD-SSB.

Proposal 10: For case2, RAN2 assumes the OD-SSB and always-on SSB are on the same frequency, i.e., frequency of associated SCell. Send LS to RAN1/RAN4 to inform this conclusion.

Proposal 11: There is no impact on neighbour cell measurement with OD-SSB, e.g., the OD-SSB status of neighbour cell is transparent to UE.

4	Reference

[1] RAN1#118bis Chair notes.

[2] RAN2#127bis meeting minutes

[3] R4-2416911

[4] R4-2416913