3GPP TSG RAN WG2 Meeting #129							R2-2500099

Athens, Greece, February 17th – 22nd, 2025

Agenda item:	8.7.3

Title:			RRM measurement gaps/restrictions related enhancements

Source:		Fraunhofer IIS, Fraunhofer HHI

Document for:	Discussion

# Introduction

In the RAN2#128 meeting, the following agreements have been reached on RRM measurement gaps/restrictions related enhancements for XR services [1]:

Agreements on RRM measurement gaps impacts - No MG-specific enhancements is needed on DSR operation. - RAN2 assumes that UE follows DRX pattern as currently, even when MG is indicated as skipped - No MG-specific enhancements is needed for DRX operation.

In this contribution, we provide our views on the need for semi-static solutions for supporting the solution chosen by RAN1 to enable transmission and reception during RRM measurements.

# Discussion

According to the agreements in RAN1 #118 meeting [2], explicit DCI-based signalling was proposed as a working assumption to indicate when a measurement gap is cancelled. This solution clearly aims at enhancing capacity, and it is currently not clear how to limit the impact of RRM measurement relaxation on UE mobility performance. These two aspects are explicitly mentioned in the Work Item Description [3] and the final solution should strive to achieve a trade-off between these two contradictory objectives.

Observation 1:	The DCI-based solution adopted by RAN1 aims at providing capacity improvements. However, it cannot guarantee a limited impact on UE mobility performance.

Given this context, it seems to be beneficial to complement RAN1’s DCI-based solution with another, semi-static, solution. Here, the term “complement” should be understood as a logical “OR”. It assumes that the semi-static is always “on”, and that the DCI-based solution only comes on top of it. In other words, they run at the same time though on two totally distinct subsets of occasions of gaps, rather than they cover different XR traffic cases. For instance, some collisions between XR traffic and gaps may be missed by the (pre-)configured pattern of the semi-static solution, but this can be “corrected” dynamically by the DCI-based solution.

The more gaps are indicated dynamically by DCI signaling, the more aggressive the proposed solution is in terms of capacity enhancements, though at the cost of more occasions of gaps being cancelled and therefore a degradation of the RRM performance. In one extreme case, all the gaps not indicated in the semi-static pattern can be indicated by DCI signaling, therefore resulting in 100% of the gaps being cancelled. Conversely, a more conservative approach is to have no occasion of gaps indicated by DCI signaling, i.e., only the ones indicated by the semi-static pattern are cancelled. With our proposal, the emphasis is then to guarantee the RRM performance, while still some capacity improvements can be achieved.

Proposal 1:	Support a combination of semi-static and DCI-based solutions to enable TX/RX in gaps caused by RRM measurements. The combination is a logical “OR”.

For the semi-static solution, a pattern can be provided by RRC Reconfiguration to the UE to indicate which gaps are cancelled. This pattern continues until the next RRC Reconfiguration. Since it is very difficult (or unrealistic) to predict the pattern the UE should follow so far in the future, a solution could be to simply assume a semi-static periodic pattern irrespective of the XR traffic/channel conditions/mobility/traffic. For example, the pattern can be such that:

- X occasions of gaps out of Y are cancelled,

- T consecutive gaps out of Y (with Y greater than T) are cancelled.

In that case, the signaling would consist simply of two numbers, namely “X” (or “T”) and “Y”, and there is thus no need to provide the complete bitmap to the UE in the RRC signaling. The ratio X/Y (or T/Y) corresponds to the minimum ratio of occasions of gaps that can be cancelled to guarantee a certain level of RRM performance, which can be left up to RAN4 to define.

The semi-static pattern can be understood as operating over the long-term to enable transmission and reception in gaps caused by RRM measurements, such that it does not depend on XR traffic, channel conditions, and/or mobility. Hence, it is not necessary to update the values of X (or T) and Y, which can be (pre-)configured by the network and provided to the UE by means of RRC Reconfiguration, whose timeline is not critical.

Proposal 2:	The semi-static pattern can be kept simple, i.e., periodic, and holds until the next RRC Reconfiguration.

On top of the RRC-based semi-static pattern, the DCI signaling is used to indicate dynamically gaps that can be cancelled. The dynamic signaling can only apply to gaps which are not already indicated as cancelled by the semi-static pattern. Hence, gaps which are already indicated as cancelled by the semi-static pattern should remain cancelled, in agreement with the RAN1 solution.

Proposal 3:	The dynamic cancellation applies only to gaps that are not already indicated as cancelled by the semi-static pattern.

# Summary

In this contribution, we discussed some enhancements to enable transmission/reception for XR during RRM measurements. The following lists the observations and proposals made in the contribution:

Observation 1:	The DCI-based solution adopted by RAN1 aims at providing capacity improvements. However, it cannot guarantee a limited impact on UE mobility performance.

Proposal 1:	Support a combination of semi-static and DCI-based solutions to enable TX/RX in gaps caused by RRM measurements. The combination is a logical “OR”.

Proposal 2:	The semi-static pattern can be kept simple, i.e., periodic, and holds until the next RRC Reconfiguration.

Proposal 3:	The dynamic cancellation applies only to gaps that are not already indicated as cancelled by the semi-static pattern.

# References

- Draft “Report of 3GPP TSG RAN WG2 meeting #128,” 3GPP RAN2 #128, Orlando (USA), Nov. 2024.

- R1-2407587, “Final Report of 3GPP TSG RAN WG1 #118 v1.0.0,” 3GPP RAN1 #118b, Hefei (China), Oct. 2024.

- RP-243318, “Revised WID on XR (eXtended Reality) for NR Phase 3,” Madrid (Spain), Dec. 2024.