3GPP TSG-RAN WG2 Meeting #129                               		R2- 2500109

Athens, Greece, Feb. 17th – 21nd, 2025

Agenda Item:		8.11.2

Source:				Charter Communications

Title:					SBFD Configuration for initial random access and operations

WID/SID:	NR_duplex_evo-Core

Document for:		Discussion and decision

# Introduction

Sub-Band Full duplex (SBFD) WID [1] highlights the core objective of specifying SBFD operation at the gNB side within a TDD carrier. Some of the objectives of enhancing SBFD operation include time/frequency resource location indication, support for random access in SBFD symbols and handling gNB-to-gNB CLI and UE-to-UE CLI. Latency is one of the important KPIs of SBFD enhancements. In RAN2#128, as regards to RACH occasion (RO) selection for random access, RAN2 made progress and reached the following agreements:

CFRA

The RO type is indicated by NW for CFRA. FFS on signaling (can FFS for the SI request case if needed).

CBRA

FFS on the following options

Option 1

Upon initiation of RACH procedure for a SBFD-aware UE, network provides the indication on the prioritization of the additional ROs over legacy RO.

If there is no such indication from the NW, FFS on the following mechanism

- UE select legacy RO or SBFD RO based on SSB RSRP, or

- UE select the legacy RO, or

- UE select the SBFD RO, or

- Other metrics than SSB RSRP.

Option 2

UE select legacy RO or SBFD RO based on SSB RSRP if such condition is configured, and if not configured, then UE can prioritize one type of the ROs, FFS which one.

RACH configuration

Only one RACH configuration option (i.e., either RACH configuration Option 1 with Alt 1-1 or RACH configuration Option 2) is supported in a cell.

RAN2 agreed that the RO type in CFRA is indicated by the NW. However, for CBRA in SBFD symbols, agreement needs to be made on the prioritization of the additional ROs over legacy ROs. Option 1 is for NW to provide the indicator of the prioritization upon initial access of RACH procedure for SBFD-aware UE. Option 2 is for UE to select legacy RO or SBFD (additional) RO based on SSB RSRP if configured and if not configured, UE can prioritize one type of the ROs.

In this contribution, we discuss our observations on Option 1 and Option 2 for the CBRA RO selection at initial access. And suggest that RAN2 considers UE SBFD capability indicator that will allow UE to initiate CBRA (RACH procedure) using SBFD symbols if the UE knows it is SBFD-aware.

# Discussion

Since one of the KPIs targeted by SBFD enhancements is latency, Option 1 could impact latency if the UE needs to wait for the NW to provide an indication of which RO to select for initial access. This latency KPI could be improved if the SBFD (additional) RO is configured as the default or initial RO for SBFD-aware UE – allowing SBFD-aware UE to perform initial random access, and if RA fails, the UE can fall back on the legacy RO.

Observation 1: Waiting for NW to provide RO prioritization indicator could negatively impact latency target of SBFD.

Proposal 1: RAN2 to consider additional RO as default RO for SBFD-aware UE for initial RACH.

If RO prioritization indicator is not configured by the NW, UEs that are self-aware of their SBFD capability, can select legacy or SBFD RO based on the SSB RSRP as suggested in RAN2#128. This is similar to Option 2 where UE selects one type of RO based on SSB RSRP and if such condition is not configured, UE decides to prioritize one type of RO. However, which RO to select is not decided yet, but UE needs to know if it is configured to select SBFD RO whenever possible.

Observation 2: If RO prioritization indication is not configured, UE is unaware it could select additional RO based on SSB RSRP.

Observation 3: If RO selection based on SSB RSRP condition is not configured, UE is unaware of its SBFD capability to select additional RO.

Proposal 2: RAN2 to discuss how SBFD-aware UEs know to select additional RO when SSB RSRP condition or prioritization indication is not configured.

As a result of Observations 2 and 3, RAN2 should discuss if there is need to include SBFD capability information element in the UE NR capability information list, which helps the UE know that it is SBFD capable or aware and can use SBFD resources in SBFD symbols if prioritization indication and/or SSB RSRP condition are configured.

sbfd-CapabilitySupport-r19

Figure 1: UE Capabilities Enquiry [TS 38.331, 5.6.1.3]

Consequently, the SBFD-aware UEs can be configured with SBFD UE capability (by defining an IE like sbfd-CapabilitySupport-R19 or something similar) to enable the UE to become aware of its SBFD capability and can select additional RO whenever possible. Similarly, gNB may inquire SBFD UE capability via some UE capability enquiry and information as shown in Figure 1 from TS 38.331. This will allow the gNB to confidently configure the UE with some SBFD resources via TDD-UL-DL-ConfigCommon for instance.

Proposal 3: RAN2 to discuss configuring SBFD capability indicator in UE implementation.

# Conclusion

In this contribution, we have made the following observation and proposals for discussion and decision in RAN2:

Observation 1: Waiting for NW to provide RO prioritization indicator negatively impacts latency target of SBFD.

Proposal 1: RAN2 to consider additional RO as default RO for SBFD-aware UE for initial RACH.

Observation 2: If RO prioritization indication is not configured, UE is unaware it could select additional RO based on SSB RSRP.

Observation 3: If RO selection based on SSB RSRP condition is not configured, UE is unaware of its SBFD capability to select additional RO.

Proposal 2: RAN2 to discuss how SBFD-aware UEs know to select additional RO when SSB RSRP condition or prioritization indication is not configured.

Proposal 3: RAN2 to discuss configuring SBFD capability indicator in UE implementation.

# References

- RP‑241614, Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD)

- RP-242034, “Evolution of NR duplex operation: Sub-band full duplex (SBFD),” Status report, Huawei, Samsung