3GPP TSG-RAN WG2 Meeting #129                               R2-2500110

Athens, Greece, Feb. 17th – 21st, 2025

Agenda Item:		8.11.3

Source:				Charter Communications

Title:					Cross link interference resource configuration in SBFD

WID/SID:	NR_duplex_evo-Core – Release 19

Document for:		Discussion and decision

# Introduction

The WID in [1] describes the work item on the evolution of NR duplex operation – Subband Full Duplex (SBFD) with the primary objective of specifying SBFD operation for TDD carrier. One of the core objectives is to specify the semi-static indication of time and frequency domain resources for SBFD-aware UEs and cross link interference (CLI) handling in SBFD symbols – specifically CLI measurement for gNB-to-gNB and UE-to-UE CLI handling.

In RAN2#128, RAN2 made the following agreements related to SBFD cell selection and CLI measurement:

Prioritization of SBFD cells / frequencies during cell reselection is not considered.

RAN2 wait for input from the other WGs regarding whether for inter-cell CSI-RS measurements, UE needs to be provided with information of the SBFD configuration of neighbouring cells.

So far in RAN2 and based on our understanding, no agreement has been reached on the CLI resource and reporting configuration for CLI measurement. In this contribution, aspects related to CLI resource and report configuration are discussed.

# Discussion

In RAN1#118bis, RAN1 agreed that CSI-ResourceConfig will include two CLI measurement resoure set lists for SRS-RSRP ad CLI-RSSI measurements based on Rel-16 SRS-ResourceConfigCLI and rssi-ResourceConfigCLI defined in MeasObjectCLI for L3 based SRS-RSRP and CLI-RSSI measurement. In line with this RAN1 agreement, RAN2 needs to discuss the CLI resource configuration and such configuration could consist of similar components (resource set, reporting set, and measurement/reporting triggers) in the legacy CSI resource configuration and reporting framework.

- CLI Resource Configuration

Similar to legacy CSI resource configuration and reporting framework, and by RAN1’s agreement, the CLI resource configuration for CLI measurement in SBFD symbols will consist of two resource lists for SRS-RSRP and CLI-RSSI. Based on our understanding, to measure UE-to-UE CLI and gNB-to-gNB CLI, the RS for CLI needs to be flexible and less complex in signalling to connected UE in the case of UE-to-UE or neighbouring gNBs in the case of gNB-to-gNB CLI measurement. Therefore, the RS should consist of a sequence of sub-regions in time and frequency – the time resource allocation for CLI should be flexible to indicate any starting OFDM symbol, number of consecutive OFDM symbols or list of OFDM symbols containing the CLI resources in time domain. For frequency domain, the flexibility should indicate the starting REs, number of REs, and comb pattern.

Proposal 1: RAN2 to consider CLI resource configuration with flexible time and frequency indication.

Based on the above resource allocation requirements, in Annex A, we define the ASN.1 signalling to be discussed in RAN2 as the baseline for CLI RS resource configuration. This new CSI-RS configuration CSI-SBFD-IM-Resource-r19 is signalled to the UEs via RRCReconfigiration.

Proposal 2: RAN2 to discuss CLI RS resource configuration in Annex A as the baseline.

In addition, UE-to-UE interference measurement could also be performed using ZP CSI-RS resources. Such configured ZP CSI-RS for SBFD CLI could be used for measurement within a gNB. A new ZP CSI-RS resource mapping for SBFD CLI is added to enhance the legacy ZP CSI RS (ZP-CSI-RS-Resource) configuration as presented in Annex B. The ZP-CSI-RS-SBFD-ResourceMapping-r19 configuration will be signalled to the UE via PDSCH-Config, which is carried by BWP-DownlinkDedicated. A trigger indicator (zp-SBFD-csi-RS-Trigger-r19) is added to this configuration to allow only SBFD-aware UEs to apply this resource configuration, and non-SBFD UEs using the legacy ZP-CSI-RS-Resource can ignore the SBFD CLI ZP-CSI-RS.

Proposal 3: RAN2 to discuss using legacy ZP-CSI-RS-Resource with the addition of SBFD CLI ZP-CSI-RS for SBFD-aware UEs to perform UE-to-UE CLI measurement.

- CLI Measurement and Reporting Triggers

By adapting the legacy CSI measurement/reporting framework, NW could trigger CSI reporting based on the resource type. For Semi-Persistent resource configuration, existing SP CSI-RS/CSI-IM MAC CE (see TS 38.321, Section 6.1.3.12) could be re-used as presented in Figure 1, by adding a new bit flag (SBFD-IM) in bit-7 of Octet 2. If this SBFD-IM bit is set to 1, UE-to-UE CLI measurement is performed using the resource Set ID indicated by the “SP CSI-IM resource set ID” in the MAC CE.

SBFD-IM

Figure 1. Semi-Persistent CSI-RS/CSI-IM Set Activation/Deactivation MAC CE [TS 38.321, Section 6.1.3.12]

Proposal 4: RAN2 to consider using reserved-bit in legacy MAC CE to trigger UE-to-UE CLI measurement reporting for semi-persistent resource type.

Periodic SBFD CLI measurement may not be needed because a gNB can trigger gNB-to-gNB CLI measurement if it is having difficulty in receiving UL transmissions in SBFD symbols/slots. This CLI measurement may be needed if the gNB is around an aggressor gNB. Similarly, a gNB can trigger a UE-to-UE CLI measurement if the UEs are having difficulty in receiving DL transmissions – this means that this kind of measurement is on a need basis, hence, it may not require periodic CLI measurements. Enabling periodic measurement may use up resources in the SBFD symbols/slots for CLI measurements, rather than periodic, aperiodic and semi-persistent can be considered.

Observation 1: Supporting periodic SBFD CLI measurement may be resource intensive to perform across subbands.

Observation 2: Supporting aperiodic and semi-persistent SBFD CLI measurements may be sufficient.

Proposal 5: RAN2 to consider prioritizing aperiodic and semi-persistent measurements over periodic CLI measurement in SBFD symbols/slots.

For Semi-Persistent ZP CSI-RS resource configuration, the legacy “SP ZP CSI-RS” MAC CE (see Section 6.1.3.19, TS 38.321) could be re-used to support SBFD CLI measurement report as presented in Figure 2. If A/D is set to 1, activate the ZP CSI-RS resource set ID in SP ZP CSI-RS resource set ID to correspond to the SBFD ZP-CSI-RS resource configuration mapping to ZP-CSI-RS-SBFD-ResourceMapping-r19.

Figure 2. SP ZP CSI-RS Set Activation/Deactivation MAC CE [TS 38.321, Section 6.1.3.19]

Proposal 6: RAN2 to support using legacy “SP ZP CSI-RS” MAC CE for SBFD CLI report trigger using ZP CSI-RS resource set.

For aPeriodic resource configuration type, the legacy “ZP CSI-RS Trigger” field in DCI Format 1_1 can be used to trigger CLI measurement/reporting. Currently, “ZP CSI-RS Trigger” supports up to three (3) resource sets for aPeriodic measurement and one solution is to expand the 3 resource sets to include the resource set for SBFD measurement or indicate that via the zp-SBFD-csi-RS-Trigger-r19 flag as defined in Annex B.

Proposal 7: RAN2 to agree on re-using and expanding “ZP CSI-RS Trigger” field in DCI Format 1_1 for aperiodic SBFD CLI reporting.

Proposal 8: If “ZP CSI-RS Trigger” field in DCI Format 1_1 is not expanded, CLI measurement can be triggered using the zp-SBFD-csi-RS-Trigger-r19 flag as defined in Annex B.

# Conclusions

We have discussed possible resource configuration and report trigger options for CLI measurement in SBFD:

Proposal 1: RAN2 to consider CLI resource configuration with flexible time and frequency indication.

Proposal 2: RAN2 to discuss CLI RS resource configuration in Annex A as the baseline.

Proposal 3: RAN2 to discuss using legacy ZP-CSI-RS-Resource with the addition of SBFD CLI ZP-CSI-RS for SBFD-aware UEs to perform UE-to-UE CLI measurement.

Proposal 4: RAN2 to consider using reserved-bit in legacy MAC CE to trigger UE-to-UE CLI measurement reporting for semi-persistent resource type.

Observation 1: Supporting periodic SBFD CLI measurement may be resource intensive to perform across subbands.

Observation 2: Supporting aperiodic and semi-persistent SBFD CLI measurements may be sufficient.

Proposal 5: RAN2 to consider prioritizing aperiodic and semi-persistent measurements over periodic CLI measurement in SBFD symbols/slots.

Proposal 6: RAN2 to support using legacy “SP ZP CSI-RS” MAC CE for SBFD CLI report trigger using ZP CSI-RS resource set.

Proposal 7: RAN2 to agree on re-using and expanding “ZP CSI-RS Trigger” field in DCI Format 1_1 for aperiodic SBFD CLI reporting.

Proposal 8: If “ZP CSI-RS Trigger” field in DCI Format 1_1 is not expanded, CLI measurement can be triggered using the zp-SBFD-csi-RS-Trigger-r19 flag as defined in Annex B.

# References

- RP‑241614, Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD)

- RP-242034, “Evolution of NR duplex operation: Sub-band full duplex (SBFD),” Status report, Huawei, Samsung

# Annex A – CSI-IM CLI Resource Configuration

CSI-ResourceConfig ::=      SEQUENCE {

csi-ResourceConfigId        CSI-ResourceConfigId,

csi-RS-ResourceSetList      CHOICE {

nzp-CSI-RS-SSB              SEQUENCE {

nzp-CSI-RS-ResourceSetList  SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourceSetsPerConfig)) OF NZP-CSI-RS-ResourceSetId

OPTIONAL, -- Need R

csi-SSB-ResourceSetList     SEQUENCE (SIZE (1..maxNrofCSI-SSB-ResourceSetsPerConfig)) OF CSI-SSB-ResourceSetId  OPTIONAL  -- Need R

},

csi-IM-ResourceSetList      SEQUENCE (SIZE (1..maxNrofCSI-IM-ResourceSetsPerConfig)) OF CSI-IM-ResourceSetId

},

bwp-Id                      BWP-Id,

resourceType                ENUMERATED { aperiodic, semiPersistent, periodic },

...,

[[

csi-SSB-ResourceSetListExt-r17      CSI-SSB-ResourceSetId                                                  OPTIONAL  -- Need R

]],

csi-SBFD-IM-ResourceSetList SEQUENCE (SIZE(1..maxNrofCSI-SBFD-IM-ResourceSetsPerConfig)) OF CSI-SBFD-IM-ResourceSetId,

csi-SBFD-IM-Trigger ENUMERATED{true}

}

CSI-SBFD-IM-ResourceSetId INTEGER ::= (0..maxNrofCSI-SBFD-IM-ResourceSets-1)

maxNrofCSI-SBFD-IM-ResourceSets INTEGER ::= 8 -- Maximum number of CSI-SBFD-IM-ResourceConfig-r19 resources per set

CSI-SBFD-IM-ResourceSet-r19 ::= SEQUENCE {

csi-SBFD-IM-ResourceSetId CSI-SBFD-IM-ResourceSetId,

csi-SBFD-IM-Resource-r19 SEQUENCE (SIZE(1..maxNrofCSI-SBFD-IM-ResourcesPerSet)) OF CSI-SBFD-IM-ResourceId-r19,

...

}

CSI-SBFD-IM-Resource-r19 ::= SEQUENCE {

csi-SBFD-IM-ResourceId              CSI-SBFD-IM-ResourceId-r19,

csi-SBFD-IM-ResourceConfig-r19	    CSI-SBFD-IM-ResourceConfig-r19

freqBand		    CSI-FrequencyOccupation 	            OPTIONAL, -- Need M

csi-SBFD-IM-periodicityAndOffset    CSI-ResourcePeriodicityAndOffset 	OPTIONAL, -- Cond PeriodicOrSemiPersistent

}

CSI-SBFD-IM-ResourceConfig-r19 ::= SEQUENCE {

nrofSBFDIMResourceElementPattern           INTEGER (0..maxNrofSBFDIMResourceElementPattern-1)

csi-SBFD-IM-Comb	ENUMERATED{1, 2, 4}

csi-SBFD-IM-ResourceElementPattern       CHOICE {

pattern0      SEQUENCE {

slotSpecificConfigPattern-p0    SEQUENCE ( SIZE (1..maxNrofSlots)) OF TDD-UL-DL-SlotIndex

symbolLocationConfigPattern-p0  BIT STRING (SIZE(14))

startingRB	     INTEGER (0..maxNrofPhysicalResourceBlocks),

nrofRBs	     INTEGER (4..maxNrofPhysicalResourceBlocksPlus1),

startingRE      INTEGER (0..maxNrofResourceElementsMinus12),

nrofREs         INTEGER (4..maxNrofResourceElements),

},

pattern1      SEQUENCE {

slotSpecificConfigPattern-p1    SEQUENCE ( SIZE (1..maxNrofSlots)) OF TDD-UL-DL-SlotIndex

symbolLocationConfigPattern-p1  BIT STRING (SIZE(14))

startingRB	     INTEGER (0..maxNrofPhysicalResourceBlocks),

nrofRBs	     INTEGER (4..maxNrofPhysicalResourceBlocksPlus1),

startingRE      INTEGER (0..maxNrofResourceElementsMinus12),

nrofREs         INTEGER (4..maxNrofResourceElements),

},

}

}

# Annex B – ZP-CSI-RS CLI Resource Configuration

ZP-CSI-RS-Resource ::=              SEQUENCE {

zp-CSI-RS-ResourceId                ZP-CSI-RS-ResourceId,

resourceMapping                     CSI-RS-ResourceMapping,

zp-SBFD-csi-RS-Trigger-r19   ENUMERATED {true}

zpCSISBFDResourceMapping-r19 ZP-CSI-RS-SBFD-ResourceMapping-r19

periodicityAndOffset                CSI-ResourcePeriodicityAndOffset                OPTIONAL, --Cond PeriodicOrSemiPersistent

...

}

ZP-CSI-RS-SBFD-ResourceMapping-r19 ::= SEQUENCE {

frequencyDomainAllocation CHOICE {

row1 BIT STRING (SIZE (4)),

row2 BIT STRING (SIZE (12)),

row4 BIT STRING (SIZE (3)),

other BIT STRING (SIZE (6))

},

nrofPorts ENUMERATED {p1,p2,p4,p8,p12,p16,p24,p32},

nrofSBFDIMResourceElementPattern         INTEGER (0..maxNrofSBFDIMResourceElementPattern-1)

csi-SBFD-IM-ResourceElementPattern       CHOICE {

pattern0                     SEQUENCE {

slotSpecificConfigPattern-p0    SEQUENCE ( SIZE (1..maxNrofSlots)) OF TDD-UL-DL-SlotIndex

symbolLocationConfigPattern-p0  BIT STRING (SIZE(14))

startingRB	     INTEGER (0..maxNrofPhysicalResourceBlocks),

nrofRBs	     INTEGER (4..maxNrofPhysicalResourceBlocksPlus1),

startingRE            INTEGER (0..maxNrofResourceElementsMinus12),

nrofREs               INTEGER (4..maxNrofResourceElements),

},

pattern1                     SEQUENCE {

slotSpecificConfigPattern-p1    SEQUENCE ( SIZE (1..maxNrofSlots)) OF TDD-UL-DL-SlotIndex

symbolLocationConfigPattern-p1  BIT STRING (SIZE(14))

startingRB	     INTEGER (0..maxNrofPhysicalResourceBlocks),

nrofRBs	     INTEGER (4..maxNrofPhysicalResourceBlocksPlus1),

startingRE            INTEGER (0..maxNrofResourceElementsMinus12),

nrofREs               INTEGER (4..maxNrofResourceElements),

},

}

cdm-Type ENUMERATED {noCDM, fd-CDM2, cdm4-FD2-TD2, cdm8-FD2-TD4},

density CHOICE {

dot5 ENUMERATED {evenPRBs, oddPRBs},

one NULL,

three NULL,

spare NULL

},

freqBand CSI-FrequencyOccupation,

...

}