3GPP TSG-RAN WG2 Meeting #129                                                  	 R2-2500121

Athens, Greece, Feb. 17th – 21st, 2025

Agenda Item:	8.1.2.2 LCM for UE-sided model for Beam Management use case

Source:	NEC

Title:	Discussion on LCM for UE sided model

Document for:	Discussion, Decision

Introduction

RAN2 has discussed some issues related to functionality reporting and LCM configuration. In this contribution, we will further investigate the remaining issue and some additional issues related to functionality training and monitoring.

Discussion

RAN1 sent LS reply for RAN2 LS on applicable functionality reporting for beam management UE-sided model in [1].

Firstly, we would like to focus on concept/terminologies of “supported functionalities”, “applicable functionalities” and “activated functionalities”. RAN1 and RAN2 understanding of these three functionalities in [1] and [2] are shown below.

Table 1 Concept/terminology of supported/applicable/activated functionalities in RAN1 and RAN2

|  | Supported functionalities | Applicable functionalities | Activated functionalities |
| --- | --- | --- | --- |
| RAN2 understanding stated in [1] | may refer to UE-capability information/parameters i.e., Rel-19 AI/ML-enabled Features/FGs | may refer to CSI-ReportConfig for inference configuration or a set of inference related parameters | may be enabled based on CSI framework |
| RAN2 understanding stated in [2] | refer to functionalities that UE can indicate by using UE capability information (via RRC/LPP signalling) | refer to functionalities that the UE is ready to apply for inference | refer to functionalities already enabled for performing inference |

RAN2 understanding of supported/applicable/activated functionalities are based on the same meaning and granularity of “functionality”. However, in RAN1 understanding, the meaning and the granularity of “functionality” for “Supported functionalities”, “Applicable functionalities” and “Activated functionalities” may not be the same. For instance, the meaning/granularity of “functionality” for “Supported functionalities” is Rel-19 AI/ML-enabled features/feature groups, while the meaning/granularity of “functionality” for “Applicable functionalities” is CSI-ReportConfig for inference configuration or a set of inference related parameters and the meaning/granularity of “functionality” for “activated functionalities” is based on CSI framework (e.g., the full configuration including AI/ML report and performance monitoring configurations.).

The meaning and the granularity of “functionality” for “Supported functionalities”, “Applicable functionalities” and “Activated functionalities” may not be the same.

Main RAN1 agreements related to the Questions in RAN2 LS [2] are copied below:

Agreement - In Step 3, following configurations are provided from NW to UE: - UE is allowed to do UAI reporting via OtherConfig, - The applicability report is based on A) and/or B) - It is up to RAN 2 to design the container - A) one or more of CSI-ReportConfig for inference configuration (wherein the associated ID may be configured in CSI framework as working assumption applied) - Note: CSI report configuration for UE-side model inference can’t be activated immediately upon receiving Step 3 - B) One set or multiple sets of inference related parameters for applicability report only (not for inference) - It is up to RAN2 to design the container. - The set of inference related parameters selected from the IEs in/or the IEs referred by CSI-ReportConfig as a starting point, e.g., - the associated ID - Note: this doesn’t imply the associated ID is mandatory - Set A related information - Set B related information - Report content related information - For BM-Case 2, - Time instances related information for measurements - Time instances related information for prediction - In Step 4, UE reports applicability for all the above A) one or more CSI-ReportConfig and/or B) set(s) of inference related parameters - FFS on whether/what other information along with the applicability is needed - If A) is configured in Step 3, - Applicable aperiodic CSI Report and semi-persistent CSI report can be activated/triggered by NW after the applicability reported. - Applicable periodic CSI Report is considered as activated only if the applicability of the corresponding CSI-ReportConfig is reported in RRCReconfigurationComplete. - In Step 5, NW can optionally configure CSI-ReportConfig for inference configuration in RRCReconfiguration, where the associated ID may be configured in CSI framework as working assumption applied. - Note: Step 5 may be optional if UE has already been configured with CSI-ReportConfig in Step 3 Agreement For beam management, multiple CSI reports for inference for UE-side model can be configured/activated/triggered, which is up to UE capability. Conclusion For the CSI-ReportConfig for inference configuration provided in Step 5, - aperiodic CSI Report and semi-persistent CSI report can be activated/triggered by NW after RRCReconfigurationComplete. - periodic CSI Report is considered as activated after RRCReconfigurationComplete. - Note: UE is not expected to be configured with a CSI-Reapportioning for inference configuration for a non-applicable set of inference parameters or a non-applicable CSI-ReportConfig - Any specification impact is a separate discussion

Two mechanisms, naming A) and B) were agreed in RAN1 for providing inference configurations in Step 3.  Our understanding is that there is no further down-selection between A) and B), i.e., both mechanism A) and B) should be supported.

Two mechanisms, naming A) and B) agreed in RAN1 for providing inference configurations in Step 3 should be supported.

We will discuss mechanism A) and B) one by one.

Mechanism A:

Step 3: NW provides one or more of CSI-ReportConfig for inference configuration (wherein the associated ID may be configured in CSI framework as working assumption applied)

Note: CSI report configuration for UE-side model inference can’t be activated immediately upon receiving Step 3

Step 4: UE reports applicability for one or more CSI-ReportConfig provided in Step 3

If the CSI-ReportConfig provided in Step 3 is aperiodic CSI Report or semi-persistent CSI report, after the applicability reported, it can be activated/triggered by NW.

If the CSI-ReportConfig provided in Step 3 is periodic CSI Report, it is considered as activated only if the applicability of the corresponding CSI-ReportConfig is reported in RRCReconfigurationComplete.

Step 5: NW can optionally configure CSI-ReportConfig for inference configuration in RRCReconfiguration, where the associated ID may be configured in CSI framework as working assumption applied.

Note: Step 5 may be optional if UE has already been configured with CSI-ReportConfig in Step 3.

For the CSI-ReportConfig for inference configuration provided in Step 5,

If the CSI-ReportConfig provided in Step 5 is aperiodic CSI Report or semi-persistent CSI report, it can be activated/triggered by NW after RRCReconfigurationComplete.

If the CSI-ReportConfig provided in Step 5 is periodic CSI Report, it is considered as activated after RRCReconfigurationComplete.

Note: UE is not expected to be configured with a CSI-ReportConfig for inference configuration for a non-applicable set of inference parameters or a non-applicable CSI-ReportConfig

o	Any specification impact is a separate discussion

According to RAN1 LS reply [1], when mechanism A) is applied (i.e., one or more of CSI-ReportConfig for inference configuration is provided in Step 3), UE is only requested to report applicability of the corresponding CSI-ReportConfig for responding to Step 3.

Since RRCReconfiguration message containing CSI-ReportConfig are used for both Step 3 and optional Step 5, one potential issue is how UE to differentiate whether the receiving RRCReconfiguration message containing CSI-ReportConfig is Step 3 or Step 5. Moreover, the NW may enquire applicability of different CSI-ReportConfig for inference configuration one by one, then an RRCReconfiguration message may act as Step 3 for CSI-ReportConfig A while also acting as Step 5 for CSI-ReportConfig B. One potential solution may be to always report applicability upon receiving RRCReconfiguration message containing CSI-ReportConfig regardless of whether it is in Step 3 or not. However, such a way makes Step 3 and Step 4 in mechanism B invalid eventually (details of mechanism B can refer to the related context). It goes against the RAN1 agreement to support both mechanism A and mechanism B.

To address this issue, we propose explicitly indicating whether an applicability report is requested for each CSI-ReportConfig for inference configuration within the RRCReconfiguration message. For example, NW can provide the activation status indication associated with each CSI-ReporConfig for inference configuration in RRCReconfiguration message. The UE only reports applicability in RRCReconfigurationComplete for those CSI-ReportConfig whose status is indicated as “deactivated”.

Support explicitly indicating whether an applicability report is requested for each CSI-ReportConfig for inference configuration within the RRCReconfiguration message.

Upon receiving RRCReconfiguration message containing CSI-ReportConfig, the UE only reports applicability in RRCReconfigurationComplete for those CSI-ReportConfig whose status is indicated as “deactivated”.

Mechanism B:

Step 3: NW provides one set or multiple sets of inference related parameters for applicability report only (not for inference)

Step 4: UE reports applicability for all set(s) of inference related parameters.

Step 5: NW configures CSI-ReportConfig for inference configuration in RRCReconfiguration, where the associated ID may be configured in CSI framework as working assumption applied.

For the CSI-ReportConfig for inference configuration provided in Step 5,

If the CSI-ReportConfig provided in Step 5 is aperiodic CSI Report or semi-persistent CSI report, it can be activated/triggered by NW after RRCReconfigurationComplete.

If the CSI-ReportConfig provided in Step 5 is periodic CSI Report, it is considered as activated after RRCReconfigurationComplete.

Note: UE is not expected to be configured with a CSI-Reapportioning for inference configuration for a non-applicable set of inference parameters or a non-applicable CSI-ReportConfig.

o	Any specification impact is a separate discussion

According to RAN1 LS reply [1], when mechanism B) is applied (i.e., one set or multiple sets of inference related parameters is provided in Step 3), UE is only requested to report applicability for set(s) of inference related parameters.

- Even though the RRCReconfiguration message are used for Step 3 and Step 5 in mechanism B), the contents are different, i.e., in Step 3 RRCReconfiguration message contains one set or multiple sets of inference related parameters whereas in Step 5 RRCReconfiguration message contains CSI-ReportConfig. There seems no issue on differentiating Step 3 and Step 5 from UE perspective.

- In mechanism B, the set of inference related parameters is still open. RAN1 LS reply [1] agreed to select from the IEs in/or the IEs referred by CSI-ReportConfig as a starting point, e.g.,

    - the associated ID

    - Set A related information

    - Set B related information

    - Report content related information

    - For BM-Case 2,

        - Time instances related information for measurements

        - Time instances related information for prediction

- Among above examples listed by RAN1, we think some information would be more essential. For example, limiting Set B to SSB may make the UEcapabilityInformation containing supported functionalities which can only be enabled with the CSI-RS-based Set B no meaningful. To avoid unnecessary applicable functionality reporting for beam management UE-sided model, RAN2 further discuss to include a part of essential information of the set of inference related parameters in Step 1 and/or Step 2, i.e., UECapabilityEnqiry and/or UECapablityInformation message.

To avoid unnecessary applicable functionality reporting, RAN2 further discuss to include a part of essential information of the set of inference related parameters in the set of inference related parameters in Step 1 and/or Step 2, i.e., UECapabilityEnqiry and/or UECapablityInformation message.

Currently RAN2 has some discussions on model/functionality training. In the real case, network side additional condition for specific functionality would possibly be the same, on the other side, UE side additional condition for the functionality among different UEs would also possibly be the same. For the case of network side additional condition and UE side additional condition are similar, repetitive model training procedure is a waste of radio resource. Therefore, it is a possible alternative that once the model has been well trained by one UE, it can be shared towards other UE for model inference directly, unless some of the following conditions have been met:

-	Network side additional condition is changed, since according to RAN1’s intention, the model shall be available for specific network side additional condition, once the network side additional condition is changed, UE shall use updated data set to train the model again.

-	UE side additional condition is changed, similar to network side additional condition, the model shall also be applicable to specific UE side additional condition, e.g. UE’s mobility speed, once the speed is changed, the previous model may no longer be applicable.

-	Poor performance result, the well-trained model does not have a good performance result, which needs to be further trained.

UE does not need to perform model training if it has received one well trained model, unless some of the following condition is met:
-	Network side additional condition is changed
-	UE side additional condition is changed
-	Poor performance result is detected

In addition, even though UE is configured to perform model training, it also needs to fulfill the following two conditions:

-	UE capability is fulfilled, since for UE sided model, the model training procedure is performed at either UE side or UE’s OTT server side, the procedure will consume quite a lot of resource for either UE or UE’s OTT server. Thus, model training cannot always be supported at UE side.

-	UE side additional condition is limited, even though the capability is good enough, UE side additional condition is dynamically changed, which cannot always ensure UE to perform model training, e.g., UE’s temperature is too high, UE’s battery life or current memory is too low.

Thus, it can be observed that for some UEs under some specific conditions, the model training procedure can be skipped. Instead, network can receive the well-trained models from one UE, and configure the model towards the other UEs directly.

It is suggested that for some UEs, the model training procedure can be configured to skip.

It is suggested to configure the well-trained model towards the UE which has been configured to skip model training procedure.

Model monitoring will be performed at both network side and UE side. When the model monitoring is performed at network side, network does not need to inform UE about the model monitoring performance result except further LCM functionality management.

When the model monitoring is performed at network side, network does not need to inform UE about the model monitoring performance result except further LCM functionality management instructions.

When the model monitoring is performed at UE side, UE should report the model monitoring performance result towards network. In order to improve the monitoring accuracy, UE should perform model monitoring procedures for certain times as a whole model monitoring procedure. In details, every time UE get the inference output, UE should be based on the inference output to perform model monitoring. Therefore, RAN2 should define model monitoring cycle. For each model monitoring cycle, UE will generate one model monitoring result. UE should report multiple model monitoring results towards network. In details, there are two reporting principles as listed in below:

-	Dynamic reporting: for each monitoring cycle, as long as UE get the monitoring performance, it should feedback towards network immediately

-	One time reporting: after the model monitoring is accomplished, UE will collect the monitoring result for all monitoring cycles as an output set and report towards network.

The pros and cons of the two performance feedback approaches are obviously observed, which is listed in the following table:

|  | Dynamic reporting | One time reporting |
| --- | --- | --- |
| Pros | Network can be aware of the model performance in a real-time way so that to perform LCM management more accurate | Decrease the complex of signalling exchange between UE and network |
| Cons | Every time UE get the monitoring result, it needs to report towards network, which needs extra procedure for uplink resource request. | Network cannot be aware of the model performance on time. |

According to the pro/con analysis, RAN2 should discuss which reporting principle of monitoring performance should be adopted.

The model monitoring accuracy can be improved by repeating multiple times of model monitoring procedure.

If model monitoring is performed at UE side, RAN2 should define model monitoring cycle with regard to each model inference output.

If model monitoring is performed at UE side, UE should report the performance result towards network for each model monitoring cycle.

If model monitor is performed at UE side, RAN2 should consider two reporting principles for performance reporting:
-	Dynamic reporting: for each monitoring cycle, as long as UE get the monitoring performance, it should feedback towards network immediately
-	One time reporting: after the model monitoring is accomplished, UE will collect the monitoring result for all monitoring cycles as an output set and report towards network.

The pros and cons of dynamic reporting and one time reporting are listed in the below table:

|  | Dynamic reporting | One time reporting |
| --- | --- | --- |
| Pros | Network can be aware of the model performance in a real-time way so that to perform LCM management more accurate | Decrease the complex of signalling exchange between UE and network |
| Cons | Every time UE get the monitoring result, it needs to report towards network, which needs extra procedure for uplink resource request. | Network cannot be aware of the model performance on time. |

RAN2 should determine which reporting principle should be adopted.

Considering that AI/ML based functionality activation will cause severe pressure on UE’s power, radio resource consumption and memory. So that UE has to perform fallback even if the model performance is still good enough. Similar as model monitoring, the AI/ML fallback strategy can also be performed at network side or UE side. If the AI/ML fallback decision is performed at network side, network does not need to inform UE about the strategy, e.g., fallback cause value, except further LCM management instructions.

Conclusion

In this contribution, we have discussed some remaining issues related to LCM configuration and functionality reporting, a brunch of proposals and observations are provided in below:

Observation 1	The meaning and the granularity of “functionality” for “Supported functionalities”, “Applicable functionalities” and “Activated functionalities” may not be the same.

Observation 2	Two mechanisms, naming A) and B) agreed in RAN1 for providing inference configurations in Step 3 should be supported.

Observation 3	According to RAN1 LS reply [1], when mechanism A) is applied (i.e., one or more of CSI-ReportConfig for inference configuration is provided in Step 3), UE is only requested to report applicability of the corresponding CSI-ReportConfig for responding to Step 3.

Observation 4	According to RAN1 LS reply [1], when mechanism B) is applied (i.e., one set or multiple sets of inference related parameters is provided in Step 3), UE is only requested to report applicability for set(s) of inference related parameters.

Observation 5	UE does not need to perform model training if it has received one well trained model, unless some of the following condition is met:

-	Network side additional condition is changed

-	UE side additional condition is changed

-	Poor performance result is detected

Observation 6	The model monitoring accuracy can be improved by repeating multiple times of model monitoring procedure.

Observation 7	The pros and cons of dynamic reporting and one time reporting are listed in the below table:

|  | Dynamic reporting | One time reporting |
| --- | --- | --- |
| Pros | Network can be aware of the model performance in a real-time way so that to perform LCM management more accurate | Decrease the complex of signalling exchange between UE and network |
| Cons | Every time UE get the monitoring result, it needs to report towards network, which needs extra procedure for uplink resource request. | Network cannot be aware of the model performance on time. |

Proposal 1	Support explicit indication on whether the applicability report is requested (e.g., the activation status indication) associated with each CSI-ReportConfig in RRCReconfiguration message.

Proposal 2	Upon receiving RRCReconfiguration message containing CSI-ReprotConfig, the UE only reports applicability in RRCReconfigurationComplete for those CSI-ReportConfig whose status is indicated as “deactivated”.

Proposal 3	To avoid unnecessary applicable functionality reporting, RAN2 further discuss to include a part of essential information of the set of inference related parameters in Step 1 and Step 2, i.e., UECapabilityEnqiry message and UECapablityInformation message.

Proposal 4	It is suggested that for some UEs, the model training procedure can be configured to skip.

Proposal 5	It is suggested to configure the well-trained model towards the UE which has been configured to skip model training procedure.

Proposal 6	When the model monitoring is performed at network side, network does not need to inform UE about the model monitoring performance result except further LCM functionality management instructions.

Proposal 7	If model monitoring is performed at UE side, RAN2 should define model monitoring cycle with regard to each model inference output.

Proposal 8	If model monitoring is performed at UE side, UE should report the performance result towards network for each model monitoring cycle.

Proposal 9	If model monitoring is performed at UE side, RAN2 should consider two reporting principles for performance reporting:

-	Dynamic reporting: for each monitoring cycle, as long as UE get the monitoring performance, it should feedback towards network immediately

-	One time reporting: after the model monitoring is accomplished, UE will collect the monitoring result for all monitoring cycles as an output set and report towards network.

Proposal 10	RAN2 should determine which reporting principle should be adopted.

- References

[1] R1-2410898, Reply LS on applicable functionality reporting for beam management UE-sided model

[2] R2-2407848, LS on applicable functionality reporting for beam management UE-sided model