3GPP TSG-RAN WG2 #129	R2-24xxxx

Athens, Greece, Feb 15th – Feb 19th

Agenda Item:	8.13.2

Source:	NEC

Title:	Discussion on multi-hop relay discovery and relay reselection

Document for:	Discussion, Decision

Introduction

During RAN2 128 meeting, RAN2 has made a brunch of conclusions related to relay discovery and reselection for U2N multi-hop relay. In this meeting, we will further investigate related issues based on the current progress.

Discussion

Previously, RAN2 has made the following agreement:

RAN2 #127 Agreements: n-hop relaying involves n relay UEs by definition. Distinguish the “last” relay UE as the U2N relay UE directly connected to the gNB, for RAN2 discussion Distinguish the “intermediate” relay UEs as the other relay UEs, for RAN2 discussion Distinguish the “first” relay UE as the intermediate relay UE directly connected to the remote UE, for RAN2 discussion FFS if we will capture the requirements according to these terms when implementing CRs. FFS link terminology. RAN2 intend to align with SA2 terminology when it is stable. RAN2 #128 agreements: Agreements: A lower bound of Uu RSRP is not required for the UE to operate as an intermediate Relay UE. If a UE can operate as a last relay UE according to the restrictions configured by the network, it does not operate as an intermediate relay UE towards the same cell for any remote UE (i.e., minimise number of hops when possible). FFS if operation as an intermediate relay UE towards a different cell would be supported. The network can configure an upper bound of Uu RSRP for the UE to operate as an intermediate relay UE. If the upper bound is not configured, there is no threshold, but this does not override the previous agreement. FFS if there would be additional restrictions in the case of no upper bound. Agreements: The following cases are supported for L2 multihop relay: - One last Relay UE can have two connections with one intermediate Relay UE and one Remote UE (the intermediate Relay UE and Remote UE are physically different UE). - Two physically different Remote UE(s) can have each indirect path via the same intermediate Relay UE and the same last Relay UE. FFS if the last relay UE can use the same L2ID for both of the connections in either case. Cases with two indirect paths to the gNB for the same remote UE are not supported. An Intermediate Relay UE can serve multiple multi-hop indirect paths of different Remote UEs. If the intermediate Relay UE also is acting as a Remote UE, it cannot support different indirect paths to the gNB with same/different last/U2N/parent intermediate Relay UE(s) based on different PC5 unicast links. Agreement: RAN2 sends LS to ask SA2 to clarify whether an intermediate relay needs to establish PC5 link with the relay from which the discovery announcement message is received before sending/forwarding the discovery announcement message.

According to the previous discussion, one can find that to act as a U2N last relay UE or an U2N intermediate relay UE, the UE shall try to satisfy specific radio condition, which is a upper bound and/or a lower bound of Uu RSRP for U2N last relay UE, and the upper bound of Uu RSRP for the U2N intermediate relay UE.

Yet since the UE can also move around and its radio condition may not be satisfied to act as the U2N last relay UE anymore. In Rel-18, the UE will not perform relay discovery anymore, and it will release the PC5 connection with other U2N remote UE. However, in Rel-19, even if the U2N last relay UE cannot satisfy the last relay UE’s radio condition, it can still try to judge whether it can act as a U2N intermediate relay UE.

Therefore, it is suggested that once the U2N last relay UE detecting its Uu radio condition cannot satisfy the configured last relay UE’s network condition, it will report its radio measurement result towards network. If the network does not configure the radio threshold of the U2N intermediate relay UE towards the UE previously. Furthermore, if the U2N last relay UE is in RRC_IDLE/RRC_INACTIVE state, the UE shall access into RRC_CONNECTED state to perform the measurement results reporting.

It is suggested that once the U2N last relay UE detecting its Uu radio condition cannot satisfy the configured network condition for the U2N last relay UE, it will report its radio measurement result towards network, if the network does not configure the radio threshold of the U2N intermediate relay UE towards the UE previously.

If the U2N last relay UE is in RRC_IDLE/RRC_INACTIVE state, the UE shall access into RRC_CONNECTED state to perform the measurement results reporting.

On the other hand, if the U2N last relay UE has already been configured with the radio threshold of the U2N intermediate relay UE, it can decide to change its role from U2N last relay UE towards U2N intermediate relay UE and reporting the role type change towards the network.

If the U2N last relay UE has already been configured with the radio threshold of the U2N intermediate relay UE, it can decide to change its role from U2N last relay UE towards U2N intermediate relay UE and reporting the role type change towards the network.

Similarly, in order to report the role type change towards the network, if the UE is in RRC_IDLE/RRC_INACTIVE state, the UE shall access into RRC_CONNECTED state.

In order to report the role type change towards the network, if the UE is in RRC_IDLE/RRC_INACTIVE state, the UE shall access into RRC_CONNECTED state.

Afterwards, if the U2N last relay UE is performing relay discovery and decide to perform role type change from U2N last relay UE towards U2N intermediate relay UE and has already reported towards network, it shall announce its role type change information within the discovery message.

If the U2N last relay UE is performing relay discovery and decide to perform role type change from U2N last relay UE towards U2N intermediate relay UE and has already reported towards network, it shall announce its role type change information within the discovery message.

While the related conclusion made from the above proposal shall be sent via an LS[1] towards SA2 since it will impact their specification.

On the other hand, if one UE, which is a U2N intermediate UE, is performing relay discovery, when its Uu signal becomes better, the UE may be satisfied to be a U2N last relay UE. In details, if the UE is configured with U2N last relay UE’s configuration, the UE can decide whether to perform role type change towards a U2N last relay UE. Furthermore, if the UE is in RRC IDLE/INACTIVE state, the UE shall access into RRC CONNECTED state to report the role type change decision towards network.

It is suggested that once the U2N intermediate UE detecting its Uu signal becomes better (i.e. better than the configured upper bound), and if the UE is configured with U2N last relay UE’s configuration, the UE can decide whether to perform role type change towards a U2N last relay UE.

If the UE is in RRC_IDLE/INACITVE state and decide to perform a role type change, it shall access into RRC_CONNECTED state and report role type change decision towards network.

Otherwise, if the UE is not configured with the last relay UE’s configuration, it shall access into RRC_CONNECTED state to report the measurement results towards network.

If UE is not configured with the last relay UE’s configuration, it shall access into RRC_CONNECTED state to report the measurement results towards network.

Afterwards, if the U2N intermediate relay UE is performing relay discovery and decide to perform role type change from U2N intermediate relay UE towards U2N last relay UE and has already reported towards network, it shall announce its role type change information within the discovery message.

If the U2N intermediate relay UE is performing relay discovery and decide to perform role type change towards a U2N last relay UE and has already reported towards network, it shall announce its role type change information within the discovery message.

While the related conclusion made from the above proposal about discovery message shall be sent via an LS[1] towards SA2 since it will impact their specification.

RAN2 should send the LS towards SA2 if proposal 5 is agreed.

Conclusion

In this contribution, we have primarily discussed further enhancement at RAN3 for U2N multi-hop relay. A brunch of proposals has been made as following:

Proposal 1	It is suggested that once the U2N last relay UE detecting its Uu radio condition cannot satisfy the configured network condition for the U2N last relay UE, it will report its radio measurement result towards network, if the network does not configure the radio threshold of the U2N intermediate relay UE towards the UE previously.

Proposal 2	If the U2N last relay UE is in RRC_IDLE/RRC_INACTIVE state, the UE shall access into RRC_CONNECTED state to perform the measurement results reporting.

Proposal 3	If the U2N last relay UE has already been configured with the radio threshold of the U2N intermediate relay UE, it can decide to change its role from U2N last relay UE towards U2N intermediate relay UE and reporting the role type change towards the network.

Proposal 4	In order to report the role type change towards the network, if the UE is in RRC_IDLE/RRC_INACTIVE state, the UE shall access into RRC_CONNECTED state.

Proposal 5	If the U2N last relay UE is performing relay discovery and decide to perform role type change from U2N last relay UE towards U2N intermediate relay UE and has already reported towards network, it shall announce its role type change information within the discovery message.

Proposal 6	It is suggested that once the U2N intermediate UE detecting its Uu signal becomes better (i.e. better than the configured upper bound), and if the UE is configured with U2N last relay UE’s configuration, the UE can decide whether to perform role type change towards a U2N last relay UE.

Proposal 7	If the UE is in RRC_IDLE/INACITVE state and decide to perform a role type change, it shall access into RRC_CONNECTED state and report role type change decision towards network.

Proposal 8	If UE is not configured with the last relay UE’s configuration, it shall access into RRC_CONNECTED state to report the measurement results towards network.

Proposal 9	If the U2N intermediate relay UE is performing relay discovery and decide to perform role type change towards a U2N last relay UE and has already reported towards network, it shall announce its role type change information within the discovery message.

Proposal 10	RAN2 should send the LS towards SA2 if proposal 5 is agreed.

Reference

3GPP TSG-RAN WG2 #129 R2-24xxxx Athens, Greece, Feb 15th – Feb 19th Title: [Draft] LS on information update within discovery message Release: Rel-19 Work Item: NR_SL_relay_enh2 Source: NEC (to be TSG RAN WG2) To: TSG RAN SA2 Contact person: Boyuan Zhang, zhang_boyuan@nec.cn # 1 Overall description RAN2 has agreed that for the following two cases: - U2N intermediate UE decides to change its role towards U2N last relay UE while performing relay discovery - U2N last relay UE decides to change its role towards U2N last intermediate UE while performing relay discovery The UE shall announce its role type change information within discovery message. # 2 Actions TO SA2 ACTIONS: RAN2 kindly requests SA2 to take the above conclusion into consideration and kindly provide feedback if any