3GPP TSG-RAN WG2 Meeting #129	R2-2500137

Athens, Greece, Feb. 17th – 21st, 2025

| Agenda Item: | 8.6.4 (NR_Mob_Ph4-Core) |
| --- | --- |
| Source: | LG Electronics Inc. |
| Title: | Discussion on remaining issues of CLTM |
| Document for: | Discussion and Decision |

1.	Introduction

In the previous meetings RAN2#127bis and RAN2#128, the following agreements for CLTM are made:

Agreements on C-LTM (RAN2#127bis)

- Source cell sends the conditional LTM configuration via RRCReconfiguration to UE, which includes the LTM candidate configurations, and the corresponding execution conditions.

- Event LTM3-like and LTM5-like are used as the conditional LTM execution condition. FFS on reuse of CHO conditions.

- Source cell and each candidate cell provides its own execution condition for conditional LTM.

- It is DU to generate the L1 execution condition. FFS on a case that L3 measurement is used.

- RACH-less Conditional intra-CU LTM is supported.

- RACH based conditional intra-CU LTM is supported.

- UE based TA measurement mechanism is supported for conditional intra-CU LTM.

- PDCCH ordered early TA acquisition is supported for conditional LTM.

- Rel-18 Early candidate TCI State activation/deactivation is supported for conditional intra-CU LTM.

- For RACH-less conditional LTM, CG-based first UL transmission on target cell is supported. FFS on DG-based approach.

- The LTM completion defined for Rel-18 intra-CU LTM is reused for conditional LTM.

Agreements on C-LTM (RAN2#128)

- The triggering condition of conditional LTM can be based on L3 measurement.

- CondEventA3 and CondEventA5 conditions can be baseline for the conditional LTM execution.

- The L1 execution condition of a candidate cell is associated to only one triggering event.

- For L3 execution condition, it may consist of one or two triggering condition(s). If there are two triggering conditions associated with the same candidate cell, the UE shall consider the execution condition is fulfilled only when both triggering conditions are met. Only single RS type is supported and at most two different trigger quantities can be configured simultaneously for the evaluation of execution condition of a single candidate cell.

- To support initial and subsequent conditional LTM, the following items can be considered for the configuration of execution condition:

- The CLTM configuration of each candidate cell shall include the execution condition for initial conditional LTM, which is generated by the initial source cell to trigger the CLTM for the candidate cell.

- The CLTM configuration of each candidate cell may include execution conditions for subsequent conditional LTM, which is generated by the candidate cell to trigger the CLTM for other candidate cells when the candidate cell becomes a serving cell.

- The network can configure measurement reports e.g., L1 periodic, semi-persistent, aperiodic and event triggered report, or L3 measurement reports for conditional LTM, e.g., to trigger PDCCH ordered early RACH.

- For CLTM, the Candidate Cell TCI States Activation/Deactivation MAC CE is re-used for the early activation/deactivation of TCI state(s) of a CLTM candidate configuration.

- The Early TA is signalled to the UE from the source cell (i.e., not from the candidate cell directly to the UE). This agreement will be included in the LS to RAN1/3/4.

- The network can inform the candidate cell’s TA information to UE via new MAC CE, which is the TA value when UE switches to that candidate cell during CLTM.

- Candidate cell TA is maintained by a new timer.

- For L1-based conditional LTM the condition evaluation is at MAC level and for L3-based conditional LTM the condition evaluation is at RRC level.

In this paper, we discuss further details on CLTM. In detail, the following aspects are handled in this paper:

- Coexistence of LTM and CLTM

- CLTM execution condition

- Early TA maintenance

- CLTM cell switch execution procedure

2.	Discussion

2.1.	Coexistence of LTM and CLTM

Based on [1], RAN2 has the following issue about LTM configuration:

Editor’s Note: FFS whether the 8 is the number of LTM plus CLTM candidate configurations or we have 8 LTM candidate configurations and 8 CLTM candidate configurations

In our view, this EN is related to whether or not CLTM configuration is provided to UE separately from Rel-18 LTM configuration. From signaling perspective, the difference between Rel-18 LTM and CLTM is whether or not the configuration includes execution conditions. This implies that CLTM configuration can be generated by including execution conditions in the Rel-18 LTM configuration for a specific candidate cell. Consider the case where CLTM configuration is provided to the UE separately from the Rel-18 LTM configuration. In this case, it is likely that most candidate cells for CLTM are same as those of Rel-18. This may lead to signaling redundancy. Hence, for signaling efficiency, the execution conditions are included in Rel-18 LTM candidate cell configuration as implemented in the Rapporteur CR [1], i.e., CLTM and Rel-18 LTM can be configured together. Consequently, it is natural that the 8 is the number of LTM plus CLTM candidate configurations.

Proposal 1. The execution conditions are included in Rel-18 LTM candidate cell configuration (i.e., CLTM and Rel-18 LTM can be configured together).

Proposal 2. The number of LTM plus CLTM candidate configurations is 8.

It is not desirable to put much restriction on the NW implementation, e.g., network cannot trigger the LTM cell switch to a candidate cell associated with execution condition. This is because an LTM cell switch can be triggered for reasons other than link quality, such as network load balancing. Therefore, all LTM candidate cells in the configuration can be used for NW trggered LTM (i.e., Rel-18 LTM CSC MAC CE based LTM cell switch), while the LTM candidate cells associated with execution conditions are used for CLTM.

Proposal 3. The LTM candidate cells associated with execution conditions are used for CLTM.

Proposal 4. The LTM candidate cells configured for CLTM can be used for NW trggered LTM (i.e., Rel-18 LTM CSC MAC CE based LTM cell switch).

2.2.	CLTM execution condition

In [1], CLTM execution conditions for a candidate cell configuration is implemented as follows:

CLTM-ExecutionConditions-r19 ::= SEQUENCE (SIZE (1..maxNrofLTM-Configs-r18)) OF ExecutionConditions-r19 ExecutionConditions-r19 ::= SEQUENCE { ltm-CandidateId-r19 LTM-CandidateId-r18, executionCondition-r19 CHOICE { l1-Conditions-r19 LTM-CSI-ReportConfigId-r18, l3-Conditions-r19 SEQUENCE (SIZE (1..2)) OF MeasId } OPTIONAL, -- Need R Editor’s Note: FFS whether L1 and L3 execution conditions can be configured simultaneously or not. ... }

As noted above, it is FFS whether L1 and L3 execution conditions can be configured simultaneously or not.

L3 measurement result is derived based on multi-beam measurement and time-domain filtering while L1 measurement is derived based on per-beam measurement. This implies that long-term stable cell with several good beams can be found by L3 measurement. On the other hand, the best beam can be dynamically found by based on L1 measurement. From these observations, simultaneous configuration of L1 and L3 execution conditions to a CLTM execution condition may be beneficial for UE to find a good CLTM candidate cell to perform CLTM cell switch.

Consider the case that both L1 and L3 execution condtions are configured in CLTM execution condition for a candidate cell, where L1 execution condition is configured with per-beam RSRP measurement and L3 execution condition is configured with cell-level RSRQ measurement. In this case, the UE can find a candidate cell whose best beam quality is quite good enough and other beam qualities are not bad (e.g., some beams are good to serve UE). If the UE switches the serving cell to the candidate cell (i.e., new serving cell) by CLTM execution, it is more likely that the UE will stay in the new serving cell through serving cell beam switching rather than moving to another candidate cell through LTM cell switch. This implies time of stay at a serving cell become longer, which in turn reduces cell switch interruption in long-term.

Proposal 5. L1 and L3 execution conditions can be simultaneously configured for evaluating a CLTM candidate cell.

Proposal 6. When a CLTM execution condition for a candidate cell is simultaneously configured with L1 and L3 execution conditions, the UE initiates the CLTM cell switch to the candidate cell if both L1 and L3 execution conditions are satisfied.

2.3.	CLTM cell switch execution procedure

FFS which beam is selected for RACH-less CLTM execution

For RACH-less CLTM, the UE determines the target cell and the target beam based on evaluation of execution conditions. It has agreed for event triggered LTM L1 report that TTT for an event is supported and TTT is evaluated per beam. This implies that an execution condition for CLTM is evaluated per beam. Therefore, it is straighforward that the beam that fulfils the execution condition and the candidate cell associated with the beam should be the target beam and the target cell of CLTM execution, respectively.

Proposal 7. The beam that fulfils the execution condition and the corresponding candidate cell are the target beam and the target cell of CLTM execution, respectively.

FFS if more than one LTM candidate cells fulfil the LTM cell switch execution

In legacy CHO, it is up to UE implementation which candidate cell to select if more than one candidate cells fulfil the execution condition. For example, in CHO, the UE implementation considers beams and beam quality to select one of the triggered cells for execution. In our view, the legacy principle can be applied to the case where more than one LTM candidate cells fulfil the LTM cell switch execution. In other words, it is up to UE implementation which candidate cell to select to perform CLTM execution if two candidate cells fulfil the execution condition. The UE implementation considers, e.g., beam quality and/or whether or not to early DL/UL synchronize to a candidate cell in order to select one of the triggered candidate cells for CLTM execution.

Proposal 8. It is up to UE implementation which candidate cell to select if more than one candidate cells fulfil the execution condition.

FFS whether to indicate the initiation of CLTM execution to network

In RAN2#127bis meeting, RAN2 agreed the following:

For RACH-less conditional LTM, CG-based first UL transmission on target cell is supported. FFS on DG-based approach.

Based on the agreement, DG-based RACH-less CLTM is FFS (i.e., green highlighted part). Some companies have a concern that it is not clear how DG-based approach works because network does not know when the UE comes to the candidate cell (i.e., the target cell of CLTM cell switch). If DG-based approach is not introduced for CLTM, network is forced to configure only CG and not DG for RACH-less CLTM cell switch. This can lead to less flexibility in network configuration. Since network does not have information about when the UE triggers CLTM execution, it is not clear for the target cell to start reserving the CG resource in CG-based RACH-less CLTM. This can result in inefficient network resource utilization.

Observation 1. Network configuration may be inflexible and/or network resources may be utilized inefficiently because network has no information about when the UE triggers CLTM execution.

In our view, the problem described above can be addessed by the existing cell switch notification procedure. Based on TS 38.401, the cell switch notification procedure between the source cell and the target cell should be performed before the UE access. Specifically in inter-DU LTM case (i.e., steps 17-21 in Figure 1), after the source gNB-DU decides to trigger LTM cell switch, the source gNB-DU sends LTM CSC MAC CE to the UE and then initiates the cell switch procedure. The target gNB-DU detects the UE access due to LTM cell switch after receiving the cell switch notification from the source gNB-DU.

Figure 1. Inter-DU LTM [2]

Since cell switch information such as target cell ID and beam information can be included in the cell switch notification message, the target cell can detect the UE access agilely based on the cell switch notification message. For example, the target cell can start providing to the UE with the configured grant (CG) associated with the beam indicated by LTM CSC MAC CE if the cell switch notification message includes the TCI state ID. Upon receiving an UL signal on the CG, the target cell considers that the UE access due to the LTM cell switch is detected. This means that the cell switch notification procedure allows the target cell to know the appropriate timing to transmit PDCCH to the UE, thus reducing LTM cell switch delay.

However in CLTM case, the cell switch notification procedure may not be initiated at an appropriate timing because the UE autonomously triggers the LTM cell switch execution based on an execution condition configured in LTM-Config. In other words, the source cell cannot initiate the cell switch notification procedure at a right time since the source cell does not know the timing of CLTM execution. If the cell switch notification procedure from the source cell to a candidate cell is initiated earlier than CLTM execution (e.g., reception of L1/L3 measurement report of the candidate cell, indication of TA value of the candidate cell, etc.), the UL grant configured for the CLTM cell switch may be wasted until CLTM execution. The waste of network resources can become severe because the target cell does not know which beam is selected by the UE for CLTM execution. In this case, as the number of candidate cells that have performed the cell switch notification procedure in advance increases, the waste of network resources becomes more severe. Moreover, after the CLTM completion, additional network signaling between the target cell and the candidate cells that are not the target cell of CLTM execution may be required to stop PDCCH transmission from the candidate cells to the UE.

Observation 2. In CLTM case, LTM cell switch delay can increase and/or network resource can be inefficiently utilized because the cell switch notification procedure is not initiated at a proper time.

In order to initiate the cell switch notification procedure at a proper time for CLTM, it would be good to introduce a bye message. For instance, the UE notifies the source cell of the initiation of CLTM execution when an execution condition is satisfied. As shown in Annex 5.1, the cell switch notification message includes the target cell ID (mandatory) and beam information to be used for LTM cell switch (optional). Hence, it makes sense that the bye message mandatorily includes the identifier of the candidate cell associated with the execution condition being satisfied.

Proposal 9. The UE sends the bye message to the source cell once an execution condition is satisfied, where the bye message mandatorily includes the identifier of the candidate cell associated with the execution condition.

As described above, the absence of beam information at the target cell may lead to inefficient network resource utilization during CLTM cell switch. In Rel-18, for RACH-less LTM cell switch, the cell switch notification message from the source cell to the target cell can include beam information such as TCI state ID to prevent the waste of network resources due to lack of beam information at the target cell. In line with the Rel-18 principle, it is beneficial that the cell switch notification message triggered by CLTM execution includes the beam information for RACH-less CLTM. To this end, once an execution condition is satisfied, the UE sends the bye message to the source cell including beam information associated with the execution condition being satisfied if the UE has the valid TA value of candidate cell associated with the execution condition. Upon receiving the notification of CLTM execution from the UE, the source cell initiates the cell switch notification procedure as Rel-18 LTM.

Proposal 10. Beam information associated with the execution condition being satisfied is included in the bye message if the UE has the valid TA value of candidate cell associated with the execution condition.

After sending the bye message to the source cell, the UE initiates the CLTM cell switch to the target cell. If the UE has the valid TA value of the target cell, then the UE performs RACH-less LTM cell switch to the target cell, i.e., the UE sends the RRCReconfigurationComplete message to the target cell. Otherwise, the UE performs RA-based LTM cell switch to the target cell. In other words, Rel-18 LTM cell switch execution procedure is baseline of CLTM cell switch execution procedure.

Proposal 11. Rel-18 LTM cell switch execution procedure is baseline of CLTM cell switch execution procedure.

3.	Conclusion

In this paper, we have made the following observations and proposals:

Coexistence of LTM and CLTM

Proposal 1. The execution conditions are included in Rel-18 LTM candidate cell configuration (i.e., CLTM and Rel-18 LTM can be configured together).

Proposal 2. The number of LTM plus CLTM candidate configurations is 8.

Proposal 3. The LTM candidate cells associated with execution conditions are used for CLTM.

Proposal 4. The LTM candidate cells configured for CLTM can be used for NW trggered LTM (i.e., Rel-18 LTM CSC MAC CE based LTM cell switch).

CLTM execution condition

Proposal 5. L1 and L3 execution conditions can be simultaneously configured for evaluating a CLTM candidate cell.

Proposal 6. When a CLTM execution condition for a candidate cell is simultaneously configured with L1 and L3 execution conditions, the UE initiates the CLTM cell switch to the candidate cell if both L1 and L3 execution conditions are satisfied.

CLTM cell switch execution procedure

Proposal 7. The beam that fulfils the execution condition and the corresponding candidate cell are the target beam and the target cell of CLTM execution, respectively.

Proposal 8. It is up to UE implementation which candidate cell to select if more than one candidate cells fulfil the execution condition.

Observation 1. Network configuration may be inflexible and/or network resources may be utilized inefficiently because network has no information about when the UE triggers CLTM execution.

Observation 2. In CLTM case, LTM cell switch delay can increase and/or network resource can be inefficiently utilized because the cell switch notification procedure is not initiated at a proper time.

Proposal 9. The UE sends the bye message to the source cell once an execution condition is satisfied, where the bye message mandatorily includes the identifier of the candidate cell associated with the execution condition.

Proposal 10. Beam information associated with the execution condition being satisfied is included in the bye message if the UE has the valid TA value of candidate cell associated with the execution condition.

Proposal 11. Rel-18 LTM cell switch execution procedure is baseline of CLTM cell switch execution procedure.

4.	References

[1] Rapporteur RRC for inter-CU and conditional LTM, [POST128][107][MOB] RRC CR for LTM and CLTM (Ericsson)

[2] TS 38.473 F1 application protocol (F1AP) (Release 18)

5.	Annex

5.1.	Content of cell switch notification message (TS 38.473 [2])

9.2.2.15	DU-CU CELL SWITCH NOTIFICATION

This message is sent by the gNB-DU to inform the gNB-CU about the initiation of the cell switch  command to the UE.

Direction: gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| Cell ID | M |  | NR CGI 9.3.1.12 |  | YES | reject |
| LTM Cell Switch Information |  | 0..1 |  |  | YES | ignore |
| >Joint or DL TCI State ID | M |  | OCTET STRING | Includes the TCI-StateId IE, as defined in TS 38.331 [8]. | - |  |
| >UL TCI State ID | O |  | OCTET STRING | Includes the TCI-UL-StateId IE, as defined in TS 38.331 [8]. | - |  |
| TA Information List |  | 0..1 |  |  | YES | ignore |
| >TA Information Item IEs |  | 1 .. <maxnoofTAList> |  |  | EACH | ignore |
| >>Candidate Cell ID | M |  | NR CGI 9.3.1.12 |  | - |  |
| >>TA Value | M |  | INTEGER (0..4095) | Indicates the TA value as defined in TS 38.213 [31]. | - |  |
| >>Tag ID Pointer | O |  | OCTET STRING | Includes the tag-Id-ptr contained in the TCI-UL-State IE or the TCI-State IE, as defined in TS 38.331 [8]. | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofTAList | Maximum no. of TA values to be sent, the maximum value is 8. |

9.2.2.16	CU-DU CELL SWITCH NOTIFICATION

This message is sent by the gNB-CU to inform the gNB-DU about the initiation of the cell switch command to the UE.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| Cell ID | M |  | NR CGI 9.3.1.12 |  | YES | reject |
| LTM Cell Switch Information |  | 0..1 |  |  | YES | ignore |
| >Joint or DL TCI State ID | M |  | OCTET STRING | Includes the TCI-StateId IE, as defined in TS 38.331 [8]. | - |  |
| >UL TCI State ID | O |  | OCTET STRING | Includes the TCI-UL-StateId IE, as defined in TS 38.331 [8]. | - |  |
| TA Information List |  | 0..1 |  |  | YES | ignore |
| >TA Information Item IEs |  | 1 .. <maxnoofTAList> |  |  | EACH | ignore |
| >>Candidate Cell ID | M |  | NR CGI 9.3.1.12 |  | - |  |
| >>TA Value | M |  | INTEGER (0..4095) | Indicates the TA value as defined in TS 38.213 [31]. | - |  |
| >>Tag ID Pointer | O |  | OCTET STRING | Includes the tag-Id-ptr contained in the TCI-UL-State IE or the TCI-State IE, as defined in TS 38.331 [8]. | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofTAList | Maximum no. of TA values to be sent, the maximum value is 8. |