3GPP TSG-RAN WG2 Meeting #129	R2-2500140

Athens, Greece, Feb. 17th – 21st, 2025

Agenda item:	8.1.2.2

Source:	LG Electronics Inc.

Title:	Functionality based LCM for BM

Document for:	Discussion and Decision

1. Introduction

During the RAN2 #128 meeting, RAN2 reached the following agreements.

- When a functionality configured by the network to be reported via UAI, becomes from non-applicable to applicable, the UE can reports it to the network. FFS detailed design - When a functionality becomes non-applicable the UE doesn’t autonomously deactivate. NW is expected to deactivate active functionality when it receives report from UE that it is non-applicable. - FFS whether the UE reports explicitly “non-applicable” functionality when there is a change of applicability. Verify this aligns with RAN1 configuration design - Applicable functionality reporting at handover is supported with the same RRC procedure that will be specified within a cell, as a baseline, i.e. the NW-side additional conditions and/or the inference configuration related to the target gNB are transmitted by the target gNB as part of the HO command, and the UE in response transmits the applicability report (either in RRCReconfigurationComplete or in UAI) to the target gNB after completing the handover. - Source cell UAI (as is) can be sent from source cell to target cell using existing signaling. No further optimizations will be considered in RAN2 related to UAI. - For BM use case for UE-side model, data collection related configuration(s) (e.g., measurement resources configuration) and associated ID(s) can be included in training data collection configuration. - For data collection configuration UE-side model training, the UE can send a request for data collection. FFS what the request contains. - The network can provide the data collection configuration (at any point in time), with or without UE request. - The following methods for network control of the initiation and configuration for data collection: - The network can decide when to start/stop the data collection and send configuration. - The network can configure whether UE is allowed to initiate request for data collection. - FFS whether an indication from UE to network is needed when UE can’t perform data collection based on received configuration

This contribution aims to address issued regarding applicable functionality reporting, data collection for UE sided training, and functionality continuity.

2. Discussion

Applicable Functionality Report

Definition assumption

In the last RAN1 meeting, RAN1 made an agreement regarding functionality terminology.

RAN1 # 119 - The concept/terminology “functionality” of Supported functionalities may refer to UE-capability information/parameters i.e., Rel-19 AI/ML-enabled Features/FGs - The concept/terminology “functionality” of Applicable functionalities may refer to CSI-ReportConfig for inference configuration or a set of inference related parameters - The Activated functionalities may be enabled based on CSI framework. - The meaning and the granularity of “functionality” for Applicable functionalities, Activated functionalities and Supported functionalities may or may not be the same.

For further discussion on applicable functionality report, RAN2 can assume the following definitions:

- Supported functionality: UE capability parameters

- Applicable functionality: applicable CSI-ReportConfig(option A in RAN1) or *inference related parameter(option B in RAN1)

*Inference related params: associated ID, resource set A/B, report content, Time instances related information for measurement/prediction (for BM2)

- Activate functionality: active CSI reporting based on CSI-ReportConfig

Proposal 1. RAN2 assumes the following definitions for further discussion on applicable functionality report

- Supported functionality: UE capability parameters

- Applicable functionality: applicable CSI-ReportConfig(option A in RAN1) or applicable *inference related parameter(option B in RAN1)

Inference related params: associated ID, resource set A/B, report content, or Time instances related information for measurement/prediction (for BM2)

- Activate functionality: active CSI reporting based on CSI-ReportConfig

Activation/Deactivation Procedure

RAN1 and RAN2 is under discussion the applicable functionality report procedure with the figure below.

<Option A>

According to the RAN1 agreement, for option A, the activation timing may vary depending on CSI reporting type.

- Applicable aperiodic CSI Report and semi-persistent CSI report can be activated/triggered by NW after the applicability reported.

- Applicable periodic CSI Report is considered as activated only if the applicability of the corresponding CSI-ReportConfig is reported in RRCReconfigurationComplete

Additionally, the deactivation timing may also differ based on the reporting method on each report type. For example, in the case of aperiodic CSI reporting, it can be considered deactivated immediately after a one-shot report is sent. In contrast, periodic CSI reporting can be deactivated when the corresponding configuration is released. RAN2 can assume the activation and deactivation timing for each CSI reporting type as follows:

- Aperiodic reporting:

  - Activation: When the UE receives reporting command via DCI, corresponding aperiodic CSI reporting is activated

  - Deactivation: Once the UE sends an aperiodic CSI reporting, corresponding aperiodic CSI reporting is deactivated

- Semi-persistent reporting:

  - Activation: When the UE receives reporting command via DCI/MAC CE, corresponding semi-persistent CSI reporting is activated

  - Deactivation: When the UE receives deactivation command via DCI/MAC CE, corresponding semi-persistent CSI reporting is deactivated

- Periodic reporting:

  - Activation: When the UE receives periodic CSI reporting related configuration and sends applicable reporting for corresponding periodic CSI reporting

  - Deactivation: When the NW releases corresponding periodic CSI reporting configuration, corresponding periodic CSI reporting is deactivated

With these assumptions, RAN2 can define procedures for each CSI reporting type.

Procedure for Aperiodic CSI report

- Step 3. NW configures one or more of CSI-ReportConfig for inference configuration

  - Associated ID (working assumption)

- Step 4. UE reports applicability for one or more CSI-ReportConfig

- Step 5. Optionally, NW configures CSI-ReportConfig for inference configuration

- Step 6. NW triggers applicable aperiodic CSI report (via DCI)

- Step 7. UE reports corresponding aperiodic CSI report

  - UE consider corresponding aperiodic CSI report is deactivated after reporting to NW

Procedure for Semi-persistent CSI report

- Step 3. NW configures one or more of CSI-ReportConfig for inference configuration

  - Associated ID (working assumption)

- Step 4. UE reports applicability for one or more CSI-ReportConfig

- Step 5. Optionally, NW configures CSI-ReportConfig for inference configuration

- Step 6. NW activates applicable semi-persistent CSI report (via DCI/MAC CE)

- Step 7. UE reports corresponding semi-persistent CSI report

  - UE consider corresponding semi-persistent CSI report is activated until it receives deactivate command for semi-persistent reporting

- Step 8. For semi-persistent CSI reporting, NW deactivate semi-persistent CSI reporting (via DCI/MAC CE)

Procedure for Periodic CSI report

- Step 3. NW configures one or more of CSI-ReportConfig for interference configuration

  - Associated ID (working assumption)

- Step 4. UE reports applicability for one or more CSI-ReportConfig

  - UE considers applicable periodic CSI report is activated

- Step 5. Optionally, NW configures CSI-ReportConfig for inference configuration

- Step 6. UE reports applicable periodic CSI report

  - UE consider corresponding periodic CSI report is activated until NW releases periodic CSI reporting related configuration

- Step 7. NW releases configuration related to corresponding periodic CSI reporting

Proposal 2. For option A, RAN2 assumes the activation and deactivation timing for each CSI reporting type as follows:

- Aperiodic reporting:

  - Activation: Upon receiving a reporting command via DCI.

  - Deactivation: After the UE sends an aperiodic CSI report.

- Semi-persistent reporting:

  - Activation: Upon receiving a reporting command via DCI/MAC CE.

  - Deactivation: Upon receiving a deactivation command via DCI/MAC CE.

- Periodic reporting:

  - Activation: Upon reporting applicability (e.g., via RRCReconfigurationComplete).

  - Deactivation: When the network releases the configuration.

In the case of periodic CSI reporting, if only a part of the configured CSI report configuration is applicable, transmitting all reference signals related to the configured CSI report configuration may be wasteful. Therefore, the network can start transmitting the reference signals after step 4.

In this case, unlike in conventional periodic CSI reporting, considering the time required for the network to start transmitting the relevant reference signals after receiving the RRCReconfigurationComplete message from the UE in step 4, it may be unclear when the UE should begin periodic CSI reporting. If the UE performs measurement and reporting immediately after sending RRCReconfigurationComplete, the initial reported values may not be accurate because the network may not have started transmitting the necessary reference signals (e.g., CSI-RS) yet. If the UE performs measurement before the network begins transmission, it may report CSI values without valid reference signals, leading to inaccurate results. Therefore, the UE should start periodic CSI reporting after receiving confirmation of the applicable functionality report. For example, the UE can start periodic CSI reporting after receiving acknowledgement from the network for the RRCReconfigurationComplete message.

Proposal 3. For option A, UE starts periodic CSI reporting for inference after receiving acknowledgement from the network for the applicable functionality report (e.g., RRCReconfigurationComplete message)

<Option B>

On the other hand, in the case of Option B, the CSI report configuration is set according to the applicable parameters reported by the UE. Compared to Option A, the only difference in the activation/deactivation timing is that periodic CSI reporting can be activated at the configured time (similar to legacy CSI reporting).

Proposal 4. For option B, RAN2 assumes the activation and deactivation timing for each CSI reporting type as follows:

- Aperiodic reporting:

  - Activation: Upon receiving a reporting command via DCI.

  - Deactivation: After the UE sends an aperiodic CSI report.

- Semi-persistent reporting:

  - Activation: Upon receiving a reporting command via DCI/MAC CE.

  - Deactivation: Upon receiving a deactivation command via DCI/MAC CE.

- Periodic reporting:

  - Activation: Upon receiving the configuration

  - Deactivation: When the network releases the configuration.

Non-applicable Functionality Report

Applicability for a certain functionality can be updated. When the status of applicability has been changed, the UE can trigger applicable functionality reports via RRC message such as RRCReconfigurationComplete or UEAssistanceInfomation.

If the applicability of a functionality becomes non-applicable, there are two approaches to inform the network of the non-applicable functionality: (i) explicit reporting and (ii) implicit reporting. The former explicitly lists the non-applicable functionalities, while the latter indicates them implicitly by reporting only the applicable functionalities.

Explicit reporting can be unambiguous to the network. However, since applicable functionality must be reported for the activation of a specific functionality, separately indicating non-applicable functionality may be redundant and could introduce signalling overhead, as the applicable functionality already indicates what can be active. In this sense, implicit reporting is more advantageous, considering both signalling overhead and the initial purpose of reporting.

- Efficiency in Signalling: Implicit reporting reduces unnecessary signalling overhead because the UE only reports applicable functionalities. Since the network is primarily concerned with activating applicable functionalities, listing non-applicable ones is redundant and inefficient.

- No Need to Track Non-Applicable Functionalities: The network only needs to manage what can be activated, not what cannot. As long as the UE provides a list of applicable functionalities, the network can implicitly infer that the rest are non-applicable.

- Alignment with the Purpose of Reporting: The primary goal of reporting is to ensure the proper activation of applicable functionalities. Whether a functionality is non-applicable is not directly useful to the network; what matters is knowing what can be used.

- More Scalable Approach: As the number of functionalities grows, explicit reporting becomes inefficient, requiring longer lists of non-applicable functionalities. In contrast, implicit reporting naturally scales, as the UE only transmits a compact list of applicable functionalities, regardless of how many exist.

During the discussion, some companies suggested that the applicable functionality report can be utilized for specific purposes, such as requesting a specific configuration when retraining is needed. In this context, they argue that non-applicable functionality should be explicitly reported, along with a reason, so that the network can understand which configuration should be used for training instead of activating a particular functionality. However, consistently reporting non-applicable functionalities for these cases could lead to inefficiencies, such as unnecessary signaling overhead.

To address this, RAN2 should separate the procedures for reporting non-applicable functionalities depending on the purpose of report.

- For deactivating a non-applicable functionality:

  - There is no need to explicitly report non-applicable functionality. Simply reporting the applicable functionality is sufficient.

- For informing the network about a preferred configuration (e.g., in cases related to training purposes):

  - The UE may explicitly report non-applicable functionality only when it intends to request a specific configuration/transmission

Proposal 5. To discuss separately the applicable functionality report procedure for non-applicable functionality

- For deactivating a non-applicable functionality: Simply reporting the applicable functionality

- For informing the network about a preferred configuration/transmssion (e.g., in cases related to training purposes): Explicitly reporting non-applicable functionality

Data collection for UE sided training

RS configuration

RS configuration for training is related to CSI reporting configuration. The applicable functionality report is already under discussion for requesting applicable RS configuration, and the framework for the requests is being discussed via Option A and Option B. If Proposal 5 is agreed, the network can use the applicable functionality report for RS configuration related to training.

Proposal 6: gNB can recognize required RS configuration for training through the applicable functionality report

Initiation/Termination of Data Collection and Data Transfer

In data transfer options, since options 1a and 1b involve a UP solution, and option 2 involves NAS, decisions on when to initiate/terminate data collection or conduct data transfers may be considered outside RAN2's scope. However, option 3, as an OAM-related solution, could leverage the MDT approach being discussed for network-sided data collection. In network-sided data collection, discussions are ongoing regarding how to configure initiation/termination of data collection on the UE and set up reporting procedures.

For the BM case, if option 3 (OAM solution) is feasible for UE-sided data collection in SA5, this may impact RAN2 in initiation/termination of data collection and data transfer. However, RAN2 needs to discuss how MDT for UE-sided data collection differs from MDT for network-sided data collection. This discussion can help identify any additional considerations that may be necessary for the ongoing discussions regarding network-sided data collection.

Proposal 7. For BM use case, RAN2 understands that only option 3 (OAM solution) for initiation/termination of data collection and data transfer has an impact on RAN2. FFS on if there may be differences compared to network-sided data collection with immediate MDT

3. Conclusion

Proposal 1. RAN2 assumes the following definitions for further discussion on applicable functionality report

- Supported functionality: UE capability parameters

- Applicable functionality: applicable CSI-ReportConfig(option A in RAN1) or applicable *inference related parameter(option B in RAN1)

Inference related params: associated ID, resource set A/B, report content, or Time instances related information for measurement/prediction (for BM2)

- Activate functionality: active CSI reporting based on CSI-ReportConfig

Proposal 2. For option A, RAN2 assumes the activation and deactivation timing for each CSI reporting type as follows:

- Aperiodic reporting:

  - Activation: Upon receiving a reporting command via DCI.

  - Deactivation: After the UE sends an aperiodic CSI report.

- Semi-persistent reporting:

  - Activation: Upon receiving a reporting command via DCI/MAC CE.

  - Deactivation: Upon receiving a deactivation command via DCI/MAC CE.

- Periodic reporting:

  - Activation: Upon reporting applicability (e.g., via RRCReconfigurationComplete).

  - Deactivation: When the network releases the configuration.

Proposal 3. For option A, UE starts periodic CSI reporting for inference after receiving acknowledgement from the network for the applicable functionality report (e.g., RRCReconfigurationComplete message)

Proposal 4. For option B, RAN2 assumes the activation and deactivation timing for each CSI reporting type as follows:

- Aperiodic reporting:

  - Activation: Upon receiving a reporting command via DCI.

  - Deactivation: After the UE sends an aperiodic CSI report.

- Semi-persistent reporting:

  - Activation: Upon receiving a reporting command via DCI/MAC CE.

  - Deactivation: Upon receiving a deactivation command via DCI/MAC CE.

- Periodic reporting:

  - Activation: Upon receiving the configuration

  - Deactivation: When the network releases the configuration.

Proposal 5. To discuss separately the applicable functionality report procedure for non-applicable functionality

- For deactivating a non-applicable functionality: Simply reporting the applicable functionality

- For informing the network about a preferred configuration/transmssion (e.g., in cases related to training purposes): Explicitly reporting non-applicable functionality

Proposal 6: gNB can recognize required RS configuration for training through the applicable functionality report

Proposal 7. For BM use case, RAN2 understands that only option 3 (OAM solution) for initiation/termination of data collection and data transfer has an impact on RAN2. FFS on if there may be differences compared to network-sided data collection with immediate MDT