3GPP TSG-RAN WG2 Meeting #129	R2-2500141

Athens, Greece, Feb. 17th – 21st, 2025

Agenda item:	8.1.2.2

Source:	LG Electronics Inc.

Title:	NW side Data Collection

Document for:	Discussion and Decision

1. Introduction

During the RAN2 #128 meeting, RAN2 reached the following agreements.

- Focus on the following three radio condition event based logging - L3 serving cell measurement based (e.g. X1/X2 similar to A1/A2) - Beam based events (e.g. beam becomes top-1 beam and number of measurements is less than configured value) - L1 beam level measurement - Measurements on aperiodic CSI resources are not reported for NW sided data collection. - Data collection is controlled by the network. The UE will not autonomously stop when low power state is detected. - The UE reports to the network when the power state is low. We will not specify how the UE determines low power state. The network should de-configure the data collection (this can be captured in stage 2). - The UE reports to the network when buffer is or may become full. FFS when it reports (before and/or after). - The UE can report the reason for triggering of indication for the status (e.g. low power state, low memory). FFS how this is signalled and if the reporting can be part of availability indication.

This contribution aims to specify the configuration and procedure involved in data collection for BM use case.

2. Discussion

Configuration

In order to enable logging within the RRC, a dedicated CSI configuration for RRC logging can be newly established. This involves configuring the resource configuration for periodic, aperiodic, and semi-persistent CSI-RS, separate from the logging and reporting settings for each resource/resource set. However, CSI reporting may still be required for inference and monitoring operations where logging is unnecessary for the same resource/resource set, in which case the conventional CSI reporting via the L1 layer will continue to be used. Establishing a separate resource configuration solely for logging, identical to the one used for CSI reporting, would result in unnecessary duplication. Therefore, it is preferable that resource configuration for logging refer to the resource configuration of conventional CSI framework.

Observation 1. CSI reporting may still be required for inference and monitoring operations where logging is unnecessary, in which case the conventional CSI reporting via the L1 layer will continue to be used. Establishing a separate resource configuration solely for logging, identical to the one used for CSI reporting, would result in unnecessary duplication.

Proposal 1. Resource configuration for logging refers to the resource configuration of conventional CSI framework.

To set L3 reporting configuration for L1 measurements, the configuration can be affected from two parts. One part is for L1 measurement-related configuration (e.g., conventional CSI framework), while the other part is for L3 logging-related configuration.

<Figure 2. Configuration structure>

The L1 measurement-related configuration can be further divided into three sub-parts: Triggering State, Report and Resource Configuration, and Physical Layer Configuration.

- Physical Layer configuration: Contain references to NZP CSI-RS resources, CSI-IM resources, and SSB resources in a CSI-RS resource set

- Triggering State: Contain trigger states for dynamically selecting one or more aperiodic and semi-persistent reporting configuration and/or triggering one or more aperiodic CSI-RS resource sets

- Report and Resource Configuration: Contain reporting configuration for a periodic/semi-persistent/aperiodic report and resource configuration including a group of one or more NZP-CSI-RS-ResourceSet, CSI-IM-ResourceSet and/or CSI-SSB-ResourceSet

The L3 Logging configuration can be further divided into two sub-parts: Logging object and Logging-related configuration

- Logging Object: Logging object configuration can include resource configuration for obtaining L1 measurement results

- Logging-related configuration:  Logging configuration can include logging interval to indicate the periodicity for logging.

According to Observation 1, Logging Object can be linked to one of configurations in CSI reporting settings. There may five alternatives as below:

- Alternative 1. Link to reference signal, such as certain CSI-RS(s)/SSB(s) in L1 measurement-related configuration as shown in (1) of Figure 2.

- Alternative 2. Link to resource set ID associated with nzp-CSI-RS/csi-IM/csi-SSB resource(s) in L1 measurement-related configuration as shown in (2) of Figure 2.

- Alternative 3. Link to resource config ID associated with at least one resource set ID in L1 measurement-related configuration as shown in (3) of Figure 2.

- Alternative 4. Link to report config ID associated with resource config ID(s) in L1 measurement-related configuration as shown in (4) of Figure 2.

- Alternative 5. Link to entry index value of triggering state list associated with report config ID in L1 measurement-related configuration as shown in (5) of Figure 2.

From the perspective that the logging object is ultimately the reporting object, Alternative 4 makes sense to indicate logging object. Alternative 4 can also reuse the legacy DCI/MAC CE for CSI reporting to trigger logging. On the other hand, for Alternative 1-3, a separate DCI/MAC CE/RRC signalling is needed to indicate when to start/stop logging, especially for semi-persistent CSI-RS. Therefore, RAN2 could consider designing the signaling based on Alternative 4 to enable simpler UE operations. However, an LS should be sent to RAN1 to consult on whether the RS designated for logging can be specified through the CSI report config(CSI-ReportConfig).

Proposal 2. Send an LS to RAN1 to consult on whether the RS for logging object can be specified through the CSI report config

Logging mechanism

RAN2 should decide when to start and how to perform logging.

Initiation of logging

In legacy CSI reporting, periodic CSI reporting is triggered if the UE receives CSI configuration. Likewise, the logging of periodic CSI-RS can be triggered if the UE receives the logging configuration associated with the periodic CSI-RS.

Proposal 3. For periodic CSI-RSs, logging is initiated if logging configuration associated with the periodic CSI-RSs is received

Conversely, according to TS 38.214, semi-persistent CSI-RS do not support periodic CSI reporting and therefore require additional DCI/MAC CE for reporting. Note that the logging object is essentially a reporting object. A straightforward approach to activating logging would be to reuse the existing DCI/MAC CE for CSI reporting, thus leveraging existing signalling. When the UE receives a DCI/MAC CE for CSI reporting, it can decide to either log or report the L1 measurement results based on its logging requirements. However, further discussion in RAN2 is necessary to fully evaluate this approach.

| CSI-RS Configuration | Periodic CSI Reporting | Semi-Persistent CSI Reporting | Aperiodic CSI Reporting |
| --- | --- | --- | --- |
| Periodic CSI-RS | No dynamic triggering/activation | For reporting on PUCCH, the UE receives an activation command, as described in clause 6.1.3.16 of [10, TS 38.321]; for reporting on PUSCH, the UE receives triggering on DCI | Triggered by DCI; additionally, subselection indication as described in clause 6.1.3.13 of [10, TS 38.321] possible as defined in Clause 5.2.1.5.1. |
| Semi-Persistent CSI-RS | Not Supported | For reporting on PUCCH, the UE receives an activation command, as described in clause 6.1.3.16 of [10, TS 38.321]; for reporting on PUSCH, the UE receives triggering on DCI | Triggered by DCI; additionally, subselection indication as described in clause 6.1.3.13 of [10, TS 38.321] possible as defined in Clause 5.2.1.5.1. |
| Aperiodic CSI-RS | Not Supported | Not Supported | Triggered by DCI; additionally, subselection indication as described in clause 6.1.3.13 of [10, TS 38.321] possible as defined in Clause 5.2.1.5.1. |

<TS 38.214. Table 5.2.1.4-1: Triggering/Activation of CSI Reporting for the possible CSI-RS Configurations. >

Proposal 4. RAN2 needs to discuss how/when logging for semi-persistent CSI-RS can be activated, e.g., whether to activate logging upon receiving CSI reporting DCI/MAC CE as legacy CSI reporting mechanism. Send an LS to RAN1 to consult on the conclusion of Proposal 4.

logging operation

In legacy MDT operations, if the report type is periodic, the UE logs data at the specified logging interval. Similarly, for periodic/semi-persistent CSI-RS logging, the UE can log data at each logging interval, and the minimum logging interval can be the transmission periodicity of the CSI-RS.

<logging with periodic reporting w/ logging interval>

<logging with semi-persistent reporting w/ logging interval>

Proposal 5. For periodic logging, UE performs logging at each logging interval, and the minimum logging interval can be the transmission periodicity of the CSI-RS/SSB

For event-triggered reporting, the initial motivation of companies proposing event-based logging is centered on the need to collect data from different UE environments, such as the cell edge and cell center, to train AI/ML models for beam management. However, this approach assumes that collecting data from specific environments alone is sufficient or meaningful for the use cases defined by RAN1. From our perspective, this assumption has not been sufficiently validated by RAN1 for the defined beam management use cases. Furthermore, introducing event-based filtering at the RAN2 level seems unnecessary. A more comprehensive approach that gathers data from all conditions a UE encounters within a serving cell would ensure a better training dataset for generalized model performance. Pre-filtering data in RAN2 may risk excluding valuable information that could degrade the model’s effectiveness. Therefore, we believe it would be more appropriate to validate the need for environment-specific data collection with RAN1 before implementing any event-based mechanisms in RAN2.

Proposal 6. Send an LS to RAN1 to consult on whether the event-based logging can be useful for RAN1 use cases

Additional case for consideration

For logging interval-based operation, there may be instances where more than one L1(CSI-RS/SSB) measurement result is delivered within a single interval. In such cases, logging the most recent L1 measurement result at the logging interval would ensure that L3 always provides the latest data available, minimizing the impact of multiple results within a single interval. This approach keeps reporting consistent and prevents less relevant data from logging. There may also be cases where no L1 measurement result is delivered within a single interval. In this case, since there is no issue if the UE skips the logging operation, the logging can simply be skipped.

Proposal 7. For logging interval-based operations:

- If multiple L1 measurement results are delivered within a single interval, the UE logs only the most recent result to ensure the latest data is logged

- If no L1 measurement result is delivered, the UE skips the logging operation for that interval

Logging resume

According to RAN2 agreement, when a UE reaches its buffer limit due to logging, the UE can stop logging. In legacy MDT operation, if measurement logging is stopped due to an IDC problems, the UE can resume measurement logging if the IDC issue detected by the UE during the last logging interval is resolved. By resuming logging autonomously, the UE does not require additional signaling from the network to start logging. Similarly, when the buffer limit is reached, the UE can stop logging but keep the logging configuration. In this case, when the buffer limit issue is resolved, the UE can perform logging according to the available logging configuration, if any.

Proposal 8. When the buffer limit issue is resolved, UE can resume logging according to the available logging configuration, if any

Reporting Mechanism

The UE can report to the network when the buffer is/or become full or when the power state is low. In case the UE encounters a buffer limitation, it can report to the network to enable data retrieval, aligning with the legacy data availability indication used in logged MDT operations. Therefore, instead of introducing new signaling, the existing indication can be reused to inform the network. Given that buffer limitations can occur in connected state, the data availability indication can be transmitted via UAI messages. Since the expected action (i.e., retrieving data from the network) remains consistent with legacy MDT, no additional reason information is required for the report.

Proposal 9. For buffer limitation cases, the legacy data availability indication from logged MDT operations is utilized without specifying the reason for the report. The data availability indication can be sent via UAI messages

On the other hand, for the low power cases, the report aims to deactivate logging operations from the network. Given that power issues affect not only logging but also other operations, a more generalized solution is necessary. Since this issue is not specific to AIML, it should be addressed through a new, extensible indication applicable to various operations.

Proposal 10. For low power scenarios, a general indication is introduced. This new indication can be sent via UAI messages

It was also left as FFS whether new SRB is introduced or SRB4 can be used for reporting of logged data.

RAN2 #127bis Low priority SRB will be used. FFS new SRB or use of SRB4

The UE's logged data can only be transmitted when requested by the network. In other words, even if both QoE and logging operations are configured, the network can prioritize either logging data or QoE data based on its implementation. Therefore, there is no issue with using the existing SRB4 for transmitting the logged data. As such, SRB4 can be used for the transmission of logged data.

Proposal 11. SRB4 is used for the transmission of logged data

UE behavior during HO, RRC state transitions, and RRC Re-establishment

There is no guarantee that the UE can transmit logged data to the gNB that has configured logging. Specifically, during handover processes, transmission of logging data might fail due to radio quality issues in the serving cell before the handover is completed. If the UE discards such logged data during handover, it would be inefficient given the resources (memory and power) consumed by the UE in collecting this data. Therefore, the UE can retain logged data during handover processes and can indicate availability of logged data to the target gNB during handover.

Proposal 12: UE retains logged data during handover (HO)

Proposal 13: UE indicates availability of logged data during handover (e.g., within the RRCReconfigurationComplete message).

When transitioning to RRC Idle or RRC Inactive states, the network might face challenges in retrieving logged data due to constraints such as high network load. Additionally, during RRC Re-establishment procedures, which often occur due to abnormal UE conditions (e.g., RLF, invalid configuration, etc), transmitting logged data to the network can also be problematic. Therefore, to mitigate these issues, the UE can retrain logged data during transitions to RRC Idle/Inactive states or during RRC Re-establishment procedures. Then, the UE can indicate availability of logged data to the network during RRC connection establishment procedure (e.g., RRC Setup, RRC Resume, or RRC Reestablishment).

Proposal 14: UE retains logged data during transitions to RRC Idle/Inactive states or during RRC Re-establishment procedures.

Proposal 15: UE informs the network about the availability of logged data upon transitioning back to the RRC Connected state (e.g., through RRCSetupComplete, RRCResumeComplete or RRCReestablishment messages).

On the contrary, if the UE receives a new logging configuration or determines that the valid time has expired, it should consider the logged data as no longer valid and clear it for the next logging operation. The conditions for releasing logged data can follow the existing logged MDT operation.

Proposal 16. UE releases logged data under the following conditions (same as Logged MDT):

- upon receiving new configuration (e.g., for the modification of configuration associated with logging object)

- upon power off

- upon deregistration

- upon a certain time later, FFS length of timer (like 48-hour timer in logged MDT)

- upon successful delivery of the UEInformationResponse message (w/ logged data)

3. Conclusion

Observation 1. CSI reporting may still be required for inference and monitoring operations where logging is unnecessary, in which case the conventional CSI reporting via the L1 layer will continue to be used. Establishing a separate resource configuration solely for logging, identical to the one used for CSI reporting, would result in unnecessary duplication.

Proposal 1. Resource configuration for logging refers to the resource configuration of conventional CSI framework.

Proposal 2. Send an LS to RAN1 to consult on whether the RS for logging object can be specified through the CSI report config

Proposal 3. For periodic CSI-RSs, logging is initiated if logging configuration associated with the periodic CSI-RSs is received

Proposal 4. RAN2 needs to discuss how/when logging for semi-persistent CSI-RS can be activated, e.g., whether to activate logging upon receiving CSI reporting DCI/MAC CE as legacy CSI reporting mechanism. Send an LS to RAN1 to consult on the conclusion of Proposal 4.

Proposal 5. For periodic logging, UE performs logging at each logging interval, and the minimum logging interval can be the transmission periodicity of the CSI-RS/SSB

Proposal 6. Send an LS to RAN1 to consult on whether the event-based logging can be useful for RAN1 use cases

Proposal 7. For logging interval-based operations:

- If multiple L1 measurement results are delivered within a single interval, the UE logs only the most recent result to ensure the latest data is logged

- If no L1 measurement result is delivered, the UE skips the logging operation for that interval

Proposal 8. When the buffer limit issue is resolved, UE can resume logging according to the available logging configuration, if any

Proposal 9. For buffer limitation cases, the legacy data availability indication from logged MDT operations is utilized without specifying the reason for the report. The data availability indication can be sent via UAI messages

Proposal 10. For low power scenarios, a general indication is introduced. This new indication can be sent via UAI messages

Proposal 11. SRB4 is used for the transmission of logged data

Proposal 12: UE retains logged data during handover (HO)

Proposal 13: UE indicates availability of logged data during handover (e.g., within the RRCReconfigurationComplete message).

Proposal 14: UE retains logged data during transitions to RRC Idle/Inactive states or during RRC Re-establishment procedures.

Proposal 15: UE informs the network about the availability of logged data upon transitioning back to the RRC Connected state (e.g., through RRCSetupComplete, RRCResumeComplete or RRCReestablishment messages).

Proposal 16. UE releases logged data under the following conditions (same as Logged MDT):

- upon receiving new configuration (e.g., for the modification of configuration associated with logging object)

- upon power off

- upon deregistration

- upon a certain time later, FFS length of timer (like 48-hour timer in logged MDT)

- upon successful delivery of the UEInformationResponse message (w/ logged data)