3GPP TSG-RAN WG2 Meeting #129		R2-2500168

Athens, Greece, Feb 17th – 21st, 2025

Source:	Baicells

Title:		Discussion on Conditional intra-CU LTM

Agenda Item:		8.6.4

Document for:	Discussion and Decision

- Introduction

The following agreements to support the Conditional intra-CU LTM are made in the RAN2#127bis[1] and RAN2#128[2]:

RAN2#127bis Agreements on C-LTM: - Source cell sends the conditional LTM configuration via RRCReconfiguration to UE, which includes the LTM candidate configurations, and the corresponding execution conditions. - Event LTM3-like and LTM5-like are used as the conditional LTM execution condition. FFS on reuse of CHO conditions. - Source cell and each candidate cell provides its own execution condition for conditional LTM. - It is DU to generate the L1 execution condition. FFS on a case that L3 measurement is used. - RACH-less Conditional intra-CU LTM is supported. - RACH based conditional intra-CU LTM is supported. - UE based TA measurement mechanism is supported for conditional intra-CU LTM. - PDCCH ordered early TA acquisition is supported for conditional LTM. - Rel-18 Early candidate TCI State activation/deactivation is supported for conditional intra-CU LTM. - For RACH-less conditional LTM, CG-based first UL transmission on target cell is supported. FFS on DG-based approach. - The LTM completion defined for Rel-18 intra-CU LTM is reused for conditional LTM.

RAN2#128 Agreements on C-LTM: - The triggering condition of conditional LTM can be based on L3 measurement. - CondEventA3 and CondEventA5 conditions can be baseline for the conditional LTM execution. - The L1 execution condition of a candidate cell is associated to only one triggering event. - For L3 execution condition, it may consist of one or two triggering condition(s). If there are two triggering conditions associated with the same candidate cell, the UE shall consider the execution condition is fulfilled only when both triggering conditions are met. Only single RS type is supported and at most two different trigger quantities can be configured simultaneously for the evaluation of execution condition of a single candidate cell. - To support initial and subsequent conditional LTM, the following items can be considered for the configuration of execution condition: - The CLTM configuration of each candidate cell shall include the execution condition for initial conditional LTM, which is generated by the initial source cell to trigger the CLTM for the candidate cell. - The CLTM configuration of each candidate cell may include execution conditions for subsequent conditional LTM, which is generated by the candidate cell to trigger the CLTM for other candidate cells when the candidate cell becomes a serving cell. - The network can configure measurement reports e.g., L1 periodic, semi-persistent, aperiodic and event triggered report, or L3 measurement reports for conditional LTM, e.g., to trigger PDCCH ordered early RACH. - For CLTM, the Candidate Cell TCI States Activation/Deactivation MAC CE is re-used for the early activation/deactivation of TCI state(s) of a CLTM candidate configuration. - The Early TA is signalled to the UE from the source cell (i.e., not from the candidate cell directly to the UE). This agreement will be included in the LS to RAN1/3/4. - The network can inform the candidate cell’s TA information to UE via new MAC CE, which is the TA value when UE switches to that candidate cell during CLTM. - Candidate cell TA is maintained by a new timer. - For L1-based conditional LTM the condition evaluation is at MAC level and for L3-based conditional LTM the condition evaluation is at RRC level.

In this contribution, we will discuss the remaining open issues and further details of conditional LTM, including:

- The details for configuring the L1 execution conditions and L3 execution conditions

- The co-existence between LTM and C-LTM

- When should UE evaluate the execution conditions

- Which beam can be used by the UE for triggering the C-LTM cell switching

- Which beam can be used by the UE for accessing the C-LTM target cell

- Discussion

  - CLTM preparation

L1 executions

It has been agreed that it is DU to generate the L1 execution condition in RAN2#127bis. In R18 LTM, DU has already generated the LTM-CSI-ReportConfig for the LTM, so DU can directly configure the L1 execution condition in the LTM-CSI-ReportConfig. In this way, the UE can find the L1 measurement report configuration for LTM and the L1 execution condition for CLTM at the same time without decoding another IE.

Observation1: In R18 LTM, DU has already generated the LTM-CSI-ReportConfig for the LTM.

Proposal1: DU can configure the L1 execution condition in the LTM-CSI-ReportConfig.

L3 execution conditions

In the last R2#128 meeting, it was agreed that the L3 execution conditions are evaluated by the RRC layer.  Thus, it should be the source CU to generate the L3 execution conditions and the legacy L3 CHO measurement report configuration can be the baseline i.e. the CLTM L3 execution conditions and the CLTM candidate cell can both be configured in the legacy MeasConfig IE and they are also associated with a MeasId.

Observation2: It was agreed that the L3 execution conditions are evaluated by the RRC layer in the R2#128 meeting.

Proposal2: The CLTM L3 execution conditions and the CLTM candidate cell can both be configured in the legacy MeasConfig IE and they are also associated with a MeasId.

The co-existence between LTM and C-LTM

Since the LTM procedure in R18 has specified the configuration for the LTM candidate cell. It would be nice to reuse what has been done for LTM R18. Thus, the conditional LTM should be seen as an enhancement to the LTM. In this way, the conditional LTM candidate cells should be determined from the LTM candidate cells. The maximum number of the LTM candidate cells is eight and the number of the CLTM candidate cells should be less than or equal to that of the LTM candidate cells.

Observation3: The LTM procedure in R18 has specified the configuration for the LTM candidate cell.

Proposal3a: The LTM-Candidate IE is reused to model a CLTM candidate cell configuration.

Proposal3b: The CLTM candidate cells should be determined among the LTM candidate cells.

  - Conditional LTM Evaluation

When to evaluate the execution conditions

In legacy CHO, the UE evaluates the CHO execution conditions once UE receives them. In CLTM, since both the RACH-based and RACH-less CLTM are supported, the UE should evaluate the configurated L1 execution conditions and L3 execution conditions once the UE received the configuration.

Observation4a: In legacy CHO, the UE evaluates the CHO execution conditions once UE receives them.

Observation4b: Both the RACH-based and RACH-less CLTM are supported.

Proposal4: In CLTM, the UE should evaluate the configurated L1 execution conditions and L3 execution conditions once the UE received the configuration.

How to evaluate the execution condition

For the L3 execution condition, the L3 cell level measurement results should be used to trigger the cell switching i.e. the cell level measurement results of which CLTM candidate cell fulfills the execution condition, UE can trigger the cell switching to this CLTM candidate cell.

For the L1 execution condition, the L1 beam level measurement results can be used by UE for evaluating the trigger event. Any beam of a CLTM candidate cell that fulfills the execution condition can trigger the CLTM cell switching.

Proposal5: The L3 cell level measurement results should be used for evaluating L3 execution condition to trigger CLTM cell switching.

Proposal6: Any beam of a CLTM candidate cell that fulfills the L1 execution condition can trigger the CLTM cell switching.

  - Conditional LTM Execution

Which beam to perform the CLTM cell switch

In the R18 LTM, the network chooses an activated TCI state to be included in the cell switch command MAC CE or activate a TCI state in the cell switch command MAC CE. Then the UE can use the beam associated with the indicated TCI state for accessing the target cell. In CLTM, there is no cell switch command MAC CE, thus it’s UE to determine the beam for performing CLTM switching. Since the TCI state activating is not necessary before the cell switching, the UE can choose beams which are not associated with an activated TCI state. If the cell switching is triggered by the L3 execution conditions to a CLTM candidate cell, and there are activated TCI states for this CLTM candidate cell, UE select one of the activated TCI states for accessing the cell. However, if there is no activated TCI state for this CLTM candidate cell, then the L3 beam level measurement results of this CTLM candidate cell can be used by UE for selecting the best beam as the target beam for accessing the target candidate cell. If the cell switching is triggered by the L1 execution conditions to a CLTM candidate cell, and there are activated TCI states for this CLTM candidate cell, UE select one of the activated TCI states for accessing the cell. However, if there is no activated TCI state for this CLTM candidate cell, then the beam that fulfills the L1 execution condition can be the target beam.

Proposal7: If the CLTM cell switching is triggered by the L3 execution conditions, the beam used by UE for accessing the cell can have the following options:

Option A: UE selects the beam associated with an activated TCI state as the target beam for accessing the target candidate cell.

Option B: If there is no activated TCI state available, UE selects the best beam as the target beam for accessing the target candidate cell.

Proposal8: If the CLTM cell switching is triggered by the L1 execution conditions, the beam used by UE for accessing the cell can have the following options:

Option A: UE selects the beam associated with an activated TCI state as the target beam for accessing the target candidate cell.

Option B: If there is no activated TCI state available, UE selects the beam that fulfills the execution condition as the target beam for accessing the target candidate cell.

- Conclusion

In this contribution, we discussed the issues related to Conditional LTM. We have the following Observations and Proposals:

Observation1: In R18 LTM, DU has already generated the LTM-CSI-ReportConfig for the LTM.

Proposal1: DU can configure the L1 execution condition in the LTM-CSI-ReportConfig with some necessary modifications.

Observation2: It was agreed that the L3 execution conditions are evaluated by the RRC layer in the R2#128 meeting.

Proposal2: The CLTM L3 execution conditions and the CLTM candidate cell can both be configured in the legacy MeasConfig IE and they are also associated with a measId.

Observation3: The LTM procedure in R18 has specified the configuration for the LTM candidate cell.

Proposal3a: The LTM-Candidate IE is reused to model a CLTM candidate cell configuration.

Proposal3b: The CLTM candidate cells should be determined among the LTM candidate cells.

Observation4a: In CHO, the UE evaluates the CHO execution conditions once UE receives them.

Observation4b: Both the RACH-based and RACH-less CLTM are supported.

Proposal4: In CLTM, the UE should evaluate the configurated L1 execution conditions and L3 execution conditions once the UE received the configuration.

Proposal5: The L3 cell level measurement results should be used for evaluating L3 execution condition to trigger CLTM cell switching.

Proposal6: Any beam of a CLTM candidate cell that fulfills the L1 execution condition can trigger the CLTM cell switching.

Proposal7: If the CLTM cell switching is triggered by the L3 execution conditions, the beam used by UE for accessing the cell can have the following options:

Option A: UE selects the beam associated with an activated TCI state as the target beam for accessing the target candidate cell.

Option B: If there is no activated TCI state available, UE selects the best beam as the target beam for accessing the target candidate cell.

Proposal8: If the CLTM cell switching is triggered by the L1 execution conditions, the beam used by UE for accessing the cell can have the following options:

Option A: UE selects the beam associated with an activated TCI state as the target beam for accessing the target candidate cell.

Option B: If there is no activated TCI state available, UE selects the beam that fulfills the execution condition as the target beam for accessing the target candidate cell.

- References

- RAN2 #127bis chairman notes

- RAN2 #128 chairman notes