3GPP TSG-RAN WG2 Meeting #129						R2-2500170

Athens, Greece, Feb. 17th – 21st, 2025

Agenda item:	8.5.3

Source:	Samsung

Title:	On-demand SIB1

Document for:	Discussion and decision

# Introduction

RAN2 discussed procedures and signaling method(s) to support on-demand SIB1 for UEs in idle/inactive mode in RAN2#128 and following agreements were made:

- Periodic SIB1 transmission in NES Cell

  - NES UE with SIB1 request configuration of a NES cell assumes that a NES cell, with SSB containing K_SSB < 24 for FR1 and K_SSB < 12 for FR2, will acquire SIB1 as in legacy.

- Excluded-cell list and NES dedicated reselection priority

  - New NES-specific reselection priority parameters for NES UEs are defined for the purpose of prioritizing/deprioritizing a NES frequency.

  - Introduce new IntraFreqExcludedCellList-NES / InterFreqExcludedCellList-NES IEs enable proper reselection behaviour of legacy and NES UEs.

- Triggering condition for OD-SIB1

  - Reuse legacy cell reselection criterion as trigger condition of OD-SIB1 acquisition. No need to specify other conditions (e.g. a new RSRP threshold).

- RACH Failure for OD-SIB1

  - The UE considers the cell as barred after MAC indicates max number of preamble transmission for the OD-SIB1 request.

  - If MSG2 (ACK) is received, but UE fails to receive SIB1 then the UE may consider this cell as barred, no spec impact.

- Whether to alleviate 300s restriction after NES UE acquires UL WUS configuration

  - A UE bars the NES/SIB1 less cell and/or excludes it as a candidate for reselection since the UE had no corresponding UL WUS configuration, the UE would treat this cell as if cell status is “not barred” and consider it as candidate for cell reselection once it has received a UL-WUS configuration to request SIB1 for this cell.

- Validity of UL WUS configuration

  - UE considers WUS configuration only valid in directly succeeding cell reselection from cell where UE acquired WUS configuration. FFS on SI validity area rather than cell level.

- Handling of RRC connected UEs

  - Wait one cycle of RAN discussion (i.e. to see whether WID is updated or not to handle RRC connected UEs)

- UL WUS configuration by RRC Release

  - Working assumption: UL-WUS configuration in RRC release is not supported.

In this contribution we discuss further details of on-demand SIB1.

# Discussion

## 2.1. SIB1 request configuration validity

UE considers WUS configuration only valid in directly succeeding cell reselection from cell where UE acquired WUS configuration. FFS on SI validity area rather than cell level.

Scenario 1:

- UE is camped on Cell A. Cell A supports SIB X. SIB X includes SIB1 request configuration for multiples cells (e.g. Cell B, Cell C and Cell D).

- UE acquires SIB X transmitted by Cell A.

- UE reselects to Cell B from Cell A. Cell B provides SIB1 on demand. UE applies SIB1 request configuration of Cell B from SIB X of Cell A to acquire SIB1 of Cell A.

Scenario 2:

- UE is camped on Cell A. Cell A supports SIB X. SIB X includes SIB1 request configuration for multiples cells (e.g. Cell B, Cell C and Cell D).

- UE has a stored SIB X acquired from another cell (say Cell P). This stored SIB X is associated with same area id and value tag as the SIB X transmitted by Cell A. So, as per current principle, UE does not acquire SIB X transmitted by cell A.

- UE reselects to Cell B from Cell A. Cell B provides SIB1 on demand. Question is whether UE can apply SIB1 request configuration of Cell B from SIB X of Cell A to acquire SIB1 of Cell B even though SIB X was not directly acquired from Cell A. If we strictly follow the wording of current agreement, it seems that this is not allowed. In our view, this should be allowed as SIB X of Cell P and Cell A is same and UE is not required to acquire SIBX again in Cell A.

Proposal 1: UE considers SIB1 request configuration only valid in directly succeeding cell reselection to Cell P from Cell Q where the SIB1 request configuration is included in the SIB X of Cell Q. This SIB X of Cell Q is the SIB X acquired from Cell Q, if SIB X is cell specific. This SIB X of Cell Q is the SIB X acquired from Cell Q or any other cell with same system information area ID as the Cell Q, if SIB X is area specific.

## 2.2. SIB1 reception after SIB request

RAN2#126 agreement

- FFS if / when the UE monitors the OD-SIB1 upon reception of RAR

RAN2#127 agreement

- RAN2 assumes the UE is expected to receive the RAR responding to the preamble transmission for Msg1-based on-demand SIB1 procedure, as the baseline

Given the RAN2#127 agreement, it would be good to clarify that UE monitors/receive SIB1 after receiving the ack i.e. RAR responding to the preamble transmission for Msg1-based on-demand SIB1 procedure. Monitoring will be in window defined by RAN1.

Proposal 2: UE monitors/receive SIB1 after receiving the ack i.e. RAR responding to the preamble transmission for Msg1-based on-demand SIB1 procedure. Monitoring will be in window defined by RAN1.

## 2.3. UL carrier for SIB1 request

In legacy SI request is supported for both SUL and NUL. Cell which supports on demand SIB1 can be configured with both SUL and NUL. So SIB1 request should be supported for both SUL and NUL. UE should select the UL carrier before transmitting SIB1 request. Since SIB1 request is based on random access and UL carrier selection is performed at the initiation of random access procedure in legacy, both NUL and SUL can easily be supported for SIB1 request.

Not supporting SUL for SIB1 request has more specification effort:

- a) there is impact on random access procedure as exception (i.e. no UL carrier selection for the case random access procedure is initiated for SIB1 request) need to be defined in MAC spec.

- b) we need to specify that cell supporting SUL does not provide SIB1 on demand. The reason is that if cell supporting SUL provides SIB1 on demand and UE is not in NUL coverage, UE will not be able to acquire SIB1 in the absence of SIB1 request configuration for SUL.

Proposal 3: SIB 1 request is supported for both SUL and NUL.

## 2.4 Msg1 repetitions for SIB1 request

Msg1 repetitions is supported for UL coverage in R18 for random access (including Msg1 based SI request). RACH based SIB1 request (similar to Msg1 based SI request) is agreed for on-demand SIB1. So, it is reasonable to support Msg1 repetitions for SIB1 request for better UL coverage.

Proposal 4: Msg1 repetitions is supported for SIB1 request.

## 2.5 SIB1 request in RRC_CONNECTED

Figure 1

Input from RAN Plenary on RRC Connected mode

- The scope of WID will not be updated to include RRC Connected.

- However, RAN2 will address the specific case of RLF, to use the same procedure as idle/inactive to request OD-SIB1 transmission.  No need to change the scope of WI to indicate this.

Figure 1 illustrates the scenario of SIB1 request to the selected cell upon RLF:

Inorder for this procedure to work, we need to specify the following:

- UE supporting SIB1 request acquires and maintains SIB X in RRC_CONNECTED.

- RRC system information request needs to include SIB X

- RRC reconfiguration message needs to include SIB X

- Mechanism to validate that last acquired SIB X in current PCell is valid at the time SIB1 request is sent to cell selected for connection re-establishment.

  - Example, UE may send SIB1 request to Cell B, if the SIB1 request configuration acquired from Cell A includes SIB1 request configuration of Cell B and the SIB1 request configuration was acquired in the current modification period.

Proposal 5: RAN2 to discuss the following for SIB1 request to cell selected for connection re-establishment

- UE supporting SIB1 request acquires and maintains SIB X in RRC_CONNECTED.

- SIB X can be requested using RRC system information request in RRC_CONNECTED

- SIB X can be delivered to UE using RRC reconfiguration message in RRC_CONNECTED (for the case SIB X cannot be broadcasted in active DL BWP)

- Mechanism to validate that last acquired SIB X in current PCell is valid at the time SIB1 request is sent to cell selected for connection re-establishment.

In the RRC CONNECTED while T311 is not running, UE needs SIB1 to acquire other SIBs (pink highlighted conditions below). This was discussed in last meeting and companies thought that WID needs to be updated for SIB1 request in connected. Given that we will define SIB1 request in connected for the case T311 is running without any WID update, the above argument is not valid any more. SIB request is supported in RRC_CONNECTED using DedicatedSIBRequest message. SIB1 request can be easily supported by adding a new code point for SIB1 in DedicatedSIBRequest.

“5.2.2.3.1              Acquisition of MIB and SIB1

1> if the UE is in RRC_CONNECTED with an active BWP with common search space configured by searchSpaceSIB1 and pagingSearchSpace and has received an indication about change of system information; or

1> if the UE is in RRC_CONNECTED with an active BWP with common search space configured by searchSpaceSIB1 and the UE has not stored a valid version of a SIB or posSIB, in accordance with clause 5.2.2.2.1, of one or several required SIB(s) or posSIB(s) in accordance with clause 5.2.2.1, and, UE has not acquired SIB1 in current modification period; or

1> if the UE is in RRC_CONNECTED with an active BWP with common search space configured by searchSpaceSIB1, and, the UE has not stored a valid version of a SIB or posSIB, in accordance with clause 5.2.2.2.1, of one or several required SIB(s) or posSIB(s) in accordance with clause 5.2.2.1, and, si-BroadcastStatus for the required SIB(s) or posSI-BroadcastStatus for the required posSIB(s) is set to notBroadcasting in acquired SIB1 in current modification period; or

:

2> if ssb-SubcarrierOffset indicates SIB1 is transmitted in the cell (TS 38.213 [13]) and if SIB1 acquisition is required for the UE:

3> acquire the SIB1, which is scheduled as specified in TS 38.213 [13];

“

Proposal 6: DedicatedSIBRequest is updated to include request for SIB1 in RRC_CONNECTED.

## 2.6 SSB associated with on-demand SIB1 is CD-SSB or NCD-SSB

TS 38.330

“

Within the frequency span of a carrier, multiple SSBs can be transmitted. The PCIs of SSBs transmitted in different frequency locations do not have to be unique, i.e. different SSBs in the frequency domain can have different PCIs. However, when an SSB is associated with an RMSI, the SSB is referred to as a Cell-Defining SSB (CD-SSB). A PCell is always associated to a CD-SSB located on the synchronization raster.

When an SSB is not associated with an RMSI, the SSB is referred to as a non-Cell Defining SSB (NCD-SSB), which can be used to perform RLM, BFD, and RRM measurements and measurements for RA resource selection inside the active DL BWP when the active BWP does not contain the CD-SSB. A UE may be configured with multiple SSBs provided that each BWP is configured with at most one SSB (CD-SSB or NCD-SSB).

“

RAN1 has agreed that  kSSB>23 for FR1 or kSSB>11 for FR2 is used in MIB of a cell supporting on-demand SIB1. Based on this information, UE supporting on demand SIB1 can know that cell supports on demand SIB1 and can obtain the SIB1 from the cell upon SIB1 request. Given that cell supports SIB1 and provides the SIB1, SSB associated with on-demand SIB1 should be considered as CD-SSB.

Proposal 7: SSB associated with on-demand SIB1 is considered as CD-SSB

# Conclusion

Proposal 1: UE considers SIB1 request configuration only valid in directly succeeding cell reselection to Cell P from Cell Q where the SIB1 request configuration is included in the SIB X of Cell Q. This SIB X of Cell Q is the SIB X acquired from Cell Q, if SIB X is cell specific. This SIB X of Cell Q is the SIB X acquired from Cell Q or any other cell with same system information area ID as the Cell Q, if SIB X is area specific.

Proposal 2: UE monitors/receive SIB1 after receiving the ack i.e. RAR responding to the preamble transmission for Msg1-based on-demand SIB1 procedure. Monitoring will be in window defined by RAN1.

Proposal 3: SIB 1 request is supported for both SUL and NUL.

Proposal 4: Msg1 repetitions is supported for SIB1 request.

Proposal 5: RAN2 to discuss the following for SIB1 request to cell selected for connection re-establishment

- UE supporting SIB1 request acquires and maintains SIBX in RRC_CONNECTED.

- SIB X can be requested using RRC system information request in RRC_CONNECTED

- SIB X can be delivered to UE using RRC reconfiguration message in RRC_CONNECTED (for the case SIB X can not be broadcasted in active DL BWP)

- Mechanism to validate that last acquired SIBX in current PCell is valid at the time SIB1 request is sent to cell selected for connection re-establishment.

Proposal 6: DedicatedSIBRequest is updated to include request for SIB1 in RRC_CONNECTED.

Proposal 7: SSB associated with on-demand SIB1 is considered as CD-SSB

# References