3GPP TSG-RAN WG2 Meeting #129						R2-2500172

Athens, Greece, Feb. 17th – 21st, 2025

Agenda item:	8.5.4

Source:	Samsung

Title:	Adaptation of common signal transmissions

Document for:	Discussion and Decision

# Introduction

In this contribution we discuss the RAN2 aspects of Paging clustering/bundling and PRACH adaptation in time domain.

# Discussion

## 2.1. Paging clustering/bundling

RAN2 has agreed option-b (i.e. Extension of N and Ns) as baseline for R19 NES paging enhancement for paging clustering/bundling.

In option-b POs are clustered at the beginning of DRX cycle by extending the values of N to have increased interval between PFs (e.g. T/32, T/64, T/128 ...) and compensating decrease in number of PFs by increasing POs per PF. For example, let’s say 32 POs are needed in a DRX cycle to meet the paging load in a cell. Network can configure 1 PF in DRX cycle and 32 POs per PF.

If cell supports paging clustering/bundling,

- it configures new parameters N’ and Ns’ for indicating number of paging frames and number of POs for UEs supporting paging clustering/bundling.

- Starting PDCCH monitoring occasion number (firstPDCCH-MonitoringOccasionOfPO’) of PO is also configured separately for paging clustering/bundling. Starting PDCCH monitoring occasion number of PO is optional as in legacy.

- PF_Offset’ is also configured separately for paging clustering/bundling for flexible allocation of PF with paging clustering/bundling and legacy PF.

Legacy UEs in the cell will apply legacy parameters N, Ns, PF_Offset and firstPDCCH-MonitoringOccasionOfPO (if configured) whereas NES UEs supporting paging clustering/bundling applies N’, Ns’. PF_Offset’ and firstPDCCH-MonitoringOccasionOfPO’.

Proposal 1: If cell supports paging clustering/bundling,

- it configures N’ and Ns’ for indicating number of paging frames and number of POs for UEs supporting paging clustering/bundling.

- if may configure starting PDCCH monitoring occasion number (firstPDCCH-MonitoringOccasionOfPO’) of PO for UEs supporting paging clustering/bundling. Starting PDCCH monitoring occasion number of PO is optional as in legacy.

- it configures new parameter PF_Offset’ for UEs supporting paging clustering/bundling.

RAN2 has agreed the following in last meeting for the extended values of N and Ns

- Introduce value for Ns=8. FFS on 16.

- Introduce value for N= T/32. FFS on T/64, T/128, T/256.

In order to maximize network energy savings, we prefer to support longer intervals between PFs and more POs per PF. So, we prefer to support a value of 16 for Ns and N = T/64, T/128, T/256.

Proposal 2: Support Ns = 16 and N= T/64, T/128, T/256 for paging clustering/bundling.

UE specific DRX cycle can be smaller or larger than default DRX cycle. Due to clustering of POs at the beginning of DRX cycle (i.e. default DRX cycle), DRX cycle T used by UE for PF/PO determination should be max (default DRX cycle, UE specific DRX cycle) or equal to default DRX cycle.

Proposal 3: For paging clustering/bundling, DRX cycle T used by UE for PF/PO determination is equal to default DRX cycle.

If UE specific DRX cycle is smaller than default DRX cycle, clustering of POs may result in increased paging latency for UE. To overcome this, following approach can be considered:

- if UE specific DRX cycle is smaller than default DRX cycle,

  - UE monitors paging using legacy configuration.

- Else

  - UE monitors paging using new configuration (i.e. configuration for paging adaptation)

Proposal 4: if UE specific DRX cycle is smaller than default DRX cycle, UE monitors paging using legacy configuration. Otherwise, UE monitors paging using new configuration (i.e. configuration for paging adaptation)

## 2.2. PRACH adaptation in time domain

For network energy savings, legacy RACH resources can be configured sparsely and additional RACH resources can be configured dynamically as and when needed. RAN1 has discussed configuration of additional RACH resources and signaling to (de)activate additional RACH resources.

RAN1#118b Agreements

- For adaptation of PRACH in time-domain, the same PRACH preamble format is used for the additional RACH resources and legacy PRACH resources.

- For configuration of additional PRACH resources, support both of the following:

  - Alt 1: The PRACH configuration index for the additional PRACH resources is same as the PRACH configuration index for the legacy resources

  - Alt 2: The PRACH configuration index for the additional PRACH resources is different from the PRACH configuration index for the legacy resources

  - For each of the alternatives, (jointly or independently) select one or more from:

    - Opt 1-2a: up to N1 additional value(s) of (x, y)

    - Opt 1-2b: up to N2 additional value(s) of y

    - Opt 1-3: Muting/masking ROs

    - Opt 1-4:

    - up to N4_1 additional timing offset(s) at frame-level

    - up to N4_2 additional timing offset(s) at slot-level

    - Opt 1-5: No additional mechanism is selected.

    - Note: x and y refer to the parameters from the random-access configuration tables (from 38.211)

- For adaptation of PRACH in time-domain, the frequency domain resources for the additional PRACH resources and legacy PRACH resources can be same or different

  - FFS: applicable case(s) (i.e. case(s) from the RAN1#117 agreement).

  - Discuss further following options for signaling

    - Option 1: at least the following parameter(s) can be configured separately for the additional PRACH resources.

      - msg1-FrequencyStart at least for 4-step RACH

      - FFS: other applicable legacy frequency domain parameter(s)

    - Option 2: Offset to legacy frequency domain parameter(s) are configured for the additional PRACH resources

      - Note: Offset to legacy frequency domain parameter(s) is a new parameter

      - FFS: applicable legacy frequency domain parameter(s)

- (WA) For DCI-based adaptation for additional PRACH resources, at least DCI format 1_0 can carry the adaptation indication for UEs in idle/inactive and connected mode.

  - P-RNTI is used

RAN1#119 Agreements

- Frequency domain configuration:

  - At least msg1-FrequencyStart can be configured separately for the additional PRACH resources at least for 4-step RACH.

- Adaptation mechanism:

  - Alt 1: DCI-based adaptation to indicate whether the additional PRACH resources provided by semi-static signalling are available or not

    - [DCI payload size = 1 bit]

    - FFS: A single PRACH mask provided by semi-static signalling is used to identify the subset of the additional PRACH resources

- Bits in DCI format 1_0 for carrying adaptation indication:

  - Option 1: Use reserved bits in the DCI format

  - Option 2: Use Bits 5-8 within the Short Message (from upper layers)

  - Option 3: Use bits available for both Option 1 and Option 2

Discussion

Based on the above agreements,  following configuration are allowed for random access configuration/random access resource set

- Separate PRACH configuration index for the additional PRACH resources and legacy RACH resources + msg1-FrequencyStart is same for legacy RACH resources and additional PRACH resources

- Separate PRACH configuration index for the additional PRACH resources and legacy RACH resources + separate msg1-FrequencyStart for the legacy RACH resources and additional PRACH resources

- Same PRACH configuration index for the additional PRACH resources and legacy RACH resources + msg1-FrequencyStart is same for legacy RACH resources and additional PRACH resources

- Same PRACH configuration index for the additional PRACH resources and legacy RACH resources + separate msg1-FrequencyStart for the legacy RACH resources and additional PRACH resources

Further details of enhancing the configuration in random-access configuration table (such as values of x, y, offsets etc) indicated by PRACH configuration index is FFS.

prach-ConfigurationIndex and msg1-FrequencyStart are signalled in RACH-ConfigGeneric IE. So, it’s straightforward to include additional RACH resources configuration in RACH-ConfigGeneric IE. An example is shown below. An alternate would be to signal a seperate RACH-ConfigCommon for additional RACH resources. However, this is unnecessary overhead as it would lead to duplicate signalling of several random access parameters.

RACH-ConfigGeneric information element

RACH-ConfigGeneric ::=              SEQUENCE {

prach-ConfigurationIndex            INTEGER (0..255),

msg1-FDM                            ENUMERATED {one, two, four, eight},

msg1-FrequencyStart                 INTEGER (0..maxNrofPhysicalResourceBlocks-1),

zeroCorrelationZoneConfig           INTEGER(0..15),

preambleReceivedTargetPower         INTEGER (-202..-60),

preambleTransMax                    ENUMERATED {n3, n4, n5, n6, n7, n8, n10, n20, n50, n100, n200},

powerRampingStep                    ENUMERATED {dB0, dB2, dB4, dB6},

ra-ResponseWindow                   ENUMERATED {sl1, sl2, sl4, sl8, sl10, sl20, sl40, sl80},

...,

[[

prach-ConfigurationPeriodScaling-IAB-r16    ENUMERATED {scf1,scf2,scf4,scf8,scf16,scf32,scf64}              OPTIONAL,   -- Need R

prach-ConfigurationFrameOffset-IAB-r16      INTEGER (0..63)                                                 OPTIONAL,   -- Need R

prach-ConfigurationSOffset-IAB-r16          INTEGER (0..39)                                                 OPTIONAL,   -- Need R

ra-ResponseWindow-v1610                     ENUMERATED { sl60, sl160}                                       OPTIONAL,   -- Need R

prach-ConfigurationIndex-v1610              INTEGER (256..262)                                              OPTIONAL    -- Need R

]],

[[

ra-ResponseWindow-v1700                     ENUMERATED {sl240, sl320, sl640, sl960, sl1280, sl1920, sl2560} OPTIONAL    -- Need R

]],

[[

additionalROConfig			 SEQUENCE {

prach-ConfigurationIndex            INTEGER (0..255), OPTIONAL    -- Need R

msg1-FrequencyStart    INTEGER (0..maxNrofPhysicalResourceBlocks-1), OPTIONAL    -- Need R

// other parameters, if any

} OPTIONAL    -- Need R

]]

}

Proposal 5: Introduce prach-ConfigurationIndex and msg1-FrequencyStart for additional rach resouces in RACH-ConfigGeneric IE.

PRACH adpatation in time domain for 2 step RACH

From RAN1 agreements it is not clear whether the PRACH adpatation in time domain is supported only for 4 step RACH or supported for both 2 step RACH and 4 step RACH. RAN2 can send LS to RAN1 or wait for further progress in RAN1 for PRACH adpatation in time domain for 2 step RACH.

Proposal 6: RAN2 to wait for further progress in RAN1 for PRACH adpatation in time domain for 2 step RACH.

PRACH adpatation in time domain for NUL or SUL or both

RAN1 is discussing a general design for PRACH adpatation in time domain. There is no difference in random access configuration for SUL and NUL. So there is no reason to exclude PRACH adpatation in time domain for either carrier. Network can configure additional RACH resources in random access resource configuration of SUL or random access resource configuration of NUL or both.

Proposal 7: PRACH adpatation in time domain is supported for both NUL and SUL.

PRACH adpatation in time domain for feature/feature combination specific random access resources

In the current design random access resources are configured per random access resource configuration/random access resource set. Several random access resource configurations/random access resource sets can be configured where per BWP. Each of these random access resource configurations/random access resource sets are mapped to different feature/feature combination. One of these random access resource configurations/random access resource sets may be not mapped to any feature/feature combination.

By PRACH adpatation in time domain network can control the density of ROs used for random access dynamically. This is beneficial for any random access resoure configuration.

Proposal 8: PRACH adpatation in time domain is supported for feature/feature combination specific random access resources.

RAN1 has agreed that for DCI-based adaptation for additional PRACH resources, at least DCI format 1_0 can carry the adaptation indication for UEs in idle/inactive and connected mode. This DCI is paging DCI and transmitted in PO and addressed to P-RNTI.

Inorder to receive this, UE needs to monitor PO in RRC_CONNECTED state. However, PO is monitored only if paging search space is configured in active UL BWP. In case paging search space is not configured in active UL BWP, UE cannot receive the DCI format 1_0. So other mechanism (such RRC Reconfiguration message or MAC CE or group DCI) for adaptation for additional PRACH resources may be needed.

Proposal 9: RAN2 to discuss and agree one of the following for RRC_CONNECTED

Option 1: if adaptation for additional PRACH resources is supported in active UL BWP, network ensures that paging search space is configured in active DL BWP.

Option 2: Introduce other mechanism (such RRC Reconfiguration message or MAC CE or group DCI) for adaptation for additional PRACH resources in RRC_CONNECTED.

If paging search space is configured in active UL BWP, UE can monitor PO in RRC_CONNECTED for DCI-based adaptation for additional PRACH resources. However, current requirement is that UE monitors any PO at least once every modification period. If adaptation faster than modification period is needed, this will not work. Alternate would be that for DCI-based adaptation for additional PRACH resources in RRC_CONNECTED, UE monitors any PO at least once every defaultPagingCycle similar to emergency notifications.

Proposal 10: RAN2 to discuss and agree one of the following for RRC_CONNECTED

Option 1: For DCI-based adaptation indication for additional PRACH resources in RRC_CONNECTED, UE monitors the indication in any paging occasion at least once every defaultPagingCycle.

Option 2: For DCI-based adaptation for additional PRACH resources in RRC_CONNECTED, UE monitors the indication in any paging occasion at least once every modification period.

# Conclusion

Proposal 1: If cell supports paging clustering/bundling,

- it configures N’ and Ns’ for indicating number of paging frames and number of POs for UEs supporting paging clustering/bundling.

- if may configure starting PDCCH monitoring occasion number (firstPDCCH-MonitoringOccasionOfPO’) of PO for UEs supporting paging clustering/bundling. Starting PDCCH monitoring occasion number of PO is optional as in legacy.

- it configures new parameter PF_Offset’ for UEs supporting paging clustering/bundling.

Proposal 2: Support Ns = 16 and N= T/64, T/128, T/256 for paging clustering/bundling.

Proposal 3: For paging clustering/bundling, DRX cycle T used by UE for PF/PO determination is equal to default DRX cycle.

Proposal 4: if UE specific DRX cycle is smaller than default DRX cycle, UE monitors paging using legacy configuration. Otherwise, UE monitors paging using new configuration (i.e. configuration for paging adaptation)

Proposal 5: Introduce prach-ConfigurationIndex and msg1-FrequencyStart for additional rach resouces in RACH-ConfigGeneric IE.

Proposal 6: RAN2 to wait for further progress in RAN1 for PRACH adpatation in time domain for 2 step RACH.

Proposal 7: PRACH adpatation in time domain is supported for both NUL and SUL.

Proposal 8: PRACH adpatation in time domain is supported for feature/feature combination specific random access resources.

Proposal 9: RAN2 to discuss and agree one of the following for RRC_CONNECTED

Option 1: if adaptation for additional PRACH resources is supported in active UL BWP, network ensures that  paging search space is configured in active DL BWP.

Option 2: Introduce other mechanism (such RRC Reconfiguration message or MAC CE or group DCI) for adaptation for additional PRACH resources in RRC_CONNECTED.

Proposal 10: RAN2 to discuss and agree one of the following for RRC_CONNECTED

Option 1: For DCI-based adaptation indication for additional PRACH resources in RRC_CONNECTED, UE monitors the indication in any paging occasion at least once every defaultPagingCycle

Option 2: For DCI-based adaptation for additional PRACH resources in RRC_CONNECTED, UE monitors the indication in any paging occasion at least once every modification period

# References