3GPP TSG-RAN WG2 Meeting #129						R2-2500173

Athens, Greece, Feb. 17th – 21st, 2025

Agenda item:	7.2

Source:	Samsung

Title:	Corrections related to random access for LTM

Document for:	Discussion and Decision

# Introduction

This contribution discusses some corrections related to random access for LTM.

# Discussion

## 2.1 CFRA configuration handling

LTM configuration of a candidate cell (say Cell A) includes rach-ConfigDedicated in Reconfiguration with sync IE. UE then receives LTM cell switch command to switch to Cell A.

Case 1:  UE performs RACH based LTM Cell switch

Upon successful completion of random access procedure, UE releases configured contention-free random access resources in rach-ConfigDedicated as per the below procedure in TS 38.321.

“

5.1.6   Completion of the Random Access procedure

Upon completion of the Random Access procedure, the MAC entity shall:

- discard any explicitly signalled contention-free Random Access Resources for 2-step RA type and 4-step RA type except the 4-step RA type contention-free Random Access Resources for beam failure recovery request, if any;

“

Case 2: LTM cell switch failure

UE releases configured contention-free random access resources in rach-ConfigDedicated as per the below procedure in TS 38.331.

“

5.3.5.8.3      T304 expiry (Reconfiguration with sync Failure) or T420 expiry (Path switch failure)

The UE shall:

1> if T304 of the MCG expires; or

1> if T420 expires; or,

1> if the target L2 U2N Relay UE (i.e., the UE indicated by targetRelayUE-Identity in the received RRCReconfiguration message containing reconfigurationWithSync indicating path switch as specified in 5.3.5.5.2) changes its serving PCell before path switch:

2> release dedicated preambles provided in rach-ConfigDedicated if configured;

2> release dedicated msgA PUSCH resources provided in rach-ConfigDedicated if configured;

“

Case 3:  UE performs RACH less LTM Cell switch

Network may configure contention free random access resources via RRC but UE may not use it if UE has valid TA available. If UE has valid TA available it performs RACH less LTM cell switch.

Upon successful completion of cell switch, UE should release configured contention-free Random Access Resources in rach-ConfigDedicated. This behaviour seems missing in current spec.

One point of view can be that configured CFRA resources can be used for subsequent LTM. If so, it’s not clear why we are releasing the configuration in case 1 and case 2.

To have consistent behaviour across all cases, we propose to specify that upon successful completion of cell switch, UE releases configured contention-free Random Access Resources in rach-ConfigDedicated if configured in the reconfigurationWithSync.

Proposal 1: upon successful completion of cell switch, UE releases configured contention-free Random Access Resources in rach-ConfigDedicated if configured in the reconfigurationWithSync. Adopt the TP in annexure 1.

## 2.2 Random access resource set selection for early TA

LTM-Candidate-r18 ::=     SEQUENCE {

ltm-CandidateId-r18                            LTM-CandidateId-r18,

ltm-CandidatePCI-r18                           PhysCellId                                            OPTIONAL,    -- Need M

ltm-SSB-Config-r18                             LTM-SSB-Config-r18                                    OPTIONAL,    -- Need M

ltm-CandidateConfig-r18                        OCTET STRING (CONTAINING RRCReconfiguration)          OPTIONAL,    -- Need M

ltm-ConfigComplete-r18                         ENUMERATED {true}                                     OPTIONAL,    -- Need R

ltm-EarlyUL-SyncConfig-r18                     OCTET STRING (CONTAINING EarlyUL-SyncConfig-r18)      OPTIONAL,    -- Need R

ltm-EarlyUL-SyncConfigSUL-r18                  OCTET STRING (CONTAINING EarlyUL-SyncConfig-r18)      OPTIONAL,    -- Need R

ltm-TCI-Info-r18                               LTM-TCI-Info-r18                                      OPTIONAL,    -- Need M

ltm-NoResetID-r18                              INTEGER (1..maxNrofLTM-Configs-plus1-r18)             OPTIONAL,    -- Need M

ltm-UE-MeasuredTA-ID-r18                       INTEGER (1..maxNrofLTM-Configs-plus1-r18)             OPTIONAL,    -- Need M

...

}

EarlyUL-SyncConfig-r18 ::=         SEQUENCE {

frequencyInfoUL-r18                FrequencyInfoUL,

rach-ConfigGeneric-r18             RACH-ConfigGeneric,

bwp-GenericParameters-r18          BWP,

ssb-PerRACH-Occasion-r18           ENUMERATED {oneEighth, oneFourth, oneHalf, one, two, four, eight, sixteen} OPTIONAL, -- Need R

prach-RootSequenceIndex-r18        CHOICE {

l839                               INTEGER (0..837),

l139                               INTEGER (0..137)

}                                                                                                             OPTIONAL, -- Need R

ltm-PRACH-SubcarrierSpacing-r18    SubcarrierSpacing                                                          OPTIONAL, -- Cond L139

n-TimingAdvanceOffset-r18          ENUMERATED { n0, n25600, n39936, spare1 }                                  OPTIONAL, -- Need R

...,

[[

ltm-tdd-UL-DL-ConfigurationCommon-r18 TDD-UL-DL-ConfigCommon                                                  OPTIONAL, -- Cond TDD

ltm-restrictedSetConfig-r18        ENUMERATED {unrestrictedSet, restrictedSetTypeA, restrictedSetTypeB}       OPTIONAL  -- Need R

]]

}

EarlyUL-SyncConfig-r18 includes random access resource configuration. It is separately signalled for SUL and NUL. UL carrier to be used for early TA is indicated in PDCCH order.

Generally during the random access procedure, UE select set of Random Access resources configured in the selected BWP for the random access procedure. However, there is no BWP selected for early TA. When random access procedure for early TA is initiated, UE selects the set of Random Access resources configured in EarlyUL-SyncConfig as per the below procedure. The issue with this procedure is that it is not clear which EarlyUL-SyncConfig is selected by UE as there can be two EarlyUL-SyncConfig IEs, one each for SUL and NUL.

5.1.1b	Selection of the set of Random Access resources for the Random Access procedure

:

1>	else if this Random Access procedure is initiated by PDCCH order for an LTM candidate cell:

2>	select the set of Random Access resources configured in EarlyUL-SyncConfig corresponding to the cell indicated by the field Cell indicator in the PDCCH order as specified in TS 38.212 [9].

We propose to correct the procedure as follows:

5.1.1b	Selection of the set of Random Access resources for the Random Access procedure

:

1>	else if this Random Access procedure is initiated by PDCCH order for an LTM candidate cell:

2>	select the set of Random Access resources configured in EarlyUL-SyncConfig of the selected UL carrier, corresponding to the cell indicated by the field Cell indicator in the PDCCH order as specified in TS 38.212 [9].

Proposal 2: Update the TS 38.321 as follows:

5.1.1b	Selection of the set of Random Access resources for the Random Access procedure

:

1>	else if this Random Access procedure is initiated by PDCCH order for an LTM candidate cell:

2>	select the set of Random Access resources configured in EarlyUL-SyncConfig of the selected UL carrier, corresponding to the cell indicated by the field Cell indicator in the PDCCH order as specified in TS 38.212 [9].

# Conclusion

Proposal 1: upon successful completion of cell switch, UE releases configured contention-free Random Access Resources in rach-ConfigDedicated if configured in the reconfigurationWithSync. Adopt the TP in annexure 1.

Proposal 2: Update the TS 38.321 as follows:

5.1.1b	Selection of the set of Random Access resources for the Random Access procedure

:

- else if this Random Access procedure is initiated by PDCCH order for an LTM candidate cell:

2>	select the set of Random Access resources configured in EarlyUL-SyncConfig of the selected UL carrier, corresponding to the cell indicated by the field Cell indicator in the PDCCH order as specified in TS 38.212 [9].

# Annexure 1

5.3.5.3   Reception of an RRCReconfiguration by the UE

:

1> if reconfigurationWithSync was included in spCellConfig of an MCG or SCG and when MAC of an NR cell group successfully completes a Random Access procedure triggered above; or,

1> if sl-PathSwitchConfig was included in reconfigurationWithSync included in spCellConfig of an MCG, and when successfully sending RRCReconfigurationComplete message (i.e., PC5 RLC acknowledgement is received from target L2 U2N Relay UE); or,

1> if rach-LessHO was included in reconfigurationWithSync included in spCellConfig of an MCG, and upon indication from lower layers that the RACH-less handover has been successfully completed; or,

1> if reconfigurationWithSync was included in spCellConfig of an MCG or SCG and the RRCReconfiguration message is applied due to an LTM cell switch execution and upon an indication from lower layer that the LTM cell switch execution has been successfully completed:

2> release dedicated preambles provided in rach-ConfigDedicated if configured in the reconfigurationWithSync;

2> release dedicated msgA PUSCH resources provided in rach-ConfigDedicated if configured in the reconfigurationWithSync;