3GPP TSG RAN WG2 Meeting #129	R2-2500174

Athens, Greece  17-21 Feb 2025	(revision of R2-2409907)

Agenda Item:	8.1.2.3 LCM for Positioning use case

Source:	Fraunhofer IIS, Fraunhofer HHI

Title:	Association of measurements and ground truth labels for positioning use-cases

Document for:	Discussion & Decision

# Introduction

The WID [1] , lists, among others, the following as objective for RAN2:

- Positioning accuracy enhancements, encompassing [RAN1/RAN2/RAN3]: - Direct AI/ML positioning: - (1st priority) Case 1: UE-based positioning with UE-side model, direct AI/ML positioning - (2nd priority) Case 2b: UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning - (1st priority) Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning - AI/ML assisted positioning - (2nd priority) Case 2a: UE-assisted/LMF-based positioning with UE-side model, AI/ML assisted positioning - (1st priority) Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning - Specify necessary measurements, signalling/mechanism(s) to facilitate LCM operations specific to the Positioning accuracy enhancements use cases, if any - Investigate and specify the necessary signalling of necessary measurement enhancements (if any) - Enabling method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE for relevant positioning sub use cases

In RAN2#127bis, we reached the following agreements:

Agreements: 1: The following procedures for LCM for UE sided model for AI positioning case 1 is the baseline: Step 1: LMF may request the UE to report the supported functionalities at the UE side by LPP request capabilities message. Step 2: UE sends LPP provide capabilities message to LMF with the supported functionalities at the UE side. Step 3: LMF sends the LPP provide assistance data message (which may contain network side additional condition). Step 4: UE reports the applicable functionality to the LMF by the LPP provide capabilities message. Step 5: The LMF requests the inferred location information using the LPP request location information message. Step 6: UE reports the inferred location using LPP provide location information message. 2: Whether the inference configuration is provided in step 3 or/and step 5 is FFS (to be revised based on RAN1 progress). 3: Whether network side additional condition is needed and what it contains is FFS (to be revised based on RAN1 progress). 4: FFS whether LMF controls the UE sending unsolicited LPP provide capabilities (i.e. whether step4 is sent reactively or proactively). FFS the signalling details. 5: RAN2 will decide whether AI positioning will be a new method after further details from RAN1 are received.

In this contribution, we discuss the importance of acquiring accurate ground truth labels for AI/ML model to function, and why the labels acquired using the currently supported positioning methods are do not provide enough high quality labels for training and monitoring AI/ML models.

In order to have an AI/ML model that provides good inference results, it becomes imperative that there are sufficient labels collected with good quality, even in challenging scenarios. It would not be practical to deploy an extremely high number of PRUs, which would  have enabled collection of a large number of high quality labels and measurements. Therefore, we have to rely also on UEs that are not deployed as PRUs to provide labels.

Unfortunately, it is well known that GNSS is generally unavailable indoors, and in scenarios that are dominated with NLOS and heavy multipath, the labels acquired with existing RAT dependent positioning methods are inaccurate. This inadvently leads to the situation that the high quality labels are collected in the area where where GNSS or RAT dependent methods are already accurate enough. This means, we do not have enough high quality data labels in the areas where AI/ML is envisaged to improve positioning performance. Training the AI/ML models using channel measurements that are associated mainly with inaccurate labels will lead to inference results that are also inaccurate, when the AI/ML model is deployed.

Therefore, the use of an external source of ground truth for acquiring labels, and association to the measurements reported in the ProvideLocationInformation becomes necessary.

To achieve this goal, the ProvideAssistanceData information may be used to provide assistance information to the UE to assist the UE in identifying sources of groundtruth, which can be used to determine ground truth labels in scenarios where existing positioning methods cannot be relied upon to provide good quality labels.

The labels may be collected from the UEs subject to their capabilities and consent in detecting and processing external source of information, such as QR code, RFID tags, landmarks etc.

Observation 1: Using existing positioning methods specified in 3GPP as source of labels does provides inaccurate labels in training data in the very scenarios where the AI/ML methods (NLOS heavy, multipath dominated) are supposed to perform better than existing methods.

Proposal 1: UE location for the purpose of ground truth shall be determined  using secondary sources (e.g. camera-based), which different from GNSS or RAT dependent methods for UEs that are not provisioned as PRUs (“non-PRU UEs”).

Proposal 2: RAN2 shall discuss association between channel measurement and ground truth for each of the positioning use case, including indication of the method used to obtain the ground truth label.

# LCM for UE-based positioning with UE-side model, direct AI/ML positioning (Case 1)

## Ground truth label

RAN1 agreed in RAN1#116bis the following regarding data collection, separating the data collection into two components – Part A (which pertains to radio measurements) and Part B (which pertains to ground truth).

RAN1 agreements:

For training data collection of AI/ML based positioning, the collected data sample can include the following components:

Part A:

•	channel measurement

•	quality indicator of channel measurement

•	time stamp of channel measurement

Part B:

•	ground truth label (or its approximation)

•	quality indicator of label

•	time stamp of label

Note: "Part A" and "Part B" terminologies are only for RAN1 discussion purpose, and may not be used in specification.

Note: contents in Part A and Part B may or may not be generated by different entities.

Note: Part A and/or Part B, and their contents may or may not apply for each case

FFS: detailed definition of channel measurement

For Case 1 (UE-based positioning with UE-side model, direct AI/ML positioning), this means that

- Training is based on channel measurements and UE position (label) both generated by the UE.

- Training is based on channel measurements generated by the UE and the UE position (label) generated by the LMF.

Both training an AI/ML model and monitoring the performance of the AI/ML model requires high quality ground truth labels associated with it.

Especially in indoor scenarios, it is challenging to get accurate ground truth labels, because GNSS does not work indoors, and RAT dependent and RAT-independent methods currently specified cannot deliver high quality labels in scenarios dominated by heavy multipath.

Of course, the location of UE may be well known when they are commissioned as PRUs in a fixed location. However, this limits the training and monitoring information to the sub-set of UEs that are stationary and at deployed at certain location.

Observation 2: Use of PRU type UEs limit the data collected to static UEs and spatially limited to where the PRUs are provisioned.

Use of non-PRU type UEs enable data collection with variety of UE speeds and UE locations.

As described above, the UE position obtained by UE-based or LMF-based methods may be available for a non-PRU UE. However, the labels thus obtained may not necessarily be reliable.  Therefore, it may be beneficial to utilize other sensor information available at the UE.

Supporting landmarks in the 5G AI/ML framework can be crucial for enabling data collection and monitoring, particularly in indoor scenarios where landmarks can be the only mechanism to obtain labeled data. In this context, these landmarks are associated with specific SRS transmissions or PRS measurements. It is currently unclear if the existing framework adequately supports these mechanisms.

Figure 1-  A landmark at Frankfurt Airport's wall can serve as a reference point for such DL PRS data collection or monitoring.

Observation 3: In indoor scenarios, landmarks can be the only mechanism to obtain labeled data, enabling the Network/UE to identify and trigger the need for labels in specific areas.

Based on UE capabilities, the UE may be equipped with additional sensors, such as Lidar, camera, or radar sensors. The UE may be configured to search for certain features containing the ground truth labels, such as landmarks, which may be QR codes, visible markers, orientation markers and so on. For example, an autonomous guided vehicle in an indoor factory scenario may be configured to detect QR codes or NFC tags as it moves along a path. Detection of such information could provide a ground truth label in such scenario.

Observation 4: UEs may be equipped with additional sensors (such as camera and radars) and may have applications that are able to detect and provide or enhance ground truth labels.

Additionally, the detection accuracy of landmarks can benefit from device sensors and user usage patterns. The network can identify or indicate areas where labeled data is needed and trigger action associated with these landmarks accordingly.

Proposal 3: RAN2 shall discuss what additional information from the UE can be provided to the LMF or OTT server which trains the UE-sided model to ascertain the ground truth. The following information could be transferred from the UE to obtain ground truth labels:

      - Information obtained from on-board sensors, such as camera and radars

      - Information obtained from processing tags, such as QR codes, NFC tags

      - Information obtained from other positioning systems deployed in parallel to 3GPP system.

Although the ground truth information is available at the UE and can be used for training and monitoring purposes, training a model at the UE side may be resource (e.g. battery, processing power) heavy at the UE side. There may be need to transmit the ground truth information to the network side.

Proposal 4: RAN2 shall consider privacy preserving mechanisms to provide training data containing ground truth label to the network or external server, for training the UE-sided model.

## Model monitoring

The output of the AI/ML model at the UE needs to be monitored regularly to see if the model is still correctly estimating the UE location. There may be two approaches, depending on the availability of high-quality labels.

### Label-free monitoring of performance

If the input data distribution to the AI/ML model has similar statistics to the input data distribution used for AI/ML model training, the model can expect to perform well, i.e. it has observed similar data during training phase. If the input data statistics are different, the model may not be suitable for inference.

Proposal 5: RAN2 shall consider filtering on the input statistics and output statistics (or monitoring output, e.g. position error) using the filtering configuration provided by NW and inform LMF

Alt 1: Report the statistics to LMF for monitoring

Alt 2: Provide a fault indication, if the statistics differ by a threshold value configured by NW.

There may be scenarios, where the UE observes certain characteristics behaviour. For example, a UE may observe differences in statistics at certain locations. Alternatively, the network may have observed that a group of UEs observe certain patterns at certain location. To obtain data for training or fine-tuning the network, the UE may be configured to report certain information, in response to configured events.

Proposal 6: A UE may be configured to collect and report data to NW, if certain events are triggered.

### Ground truth label for monitoring

Inaccurate ground truth labels may cause error in inference or monitoring. One source of ground truth labels for positioning may come from classical positioning methods already specified based on RAT dependent and RAT-independent techniques. However, AI/ML models are good candidates for deployment in scenarios where the existing classical positioning methods fail to deliver good results. Therefore, the labels from existing positioning methods may not be sufficient to be used as ground truth labels in all cases.

A ground truth label (the actual position of the UE in case of direct positioning) may not be known for all UEs with the necessary accuracy, or this may not be known at all, except for the PRUs that are provisioned whose position can be assumed to be reasonably accurate. On the other hand, it is reasonable to assume that certain UEs are equipped with on-board sensors, that can provide some information to derive the ground truth labels. For example, A UE can utilize a camera to detect its surroundings, use tags (such as QR codes/NFC tags) to determine its position, etc.

Proposal 7: UE shall report its capability to provide ground truth labels or its ability to provide additional information to improve the ground truth labels to the NW.

Proposal 8: If the UE needs assistance from NW to perform monitoring, then the UE shall send a request to NW, indicating the type of assistance data needed.

# LCM for NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning (Case 3b)

For the Case 3b, data collection from the UE to enable the model monitoring and management functionalities need to be supported. Other LCM components for NW-sided model may be outside the scope of RAN2 specifications.

## Ground truth label

For Case 3b (NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning), this means that

- Training is based on channel measurements provided by the NG-RAN node and the label generated by the LMF.

- Training is based on channel measurements provided by the NG-RAN node and the label provided by the UE (PRU or non-PRU).

In both cases, the training data and label are generated by different entities.

Observation 6: Training data and labels are generated by different entities for Case 3b, thereby requiring alignment between training data and the label.

Nevertheless, the position obtained by UE-based and LMF-based positioning methods still suffer from reduced accuracy in scenarios where heavy multipath and NLOS scenarios occur more frequently. These are exactly the scenarios where classical positioning methods show performance degradation. Therefore, it may be beneficial to utilize other sensor information available at the UE.

As discussed earlier for Case 1, the UE shall provide additional information to provide ground truth labels to the NW.

Proposal 9: UE shall report its capability to provide ground truth labels or its ability to provide additional information to improve the ground truth labels to the NW for LCM of NW-based AI/ML models.

Proposal 10:  The UE shall provide ground truth labels or additional information to improve the ground truth labels, when configured by the network.

## Model monitoring

There may be scenarios, where the LMF observes certain characteristics behaviour. For example, an LMF may observe certain characteristics, such deviation from the expected performance, when a UE is in certain region. To obtain data for training or fine-tuning the model, the LMF may configure UE to report certain information, in response to configured events.

Proposal 11: A UE may be configured to collect and report data to the network, if certain events are triggered. The events may be:

      - UE travelling in a preconfigured track, where the NW intends to collect data.

      - UE entering a certain area.

      - UE detecting certain signals above a certain threshold.

      - UE performance degrading beyond a certain threshold.

Additionally, the source of ground truth (e.g. UE) and the source of  the input data and interence output (e.g. LMF) may need time-alignment.  For example, the ground truth label that was generated using on-board sensors may have certain processing delays, in addition to mismatch between UE local timing and NW-timing  and may not be aligned with the measurements performed at the TRPs in the network.

Observation 7: The LMF may need to time-synchronize the  input and ground truth label for training and monitoring purposes.

Proposal 12: The UE shall provide the timestamp aligned with the network time to associate the ground truth label with measurements from the network.

# NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning (Case 3a)

In NG-RAN node assisted positioning, the NG-RAN node provides measurement results, such as timing and angular measurements, for different positioning methods, using the AI/ML model instead of classical algorithms.

The NG-RAN node is not an LCS-client to receive UE position. Therefore, gNB is unable to determine whether the inference output corresponds to the anticipated output, i.e. the gNB lacks ground truth label to perform monitoring or training.

On the other hand, the LMF may have the ground truth label of a given UE. The ground truth label may have different quality depending on the method used to acquire the label. The ground truth label may be computed by the LMF using classical positioning methods (but may not have good quality in an environment dominated by NLOS or multipath) or a location report received from the UE (for example, using GNSS based positioning). Alternatively, signalling may be enhanced such that UE may provide information from its sensors (such as camera).  However, the LMF only has access to measurement reports provided by the gNB and not the raw measurements of the radio unit.

Observation 8: The NG-RAN node is not an LCS-client to receive UE position.

Observation 9: The LMF observes the measurement (inference) and ground truth, but not the raw measurements (input) of radio unit.

Therefore, for training and monitoring purposes, there are two alternatives:

Alternative 1: NG-RAN node becomes an LCS-client to receive the labels.

Alternative 2: NG-RAN node reports the raw measurement from the radio unit to the LMF.

Both of the above alternatives have stage 3 impact beyond the scope of RAN2 specifications. From RAN2 perspective, the ground truth shall be reported to the NW depending on UE capability, which may be exchanged between the NW entities.

Proposal 13: The UE shall, subject to its capabilities and privacy, be configured to report its ground truth (i.e. UE position at one or more point in time), together with uncertainties to the LMF for assisting training for Case 3a.

# Second priority use cases

## Case 2b: UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning (2nd priority)

As outlined in the WID, case 2b is 2nd priority. This option has similarities to case 3b in terms of ground truth label collection, and similarity to direct UE-positioning (case 1) in terms of input data. Consequently, what additional information needs to be collected shall be concluded after handling the 1st priority cases.

Proposal 14: RAN2 shall further study if specification is needed beyond what is needed for Case 3b.

## Case 2a: UE-assisted/LMF-based positioning with UE-side model, AI/ML assisted positioning

As outlined in the WID, case 2a is 2nd priority. The issue is associating UE-labels to the channel measurements. However, it has similarities to NG-RAN assisted AI/ML assisted positioning in terms of associating labels to measurement data, albeit the processing done in different network entity.

Consequently, what additional information needs to be collected shall be concluded after handling the 1st priority cases.

Proposal 15: RAN2 shall consider case 2a after further progress on first priority use cases.

# Conclusions

In this contribution, we have made the following observations:

Observation 1: Using existing positioning methods specified in 3GPP as source of labels does provides wrong labels in training data in the very scenarios where the AI/ML methods (NLOS heavy, multipath dominated) are supposed to perform better than existing methods.

Observation 2: Use of PRU type UEs limit the data collected to static UEs and spatially limited to where the PRUs are provisioned.

Observation 3: In indoor scenarios, landmarks can be the only mechanism to obtain labeled data, enabling the Network/UE to identify and trigger the need for labels in specific areas.

Observation 4: UEs may be equipped with additional sensors (such as camera and radars) and may have applications that are able to detect and provide or enhance ground truth labels.

Observation 5: Running several AI/ML models in parallel, monitoring them and switching the models is computationally expensive.

Observation 6: Training data and labels are generated by different entities for Case 3b, thereby requiring alignment between training data and the label.

Observation 7: The LMF may need to time-synchronize the  input and ground truth label for training and monitoring purposes.

Observation 8: The NG-RAN node is not an LCS-client to receive UE position.

Observation 9: The LMF observes the measurement (inference) and ground truth, but not the raw measurements (input) of radio unit.

Based on the above observations, we make the following proposals:

Proposal 1: UE location for the purpose of ground truth shall be determined  using secondary sources (e.g. camera-based), which different from GNSS or RAT dependent methods for UEs that are not provisioned as PRUs (“non-PRU UEs”).

Proposal 2: RAN2 shall discuss association between channel measurement and ground truth for each of the positioning use case, including indication of the method used to obtain the ground truth label.

Proposal 3: RAN2 shall discuss what additional information from the UE can be provided to the LMF or OTT server which trains the UE-sided model to ascertain the ground truth. The following information could be transferred from the UE to obtain ground truth labels:

      - Information obtained from on-board sensors, such as camera and radars

      - Information obtained from processing tags, such as QR codes, NFC tags

      - Information obtained from other positioning systems deployed in parallel to 3GPP system.

Proposal 4: RAN2 shall consider privacy preserving mechanisms to provide training data containing ground truth label to the network or external server, for training the UE-sided model.

Proposal 5: RAN2 shall consider filtering on the input statistics and output statistics (or monitoring output, e.g. position error) using the filtering configuration provided by NW and inform LMF

Alt 1: Report the statistics to LMF for monitoring

Alt 2: Provide a fault indication, if the statistics differ by a threshold value configured by NW.

Proposal 6: A UE may be configured to collect and report data to NW, if certain events are triggered.

Proposal 7: UE shall report its capability to provide ground truth labels or its ability to provide additional information to improve the ground truth labels to the NW.

Proposal 8: If the UE needs assistance from NW to perform monitoring, then the UE shall send a request to NW, indicating the type of assistance data needed.

Proposal 9: UE shall report its capability to provide ground truth labels or its ability to provide additional information to improve the ground truth labels to the NW for LCM of NW-based AI/ML models.

Proposal 10:  The UE shall provide ground truth labels or additional information to improve the ground truth labels, when configured by the network.

Proposal 11: A UE may be configured to collect and report data to the network, if certain events are triggered. The events may be:

      - UE travelling in a preconfigured track, where the NW intends to collect data.

      - UE entering a certain area.

      - UE detecting certain signals above a certain threshold.

      - UE performance degrading beyond a certain threshold.

Proposal 12: The UE shall provide the timestamp aligned with the network time to associate the ground truth label with measurements from the network.

Proposal 13: The UE shall, subject to its capabilities and privacy, be configured to report its ground truth (i.e. UE position at one or more point in time), together with uncertainties to the LMF for assisting training for Case 3a.

Proposal 14: RAN2 shall further study if specification is needed beyond what is needed for Case 3b.

Proposal 15: RAN2 shall consider case 2a after further progress on first priority use cases.

# References

| [1] | RP-234039, "New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface," 3GPP, Online, 2023-12. |
| --- | --- |