3GPP TSG-RAN WG2#129	R2-2500176

Athens, Greece, 17-21 February 2025

Agenda Item:	8.2.3

Source: 	MediaTek Inc.

Title:  	Downselection of ambient IoT random access procedures

Document for:	Discussion, decision

# 1	Introduction

The ambient IoT work item in [1] includes guidance for RAN2 to focus on solution 1 (3-step only) for AIoT random access, unless a decision is taken in RAN2#129 to pursue solution 3 (unified 2-step and 3-step).  This document addresses the question of which form of random access should be supported.

# 2	Discussion

## 2.1	Solution 1

The description in [2] of solution 1 does not actually include Msg3 as part of the random access procedure, but the term “3-step” seems still to be in use.  The device transmits Msg1 without data (just the 16-bit random ID, “RN16” henceforth) and receives contention resolution in Msg2, after which a long device ID or other upper-layer data can be transmitted in the D2R direction (“Msg3”).  The (straightforward) procedure is shown in figure 1.

Figure 1: Solution 1 for AIoT random access

From the device implementation perspective, this procedure involves five steps triggered by the reception of a paging message (assuming the needed data are already available at the device):

- Generate and store RN16

- Transmit RN16 in Msg1

- Receive Msg2

- Compare Msg2 payload to the stored RN16

- In case of a match, transmit data in Msg3

This form of the procedure assumes that there is no assignment of a persistent AS ID in Msg2, which would add a sixth step: In case of a match, store the assigned ID in place of RN16 (this step may need to precede step 5).  There is one conditional operation (at step 4, with one branch comprising the rest of the procedure and the other branch terminating the procedure immediately).

Observation 1: From the device implementation perspective, solution 1 requires 5-6 steps with one conditional.

## 2.2	Solution 3

Solution 3 takes two forms depending on whether data are requested by the triggering paging message (note that the description of solution 3 in [2] is explicit that including the data is controlled by the reader).  The first form is identical to figure 1 above; the second, with data in Msg1, is shown in figure 2.

Figure 2: Solution 3 for AIoT random access

For the device, this solution could be implemented as follows:

- Generate and store RN16

- If data were requested to be transmitted in Msg1, transmit RN16+data in Msg1; otherwise, transmit RN16 in Msg1

- Receive Msg2

- Compare Msg2 payload to the stored RN16 (this step is also necessary in the “2-step” case, because it may affect subsequent processing of stored data, e.g., to flag it as successfully transmitted)

- In case of a match:

  - If data were requested to be transmitted in Msg1, the procedure ends

  - Otherwise, transmit data in Msg3

(This description is optimised to minimise the number of steps; an alternative formulation could branch at step 2 into two completely separate, though largely similar, procedures, resulting in a larger total number of steps, some code redundancy, and two conditionals instead of three.)

The slightly greater number of steps compared to solution 1 (considering steps 5a and 5b, resulting in a six-step unified procedure) is not the main concern; it is more significant that there are three conditional points in steps 2, 5, and 5a.  (As with solution 1, adding a persistent AS ID in Msg2 would add one additional step before step 5.)

Observation 2: From the device implementation perspective, solution 3 requires 6-7 steps with three conditionals.

## 2.3	Comparison

It is clear from observations 1 and 2 (and not surprising) that solution 1 is nominally simpler than solution 3.  As suggested above, the greatest difference is in the number of conditionals, particularly since each branch requires testing.  The alternative formulation of solution 3 (separate code branches from step 2, as described above) would reduce this difference at the cost of increased code footprint (which has its own costs) and redundancy.

In addition to the complexity comparison, solution 1 has the advantage of better coverage than solution 3 due to the smaller Msg1 size, as noted in [2].  (The tradeoff is the avoidance of Msg3 in solution 3, which applies in case contention is reasonably infrequent.)  It should also be noted that the reader cannot be expected to have advance knowledge of the coverage situation; a reader may send a page with data requested to a device that cannot support transmitting the larger transport block due to coverage limitations.  Handling this possibility at the device would introduce another conditional and complicate step 2 of the description (“If data were requested to be transmitted in Msg1 and the corresponding TB size can be supported…”), and ignoring it would result in lost transmissions.

The main advantage of the 2-step solution is power saving (equiv. latency reduction), giving a low-power device a better chance of completing the procedure within the limits of its energy reserves.  With only device type 1 in scope for the work item, however, this concern is less important; as indicated in several of the analyses in [2] (e.g., from source [78] as documented in section 6.2.1), it seems reasonable to expect that device type 1 can complete an inventory procedure on a single charge even in the presence of multiple responding devices.  We therefore see no compelling benefit of solution 3 in the scope of this work item.

On balance, we suggest adopting solution 1.

Proposal 1: Adopt solution 1 (3-step only) for AIoT random access.

# 3	Conclusion

This document promulgated the following proposal:

Proposal 1: Adopt solution 1 (3-step only) for AIoT random access.

# 4	References

[1]	RP-243326, “New Work Item: Solutions for Ambient IoT (Internet of Things) in NR”, Huawei, RAN#106

[2]	TR 38.769