3GPP TSG-RAN WG2 Meeting #129	R2-2500217

Athens, Greece, Feb. 17th – 21st, 2025

Agenda item:	8.12.2

Source:	Samsung

Title:	Discussion on Asymmetric DL sTRP/UL mTRP

WID/SID:	NR_MIMO_Ph5

Document for:	Discussion and Decision2

# Introduction

In the so-called “asymmetric DL sTRP/UL mTRP” scenario, we consider dense UL-only TRPs, where UEs receive DL from a macro DL/UL TRP and can transmit UL to micro UL-only TRPs, as shown in the Figure below.

In this scenario, we consider a pathloss offset between the DL pathloss RS from the DL TRP and the actual pathloss for UL-only TRP.  Due to this pathloss offset, UL power control to the UL-only TRP are determined differently from the legacy power control. The PL offset is used in UL power control in a way that UE reuses the legacy uplink power control formulation by replacing legacy PL with a PL which is derived from the DL PL RS and the PL offset. The PL offset is applied in PUCCH, PUSCH, SRS, PDCCH-order PRACH as RAN1 agreed.

In RAN2#128 meeting, a new MAC CE for pathloss offset update is agreed.

New MAC CE is introduced for PL offset update for asymmetric DL sTRP/UL mTRP. This new MAC CE is identified by new eLCID.

Absolute value of PL offset is indicated in the new MAC CE. For the offset value, the value range is [-12, 60] dB and the step size is 4dB.

In the MAC CE, PL offset value can be updated for any configured TCI states with RRC configured PL offset, i.e., not limited to the activated TCI states.

In this paper, we discuss more details on the PL offset MAC CE and other impacts due to the pathloss offset from RAN2 perspective.

# Discussion

## PL offset MAC CE

RAN1 has agreed to configure/indicate PL offset for joint/UL TCI state(s).

- A PL offset value is configured per joint/UL TCI state, MAC CE is used to update the PL offset value(s) for joint/UL TCI states.

- The value range of PL offset is [-12, 60] dB, with a step size of 4dB.

In the last meeting, we have agreed that the absolute value of PL offset is indicated, with value range [-12, 60] dB and step size 4 dB. The PL offset is configured by RRC per TCI state, thus, the new MAC CE should contain a joint/UL TCI state ID to indicate to which TCI state the updated PL offset is applied.

Proposal 1: The PL offset update MAC CE contains joint/UL TCI state ID(s) for the indicated PL offset(s).

Regarding the mapping between PL offset and TCI state in the MAC CE, we consider

- Option 1: 1-1 mapping, i.e., a single PL offset is indicated for each joint/UL TCI state ID.

- Option 2: 1-many mapping, i.e., a single PL offset is indicated for all joint/UL TCI states IDs with the same update value.

Option 1 should be the baseline. Although option 2 can save some bits, it complicates the MAC CE design. In practice, different joint/UL TCI states may or may not have same PL offset, and NW may only update the PL offset for the DCI-indicated TCI state, i.e., the currently used TCI state. Therefore, option 1 is preferred.

Proposal 2: In the PL offset update MAC CE, one PL offset value is indicated for each joint/UL TCI state.

For unified TCI state framework introduced since Rel-17, if unifiedTCI-StateType is configured with joint for the serving cell, a list of joint TCI states are configured per DL BWP and each joint TCI state is used for both DL reception and UL transmission;  if unifiedTCI-StateType is configured with separate for the serving cell, a list of DL TCI states is configured per DL BWP to be used for DL reception and a list of UL TCI states is configured per UL BWP to be used for UL transmission. As the list of joint/UL TCI states is configured per BWP, a BWP ID is needed to indicate from which BWP the indicated joint/UL TCI states are configured.

Proposal 3: The PL offset update MAC CE includes a BWP ID. If unifiedTCI-StateType is configured with joint for the serving cell, the BWP ID refers to a DL BWP and TCI state ID(s) refer to joint TCI state(s) configured for the DL BWP; if unifiedTCI-StateType is configured with separate for the serving cell, the BWP ID refers to a UL BWP and TCI state ID(s) refer to UL TCI state(s) configured for the UL BWP.

For simplicity, we propose the MAC CE is applied to a single serving cell with a Serving Cell ID field.

Proposal 4: The PL offset MAC CE includes a serving cell ID.

As the number of updated PL offsets (i.e., TCI states) can be dynamic, the MAC CE should have a variable size. The maximum number of updated PL offsets can be considered same as the maximum number of activated joint/UL TCI states, which is 8 for Rel-17 unified TCI states and 16 for Rel-18 unified TCI states (for sDCI mTRP).

Proposal 5: The MAC CE has a variable size and the maximum number of PL offsets (i.e., TCI states) is 16.

Based on the above discussion, we propose a baseline format for the new MAC CE.

Proposal 6: Adopt the baseline format for PL offset update MAC CE.

Some company raised an issue that it is not clear how to handle the RRC configured PL offset and the MAC CE updated PL offset for the same joint/UL TCI state. For instance, for BWP#1, UE has an RRC configured PL offset for a TCI state, and then also received an updated value for the same TCI state in MAC CE. In this case, UE should apply the latest PL offset value updated in the MAC CE. The PL offset updated by MAC CE overrides the RRC configured value. This is same as the function of SRS Pathloss Reference RS Update MAC CE, which is used to update the RRC-configured PL RS for SRS resource set.

Then, consider a scenario that BWP is switched to BWP#2 and switched back to BWP#1. Should UE apply the RRC configured PL offset for BWP#1 or apply the previously MAC-CE updated value? Does UE need to maintain the updated value previously received in the MAC CE or treat it as a one-shot update? In this case, it seems more reasonable that UE maintains and applies the MAC-CE updated PL offset which reflects the latest pathloss. Basically, UE should always apply the latest received PL offset which can be either configured by RRC or updated by MAC CE.

Proposal 7: UE updates the RRC configured PL offset with MAC CE indicated value. UE applies the latest PL offset value (e.g., upon BWP switching) received in RRC or MAC CE.

However, PL offset is applied in PHY layer instead of MAC layer and MAC only delivers the received information to PHY. It is RAN1’s regime to define how to maintain/apply the configured/updated PL offset. We can specify in MAC that the MAC entity indicates to the lower layer the information in the PL offset update MAC CE. This is same as the handling of SRS Pathloss Reference RS Update MAC CE.

Proposal 8: Upon receiving a PL offset update MAC CE, the MAC entity indicates to lower layer the information in the MAC CE.

## PHR

For PHR aspect, RAN1 has agreed that the PL offset is applied to Type 1 PHR calculation for actual/reference PUSCH, FFS whether/how to consider PL offset for Type 3 PHR calculation. As PHR trigger is specified in MAC specification, one issue to be discussed in RAN2 is whether PHR trigger is impacted. Currently in TS 38.321 clause 5.4.6, the key trigger of PHR is that pathloss of any PL RS changes more than a threshold.

- phr-ProhibitTimer expires or has expired and the path loss has changed more than phr-Tx-PowerFactorChange dB for at least one RS used as pathloss reference for one activated Serving Cell of any MAC entity of which the active DL BWP is not dormant BWP since the last transmission of a PHR in this MAC entity when the MAC entity has UL resources for new transmission; NOTE 1: The path loss variation for one cell assessed above is between the pathloss measured at present time on the current pathloss reference and the pathloss measured at the transmission time of the last transmission of PHR on the pathloss reference in use at that time, irrespective of whether the pathloss reference has changed in between. The current pathloss reference for this purpose does not include any pathloss reference configured using pathlossReferenceRS-Pos in TS 38.331 [5].

PHR is used to inform the UL transmission power headroom, i.e., how much more transmission power UE is able to add to the power being used by current transmission. PHR should be triggered whenever UL transmission power is changed more than a threshold. As specified in TS 38.213 clause 7, the PUSCH/PUCCH/SRS transmission power are formulated as follows.

PPUSCH,b,f,ci,j,qd,l=minPCMAX,f,ciPO_PUSCH,b,f,cj+10log102μ∙MRB,b,f,cPUSCHi+αb,f,cj∙PLb,f,cqd+∆TF,b,f,ci+fb,f,ci,l   [dBm]

PPUCCH,b,f,ci,qu,qd,l=minPCMAX,f,ciPO_PUCCH,b,f,cqu+10log102μ∙MRB,b,f,cPUCCHi+PLb,f,cqd+∆F_PUCCHF+∆TF,b,f,ci+gb,f,ci,l   [dBm]

[dBm]

As we can see, UE determines UL transmission power based on pathloss of PL RS. The pathloss change implies the change of UL transmission power, thus, PHR is triggered by the pathloss change of PL RS in the legacy scenario.

Differently, in asymmetric DL sTRP/UL-only mTRP scenario, for a joint/UL TCI state configured with a PL offset, the UL transmission power is determined based on not only the pathloss but also the pathloss offset of the PL RS. As RAN1 agreed, the PL offset is used in UL power control in a way that UE reuses the legacy uplink power control formulation by replacing legacy PL with a PL which is derived from the DL PL RS and the PL offset.

Thus, the PHR should be triggered when the pathloss change of a PL RS is larger than a threshold, where the pathloss takes into account the PL offset and the PL RS is associated to the joint/UL TCI configured with the PL offset. How to apply the PL offset is up to RAN1, e.g., (PL-PL_offset) or (PL+PL_offset).

According to RAN1 previous discussion (R1-2405347), the impact to PHR trigger is left to RAN2. For the sake of progress, we propose to add a note similar to the existing NOTE 1 to clarify that the PL change is determined based on PL offset. Whether/how to capture anything relevant in RAN1 specification is up to RAN1.

Proposal 9: For PHR trigger, add a note in MAC specification: If a joint/UL TCI state is configured with a PL offset, PHR trigger is based on the PL change of the PL-RS associated to the joint/UL TCI, where the PL change takes into account the PL offset.

## 2TA for single-DCI mTRP

In Rel-18, 2TA operation has been introduced for mDCI mTRP. For Rel-19, the scope of the latest WID includes 2TA for sDCI mTRP to support asymmetric DL sTRP/UL mTRP. RAN1 informs in LS R1-2410758 the following agreement regarding 2TA for sDCI mTRP.

RAN1#118bis Agreement Support 2TA for the asymmetric DL sTRP/UL mTRP deployment scenarios: - Remove the restriction that coresetPoolIndex needs to be configured for the 2TA feature. - One downlink reference timing is supported and applied to both TAGs. - (FFS) Note: UE autonomous TA adjustment is only applicable to the first TAG - One single n-TimingAdvanceoffset is configured and applied to both TAGs. - Any of the TCI states can be associated with any one of the two TAGs. - The RAR carrying TA adjustment for those 2 TAGs is reused for Rel-19 2TA - The MAC CE based TA adjustment for 2 TAGs is reused for Rel-19 2TA. - Introduce the optional UE capability of “Overlapping UL transmission reduction” for Rel-19 2TA - If UE does not report this UE capability, UE does not expect two UL transmissions associated with different TAGs are overlapped. - FFS: UE does not expect that in intra-slot TDM PUSCH type-B repetition transmission, two consecutive repetitions associated with different TAGs are overlapped.

In the current 2TA operation, each TCI state is applied with a TA and associated to the corresponding TAG. To extend 2TA operation to asymmetric DL sTRP/UL mTRP, Rel-18 RA procedure with 2TA and the procedure of TAT/TAG management can be directly applied since mDCI/sDCI is transparent to the TA operation in MAC. The current 2TA operation can be applied with minor changes on RRC configuration.

We can confirm the RRC impacts based on RAN1 agreements.

- remove in RRC field tag2 the restriction that coresetPoolIndex needs to be configured for the 2TA;

- a single n-TimingAdvanceoffset is configured, i.e., n-TimingAdvanceOffset2 is not configured for 2TA in asymmetric DL sTRP/UL mTRP scenario.

Proposal 10: For 2TA in asymmetric DL sTRP/UL mTRP scenario, Rel-18 2TA operation is applied with the following RRC changes:

- remove the restriction that RRC field tag2 is configured only if coresetPoolIndex is configured with more than one value;

- a single n-TimingAdvanceoffset is configured, i.e., n-TimingAdvanceOffset2 is not configured for 2TA in asymmetric DL sTRP/UL mTRP scenario.

## PL offset in RACH

For PRACH transmission, RAN1 has agreed the following aspects up to the last meeting.

- Support applying PL offset on PDCCH-order PRACH towards a UL-only TRP in FR1.

- Support to apply PL offset on PDCCH-order PRACH in FR2. The design for applying PL offset on PDCCH-order PRACH in FR1 is fully reused here. Note: there is no extra enhancement for Tx beam determination for PDCCH-order PRACH in FR2 in Rel-19.

- The PL offset associated with one of the indicated joint/UL TCI state for UL-only TRP in unified TCI framework is applied on the PDCCH-order PRACH transmission.

- A one-bit DCI field indicates whether the PL offset of the indicated joint/UL TCI state is used for PRACH transmission.

We notice that RAN1 has only considered PDCCH-order PRACH. It is not clear whether it refers to both PDCCH-order CBRA and PDCCH-order CFRA or not. In our understanding, to obtain the TA for UL-only TRP in the 2TA operation, UE has to apply the PL offset in PDCCH-order PRACH towards the UL-only TRP so that the UL-only TRP can receive the PRACH with a proper UL power control. Since both PDCCH-order CBRA and PDCCH-order CFRA can be used to obtain TA, we assume PL offset is applied to both CBRA and CFRA.

Proposal 11: RAN2 assumes PL offset is applied to both PDCCH-order CBRA and PDCCH-order CFRA towards a UL-only TRP.

On the other hand, it is also not clear whether PL offset is needed for PRACH in UE initiated RA (e.g., for BFR, SR, etc.) or not. In our view, PL offset is not necessary for UE initiated RACH for BFR, SR, etc. since the macro DL TRP should be able to receive the PRACH with legacy power control. This is different from PDCCH-order PRACH to the UL-only TRP which aims to acquire TA. Thus, we propose not to consider PL offset in UE initiated RACH.

Proposal 12: RAN2 does not consider PL offset in UE initiated RACH.

Moreover, in the current Random Access procedure UE measures RSRP of DL pathloss RS and compares it with a RSRP threshold to determine RA resource selection (e.g., for NUL/SUL, 4-step/2-step, Msg1/3 repetition, preamble group A/B) for UL coverage consideration.

As specified in TS 38.213 clause 7, the pathloss in the above formula is calculated as the difference between the reference signal power and the filtered RSRP.

PLb,f,c(qd) = referenceSignalPower – higher layer filtered RSRP,

where referenceSignalPower is provided by higher layers and RSRP is defined in [7, TS 38.215] for the reference serving cell and the higher layer filter configuration provided by QuantityConfig is defined in [12, TS 38.331] for the reference serving cell

If the UE is not configured periodic CSI-RS reception, referenceSignalPower is provided by ss-PBCH-BlockPower. If the UE is configured periodic CSI-RS reception, referenceSignalPower is provided either by ss-PBCH-BlockPower or by powerControlOffsetSS providing an offset of the CSI-RS transmission power relative to the SS/PBCH block transmission power [6, TS 38.214]. If powerControlOffsetSS is not provided to the UE, the UE assumes an offset of 0 dB.

In legacy operation, UE measures RSRP from a DL TRP and transmits PRACH to the same TRP. However, in asymmetric DL/UL scenario, UE measures RSRP from a DL TRP and transmits PRACH to a UL-only TRP. When transmitting to UL-only TRP instead of to DL TRP, a path different from DL RS is taken. Consequently the pathloss changes by an offset, and the pathloss change leads to a change in the measured RSRP since the reference signal power from the DL TRP is not changed.

PLb,f,c(qd) + PLoffset = referenceSignalPower – higher layer filtered RSRP + PLoffset

= referenceSignalPower – (higher layer filtered RSRP– PLoffset)

Equivalently, from physical channel perspective, received power (i.e., RSRP) equals the difference between the transmit power and channel pathloss. When the PL offset is changed by an offset, so is the RSRP.

higher layer filtered RSRP– PLoffset = referenceSignalPower – (PLb,f,c(qd) + PLoffset)

An example is shown in the Figure below.

For UE1, the pathloss between DL TRP and UE1 and the pathloss between UE1 and UL-only TRP may not be much different, so the actual measured RSRP for DL RS can be used as the virtual measured RSRP from UL-only TRP. Thus, the RSRP of DL PL RS can still work well in the comparison with the configured RSRP threshold.

For UE2 and UE3, the pathloss between DL TRP and UE2/3 and the pathloss between UE2/3 and UL-only TRP may be much different due to the different distances, so that the actual measured RSRP for DL RS deviates from the virtual measured RSRP from UL-only TRP. As a result, the RSRP of DL PL RS may not work well.

Take NUL/SUL selection as an example. UE2 is far from DL TRP and outside the boundary of RSRP threshold, and the measured RSRP for DL PL RS is smaller than the configured RSRP threshold. In this case, SUL will be selected for PRACH transmission according to the current RA procedure. However, UE2 is close to the UL-only TRP and the virtual RSRP should be larger than the configured RSRP, so for PRACH towards UL-only TRP NUL should be selected. Similarly, UE3 will select NUL as the measured RSRP is larger than the configured RSRP threshold. However, UE3 is far away from the UL-only TRP and the virtual RSRP should be smaller than the configured RSRP, so for PRACH towards UL-only TRP SUL should be selected. Thus, the current NUL/SUL selection base on RSRP comparison does not work well.

For UE2 and UE3, this issue can be solved by applying the pathloss offset to the RSRP of DL RS for RA resource selection, as the same principle of applying the pathloss offset to the pathloss of DL RS for PRACH power control towards UL-only TRP.

Proposal 13: RAN2 confirms that PL offset is applied to the RSRP of DL RS in RA procedure.

Proposal 14: For PDCCH-order CBRA and PDCCH-order CFRA, discuss details on how to apply the PL offset to the RSRP of DL RS for RA resource selection (e.g., for NUL/SUL, 4-step/2-step, Msg1/3 repetition, preamble group A/B).

# Conclusion

Based on the discussion, we have the following proposals.

Proposal 1: The PL offset update MAC CE contains joint/UL TCI state ID(s) for the indicated PL offset(s).

Proposal 2: In the PL offset update MAC CE, one PL offset value is indicated for each joint/UL TCI state.

Proposal 3: The PL offset update MAC CE includes a BWP ID. If unifiedTCI-StateType is configured with joint for the serving cell, the BWP ID refers to a DL BWP and TCI state ID(s) refer to joint TCI state(s) configured for the DL BWP; if unifiedTCI-StateType is configured with separate for the serving cell, the BWP ID refers to a UL BWP and TCI state ID(s) refer to UL TCI state(s) configured for the UL BWP.

Proposal 4: The PL offset MAC CE includes a serving cell ID.

Proposal 5: The MAC CE has a variable size and the maximum number of PL offsets (i.e., TCI states) is 16.

Proposal 6: Adopt the baseline format for PL offset update MAC CE.

Proposal 7: UE updates the RRC configured PL offset with MAC CE indicated value. UE applies the latest PL offset value (e.g., upon BWP switching) received in RRC or MAC CE.

Proposal 8: Upon receiving a PL offset update MAC CE, the MAC entity indicates to lower layer the information in the MAC CE.

Proposal 9: For PHR trigger, add a note in MAC specification: If a joint/UL TCI state is configured with a PL offset, PHR trigger is based on the PL change of the PL-RS associated to the joint/UL TCI, where the PL change takes into account the PL offset.

Proposal 10: For 2TA in asymmetric DL sTRP/UL mTRP scenario, Rel-18 2TA operation is applied with the following RRC changes:

- remove the restriction that RRC field tag2 is configured only if coresetPoolIndex is configured with more than one value;

- a single n-TimingAdvanceoffset is configured, i.e., n-TimingAdvanceOffset2 is not configured for 2TA in asymmetric DL sTRP/UL mTRP scenario.

Proposal 11: RAN2 assumes PL offset is applied to both PDCCH-order CBRA and PDCCH-order CFRA towards a UL-only TRP.

Proposal 12: RAN2 does not consider PL offset in UE initiated RACH.

Proposal 13: RAN2 confirms that PL offset is applied to the RSRP of DL RS in RA procedure.

Proposal 14: For PDCCH-order CBRA and PDCCH-order CFRA, discuss details on how to apply the PL offset to the RSRP of DL RS for RA resource selection (e.g., for NUL/SUL, 4-step/2-step, Msg1/3 repetition, preamble group A/B).

# References

- RP‑242356	Revised Work Item: NR mobility enhancements Phase 4, RAN#105.