3GPP TSG-RAN WG2 Meeting #129	R2-2500218

Athens, Greece, Feb. 17th – 21st, 2025

Agenda item:	8.12.3

Source:	Samsung

Title:	Discussion on UE-initiated Beam Reporting and CSI enhancement

WID/SID:	NR_MIMO_Ph5

Document for:	Discussion and Decision

# Introduction

In the last meeting, RAN2 discussed whether there is RAN2 impacts on event evaluation/triggering of UE initiated beam reporting. In this paper, we share our views on way forward for UE initiated/event-triggered (UEI) beam reporting from RAN2 perspective and discuss some RRC parameters for UE initiated/event-triggered (UEI) beam reporting and CSI enhancements.

# Discussion

## 2.1 UEI beam reporting

Based on RAN1 progress up to last meeting, we summarize the current agreed procedure of UEI beam reporting from several aspects as follows. The remaining FFSs in RAN1 are not included, which may lead to additional procedure and details.

- Measurement resource: intra/inter-cell SSB, and periodic CSI-RS for BM

  - The same RS type for current beam and new beam is ensured implicitly:

    - If the RS(s) for new beam are CSI-RS configured in a CSI-RS resource set configured with repetition (i.e., new beam is CSI-RS for BM), Scheme-1 is enabled, which means RS for current beam is the QCL RS in the indicated TCI state (Note QCL RS of the indicated TCI state is always a CSI-RS);

    - Otherwise (i.e., new beam is SSB), Scheme-2 is enabled, meaning the RS for current beam is the SSB which is QCLed with the QCL RS in the indicated TCI state.

- Trigger events

  - Event-2:

    - Current beam: QCL RS of indicated TCI state, or SSB QCLed to the indicated TCI state

    - New beam: either CSI-RS or SSB, explicitly configured in RS resource set associated to the report configuration

    - Basic trigger: L1-RSRP of at least one new beam becomes a threshold (RRC configured) better than the current beam

    - Optional trigger by UE capability: If within a configurable time window, the number of Event-2 instance(s) for at least one same new beam is greater than or equal to a configurable conter M (counter M is per new beam).

    - Report content: CRI/SSBRI and L1-RSRP (differential value) for N ≥ 1 beam(s) (and current beam if enabled by RRC) in the report instance, with at least one beam meeting the event.

  - Event-1:

    - Quality of the current beam is worse than a certain threshold.

    - Same type of beam comparison: FFS Scheme-1/2 is explicitly configured or implicitly enabled.

  - Event-7:

    - Quality (e.g., L1-RSRP) of at least one new beam becomes a threshold (RRC configured) better than the Q-th best beam among the activated TCI states

    - Same type of beam comparison:

      - Scheme-1: ‘RS for current beam’ is the QCL RS in the activated TCI state with the Q-th best quality.

      - Scheme-2: ‘RS for current beam’ is the SSB which is QCLed with the QCL RS in the activated TCI state with the Q-th best quality.

- Report procedure

- Mode A (report in UCI using DG)

- Step 1: Transmit a one-bit report notification in UCI in PUCCH

  - dedicated PUCCH resource, not SR

- Step 2: Receive DCI indicating CSI trigger state associated with UEI beam report configuration(s) and DG for UCI,

- Step 3: Transmit report in UCI using DG in PUSCH

  - Mode B (report in UCI using type-1 CG)

    - Step 1: Transmit a one-bit report notification in UCI in PUCCH

  - dedicated PUCCH resource, not SR

    - Step 2: Transmit report in UCI using type-1 CG PUSCH, after at least X symbols

      - One-to-one mapping between the PUCCH resource and type-1 CG PUSCH resource: Only one periodic PUCCH resource and only one type-1 CG with the same periodicity.

      - type-1 CG PUSCH for dedicated for the beam report, which can carry any other UCI but can NOT carry UL-SCH.

  - Support cross-CC report for both modes, the report notification in PUCCH and report in PUSCH should be in the same CG and can be from the same or different CC(s).

MAC impact

On the MAC aspect, we discussed the following two issues in the previous meetings.

- Issue-1: whether to use common framework for UEI beam reporting and LTM event-triggered L1 measurement reporting.

- Issue-2: whether to capture UEI beam report event evaluation/triggering procedure in MAC or in RAN1 specification.

For Issue-1, the event evaluation/triggering of the two features are technically different in our view, which cannot share a common framework.

For UEI beam reporting Event-2, as summarized above,

- the basic trigger is that L1-RSRP of at least one new beam becomes a threshold (RRC configured) better than the current beam;

- the optional trigger up to UE capability is that in a configurable time window the L1-RSRP of one same new beam becomes a threshold (RRC configured) better than the current beam for at least M times (counter M is per new beam).

- Only Event-1 and Event-7 are additionally supported.

For LTM event-triggered L1 measurement reporting,

- TTT (like L3 event triggered MR) is used for event evaluation procedure.

- A list of events similar to L3 events are agreed.

Observation 1: UEI beam reporting and LTM event- triggered L1 measurement reporting are designed independently with different events, event evaluation, and reporting trigger agreed.

Since UEI beam reporting and LTM event-triggered L1 measurement reporting are progressed in different WIs and will be continued separately as RAN plenary confirmed, we think there is no need to align the two. It will definitely bring more coordination efforts in order to build a common framework.

Proposal 1: RAN2 confirms that UEI beam reporting does not share common framework of LTM event-triggered L1 measurement.

For Issue-2, RAN1 has been discussing on the details of event evaluation/triggering, such as whether/how to reset the timer and counter, when to evaluate, what’s the evaluation periodicity, whether the evaluation window is sliding or fixed, and so on. RAN2 cannot make decision on these details. Thus, it is immature to decide specifying anything in MAC.

Considering the current RAN1 progress and their leading role on the whole feature, we think there is no other choice than waiting for RAN1 progress. If necessary, we can ask RAN1 whether they expect anything to be captured in MAC. If RAN1 decides something needs to be captured in MAC, a LS with detailed procedure is needed to RAN2.

Proposal 2: Send a LS to RAN1 to ask whether the procedure of handling time window and counter for event evaluation/triggering is to be captured in RAN1 or RAN2 specification. Adopt the draft LS in the appendix as baseline.

RRC impact

Based on RAN1 LS R1-2410758, the UEI beam reporting configuration includes the following aspects.

- Measurement resource

  - newBeamResourceSetEvent2-r19

- Event related:

  - eventType-r19

  - eventThresholdEvent1-r19

  - eventThresholdEvent2-r19

  - valueOfQ-r19

  - eventDetectionTimeWindowLength-r19

  - eventInstanceCount-r19

- Report configuration:

  - reportTransmissionMode-r19

  - enabledCurrentBeamReport-r19

  - nrofReportedRS-UEIBR-r19

  - configuredResourceForSecondChannelOfModeB-r19

  - firstPUCCHResourceConfig-UEIBR-r19

It is noticed that all parameters are all included in CSI-ReportConfig (although not stabilized). The IE CSI-ReportConfig currently includes periodic/semi-persistent/aperiodic CSI report configuration. Some existing fields in CSI-ReportConfig can be considered as baseline for UEI beam reporting, for example, reportConfigId, carrier, resourcesForChannelMeasurement, reportConfigType, reportQuantity.

CSI-ReportConfig ::=                SEQUENCE {

reportConfigId                          CSI-ReportConfigId,

carrier                                 ServCellIndex                   OPTIONAL,   -- Need S

resourcesForChannelMeasurement          CSI-ResourceConfigId,

csi-IM-ResourcesForInterference         CSI-ResourceConfigId            OPTIONAL,   -- Need R

nzp-CSI-RS-ResourcesForInterference     CSI-ResourceConfigId            OPTIONAL,   -- Need R

reportConfigType                        CHOICE {

periodic                                SEQUENCE {

reportSlotConfig                        CSI-ReportPeriodicityAndOffset,

pucch-CSI-ResourceList                  SEQUENCE (SIZE (1..maxNrofBWPs)) OF PUCCH-CSI-Resource

},

semiPersistentOnPUCCH                   SEQUENCE {

reportSlotConfig                        CSI-ReportPeriodicityAndOffset,

pucch-CSI-ResourceList                  SEQUENCE (SIZE (1..maxNrofBWPs)) OF PUCCH-CSI-Resource

},

semiPersistentOnPUSCH                   SEQUENCE {

reportSlotConfig                        ENUMERATED {sl5, sl10, sl20, sl40, sl80, sl160, sl320},

reportSlotOffsetList                SEQUENCE (SIZE (1.. maxNrofUL-Allocations)) OF INTEGER(0..32),

p0alpha                                 P0-PUSCH-AlphaSetId

},

aperiodic                               SEQUENCE {

reportSlotOffsetList                SEQUENCE (SIZE (1..maxNrofUL-Allocations)) OF INTEGER(0..32)

}

},

reportQuantity                          CHOICE {

none                                    NULL,

cri-RI-PMI-CQI                          NULL,

cri-RI-i1                               NULL,

cri-RI-i1-CQI                           SEQUENCE {

pdsch-BundleSizeForCSI                  ENUMERATED {n2, n4}                                         OPTIONAL    -- Need S

},

cri-RI-CQI                              NULL,

cri-RSRP                                NULL,

ssb-Index-RSRP                          NULL,

cri-RI-LI-PMI-CQI                       NULL

},

reportFreqConfiguration                 SEQUENCE {

cqi-FormatIndicator                     ENUMERATED { widebandCQI, subbandCQI }                          OPTIONAL,   -- Need R

pmi-FormatIndicator                     ENUMERATED { widebandPMI, subbandPMI }                          OPTIONAL,   -- Need R

csi-ReportingBand                       CHOICE {

subbands3                               BIT STRING(SIZE(3)),

subbands4                               BIT STRING(SIZE(4)),

subbands5                               BIT STRING(SIZE(5)),

subbands6                               BIT STRING(SIZE(6)),

subbands7                               BIT STRING(SIZE(7)),

subbands8                               BIT STRING(SIZE(8)),

subbands9                               BIT STRING(SIZE(9)),

subbands10                              BIT STRING(SIZE(10)),

subbands11                              BIT STRING(SIZE(11)),

subbands12                              BIT STRING(SIZE(12)),

subbands13                              BIT STRING(SIZE(13)),

subbands14                              BIT STRING(SIZE(14)),

subbands15                              BIT STRING(SIZE(15)),

subbands16                              BIT STRING(SIZE(16)),

subbands17                              BIT STRING(SIZE(17)),

subbands18                              BIT STRING(SIZE(18)),

...,

subbands19-v1530                        BIT STRING(SIZE(19))

}   OPTIONAL    -- Need S

}                                                                                                           OPTIONAL,   -- Need R

timeRestrictionForChannelMeasurements           ENUMERATED {configured, notConfigured},

timeRestrictionForInterferenceMeasurements      ENUMERATED {configured, notConfigured},

codebookConfig                                  CodebookConfig                                              OPTIONAL,   -- Need R

dummy                                           ENUMERATED {n1, n2}                                         OPTIONAL,   -- Need R

groupBasedBeamReporting                     CHOICE {

enabled                                     NULL,

disabled                                    SEQUENCE {

nrofReportedRS                          ENUMERATED {n1, n2, n3, n4}                                 OPTIONAL    -- Need S

}

},

...omitting other fields...

}

For UEI-BR new beam measurement resource, resourcesForChannelMeasurement may or may not be reused. Note currently resourcesForChannelMeasurement points to a CSI-ResourceConfig which can include both CSI-RS resource sets (nzp-CSI-RS-ResourceSetList) and SSB resource sets (csi-SSB-ResourceSetList), however, for UEI beam reporting either CSI-RS resource set or SSB resource set is configured for new beam measurement.

For the event related parameters, a new report type for UEI-BR should be defined. The legacy reportConfigType for P/SP/AP CSI reporting cannot be configured in a CSI report configuration for UEI-BR.

Proposal 3: Discuss the following points for the configuration of UEI beam reporting.

- Whether resourcesForChannelMeasurement can be used to indicate a CSI-ResourceConfig for the new beam measurement of UEI beam reporting. Note the new beam is either CSI-RS (nzp-CSI-RS-ResourceSetList) or SSB (csi-SSB-ResourceSetList).

- Define a new report type, i.e., UEI-BR, to include the event related parameters. Discuss how to handle legacy reportConfigType.

- In reportQuantity, either cri-RSRP or ssb-Index-RSRP is configured for UEI-BR.

- A new IE under CSI-ReportConfig can be introduced to include all the other required parameters for UEI-BR.

- Currently, timeRestrictionForChannelMeasurements, timeRestrictionForInterferenceMeasurements, and groupBasedBeamReporting are mandatory present in CSI-ReportConfig. We can ask RAN1 whether these fields are applicable for UEI-BR.

## 2.2 CSI enhancements

We group several objectives in WID into CSI enhancements , including increased number of ports (up to 128) for CSI, inter-TRP calibration for CJT, and 3Tx for UL.

Here we discuss the CBSR (codebook subset restriction) parameters which are informed in particular by RAN1 LS R2-2404114.

[116bis] Agreement For the Rel-19 Type-II codebook refinement for 48, 64, and 128 CSI-RS ports, on CBSR, refine the legacy CBSR as follows: - Only 1-bit hard restriction is supported (analogous to Rel-18 Type-II) - Moving (N1, N2) configuration out from CBSR IE and the CBSR can be optional configured - Send LS to RAN2, and subject to RAN2 consent - Group-based CBSR granularity where each bit in the CBSR is associated with a set of X1X2 SD basis vectors, where the set includes X1 adjacent SD basis vectors along the N1 direction and/or X2 adjacent SD bases along the N2 direction - FFS: Value(s) of X1 and X2 and detailed design/spec impact FFS: Whether/how to enable shared CBSR in RRC configuration for Type-I/-II codebooks with a same (N1,N2). [116bis] Agreement For the Rel-19 Type-I SP codebook refinement for 48, 64, and 128 CSI-RS ports, regarding CBSR design: - 1-bit hard restriction is supported (analogous to Rel-15 Type-I) - FFS: 3-bit scaling factor for soft restriction is supported with the scaling factor taken into account in CQI/PMI calculation - Moving (N1, N2) configuration out from CBSR IE and the CBSR can be optional configured - Send LS to RAN2, and subject to RAN2 consent - N1⋅O1⋅N2⋅O2X1⋅X2-bit CBSR where each bit in the CBSR is associated with a set of X1X2 SD basis vectors, where the set includes X1 adjacent SD basis vectors along the N1 direction and/or X2 adjacent SD bases along the N2 direction - FFS: Value(s) of X1 and X2 and detailed design/spec impact FFS: Whether/how to enable shared CBSR in RRC configuration for Type-I/II codebooks with a same (N1,N2).

Further, in RAN1 LS R1-2410758 more information on the relevant parameters are provided. For Rel-19 type-I single-panel codebook, the following parameters are required by RAN1.

As informed, CBSR bit string is optional. The length of the CBSR bit string is (N1O1N2O2)/(X1X2), where (N1, N2) and (X1, X2) take the defined values, and O1=O2=4 according to RAN1 agreement. Table1 below shows the CBSR bit string length for different cases for Rel-19 type-I single-panel codebook.

Table 1: CBSR bit string length for Rel-19 type-I single-panel codebook

For Rel-19 type-II and type-II Doppler codebook, the following parameters are required by RAN1.

As informed, CBSR bit string is optional. The length of the CBSR bit string is (N1N2)/(X1X2), where (N1, N2) and (X1, X2) take the defined values. Table 2 below shows the CBSR bit string length for different cases for Rel-19 type-II and type-II Doppler codebook.

Table 2: CBSR bit string length for Rel-19 type-II codebook

In Rel-15/16/17 (N1, N2) and CBSR configuration, the signaling of (N1, N2) and CBSR bit string are coupled since the length of CBSR bit string is a function of (N1, N2). Although CBSR is not a mandatory parameter for a configured codebook, it is always signaled with (N1, N2) because (N1, N2) is a mandatory parameter for a configured codebook. The cost of such signaling design is the overhead. Even if CBSR is not configured, a bit string of all 0s is still signaled. In Rel-18, in order to make CBSR bit string optional, a CHOICE of no CBSR is used, as shown below.

N1-N2-CBSR-r18 ::=     CHOICE {

two-one-r18            CHOICE {no-cbsr-r18 NULL, cbsr-r18 BIT STRING (SIZE (8))},

two-two-r18            CHOICE {no-cbsr-r18 NULL, cbsr-r18 BIT STRING (SIZE (27))},

four-one-r18           CHOICE {no-cbsr-r18 NULL, cbsr-r18 BIT STRING (SIZE (16))},

three-two-r18          CHOICE {no-cbsr-r18 NULL, cbsr-r18 BIT STRING (SIZE (35))},

six-one-r18            CHOICE {no-cbsr-r18 NULL, cbsr-r18 BIT STRING (SIZE (24))},

four-two-r18           CHOICE {no-cbsr-r18 NULL, cbsr-r18 BIT STRING (SIZE (43))},

eight-one-r18          CHOICE {no-cbsr-r18 NULL, cbsr-r18 BIT STRING (SIZE (32))},

four-three-r18         CHOICE {no-cbsr-r18 NULL, cbsr-r18 BIT STRING (SIZE (59))},

twelve-one-r18         CHOICE {no-cbsr-r18 NULL, cbsr-r18 BIT STRING (SIZE (48))},

four-four-r18          CHOICE {no-cbsr-r18 NULL, cbsr-r18 BIT STRING (SIZE (75))},

eight-two-r18          CHOICE {no-cbsr-r18 NULL, cbsr-r18 BIT STRING (SIZE (75))},

sixteen-one-r18        CHOICE {no-cbsr-r18 NULL, cbsr-r18 BIT STRING (SIZE (64))}

}

For Rel-19 codebook, as informed by RAN1 LS R2-2404114, (N1, N2) configuration should be moved out from CBSR IE. If we reuse the Rel-18 CHOICE structure, the CBSR bit string has to be signaled also as a CHOICE of (X1, X2), which leads to CHOICE embedded in 3 layers. Here we provide some other options for discussion.

Option 1:

Reuse the Rel-18 CHOICE structure for the CBSR of Rel-19 type-1 and type-2 codebooks, i.e., bit string length is signaled as a CHOICE of (N1, N2), a CHOICE of (no CBSR), and a CHOICE of (X1, X2), which leads to CHOICE embedded in 3 layers. This option does not need separate parameters for N1, N2, X1, X2, but cannot reflect nonapplicable (X1, X2) pairs so extra configuration restrictions have to be capture in field description.

Option 2:

Alternatively, (N1, N2) can be signaled as a separate parameter, and CBSR can be signaled as a CHOICE of (X1, X2) and a CHOICE of N1N2. Note there are many (X1, X2) values not applicable to type-II and type-II Doppler codebook, but only applicable to type-I single-panel codebook. This option does not need separate parameter/description for X1, X2, and can directly reflect applicable (X1, X2) pairs in the CHOICE of (X1, X2). An example is shown below.

n1-n2-r19		ENUMERATED {eight-three, six-four, sixteen-two, eight-four, sixteen-four, eight-eight},

typeI-codebookSubsetRestriction-r19		typeI-X1-X2-CBSR-r19							 OPTIONAL    -- Need R

typeI-X1-X2-CBSR-r19 ::=     CHOICE {

one-one-r19            CHOICE {twentyfour BIT STRING (SIZE (384)), thirtytwo BIT STRING (SIZE (192)), sixtyfour BIT STRING (SIZE (1024))},

two-one-r19            CHOICE {twentyfour BIT STRING (SIZE (192)), thirtytwo BIT STRING (SIZE (256)), sixtyfour BIT STRING (SIZE (512))},

two-two-r19            CHOICE {twentyfour BIT STRING (SIZE (96)), thirtytwo BIT STRING (SIZE (128)), sixtyfour BIT STRING (SIZE (256))},

four-one-r19           CHOICE {twentyfour BIT STRING (SIZE (96)), thirtytwo BIT STRING (SIZE (128)), sixtyfour BIT STRING (SIZE (256))},

four-two-r19           CHOICE {twentyfour BIT STRING (SIZE (48)), thirtytwo BIT STRING (SIZE (64)), sixtyfour BIT STRING (SIZE (128))},

four-four-r19          CHOICE {twentyfour BIT STRING (SIZE (24)), thirtytwo BIT STRING (SIZE (32)), sixtyfour BIT STRING (SIZE (64))}

}

typeII-codebookSubsetRestriction-r19		typeII-X1-X2-CBSR-r19							 OPTIONAL    -- Need R

typeII-X1-X2-CBSR-r19 ::=     CHOICE {

one-one-r19            CHOICE {twentyfour BIT STRING (SIZE (24)), thirtytwo BIT STRING (SIZE (32)), sixtyfour BIT STRING (SIZE (64))},

two-one-r19            CHOICE {twentyfour BIT STRING (SIZE (12)), thirtytwo BIT STRING (SIZE (16)), sixtyfour BIT STRING (SIZE (32))},

two-two-r19            CHOICE {twentyfour BIT STRING (SIZE (6)), thirtytwo BIT STRING (SIZE (8)), sixtyfour BIT STRING (SIZE (16))},

four-one-r19           CHOICE {twentyfour BIT STRING (SIZE (6)), thirtytwo BIT STRING (SIZE (8)), sixtyfour BIT STRING (SIZE (16))},

four-two-r19           CHOICE {thirtytwo BIT STRING (SIZE (4)), sixtyfour BIT STRING (SIZE (8))}

}

Option 3:

(N1, N2) is signaled as a separate parameter, and CBSR is optionally signaled as a variable BIT STRING. Variable BIT STRING is widely used in UE capability signaling, which can be utilized to simplify the signaling of CBSR. This option needs extra parameters for (X1, X2) and capture nonapplicable pairs in FD. An example is shown below.

n1-n2-r19		ENUMERATED {eight-three, six-four, sixteen-two, eight-four, sixteen-four, eight-eight},

typeI-x1-x2-r19		ENUMERATED {one-one, two-one, two-two, four-one, four-two, four-four}	OPTIONAL    -- Need R

typeII-x1-x2-r19		ENUMERATED {one-one, two-one, two-two, four-one, four-two}	OPTIONAL    -- Need R

typeI-codebookSubsetRestriction-r19		BIT STRING (SIZE (24, 32, 48, 64, 96, 128, 192, 256, 384,   512, 1024))							 OPTIONAL    -- Need R

typeII-codebookSubsetRestriction-r19		BIT STRING (SIZE (4, 6, 8, 12, 16, 24, 32, 64))		OPTIONAL    -- Need R

Proposal 4: Discuss how to signal CBSR for Rel-19 type-I and type-II codebooks.

- Option 1: Reuse the Rel-18 CHOICE structure for the CBSR of Rel-19 type-1 and type-2 codebooks, i.e., CBSR bit string length is as a CHOICE of (N1, N2), a CHOICE of (no CBSR), and a CHOICE of (X1, X2), which leads to CHOICE embedded in 3 layers.

- Option 2: (N1, N2) is configured as a separate mandatory parameter not coupled with CBSR, and CBSR is optionally configured as a CHOICE of (X1, X2) and a CHOICE of N1N2, i.e., 2-layer CHOICE structure.

- Option 3: (N1, N2) is configured as a separate mandatory parameter not coupled with CBSR, (X1, X2) is optionally configured as a separate parameter, and CBSR is optionally configured using variable BIT STRING.

# Conclusion

Based on the discussion, we have the following proposals.

Observation 1: UEI beam reporting and LTM event- triggered L1 measurement reporting are designed independently with different events, event evaluation, and reporting trigger agreed.

Proposal 1: RAN2 confirms that UEI beam reporting does not share common framework of LTM event-triggered L1 measurement.

Proposal 2: Send a LS to RAN1 to ask whether the procedure of handling time window and counter for event evaluation/triggering is to be captured in RAN1 or RAN2 specification. Adopt the draft LS in the appendix as baseline.

Proposal 3: Discuss the following points for the configuration of UEI beam reporting.

- Whether resourcesForChannelMeasurement can be used to indicate a CSI-ResourceConfig for the new beam measurement of UEI beam reporting. Note the new beam is either CSI-RS (nzp-CSI-RS-ResourceSetList) or SSB (csi-SSB-ResourceSetList).

- Define a new report type, i.e., UEI-BR, to include the event related parameters. Discuss how to handle legacy reportConfigType.

- In reportQuantity, either cri-RSRP or ssb-Index-RSRP is configured for UEI-BR.

- A new IE under CSI-ReportConfig can be introduced to include all the other required parameters for UEI-BR.

- Currently, timeRestrictionForChannelMeasurements, timeRestrictionForInterferenceMeasurements, and groupBasedBeamReporting are mandatory present in CSI-ReportConfig. We can ask RAN1 whether these fields are applicable for UEI-BR.

Proposal 4: Discuss how to signal CBSR for Rel-19 type-I and type-II codebooks.

- Option 1: Reuse the Rel-18 CHOICE structure for the CBSR of Rel-19 type-1 and type-2 codebooks, i.e., CBSR bit string length is as a CHOICE of (N1, N2), a CHOICE of (no CBSR), and a CHOICE of (X1, X2), which leads to CHOICE embedded in 3 layers.

- Option 2: (N1, N2) is configured as a separate mandatory parameter not coupled with CBSR, and CBSR is optionally configured as a CHOICE of (X1, X2) and a CHOICE of N1N2, i.e., 2-layer CHOICE structure.

- Option 3: (N1, N2) is configured as a separate mandatory parameter not coupled with CBSR, (X1, X2) is optionally configured as a separate parameter, and CBSR is optionally configured using variable BIT STRING.

# References

- RP‑242356	Revised Work Item: NR mobility enhancements Phase 4, RAN#105.

- R2-2404114, LS to RAN2 on CBSR for Rel-19 MIMO

- R1-2410758, LS to RAN2 on RRC and MAC CE impacts for Rel-19 NR MIMO Ph5

# Appendix: LS to RAN1 on UE initiated beam reporting

1	Overall description

For UE initiated beam reporting, RAN2 has been discussing potential RAN2 specification impacts. We noticed the following RAN1 agreement.

Agreement

Regarding the triggering event determination for Event 2:

- If within a time window (which is configurable), the number of Event-2 instance(s) for at least one same new beam is greater than or equal to a configurable number M, UE initiated beam report occurs.

  - Note: Event-2 instance for a new beam is determined if the L1-RSRP of the new beam becomes a threshold value better than the current beam

Above feature is subject to UE capability.

- Basic feature: Once the L1-RSRP of the new beam becomes a threshold value better than the current beam, UE initiated beam report occurs

FFS: Whether the above is captured in RAN1 or RAN2 specification.

As RAN1 has discussed details on event evaluation/triggering, we would like to ask the following questions to identify RAN2 specification impacts and avoid work duplication.

Q1: Is the procedure of handling the time window and the counter to be captured in RAN1 or RAN2 specification?

Q2: If the procedure is to be captured in RAN2 specification, what is the detailed procedure RAN2 specification has to capture?

2	Actions

To RAN1

ACTION:   RAN2 respectfully asks RAN1 to provide answers to the above questions.