3GPP TSG-RAN WG2 Meeting #129	R2-2500219

Athens, Greece, Feb. 17th – 21st, 2025

Agenda item:	8.8.2

Source:	Samsung

Title:	Discussion on Downlink Coverage Enhancement

WID/SID:	NR_NTN_Ph3-Core

Document for:	Discussion and Decision

# Introduction

For Rel-19 NR NTN, the latest WID has confirmed the objective of DL coverage enhancement (DL-CE) [1].

- Specify downlink coverage enhancements targeting support for additional reference satellite payload parameters covering both GSO and NGSO constellations operating in FR1-NTN or FR2-NTN [RAN1, RAN2, RAN4] - Define additional reference satellite payload parameters assuming power sharing among satellite beams or different satellite beam patterns/size (i.e. wide or narrow) across the satellite footprint, such that satellite beams may not all be simultaneously active or may be active below the nominal EIRP density per satellite beam (see section 6.1.1 in TR 38.821) due to limited power and limited feeder link bandwidth. - Define the corresponding power sharing assumptions and necessary link level and system level evaluation methodology and relevant KPIs for evaluations of the coverage, to allow for identification of physical channels/signals and system-level aspects that need enhancements and the corresponding needed improvements. - Specify solutions, including link level enhancements for FR1-NTN and system level enhancements for FR1-NTN and FR2-NTN, allowing dynamic and flexible power sharing between satellite beams or different satellite beam patterns/size (i.e. wide or narrow) across the satellite footprint. - Link level enhancements are to be specified for the following channels: - PDCCH at least for CSS (except for Type-3) via PDCCH repetition - PDSCH with Msg4 via PDSCH repetition - PDSCH with SIB1 via 2 PDSCH repetitions within 20 ms duration - System-level enhancements are to be specified for the following: - Support of extended periodicity of the half frames with SS/PBCH blocks assumed by UE during initial access. - The maximum of the additional default value (apart from the existing 20ms value) is 160 ms - Notes for this objective: - SSB channel enhancement other than SSB periodicity extension is not considered - RAN1 should consider issues such as UE’s cell search complexity and impact to initial cell selection, latency and success rate, for the above extension - The SSB periodicity enhancements potentially defined in this WID only apply to NTN operation - Antenna gain of UE shall be assumed to be -5.5dBi in case of smartphone in FR1-NTN, the UE is assumed to be a full duplex UE, and at least 2Rx are considered at the UE - NGSO to be considered in priority: LEO Set-1 @ 600 km

According to the objective, link-level enhancements to be specified are PDCCH repetition and PDSCH repetition, system-level enhancement to be specified is SSB periodicity extension up to 160ms.

For link-level enhancements, we expect to wait more RAN1 progress before we can identify the relevant RAN2 impacts. In this contribution, we share our views on RAN2 impacts of SSB periodicity extension.

# Discussion

As clarified by WID that SSB periodicity is up to 160ms (i.e., no larger than 160ms), we can confirm there is no impact to RRC configurations that are relevant to SSB periodicity as the value range up to 160ms is supported. These RRC configurations include what we have identified before, i.e., ssb-PeriodicityServingCell, measurement gap periodicity, SMTC periodicity, ssb-Periodicity-r17 for NonCellDefiningSSB-r17, nAndPagingFrameOffset.

Observation 1: There is no impact to RRC configurations that are relevant to SSB periodicity as the value range up to 160ms is supported, including ssb-PeriodicityServingCell, measurement gap periodicity, SMTC periodicity, ssb-Periodicity-r17 for NonCellDefiningSSB-r17, nAndPagingFrameOffset.

One aspect we have discussed a lot is how to bar legacy UEs to access DL-CE cells if necessary. RAN1 has reached an observation regarding the backward compatibility.

Our interpretation is that legacy UEs assuming default SSB periodicity of 20ms may have difficulties in initial search if the cell used for initial frequency scan has SSB periodicity larger than 20ms, but they may still be able to camp on such a cell. It is possible that a legacy UE happens to detect SSBs within 20ms even for a cell with larger SSB periodicity, or UE is able to detect longer-period SSBs by implementation. Besides, as specified in TS 38.304 UE itself will treat the cell as barred due to being unable to acquire the MIB/SIB1 (e.g., when UE fails to detect SSBs of longer periodicity). Thus, barring legacy UE to DL-CE cells seems not a necessity.

Observation 2: A legacy UE assuming default SSB periodicity of 20ms can still access DL-CE cells with longer SSB periodicity if it happens to detect SSBs for such a cell or is able to access by implementation. As specified, UE will treat the cell as barred due to being unable to acquire the MIB/SIB1 (e.g., when UE fails to detect SSBs of longer periodicity).

Similarly, considering the desire to provide NTN service with a broader coverage, it is not strongly motivated either to prevent legacy NTN UEs from reselecting DL-CE cells with longer SSB periodicity. The current SMTC periodicity {sf5, sf10, sf20, sf40, sf80, sf160} can imply the SSB periodicity of neighbor cells per frequency, and UE can select cells properly by implementation, e.g., with or without (de)prioritization of cells operating with larger SSB periodicity.

Proposal 1: Do not proactively bar/prevent legacy NTN UEs to access/select DL-CE cells with SSB periodicity larger than 20ms.

Another aspect is potential SMTC impacts/enhancements considering the SSB periodicity extension and/or beam hopping. Currently, SMTC which is configured per frequency layer for neighbor cell measurement in connected mode (in measConfig) and in idle/inactive mode (in SIB2/4) supports one periodicity and up to 4 different offsets for NTN. Take an example of the scattered beam hopping pattern analyzed in [2], as shown in the figure below. The serving cell in the center only needs to provide SMTCs for neighbor cells around it, for instance, up to 6 neighbor cells in the figure below. Those neighbor cells can operate with hopping beams, as well as frequency reuse. Different colors can represent beams illuminated at the same time, and can also represent different frequencies. Suppose some of the rounding neighbor cells are operating with hopping beams on the intra-frequency as the serving cell, while some others are operating with inter-frequencies. For each frequency, SMTC can support up to 4 offsets to accommodate beam hopping among neighbor cells on the same frequency (e.g., 4 neighbor cells on this same frequency are illuminated one by one, and their associated SMTC windows are shifting in time which correspond to 4 offsets). In this sense, the current SMTC supporting up to 4 offsets per periodicity should be sufficient.

Observation 3: Considering neighbor cells can operating with beam hopping pattern on the intra-frequency as the serving cell, as well as with frequency reuse pattern on inter-frequencies, the current SMTC per frequency layer supporting up to 4 offsets should be sufficient.

Proposal 2: Do not introduce extra SMTC for NTN.

Figure. Scattered beam hopping pattern [2]

Having said that, beam hopping which causes neighbor cells not illuminated/served all the time renders an opportunity for UE power saving. Basically, NW should configure SMTC properly to ensure the SSB beams of cells associated to the SMTC are illuminated during the corresponding SMTC window. From UE perspective, UE does not need to measure neighbor cells that are not in its proximity. To this end, location information can be provided for neighbor cells or for the associated SMTC to help UE measure only close-enough neighbor cells.

Proposal 3: Provide location information for neighbor cells or for the associated SMTCs to help UE measure only close-enough neighbor cells.

# Conclusion

In this contribution we have the following proposals.

Observation 1: There is no impact to RRC configurations that are relevant to SSB periodicity as the value range up to 160ms is supported, including ssb-PeriodicityServingCell, measurement gap periodicity, SMTC periodicity, ssb-Periodicity-r17 for NonCellDefiningSSB-r17, nAndPagingFrameOffset.

Observation 2: A legacy UE assuming default SSB periodicity of 20ms can still access DL-CE cells with longer SSB periodicity if it happens to detect SSBs for such a cell or is able to access by implementation. As specified, UE will treat the cell as barred due to being unable to acquire the MIB/SIB1 (e.g., when UE fails to detect SSBs of longer periodicity).

Proposal 1: Do not proactively bar/prevent legacy NTN UEs to access/select DL-CE cells with SSB periodicity larger than 20ms.

Observation 3: Considering neighbor cells can operating with beam hopping pattern on the intra-frequency as the serving cell, as well as with frequency reuse pattern on inter-frequencies, the current SMTC per frequency layer supporting up to 4 offsets should be sufficient.

Proposal 2: Do not introduce extra SMTC for NTN.

Proposal 3: Provide location information for neighbor cells or for the associated SMTCs to help UE measure only close-enough neighbor cells.

# Reference

RP-243300, Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3

R2-2410804, Downlink coverage enhancement SMTC impacts, Sequans Communications