3GPP TSG-RAN WG2 Meeting #129	R2-2500220

Athens, Greece, Feb. 17th – 21st, 2025

Agenda item:	8.8.4

Source:	Samsung

Title:	Discussion on Broadcast service area

WID/SID:	NR_NTN_Ph3-Core

Document for:	Discussion and Decision

# Introduction

For Rel-19 NTN, one objective is to support MBS broadcast service for smaller areas in an NTN cell. The detailed justification and objective in WID are provided as follows.

MBS feature provides an important add-value for NR NTN system, leveraging the large coverage of the NTN compared to TN. Terrestrial MBS features are equally available for NR NTN in the 5G specifications, but for some cases the intended service area is expected to be smaller than the coverage of a Uu cell, some enhancements need to be done to notify the service area of a Broadcast service. Specify signaling of the intended service area of a broadcast service (e.g. MBS broadcast) via NR NTN [RAN2, RAN3] - Specify SIB signaling to indicate the intended service area in case the satellite footprint covers a larger area. [RAN2] - Specify the necessary signaling between CN and NG-RAN. [RAN3]

In this contribution, we continue to share our views on the MBS broadcast intended service area, and discuss service area for ETWS.

# Discussion

## MBS intended service area

RAN2 has made several agreements regarding the signaling of MBS intended service area. As a summary,

- a new SIB provides a list of intended service areas, FFS how to refer to an intended service area (e.g., index or ID);

- in MCCH, each MBS session is associated to intended service area ID(s).

Regarding the FFS, we think the simple solution is to define an ID for each intended service area. To associated to a MBS session, one or multiple intended service area IDs can be included in MBS-SessionInfo in MCCH.

Proposal 1: In the new SIB, an ID is used to identify each MBS intended service area.

UE can utilize the list of intended service areas to skip acquiring MCCH if UE is not in any intended areas. If UE is in an intended service area, UE acquires MCCH and receives the associated MBS session by receiving MTCH. Note this requires that NW indicates the intended service areas of all MBS sessions.

Proposal 2: If UE is not in any intended service areas, UE does not need to acquire MCCH.

RAN2 has agreed that both (quasi)-earth-fixed and earth-moving cells are supported for MBS broadcast service. We understand the intended service area represents a fixed geographic area on the ground for both cases. However, due to the fast movement of quasi-earth-fixed or earth-moving cells, the cell may provide an MBS broadcast service to an intended area for a quite short duration, which will lead to frequent updates of MBS broadcast service and the associate intended area and, thus, UE frequent acquisitions of SIB or MCCH. This is different from unicast service which is UE specific and does not need to provide intended service area. To avoid this issue, some additional information (e.g., duration, etc.) can be provided for an MBS broadcast service and its intended area for quasi-earth-fixed or earth-moving cells.

For example, a quasi-earth-fixed or earth-moving cell can signal intended service areas for MBS broadcast services that are to be provided in the future and indicate the time interval during which the service is going to provide. In this way, the intended service areas and the associated MBS broadcast services are announced in advance, and UE only considers to receive a service when the service provision time comes. If NW can plan in advance the MBS broadcast service in quasi-earth-fixed or earth-moving cells (e.g., based on the trajectory and constellation of satellites), this approach can reduce frequent updates/acquisition of the intended service areas and the associated MBS broadcast services in SIB and MCCH.

Proposal 3: Discuss for quasi-earth-fixed or earth-moving cells how to avoid NW frequent update and UE frequent acquisition of information on MBS broadcast service and the intended service area.

## ETWS service area

In the SA1 LS reply R2-2500045, SA1 informs the specification of warning notification area for ETWS, introduced in TS 22.268.

Based on the SA1’s reply, ETWS shall be able to broadcast Warning Notification Area(s) for Primary or Secondary Notification(s). RAN2 can confirm Warning Notification Area is broadcasted in system information. Where to include the Warning Notification Area has to be discussed. In our view, the approach similar to CMAS can be considered, where a warningAreaCoordinatesSegment is included in the notification message. For ETWS, a warningAreaCoordinatesSegment can be included in Primary/Secondary Notification message in SIB7.

Proposal 4: Introduce warningAreaCoordinatesSegment in ETWS Primary Notification (SIB6) and in ETWS Secondary Notification (SIB7).

# Conclusion

In this contribution we discussed the following proposals.

Proposal 1: In the new SIB, an ID is used to identify each MBS intended service area.

Proposal 2: If UE is not in any intended service areas, UE does not need to acquire MCCH.

Proposal 3: Discuss for quasi-earth-fixed or earth-moving cells how to avoid NW frequent update and UE frequent acquisition of information on MBS broadcast service and the intended service area.

Proposal 4: Introduce warningAreaCoordinatesSegment in ETWS Primary Notification (SIB6) and in ETWS Secondary Notification (SIB7).

# Reference

RP-240775, Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3

R2-2500045, Reply LS on requirements for ETWS primary notification.