3GPP TSG-RAN WG2 Meeting #129										 					R2-2500255

Athens, GR 17th Feb. 2025 – 21st Feb. 2025             				     revision of R2-2409583

Agenda Item:	8.2.2

Source: 	Xiaomi

Title:  	Discussion on A-IOT paging procedure

Document for:	Discussion and Decision

1	Introduction

In R19, a new WI for Ambient IoT (Internet of Things) in NR is setup and for paging part the objective is as below:

- RAN2 scope: - Specify the necessary functions and procedures for an Ambient IoT compact protocol stack and lightweight signalling procedure to enable DO-DTT and DT data transmission: - A-IoT Paging, including subsequent paging for the same service. Support the options that a paging message contains one identifier, and that a paging message contains no identifier. Temporary identifier is not supported, unless required by SA WGs. Note: RAN2 aims to design a paging message format such that multiple identifiers can be contained in one paging message, for forward compatibility purposes.

In this paper, we would like to discuss the issues in paging procedure.

2	Discussion

2.1 Paging duplication detection

In RAN2#127 meeting, to solve the device availability issue and paging reception failure issue, RAN2 greed that “Support mechanism for reader to be able to trigger multiple/subsequent AIoT paging messages that are associated with the same request from the CN.”

In RAN2#128 meeting, RAN2 reached following agreements on paging.

Agreements on paging

- An ID in paging message is beneficial to avoid duplicated response for same service request. FFS whether ID is generated by the reader or CN. FFS on the ID size.

- Capture the scenario that different readers may page the same device for same service request.  We will discuss this in WI phase (if in scope of WI) and consider progress from all the WGs

- Whether we include further information in paging message to indicate service type/command type or to indicate that further subsequent messages are coming can be discussed in WI phase once we understand more the paging design and AS ID.

Regarding open issues FFS whether ID is generated by the reader or CN, as RAN2 already agreed that An ID in paging message is beneficial to avoid duplicated response for same service request, it should be applied for the scenario that different readers may page the same device for same service request as well.

There are two options:

Option 1: duplication ID is generated by the CN

The benefit of option 1 is that it is easy for the CN to generate the unique duplication ID among multiple readers for the same session. It can be used by a device to determine whether the response is needed, especially for the scenario that different readers may page the same device for same service request.

Option 2: duplication ID is generated by the reader

If the paging duplication ID is generated by the reader, it should be unique in the reader. To avoid the duplicated reporting from the device for the paging from the same reader, the paging message should include the reader ID. The device shall skip the paging when both the paging duplication id and reader ID in paging message are same. The drawback of the scenario is that additional overhead is needed, and it cannot avoid the duplicated response for the scenario that different readers may page the same device for same service request.

Proposal 1: The paging duplication id is generated by CN and it is unique among multiple readers for the same session. Send LS to SA2 to inform them of our decision.

For the paging duplication times, the reader has no idea how many devices should respond the paging unless CN can provide the number of devices to be paged. Therefore, CN is in the best position to configure it, i.e., it can be controlled by CN, e.g., configure a times of paging duplication and reader can repeat the paging based on CN configured paging duplication times. If the CN consider that the paging duplication is not needed anymore, e.g., the CN can check whether it has received enough response and know the paging duplication is not needed any more.  The CN can ask RAN to stop the paging duplication by signaling.

Proposal 2: The paging duplication times should be configured by CN.

Proposal 3: Paging repetition can also be stopped by CN via signaling to reader.

Regarding the open issue FFS on the ID size, some proposals were mentioned in last RAN2 meeting, e.g.,

- A-IOT Session id or task ID

- Part of A-IOT Session id or task ID

- Random ID;

In NR, there are several mechanisms for changing/duplication detection, E.g.,

- SIB valueTag (0..31) is used to identify whether the SIB is changed or not if the UE comes from a coverage hole.

- NDI (1 bit) in DCI is used to indicated the current transmission is new transmission or retransmission.

- SN number in PDCP or RLC header is used for duplication detection.

The ID size should be the balance between resources efficiency and reliability of duplication detection. The AIoT device may miss several paging since it needs longer time for charging. If a device has responded a paging, and then went to sleep for charging, it may miss several paging in between.  If the ID size is not larger enough, the device may see the paging with the same ID when ID wrap around happens. The device will not respond this paging although it is for the different session, i.e., the paging will be missed.  However longer ID size needs more resources for the transmission from the reader side and more power consumption for the reception and decoding from device side. To our understanding, one of the purposes of the paging duplication is to avoid the case that a device misses paging due to charging, i.e., the repetition times should be longer enough. Additional larger ID size, e.g., 4 bits should be sufficient to avoid wrap around issue.

If proposal 1 is agreed, i.e., the duplication ID is generated by the CN, it is up to CN how to generate it, e.g., based on session ID, task ID or random ID.

Proposal 4: The ID size for paging duplication detection is 4 bits.

For paging triggered CF access, the target device id will be included in paging message, and the reader will also know whether the target device complete the service or not, then network can send paging only include the device id who did not complete the service, no need to repeat the paging message to include the device id who had complete the service.

Proposal 5: Multiple/subsequent AIoT paging is only used for paging triggered CB access, i.e., paging triggered CF access will not support multiple/subsequent AIoT paging.

2.2 Content of A-IOT paging message

According to WI, the AIOT paging may contain (1) an ID of a single A-IoT device; (2) a group ID that maps to multiple A-IoT devices; (3) all devices;

It is well known that the device related ID will be included in A-IOT paging message and it is not clear whether it is visible for IOT MAC or container in IOT MAC. In NR paging, the UE ID, e.g., 5G S-TMSI is one CN ID but it is visible for AS in paging message. We also think the paging procedure and RACH procedure are IOT MAC related procedure. The reader will determine CF access or CB access based on device related ID, but the IOT MAC does not need to know the details of device ID or Group ID.

Proposal 6a:  A-IOT Paging message includes one device ID, or one device Group ID (or MASK) or NO device related ID (i.e., All). The device related ID is visible to IOT MAC and details of device related ID is transparent to A-IOT MAC in device.

Regarding how to determine CF or CB, RAN2 asked SA2 whether the CN can indicate that the service is targeted for a single or multiple devices; However according to the reply LS from SA2, CN can provide the information to the reader if available. That means this information may not always be available.

Q1: Is it feasible for CN to provide the following information to the reader? - the service type of A-IoT (e.g. inventory, command) - whether the service is targeted for a single or multiple devices; - the approximate number of target devices (if available). SA2 answer: CN can provide the information listed above to the reader if available.

If the reader can get the information from the CN, i.e., single device, the reader can trigger CF access for paging, otherwise CB based access should be used.

Proposal 6b: Reader can decide whether CB or CF access should be used based on the information from CN, i.e., single device ID from CN will trigger CF access, otherwise trigger CB access.

According to RAN1 agreements, the TDM/FDM resource will be used for initial access and the concerned devices can access the network on different TDM/FDM resource. It is common understanding that CB/CF access resource should be indicated to the devices before the device initial the initial access procedure. If CF access is triggered, the reader will also allocate specific resource for each device. So, the reader should know some information to decide CB or CF access is triggered and corresponding resource allocation.

In RAN2#127 meeting, RAN2 agreed that some information from CN is visible to the reader. The reader can decide the slot number based on the number of target devices from CN. So, the reader can indicate the slot number for access following this paging.

Agreements - At least the following information are considered useful to be visible to the reader from CN - The service type of A-IoT (e.g. inventory, command) . FFS if more information on command type (e.g. read/write/disable) is useful - targeted for one or more than one devices; - approximate number of target devices (if available). FFS on mandatory/optional 2 RAN2 assumes that commands (e.g., read/write/disable) and/or inventory are carried over the AIOT interface as upper layer data.

Proposal 7: Access occasion configuration and slot number are configured in paging message.

The key point of contention-free access is that there is no collision of RN16 and access occasion for the access and followed communication. So, the per device access occasion should be configured for the target device in paging message, the same reason for RN16 if needed in CF access.

Proposal 8: For CF access trigger, device specific access occasion and RN 16 if needed is included in paging message.

From device perspective, the device can know the CF or CB access according to the device id related information and resource allocation for access.

Proposal 9: The device will know CF access or CB access according to the device ID information and corresponding resource allocation in paging message, i.e., CF or CB access indication will not be included in paging message.

In R19, only DT and DO-DTT will be triggered, the device will response based on network R2D command trigger step by step. So, we cannot see the motivation to make the device know the service type, i.e., inventory only or command only or inventory + command.

Proposal 10: The service type of A-IoT (e.g., inventory only, command only, inventory + command) is not included in paging message.

3	Conclusion

In this contribution, we discussed the procedures and signalling for the initial access in A-IOT and have corresponding proposals.

Paging duplication detection:

Proposal 1: The paging duplication id is generated by CN and it is unique among multiple readers for the same session. Send LS to SA2 to inform them of our decision.

Proposal 2: The paging duplication times should be configured by CN.

Proposal 3: Paging repetition can also be stopped by CN via signaling to reader.

Proposal 4: The ID size for paging duplication detection is 4 bits.

Proposal 5: Multiple/subsequent AIoT paging is only used for paging triggered CB access, i.e., paging triggered CF access will not support multiple/subsequent AIoT paging.

Content of A-IOT paging message:

Proposal 6a:  A-IOT Paging message includes one device ID, or one device Group ID (or MASK) or NO device related ID (i.e., All). The device related ID is visible to IOT MAC and details of device related ID is transparent to A-IOT MAC in device.

Proposal 6b: Reader can decide whether CB or CF access should be used based on the information from CN, i.e., single device ID from CN will trigger CF access, otherwise trigger CB access.

Proposal 7: Access occasion configuration and slot number are configured in paging message.

Proposal 8: For CF access trigger, device specific access occasion and RN 16 if needed is included in paging message.

Proposal 9: The device will know CF access or CB access according to the device ID information and corresponding resource allocation in paging message, i.e., CF or CB access indication will not be included in paging message.

Proposal 10: The service type of A-IoT (e.g., inventory only, command only, inventory + command) is not included in paging message.

Annex: agreements in SI

In RAN2#125bis meeting, RAN2 reached following agreements on paging.

Agreements

- Legacy paging message for device will not be supported.

- Legacy paging occasion and legacy DRX for the device is not supported.  This doesn’t preclude solutions that address device monitoring (taking into account discussions from RAN1 as well).

- RAN2 assumes that the device will not support tracking/RAN area update procedure.

- For the case of reaching single or group of devices, an identifier may be required to identify the device/group of devices in the trigger message.    FFS pending the details from SA2

In RAN2#126 meeting, RAN2 reached following agreements on paging.

Agreements 1 RAN2 will study the following cases for AIoT paging message: - a message containing an ID of a single A-IoT device. - a message containing a group ID that maps to multiple A-IoT devices. - a message that does not contain an ID, i.e., addressed for all devices that can receive the AIoT message. - a message containing multiple IDs of A-IoT devices. Need to confirm the need for this use case based on SA2 discussion. What device ID and group ID and scenarios is depending on SA2 discussion. 2 AIoT paging message indicate information from which the device can determine resources to be used for response (D2R message). FFS how (e.g. implicit/explicit/configured/preconfigured) and what resources (dedicated and/or shared) are provided to the device taking into account RAN1 discussion. 3 From RAN2 perspective, we assume the device can receive as long as there is enough energy. We will wait for RAN1 further progress on device monitoring details.

In RAN2#127 meeting, RAN2 reached following agreements on paging.

Agreements:

- Support mechanism for reader to be able to trigger multiple/subsequent AIoT paging messages that are associated with the same request from the CN. Study how to avoid duplicated response from devices

=>	wait for further RAN1 progress on indication of the start of access occasion.

In RAN2#127bis meeting, RAN2 reached following agreements on paging.

Agreements

- The A-IoT paging message can include information to avoid duplicate response from the device to a reader.  This information should be short and simple.  FFS how to indicate.  Wait for further information and requirements from other WGs and make this decision in normative phase.

- Based on this information the device determines whether to skip sending the response to paging.

- From RAN2 perspective it is feasible to support paging multiple device IDs, however depending on TB size it may not be possible to include multiple device IDs.    RAN2 will determine the need for this after considering aspects related to RAN2 multiplexing and other WGs.

- If multiple device IDs in single paging is supported, if A-IoT paging message contains multiple device IDs, reader can configure either contention free RA or contention-based RA

- For all use cases, device determines the random access type from the paging message.   FFS whether it is explicitly or implicitly.   FFS whether we will down selection for all RA types (2step, 3step, CFRA) or try to unify the design for 2step, 3step

In RAN2#128 meeting, RAN2 reached following agreements on paging.

Agreements on paging

- An ID in paging message is beneficial to avoid duplicated response for same service request. FFS whether ID is generated by the reader or CN. FFS on the ID size.

- Capture the scenario that different readers may page the same device for same service request.  We will discuss this in WI phase (if in scope of WI) and consider progress from all the WGs

- Whether we include further information in paging message to indicate service type/command type or to indicate that further subsequent messages are coming can be discussed in WI phase once we understand more the paging design and AS ID.