3GPP TSG-RAN WG2 Meeting #129	R2-2500281

Athens, Greece, Feb. 17th – 21st, 2025

Source:	TCL

Title:	Discussion on the adjustment of Bj/PBR for delay-aware LCP

Agenda item:	8.7.4.1

Document for:	Discussion and decision

- Introduction

- According to the RAN2#128 meeting[1], the following agreements on LCP enhancements for XR are reached:

- How/when to apply additional priority As a baseline, the additional LCH priority is applied to both the first round and the second round of the LCP procedure. The UE does not fallback to the default LCH priority in the second round even if there is no more LCH priority-adjusted data after the first round. As an optional capability, the UE can also support to fallback to default priority in the 2nd round of LCP.

In this contribution, we aim to discuss about further details of handling of the additional priority for LCH, e.g. configuration of additional LCH priority, impact of additional LCH priority on LCP resource allocation (Bj/PBR).

- Discussion

  - Issue 1: Configuration of additional LCH priority

As a baseline, in order to support delay-aware LCP, it was agreed to introduce an additional LCH priority for a logical channel (LCH) when the LCH has LCH priority-adjusted data. According to section 5.4.3.1.1 of TS38.321, during the Logical Channel Prioritization (LCP) process, the network side (NW) configures relevant parameters for the UE through RRC messages, as follows:

The Logical Channel Prioritization (LCP) procedure is applied whenever a new transmission is performed.

RRC controls the scheduling of uplink data by signalling for each logical channel per MAC entity:

-	priority where an increasing priority value indicates a lower priority level;

-	prioritisedBitRate which sets the Prioritized Bit Rate (PBR);

-	bucketSizeDuration which sets the Bucket Size Duration (BSD).

The newly introduced additional LCH priority is also a parameter used by the UE for resource allocation in the LCP process. Considering consistency, it is suggested that the network configure the additional LCH priority for each LCH of the MAC entity through RRC message.

Proposal 1: It is suggested that the network configure the additional LCH priority for each LCH of the MAC entity through RRC message.

  - Issue 2: Impact of additional LCH priority on LCP resource allocation (Bj/PBR)

Regarding how to apply the additional priority for delay-aware LCP, another question arises: if an LCH uses the additional priority during the first round of resource allocation, should the PBR or Bj be updated accordingly?

According to TS38.321, when the network allocates a priority to the UE, it simultaneously allocates a PBR to prevent resource allocation starvation. The UE determines the total amount of resources allocated to the current LCH based on the PBR, Bj, and the total size of MAC SDUs of the logical channel. The specific process of the first round resource allocation in the LCP process is as follows:

1>Select the LCHs that meet the UL Grant limit conditions and have Bj > 0, and allocate resources in descending order of the LCH priority. If the PBR of a logical channel is set to infinity, the MAC entity shall allocate resources for all the data that is available for transmission on the logical channel before meeting the PBR of the lower priority logical channel(s); 2> decrement Bj by the total size of MAC SDUs served to logical channel j above; The following UE variable is used for the Logical channel prioritization procedure: - Bj which is maintained for each logical channel j. The MAC entity shall initialize Bj of the logical channel to zero when the logical channel is established. For each logical channel j, the MAC entity shall: 1> increment Bj by the product PBR × T before every instance of the LCP procedure, where T is the time elapsed since Bj was last incremented; 1> if the value of Bj is greater than the bucket size (i.e. PBR × BSD): 2> set Bj to the bucket size.

- As can be seen from the above process, for an LCH with a non - infinite PBR, the UE mainly allocates resources to the LCH based on its token Bj/PBR or the total size of MAC SDUs of the logical channel. When an LCH contains LCH priority-adjusted data, i.e., data with a remaining delay less than the threshold configured by the network, the UE uses the additional priority as the new priority for that LCH during resource allocation. During the first round resource allocation of the LCH in the LCP process, the following situations may occur:

- Figure 1: Three possible scenarios in the first round resource allocation of LCP

Case1: Bj < total size of LCH priority-adjusted data;

In this case, Bj/PBR cannot meet the resource requirements of the LCH priority-adjusted data in the current LCH. In this scenario, the UE needs to decide whether to adjust Bj/PBR so that all the LCH priority-adjusted data in the LCH can be allocated UL resources. To support the priority scheduling of delay-critical data, it is suggested that, on the premise of having enough remaining resources, Bj/PBR be adjusted so that all the LCH priority-adjusted data in the LCH can be scheduled.

Observation 1: The Bj/PBR of the LCH may not be able to meet the resource requirements of the LCH priority-adjusted data in the LCH.

Case2: total size of LCH priority-adjusted data < Bj < total size of available data on LCH;

In this case, Bj/PBR can meet the resource requirements of the LCH priority-adjusted data, but cannot meet the resource requirements of all the data that is available for transmission on the logical channel. In this scenario, the UE needs to decide whether to allocate resources to the non-LCH priority-adjusted data in the LCH. To consider the fairness among the LCHs in the UE, it is suggested that during the first round resource allocation, Bj/PBR is only used for the resource allocation of the LCH priority-adjusted data, i.e., the non-LCH priority-adjusted data is not considered for resource allocation.

Case3: total size of available data on LCH < Bj ;

- In this case, Bj/PBR can meet the resource requirements of all the data on the LCH. This scenario is similar to Case 2. It is suggested that during the first round resource allocation, Bj/PBR is only used for the resource allocation of the LCH priority-adjusted data, i.e., the non - LCH priority-adjusted data is not considered for resource allocation.

- Proposal 2: During the first round resource allocation of LCP, only the LCH priority-adjusted data of the LCH using the additional LCH priority should be considered for resource allocation.

- In summary, during the first-round resource allocation, when the Bj/PBR of the LCH cannot meet the resource requirements of all the LCH priority-adjusted data, Bj should be adjusted so that all the LCH priority-adjusted data in the LCH can be scheduled. When the Bj of the LCH is greater than the resource requirements of the LCH priority-adjusted data, the resources of Bj/PBR are limited to the resource allocation of the LCH priority-adjusted data in the LCH.

- Proposal 3: During the first round resource allocation of LCP, when the Bj/PBR of the LCH cannot meet the resource requirements of all the LCH priority-adjusted data, Bj should be adjusted.

- The adjustment solution of Bj/PBR should be considered. For example, the network could configure an additional PBR when assigning additional priority to the LCH, or the UE could adjust the Bj/PBR based on the proportion of LCH priority-adjusted data within the LCH, as authorized by the network during resource allocation. Through the adjustment of PBR, the value of Bj of the LCH can be adjusted, thus ensuring that the LCH priority-adjusted data can be fully scheduled.

Proposal 4: It is suggested that the UE update the Bj/PBR when using additional LCH priority, such as by using an additional PBR configured by the network or UE could adjust the Bj/PBR based on the proportion of LCH priority-adjusted data within the LCH.

- Conclusion

In this contribution, we provided our observations and proposals on LCP enhancements mechanism for XR as follows:

Observation 1: The Bj/PBR of the LCH may not be able to meet the resource requirements of the LCH priority-adjusted data in the LCH.

Proposal 1: It is suggested that the network configure the additional LCH priority for each LCH of the MAC entity through RRC message.

- Proposal 2: During the first round resource allocation of LCP, only the LCH priority-adjusted data of the LCH using the additional LCH priority should be considered for resource allocation.

- Proposal 3: During the first round resource allocation of LCP, when the Bj/PBR of the LCH cannot meet the resource requirements of all the LCH priority-adjusted data, Bj should be adjusted.

Proposal 4: It is suggested that the UE update the Bj/PBR when using additional LCH priority, such as by using an additional PBR configured by the network or UE could adjust the Bj/PBR based on the proportion of LCH priority-adjusted data within the LCH.

- References

- RAN2#128 meeting agreements.