3GPP TSG-RAN WG2 Meeting #129	R2-2500302

Athens, Greece, Feb 17th – 21st, 2025

Agenda item:	8.4.1

Source:	Huawei, HiSilicon

Title:	Discussion on SA2 LS on LP-WUS subgrouping

Document for:	Discussion and Decision

- Introduction

SA2 sent an LS to RAN2 on LP-WUS subgrouping with the following two questions and requested feedback from RAN2 [1].

Additionally, SA2 discussed the following aspects: 1) SA2 noted that the name “LP-WUS” can be mis-understood, as SA2 understands that the Wake Up Signal is transmitted with high power to enable a large coverage area, and low power refers to a low power wake up receiver used in the mobile. SA2 invites feedback on appropriate terminology and naming of the “LP-WUS” feature. SA2 will then align the terminology and name, based on the feedback. 2) Initial Registration may be performed on a pre-Rel 19 gNB (or a later gNB that does not support LP-WUS) that analyses the UE’s UE Radio Access Capabilities to create the UE Radio Capability for Paging Information and send it to the AMF, see clause 5.4.4.3 of TS23.501. It will be important that LP-WUS related aspects of UE Radio Capability for Paging Information are not lost when Initial Registration is performed on a pre-Rel-19 gNB. If the LP-WUS aspects are lost, then there can be paging problems, when for example, paging is performed by a gNB that supports LP-WUS. SA2 invites RAN2 and RAN3 to investigate this and provides solution if any issue is identified. SA2 was not sure whether this issue also impacts RedCap, eRedCap and other features that are signalled within the UE Radio Capability for Paging Information. SA2 invites RAN2 and RAN3 to investigate this.

In this contribution, we discuss the two questions and provide the draft reply LS.

- Discussion

  - “LP-WUS” terminology

Low Power-Wake Up Signal (LP-WUS) is a wake-up signal received with low power consumption and a Low Power-Wake Up Receiver (LP-WUR) is introduced in the mobile to receive the LP-WUS. As SA2 mentioned, “low power” in LP-WUS does not reflect the transmitted power by the network. However, it’s not necessary that LP-WUS is transmitted with high power. Higher power (e.g., power boosting) is just one possible way to ensure the coverage performance. There are also other ways to improve the coverage performance as agreed in RAN1#113:

RAN1#113 Agreement - Study the following techniques/mechanisms to enhance coverage performance of LP-WUS - low complex channel coding - FEC - spreading code in time domain - time domain repetition - with combining before or after ED - time-domain interleaving - Note: Also Manchester coding can be considered as channel code - non-contiguous transmission in the frequency domain - frequency domain repetition - frequency-hopping - power-boosting - transmit diversity - study whether any above techniques could be transparent to UE.

Therefore, from RAN2 perspective, there is no ambiguity caused by “LP-WUS”, it is suggested to continue using “LP-WUS” terminology. The draft reply LS is provided in appendix. It is suggested to also CC to RAN1/4 to align the terminology across all WGs.

Proposal 1: RAN2 to confirm that the “LP-WUS” terminology can be used, since LP-WUS is received with low power consumption by LP-WUR.

Proposal 2: RAN2 sends reply LS to SA2/RAN3/CT1, and CC RAN1 and RAN4 on “LP-WUS” terminology.

  - UE radio capability for paging information

2.2.1 LP-WUS

As mentioned in RAN1’s LS [2], UE may not support LP-WUS reception on all the bands supported by the UE. This means that LP-WUS capability would be a band-specific capability for the UE. Therefore, similar to Rel-17 PEI, a new Rel-19 capability of LP-WUS for paging information needs to be introduced.

According to the current mechanism, the UE shall report its LP-WUS capability via the UE capability reporting procedure. After receiving the UECapabilityInformation message, the gNB decodes it and accordingly generates the UERadioPagingInformation inter-node message which contains UE’s paging related capabilities including the LP-WUS capability. For example, LP-WUS related capability is added in UE-RadioPagingInfo-r17, or added in UERadioPagingInformation but outside of UE-RadioPagingInfo-r17. Then the gNB transfer this message to the CN or other gNBs.

Observation 1: LP-WUS related capability needs to be introduced in UE radio capabilities for Uu interface and UE radio paging capabilities for inter-node interface.

As illustrated in Figure 1, if the UE reports its LP-WUS capability to a gNB that cannot include LP-WUS capability in UERadioPagingInformation inter-node message as it cannot recognize LP-WUS capability. So the CN does not have the possibility to know about UE’s LP-WUS capability. When there is a paging for the UE, CN does not include any LP-WUS capability and the gNB sends paging message to the UE directly. However, LP-WUS UE is not aware of this and will monitor LP-WUS. As there is no LP-WUS indication, UE does not wake up the MR and thus miss paging.

Fig.1 An example for the LP-WUS capability maintenance and mismatch issue

Observation 2: UE will miss paging if the UE reports its LP-WUS capability to a gNB that cannot forward LP-WUS capability to CN as the CN does not receive LP-WUS capability.

Proposal 3: RAN2 to confirm that the UE will miss the paging message due to the mismatch of using LP-WUS between UE and gNB, if UE does not know the gNB is not able to forward the LP-WUS capability to CN.

To further consider the solution to address the capability mismatch issue between the UE and the network, we provide the following possible solutions.

- Option 1: Up to the network deployment to upgrade simultaneously

One simple approach is to leave it to the network vendors to upgrade their gNBs to support LP-WUS in a given area, e.g., across the entire TA area. Once all gNBs in a given area are upgraded simultaneously, the capability mismatch issue would no longer exist. This solution does not have any impact on the specifications. However, SA2 discussions emphasized multi-vendors scenario and achieving simultaneous upgrades for multi-vendors may be challenging.

Observation 4: Leaving to network deployment does not have any impact on the specifications, but may be challenging for multi-vendors scenario.

- Option 2: UE gets information from gNB whether LP-WUS capability can be maintained

To overcome the draw back from Option 1, another option is to let the UE know whether gNB will/is able to forward the reported LP-WUS capability to the CN. For example, when gNB enquires the UE to report LP-WUS capability or indicates the UE that it will/is able to forward LP-WUS capability, then UE reports LP-WUS capability and can use LP-WUS functionality in a cell that supports it.

Proposal 4: It is suggested to consider to specify a solution to solve the mismatch of LP-WUS capability between the UE and the network.

2.2.2 Other features

In SA2 LS [1]:

SA2 was not sure whether this issue also impacts RedCap, eRedCap and other features that are signalled within the UE Radio Capability for Paging Information. SA2 invites RAN2 and RAN3 to investigate this.

In UERadioPagingInformation message, as shown below, there are several capabilities for other features introduced after Rel-15, e.g., PEI, inactiveStatePO-Determination, (e)RedCap, 2 Rx XR, DL scheduling offset for NTN. RAN2 is requested to further investigate whether the same issue exists for these features.

In our understanding, the same issue exists for PEI and inactiveStatePO-Determination, the misalignment will lead to missing paging message. However, for (e)RedCap, the (e)RedCap UEs can only camp on/access to a cell that supports (e)Redcap, so the gNBs can only be the upgraded gNBs. As for 2 Rx XR and DL scheduling offset for NTN, it is an enhancement for paging transmission, if the gNB does not know these capabilities, it uses the legacy way to page UEs, there is no negative impact on the paging reception for UEs.

But this is out of the scope of LP-WUS WI and needs further discussion in general session. We can first focus on LP-WUS aspect. Based on the progress for LP-WUS, a consistent solution can be considered to reduce the specification impacts for the identified features with the same issue.

Proposal 5: It is suggested to focus on LP-WUS aspect first and a consistent solution for all identified features with the same issue can be considered in general agenda item.

- Conclusion

In this paper, we discuss the two questions from SA2, and have the following proposals.

“LP-WUS” terminology

Proposal 1: RAN2 to confirm that the “LP-WUS” terminology can be used, since LP-WUS is received with low power consumption by LP-WUR.

Proposal 2: RAN2 sends reply LS to SA2/RAN3/CT1, and CC RAN1 and RAN4 on “LP-WUS” terminology.

UE Radio Capability for Paging Information

Proposal 3: RAN2 to confirm that the UE will miss the paging message due to the mismatch of using LP-WUS between UE and gNB, if UE does not know the gNB is not able to forward the LP-WUS capability to CN.

Proposal 4: It is suggested to consider to specify a solution to solve the mismatch of LP-WUS capability between the UE and the network.

Proposal 5: It is suggested to focus on LP-WUS aspect first and a consistent solution for all identified features with the same issue can be considered in general agenda item.

- References

R2-2500050/S2-2412876, LS Reply on LP-WUS subgrouping.

R1-2407559 LS on LP-WUS operation in IDLE/INACTIVE mode

- Appendix

3GPP TSG-RAN WG2 Meeting #129	R2-240xxxx

Athens, Greece, Feb. 17th – 21st, 2025

Title:	[DRAFT] Reply LS on LP-WUS subgrouping

Response to:	R2-2500050/S2-2412876

Release:	Rel-19

Work Item:	NR_LPWUS-Core

Source:	Huawei, HiSilicon [to be RAN2]

To:	SA2

Cc:	RAN3, CT1, RAN1, RAN4

Contact Person:

#### Name:	Yiru Kuang

E-mail Address:	kuangyiru@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1. Overall Description:

RAN2 thanks SA2 for the LS in S2-2412876 and provides the following feedback:

- On the terminology and naming of the “LP-WUS”

LP-WUS is a wake-up signal that can be received by a receiver with low power consumption. From RAN2 perspective, it’s not necessary that LP-WUS is transmitted with high power, there are also other ways to improve the coverage performance. Thus, RAN2 understands that the “LP-WUS” terminology can be used.

- On the issue of UE radio capability for paging information

[To be added, based on RAN2 progress]

2. Actions:

To SA2

RAN2 respectfully asks SA2 to take the above answer into account.

3. Date of Next TSG-RAN WG2 Meeting:

3GPP RAN2#129bis	                       07 - 11 April, 2025		        China

3GPP RAN2#130	                       19 - 23 May, 2025		        Malta