3GPP TSG-RAN WG2 Meeting #129	R2-2500303

Athens, Greece, Feb 17th – 21st, 2025

Agenda item:	8.4.4

Source:	Huawei, HiSilicon

Title:	Further discussion on LP-WUS for RRC_CONNECTED mode

Document for:	Discussion and Decision

- Introduction

The following agreements are reached on LP-WUS for RRC_CONNECTED mode in RAN2#128 meeting [1].

- The drx-onDurationTimer is not started with Option 1-2 LP-WUS.

- For Option 1-2, network can configure whether UE reports periodic CSI/L1-RSRP during the time given by the configured drx-onDurationTimer, for the case when UE is outside C-DRX active time.

- For option 1-2, if UE receives DRX command MAC CE or Long DRX command MAC CE, UE stops the new timer triggered by LP-WUS.

- Don’t support Option 1-1 and Option 1-2 simultaneously configured for the same UE.

In this contribution, we further discuss higher layer design for CONNECTED mode.

- Discussion

  - Configuration for LP-WUS

Regarding the issue of how the NW provides the configuration to the UE, it can rely on the RRC measurement report. For example, the NW configures the UE to report measurement results based on some events and periodicity, then the NW can know whether it is suitable for the UE to start using LP-WUS functionality, e.g. the NW finds that the measurement result is good enough, or the change of the measurement result is small enough, the NW can configure the UE to use LP-WUS. However, the NW only knows the measurement results from MR. Sometimes even when the measurement result from MR is good, the link quality of LR can be bad due to the weaker tolerance for adjacent-channel interference for LR. In this case, the UE can inform the NW to stop using LP-WUS since the UE knows the link quality of LR, and also indicate whether the LP-WUS can be used again if it finds that the LR link quality is good enough. For example, the UE is monitoring LP-WUS and measures LP-SS/SSB by LR, once it finds that the LR link quality is not good enough, the UE switches to MR and sends assistance information to the NW to disable the LP-WUS.

Proposal 1: UE can send assistance information to the network indicating to disable the LP-WUS functionality or whether the LP-WUS can be enabled again.

After the NW provides the LP-WUS configuration to the UE by RRC signaling, the UE considers the LP-WUS is enabled and starts to use the LP-WUS functionality. However, one issue that needs discussion is whether the UE starts LP-WUS monitoring once it receives the LP-WUS configuration or not.

One option is to start LP-WUS monitoring after receiving the LP-WUS configuration. However, if the UE applies LP-WUS monitoring ahead of the NW, ongoing services from the NW may be affected which causes the UE’s data loss. On another hand, if the LP-WUS function takes effect on the NW before the UE, UE may miss the LP-WUS which leads to LR/MR state mismatch between the UE and the NW. Thus, it is beneficial that the UE and NW can align the starting point for monitoring LP-WUS, e.g. the UE and the NW start LP-WUS monitoring after a time period after the UE sends RRC reconfiguration complete message.

Another option is to keep the UE stay in C-DRX Active Time (i.e., using MR) after it receives the LP-WUS configuration. As the UE receives RRC reconfiguration message and needs to receive PDCCH scheduling PUSCH for sending RRC reconfiguration complete message, the UE can naturally stay in C-DRX Active Time. And there is no interruption for ongoing service if any. After the data transmission is finished and C-DRX Active Time ends, the UE goes back to LP-WUS monitoring according to the agreed UE’s behavior.

Proposal 2: RAN2 to further discuss the following alternatives:

- Alt1: UE starts LP-WUS monitoring after it receives the LP-WUS configuration, alignment of the starting point for LP-WUS monitoring needs to be further studied;

- Alt2: UE stays in MR (C-DRX Active Time) after it receives the LP-WUS configuration, UE goes back to LP-WUS monitoring after C-DRX Active Time ends according to the agreed UE’s behavior.

  - LP-WUS with DC/CA

In last RAN2 meeting, LP-WUS with DC/CA was discussed but there are no conclusions reached.

For both option 1-1 and option 1-2 - ?? The LP-WUS can also be configured with secondary per DRX group. - ?? In NR-DC case, LP-WUS can be configured separately in MCG and SCG, and LP-WUS could trigger the PDCCH monitoring of all activated serving cells within the same cell group.

In NR-DC, since there are two separate MAC entities, the LP-WUS can be configured and operated separately in MCG and SCG, without additional standardization work. This is applicable to both Option 1-1 and Option 1-2.

Proposal 3: For both Option 1-1 and Option 1-2, in NR-DC case, LP-WUS can be configured separately in MCG and/or SCG.

Within a cell group, LP-WUS can be further supported in CA scenario. Legacy DCP monitoring is only performed in PCell, and it triggers the PDCCH monitoring of all activated serving cells within the same cell group. The basic principle can be inherited to LP-WUS, i.e. the detection of LP-WUS triggers the PDCCH monitoring of all activated serving cells within the same cell group. However, the next question is that whether LP-WUS monitoring can only be performed in PCell. Currently, the common understanding for LP-WUS deployment is that LP-WUS may only be deployed on lower frequency (e.g. 700MHz), but some UEs are still expected to access on higher frequency (e.g. 3.5GHz) to avoid overload on lower frequency. In this case, UE’s PCell is on 3.5GHz but LP-WUS is not supported on that frequency. To maximum LP-WUS utilization, one way is to always perform a PCell change for the UE if LP-WUS can only be monitored in PCell/PSCell, it makes the procedure complicated and restricted. Another way is to allow LP-WUS to be monitored in SCell as well, the network can configure LP-WUS monitoring occasions in SCell, which is more flexible for network configuration. Thus LP-WUS monitoring is only performed in a serving cell, including PCell, PSCell or SCell.

Proposal 4: For both Option 1-1 and Option 1-2, LP-WUS monitoring is only performed in a serving cell including PCell, PSCell or SCell, and it triggers the PDCCH monitoring of all activated serving cells within the same cell group.

  - Coexistence with other features

    - Combination with Rel-16 DCP

In RAN2#126 meeting, we had the following agreement:

- RAN2 assume that legacy DCP and Option 1-1 is not configured simultaneously for a UE.

The same principle can be applied to Option 1-2, since Option 1-2 can technically provide better performance for power saving and latency compared with DCP. We don’t see the motivation to configure both legacy DCP and Option 1-2, besides, such combination gives additional complexity for the specification.

Proposal 5: Legacy DCP and Option 1-2 are not configured simultaneously for the same UE.

    - Combination with short DRX cycle

Legacy DCP and short DRX cycle are not configured simultaneously. During the Rel-16 discussion, some companies thought that if short DRX cycle is configured, it means there is delay sensitive service ongoing, and the performance for the service is more important than power saving. However, it is no harm if power saving can be obtained without negative impact on data transmission. LP-WUS configured with short DRX cycle simultaneously can be considered if it can be supported easily without impacts on performance and standardization work.

Proposal 6: If LP-WUS can be configured with short DRX cycle simultaneously for the same UE, no further specification impacts are expected.

    - Impacts on SPS and CG

LP-WUS indication in CONNECTED mode is to trigger MR PDCCH monitoring, the LP-WUS only impacts MR PDCCH monitoring. For DL configured resources, e.g. DL SPS, as the gNB allocates DL SPS resources despite knowing the LP-WUS is enabled, gNB expects the UE to perform legacy operation on DL SPS resources, and there is no mismatch between UE and gNB on LP-WUR/MR monitoring. From UE perspective, UE should monitor DL SPS resources by MR to avoid missing any DL data.

Proposal 7: DL SPS is not impacted by LP-WUS, i.e. UE still needs to wake up MR for reception on DL SPS resources.

However, for UL configured resources, e.g. configured grant Type 1 or configured grant Type 2 (UL SPS), sometimes there is a UL configured resource but UE does not have the UL data for transmission. If UL skipping is not enabled, the UE should still transmit padding bits in the UL configured resource which leads to additional power consumption. If the UE is operating with LP-WUR, it is beneficial to skip UL configured resource if no UL data available so the UE does not need to turn on the MR and keeps monitoring LP-WUS.

Proposal 8: For UL configured resources, RAN2 to discuss whether UE can skip UL configured resource if the UE is operating with LP-WUR and does not have the UL data for transmission.

  - Discussion on LS R1-2410909

Conclusion - For Option 1-1 of LP-WUS CONNECTED mode operation, RAN1 assumes existing DRX Command MAC CE and Long DRX Command MAC CE can be used to stop drx-onDurationTimer and drx-InactivityTimer for the DRX group - When UE is not in C-DRX active time, UE monitors LP-WUS - Send LS to RAN2 to ask whether RAN2 has same understanding or not. LS in R1-2410909. Agreement - For Option 1-2 of LP-WUS CONNECTED mode operation, for explicit indication to stop the new timer for PDCCH monitoring in Option 1-2 and drx-InactivityTimer for the DRX group, RAN1 identified the following potential solution - Use existing DRX Command MAC CE and Long DRX Command MAC CE - When UE is not in C-DRX active time, UE monitors LP-WUS - Send LS to RAN2 to ask the feasibility of the above potential solution. LS in R1-2410909.

RAN1 sent LS: 1) Regarding the above conclusion on Option 1-1, RAN1 would like to ask RAN2 whether RAN2 has same understanding or not; 2) Regarding the above agreement on Option 1-2, RAN1 would like to ask RAN2 the feasibility of the above potential solution.

Regarding Option 1-1, since operations introduced for Rel-16 DCP mechanism are taken as baseline, UE monitors DCP outside C-DRX Active Time, DCP can trigger the start of drx-onDurationTimer and UE is in C-DRX Active Time, UE skips DCP monitoring if the DCP occasion is overlapped with C-DRX Active Time. Thus, for Option 1-1, the above RAN1 understanding is correct. To be noted, “UE monitors LP-WUS” is understood as “UE monitors LP-WUS according to configured LP-WUS monitoring occasions”.

Proposal 9: RAN2 replies to RAN1 that RAN2 shares the same understanding with RAN1 for Option 1-1.

Regarding Option 1-2, in some cases, there is no following data to be received before C-DRX Active Time related timers expire, it wastes power if UE keeps monitoring PDCCH. To overcome this issue, similar mechanism can be supported naturally, e.g. MR (Long) DRX Command MAC CE can be used to stop drx- InactivityTimer, which may terminate the C-DRX Active Time and trigger LP-WUS monitoring. RAN2 also agreed that “For option 1-2, if UE receives DRX command MAC CE or Long DRX command MAC CE, UE stops the new timer triggered by LP-WUS” In last meeting. Thus, for Option 1-2, the potential solution provided by RAN1 is feasible.

Proposal 10: RAN2 to confirm that legacy (Long) DRX Command MAC CE can be used to stop drx-InactivityTimer, which may terminate the C-DRX Active Time and trigger LP-WUS monitoring without additional specification impacts.

Proposal 11: RAN2 replies to RAN1 that the potential solution provided by RAN1 for Option 1-2 is feasible from RAN2 perspective.

- Conclusion

In this paper, we discuss higher layer design for CONNECTED mode, and have the following proposals.

Configuration for LP-WUS

Proposal 1: UE can send assistance information to the network indicating to disable the LP-WUS or whether the LP-WUS functionality can be enabled again.

Proposal 2: RAN2 to further discuss the following alternatives:

- Alt1: UE starts LP-WUS monitoring after it receives the LP-WUS configuration, alignment of the start point for LP-WUS monitoring needs further studied;

- Alt2: UE keeps in MR (C-DRX Active Time) after it receives the LP-WUS configuration, UE goes back to LP-WUS monitoring after C-DRX Active Time ends according to the agreed UE behavior.

LP-WUS with DC/CA

Proposal 3: For both Option 1-1 and Option 1-2, in NR-DC case, LP-WUS can be configured separately in MCG and/or SCG.

Proposal 4: For both Option 1-1 and Option 1-2, LP-WUS monitoring is only performed in a serving cell including PCell, PSCell or SCell, and it triggers the PDCCH monitoring of all activated serving cells within the same cell group.

Coexistence with other features

Proposal 5: Legacy DCP and Option 1-2 are not configured simultaneously for the same UE.

Proposal 6: If LP-WUS can be configured with short DRX cycle simultaneously for the same UE, no further specification impacts are expected.

Proposal 7: DL SPS is not impacted by LP-WUS, i.e. UE still needs to wake up MR for reception on DL SPS resources.

Proposal 8: For UL configured resources, RAN2 to discuss whether UE can skip UL configured resource if the UE is operating with LP-WUR and does not have the UL data for transmission.

Discussion on LS R1-2410909

Proposal 9: RAN2 replies to RAN1 that RAN2 shares the same understanding with RAN1 for Option 1-1.

Proposal 10: RAN2 to confirm that legacy (Long) DRX Command MAC CE can be used to stop drx-InactivityTimer, which may terminate the C-DRX Active Time and trigger LP-WUS monitoring without additional specification impacts.

Proposal 11: RAN2 replies to RAN1 that the potential solution provided by RAN1 for Option 1-2 is feasible from RAN2 perspective.

- References

R2-2410912, Rel-18 MIMO, Rel-19 MIMO, LPWUS, and SBFD.