3GPP TSG-RAN WG2 Meeting #129	R2-2500320

Athens, Greece, Feb. 17th – 21st, 2025

Agenda Item:	8.1.2.2

Source:	Mediatek Inc.

Title:	Further Discussion on LCM for UE-side Model for AI-BM

Document for:	Discussion, Decision

Introduction

During the RAN2#127 meeting, the following procedures were agreed upon for the applicability report concerning the functionality-based LCM for the UE-side model, specifically for the use case of beam management. Additionally, one LS[1] was sent to RAN1, containing questions whose answers are essential for facilitating further progress in RAN2.

According to the reply LS[2] from RAN1 (R1-2410898), RAN1 provided agreements related to the questions from RAN2, although there was limited consensus on the answers to RAN2’s specific questions.

Agreement - In Step 3, following configurations are provided from NW to UE: - UE is allowed to do UAI reporting via OtherConfig, - The applicability report is based on A) and/or B) - It is up to RAN 2 to design the container - A) one or more of CSI-ReportConfig for inference configuration (wherein the associated ID may be configured in CSI framework as working assumption applied) - Note: CSI report configuration for UE-side model inference can’t be activated immediately upon receiving Step 3 - B) One set or multiple sets of inference related parameters for applicability report only (not for inference) - It is up to RAN2 to design the container. - The set of inference related parameters selected from the IEs in/or the IEs referred by CSI-ReportConfig as a starting point, e.g., - the associated ID - Note: this doesn’t imply the associated ID is mandatory - Set A related information - Set B related information - Report content related information - For BM-Case 2, - Time instances related information for measurements - Time instances related information for prediction - In Step 4, UE reports applicability for all the above A) one or more CSI-ReportConfig and/or B) set(s) of inference related parameters - FFS on whether/what other information along with the applicability is needed - If A) is configured in Step 3, - Applicable aperiodic CSI Report and semi-persistent CSI report can be activated/triggered by NW after the applicability reported. - Applicable periodic CSI Report is considered as activated only if the applicability of the corresponding CSI-ReportConfig is reported in RRCReconfigurationComplete. - In Step 5, NW can optionally configure CSI-ReportConfig for inference configuration in RRCReconfiguration, where the associated ID may be configured in CSI framework as working assumption applied. - Note: Step 5 may be optional if UE has already been configured with CSI-ReportConfig in Step 3 Agreement For beam management, multiple CSI reports for inference for UE-side model can be configured/activated/triggered, which is up to UE capability. Conclusion For the CSI-ReportConfig for inference configuration provided in Step 5, - aperiodic CSI Report and semi-persistent CSI report can be activated/triggered by NW after RRCReconfigurationComplete. - periodic CSI Report is considered as activated after RRCReconfigurationComplete. - Note: UE is not expected to be configured with a CSI-ReportConfig for inference configuration for a non-applicable set of inference parameters or a non-applicable CSI-ReportConfig Any specification impact is a separate discussion

In this contribution, we further discuss how inference configurations and their applicability are managed and reported by the UE, taking into consideration the reply LS[2] from RAN1 (R1-2410898).

.Discussion

Terminology of ‘Functionality’

According to the reply LS from RAN1, RAN1 discussed the terminologies defined by RAN2 and make following agreements:

Agreement Incorporating below text into the general part of the LS In RAN1’s discussion of RAN 2 terminologies on beam management, - The concept/terminology “functionality“ of Supported functionalities may refer to UE-capability information/parameters i.e., Rel-19 AI/ML-specific FGs - The concept/terminology “ functionality“ of Applicable functionalities may refer to CSI-ReportConfig for inference configuration or a set of inference related parameters or information/parameters indicated by UE - The Activated functionalities may be enabled based on CSI framework. Therefore, the meaning and the granularity of “functionality“ for Applicable functionalities, Activated functionalities and Supported functionalities may or may not be the same, depends on certain option in RAN1, and the discussion is still ongoing.

Although RAN1 created the term ‘functionality’, it lacks a clear definition, and discussions on its granularity have been suspended for some time. Additionally, the term 'functionality' may have different meanings for various procedures, such as UE capability report, applicability report, and activation/deactivation.

It appears that RAN1 has abandoned the term 'functionality' and is now focusing on the impact on signaling for inference configuration. According to RAN1 agreement, UE reports applicability for all the above A) one or more CSI-ReportConfig and/or B) set(s) of inference related parameters in step 4.

From the RAN2 perspective, it is unclear whether it is still efficient to continue using this terminology and define procedures based on it. Alternatively, RAN2 could focus on inference configuration or inference-related parameters (not for inference), based on which the applicability report is generated.

Observation 1: For different procedures, e.g. UE capability report, applicability report and activation/deactivation, the term of ‘functionality’ in RAN1 may have different meaning. RAN1 has abandoned the term of ‘functionality’ and focuses more on the impact on the signaling for inference configuration.

Proposal 1: RAN2 confirms that the applicability refers to the applicability of the inference configuration and/or inference related parameters, i.e. the UE reports the applicability for A) one or more CSI-ReportConfig and/or B) set(s) of inference related parameters (not for inference) provided in step3.

Applicability Determination

In RAN2#127 meeting, RAN2 made following agreement on how UE determines the applicability:

- UE decides the applicable functionalities based on NW-side additional conditions (if provided), UE-side additional conditions (internally known by UE) and model availability in device. FFS whether other configuration can considered by UE (e.g. inference configuration). FFS how the applicable functionality is decided if NW-side additional condition is not provided in step 3.

According to RAN1 discussion, RAN1 did not have agreement on the content of NW-side additional condition.  there is no consensus yet on whether the network-side condition is mandatory or optional. There is no conclusion yet on whether it is feasible or not for the UE to decide the applicability without it.

RAN1 agreed to support associated ID and it can be used to ensure the consistency of NW-side additional condition across training and inference for UE-sided model for BM-Case 1 and BM Case 2. However, it doesn’t imply the associated ID is mandatory to be provided by the network. Therefore, how the UE determines the applicable inference configurations and/or inference-related parameters need to considered. Just illustrated in Figure 1, different CSI-ReportConfigIds may be linked to different associated IDs.

Figure 1 Applicability Determination w/o and w/ NW-sided additional condition in Step 3

- Option 1 w/o network-side additional condition information: If the network condition and/or the associated IDs are not provided, the UE determines the applicability based on the model availability and the UE-side additional condition. The prerequisite is that the network needs to maintain the linkage between the inference configurations and/or sets of inference-related parameters and the network-side additional condition and/or the associated IDs. Then, the network chooses the truly applicable inference configurations according to network-side additional condition. In Figture 1-A), gNB provides CSI-ReportConfig with CSI-ReportConfigId 0~9 and the UE replies that CSI-ReportConfig with CSI-ReportConfigId 0~6 are applicable without the associated ID information.

- Option 2 w/ network-side additional condition information: If the network condition or the associated IDs are provided, the UE determines the applicability based on the model availability, the UE-side additional condition, and the network-side additional condition or the associated ID. The UE can further filter the applicability based on the network-side additional condition or the associated ID. The prerequisite is that the linkage between the inference configurations and/or sets of inference-related parameters and the network-side additional condition as well as the associated IDs is known to the UE, for example, through the Model Identification procedure, such as MI-Option1. In Figture 1-B), gNB provides CSI-ReportConfig with CSI-ReportConfigId 0~9 and additionally the associated ID=2. The UE replies that CSI-ReportConfig with CSI-ReportConfigId 2~5 are applicable with the associated ID.

In option 1, the UE determines applicability based on model availability and UE-side additional conditions, which may result in a broader set of applicable inference configurations. While in Option 2, the UE can further filter applicability based on network-side additional conditions or associated IDs, resulting in a more precise and potentially smaller set of applicable inference configurations or sets of inference-related parameters being reported to the network.

For option 1, considering the dynamic changes in UE internal conditions, any changes in applicability could trigger an applicability report. For example, a change of applicable inference configurations from [CSI-ReportConfigId 0~6] to [CSI-ReportConfigId 0~5] could lead to an applicability report. The frequent applicability report is also a burden to both the UE and network.

For option 2, even though the UE internal conditions are dynamically changing, the UE only needs to consider applicability under specific NW-side conditions, if the information is provided. Therefore, in the same example, a change of applicable inference configurations from [CSI-ReportConfigId 0~6] to [CSI-ReportConfigId 0~5]  would not trigger an applicability report for the UE.

Observation 2: Compared with Option 1, Option 2 can reduce the frequency of the applicability report and result in a more precise and smaller set of applicable inference configurations and/or sets of inference-related parameters being reported to the network.

Proposal 2: RAN2 considers both option 1 and option 2 for the UE to determine the appliable inference configurations and/or sets of inference-related parameters provided in step 3:

- Option 1 w/o network-side additional condition information: the UE determines the applicability based on the model availability and the UE-side additional condition. FFS how to reduce the frequency of the applicability report.

- Option 2: w/ network-side additional condition information: on top of Option 1, the UE furtherfilters the applicability based on the network-side additional condition and/or the associated ID.

Proposal 3: In option 2 that the network-side additional condition information is provided to the UE, it is assumed that the linkage between the inference configurations and/or sets of inference-related parameters and the network-side additional condition and/or the associated IDs is known to the UE.

Applicability Report

Since the UE reports the applicability for inference configuration and/or inference-related parameters in step 4, certain IDs are required to represent the corresponding inference configurations or sets of inference-related parameters to save signaling overhead. If one or more CSI-ReportConfig is provided as the inference configuration in step 3, the existing CSI-ReportConfigId can be used to identify each CSI-ReportConfig. Therefore, if sets of inference-related parameters are provided in step 3, a new type of ID is required to identify the sets of inference-related parameters. The UE can reports the applicability in the form of IDs, which can be decided in stage-3.

Proposal 4: The UE reports the applicability in the form of IDs, specifically the CSI-ReportConfigIds for applicable inference configurations and/or the IDs for applicable sets of inference-related parameters.

In previous RAN2 meetings, following agreements were made regarding the applicability change from applicable to non-applicable, and vice versa.

- UE can report to the network when an applicable AI functionality becomes non-applicable. FFS how this is signaled (e.g. explicitly/implicitly). - When a functionality configured by the network to be reported via UAI, becomes from non-applicable to applicable, the UE can reports it to the network. - When a functionality becomes non-applicable the UE doesn’t autonomously deactivate. NW is expected to deactivate active functionality when it receives report from UE that it is non-applicable.

Considering the dynamic changes in the UE-side additional condition, it is possible that none of the inference configurations provided in step 3 or step 5 is applicable for the UE.

- Step 3 Scenario: If none of the inference configurations provided in step 3 are applicable, the UE should keep the inference configurations and report 0 applicable inference configurations in step 4.

- Step 5 Scenario: If none of the inference configurations provided in step 5, which are intended to provide the final inference configurations, are applicable, the UE should behave the same as in the former case and should not trigger the re-establishment procedure. Instead, the network may trigger a fallback procedure and provide legacy configurations.

Proposal 5: If none of the inference configurations provided in step 3 are applicable, the UE keeps the inference configurations and reports 0 applicable inference configurations in step 4.

Proposal 6: If none of the inference configurations provided in step 5 are applicable, the UE keeps the inference configurations and reports 0 applicable inference configurations. Re-establishment should not be triggered.

Conclusion

Terminology of ‘Functionality’

Observation 1: For different procedures, e.g. UE capability report, applicability report and activation/deactivation, the term of ‘functionality’ in RAN1 may have different meaning. RAN1 has abandoned the term of ‘functionality’ and focuses more on the impact on the signaling for inference configuration.

Proposal 1: RAN2 confirms that the applicability refers to the applicability of the inference configuration and/or inference related parameters, i.e. the UE reports the applicability for A) one or more CSI-ReportConfig and/or B) set(s) of inference related parameters (not for inference) provided in step3.

Applicability Determination

Observation 2: Compared with Option 1, Option 2 can reduce the frequency of the applicability report and result in a more precise and smaller set of applicable inference configurations and/or sets of inference-related parameters being reported to the network.

Proposal 2: RAN2 considers both option 1 and option 2 for the UE to determine the appliable inference configurations and/or sets of inference-related parameters provided in step 3:

- Option 1 w/o network-side additional condition information: the UE determines the applicability based on the model availability and the UE-side additional condition. FFS how to reduce the frequency of the applicability report.

- Option 2: w/ network-side additional condition information: on top of Option 1, the UE furtherfilters the applicability based on the network-side additional condition and/or the associated ID.

Proposal 3: In option 2 that the network-side additional condition information is provided to the UE, it is assumed that the linkage between the inference configurations and/or sets of inference-related parameters and the network-side additional condition and/or the associated IDs is known to the UE.

Applicability Report

Proposal 4: The UE reports the applicability in the form of IDs, specifically the CSI-ReportConfigIds for applicable inference configurations and/or the IDs for applicable sets of inference-related parameters.

Proposal 5: If none of the inference configurations provided in step 3 are applicable, the UE keeps the inference configurations and reports 0 applicable inference configurations in step 4.

Proposal 6: If none of the inference configurations provided in step 5 are applicable, the UE keeps the inference configurations and reports 0 applicable inference configurations. Re-establishment should not be triggered.

Reference

[1] R2-2407848, LS on applicable functionality reporting for beam management UE-sided model

[2] R1-2410898, Reply LS on applicable functionality reporting for beam management UE-sided model