3GPP TSG-RAN WG2 Meeting #129	R2-2500321

Athens, Greece, Feb. 17th – 21st, 2025

Agenda Item:	8.1.3

Source:	Mediatek Inc.

Title:	Further Discussion on NW-side Data Collection

Document for:	Discussion, Decision

Introduction

In the RAN2#127bis meeting, following agreements were reached specifically for network-side training data collection pertaining to the use case of AI BM.

Agreements on NW side data collection

- Periodic logging is supported for training data collection procedure in R19

- Event-triggered data logging will be supported.  At least radio condition based event triggered logging will be supported.  FFS the details of radio condition based event.  FFS if other events are supported.

- Periodic reporting of logged data is not supported.

- On-demand reporting of the logged measurements will be specified

- UEInformationRequest/UEInformationResponse is used for on-demand reporting of AI/ML training data collection.   FFS of details of the message

- The UE can indicates the availability of logged data to the network to assist network to trigger UEInformationRequest.  FFS trigger/definition of availability indication.   and FFS how data availability indication is sent to the network.

- Low priority SRB will be used.    FFS new SRB or use of SRB4

- For data collection for both NW-sided/UE sided BM model training, at least L1-RSRPs and/or beam-IDs needs to be collected by UE.  FFS if other data needs to be collected based on RAN1 progress.

Subsequently, in the RAN2#127 meeting, further agreements were made regarding the data collection of network-side model training.

Agreements on NW side data collection - Focus on the following three radio condition event based logging - L3 serving cell measurement based (e.g. X1/X2 similar to A1/A2) - Beam based events (e.g. beam becomes top-1 beam and number of measurements is less than configured value) - L1 beam level measurement - Measurements on aperiodic CSI resources are not reported for NW sided data collection. - Data collection is controlled by the network. The UE will not autonomously stop when low power state is detected. - The UE reports to the network when the power state is low. We will not specify how the UE determines low power state. The network should de-configure the data collection (this can be captured in stage 2). - The UE reports to the network when buffer is or may become full. FFS when it reports (before and/or after). - The UE can report the reason for triggering of indication for the status (e.g. low power state, low memory). FFS how this is signalled and if the reporting can be part of availability indication.

This paper delves into the specifics of data logging and reporting for data collection aimed at network-side model training, discusses the handling of logged data and measurements during RRC state transitions, and discusses the data content for both network-side and UE-side data collection.

Discussion

Logged Data Handling during RRC State Transition

RRC_CONNECTED (Incl. Handover)

Normal Operation in RRC_CONNECTED

In RAN2#126 meeting, it was agreed that Immediate MDT is the baseline framework for OAM-centric data collection for the training of a network-sided model. In the following meetings, RAN2 discussed how to support data logging and reporting on top of it and agreed that on-demand reporting of the logged data will be specified.

For logged MDT, the UE maintains only one RAT-specific logged measurement configuration at a time. When the network provides a new configuration, it completely replaces any previously configured logged measurement configuration. This means that only the new configuration will take effect. Additionally, any logged measurements associated with the previous configuration will be cleared. It is the network's responsibility to retrieve any relevant data before providing a new configuration.

Observation 1: In logged MDT, the UE maintains only one logged measurement configuration. It is the network's responsibility to retrieve any relevant data before providing a new configuration.

The same principle should be applied to data collection for network-side model training. Firstly, networks typically conduct data collection targeted to a specific RAN configuration or scenario, such as set A or set B configuration for AI BM use cases from large amount of UEs. Secondly, it simplifies the management of measurement configurations and logged data for both the network and the UEs. Otherwise, both the UE and the network would need to maintain the association between the logged measurement configuration and the logged data, and the network would need to differentiate under which logged measurement configuration the reported data is targeted. Thirdly, considering that there is no latency requirement for data collection for model training, supporting the reconfiguration of the measurement configuration before retrieving the logged data offers little benefit. Therefore, for data collection configuration for network-side model training, only one logged measurement configuration should be in effect. It should be noted that multiple measurement resources and measurement reports can be configured in the logged measurement configuration, such as CSI-MeasConfig.

Observation 2: It simplify the management of the data collection configuration and the logged data if the UE maintains only one logged measurement configuration for the data collection for network-side model training:

- When the network provides a new logged measurement configuration, it completely replaces any previously configured measurement configuration.

- When the network provides a new logged measurement configuration, any logged measurements associated with the previous configuration will be cleared.

- It’s up to the network to retrieve any relevant data before providing a new configuration for data collection.

In logged MDT, the release of a logged measurement configuration in the UE is achieved either by configuration replacement when the configuration is overwritten or by configuration clearance when a duration timer stops or expires. For data collection for network-side model training, since the data collection is conducted exclusively in the RRC_CONNECTED state and any control over the data collection configuration and logged data can be managed through network signaling, there is no need for a duration timer to handle the data collection operation. The duration timer was originally intended to control UE data logging in the IDLE/INACTIVE state. Therefore, explicit release of the measurement configuration needs to be supported for data collection for network-side model training. When the measurement configuration is released, the data associated to the configuration is also discarded.

Observation 3: Since the data collection for network-side model training is conducted exclusively in the RRC_CONNECTED state, any control over the data collection configuration and logged data can be managed through network signaling.

Handover

During normal handover procedure, the UE is still in RRC_CONNECTED state. The above principle can still be applied. Same as the normal RRC reconfiguration procedure, the network has the freedom to decide whether to provide a new measurement configuration for data collection for network-side model training during handover.

If the handover command provides a new configuration, the UE replaces any previously configured measurement configuration with the new configuration and discards any logged measurements associated with the previous configuration. If the network is not willing to initiate data collection in the target cell, the network will not provide new measurement configuration and may request the logged data after handover. In this case, the UE keeps the logged data associated to the original measurement configuration and send the data availability indication in the RRCReconfigurationComplete message to the target cell.

Proposal 1: In RRC_CONNECTED state (incl. handover), the UE maintains only one logged measurement configuration for data collection for network-side model training. When the network provides a new logged measurement configuration, it completely replaces any previously configured logged measurement configuration.

Proposal 2: In RRC_CONNECTED state (incl. handover), when the network provides a new logged measurement configuration for data collection for network-side model training, any logged measurements associated with the previous configuration will be cleared.

Proposal 3: In normal operation of RRC_CONNECTE state (incl. handover), it’s up to the network to retrieve any relevant data before providing a new logged measurement configuration for data collection for network-side model training.

Proposal 4: In RRC_CONNECTED state (incl. handover), the logged measurement configuration can be released explicitly via RRCReconfiguration message by the network. The UE discards the associated data when the measurement configuration is released.

Proposal 5: During handover, if the original logged measurement configuration still takes effect, i.e. the network doesn’t provide new logged measurement configuration or release the original logged measurement configuration in handover command, the UE sends the data availability indication in the RRCReconfigurationComplete message to the target cell.

The terminology of ‘functionality’ in RAN1 discussions is confusing and may refer to certain CSI-ReportConfig or inference-related parameters. It does not exactly align with RAN2’s modeling of ‘functionality’. Therefore, it is unclear whether the logged measurement configuration for data collection for network-side model training is per functionality, per AI/ML enabled feature/FGs or even per UE, which can be considered in stage-3 RRC siganling.

Proposal 6: It is FFS whether the measurement configuration for data collection is per ‘functionality’, per AI/ML-enablde feature/FG or per UE.

RRC Re-establishment

When the UE is in RRC_CONNECTED state and endures failures, e.g. RLF or handover failure, the UE will initated RRC reestablishement procedure. In this case, the UE can send data availability indication in the RRCReestablishmentComplete message to the cell where the RRC connection is sucessfully re-established. Meanwhile, the UE keeps the original logged measurement `configuration and the relevant logged data associated to the original configuration. In this case, the network can decide whether to retrieve the stored data from the recovered UE or release the logged measurement configuration and the logged data.

Proposal 7: When the UE initiates the Re-establishment procedure, the UE keeps the original logged measurement configuration and the relevant data.  The UE sends the data availability indication to the network in the RRCReestablishComplete message.

State Transition

In logged MDT, under normal operation, the network should retrieve all stored data and logs before transmitting an RRCRelease message to the UE. Given that on-demand requests from the network are supported for data collection for model training, the network should adhere to the same requirement, ensuring no residual data or logs are left behind when transitioning the UE to the RRC_INACTIVE or RRC_IDLE state. It is the network's responsibility to collect any stored data/logs before transmitting the RRCRelease message. In abnormal case that there is residual data or logs left when receiving the RRCRelease message, the UE discards the stored data or logs.

From complexity point of view, we also don’t see much benefits to make the UE store the measurement configuration and the logged the data when the UE is RRC_INACTIVE mode, since the collected data may targets to specific site/area. Considering the UE mobility, the UE may moves to another unrelavant area and the measurement configuration is not valid anymore. In order to simplify the data collection operation, the same operation is applied to RRC_IDLE and RRC_INACTIVE state.

Proposal 8: In normal operation, the network should retrieve all stored data and logs before transmitting an RRCRelease message to the UE, which moves the UE from RRC_CONNECTED to RRC_IDLE/RRC_INACTIVE state.

Proposal 9: When the UE receives RRCRelease message which moves the UE from RRC_CONNECTED to RRC_IDLE/RRC_INACTIVE state, the UE releases the configuration for data collection and discard the logged data.

Data Content of Data Collection for Model Training

In RAN1, the discussion on data content and the corresponding report configuration primarily focuses on inference rather than model training.

In the RAN1 AI-BM FL summary[1], the following types of data content for network-side model training via higher layer signaling were discussed during the RAN1#119 meeting for BM-Case 1, but no agreement was reached. There has been no discussion on BM-Case 2 yet. It is expected that RAN1 will continue the discussion, and the down-selection of different types of data content may take place.

Send LS to RAN 2, to inform RAN 2 that from RAN 1 perspective, for content for data collection for training for NW-sided model via higher layer signaling, for BM-Case1, the following three types of contents for each instance can be considered: - Type 1: All L1-RSRPs of measured based the resources (for Set A/B) configured to UE - Type 2: All L1-RSRPs measured based on resources (for Set B) configured to UE, and beam information of K beams based on some resources (for Set A) configured to UE - Type 3: All L1-RSRPs measured based on resources (for Set B) configured to UE, and beam information and L1-RSRPs of K beams based on some resources (for Set A) configured to UE - Wherein the K beams are the beams with the largest K L1-RSRPs based on the resources (for Set A) in one instance - K is configured by NW.

The reason for discussing the three types of data content is that for AI-BM, generally only two types of AI algorithms are used: regression and classification. In both cases, the inputs are the L1-RSRPs of Set B.

- For regression algorithms, the outputs are the L1-RSRPs of Set A, corresponding to Type 1 data content.

- For classification algorithms, the outputs are the beam information (beam-IDs) of K beams (on resources for Set A), corresponding to Type 2 data content.

- If both regression and classification algorithms are used, Type 3 data content will be employed.

Since RAN2 already agreed that for data collection for both NW-sided/UE sided BM model training, at least L1-RSRPs and/or beam-IDs needs to be collected by UE.  FFS if other data needs to be collected based on RAN1 progress. Based on RAN1 discussion, there is no other data needs to be collected for AI-BM.

One advantage of Type 1 is that Set A/Set B information is not required. Therefore, the data content can be used to train AI models with different Set A/Set B configurations. Additionally, the data content can be utilized for both model training and model monitoring. However, the drawback is that it results in significant signaling overhead. The advantage of Type 2 is that it significantly reduces the signaling overhead, however, it requires the provision of Set A/Set B information.

Observation 4: For content for data collection for training for NW-sided model via higher layer signaling, for BM-Case1, Type 2/3 data content requires Set A/Set B configuration information, while Type 1 data content does not. However, RAN1 has not reached a conclusion regarding the data content.

Proposal 10: For the data content for both NW-side/UE-sdie model training, RAN2 waits for RAN1’s progress on BM-Case 1 and BM-Case 2.

If Type 1 was selected as the data content, for data collection for network-side model training, the information related to Set A and Set B is not required to be known to the UE, as how to configure Set A and Set B during inference operation is network implementation. The network side can choose different SetA/SetB configurations and traing the NW-side accordingly. During inference operation, the network only needs to provide measurement resources on Set B for UE to measure and report, which are used as inference inputs at the gNB.

However, the case would be different for the UE-side model training, as the UE needs to know the Set A/Set B configuration that the network may configure later for model inference and train the AI models targeted to the specific configuration. How to the UE know the Set A/Set B configuration for model training needs to be discussed. Following options can be considered:

- Option 1: Set A and Set B configurations are provided to the UE by gNB over the air interface.

- Option 2: Set A and Set B configurations are shared to the UE-side server by gNB via CN/OAM.

Considering the data collection for model training is targeted to a group of UEs instead of a specific UE, option 1 will result in lots of signaling overhead, as such information also needs to be attached to the collected data from the UE for purpose of dataset categorization. Option 2 more aligns the UE-sided data collection solution 2 and 3 assuming that there is interface between the UE-side server for data collection for model training and CN/OAM.

Proposal 11: For data collection for UE-side model training, the Set A and Set B configurations needs to be known to the UE-side.

Proposal 12: For data collection for UE-side model training, RAN2 consifer following options to make the UE know the Set A and Set B configurations:

- Option 1: Set A and Set B configurations are provided to the UE by gNB over the air interface.

- Option 2: Set A and Set B configurations are shared to the UE-side server by gNB via CN/OAM.

Conclusion

Logged data handling

Observation 1: In logged MDT, the UE maintains only one logged measurement configuration. It is the network's responsibility to retrieve any relevant data before providing a new configuration.

Observation 2: It simplify the management of the data collection configuration and the logged data if the UE maintains only one logged measurement configuration for the data collection for network-side model training:

- When the network provides a new logged measurement configuration, it completely replaces any previously configured measurement configuration.

- When the network provides a new logged measurement configuration, any logged measurements associated with the previous configuration will be cleared.

- It’s up to the network to retrieve any relevant data before providing a new configuration for data collection.

Observation 3: Since the data collection for network-side model training is conducted exclusively in the RRC_CONNECTED state, any control over the data collection configuration and logged data can be managed through network signaling.

Proposal 1: In RRC_CONNECTED state (incl. handover), the UE maintains only one logged measurement configuration for data collection for network-side model training. When the network provides a new logged measurement configuration, it completely replaces any previously configured logged measurement configuration.

Proposal 2: In RRC_CONNECTED state (incl. handover), when the network provides a new logged measurement configuration for data collection for network-side model training, any logged measurements associated with the previous configuration will be cleared.

Proposal 3: In normal operation of RRC_CONNECTE state (incl. handover), it’s up to the network to retrieve any relevant data before providing a new logged measurement configuration for data collection for network-side model training.

Proposal 4: In RRC_CONNECTED state (incl. handover), the logged measurement configuration can be released explicitly via RRCReconfiguration message by the network. The UE discards the associated data when the measurement configuration is released.

Proposal 5: During handover, if the original logged measurement configuration still takes effect, i.e. the network doesn’t provide new logged measurement configuration or release the original logged measurement configuration in handover command, the UE sends the data availability indication in the RRCReconfigurationComplete message to the target cell.

Proposal 6: It is FFS whether the measurement configuration for data collection is per ‘functionality’, per AI/ML-enablde feature/FG or per UE.

Proposal 7: When the UE initiates the Re-establishment procedure, the UE keeps the original logged measurement configuration and the relevant data.  The UE sends the data availability indication to the network in the RRCReestablishComplete message.

Proposal 8: In normal operation, the network should retrieve all stored data and logs before transmitting an RRCRelease message to the UE, which moves the UE from RRC_CONNECTED to RRC_IDLE/RRC_INACTIVE state.

Proposal 9: When the UE receives RRCRelease message which moves the UE from RRC_CONNECTED to RRC_IDLE/RRC_INACTIVE state, the UE releases the configuration for data collection and discard the logged data.

Data Content of Data Collection for Model Training

Observation 4: For content for data collection for training for NW-sided model via higher layer signaling, for BM-Case1, Type 2/3 data content requires Set A/Set B configuration information, while Type 1 data content does not. However, RAN1 has not reached a conclusion regarding the data content.

Proposal 10: For the data content for both NW-side/UE-sdie model training, RAN2 waits for RAN1’s progress on BM-Case 1 and BM-Case 2.

Proposal 11: For data collection for UE-side model training, the Set A and Set B configurations needs to be known to the UE-side.

Proposal 12: For data collection for UE-side model training, RAN2 consifer following options to make the UE know the Set A and Set B configurations:

- Option 1: Set A and Set B configurations are provided to the UE by gNB over the air interface.

- Option 2: Set A and Set B configurations are shared to the UE-side server by gNB via CN/OAM.

Reference

[1] R1-2410892, FL summary #5 for AI/ML in beam management,Samsung (Moderator)